ETSI ES 201 873-1 V4.12.1 (2020-05)

Methods for Testing and Specification (MTS);

The Testing and Test Control Notation version 3;

Part 1: TTCN-3 Core Language

ETSI Standard

Reference

RES/MTS-201873-1V4.12.1

Keywords

language, methodology, testing, TTCN-3

ETSI

650 Route des Lucioles

F-06921 Sophia Antipolis Cedex - FRANCE

Tel.: +33 4 92 94 42 00   Fax: +33 4 93 65 47 16

Siret N° 348 623 562 00017 - NAF 742 C

Association à but non lucratif enregistrée à la

Sous-Préfecture de Grasse (06) N° 7803/88

Important notice

The present document can be downloaded from:
http://www.etsi.org/standards-search

The present document may be made available in electronic versions and/or in print. The content of any electronic and/or print versions of the present document shall not be modified without the prior written authorization of ETSI. In case of any existing or perceived difference in contents between such versions and/or in print, the prevailing version of an ETSI deliverable is the one made publicly available in PDF format at www.etsi.org/deliver.

Users of the present document should be aware that the document may be subject to revision or change of status. Information on the current status of this and other ETSI documents is available at https://portal.etsi.org/TB/ETSIDeliverableStatus.aspx

If you find errors in the present document, please send your comment to one of the following services:
https://portal.etsi.org/People/CommiteeSupportStaff.aspx

Copyright Notification

No part may be reproduced or utilized in any form or by any means, electronic or mechanical, including photocopying and microfilm except as authorized by written permission of ETSI.
The content of the PDF version shall not be modified without the written authorization of ETSI.
The copyright and the foregoing restriction extend to reproduction in all media.

© ETSI 2020.

All rights reserved.

DECT™, PLUGTESTS™, UMTS™ and the ETSI logo are trademarks of ETSI registered for the benefit of its Members.
3GPP™ and LTE™ are trademarks of ETSI registered for the benefit of its Members and
of the 3GPP Organizational Partners.
oneM2M™ logo is a trademark of ETSI registered for the benefit of its Members and
of the oneM2M Partners.
GSM® and the GSM logo are trademarks registered and owned by the GSM Association.

Contents

Intellectual Property Rights	13

Foreword	13

Modal verbs terminology	13

1	Scope	14

2	References	14

2.1	Normative references	14

2.2	Informative references	15

3	Definition of terms, symbols and abbreviations	16

3.1	Terms	16

3.2	Symbols	22

3.3	Abbreviations	22

4	Introduction	23

4.0	General	23

4.1	The core language and presentation formats	24

4.2	Unanimity of the specification	25

4.3	Conformance	25

5	Basic language elements	25

5.0	General	25

5.1	Identifiers and keywords	26

5.2	Scope rules	27

5.2.0	General	27

5.2.1	Scope of formal parameters	29

5.2.2	Uniqueness of identifiers	29

5.3	Ordering of language elements	30

5.4	Parameterization	30

5.4.0	General	30

5.4.1	Formal parameters	31

5.4.1.0	General	31

5.4.1.1	Formal parameters of kind value	31

5.4.1.2	Formal parameters of kind template	34

5.4.2	Actual parameters	36

5.5	Cyclic Definitions	41

6	Types and values	42

6.0	General	42

6.1	Basic types and values	43

6.1.0	Simple basic types and values	43

6.1.1	Basic string types and values	44

6.1.1.0	General	44

6.1.1.1	Accessing individual string elements	46

6.1.2	Subtyping of basic types	47

6.1.2.0	General	47

6.1.2.1	Lists of templates	47

6.1.2.2	Lists of types	47

6.1.2.3	Ranges	48

6.1.2.4	String length restrictions	49

6.1.2.5	Pattern subtyping of character string types	49

6.1.2.6	Mixing subtyping mechanisms	49

6.1.2.6.1	Mixing patterns, lists and ranges	49

6.1.2.6.2	Using length restriction with other constraints	50

6.2	Structured types and values	50

6.2.0	General	50

6.2.1	Record type and values	52

6.2.1.0	General	52

6.2.1.1	Referencing fields of a record type	55

6.2.1.2	Optional elements in a record	56

6.2.1.3	Nested type definitions for field types	56

6.2.2	Set type and values	57

6.2.2.0	General	57

6.2.2.1	Referencing fields of a set type	57

6.2.2.2	Optional elements in a set	57

6.2.2.3	Nested type definition for field types	57

6.2.3	Records and sets of single types	57

6.2.3.0	General	57

6.2.3.1	Nested type definitions	60

6.2.3.2	Referencing elements of record of and set of types	60

6.2.4	Enumerated type and values	61

6.2.5	Unions	63

6.2.5.0	General	63

6.2.5.1	Referencing fields of a union type	64

6.2.5.2	Option and union	65

6.2.5.3	Nested type definition for field types	65

6.2.6	The anytype	66

6.2.7	Arrays	66

6.2.8	The default type	68

6.2.9	Communication port types	68

6.2.10	Component types	70

6.2.10.1	Component type definition	70

6.2.10.2	Reuse of component types	71

6.2.11	Component references	73

6.2.12	Addressing entities inside the SUT	75

6.2.13	Subtyping of structured types	77

6.2.13.0	General	77

6.2.13.1	Length subtyping of record ofs and set ofs	77

6.2.13.2	List subtyping of structured types and anytype	78

6.2.13.3	Subtyping of the iterated type of record ofs and set ofs	81

6.2.13.4	Mixing subtyping mechanisms	82

6.2.14	The timer type	82

6.2.15	Map types	82

6.2.15.0	General	82

6.2.15.1	Map Type Definition	82

6.2.15.2	Indexed Assignment Notation	83

6.2.15.3	Unmapping Keys	83

6.2.15.4	Index Notation	84

6.2.15.5	Accessing the Keys of a Map	84

6.2.15.6	Accessing the Values of a Map	85

6.2.15.7	Referencing of Elements of a Map	85

6.2.15.8	Nested type definitions	85

6.2.16	The open type	85

6.3	Type compatibility	86

6.3.0	General	86

6.3.1	Compatibility of non-structured types	86

6.3.2	Compatibility of structured types	88

6.3.2.0	General	88

6.3.2.1	Compatibility of enumerated types	88

6.3.2.2	Compatibility of record and record of types	89

6.3.2.3	Compatibility of set and set of types	90

6.3.2.4	Compatibility of union types	91

6.3.2.5	Compatibility of anytype types	91

6.3.2.6	Compatibility between sub-structures	92

6.3.2.7	Compatibility of the open type	92

6.3.3	Compatibility of component types	93

6.3.4	Type compatibility of communication and connection operations	93

6.3.5	Type conversion	94

6.3.6	Type compatibility of port types	94

6.3.7	Type compatibility of timer types	94

6.3.8	Type Compatibility of Map Types	94

6.4	Type synonym	94

7	Expressions	94

7.0	General	94

7.1	Operators	95

7.1.0	General	95

7.1.1	Arithmetic operators	97

7.1.2	List operator	97

7.1.3	Relational operators	98

7.1.4	Logical operators	101

7.1.5	Bitwise operators	101

7.1.6	Shift operators	102

7.1.7	Rotate operators	102

7.1.8	Presence checking operators	103

7.1.8.0	General	103

7.1.8.1	The ispresent operator	104

7.1.8.2	The ischosen operator	105

7.1.8.3	The isvalue operator	106

7.1.8.4	The isbound operator	108

7.2	Field references and list elements	109

7.3	Decoded field reference	109

8	Modules	110

8.0	General	110

8.1	Definition of a module	110

8.2	Module definitions part	111

8.2.0	General	111

8.2.1	Module parameters	112

8.2.2	Groups of definitions	114

8.2.3	Importing from modules	115

8.2.3.0	General	115

8.2.3.1	General format of import	115

8.2.3.2	Importing single definitions	121

8.2.3.3	Importing groups	122

8.2.3.4	Importing definitions of the same kind	123

8.2.3.5	Importing all definitions of a module	124

8.2.3.6	Import definitions from other TTCN-3 editions and from non-TTCN-3 modules	125

8.2.3.7	Importing of import statements from TTCN-3 modules	126

8.2.3.8	Compatibility of language specifications in imports	127

8.2.4	Definition of friend modules	128

8.2.5	Visibility of definitions	128

8.3	Module control part	130

9	Port types, component types and test configurations	131

9.0	General	131

9.1	Communication ports	131

9.2	Test system interface	134

10	Declaring constants	136

11	Declaring variables	136

11.0	General	136

11.1	Value variables	137

11.2	Template variables	138

12	Declaring timers	139

13	Declaring messages	140

14	Declaring procedure signatures	141

15	Declaring templates	142

15.0	General	142

15.1	Declaring message templates	143

15.2	Declaring signature templates	145

15.3	Global and local templates	146

15.4	In-line Templates	147

15.5	Modified templates	148

15.6	Referencing elements of templates or template fields	152

15.6.0	General	152

15.6.1	Referencing individual string elements	152

15.6.2	Referencing record and set fields	152

15.6.3	Referencing record of and set of elements	153

15.6.4	Referencing signature parameters	157

15.6.5	Referencing union alternatives	158

15.7	Template matching mechanisms	159

15.7.0	General	159

15.7.1	Specific values	160

15.7.2	Special symbols that can be used instead of values	161

15.7.3	Special symbols that can be used inside values	162

15.7.4	Special symbols which describe attributes of values	162

15.8	Template Restrictions	163

15.9	Match Operation	165

15.10	Valueof Operation	167

15.11	Concatenating templates of string and list types	167

16	Functions, altsteps and testcases	170

16.0	General	170

16.1	Functions	170

16.1.0	General	170

16.1.1	Invoking functions	173

16.1.2	Predefined functions	173

16.1.3	External functions	176

16.1.4	Invoking functions from specific places	176

16.1.5	Explicit control functions	178

16.2	Altsteps	178

16.2.0	General	178

16.2.1	Invoking altsteps	180

16.3	Test cases	181

17	Void	182

18	Overview of program statements and operations	182

19	Basic program statements	185

19.0	General	185

19.1	Assignments	185

19.2	The If-else statement	187

19.3	The Select statements	187

19.3.1	The Select case statement	187

19.3.2	The Select union statement	189

19.4	The For statement	190

19.5	The While statement	190

19.6	The Do-while statement	191

19.7	The Label statement	191

19.8	The Goto statement	192

19.9	The Stop execution statement	193

19.10	The Return statement	193

19.11	The Log statement	194

19.12	The Break statement	196

19.13	The Continue statement	196

19.14	Statement block	197

20	Statement and operations for alternative behaviours	197

20.0	General	197

20.1	The snapshot mechanism	198

20.2	The Alt statement	198

20.3	The Repeat statement	202

20.4	The Interleave statement	203

20.5	Default Handling	205

20.5.0	General	205

20.5.1	The default mechanism	206

20.5.2	The Activate operation	206

20.5.3	The Deactivate operation	207

21	Configuration Operations	208

21.0	General	208

21.1	Connection Operations	209

21.1.0	General	209

21.1.1	The Connect and Map operations	210

21.1.2	The Disconnect and Unmap operations	212

21.2	Test case operations	213

21.2.0	General	213

21.2.1	Test case stop operation	214

21.3	Test Component Operations	214

21.3.0	General	214

21.3.1	The Create operation	214

21.3.2	The Start test component operation	215

21.3.3	The Stop test behaviour operation	217

21.3.4	The Kill test component operation	218

21.3.5	The Alive operation	218

21.3.6	The Running operation	219

21.3.7	The Done operation	221

21.3.8	The Killed operation	223

21.3.9	Summary of the use of any and all with components	225

21.3.10	The Call test component behaviour operation	225

22	Communication operations	227

22.0	General	227

22.1	The communication mechanisms	227

22.1.0	General	227

22.1.1	Principles of message-based communication	227

22.1.2	Principles of procedure-based communication	228

22.1.3	Principles of unicast, multicast and broadcast communication	228

22.1.4	General format of communication operations	229

22.1.4.0	General	229

22.1.4.1	General format of the sending operations	229

22.1.4.2	General format of the receiving operations	230

22.2	Message-based communication	231

22.2.0	General	231

22.2.1	The Send operation	231

22.2.2	The Receive operation	232

22.2.3	The Trigger operation	236

22.3	Procedure-based communication	239

22.3.0	General	239

22.3.1	The Call operation	239

22.3.2	The Getcall operation	243

22.3.3	The Reply operation	246

22.3.4	The Getreply operation	247

22.3.5	The Raise operation	250

22.3.6	The Catch operation	251

22.4	The Check operation	255

22.5	Controlling communication ports	257

22.5.0	General	257

22.5.1	The Clear port operation	257

22.5.2	The Start port operation	258

22.5.3	The Stop port operation	258

22.5.4	The Halt port operation	259

22.5.5	The Checkstate port operation	259

22.6	Use of any and all with ports	261

23	Timer operations	261

23.0	General	261

23.1	The timer mechanism	262

23.2	The Start timer operation	262

23.3	The Stop timer operation	263

23.4	The Read timer operation	263

23.5	The Running timer operation	264

23.6	The Timeout operation	265

23.7	Summary of use of any and all with timers	266

24	Test verdict operations	266

24.0	General	266

24.1	The Verdict mechanism	266

24.2	The Setverdict operation	267

24.3	The Getverdict operation	268

25	External actions	269

26	Module control	269

26.0	General	269

26.1	The Execute statement	270

26.2	Test suite execution	271

27	Specifying attributes	273

27.0	General	273

27.1	The Attribute mechanism	273

27.1.0	General	273

27.1.1	Scope of attributes	274

27.1.2	Overwriting rules for attributes	275

27.1.2.0	General	275

27.1.2.1	Additional default overwriting rules for variant attributes	277

27.1.2.2	Overwriting rules for multiple encoding	278

27.1.3	Changing attributes of imported language elements	278

27.2	The With statement	279

27.3	Display attributes	280

27.4	Encoding attributes	281

27.5	Variant attributes	282

27.6	Extension attributes	284

27.7	Optional attributes	284

27.8	Retrieving attribute values	286

27.9	Dynamic configuration of encoding used by ports	287

Annex A (normative): 	BNF and static semantics	289

A.1	TTCN-3 BNF	289

A.1.0	General	289

A.1.1	Conventions for the syntax description	289

A.1.2	Statement terminator symbols	289

A.1.3	Identifiers	289

A.1.4	Comments	290

A.1.5	TTCN-3 terminals	290

A.1.5.0	General	290

A.1.5.1	Use of whitespaces and newlines	292

A.1.6	TTCN-3 syntax BNF productions	293

A.1.6.0	TTCN-3 module	293

A.1.6.1	Module definitions part	293

A.1.6.1.0	General	293

A.1.6.1.1	Typedef definitions	293

A.1.6.1.2	Constant definitions	295

A.1.6.1.3	Template definitions	295

A.1.6.1.4	Function definitions	297

A.1.6.1.5	Signature definitions	298

A.1.6.1.6	Testcase definitions	298

A.1.6.1.7	Altstep definitions	298

A.1.6.1.8	Import definitions	298

A.1.6.1.9	Group definitions	299

A.1.6.1.10	External function definitions	299

A.1.6.1.11	Void	299

A.1.6.1.12	Module parameter definitions	300

A.1.6.1.13	Friend module definitions	300

A.1.6.2	Module control function	300

A.1.6.3	Local definitions	300

A.1.6.3.1	Variable instantiation	300

A.1.6.3.2	Timer instantiation	300

A.1.6.4	Operations	300

A.1.6.4.1	Component operations	300

A.1.6.4.2	Port operations	301

A.1.6.4.3	Timer operations	303

A.1.6.4.4	Testcase operation	303

A.1.6.5	Type	303

A.1.6.6	Value	304

A.1.6.7	Parameterization	305

A.1.6.8	Statements	305

A.1.6.8.1	With statement	305

A.1.6.8.2	Behaviour statements	306

A.1.6.8.3	Basic statements	307

A.1.6.9	Miscellaneous productions	309

Annex B (normative): 	Matching values	310

B.1	Template matching mechanisms	310

B.1.0	General	310

B.1.1	Matching specific values	310

B.1.2	Matching mechanisms instead of values	310

B.1.2.0	General	310

B.1.2.1	Template list	310

B.1.2.2	Complemented template list	311

B.1.2.3	Any value	312

B.1.2.4	Any value or none	313

B.1.2.5	Value range	314

B.1.2.6	SuperSet	314

B.1.2.7	SubSet	315

B.1.2.8	Omitting optional fields	317

B.1.2.9	Matching decoded content	317

B.1.2.10	Matching enumerated value with value list	319

B.1.3	Matching mechanisms inside values	319

B.1.3.0	General	319

B.1.3.1	Any element	319

B.1.3.1.0	General	319

B.1.3.1.1	Using single character wildcards	319

B.1.3.2	Any number of elements or no element	320

B.1.3.2.0	General	320

B.1.3.2.1	Using multiple character wildcards	320

B.1.3.3	Permutation	320

B.1.4	Matching attributes of values	322

B.1.4.0	General	322

B.1.4.1	Length restrictions	322

B.1.4.2	The IfPresent indicator	323

B.1.5	Matching character pattern	324

B.1.5.0	General	324

B.1.5.1	Set expression	326

B.1.5.2	Reference expression	326

B.1.5.3	Match expression n times	328

B.1.5.4	Match a referenced character set	328

B.1.5.5	Type compatibility rules for patterns	329

B.1.5.6	Case insensitive pattern matching	329

Annex C (normative): 	Predefined TTCN-3 functions	330

C.0	General exception handling procedures	330

C.1	Conversion functions	330

C.1.1	Integer to character	330

C.1.2	Integer to universal character	330

C.1.3	Integer to bitstring	330

C.1.4	Integer to enumerated	331

C.1.5	Integer to hexstring	331

C.1.6	Integer to octetstring	331

C.1.7	Integer to charstring	332

C.1.8	Integer to float	332

C.1.9	Float to integer	332

C.1.10	Character to integer	332

C.1.11	Character to octetstring	332

C.1.12	Universal character to integer	333

C.1.13	Bitstring to integer	333

C.1.14	Bitstring to hexstring	333

C.1.15	Bitstring to octetstring	333

C.1.16	Bitstring to charstring	334

C.1.17	Hexstring to integer	334

C.1.18	Hexstring to bitstring	334

C.1.19	Hexstring to octetstring	335

C.1.20	Hexstring to charstring	335

C.1.21	Octetstring to integer	335

C.1.22	Octetstring to bitstring	335

C.1.23	Octetstring to hexstring	336

C.1.24	Octetstring to character string	336

C.1.25	Octetstring to character string, version II	336

C.1.26	Charstring to integer	337

C.1.27	Character string to hexstring	337

C.1.28	Character string to octetstring	337

C.1.29	Character string to float	338

C.1.30	Enumerated to integer	338

C.1.31	Octetstring to universal character string	339

C.1.32	Universal character string to octetstring	339

C.1.33	Value or template to universal charstring	340

C.2	Length/size functions	341

C.2.1	Length of strings and lists	341

C.2.2	Number of elements in a structured value	342

C.3	Presence checking functions	343

C.3.1	Void	343

C.3.2	Void	343

C.3.3	Void	343

C.3.4	Void	343

C.3.5	Matching mechanism detection	343

C.4	String/list handling functions	344

C.4.1	The Regexp function	344

C.4.2	The Substring function	346

C.4.3	The Replace function	347

C.5	Codec functions	348

C.5.1	The encoding function	348

C.5.2	The decoding function	348

C.5.3	The encoding to universal charstring function	348

C.5.4	The decoding from universal charstring function	349

C.5.5	The encoding to octetstring function	351

C.5.6	The decoding from octetstring function	351

C.5.7	Retrieving the type of string encoding	352

C.5.8	Removing BOMs of UCS encoding schemes	352

C.6	Other functions	353

C.6.1	The random number generator function	353

C.6.2	The testcasename function	353

C.6.3	The hostId function	354

Annex D (normative): 	Preprocessing macros	355

D.0	General	355

D.1	Preprocessing macro __MODULE__	355

D.2	Preprocessing macro __FILE__	355

D.3	Preprocessing macro __BFILE__	355

D.4	Preprocessing macro __LINE__	355

D.5	Preprocessing macro __SCOPE__	356

Annex E (informative): 	Library of Useful Types	358

E.1	Limitations	358

E.2	Useful TTCN-3 types	358

E.2.1	Useful simple basic types	358

E.2.1.0	Signed and unsigned single byte integers	358

E.2.1.1	Signed and unsigned short integers	358

E.2.1.2	Signed and unsigned long integers	359

E.2.1.3	Signed and unsigned longlong integers	359

E.2.1.4	IEEE 754 floats	359

E.2.2	Useful character string types	360

E.2.2.0	UTF-8 character string "utf8string"	360

E.2.2.1	BMP character string "bmpstring"	360

E.2.2.2	UTF-16 character string "utf16string"	360

E.2.2.3	ISO/IEC 10646 character string "iso8859string"	360

E.2.2.4	Status values for TTCN-3 objects	361

E.2.2.5	Template kinds of TTCN-3 objects	361

E.2.3	Useful structured types	361

E.2.3.0	Fixed-point decimal literal	361

E.2.4	Useful atomic string types	362

E.2.4.1	Single Recommendation ITU-T T.50 character type	362

E.2.4.2	Single universal character type	362

E.2.4.3	Single bit type	362

E.2.4.4	Single hex type	362

E.2.4.5	Single octet type	362

Annex F (informative): 	Operations on TTCN-3 active objects	363

F.0	General	363

F.1	Test components	363

F.1.1	Test component references	363

F.1.2	Dynamic behaviour of PTCs	364

F.1.3	Dynamic behaviour of the MTC	366

F.2	Timers	367

F.3	Ports	367

F.3.0	General	367

F.3.1	Configuration Operations	367

F.3.2	Port Controlling Operations	368

F.3.3	Communication Operations	369

Annex G (informative): 	Deprecated language features	370

G.1	Group style definition of module parameters	370

G.2	Void	370

G.3	Using all in port type definitions	370

G.4	sizeof for length of lists	370

G.5	Void	370

G.6	Mixed ports	370

G.7	Void	370

G.8	Void	371

G.9	Void	371

G.10	Void	371

G.11	Void	371

G.12	Void	371

G.13	Assignment of less restrictive templates to more restrictive templates	371

G.14	Mixing case and case else branches in select statements	371

G.15	Partially initialized global and local templates	372

Annex H (informative): 	Bibliography	373

History	374

# Intellectual Property Rights

Essential patents

IPRs essential or potentially essential to normative deliverables may have been declared to ETSI. The information pertaining to these essential IPRs, if any, is publicly available for ETSI members and non-members, and can be found in ETSI SR 000 314: "Intellectual Property Rights (IPRs); Essential, or potentially Essential, IPRs notified to ETSI in respect of ETSI standards", which is available from the ETSI Secretariat. Latest updates are available on the ETSI Web server (https://ipr.etsi.org/).

Pursuant to the ETSI IPR Policy, no investigation, including IPR searches, has been carried out by ETSI. No guarantee can be given as to the existence of other IPRs not referenced in ETSI SR 000 314 (or the updates on the ETSI Web server) which are, or may be, or may become, essential to the present document.

Trademarks

The present document may include trademarks and/or tradenames which are asserted and/or registered by their owners. ETSI claims no ownership of these except for any which are indicated as being the property of ETSI, and conveys no right to use or reproduce any trademark and/or tradename. Mention of those trademarks in the present document does not constitute an endorsement by ETSI of products, services or organizations associated with those trademarks.

# Foreword

This ETSI Standard (ES) has been produced by ETSI Technical Committee Methods for Testing and Specification (MTS).

The present document is part 1 of a multi-part deliverable covering the Testing and Test Control Notation version 3, as identified below:

Part 1:	"TTCN-3 Core Language";

Part 2:	"TTCN-3 Tabular presentation Format (TFT)";

Part 3:	"TTCN-3 Graphical presentation Format (GFT)";

Part 4:	"TTCN-3 Operational Semantics";

Part 5:	"TTCN-3 Runtime Interface (TRI)";

Part 6:	"TTCN-3 Control Interface (TCI)";

Part 7:	"Using ASN.1 with TTCN-3";

Part 8:	"The IDL to TTCN-3 Mapping";

Part 9:	"Using XML schema with TTCN-3";

Part 10:	"TTCN-3 Documentation Comment Specification";

Part 11:	"Using JSON with TTCN-3".

NOTE:	Part 2 of this multi-part deliverable is in status "historical" and is not maintained.

# Modal verbs terminology

In the present document "shall", "shall not", "should", "should not", "may", "need not", "will", "will not", "can" and "cannot" are to be interpreted as described in clause 3.2 of the ETSI Drafting Rules (Verbal forms for the expression of provisions).

"must" and "must not" are NOT allowed in ETSI deliverables except when used in direct citation.

# 1	Scope

The present document defines the Core Language of TTCN-3. TTCN-3 can be used for the specification of all types of reactive system tests over a variety of communication ports. Typical areas of application are protocol testing (including mobile and Internet protocols), service testing (including supplementary services), module testing, testing of CORBA® based platforms, APIs, etc. TTCN-3 is not restricted to conformance testing and can be used for many other kinds of testing including interoperability, robustness, regression, system and integration testing. The specification of test suites for physical layer protocols is outside the scope of the present document.

TTCN-3 is intended to be used for the specification of test suites which are independent of test methods, layers and protocols. In addition to the textual format defined in the present document, while GFT (ETSI ES 201 873-3 [i.2]) defines a graphical presentation format for TTCN-3. The specification of these formats is outside the scope of the present document.

While the design of TTCN-3 has taken the eventual implementation of TTCN-3 translators and compilers into consideration the means of realization of Executable Test Suites (ETS) from Abstract Test Suites (ATS) is outside the scope of the present document.

# 2	References

## 2.1	Normative references

References are either specific (identified by date of publication and/or edition number or version number) or non-specific. For specific references, only the cited version applies. For non-specific references, the latest version of the referenced document (including any amendments) applies.

Referenced documents which are not found to be publicly available in the expected location might be found at https://docbox.etsi.org/Reference.

NOTE:	While any hyperlinks included in this clause were valid at the time of publication ETSI cannot guarantee their long term validity.

The following referenced documents are necessary for the application of the present document.

[1]	ETSI ES 201 873-4: "Methods for Testing and Specification (MTS); The Testing and Test Control Notation version 3; Part 4: TTCN-3 Operational Semantics".

[2]	ISO/IEC 10646 (2017): "Information technology -- Universal Coded Character Set (UCS)".

[3]	Recommendation ITU-T X.292: "OSI conformance testing methodology and framework for protocol Recommendations for ITU-T applications - The Tree and Tabular Combined Notation (TTCN)".

NOTE:	The corresponding ISO/IEC standard is ISO/IEC 9646-3: "Information technology -- Open Systems Interconnection -- Conformance testing methodology and framework -- Part 3: The Tree and Tabular Combined Notation (TTCN)".

[4]	Recommendation ITU-T T.50: "International Reference Alphabet (IRA) (Formerly International Alphabet No. 5 or IA5) - Information technology - 7-bit coded character set for information interchange".

NOTE:	The corresponding ISO/IEC standard is ISO/IEC 646: "Information technology -- ISO 7-bit coded character set for information interchange".

[5]	Recommendation ITU-T X.290: "OSI conformance testing methodology and framework for protocol Recommendations for ITU-T applications - General concepts".

NOTE:	The corresponding ISO/IEC standard is ISO/IEC 9646-1: "Information technology -- Open Systems Interconnection -- Conformance testing methodology and framework; Part 1: General concepts".

[6]	IEEE 754™: "IEEE Standard for Floating-Point Arithmetic".

## 2.2	Informative references

References are either specific (identified by date of publication and/or edition number or version number) or non-specific. For specific references, only the cited version applies. For non-specific references, the latest version of the referenced document (including any amendments) applies.

NOTE:	While any hyperlinks included in this clause were valid at the time of publication ETSI cannot guarantee their long term validity.

The following referenced documents are not necessary for the application of the present document but they assist the user with regard to a particular subject area.

[i.1]	Void.

[i.2]	ETSI ES 201 873-3: "Methods for Testing and Specification (MTS); The Testing and Test Control Notation version 3; Part 3: TTCN-3 Graphical presentation Format (GFT)".

[i.3]	ETSI ES 201 873-5: "Methods for Testing and Specification (MTS); The Testing and Test Control Notation version 3; Part 5: TTCN-3 Runtime Interface (TRI)".

[i.4]	ETSI ES 201 873-6: "Methods for Testing and Specification (MTS); The Testing and Test Control Notation version 3; Part 6: TTCN-3 Control Interface (TCI)".

[i.5]	ETSI ES 201 873-7: "Methods for Testing and Specification (MTS); The Testing and Test Control Notation version 3; Part 7: Using ASN.1 with TTCN-3".

[i.6]	ETSI ES 201 873-8: "Methods for Testing and Specification (MTS); The Testing and Test Control Notation version 3; Part 8: The IDL to TTCN-3 Mapping".

[i.7]	ETSI ES 201 873-9: "Methods for Testing and Specification (MTS); The Testing and Test Control Notation version 3; Part 9: Using XML schema with TTCN-3".

[i.8]	ETSI ES 201 873-10: "Methods for Testing and Specification (MTS); The Testing and Test Control Notation version 3; Part 10: TTCN-3 Documentation Comment Specification".

[i.9]	Void.

[i.10]	Object Management Group (OMG) (2001): "The Common Object Request Broker: Architecture and Specification - IDL Syntax and Semantics". Version 2.6, FORMAL/01-12-01.

[i.11]	ETSI ES 202 781: "Methods for Testing and Specification (MTS); The Testing and Test Control Notation version 3; TTCN-3 Language Extensions: Configuration and Deployment Support".

[i.12]	ETSI ES 202 784: "Methods for Testing and Specification (MTS); The Testing and Test Control Notation version 3; TTCN-3 Language Extensions: Advanced Parameterization".

[i.13]	ETSI ES 202 785: "Methods for Testing and Specification (MTS); The Testing and Test Control Notation version 3; TTCN-3 Language Extensions: Behaviour Types".

[i.14]	ETSI ES 202 782: "Methods for Testing and Specification (MTS); The Testing and Test Control Notation version 3; TTCN-3 Language Extensions: TTCN-3 Performance and Real Time Testing".

[i.15]	Void.

[i.16]	Void.

[i.17]	ETSI ES 201 873-1 (V1.1.2): "Methods for Testing and Specification (MTS); The Tree and Tabular Combined Notation version 3; Part 1: TTCN-3 Core Language", 2001.

[i.18]	ETSI ES 201 873-1 (V2.2.1): "Methods for Testing and Specification (MTS); The Testing and Test Control Notation version 3; Part 1: TTCN-3 Core Language", 2003.

[i.19]	ETSI ES 201 873-1 (V3.1.1): "Methods for Testing and Specification (MTS); The Testing and Test Control Notation version 3; Part 1: TTCN-3 Core Language", 2005.

[i.20]	ETSI ES 201 873-1 (V3.2.1): "Methods for Testing and Specification (MTS); The Testing and Test Control Notation version 3; Part 1: TTCN-3 Core Language", 2007.

[i.21]	ETSI ES 201 873-1 (V3.3.2): "Methods for Testing and Specification (MTS); The Testing and Test Control Notation version 3; Part 1: TTCN-3 Core Language", 2008.

[i.22]	ETSI ES 201 873-1 (V3.4.1): "Methods for Testing and Specification (MTS); The Testing and Test Control Notation version 3; Part 1: TTCN-3 Core Language", 2008.

[i.23]	ETSI ES 201 873-1 (V4.1.1): "Methods for Testing and Specification (MTS); The Testing and Test Control Notation version 3; Part 1: TTCN-3 Core Language", 2009.

[i.24]	ETSI ES 201 873-1 (V4.2.1): "Methods for Testing and Specification (MTS); The Testing and Test Control Notation version 3; Part 1: TTCN-3 Core Language", 2010.

[i.25]	ETSI ES 201 873-1 (V4.3.1): "Methods for Testing and Specification (MTS); The Testing and Test Control Notation version 3; Part 1: TTCN-3 Core Language", 2011.

[i.26]	ETSI ES 201 873-1 (V4.4.1): "Methods for Testing and Specification (MTS); The Testing and Test Control Notation version 3; Part 1: TTCN-3 Core Language", 2012.

[i.27]	ETSI ES 201 873-1 (V4.5.1): "Methods for Testing and Specification (MTS); The Testing and Test Control Notation version 3; Part 1: TTCN-3 Core Language", 2013.

[i.28]	ETSI ES 201 873-1 (V4.6.1): "Methods for Testing and Specification (MTS); The Testing and Test Control Notation version 3; Part 1: TTCN-3 Core Language", 2014.

[i.29]	ETSI ES 201 873-1 (V4.7.1): "Methods for Testing and Specification (MTS); The Testing and Test Control Notation version 3; Part 1: TTCN-3 Core Language", 2015.

[i.30]	ETSI ES 201 873-1 (V4.8.1): "Methods for Testing and Specification (MTS); The Testing and Test Control Notation version 3; Part 1: TTCN-3 Core Language", 2016.

[i.31]	ETSI ES 201 873-1 (V4.9.1): "Methods for Testing and Specification (MTS); The Testing and Test Control Notation version 3; Part 1: TTCN-3 Core Language", 2017.

# 3	Definition of terms, symbols and abbreviations

## 3.1	Terms

For the purposes of the present document, the terms given in Recommendation ITU-T X.290 [5], Recommendation ITU-T X.292 [3] and the following apply:

actual parameter: value, expression, template or name reference (identifier) to be passed as parameter to the invoked entity (function, test case, altstep, etc.) as defined at the place of invoking

assignment notation: notation that can be used for record, set, record of and set of values, where the fields or the elements to which a value is assigned are identified explicitly within a pair of curly brackets ("{" and "}") by the field names or the positions of the elements

basic types: set of predefined TTCN-3 types described in clauses 6.1.0 and 6.1.1 of the present document

NOTE:	Basic types are referenced by their names.

behaviour definition: dynamic test behaviour, which is either a testcase, a function, or an altstep definition

communication port: abstract mechanism facilitating communication between test components

NOTE:	A communication port is modelled as a FIFO queue in the receiving direction. Ports can be message-based or procedure-based.

compatible type: TTCN-3 is not strongly typed but the language does require type compatibility

NOTE:	Variables, constants, templates, etc. have compatible types if conditions in clause 6.2.15 are met.

completely initialized: value or template is completely initialized if it is not uninitialized and, if its type is a structured type, all its required parts are completely initialized

NOTE 1:	Additionally, templates are completely initialized if they are assigned a matching mechanism all parts of which are completely initialized. If a value or template is completely initialized, it fulfils the requirement of being "at least partially initialized".

NOTE 2:	A value or template of a simple, component or default type is completely initialized if anything but the unchanged symbol "-" has been assigned to it. 
A value or template of a union or anytype type is completely initialized if one of its variants has been completely initialized.
A value or template of a record or set type with only optional fields and the optional "implicit omit" attribute attached, is completely initialized if the value "{}" is assigned, as all fields are implicitly set to omit. 
A value or template of a record or set type with no fields is completely initialized with assignment of the value "{}".
A value or template of a record of, set of or array type is completely initialized if at least the first n elements are completely initialized, where n is the minimal length imposed by the type length restriction or array definition. Thus in case of n equals 0, the assignment of the value "{}" also completely initializes such a record of, set of or array.

component constant: constant defined in a component type

component data types: collection of all data types, component types and structured types whose sub-elements are component data types

component port: port defined in a component type

component template: template defined in a component type

component timer: timer defined in a component type

component variable: variable defined in a component type

control behaviour: collection of module control functions with the name control and functions and altsteps called by control directly or through other control functions or altsteps, and are used for the dynamic execution of test cases

NOTE:	Such functions and altsteps are called control functions and control altsteps respectively. Module control functions can be used as an entry point of executing a test suite.  Declaring functions or altsteps with the modifier @control explicitly allows to distinguish them from test case behaviour definitions in their special role. Module control functions and behaviour definitions with the @control modifier are called explicit control behaviour definitions, i.e explicit control functions and explicit control altsteps.

data types: all types whose values or sub-elements cannot contain object references

NOTE:	Data types include simple basic types, basic string types, and the special data type anytype. Data types also include all structured types where all their sub-elements are of a data type. All user defined types based on a data type are data types as well. See more details in table 3 of the present document.

defined types (defined TTCN-3 types): set of all predefined TTCN-3 types (basic types, all structured types, the type anytype, the address, port and component types and the default type) and all user-defined types declared either in the module or imported from other TTCN-3 modules

deterministic function: function that for the same input in the in and inout parameters always yields the same output both for the return result as well as the inout and out parameters

NOTE 1:	A non-deterministic function is one that is not deterministic.

NOTE 2:	In general, it cannot be decided if a function is deterministic or not. However, a function can be specified to be deterministic, i.e. the function is supposed to be deterministic. In this case, a violation of the determinism can be detected and handled accordingly. The handling however is tool-specific.

dynamic parameterization: form of parameterization, in which actual parameters are dependent on runtime events

EXAMPLE:	The value of the actual parameter is a value received during runtime or depends on a received value by a logical relation.

exception: in cases of procedure-based communication, an exception (if defined) is raised by an answering entity if it cannot answer a remote procedure call with the normal expected response

formal parameter: typed name or typed template reference (identifier) not resolved at the time of the definition of an entity (function, test case, altstep, etc.) but at the time of invoking it

NOTE:	Actual values or templates (or their names) to be used at the place of formal parameters are passed from the place of invoking the entity (see also the definition of actual parameter).

fuzzy value or template: value or template instance that is declared to be fuzzy and consequently the expression, initializing or partly initializing it (including actual parameters passed to in formal parameters), is subject to lazy evaluation

NOTE:	During execution, this expression is re-evaluated each time when the fuzzy object is referenced, except when at the left hand side of an assignment or passing it to a fuzzy or lazy formal parameters. The result of this (re)evaluation is used as the actual value or template of the fuzzy instance. When new content is assigned to a fuzzy instance or to its subpart, the right hand side of the assignment is subject to lazy evaluation again.

global visibility: attribute of an entity (module parameter, constant, template, etc.) whose identifier can be referenced anywhere within the module where it is defined including all functions, test cases and altsteps defined within the same module

implementation conformance statement (ICS): See Recommendation ITU-T X.290 [5].

implementation extra information for testing (IXIT): See Recommendation ITU-T X.290 [5].

implementation under test (IUT): See Recommendation ITU-T X.290 [5].

in parameterization: kind of parameterization where the value of the actual parameter (the argument) is assigned to the formal parameter when the parameterized object is invoked, but the value of the formal parameter is not passed back to the actual parameter when the invoked object completes

NOTE 1:	In in parameterization, parameters are passed by value.

NOTE 2:	The arguments are evaluated before the parameterized object is entered.

NOTE 3:	Only the values of the arguments are passed and changes to the arguments within the invoked object have no effect on the arguments as seen by the invoking object.

index notation: notation to access individual elements of record of, set of, array and string values or templates, where the element to be accessed is identified explicitly by an index value enclosed in square brackets ("[" and "]") which specifies the position of that element within the referenced value or template and the index value is either an integer value, array of integers or record of integers

NOTE:	Integer values used for indexing (either directly or as elements of the record of or array values) always lie within the index range of the type of the referenced value or template. Except for those arrays which are defined with an explicit index range, the index range always has 0 as the index for the first element.

initialization: value or template, or a value or template field is initialized when a content is first assigned to it

NOTE:	The assignment may be explicit at the declaration of the given object, in which case the same restrictions apply as for the right-hand side of the assignment operation, or at first use on the left-hand side of an assignment, or may be implicit. Implicit initialization occurs when a yet uninitialized object is passed as actual parameter to an out formal parameter of a directly called testcase, function or altstep returns with a non-uninitialized value or template that is assigned to the actual parameter; or when module parameters not initialized in the TTCN-3 code get their runtime values before test suite execution.

inout parameterization: kind of parameterization that uses passing by reference, i.e. when the parameterized object is invoked, the formal parameter is linked with the actual parameter and gets direct access to the same data content that is currently represented by the actual parameter

NOTE 1:	The invoked object uses the actual parameter directly, so that all changes made in the formal parameter become immediately effective on the actual parameter. If the same actual parameter is passed to two distinct formal parameters, a change in one formal parameter becomes immediately effective in the other one (and in the actual parameter).

NOTE 2:	Inout parameters can be used for functions, altsteps, and test cases only, if not restricted by further rules, e.g. altsteps activated as defaults.

known types: set of all TTCN-3 predefined types, types defined in a TTCN-3 module and types imported into that module from other TTCN-3 modules or from non-TTCN-3 modules

lazy evaluation: evaluation of an expression, delayed during execution until the value or template instance, to which the result of the evaluation should have been assigned or passed to as actual parameter, is first referenced at another place than the left hand side of an assignment or an actual parameter passed to a fuzzy or lazy formal parameter

NOTE:	During execution, this delayed evaluation is carried out at the first actual reference, even when the result is to be used in an expression that is also subject to lazy evaluation. For the evaluation the actual values at the time of the evaluation are to be used (not the actual values at the time of the assignment or parameter passing). This implies that components of the expression may be uninitialized at the time, when execution reaches the assignment or parameter passing, but may be initialized by the time of the evaluation that can lead to successful evaluation. If, by the time of the evaluation, execution has left the scope unit, in which one or more components of the expression is defined, the actual values of the component(s) at the time of leaving the scope unit are to be stored for the purpose of the delayed evaluation (but only for that, i.e. the values are not accessible for the user).

lazy value or template: value or template instance for which the expression, initializing or partly initializing it (including actual parameters passed to in formal parameters), is subject to lazy evaluation

NOTE:	When, during execution, the delayed (lazy) evaluation is taking place, its result is stored in the lazy value or template and the lazy instance is used further on like ordinary values and templates, until the next use of the lazy variable or parameter on the left hand side of an assignment. When a new content is assigned to a lazy instance or to its subpart, the right hand side of the assignment is subject to lazy evaluation again. If, during execution, no expression referencing the lazy object is evaluated, the lazy value or template instance is never evaluated.

left hand side (of assignment): value or template variable identifier or a field name of a structured type value or template variable (including array index if any), which stands left to an assignment symbol (:=)

NOTE:	A constant, module parameter, timer, structured type field name or a template header (including template type, name and formal parameter list) standing left of an assignment symbol (:=) in declarations and or a modified template definitions are out of the scope of this definition as not being part of an assignment.

local visibility: attribute of an entity (constant, variable, etc.) that its identifier can be referenced only within the function, test case or altstep where it is defined

main test component (MTC): See Recommendation ITU-T X.292 [3].

object: instance of one of the object types (component, default, port and timer)

NOTE:	Objects of type default, port or timer, which are owned by the component that instantiated them, are local objects while objects of type component are global objects. Global objects can be referenced from other component scopes while references to local objects can only be used by the component they are bound to.

object reference: special kind of value used for instances of component, default, port and timer types which represents a reference to an existing entity in the TE

NOTE:	When used in assignments or parameter passing, only the reference to the entity is copied, but not the entity itself. An object reference can also be initialized with the special value null in which case it does not reference an object.

out parameterization: kind of parameterization where the actual parameter's content (the argument) is not passed to the formal parameter when the parameterized object is invoked, but the content of the formal parameter is passed back to the actual parameter when the invoked object completes, if the formal parameter has been initialized during the invocation and the actual parameter is the reference evaluated at the time of the invocation

NOTE 1:	In out parameterization, parameters are passed by value.

NOTE 2:	Out parameters can be used for functions, altsteps, and test cases only, if not restricted by further rules, e.g. altsteps activated as defaults.

NOTE 3:	Formal an out parameters are uninitialized (unbound) when the invoked object is entered.

parallel test component (PTC): See Recommendation ITU-T X.292 [3].

partially initialized: value or template is partially initialized if initialization has taken place on it or to at least one of its fields or elements

NOTE:	A template variable is initialized if a matching mechanism has been assigned to it or to at least one of its fields or elements, directly or indirectly via expansion (see clause 15.6). A template is initialized if a matching mechanism has been assigned to it, directly or indirectly via expansion (see clause 15.6).

passing by reference: ability to link an actual parameter with a formal parameter of a function, altstep or test case and to control its actual value within the function, altstep or test case by using the formal parameter reference, i.e. no copy of the data content is made and the actual and formal parameters share the same data content

passing by value: ability to make a copy of a data content of an actual or formal parameter before passing it to a formal or actual parameter, i.e. the actual and formal parameters do not share the same data content

NOTE:	When passing object references by value, a new reference is created, but the referenced entity remains the same.

qualified name: TTCN-3 elements can be identified unambiguously by qualified names

NOTE:	For modules, the qualified name is the <module name>. For global definitions such as testcases, functions, etc., the qualified name is <module name>.<definition name>. For control, the qualified name is <module name>.control. For local definitions, such as variables, local templates, etc. within a global definition, the qualified name is <module name>.<global definition name>.<local definition name>.

right hand side (of assignment): expression, template reference or signature parameter identifier which stands right to an assignment symbol (:=)

NOTE:	Expressions and template references standing right of an assignment symbol (:=) in constant, module parameter, timer, template or modified template declarations are out of the scope of this definition as not being part of an assignment.

root type: root types of types derived from TTCN-3 basic types are the respective basic types

NOTE 1:	The root type of user defined record types is record, the root type of user defined record of and array types is record of, the root type of user defined set types is set, the root type of user defined set of types is set of. The root type of user defined union types is union and the root type of anytypes is anytype. The root types of special configuration types are default or component, respectively. Port types do not have a root type.

NOTE 2:	As address is more a predefined type name than a distinct type with its own properties, the root type of an address type and all of its derivatives are the same as the root type was, if the type was defined with a name different from address.

static parameterization: form of parameterization, in which actual parameters are independent of runtime events; i.e. known at compile time or in case of module parameters are known by the start of the test suite execution

NOTE 1:	A static parameter is to be known from the test suite specification, (including imported definitions), or the test system is aware of its value before execution time.

NOTE 2:	All types are known at compile time, i.e. are statically bound.

strong typing: strict enforcement of type compatibility by type name equivalence with no exceptions

system under test (SUT): See Recommendation ITU-T X.290 [5].

template: TTCN-3 data objects are values or templates by definition. A TTCN-3 template identifies a subset of the values of its type (where the subset may contain a single instance of the type, several instances or all instances) or the matching mechanism omit

NOTE:	Templates are defined by global and local templates, template variable definitions, or formal template parameters. Any of those are templates from the point of view of their usage, irrespective of their actual content; for example, a template variable containing a specific value is a template.

template parameterization: ability to pass a template as an actual parameter into a parameterized object via a template parameter

NOTE 1:	This actual template parameter is added to the specification of that object and may complete it.

NOTE 2:	Values passed to formal template parameters are considered to be in-line templates (see clause 15.4).

test behaviour: (or behaviour) test case, function or altstep started on a test component when executing an execute or a start component statement and all functions and altsteps called recursively

NOTE:	During a test case execution each test component has its own behaviour and hence several test behaviours may run concurrently in the test system (i.e. a test case can be seen as a collection of test behaviour).

test case: See Recommendation ITU-T X.290 [5].

test case error: See Recommendation ITU-T X.290 [5].

test suite: set of TTCN-3 modules that contains a completely defined set of test cases, optionally supplemented with one or more TTCN-3 control functions

test system: See Recommendation ITU-T X.290 [5].

test system interface: test component that provides a mapping of the ports available in the (abstract) TTCN-3 test system to those offered by the SUT

type compatibility: language feature that allows to use values, expressions or templates of a given type as actual values of another type

EXAMPLE:	At assignments, as actual parameters at calling a function, referencing a template, etc. or as a return value of a function.

type context: "In the context of a type" means that at least one object involved in the given TTCN-3 action (an assignment, operation, parameter passing, etc.) identifies a concrete type unambiguously

NOTE:	Either directly (e.g. an in-line template) or by means of a typed TTCN-3 object (e.g. via a constant, variable, formal parameter, etc.).

uninitialized: value or template is uninitialized as long as no initialization of it or at least one of its parts has occurred

unqualified name: unqualified name of a TTCN-3 element is its name without any qualification

user-defined type: type that is defined by subtyping of a basic type or declaring a structured type

NOTE:	User-defined types are referenced by their identifiers (names).

value: TTCN-3 data objects are values or templates by definition. A TTCN-3 value is an instance of its type

NOTE:	Values are defined by module parameters, constants, value variables, or formal value parameters. Any of those are value objects from the point of view of their usage. A template containing only specific value matching - though referring to a single instance of its type - is not a value object, but is a template object.

value list notation: notation that can be used for record, set, record of and set of values, where the values of the subsequent fields or elements are listed within a pair of curly brackets ("{" and "}"), without an explicit identification of the field name or element position

value notation: notation by which an identifier is associated with a given value or range of a particular type

NOTE:	Values may be constants or variables.

value parameterization: ability to pass a value as an actual parameter into a parameterized object via a value parameter

NOTE:	This actual value parameter is added to the specification of that object and may complete it.

## 3.2	Symbols

Void.

## 3.3	Abbreviations

For the purposes of the present document, the following abbreviations apply:

API	Application Programming Interface

ASN	Abstract Syntax Notation

ASP	Abstract Service Primitive

NOTE:	See Recommendation ITU-T X.290 [5].

ATS	Abstract Test Suite

BER	Basic Encoding Rules

BMP	Basic Multilingual Plane

BNF	Backus-Nauer Form

BOM	Byte Order Mark

CORBA®	Common Object Request Broker Architecture

ETS	Executable Test Suite

FIFO	First In First Out

GFT	Graphical presentation Format

ICS	Implementation Conformance Statement

IDL	Interface Definition Language

IRV	International Reference Version

ITU-T	International Telecommunication Union - Telecommunication standardization sector

IUT	Implementation Under Test

IXIT	Implementation eXtra Information for Testing

JSON	JavaScript Object Notation

MTC	Main Test Component

PDU	Protocol Data Unit

NOTE:	See Recommendation ITU-T X.290 [5].

PTC	Parallel Test Component

RHS	Right Hand Side (of assignment)

SDL	Specification and Description Language

SUT	System Under Test

TCI	TTCN-3 Control Interfaces

TE	TTCN-3 Executable

NOTE:	See also ETSI ES 201 873-5 [i.3].

TFT	Tabular presentation Format

TRI	TTCN-3 Runtime Interfaces

TSI	Test System Interface

TTCN-3	Testing and Test Control Notation version 3

UCS	Universal Character Set

UID	Short identifier for character code point

NOTE:	See ISO/IEC 10646 [2], clauses 6.5 and 6.6.

USI	UCS Short Identifier

UTF	UCS Transformation Format

UTF-8	Unicode Transformation Format-8

UTF-16	Unicode Transformation Format-16

UTF-16BE	Unicode Transformation Format-16 big-endian

UTF-16LE	Unicode Transformation Format-16 little-endian

UTF-32	Unicode Transformation Format-32

UTF-32BE	Unicode Transformation Format-32 big-endian

UTF-32LE	Unicode Transformation Format-32 little-endian

XML	eXtensible Markup Language

# 4	Introduction

## 4.0	General

TTCN-3 is a flexible and powerful language applicable to the specification of all types of reactive system tests over a variety of communication interfaces. Typical areas of application are protocol testing (including mobile and Internet protocols), service testing (including supplementary services), module testing, testing of CORBA® based platforms, API testing, etc. TTCN-3 is not restricted to conformance testing and can be used for many other kinds of testing including interoperability, robustness, regression, system and integration testing.

NOTE 1:	CORBA® is the trade name of a product supplied by Object Management Group®. This information is given for the convenience of users of the present document and does not constitute an endorsement by ETSI of the product named. Equivalent products should be used if they can be shown to lead to the same results.

TTCN-3 includes the following essential characteristics:

the ability to specify dynamic concurrent testing configurations;

operations for procedure-based and message-based communication;

the ability to specify encoding information and other attributes (including user extensibility);

the ability to specify data and signature templates with powerful matching mechanisms;

value parameterization;

the assignment and handling of test verdicts;

test suite parameterization and test case selection mechanisms;

combined use of TTCN-3 with other languages;

well-defined syntax, interchange format and static semantics;

different presentation formats (e.g. tabular and graphical presentation formats);

a precise execution algorithm (operational semantics).

NOTE 2:	The present document uses the following model of concept description: concepts, principles and mechanisms are explained in (introductory) text at the beginning of a clause. For every concept having concrete syntax, the syntactical structure of that concept is presented afterwards. The syntactical structure follows the conventions for the TTCN-3 syntax description in clause A.1.1 and uses rules of the TTCN-3 BNF given in clause A.1. A semantic description follows the syntactic structure. The restrictions on the concept are listed subsequently. Finally, examples on the usage of the concept are given.

In case of a contradiction between the body of the present document (clauses 5 to 27) and annex A of the present document, annex A has the priority.

## 4.1	The core language and presentation formats

The TTCN-3 specification is separated into several parts (see figure 1).

The first part, defined in the present document, is the core language.

The third part, defined in ETSI ES 201 873-3 [i.2], is the graphical presentation format.

The fourth part, ETSI ES 201 873-4 [1], contains the operational semantics of the language.

The fifth part, ETSI ES 201 873-5 [i.3], defines the TTCN-3 Runtime Interface (TRI).

The sixth part, ETSI ES 201 873-6 [i.4], defines the TTCN-3 Control Interfaces (TCI).

The seventh part, ETSI ES 201 873-7 [i.5], specifies the use of ASN.1 definitions with TTCN-3.

The eighth part, ETSI ES 201 873-8 [i.6], specifies the use of IDL definitions with TTCN-3.

The ninth part, ETSI ES 201 873-9 [i.7] specifies the use of XML definitions with TTCN-3.

The tenth part, ETSI ES 201 873-10 [i.8] specifies documentation tags for TTCN-3.

The core language serves three purposes:

a)	as a generalized text-based test language in its own right;

b)	as a standardized interchange format of TTCN-3 test suites between TTCN-3 tools;

c)	as the semantic basis (and where relevant, the syntactical basis) for various presentation formats.

The core language may be used independently of the presentation formats. However, neither the tabular format nor the graphical format can be used without the core language. Use and implementation of these presentation formats will be done on the basis of the core language.

The tabular format and the graphical format are the first in an anticipated set of different presentation formats. These other formats should be standardized presentation formats or they may be proprietary presentation formats defined by TTCN-3 users themselves. These additional formats are not defined in the present document.

TTCN-3 may optionally be used with TTCN-3 packages, which define additional concepts for specific purposes.

TTCN-3 may optionally be used with other type-value notations in which case definitions in other languages may be used as alternative data type and value syntax. Other parts of the TTCN-3 standard specify use of some other languages with TTCN-3. The support of other languages is not limited to those specified in the ETSI ES 201 873 series of documents but to support languages for which combined use with TTCN-3 is defined, rules given in the present document apply.

Figure 1: User's view of the core language, its packages and the various presentation formats

The core language is defined by a complete syntax (see annex A) and operational semantics (ETSI ES 201 873-4 [1]). It contains minimal static semantics (provided in the body of the present document and in annex A) which do not restrict the use of the language due to some underlying application domain or methodology.

## 4.2	Unanimity of the specification

The language is specified syntactically and semantically in terms of a textual description in the body of the present document (clauses 5 to 27) and in a formalized way in annex A. In each case, when the textual description is not exhaustive, the formal description completes it. If the textual and the formal specifications are contradictory, the latter should take precedence.

## 4.3	Conformance

For an implementation claiming to conform to this version of the language, all features specified in the present document should be implemented consistently with the requirements given in the present document and in ETSI ES 201 873-4 [1].

# 5	Basic language elements

## 5.0	General

The top-level unit of TTCN-3 is a module. A module cannot be structured into sub-modules. A module can import definitions from other modules. Modules can have module parameters to allow test suite parameterization.

A module consists of a set of definitions that define test components, communication ports, all kinds of types (see clause 6), constants, test data templates, functions including the module control function, signatures for procedure calls at ports, test cases, etc.

The module control function calls the test cases and controls their execution. The control function may also declare (local) variables, etc. Program statements (such as if-else and do-while) can be used to specify the selection and execution order of individual test cases. The concept of global variables is not supported in TTCN-3.

TTCN-3 has a number of predefined data types that include basic types (such as integer, float, boolean, verdicttyte and string tyes) as well as structured types (such as record, set, union, enumerated types and arrays) composed of basic types.

A special kind of data structure called a template provides parameterization and matching mechanisms for specifying test data to be sent or received over the test ports. The operations on these ports provide both message-based and procedure-based communication capabilities. Procedure calls may be used for testing implementations which are not message based.

Dynamic test behaviour is expressed as test cases. TTCN-3 program statements include powerful behaviour description mechanisms such as alternative reception of communication and timer events, interleaving and default behaviour. Test verdict assignment and logging mechanisms are also supported.

Finally, TTCN-3 language elements may be assigned attributes such as encoding information and display attributes. It is also possible to specify (non-standardized) user-defined attributes.

The TTCN-3 language elements are summarized in table 1.

Table 1: Overview of TTCN-3 language elements

| Language element | Associated keyword | Specified in module definitions | Specified in module control | Specified in functions/ altsteps/test cases | Specified in test component type |
| --- | --- | --- | --- | --- | --- |
| TTCN-3 module definition | module |  |  |  |  |
| Import of definitions from other module | import | Yes |  |  |  |
| Grouping of definitions | group | Yes |  |  |  |
| Data type definitions | type | Yes |  |  |  |
| Communication port definitions | port | Yes |  |  |  |
| Test component definitions | component | Yes |  |  |  |
| Signature definitions | signature | Yes |  |  |  |
| External function definitions | external | Yes |  |  |  |
| Constant definitions | const | Yes | Yes | Yes | Yes |
| Data/signature template definitions | template | Yes | Yes | Yes | Yes |
| Function definitions | function | Yes |  |  |  |
| Altstep definitions | altstep | Yes |  |  |  |
| Test case definitions | testcase | Yes |  |  |  |
| Value variable declarations | var |  | Yes | Yes | Yes |
| Template variable declarations | var template |  | Yes | Yes | Yes |
| Timer declarations | timer |  | Yes | Yes | Yes |
| NOTE: The notions "definition" and "declaration" of variables, constants, types and other language elements are used interchangeably throughout the present document. The distinction between both notions is useful only for implementation purposes, as it is the case in programming languages like C and C++. On the level of TTCN-3, the notions have equal meaning. |  |  |  |  |  |

## 5.1	Identifiers and keywords

TTCN-3 identifiers are case sensitive. TTCN-3 keywords shall be written in all lowercase letters (see annex A). TTCN-3 keywords shall neither be used as identifiers of TTCN-3 objects nor as identifiers of objects imported from modules of other languages. The same rules apply to names of predefined TTCN-3 functions (see annex C).

Special TTCN-3 modifiers are identifiers prefixed with the @-symbol (see annex A). They modify the default semantics of the language element they are applied to in the specified way. If more than one modifier is applied to a language element, they may be applied in any order.

NOTE:	These modifiers are useful for refining or modifying existing language features, for example in the context of the optional extension packages of TTCN-3 since they cannot lead to backward incompabilities with existing reserved keywords or identifiers.

## 5.2	Scope rules

### 5.2.0	General

TTCN-3 provides the following basic units of scope:

a)	module definitions part;

b)	component types;

c)	functions;

d)	altsteps;

e)	test cases;

f)	statement blocks;

g)	templates;

h)	user defined named types;

i)	port types.

NOTE 1:	Additional scoping rule for groups is given in clause 8.2.2.

NOTE 2:	Additional scoping rule for counters of for loops is given in clause 19.4.

NOTE 3:	Statement blocks may include declarations. They may occur as stand-alone statement blocks, embedded in another statement block or within compound statements, e.g. as body of a while loop.

NOTE 4:	Built in TTCN-3 types like integer, charstring, anytype, etc. are not scope units, but all named user defined types are scope units, independent of their kinds.

Each unit of scope consists of (optional) declarations. The scope units: functions (including the module control function), test cases, altsteps and statement blocks may additionally specify some form of behaviour by using the TTCN-3 program statements and operations (see clause 18).

Definitions made in the module definitions part but outside of other scope units are globally visible, i.e. may be used elsewhere in the module, including all functions, test cases and altsteps defined within the module. Identifiers imported from other modules are also globally visible throughout the importing module.

Definitions made in a test component type may be used in a component type extending this component type definition, and in functions, test cases and altsteps referencing that component type or a compatible test component type (see clause 6.3.2.7) by a runs on clause.

Test cases, altsteps and functions are individual scope units without any hierarchical relation between them, i.e. declarations made inside their body have local visibility and shall only be used in the given test case, altstep or function (e.g. a declaration made in a test case is not visible in a function called by the test case or in an altstep used by the test case).

Stand-alone statement blocks and statements within compound statements, like e.g. if-else, while, do-while, or alt statements may be used within test cases, altsteps, functions, or may be embedded in other statement blocks or compound statements, e.g. an if-else statement that is used within a while loop.

Statement blocks and embedded statement blocks have a hierarchical relation both to the scope unit including the given statement block and to any embedded statement block. Declarations made within a statement block have local visibility.

The hierarchy of scope units is shown in figure 2. Declarations of a scope unit at a higher hierarchical level are visible in all units at lower levels within the same branch of the hierarchy. Declarations of a scope unit in a lower level of hierarchy are not visible to those units at a higher hierarchical level.

Figure 2: Hierarchy of scope units

EXAMPLE 1:	Local scopes

module MyModule

{	:

const integer c_myConst := 0; // c_myConst is visible to f_myBehaviourA and f_myBehaviourB

:

function f_myBehaviourA()

{	:

const integer c_a := 1;   // The constant c_a is only visible to f_myBehaviourA

:

}

function f_myBehaviourB()

{	:

const integer c_b := 1;   // The constant c_b is only visible to f_myBehaviourB

:

}

}

EXAMPLE 2:	Component type scopes

type component MyComponentType {

const integer cc_myConst := 1;

...

}

type component MyExtendedComponentType extends MyComponentType {

var integer vc_myVar:= 2 * cc_myConst; // using cc_myConst of MyComponentType

...

}

### 5.2.1	Scope of formal parameters

The scope of formal parameters in a parameterized object (e.g. in a function definition) shall be restricted to the definition in which the parameters appear and to the lower levels of scope in the same scope hierarchy. That is they follow the scope rules for local definitions (see clause 5.2).

### 5.2.2	Uniqueness of identifiers

TTCN-3 requires uniqueness of identifiers, i.e. all identifiers in the same scope hierarchy shall be distinctive. This means that a declaration in a lower level of scope shall not re-use the same identifier as a declaration in a higher level of scope in the same branch of the scope hierarchy.

The identifier of a module (its module name) or of an imported module belongs to the scope unit of the module and cannot be used as identifier for other definitions inside this module. Identifiers for fields of structured types, enumerated values and groups do not have to be globally unique, however in the case of enumerated values the identifiers, within the same module, they shall only be reused for enumerated values within other enumerated types or as identifiers for fields of structured types. In addition, enumeration values shall not be used as names of value or template definitions of imported enumeration types, defining the given enumeration value (see also clause 8.2.3.1, example 4). The rules of identifier uniqueness shall also apply to identifiers of formal parameters.

EXAMPLE 1:	Nested scopes

module MyModule

{	:

const integer c_a := 1;

:

function f_myBehaviourA()

{	:

const integer c_a := 1; // Is NOT allowed: clash with global constant c_a

:

if(…)

{	:

const boolean c_a := true; // Is NOT allowed: clash with local constant c_a

:

}

}

}

EXAMPLE 2:	Independent scopes

// The following IS allowed as the constants are not declared in the same scope hierarchy

// (assuming there is no declaration of c_a in module header)

function f_myBehaviourA()

{	:

const integer c_a := 1;

:

}

function f_myBehaviourB()

{	:

const integer c_a := 1;

:

}

EXAMPLE 3:	Module scopes

module MyModuleB {

import from MyModuleA { … }

function f_myFunction() {

var integer MyModuleB:= 1; // Is NOT allowed: class with module name

:

}

type boolean MyModuleA; // Is NOT allowed: class with imported module name

}

## 5.3	Ordering of language elements

Generally, the order in which declarations can be made is arbitrary. Inside a statement block, such as a function body or a branch of an if-else statement, declarations (if any) can occur at any place where a statement can occur.

EXAMPLE:

// This is a legal mixing of TTCN-3 declarations

:

var MyVarType v_myVar2 := 3;

const integer c_myConst:= 1;

if (v_myVar2+c_myConst > 10)

{

var integer v_myVar1:= 1;

:

v_myVar1:= v_myVar1 + 10;

var integer v_myVar2:= 2*v_myVar1;

:

}

:

Declarations in the module definitions part and in a component type definition may be made in any order. However inside test case definitions, functions, altsteps, and statement blocks, all required declarations shall be given before their first place of usage. This means in particular, local variables, local timers, and local constants shall never be used before they are declared. The only exceptions to this rule are labels. Forward references to a label may be used in goto statements before the label occurs (see clause 19.8).

## 5.4	Parameterization

### 5.4.0	General

TTCN-3 allows to parameterize modules, templates, functions, altsteps and testcases. Values, templates, timers, and ports may be used as actual parameters. A summary of which language elements can be parameterized and what can be passed to them as parameters is given in table 2.

NOTE:	Type parameterization for TTCN-3 is defined in the optional package [i.12].

Table 2: Overview of parameterizable TTCN-3 objects

| Keyword | Allowed kind of Parameterization | Allowed form of Parameterization | Allowed types in formal parameter lists |
| --- | --- | --- | --- |
| module | Value parameterization | Static at start of runtime | all basic types, all user-defined types and address type. |
| template | Value and template parameterization | Dynamic at runtime | all basic types, all user-defined types, address type and template. |
| function | Value and template parameterization | Dynamic at runtime | all basic types, all user-defined types, address type, component type, port type, default, template and timer. |
| altstep | Value and template parameterization | Dynamic at runtime | all basic types, all user-defined types, address type, component type, port type, default, template and timer. |
| testcase | Value and template parameterization | Dynamic at runtime | all basic types and of all user-defined types, address type, component type and template. |
| NOTE: Signatures are not shown in the table, because a signature declares parameters only. The templates for the signatures can be parameterized, however. |  |  |  |

### 5.4.1	Formal parameters

#### 5.4.1.0	General

TTCN-3 modules, structured types, templates, functions, altsteps, and testcases may be defined incompletely, i.e. some entities (variables, templates, ports, timers, etc.) used by the above objects may not be resolved in the definition of the object. These objects are called parameterized objects. Formal entities replacing the unresolved entities in the parameterized object's definition are called formal parameters.

Formal parameters of parameterized templates, functions, altsteps, and testcases are defined in formal parameter lists. Formal parameters of modules are defined in module parameter definitions (see clause 8.2.1).

Formal parameters shall be in, inout or out parameters (see terms in clause 3.1). If not stated otherwise, a formal parameter is an in parameter. For all these three sorts of parameter passing, the formal parameters can both be read and set (i.e. get new values being assigned) within the parameterized object. Formal parameters can be used directly as actual parameters for other parameterized objects, e.g. as actual parameters in function invocations or as actual parameters in template instances.

If parameters are passed by value (i.e. in case of in and out parameters), type compatibility rules specified in clause 6.3 apply. When parameters are passed by reference, strong typing is required. Both the actual and formal parameter shall be of the same type.

Formal in parameters may have default values. This default value is used when no actual parameter is provided.

NOTE 1:	Although out parameters can be read within the parameterized object, they do not inherit the value of their actual parameter; i.e. they should be set before they are read.

Formal value or template parameters may be declared lazy using the @lazy modifier. The behaviour of fuzzy parameters is defined in clause 3.1, definition of lazy values or templates. See examples in clause 5.4.1.1.

Formal value or template parameters may be declared fuzzy using the @fuzzy modifier. The behaviour of lazy parameters is defined in clause 3.1, definition of fuzzy values or templates. See examples in clause 5.4.1.1.

Formal value or template parameters that are declared lazy or fuzzy can be additionally declared deterministic using the @deterministic modifier.

NOTE 2:	The actual values of component variables used in the delayed evaluation of a lazy or fuzzy parameter may differ from their values at the time, when the parameterized function or alstep was called.

Assigning default values for lazy and fuzzy formal parameters does not change the parameters' semantics: when the default values are used as actual values for the parameters, they shall be evaluated the same way (i.e. delayed) as if an actual parameter was provided.

Lazy and fuzzy properties are valid only in the scope, where the parameters' names are visible. For example, if a fuzzy parameter is passed to a formal parameter declared without a modifier, it loses its fuzzy feature inside the called function. Similarly, if it is passed to a lazy formal parameter, it becomes lazy within the called function.

#### 5.4.1.1	Formal parameters of kind value

Values of all basic and user-defined types, address, component, port and timer types, and the default type can be passed as value parameters.

Syntactical Structure

[ ( in | inout | out ) ] [ ( @lazy | @fuzzy ) [@deterministic] ]

Type ValueParIdentifier [ArrayDef] [ ":=" ( Expression | "-" ) ]

Semantic Description

Value formal parameters can be used within the parameterized object the same way as values, for example in expressions.

Value formal parameters may be in, inout or out parameters. The default for value formal parameters is in parameterization which may optionally be denoted by the keyword in. Using of inout or out kind of parameterization shall be specified by the keywords inout or out respectively.

In parameters may have a default value, which is given by an expression assigned to the parameter. Formal parameters of modified templates may inherit the default values from the corresponding parameters of their parent templates; this shall explicitly be denoted by using a dash (don't change) symbol at the place of the modified template parameters' default value.

NOTE 1:	If functions are used for the initialization of default values of in parameters, it is strongly advised to avoid side effects during the evaluation of default values. Side effects may cause non-deterministic test executions. They can be avoided, e.g. by adhering to the rules defined in clause 16.1.4.

TTCN-3 supports value parameterization according to the following rules:

the language element module allows static value parameterization to support test suite parameters, i.e. this parameterization may or may not be resolvable at compile-time but shall be resolved by the commencement of runtime (i.e. static at runtime). This means that, at runtime, module parameter values are globally visible but not changeable (see more details in clause 8.2);

the language elements template, testcase, altstep and function support dynamic value parameterization (i.e. this parameterization shall be resolved at runtime).

NOTE 2:	Component and default references are also handled as value parameters. In the case of component references, the corresponding component type is the type of the formal parameter. In the case of default references the TTCN-3 type default is the type of the formal parameter.

Restrictions

a)	Language elements which cannot be parameterized are: const, var, timer, control, record of, set of, enumerated, port, component and subtype definitions, group and import.

b)	Formal value parameters of templates, and of altsteps activated as defaults (see clause 20.5.2) shall always be in parameters.

c)	Restrictions on module parameters are given in clause 8.2.

d)	Default values can be provided for in parameters only.

e)	The expression of formal parameter's default value has to be compatible with the type of the parameter. The expression may be any expression that is well-defined at the beginning of the scope of the parameterized entity, but shall not refer to other parameters of the same parameter list.

f)	Default values of component type formal parameters shall be one of the special values null, mtc, self, or system.

g)	Default values of port, timer or default type formal parameters shall be the special value null.

h)	The dash (don't change) symbol shall be used with formal parameters of modified templates only (see also clause 15.5).

i)	For formal value parameters of templates the restrictions specified in clause 15 shall apply.

j)	Only in parameters can be declared lazy or fuzzy.

k)	When parameters are referenced (e.g. in assignments, expressions, template bodies, etc.), the rules for variables shall apply.

l)	Only function and altstep definitions with the exception of functions or altsteps started as test component behaviour (see clause 21.3.2) may have formal parameters of a port, timer or default type or of a type that contains a direct or indirect element or field of a port, default or timer type.

m)	Only function, altstep and testcase definitions may have formal parameters of a component type or of a type that contains a direct or indirect element or field of a component type.

n)	If a lazy or fuzzy value parameter is used in deterministic contexts (i.e. during the evaluation of a snapshot or initialization of global non-fuzzy templates), it shall be declared @deterministic.

Examples

EXAMPLE 1:	In, out and inout formal parameters

function f_myFunction1(in boolean p_myReferenceParameter){ … };

// p_myReferenceParameter is an in value parameter. The parameter can be read. It can also be

// set within the function, however, the assignment is local to the function only

function f_myFunction2(inout boolean p_myReferenceParameter){ … };

// p_myReferenceParameter is an inout value parameter. The parameter can be read and set

// within the function - the assignment is not local

function f_myFunction3(out template boolean p_myReferenceParameter){ … };

// p_myReferenceParameter is an out value parameter. The parameter can be set within the 
	// function, the assignment is not local. It can also be read, but only after it has been set.

EXAMPLE 2:	Reading and setting parameters

type record MyMessage {

integer f1,

integer f2

}

function f_myMessage (integer p_int) return MyMessage {

var integer v_f1, v_f2;

v_f1 := f_mult2 (p_int);

// parameter p_int is passed on; as the parameter of the called function f_mult2 is

// defined as an inout parameter, it passes back the changed value for p_int,

v_f2 := p_int;

return {v_f1, v_f2};

}

function f_mult2 (inout integer p_integer) return integer {

p_integer := 2 * p_integer;

// the value of the formal parameter is changed; this new value is passed back when

// f_mult2 completes

return p_integer-1

}

testcase TC_01 () runs on MTC_PT {

...

p1.send (f_myMessage(5))

// the value sent is { f1 := 9 , f2 := 10 }

...

}

EXAMPLE 3:	Function with default value for parameter

function f_comp (in integer p_int1, in integer p_int2 := 3) return integer {

var integer v_v := p_int1 + p_int2;

return v_v;

}

function f_f () {

var integer v_w;

v_w := f_comp(1);   // same as calling f_comp(1,3);

v_w := f_comp(1,2); // value 2 is taken for parameter p_int2 and not its default value 3

…

}

type component Comp { var integer i := 0 }

function g(integer x := f_comp(i)) runs on Comp return integer {

// reference to i from Comp is allowed in default value of parameter x

return x;

}

function h(integer y := g()+i) runs on Comp {

// reference to g is allowed because it has a compatible runs on clause as h

}

EXAMPLE 4:	Direct passing of formal parameters to functions

function f_myFunc2(in bitstring p_refPar1, inout integer p_refPar2) return integer {

:

}

function f_myFunc1(inout bitstring p_refPar1, out integer p_refPar2) return integer {

:

return f_myFunc2(p_refPar1, p_refPar2);

}

// p_refPar1 and p_refPar2 can be passed directly to a function invocation

EXAMPLE 5:	Lazy and fuzzy parameters

type component MyComp { var integer vc_int }

function f_MyLazyFuzzy(in @lazy integer p_lazy, in @fuzzy integer p_fuzzy) runs on MyComp {

//When called from MyCalling:

v_int := 1;

log(p_lazy); //will log 2 as function double with actual parameter vc_int equals 1 is called

//here; 2 is  stored in p_lazy (also, function double stores 2 in v_int)

log(p_lazy); //will log 2 again as p_lazy is not re-evaluated

log(p_fuzzy);//will log 4 as function double with actual parameter vc_int equals 2 is called

// here (also, function double stores 4 in vc_int)

log(p_fuzzy) //will log 8 as function double is re-evaluated with actual parameter 4

}

function f_double (in integer p_in) runs on MyComp return integer{

p_in := 2* p_in;

v_int := p_in;

return p_in

}

testcase TC_MyCalling() runs on MyComp {

vc_int := 0;

f_myLazyFuzzy (f_double(vc_int), f_double(vc_int) )

}

EXAMPLE 6:	Difference between passing by value and passing by reference

function f_byValue (in integer p_int1, in integer p_int2) {

p_int2 := p_int2 + 1;

log(p_int1);

log(p_int2);

}

function f_byReference (inout integer p_int1, inout integer p_int2) {

p_int2 := p_int2 + 1;

log(p_int1);

log(p_int2);

}

function f_f () {

var integer v_int := 1;

f_byValue(v_int, v_int); // prints 1 and 2

log(v_int); // prints 1

f_byReference(v_int, v_int); // prints 2 and 2

log(v_int); // prints 2

}

#### 5.4.1.2	Formal parameters of kind template

Template kind parameters are used to pass templates into parameterizable objects.

Syntactical Structure

[ in | inout | out ] TemplateModifier [ ( @lazy | @fuzzy ) [@deterministic] ]

Type ValueParIdentifier [ArrayDef] [ ":=" ( TemplateInstance | "-" ) ]

Semantic Description

Template parameters can be defined for templates, functions, altsteps, and test cases.

To enable a parameterized object to accept templates or matching symbols as actual parameters, the extra keyword template shall be added before the type field of the corresponding formal parameter. This makes the parameter a template parameter and in effect extends the allowed actual parameters for the associated type to include the appropriate set of matching attributes (see annex B) as well as the normal set of values.

Formal template parameters can be used within the parameterized object the same way as templates and template variables.

Formal template parameters may be in, inout or out parameters. The default for formal template parameters is in parameterization.

In parameters may have a default template, which is given by a template instance assigned to the parameter. Formal template parameters of modified templates may inherit their default templates from the corresponding parameters of their parent templates; this shall explicitly be denoted by using a dash (don't change) symbol at the place of the modified template parameter's default template. If a default template is used, it is evaluated in the scope of the parameterized entity, not the scope of the actual parameter list.

Formal template parameters can be restricted to accept actual parameters containing a restricted set of matching mechanisms only. Such limitations can be expressed by the restrictions omit, present, and value. The restriction template (omit) can be replaced by the shorthand notation omit. The meaning of the restrictions is explained in clause 15.8.

Restrictions

a)	Only function, testcase, altstep and template definitions may have formal template parameters.

b)	Formal template parameters of templates and of altsteps activated as defaults (see clause 20.5.2) shall always be in parameters.

c)	Default templates can be provided for in parameters only.

d)	The default template instance has to be compatible with the type of the parameter. The template instance may be any template expression that is well-defined at the beginning of the scope of the parameterized entity, but shall not refer to other parameters in the same parameter list.

e)	Default templates of component type formal parameters shall be built from the special values null, mtc, self, or system.

f)	Restrictions specified in clause 15 shall apply.

g)	The dash (don't change) symbol shall be used with formal parameters of modified templates only (see also clause 15.5).

h)	Only in template parameters can be declared lazy or fuzzy.

i)	When template parameters are referenced (e.g. in assignments, expressions, template bodies, etc.), the rules for template variables shall apply.

j)	If a lazy or fuzzy template parameter is used in deterministic contexts (i.e. during the evaluation of a snapshot or initialization of global non-fuzzy templates), it shall be declared @deterministic.

Examples

EXAMPLE 1:	Template with template parameter

// The template

template MyMessageType mw_myTemplate (template integer p_myFormalParam):=

{	field1 := p_myFormalParam,

field2 := pattern "abc*xyz",

field3 := true

}

// could be used as follows

pco1.receive(mw_myTemplate(?));

// or as follows

pco1.receive(mw_myTemplate(omit)); // provided that field1 is declared in MyMessageType as

// optional

EXAMPLE 2:	Function with template parameter

function f_myBehaviour(template MyMsgType p_myFormalParameter)

runs on MyComponentType

{	 :

pco1.receive(p_myFormalParameter);

:

}

EXAMPLE 3:	Template with restricted parameter

// The template

template MyMessageType mw_myTemplate1 (template ( omit ) integer p_myFormalParam):=

{	field1 := p_myFormalParam,

field2 := pattern "abc*xyz",

field3 := true

}

// could be used as follows

pco1.receive(mw_myTemplate1(omit));

// but not as follows

pco1.receive(mw_myTemplate1(?)); // AnyValue is not within the restriction

// the same template can be written shorter as

template MyMessageType mw_myTemplate2 (omit integer p_myFormalParam):=

{	field1 := p_myFormalParam,

field2 := pattern "abc*xyz",

field3 := true

}

### 5.4.2	Actual parameters

Values, templates, timers and/or ports can be passed into parameterized TTCN-3 objects as actual parameters. Actual parameters can be provided both as a list in the same order as the formal parameters as well as in an assignment notation explicitly using the associated formal parameter names or in a mixed notation where the first parameters are given in list notation and additional parameters in assignment notation.

Syntactical Structure

( Expression |												// for value parameter

TemplateInstance |										// for template parameter

"-" ) |													// to skip a parameter with default

ParameterId ":=" ( Expression | TemplateInstance | TimerRef | Port ) )

Semantic Description

Actual parameters that are passed by value to in formal value parameters shall be variables, literal values, module parameters, constants, value variables, invocations of value returning (external) functions, formal value parameters (of in, inout or out parameterization) of the current scope or expressions composed of the above.

Actual parameters that are passed to out formal value parameters shall be (template) variables, formal (template) parameters (of in, inout or out parameterization) or references to elements of (template) variables or formal (template) parameters of structured types. Furthermore it is allowed to use the dash symbol "-" as an actual out parameter, signifying that a possible result for that parameter will not be passed back.

Actual parameters that are passed to inout  formal value parameters shall be variables or formal value parameters (of in, inout or out parameterization) or references to elements of variables or formal value parameters of structured types.

NOTE 1:	Reference to a string element cannot be passed by reference as string types are not structured types.

Actual parameters that are passed to in formal template parameters shall be literal values, module parameters, constants, variables, invocations of value or template returning (external) functions, formal value parameters (of in, inout or out parameterization) of the current scope or expressions composed of the above, as well as templates, template variables or formal template parameters (of in, inout or out parameterization) of the current scope.

Actual parameters that are passed to out formal template parameters shall be template variables, formal template parameters or references to elements of  template variables or formal template parameters of structured types. Furthermore it is allowed to use the dash symbol "-" as an actual out parameter, signifying that a possible result for that parameter will not be passed back.

Actual parameters that are passed to inout formal template parameters shall be template variables or formal template parameters (of in, inout or out parameterization) of the current scope or references to elements of template variables or formal template parameters of structured types.

When actual parameters that are passed to in formal value or template parameters contain a value or template reference, rules for using references on the right hand side of assignments apply. When actual parameters that are passed to inout and out formal value or template parameters contain a value or template reference, rules for using references on the left hand side of assignments apply.

The values of out formal parameters are passed to the actual parameters in the same order as is the order of formal parameters in the definition of the parameterized TTCN-3 object. The value is passed back to the actual parameter only if within the invoked object a value is assigned to it. If no value is assigned, the actual parameter remains unchanged when the invoked object completes.

It is allowed to pass elements of structured values or templates (record, set, record of, set of, union and anytype values or templates) by reference. Modification of parameters passed this way affects the original structured value or template. Before passing the actual parameter, the rules for referencing the element on the left hand side of assignments are applied, expanding the structured value so that the referenced element becomes accessible (see clauses 6.2 and 15.6 for more details).

NOTE 2:	Because inout parameters are passed by reference and component variables are effectively also accessed by reference within a called function or altstep, passing parts of a structured component variable as an actual inout parameter may have confusing effects inside the parameterized behaviour: changing either the inout parameter or the component variable may also change the other simultaneously, which might break the intended algorithm. For this reason, such situations should be avoided.

When a formal parameter is an out parameter or has been defined with a default value or template, respectively, then it is not necessary to provide an actual parameter. In such a case the default value or template is taken as actual parameter.

The actual parameters are evaluated in the order of their appearance. If for some formal parameters, no actual parameter has been provided, if they are out parameters, the dash symbol "-" and for in parameters their default values are taken. Default values are evaluated after the evaluation of the actual parameters and the order of their evaluation corresponds to their order in the formal parameter list.

NOTE 3:	If assignment notation has been used for the actual parameter list, the order of the evaluation of actual parameters may differ from the order of the parameters in the formal parameter list.

The empty brackets for instances of parameterized templates that have only parameters with default values are optional when no actual parameters are provided, i.e. all formal parameters use their default values.

Restrictions

a)	When using list notation, the order of elements in the actual parameter list shall be the same as their order in the corresponding formal parameter list. For each formal inout parameter and for each in parameter without a default there shall be an actual parameter. The actual parameter of a formal out parameter or in parameter with default value can be skipped by using dash "-" as actual parameter. An actual parameter can also be skipped by just leaving it out if no other actual parameter follows in the actual parameter list - either because the parameter is last or because all following formal parameters are out parameters or have default values and are left out. The number of actual parameters in the list notation shall not exceed the number of parameters in the formal parameter list.

b)	Void.

c)	When using assignment notation, each formal parameter shall be assigned an actual parameter at most once. For each assigned actual parameter there shall exist a corresponding formal parameter of the same name. For each formal parameter without default value, there shall be an actual parameter. In order to use the default value of a formal parameter, no assignment for this specific parameter shall be provided.

d)	For in formal parameters, the type of the actual parameter shall be compatible with the type of the formal parameter. For out formal parameters, the type of the formal parameter shall be compatible with the type of the actual parameter. Strong typing is required for inout formal (parameters passed by reference). For in formal template parameters, the template restriction of the actual parameter shall not be less restrictive than the one of the formal parameter. For out formal template parameters, the template restriction of the actual parameter shall not be more restrictive than the one of the formal parameter. For inout formal template parameters, the template restriction of the actual and the formal parameter shall be the same.

e)	Actual parameters passed to restricted formal template parameters shall obey the restrictions given in clause 15.8.

f)	All parameterized entities specified as an actual parameter shall have their own parameters resolved in the top-level actual parameter list.

g)	If the formal parameter list of TTCN-3 objects function, testcase, altstep or external function is empty, then the empty parentheses shall be included both in the declaration and in the invocation of that object. In all other cases the empty parentheses shall be omitted.

NOTE 4:	signature objects also have formal parameters, see clauses 15.2 and 22.3 for their handling.

h)	Void.

i)	Restrictions on parameters passed to altsteps are given in clauses 16.2.1 and 20.5.2.

j)	Unless specified differently in the relevant clause(s), actual parameters passed to in or inout formal parameters shall be at least partially initialized (for an exemption see e.g. clause 16.1.2 of the present document).

k)	Functions, called by actual parameters passed to fuzzy or lazy formal parameters of the calling function, shall not have inout or out formal parameters. The called functions may use other functions with inout or out parameters internally.

l)	Actual parameters passed to out and inout parameters shall not be references to lazy or fuzzy variables.

m)	Whenever a value or template of a record, set, union, record of, set of, array and anytype type is passed as an actual parameter to an inout parameter, none of the fields or elements of this structured value or template shall be passed as an actual parameter to another inout parameter of the same parameterized TTCN-3 object. This restriction applies recursively to all sub-elements of the structured value or template in any level of nesting.

n)	If two or more actual parameters passed to inout parameters of the same parameterized TTCN-3 object contain a reference to distinct alternatives of the same union or anytype value, an error shall be produced.

o)	If the mixed notation is used, no value list notation shall be used following the first assignment notation and the parameters given in assignment notation shall not assign parameters that already have an actual parameter given in list notation.

p)	Actual parameters passed to @deterministic fuzzy or lazy formal parameters shall fulfill the restrictions imposed on content of functions used in special places given in clause 16.1.4.

Examples

EXAMPLE 1:	Formal and actual parameter lists have to match

// A function definition with a formal parameter list

function f_myFunction(integer p_formalPar1, boolean p_formalPar2, bitstring p_formalPar3) { … }

// A function call with an actual parameter list

f_myFunction(123, true,'1100'B);

// A function call with assignment notation for actual parameters

f_myFunction(p_formalPar1 := 123, p_formalPar3 := '1100'B, p_formalPar2 := true);

EXAMPLE 2:	In parameters

function f_myFunction(in template MyTemplateType p_myValueParameter){ … };

// p_myValueParameter is in parameter, the in keyword is optional

// A function call with an actual parameter

f_myFunction(m_myGlobalTemplate);

EXAMPLE 3:	Inout and out parameters

function f_myFunction(inout boolean p_myReferenceParameter){ … };

// p_myReferenceParameter is an inout parameter

// A function call with an actual parameter

f_myFunction(v_myBooleanVariable);

// The actual parameter can be read and set within the function

function f_myFunction(out template boolean p_myReferenceParameter){ … };

// p_myReferenceParameter is an out parameter

// A function call with an actual parameter

f_myFunction(v_myBooleanVariable);

// The actual parameter is initially unbound, but can be set and read within the function.

f_myFunction(-); // the outcoming value is not assigned to a variable

type record of integer RoI;

function f_swapElements (inout integer p_int1, inout integer p_int2) {

var integer v_tmp := p_int1;

p_int1 := p_int2;

p_int2 := v_tmp;

}

function f_testReferences (inout RoI p_roi, inout integer p_elem) { … }

:

var RoI v_roi := { 0, 1, 2, 3, 4, 5 };

f_swapElements(v_roi[0], v_roi[5]); // after the function call, v_roi is { 5, 1, 2, 3, 4, 0 }

f_testReferences(v_roi, v_roi[2]); // produces an error as elements of v_roi are not allowed

// to be passed by reference if the parent structure (v_roi) is passed by reference too.

function f_changeAndIncrement(out integer p_e, in integer p_v, inout integer p_i) {

p_i := p_i + 1;

p_e := p_v;

}

:

var integer v_i := 0;

f_changeAndIncrement(v_roi[v_i], 3, v_i); // increments p_i, but still assigns 3 to v_roi[0]

EXAMPLE 4:	A side effect caused by passing part of a component variable as inout parameter

type component MyComp {

var ROI v_rec := { 0, 1 }

}

testcase TC() runs on MyComp {

f_test(v_rec[1]) // passing 2nd element of component variable as inout parameter

log(v_rec); //will log { 2 , 2 }

}

function f_test(inout integer p_int) runs on MyComp {

v_rec := { 2 }; // now, isbound(p_int) == false

p_int := 2; // now, v_rec == { 2, 2 }

}

EXAMPLE 5:	Empty parameter lists

// A function definition with an empty parameter list shall be written as

function f_myFunction(){ … }

// and shall be called as

f_myFunction();

// A template definition with a default value for a formal parameter written as

template MyRecord m_mytemplate (integer p_myValue:= 1):= { … }

// may be used without actual parameter list  (i.e. the default value is used)

myPCO.send(m_mytemplate)

EXAMPLE 6:	Nested parameter lists

// Given the message definition

type record MyMessageType

{

integer 	field1,

charstring	field2,

boolean		field3

}

// A message template might be

template MyMessageType mw_myTemplate(integer p_myValue) :=

{

field1 := p_myValue,

field2 := pattern "abc*xyz",

field3 := true

}

// A test case parameterized with a template might be

testcase TC_001(template MyMessageType p_rxMsg) runs on PTC1 system TS1 {

:

myPCO.receive(p_rxMsg);

}

// When the test case is called in the control function and the parameterized template is

// passed as an actual parameter, the template's actual parameters shall be provided

control

{	:

execute(TC_001(mw_myTemplate(7)));

:

}

EXAMPLE 7:	A typical use case for lazy parameterization

modulepar boolean PX_LOG_MESSAGE := true;

function f_logMsg(@lazy charstring p_complex) {

if (PX_LOG_MESSAGE) {

log(p_complex);

}

}

function f_computeComplexMessage() return charstring {

// some complicated computation

}

f_logMsg(f_computeComplexMessage()); // f_computeComplexMessage() is only invoked if

// PX_LOG_MESSAGE is true

EXAMPLE 8:	Actual parameters passed to lazy and fuzzy formal parameters

type record MyMessage { integer id, float number }

type port MyPortType message { inout MyMessage }

type component MyMTC {

var integer vc_id;

port MyPortType p;

}

testcase TC_shootingMessages () runs on MyMTC {

connect(self:p,self:p);

f_sendLazy({vc_id, rnd()}); //note that at this point vc_id is unintialized yet

f_sendFuzzy({vc_id, rnd()})

}

function f_sendLazy(@lazy MyMessage p_pdu) runs on MyMTC {

for (vc_id := 1; vc_id<9; vc_id:=vc_id+1){

p.send(p_pdu); // the actual parameter passed to the formal parameter p_pdu is evaluated only

// in the first loop;let say rnd() returns 0.924946;

// the message { 1, 0.924946 } is sent out 8 times

}

setverdict(pass,"messages has been sent out")

}

function f_sendFuzzy(@fuzzy MyMessage p_pdu) runs on MyMTC {

for (vc_id := 1; vc_id<9; vc_id:=vc_id+1){

p.send(pdu); // the actual parameter passed to the formal parameter p_pdu is evaluated in each

// loop; let say rnd() returns 0.924946, 0.680497, 0.630836, 0.648681, 0.428501,

// 0.262539, 0.646990, 0.265262 in subsequent calls; the messages  1, 0.924946 },

// {{ 2, 0.680497 }, { 3, 0.630836 }, { 4, 0.648681 }, { 5, 0.428501 },

//  { 6, 0.262539 }, { 7, 0.646990 } and { 8, 0.265262 } are sent out in sequence

}

setverdict(pass,"messages has been sent out")

}

EXAMPLE 9:	Order of out parameters

function f_initValues (out integer p_par1, out integer p_par2) {

p_par1 := 1;

p_par2 := 2;

}

function f_f(){

var integer v_var1;

f_initValues(p_par2 := v_var1, p_par1 := v_var1);

// After this function call, v_var1 will contain 2, as parameters are assigned in

// the same order as in the definition of the f_initValues function. Thus p_par1 is

// assigned first to v_var1 and p_par2 after that overwriting the previous value.

}

EXAMPLE 10:	Skipped actual parameters

function f_skip (out integer p_par1, in integer p_par2 := 2) {

p_par1 := 1 + p_par2;

}

function f_f(){

// the following statements all have the same semantics :

f_skip (-,-); // p_par2 is initialized with default value 2 and

// the result of p_par1 is not assigned to any variable

f_skip (p_par1 := -, p_par2 := -);

f_skip (p_par2 := -); // skip p_par1

f_skip (-) ;  // skip p_par2 because it is the last

f_skip () ;   // skip p_par1 because all following are also skipped

}

EXAMPLE 11:	Mixed notation

function f_mixed (out integer p_par1, in integer p_par2 := 2, inout integer p_par3) {

p_par1 := 1 + p_par2;

}

function f_f(){

var integer v := 0;

// the following statements all have the same semantics:

f_mixed(-,2,v);

f_mixed(-,p_par2 := 2, p_par3 := v);

f_mixed(-,-,p_par3 := v);

f_mixed(-,p_par3 := v, p_par2 := 2);

// not allowed:

f_mixed(-,2,p_par3 := v, p_par2 := 5); // p_par2 is already assigned in list notation

}

## 5.5	Cyclic Definitions

Direct and indirect cyclic definitions are not allowed with the exception of the following cases:

a)	for recursive type definitions (see clause 6.2);

b)	function and altstep definitions (i.e. recursive function or altstep calls);

c)	cyclic import definitions, if the imported definitions only form allowed cyclic definitions.

NOTE 1:	Indirect cyclic definitions may be a result of imports of definitions that are needed for the usage of a definition but do not need to be known in the importing module (see clause 8.2.3.1).

NOTE 2:	For the detection of cycles only the main identifiers of the definition are used. For example, field identifiers are not used.

Examples

EXAMPLE 1:	Module with cyclic constant definition that is not allowed

module MyModule {

:

type record ARecordType { integer a, integer b };

:

// The following two lines include a cycle that is not allowed

const ARecordType c_cConst := { 1 , c_dConst.b}; // c_cConst refers to c_dConst

const ARecordType c_dConst := { 1 , c_cConst.b}; // c_dConst refers to c_cConst

}

EXAMPLE 2:	Modules with cyclic import that is allowed

module MyModuleA {

import from MyModuleB { type MyInteger }

type record of MyInteger MyIntegerList;

}

module MyModuleB {

type integer MyInteger;

import from MyModuleA { type MyIntegerList }

}

# 6	Types and values

## 6.0	General

TTCN-3 supports a number of predefined basic types. These basic types include ones normally associated with a programming language, such as integer, boolean and string types, as well as some TTCN-3 specific ones such as verdicttype. Structured types such as record types, set types and union types can be constructed from these basic types. enumerated types are specific structured types being constructed of enumerated values.

The special data type anytype is defined as the union of all known data types and the address type defined within a TTCN-3 module. In any specific module context, only the known types can be accessed in a value or template of type anytype.

Special types associated with test configurations such as address, port and component may be used to define the architecture of the test system (see clause 21).

The special type default may be used for the default handling (see clause 20.5).

The TTCN-3 types are summarized in table 3.

Table 3: Overview of TTCN-3 types

| Class of type | Keyword | Subtype |
| --- | --- | --- |
| Simple basic types | integer | range, list |
|  | float | range, list |
|  | boolean | List |
|  | verdicttype | List |
| Basic string types | bitstring | list, length |
|  | hexstring | list, length |
|  | octetstring | list, length |
|  | charstring | range, list, length, pattern |
|  | universal charstring | range, list, length, pattern |
| Structured types | record | list (see note) |
|  | record of | list (see note), length |
|  | set | list (see note) |
|  | set of | list (see note), length |
|  | enumerated | list (see note) |
|  | union | list (see note) |
| Special data type | anytype | list |
| Special configuration types | address |  |
|  | port |  |
|  | component |  |
| Special default type | default |  |
| Array notation | [] | list (see note) |
| NOTE: List subtyping of these types is possible when defining a new constrained type from an already existing parent type but not directly at the declaration of the first parent type. |  |  |

NOTE:	Behaviour types for TTCN-3 are defined in the optional package [i.13].

## 6.1	Basic types and values

### 6.1.0	Simple basic types and values

TTCN-3 supports the following basic types:

a)	integer: a type with distinguished values which are the positive and negative whole numbers, including zero:

Values of integer type shall be denoted by one or more digits; the first digit shall not be zero unless the value is 0; the value zero shall be represented by a single zero.

b)	float: a type to describe floating-point numbers and special float values:

In general, floating point numbers can be defined as:<mantissa> × <base><exponent>,

where <mantissa> is a positive or negative integer, <base> a positive integer (in most cases 2, 10 or 16) and <exponent> a positive or negative integer.

In TTCN-3, the floating-point number value notation is restricted to a base with the value of 10. Floating point values can be expressed by using two forms of value notations:

the decimal notation with a dot in a sequence of numbers like, 1.23 (which represents 123×10-2), 2.783 (i.e. 2783 × 10-3) or -123.456789 (which represents -123 456 789 × 10-6); or

by two numbers separated by E where the first number specifies the mantissa and the second specifies the exponent, for example 12.3E4 (which represents 123 × 103) or -12.3E-4 (which represents -123 × 10-5).

NOTE 1:	In contrast to the general definition of float values, the mantissa of in theTTCN-3 value notation, beside integers, allows decimal numbers as well.

The special values of the float type consist of infinity (positive infinity), -infinity (negative infinity) and the value not_a_number. For the ordering of special values see clauses 7.1.1 and 7.1.3.

NOTE 2:	-not_a_number (i.e. minus not a number) is not to be used.

c)	boolean: a type consisting of two distinguished values:

Values of boolean type shall be denoted by true and false.

d)	verdicttype: a type for use with test verdicts consisting of 5 distinguished values. Values of verdicttype shall be denoted by pass, fail, inconc, none and error.

### 6.1.1	Basic string types and values

#### 6.1.1.0	General

TTCN-3 supports the following basic string types:

a)	bitstring: a type whose distinguished values are the ordered sequences of zero, one, or more bits:

Values of type bitstring shall be denoted by an arbitrary number (possibly zero) of the bit digits: 0 1, preceded by a single quote ( ' ) and followed by the pair of characters 'B.

Within the quotes any number of whitespaces or any sequence of the following C0 control characters: LF(10), VT(11), FF(12), CR(13) which constitutes a newline (see Recommendation ITU-T T.50 [4]) (jointly called newline characters, see clause A.1.5.1) may be included. The newline shall be preceded by a backslash ("\"). Such whitespaces, control characters and backslash will be ignored for the value and length calculation.

EXAMPLE 1:	'01101'B
'0110 1001'B
'0110\
 1001'B

b)	hexstring: a type whose distinguished values are the ordered sequences of zero, one, or more hexadecimal digits, each corresponding to an ordered sequence of four bits:

Values of type hexstring shall be denoted by an arbitrary number (possibly zero) of the hexadecimal digits (uppercase and lowercase letters can equally be used as hex digits):

0 1 2 3 4 5 6 7 8 9 a b c d e f A B C D E F

preceded by a single quote ( ' ) and followed by the pair of characters 'H; each hexadecimal digit is used to denote the value of a semi-octet using a hexadecimal representation.

Within the quotes any number of whitespaces or any sequence of the following C0 control characters: LF(10), VT(11), FF(12), CR(13) which constitutes a newline (see Recommendation ITU-T T.50 [4]) (jointly called newline characters, see clause A.1.5.1) may be included. The newline shall be preceded by a backslash ("\"). Such whitespaces, control characters and backslash will be ignored for the value and length calculation.

EXAMPLE 2:	'AB01D'H
'ab01d'H
'Ab01D'H
'Ab 01 D'H
'Ab\
 01\
 D'H

c)	octetstring: a type whose distinguished values are the ordered sequences of zero or a positive even number of hexadecimal digits (every pair of digits corresponding to an ordered sequence of eight bits):

Values of type octetstring shall be denoted by an arbitrary, but even, number (possibly zero) of the hexadecimal digits (uppercase and lowercase letters can equally be used as hex digits):

0 1 2 3 4 5 6 7 8 9 a b c d e f A B C D E F

preceded by a single quote ( ' ) and followed by the pair of characters 'O; each hexadecimal digit is used to denote the value of a semi-octet using a hexadecimal representation.

Within the quotes any number of whitespaces or any sequence of the following C0 control characters: LF(10), VT(11), FF(12), CR(13) which constitutes a newline (see Recommendation ITU-T T.50 [4]) (jointly called newline characters, see clause A.1.5.1) may be included. The newline shall be preceded by a backslash ("\"). Such whitespaces, control characters and backslash will be ignored for the value and length calculation.

EXAMPLE 3:	'FF96'O
'ff96'O
'Ff96'O
'Ff 96'O
'Ff\
 96'O

d)	charstring: are types whose distinguished values are zero, one, or more characters of the version of Recommendation ITU-T T.50 [4] complying with the International Reference Version (IRV) as specified in clause 8.2 of Recommendation ITU-T T.50 [4].

NOTE 1:	The IRV version of Recommendation ITU-T T.50 [4] is equivalent to the IRV version of the International Reference Alphabet (former International Alphabet No.5 - IA5), described in Recommendation ITU-T T.50 [4].

Values of charstring type shall be denoted by an arbitrary number (possibly zero) of non-control characters from the relevant character set, preceded and followed by double quote ("). Graphical characters include the range from SP(32) to TILDE (126). Values of charstring type can also be calculated using the predefined conversion function int2char with the positive integer value of their encoding as argument (see clause C.1).

NOTE 2:	The predefined conversion function is able to return single-character-length values only.

In cases where it is necessary to define strings that include the character double quote (") the character is represented by a pair of double quotes on the same line with no intervening space characters.

EXAMPLE 4:	The charstring "ab"cd" is written in TTCN-3 code as in the following constant declaration. Each of the 3 quote characters that are part of the string is preceded by an extra quote character and the whole character string is delimited by quote characters, e.g.
const charstring c_char:= """ab""cd""";

e)	The character string type preceded by the keyword universal denotes types whose distinguished values are zero, one, or more characters from ISO/IEC 10646 [2].

universal charstring values can also be denoted by an arbitrary number (possibly zero) of characters from the relevant character set, preceded and followed by double quote ("), calculated using a predefined conversion function (see clause C.1.2) with the positive integer value of their encoding as argument, by a "quadruple" or using the USI-like notation.

NOTE 3:	If applying the double quote format all characters from any character set defined in ISO/IEC 10646 [2] are allowed. Users should be aware of the character set capabilities of their editing tool and the TTCN-3 module transfer syntax UTF-8 (see clause 8).

NOTE 4:	The predefined conversion function is able to return single-character-length values only.

In cases where it is necessary to define strings that include the character double quote (") the character is represented by a pair of double quotes on the same line with no intervening space characters.

The "quadruple" is only capable to denote a single character and denotes the character by the decimal values of its group, plane, row and cell according to ISO/IEC 10646 [2], preceded by the keyword char included into a pair of brackets and separated by commas (e.g. char ( 0, 0, 1, 113) denotes the Latin small letter u with double acute: "ű"). In cases where it is necessary to denote the character double quote (") in a string assigned according to the first method (within double quotes), the character is represented by a pair of double quotes on the same line with no intervening space characters. The two methods may be mixed within a single notation for a string value by using the concatenation operator.

EXAMPLE 5:	The expression: "the Braille character" & char (0, 0, 40, 48) & "looks like this" represents the literal string: the Braille character  looks like this.

The UCS sequence identifier-like (USI-like) notation (see also clause 6.6 of ISO/IEC 10646 [2]) can be used to denote 1..N characters, using their short identifiers of code point (similar to UIDs described in clause 6.5 of ISO/IEC 10646 [2]). The USI-like notation is composed of the keyword char followed by parentheses. The parentheses enclose a comma-separated list of short identifiers . Each short identifier represents a single character and it shall be composed of a letter U or u followed by an optional "+" PLUS SIGN character, followed by 1..8 hexadecimal digits. The hexadecimal digits represent the numeric code point of the character. (e.g. char(U0171) denotes the Latin small letter u with double acute: "ű"). In the USI-like notation, the leading zeroes can be omitted, (i.e. char(U171)is equal to char(U0171)).

EXAMPLE 6:	The expression: char (U4E2D, U56FD) represents the literal string: 中国.

NOTE 5:	Control characters can be denoted by using the predefined conversion function, the quadruple form or the USI-like notation.

By default, universal charstring shall conform to the UTF-32 encoding specified in clause 9.3 of ISO/IEC 10646 [2].

NOTE 6:	UTF-32 is an encoding format, which represents any UCS character on a fixed, 32 bits-length field.

This default encoding can be overridden using the defined variant attributes (see clause 27.5). The useful character string types utf8string, bmpstring, utf16string and iso8859string using these attributes are defined in annex E.

The general term string or string type in TTCN-3 refers to bitstring, hexstring, octetstring, charstring and universal charstring.

The general term binary string or binary string type in TTCN-3 refers to bitstring, hexstring, octetstring.

The general term character string or character string type in TTCN-3 refers to charstring and universal charstring.

#### 6.1.1.1	Accessing individual string elements

Individual elements in a string type may be accessed using an array-like syntax.

Units of length of different string type elements are indicated in table 4.

For accessing individual string elements the following rules apply:

Only single elements of the string may be accessed. Trying to assign strings with length 0 or more than 1 to a string element using the array-like syntax shall cause an error.

Indexing shall begin with the value zero (0).

The index shall be between zero and the length of the string minus one for retrieving an element from a string. Trying to retrieve an element from a string with an index outside this range shall cause an error.

For assigning an element to the end of a string, the length of the string should be used as index. Trying to assign an element to the end of a string with an index larger than the length of the string shall cause an error.

For initializing an uninitialized string with a single element, the index value zero (0) can be used as index. Trying to assign a single element to an uninitialized string with an index which is not zero (0) shall cause an error.

EXAMPLE 1:	Accessing an existing element

// Given

v_myBitString := '11110111'B;

// Then doing

v_myBitString[4] := '1'B;

// Results in the bitstring '11111111'B

EXAMPLE 2:	Specific cases

var bitstring v_myBitStringA, v_myBitStringB, v_myBitStringC, v_myBitStringD;

v_myBitStringA := '010'B;

v_myBitStringA[1] := '11'B; //causes an error as only individual elements can be accessed

v_myBitStringB := '1'B;

v_myBitStringB[4] := '1'B; //causes an error index is larger than the length of v_myBitStringB

v_myBitStringC := ''B;

v_myBitStringC[0] := '1'B; // value of v_myBitStringC is '1'B

v_myBitStringC[1] := '0'B; // value of v_myBitStringC is '10'B

// v_myBitStringD is not initialized
	v_myBitStringD[0] := '0'B; // value of v_myBitStringD is '0'B

v_myBitStringD[1] := '1'B; // value of v_myBitStringD is '01'B

var charstring v_myCharString;

v_myCharString[0] := "a" //initializing v_myCharString with a single character

v_myCharString[1] := "" //causes an error as the length of the to-be-assigned string is 0

v_myCharString[1] := "bc" //causes an error as the length of the to-be-assigned string is

//more than 1

### 6.1.2	Subtyping of basic types

#### 6.1.2.0	General

User-defined types shall be denoted by the keyword type. With user-defined types it is possible to create subtypes (such as lists, ranges and length restrictions) on basic types, structured types and anytype according to table 3.

#### 6.1.2.1	Lists of templates

TTCN-3 permits the specification of a list of distinguished templates as listed in table 3. The templates in the list shall be instances of the type being constrained and the set of values matching at least one of these templates shall be a subset of the values defined by the type being constrained. The subtype defined by this list restricts the allowed values of the subtype to those values matching at least one of the templates in the list. The templates in the list shall only (directly or indirectly) reference other templates or constant expressions. Constant expressions used (directly or indirectly) in the template expressions shall meet with the restrictions in clause 10 for constant expressions used in type definitions.

EXAMPLE:

type bitstring MyListOfBitStrings ('01'B, '10'B, '11'B);

type float PI (3.1415926);

type charstring MyStringList ("abcd", "rgy", "xyz");

type universal charstring SpecialLetters 
 							 (char(0, 0, 1, 111), char(0, 0, 1, 112), char(0, 0, 1, 113));

#### 6.1.2.2	Lists of types

TTCN-3 permits the specification of a list of subtypes as listed in table 3 for value lists. The types in the list shall be subtypes of the root type. The subtype defined by this list restricts the allowed values of the subtype to the union of the values of the referenced subtypes.

EXAMPLE:

type bitstring BitStrings1 ('0'B, '1'B );

type bitstring BitStrings2 ('00'B, '01'B, '10'B, '10'B);

type bitstring BitStrings_1_2 (Bitstrings1, Bitstrings2);

#### 6.1.2.3	Ranges

TTCN-3 permits the specification of range constraints for the types integer, charstring, universal charstring and float (or derivations of these types). For integer and float, the subtype defined by the range restricts the allowed values of the subtype to the values in the range including or excluding the lower boundary and/or the upper boundary. The upper boundary shall be greater than or equal to the lower boundary.

In order to specify an infinite integer range, the keyword -infinity or infinity can be used instead of a value indicating that there is no lower or upper boundary; -infinity shall not be used as the upper bound and infinity shall not be used as the lower bound for integer ranges.

Also for float, -infinity or infinity can be used as the bounds in range restrictions. Using the special value -infinity as the lower bound shall indicate that the allowed numerical values are not restricted downward and the special value -infinity is also included. If both the lower and upper bounds denote -infinity, no numerical values are included, only the special value -infinity. Using the special value infinity as the upper bound shall indicate that the allowed numerical values are not restricted upward and the special value infinity is also included. If both the lower and upper bounds denote infinity, no numerical values are included, only the special value infinity. If exclusive bounds (!infinity or !-infinity) is used instead, only the respective numerical float values are included in the range. In case of float, the special value not_a_number is not allowed in a range constraint.

In the case of charstring and universal charstring types, the range restricts the allowed values for each separate character in the strings. The boundaries shall evaluate to valid character positions according to the coded character set table(s) of the type (e.g. the given position shall not be empty). Empty positions between the lower and the upper boundaries are not considered to be valid values of the specified range.

Constants used in the constant expressions defining the values shall meet with the restrictions in clause 10.

EXAMPLE 1:

type integer MyIntegerRange (0 .. 255); 			// range from 0..255

// (with inclusive boundaries)

type integer MyIntegerRange (0 .. !256); 			// the same range as above (with left

// inclusive and right exclusive boundary)

type integer MyIntegerRange (!-1 .. 255); 			// the same range as above(with left

// exclusive and right inclusive boundary)

type integer MyIntegerRange (!-1 .. !256); 		// the same range as above

// (with exclusive boundaries)

type integer MyIntegerRange (-infinity .. -1); 	// all negative integer numbers

type float PiRange (3.14 .. 3142E-3);

type float LessThanPi (-infinity .. 3142E-3);

type float Numbers (-infinity .. infinity); 	//includes all float values but not_a_number

type float Wrong (-infinity .. not_a_number);	// causes an error as not_a_number is not

// allowed in range subtyping

EXAMPLE 2:

type charstring MyCharString ("a" .. "z");

// Defines a string type of any length with each character within the specified range

type universal charstring MyUCharString1 ("a" .. !"z");

// Defines a string type of any length with each character within the range from a to y

// (character codes from 97 to 121), like "abxy";

// strings containing any other character (including control characters), like

// "abc2" are disallowed.

type universal charstring MyUCharString2 (char(0, 0, 1, 111) .. char(0, 0, 1, 113));

// Defines a string type of any length with each character within the range specified using

// the quadruple notation

#### 6.1.2.4	String length restrictions

TTCN-3 permits the specification of length restrictions on string types. The length boundaries are based on different units depending on the string type with which they are used. In all cases, these boundaries shall be inclusive boundaries only and evaluate to non-negative integer values (or derived integer values).

EXAMPLE:

type bitstring MyByte length(8); 					// Exactly length 8

type bitstring MyByte length(8 .. 8); 				// Exactly length 8

type bitstring MyNibbleToByte length(4 .. 8); 	// Minimum length 4, maximum length 8

Table 4 specifies the units of length for different string types.

Table 4: Units of length used in field length specifications

| Type | Units of Length |
| --- | --- |
| bitstring | bits |
| hexstring | hexadecimal digits |
| octetstring | octets |
| character strings | characters |

For the upper bound the keyword infinity should also be used to indicate that there is no upper limit for the length. The upper boundary shall be greater than or equal to the lower boundary.

#### 6.1.2.5	Pattern subtyping of character string types

TTCN-3 allows using character patterns specified in clause B.1.5 to constrain permitted values of charstring and universal charstring types. The type constraint shall use the pattern keyword followed by a character pattern. All values denoted by the pattern shall be a subset of the type being sub typed. Constants used in the constant expressions defining the values shall meet with the restrictions in clause 10.

NOTE:	Pattern subtyping can be seen as a special form of list constraint, where members of the list are not defined by listing specific character strings but via a mechanism generating elements of the list.

EXAMPLE:

type charstring MyString (pattern "abc*xyz");

// all permitted values of MyString have prefix abc and postfix xyz

type charstring MyStringCaseAgnostic (pattern @nocase "abc*xyz");

// all permitted values of MyStringCaseAgnostic have a

// prefix abc or Abc or aBc or abC or ABc or aBC or AbC or ABC, and a

// postfix xyz or Xyz or xYz or xyZ or XYz or xYZ or XyZ or XYZ;

type universal charstring MyUString (pattern "*\r\n")

// all permitted values of MyUString are terminated by CR/LF

type charstring MyString2 (pattern "abc?\q{0,0,1,113}");

// causes an error because the character denoted by the quadruple {0,0,1,113} is not a

// legal character of the TTCN-3 charstring type

type MyString MyString3 (pattern "d*xyz");

// causes an error because the type MyString does not contain a value starting with the

// character d

#### 6.1.2.6	Mixing subtyping mechanisms

##### 6.1.2.6.1	Mixing patterns, lists and ranges

Within integer and float (or derivations of these types) subtype definitions it is allowed to mix lists and ranges. It is possible to mix both template list and type list subtyping with each other and with range subtyping. Overlapping of different constraints is not an error.

EXAMPLE 1:

type integer MyIntegerRange (1, 2, 3, 10 .. !20, 99, 100);

type float LessThanPiAndNaN (-infinity .. 3142E-3, not_a_number);

Within charstring and universal charstring subtype definitions it is not allowed to mix pattern, template list, type list, or range constraints.

EXAMPLE 2:

type charstring MyCharStr0 ("gr", "xyz");

// contains character strings gr and xyz;

type charstring MyCharStr1 ("a".."z");

// contains character strings of arbitrary length containing characters a to z.

type charstring MyCharStr2 (pattern "[a-z]#(3,9)");

// contains character strings of length from 3 to 9 characters containing characters a to z

##### 6.1.2.6.2	Using length restriction with other constraints

Within bitstring, hexstring, octetstring subtype definitions lists and length restriction may be mixed in the same subtype definition.

Within charstring and universal charstring subtype definitions it is allowed to add a length restriction to constraints containing list, range or pattern subtyping in the same subtype definition.

When mixed with other constraints the length restriction shall be the last element of the subtype definition. The length restriction takes effect jointly with other subtyping mechanisms (i.e. the value set of the type consists of the common subset of the value sets identified by the list, range or pattern subtyping and the length restriction).

EXAMPLE:

type charstring MyCharStr5 ("gr", "xyz") length (1..9);

// contains the character strings gr and xyz;

type charstring MyCharStr6 ("a".."z") length (3..9);

// contains character strings of length from 3 to 9 characters and containing characters

// a to z

type charstring MyCharStr7 (pattern "[a-z]#(3,9)") length (1..9);

// contains character strings of length from 3 to 9 characters containing characters

// a to z

type charstring MyCharStr8 (pattern @nocase "[a-z]#(3,9)") length (1..8);

// contains character strings of length from 3 to 8 characters containing characters

// a to z and A to Z

type charstring MyCharStr9 (pattern "[a-z]#(1,8)") length (1..9);

// contains any character strings of length from 1 to 8 characters containing characters

// a to z

type charstring MyCharStr10 ("gr", "xyz") length (4);

// causes an error as it contains no value

## 6.2	Structured types and values

### 6.2.0	General

The type keyword is also used to specify structured types such as record types, record of types, set types, set of types, enumerated types and union types.

Values of these types may be given using an explicit assignment notation or a short-hand value list notation or in a mixed list and assignment notation.

EXAMPLE 1:

const MyRecordType c_myRecordValue:= 									//assignment notation

{

field1 := '11001'B,

field2 := true,

field3 := "A string"

}

// Or

const MyRecordType c_myRecordValue:= {'11001'B, true, "A string"}		//value list notation

// Or

Const MyRecordType c_myRecordValue := {'11001'B, field3 := "A string", field2 := true}

// mixed notation

The assignment and the mixed notation can be used for record, record of, set and set of value notations and for arrays. In these notations each assigned field  or index shall not appear more than once and assignments to fields or indexes given in list notation are not allowed. The assignment notation can also be used for union values. The value list notation can be used for record, record of, set and set of value notations and for arrays. The index notation as part of an assignment or mixed notation can be used as the left-hand side of element assignments for record of and set of value notations and for arrays. In this notation each index shall not appear more than once and shall conform to the range of indices allowed by the type definition. See more details in the subsequent clauses.

EXAMPLE 2:

var MyRecordType v_myVariable:= 								//assignment notation

{

field1 := '11001'B,

// field2 implicitly unspecified

field3 := "A string"

}

// or
	var MyRecordType v_myVariable:= 								//assignment notation

{

field1 := '11001'B,
		field2 := -, // field2 explicitly unspecified

field3 := "A string"

}

// or

var MyRecordType v_myVariable:= {'11001'B, -, "A string"}		//value list notation

It is allowed to mix the two value notations in the same (immediate) context only in such a way that elements in list notation do not follow elements in assignment notation.

EXAMPLE 3:

// This is disallowed

const MyRecordType c_myRecordValue:= {c_myIntegerValue, field2 := true, "A string"}

Where applicable TTCN-3 type definitions may be recursive. The user, however, shall ensure that all type recursion is resolvable and that no infinite recursion occurs.

In case of record and set types, to avoid infinite recursion, fields referencing to its own type, shall be optional.

EXAMPLE 4:

// Valid recursive record type definition

type record MyRecord1
    { 
        FieldType1  field1,
        MyRecord1   field2 optional,

FieldType3  field3 
    }

// Invalid recursive record type definition causing an error

type record MyRecord2
    { 
        FieldType1  field1,
        MyRecord2   field2,

FieldType3  field3 
    }

In case of union types, to avoid infinite recursion, at least one of the alternatives shall not reference its own type.

EXAMPLE 5:

// Valid recursive union type definition

type union MyUnion1

{

MyUnion1   choice1,

charstring choice2

}

// Invalid recursive union type definition causing an error

type union MyUnion2

{

MyUnion2 choice1,

MyUnion2 choice2

}

### 6.2.1	Record type and values

#### 6.2.1.0	General

TTCN-3 supports ordered structured types known as record. The fields of a record type may be of any TTCN-3 type. The values of a record shall be compatible with the types of the record fields. The field identifiers are local to the record and shall be unique within the record (but do not have to be globally unique).

EXAMPLE 1:

type record MyRecordType

{

integer 			field1,

MyOtherRecordType 	field2 optional,

charstring 			field3

}

type record MyOtherRecordType

{

bitstring 	field1,

boolean 	field2

}

Records may be defined with no fields, i.e. as empty records.

EXAMPLE 2:

type record MyEmptyRecord {}

A record value is assigned on an individual field basis. The order of field values in the value list notation shall be the same as the order of fields in the related type definition.

EXAMPLE 3:

var integer v_myIntegerValue := 1;

const MyOtherRecordType c_myOtherRecordValue:=

{

field1 := '11001'B,

field2 := true

}

var MyRecordType v_myRecordValue :=

{

field1 := v_myIntegerValue,

field2 := c_myOtherRecordValue,

field3 := "A string"

}

The same value specified with a value list.

EXAMPLE 4:

v_myRecordValue:= {v_myIntegerValue, {'11001'B, true}, "A string"};

When the assignment notation is used for record-s, fields wished to be changed shall be identified explicitly and a value, the not used symbol "-" or the omit keyword can be associated with them. The omit keyword shall only be used for optional fields. Its result is that the given field is not present in the given value. Mandatory fields, not explicitly referred to in the notation or explicitly unspecified using the not used symbol "-", shall remain unchanged. In particular, when specifying partial values (i.e. setting the value of only a subset of the fields) using the assignment notation, at initialization, only the fields to be assigned values shall be specified. Fields not mentioned are implicitly left uninitialized. When re-assigning a previously initialized value, using the not used symbol or just skipping a field in an assignment notation, will cause that field to remain unchanged. Even when specifying partial values each field shall not appear more than once.

NOTE:	Please note the difference between omitted and uninitialized fields. Omitted optional fields are not present in the record or set value intentionally, i.e. the field is initialized and it does not prevent the whole record or set from being completely initialized.

EXAMPLE 5:

type record MyRecordType

{

bitstring 		field1,

boolean			field2 optional,

charstring 		field3

}

var MyRecordType v_myVariable :=

{

field1 := '111'B,

field2 := false,

field3 := -

}

v_myVariable := { '10111'B, -, - };

// after this, v_myVariable contains:

//	{ '10111'B, false /* unchanged */, <undefined> /* unchanged */ }

v_myVariable :=

{

field2 := true

}

// after this, v_myVariable contains:

//	{ '10111'B /* unchanged */, true, <undefined> /* unchanged */ }

v_myVariable :=

{

field1 := -,

field2 := false,

field3 := -

}

// after this, v_myVariable contains:

//	{ '10111'B /* unchanged */, false, <undefined> /* unchanged */}

When the assignment notation is used in a scope, where the optional attribute is implicitly or explicitly set to "explicit omit", optional and mandatory fields, not directly referred to in the notation shall remain unchanged. When optional fields of variables are not assigned explicitly, they are uninitialized (i.e. the optional attribute shall not have any effect on variables as described in clause 27.7 restriction a)).

When the assignment notation is used in a scope, where the optional attribute is set to "implicit omit", optional fields, not directly referred to in the notation, shall implicitly be set to omit, while mandatory fields shall remain unchanged (see also clause 27.7).

EXAMPLE 6:

type record MyRecordType

{

bitstring 		field1,

boolean			field2 optional,

charstring 		field3

}

const MyRecordType c_myConst1 :=

{

field1 := '111'B,

field3 := "A string"

} //	{ '10111'B, <undefined>, "A string"}

const MyRecordType c_myConst2 :=

{

field1 := '111'B,

field3 := "A string"

} with { optional "implicit omit" }

//	{ '10111'B, omit /* because of the optional attribute */, "A string"}

When using the value list notation, all fields listed in the notation shall be specified either with a value, the not used symbol "-" or the omit keyword. The omit keyword shall only be used for optional fields. Its result is that the given field is not present in the given value. The first component of the list (a value, a "-" or omit) is associated with the first field, the second list component is associated with the second field, etc. No empty assignment is allowed (i.e. two commas, the second immediately following the first or only with white space between them). Fields to be left unchanged, but followed by fields to which a value or template is assigned explicitly, shall be skipped by using the not used symbol "-".

When using value list notation in a scope where the optional attribute is implicitly or explicitly set to "explicit omit", all remaining fields at the end of the type definition, missing from the value list notation, are left unchanged.

When using value list notation in a scope where the optional attribute is set to "implicit omit", optional fields wished to be omitted by the implicit mechanism, but followed by fields to which a value or template is assigned explicitly, shall be skipped by using the not used symbol "-". When all remaining fields at the end of the type definition are optional and they are wished to be omitted by the implicit mechanism, either the not used symbol "-" can be used for some or all of them or they can simply be left out from the notation.

EXAMPLE 7:

type record R {
		integer f1,
		integer f2 optional,
		integer f3,
		integer f4 optional,
		integer f5 optional
	}

	const R c_x := { 1, -, 2 } with { optional "implicit omit" }
	// after the assignment v_x contains { 1, omit, 2, omit, omit }
	constR c_x2 := { 1, 2, 3, - } with { optional "implicit omit" }
	//  after the assignment v_x2 contains { 1, 2, 3, omit, omit }

When using direct assignment notation in a scope where the optional attribute is set to "implicit omit", the uninitialized optional fields in the referenced value, shall implicitly be set to omit after the assignment in the new value, while mandatory fields shall remain unchanged (see also clause 27.7).

EXAMPLE 8:

const R c_x3 := { 1, -, 2 } 
	// after the assignment c_x3 contains { 1, <undefined>, 2, <undefined>, <undefined>}
	const R c_x4 := c_x3 with { optional "implicit omit" }
	//  after the assignment c_x4 contains { 1, omit, 2, omit, omit }

#### 6.2.1.1	Referencing fields of a record type

Elements of a record shall be referenced by the dot notation TypeIdOrExpression.ElementId, where TypeIdOrExpression resolves to the name of a structured type or an expression of a structured type such as variable, formal parameter, module parameter, constant, template, or function invocation. ElementId shall resolve to the name of a field in the structured type. Fields of record type definitions shall not reference themselves.

EXAMPLE 1:

v_myVar1 := v_myRecord1.myElement1;

// If a record is nested within another type then the reference may look like this

v_myVar2 := v_myRecord1.myElement1.myElement2;

EXAMPLE 2:

type record MyType

{

integer	field1,

MyType.field2	field2	optional, // this circular reference is NOT ALLOWED

boolean	field3

}

If a field in a record type or a subtype of a record type is referenced by the dot notation, the resulting type is the set of values allowed for that field imposed by the constraints of the field declaration itself (i.e. any constraints applied to the record type itself are ignored).

EXAMPLE 3:

type record MyType2

{

integer	field1 (1 .. 10),

charstring	field2	optional

}

type MyType2 MyType3 ({1, omit}, {2, "foo"}, {3, "bar"}) ;

type MyType3.field1 MyType4; 	// MyType4 is the integer type constrained to
									// the values 1..10

type MyType3.field2 MyType5; 	// MyType5 is the charstring type

type MyType2.field1 MyType6; 	// MyType6 is the integer type constrained to
									// the values 1..10

type MyType2.field2 MyType7; 	// MyType7 is  the charstring type

Referencing a subfield of an uninitialized or omitted record field or value on the right hand side of an assignment shall cause an error.

EXAMPLE 4:

type record MyType4

{

integer field1 optional,

record

{

integer subfield1,

integer subfield2

} field2 optional

}

...

var MyType4 v_rec := { field1 := 1, field2 := omit }

var integer v_int := v_rec.field2.subfield1;

// causes an error as v_rec.field2 is omitted

When referencing a field of an uninitialized record value or field or omitted field (including omitting a field at a higher level of the embedding hierarchy) on the left hand side of an assignment, the reference shall recursively be expanded up to and including the depth of the referenced subfield as follows:

- When expanding a value or value field of record type, the subfield referenced in the dot notation shall be set to present and all unreferenced mandatory subfields shall be left uninitialized; when the assignment is used in a scope where the optional attribute is equal to "explicit omit", all unreferenced optional subfields shall be left undefined. When the assignment is used in a scope where the optional attribute is equal to "implicit omit", all unreferenced optional subfields shall be set to omit.

- Expansion of record of/set of/array, union and set values and intermediate fields shall follow the rules of item a) in clauses 6.2.7 and 6.2.5.1 and clause 6.2.2.1 correspondingly.

At the end of the expansion, the value at the right hand side of the assignment shall be assigned to the referenced subfield.

EXAMPLE 5:

var MyType4 v_rec;

v_rec.field2.subfield1 := 5;

// after the assignment v_rec is { field1 := <undefined>, field2 := { subfield1 := 5,

// subfield2 := <undefined> } }

#### 6.2.1.2	Optional elements in a record

Optional elements in a record shall be specified using the optional keyword.

EXAMPLE 1:

type record MyMessageType

{

FieldType1	field1,

FieldType2	field2	optional,

:

FieldTypeN 	fieldN

}

Optional fields shall be omitted using the omit symbol.

EXAMPLE 2:

v_myRecordValue:= {v_myIntegerValue, omit , "A string"};

// Note that this is not the same as writing,

// v_myRecordValue:= {v_myIntegerValue, -, "A string"};

// which would mean the value of field2 is unchanged

#### 6.2.1.3	Nested type definitions for field types

TTCN-3 supports the definition of types for record fields nested within the record definition. Both the definition of new structured types (record, set, enumerated, set of, record of, and union) and the specification of subtype constraints are possible.

EXAMPLE:

// record type with nested structured type definitions

type record MyNestedRecordType
	{
		record 
		{
			integer nestedField1, 
			float nestedField2
		} outerField1,
		enumerated {
			nestedEnum1,
			nestedEnum2
		} outerField2,
		record of boolean outerField3
	}

// record type with nested subtype definitions

type record MyRecordTypeWithSubtypedFields

{

integer    field1 (1 .. 100),

charstring field2 length ( 2 .. 255 )

}

### 6.2.2	Set type and values

#### 6.2.2.0	General

TTCN-3 supports unordered structured types known as set. Set types and values are similar to records except that the ordering of the set fields is not significant.

EXAMPLE:

type set MySetType

{

integer 	field1,

charstring 	field2

}

The field identifiers are local to the set and shall be unique within the set (but do not have to be globally unique).

NOTE:	When the value list notation is used for values of set types, the values are assigned to the fields in the sequential order of the fields in the type definition.

#### 6.2.2.1	Referencing fields of a set type

Elements of a set shall be referenced by the dot notation (see clause 6.2.1.1). Elements of set type definitions shall not reference themselves. For referencing field types of set types, the same rules apply as in clause 6.2.1.1 for fields of record types.

EXAMPLE:

v_myVar3 := v_mySet1.myElement1;

// If a set is nested in another type then the reference may look like this

v_myVar4 := v_myRecord1.myElement1.myElement2;

// Note, that the set type, of which the field with the identifier 'myElement2' is referenced,

// is embedded in a record type

#### 6.2.2.2	Optional elements in a set

Optional elements in a set shall be specified using the optional keyword.

#### 6.2.2.3	Nested type definition for field types

TTCN-3 supports the definition of types for set fields nested within the set definition, similar to the mechanism for record types described in clause 6.2.1.3.

### 6.2.3	Records and sets of single types

#### 6.2.3.0	General

TTCN-3 supports the specification of records and sets whose elements are all of the same type. These are denoted using the keyword of. These records and sets do not have element identifiers and can be considered similar to an ordered array and an unordered collection respectively.

NOTE 1:	For the subtyping of record of and set of types see in clause 6.2.13.

EXAMPLE 1:

type set of boolean MySetOfType; // is an unlimited set of boolean values

When the assignment notation is used for record of-s and set of-s, elements wished to be changed are identified explicitly and either a value or the not used symbol "-" can be assigned to them. Other elements, not referred to in the notation, shall remain unchanged. In particular, when specifying partial values (i.e. setting the value of only a subset of the fields) using the assignment notation, for example, at initialization, only the elements to be assigned values shall be specified: elements not mentioned are implicitly left uninitialized. It is also possible to leave fields explicitly unspecified using the not used symbol "-". When re-assigning a previously initialized value, using the not used symbol or just skipping a field or element in an assignment notation, will cause that field or element to remain unchanged.

EXAMPLE 2:

var MyRecordOfType v_myVariable := {

[0] := '111'B,

[1] := '101'B,

[2] := -

}

v_myVariable := { '10111'B, -, - };

// after this, v_myVariable contains:

//	{ '10111'B, '101'B /* unchanged */, <undefined> /* unchanged */ }

v_myVariable :=

{

[1] := '010'B,

}

// after this, v_myVariable contains:

//	{ '10111'B/* unchanged */, '010'B, <undefined>/* unchanged */ }

v_myVariable :=

{

[0] := -,

[1] := '001'B,

[2] := -

}

// after this, v_myVariable contains:

//	{ '10111'B/* unchanged */, '001'B, <undefined> /* unchanged */}

When using the value list notation, all elements in the structure shall be specified either with a value or the not used symbol "-". The first member of the list is assigned to the first element, the second list member is assigned to the second element, etc. No empty assignment is allowed (e.g. two commas, the second immediately following the first or only with white space between them). Elements to be left out of the assignment shall be explicitly skipped in the list by use of the not-used-symbol "-". Already initialized elements left without a corresponding list member in a value list notation (i.e. at the end of a list) are becoming uninitialized. In this way, a value with initialized elements can be made empty by using the empty value list notation ("{}").

Index notation can be used on both the right-hand side and left-hand side of assignments. The index notation, when used on the right-hand side, refers to the value of the identified element of a record of or a set of. When it is used at the left-hand side, only the value of the identified single element is changed, values assigned to other elements already remain unchanged. The index of the first element shall be zero and the index value shall not exceed the limitation placed by length subtyping.

If the value of the element indicated by the index at the right-hand of an assignment is undefined (uninitialized), this shall cause a semantic or runtime error. Referencing an identified element of an uninitialized or omitted record of or set of field or value on the right-hand side of an assignment shall cause an error.

If an indexing operator at the left-hand side of an assignment refers to a non-existent element, the value at the right-hand side is assigned to the element and all elements with an index smaller than the actual index and without assigned value are created with an uninitialized value.

For nested record of or set of types, an array or record of integer restricted to a single size can be used as a short-hand notation for a nested index notation.

When referencing an element of an uninitialized record of or set of value or field or omitted field (including omitting a field at a higher level of the embedding hierarchy) on the left-hand side of an assignment, the reference shall recursively be expanded up to and including the depth of the referenced element as follows:

- When expanding a value or value field of record of or set of type, the element referenced by the index notation shall be set to present and all elements with a smaller index shall be created with an uninitialized value.

Expansion of record, union and set values and intermediate fields shall follow the rules of item a) in clauses 6.2.1.1 and 6.2.5.1 and clause 6.2.2.1 correspondingly.

At the end of the expansion, the value at the right-hand side of the assignment shall be assigned to the referenced element.

Uninitialized elements are permitted only in transient states (while the value remains invisible). Sending a record of or set of value with uninitialized elements shall cause an error.

NOTE 2:	When using on the right-hand side of an assignment for record of-s or set of-s, the assignment notation and the indexed notation have similar effect, with the exception that the assignment notation is able to address multiple elements in one notation, while the index notation is able to address a single element only.

EXAMPLE 3:

// Given

type record of integer MyRecordOf;

type record of MyRecordOf RoRoI;

var integer v_myVar;

// Using the value list notation

var MyRecordOf v_myRecordOfVar := { 0, 1, 2, 3, 4 };

// The same record of, defined with the assignment notation
	var MyRecordOf v_myRecordOfVarAssignment := {
		[0] := 0,
		[1] := 1,
		[2] := 2,

[3] := 3,
		[4] := 4

};

var RoRoI v_recof;

// Using an index notation

v_myVar := v_myRecordOfVar[0]; // the first element of the "record of" value (integer 0)

// is assigned to v_myVar

// Index notations are permitted on the left-hand side of assignments as well:

v_myRecordOfVar[1] := v_myVar; // v_myVar is assigned to the second element

// value of v_myRecordOfVar is { 0, 0, 2, 3, 4 }

// The assignment

v_myRecordOfVar := { 0, 1, -, 2 };

// will change the value of v_myRecordOfVar to{ 0, 1, 2 <unchanged>, 2};
	// Note, that the 3rd element would be undefined if had no previous assigned value.

// The assignment

v_myRecordOfVar[6] := 6;

// will change the value of v_myRecordOfVar to

//   { 0, 1, 2 , 2, <uninitialized>, <uninitialized>, 6 };
	// Note the 5th and 6th elements (with indexes 4 and 5) had no assigned value before this

// last assignment and are therefore undefined.

v_myRecordOfVar[4] := 4; v_myRecordOfVar[5] := 5;

// will complete v_myRecordOfVar to the fully defined value { 0, 1, 2 , 2, 4 , 5 , 6 };

// Expansion of uninitialized record of value:

v_recof[1][2] := 0;

// after the assignment v_recof is { <undefined>, { <undefined>, <undefined>, 0 } }

// Pls. Note the difference between the two index assignment notations in

// the following example:

var MyRecordOf v_ix := { 0,1,2 }

v_ix := { [3] := 2*v_ix[2]+1 }

// the value of v_ix is: { 0, 1, 2, 5 }

// The same result can be achieved by using an index notation on the left hand side of

// the assignment:

var MyRecordOf v_ix := { 0,1,2 }

v_ix[3] := 2*v_ix[2]+1

// the value of v_ix is: { 0, 1, 2, 5 }

NOTE 3:	The index notation makes it possible e.g. to copy record of values element by element in a for loop. For example, the function below reverses the elements of a record of value:

function reverse(in MyRecordOf p_src) return MyRecordOf
	{
	var MyRecordOf v_dest;
	var integer v_i, v_srcLength := lengthof (p_src);
	for(v_i := 0; v_i < v_srcLength; v_i := v_i + 1) {
		v_dest[v_srcLength - 1 - v_i] := p_src[v_i];
	}
	return v_dest;
	}

Embedded record of and set of types will result in a data structure similar to multidimensional arrays (see clause 15.9).

EXAMPLE 4:

// Given
	type record of integer MyBasicRecordOfType;
	type record of MyBasicRecordOfType	My2DRecordOfType;

// Then, the variable myRecordOfArray will have similar attributes to a two-dimensional array:
	var My2DRecordOfType v_myRecordOfArray;
	// and reference to a particular element would look like this
	// (value of the second element of the third 'MyBasicRecordOfType' construct) 
	v_myRecordOfArray [2][1] := 3;

//with the short-hand notation this could also have been written as

var integer v_i[2] := { 2, 1 };
	v_myRecordOfArray [v_i] := 3;

// is the same as assigning element v_myRecordOfArray[v_i[0]][v_i[1]]

#### 6.2.3.1	Nested type definitions

TTCN-3 supports the definition of the aggregated type nested with the record of or set of definition. Both the definition of new structured types (record, set, enumerated, set of and record of) and the specification of subtype constraints are possible.

EXAMPLE:

type record of enumerated { red, green, blue } ColorList;

type record length (10) of record length (10) of integer Matrix;

type set of record { charstring id, charstring val } GenericParameters;

#### 6.2.3.2	Referencing elements of record of and set of types

It is also allowed to reference the inner type of record of and set of types by using the index notation but with a dash. The notation TypeId[-], where TypeId resolves to the name of a record of or set of type, references the inner type of TypeId. If the type definition restricts the element type of the record of or set of type, referencing the inner type of that type yields a type which contains all values from the constrained type.

EXAMPLE:

//Provided the definitions below

type record of integer MyRecordOfInt;

type record of record {

integer f1,

set { integer s1, boolean s2 } f2

} MyRecordOfRecord;

type record of record of integer MyRecordOfRecordOfInt;

type record of record {

integer f1,

record of boolean f2

} MyRecordOfRecord2;

// Referencing the inner integer type

type MyRecordOfInt[-] MyInteger;

const MyRecordOfInt[-] c_MyInteger:= 5;

// Referencing the nested record type

type MyRecordOfRecord[-] MyInnerRecord;

const MyRecordOfRecord[-] c_MyRecord := { f1 = 5; f2 := { s1 := 0; s2 := true }}

// Referencing the set type nested in the inner record

type MyRecordOfRecord[-].f2 MyNestedSet;

const MyRecordOfRecord[-].f2 c_MySet := { s1 := 0; s2 := true }

// Referencing the innermost boolean

type MyRecordOfRecord[-].f2.s2 MyBoolean;

const MyRecordOfRecord[-].f2.s2 c_MyBool := false;

// Referencing the inner record of

type MyRecordOfRecordOfInt[-] MyInnerRecordOfInt;

const MyRecordOfRecordOfInt[-] c_MyInnerRecordOfInt := { 0, 1, 2, 3 };

// Referencing the integer type within the inner record of

type MyRecordOfRecordOfInt[-][-] MyInteger2;

const MyRecordOfRecordOfInt[-][-] c_MyInteger2 := 1;

// Referencing the boolean type within the nested record

type MyRecordOfRecord2[-].f2[-] MyInnermostBoolean;

const MyRecordOfRecord2[-].f2[-] c_MyInnermostBoolean := true ;

type record length (5) of record of integer ConstrainedRecordOfInt (1 .. 10);

type ConstrainedRecordOfInt[-] ConstrainedInt;

// defines the type record of integer, where the integer values are restricted

// to the range 1 .. 10 but the record of has no length restriction

### 6.2.4	Enumerated type and values

TTCN-3 supports enumerated types. Enumerated types are used to model types that take only a distinct named set of values. Such distinct values are called enumerated values. Each enumerated value shall have an identifier and referencing the values shall only use these identifiers. References to enumerated values can occur in two forms: unqualified and qualified. The unqualified form uses only the identifier of the enumerated value. The qualified form consists of the enumerated type reference, followed by a dot and the enumerated value identifier. The identifiers of enumerated values shall be unique within the enumerated type (but do not have to be globally unique) and are consequently visible in the context of the given type only. This means that for any instantiation or value reference of an enumerated type, the given type shall be implicitly or explicitly identified.

NOTE 1:	For example, if the enumerated type is an element or field of a user defined structured type, the enumerated type is implicitly referenced via the given element/field (i.e. by the identifier of the field or the position of the value in a value list notation) at value assignment, instantiation, etc. Another example is passing an enumerated value as actual parameter, in which case the type of the corresponding formal parameter establishes the type context needed to make the enumeration value visible. The third example is the comparison operators: if the type of one of the operands is uniquely identified, it is used as a type context for the other operand (see example 2 below). The fourth example is the match operation, where the type of the template parameter establishes the type context for the operation, if the type of the value parameter is not identified (see example 2 in clause 15.9).

The identifiers of enumerated values, within the same module, shall only be reused within other structured type definitions and shall not be used for identifiers of local or global visibility at the same or a lower level of the same branch of the scope hierarchy (see scope hierarchy in clause 5.2).

EXAMPLE 1:	Declaration of enumerated types and values:

type enumerated MyFirstEnumType {

Monday, Tuesday, Wednesday, Thursday, Friday

};

type integer Monday;

// This definition does not clash with the previous one

// as Monday in MyFirstEnumType is of local scope

type enumerated MySecondEnumType {

Saturday, Sunday, Monday

};

// This definition is legal as it reuses the Monday identifier within

// a different enumerated type

type record MyRecordType {

integer			Monday

};

// This definition is legal as it reuses the Monday identifier within

// a distinct structured type as identifier of a given field of this type

type record MyNewRecordType {

MyFirstEnumType	firstField,

integer			secondField

};

var MyNewRecordType v_newRecordValue := { Monday, 0 }

// MyFirstEnumType is implicitly referenced via the firstField element of MyNewRecordType

EXAMPLE 2:	Using enumerated types (see also example 5 of clause 8.2.3.1):

// Valid instantiations of MyFirstEnumType and MySecondEnumType would be

var MyFirstEnumType v_today := Tuesday;

var MySecondEnumType v_tomorrow := Monday;

// The following statements however cause an error as the two variables are instances

// of different enumeration types

v_today := v_tomorrow;

v_today == v_tomorrow;

// The following operation is correct

if (v_today == Monday ) {...}

// the type of variable v_today identifies the type context of MyFirstEnumType for the

// equality operator

// But the following causes an error

if ( Tuesday == Wednesday ) {...}

// there is no TTCN-3 type(d) object to establish the type context for the equality operator

// Please note that the values Tuesday and Wednesday are defined within the type

// MyFirstEnumType only, but this is not sufficient to establish the type context

// This kind of error can be fixed using the qualified form for one of the enumerated values.

// The type referenced in the qualified form will provide the required type context as shown

// in the following condition

if (MyFirstEnumType.Tuesday == Wednesday ) {...}

Each enumerated value may optionally have a user-assigned integer expression or non-empty list of integer literal values or ranges of integer literal values, which is defined after the name of the enumerated value in parenthesis. Each user assigned expression shall be statically bound, known in compilation time, and evaluate to an integer value. Each user-assigned integer value shall be distinct within a single enumerated type, all ranges of all the values lists shall be disjoint and shall not include any of the used single integer values. For each enumerated value without an assigned integer value, the system successively associates an integer number in the textual order of the enumerated values, starting at the left-hand side, beginning with zero, by step 1 and skipping any number occupied by any of the enumerated values with a manually assigned value or value list. These values are only used by the system to allow the use of relational operators. Enumerated names with an associated value list shall only be used as values together with a specific integer value, which shall be one from the associated list, in parenthesis after the name. They can be used as a template of the enumerated type by adding a list of integer template(s) and ranges in parenthesis after the name. For enumerated values with no value assigned or with a specific integer value assigned, the user shall not directly use associated integer values, but can access them and convert integer values into enumerated values by using the predefined functions enum2int and int2enum (see clauses 16.1.2, C.1.30 and C.1.4).

NOTE 2:	The integer value also may be used by the system to encode/decode enumerated values. This, however is outside the scope of the present document (with the exception that TTCN-3 allows the association of encoding attributes to TTCN-3 items).

EXAMPLE 3:	Enumeration example with associated integers:

type enumerated MyThirdEnumType {

Blue(0),

Yellow(1),

Green(3),

Other(2, 4..255)

}

var MyThirdEnumType v_color := Other(5);

if (v_color == Other(4)) { // is false

}

if (v_color > Other(4)) { // is true

}

if (match(v_color, Other(?))) { // is true

}

if (match(v_color, Other(6..10))) { // is false

}

if (match(v_color, Other((6..10), 15, 20..25))) { // is false

}

v_color := Blue(0) //causes an error as enumerated values with a specific integer value assigned
	                   //shall not use the associated integer value

type enumerated MyEnum {

e_num (1),

e_expr (2+2),						// same as e_expr (4)

e_bin_conv (bit2int('0111'B)),	// same as e_bin_conv(7)

e_oct_conv (oct2int('34'O)),		// same as e_oct_conv(52)

e_hex_conv (hex2int('AC'H))		// same as e_hex_conv(172)

}

When a TTCN-3 module parameter, formal parameter, constant, variable, non-parameterized template or parameterized template with all formal parameters having default values of an imported enumerated type is defined, the name of that definition shall not be the same as any of the enumerated values of that type.

### 6.2.5	Unions

#### 6.2.5.0	General

TTCN-3 supports the union type. The union type is a collection of alternatives, each one identified by an identifier. Only one of the specified alternatives will ever be present in an actual union value. Union types are useful to model data which can take one of a finite number of known types.

EXAMPLE 1:

type union MyUnionType

{

integer 		number,

charstring 		string

};

// A valid instantiation of MyUnionType would be

var MyUnionType v_age, v_oneYearOlder;

var integer v_ageInMonths;

v_age.number := 34; 		// value notation by referencing the field. Note, that this 
							    // notation makes the given field to be the chosen one

v_oneYearOlder := {number := v_age.number+1};

v_ageInMonths := v_age.number * 12;

The assignment notation shall be used for union-s, and the notation shall assign a value to one field only. This field becomes the chosen field. Neither the not used symbol "-" nor omit is allowed in union value notations.

The value list notation shall not be used for setting values of union types.

At most one of the union alternatives can be declared as the default alternative by using the @default modifier before the type of the alternative. For unions with a default alternative, special type compatibility rules apply (see clause 6.3.2.4) which allow using the union value as compatible with the type of the default alternative. Therefore, the assignment notation does not have to be used to denote a value of the union type if the union's default alternative is to be chosen. Also, the default alternative selection does not have to be used to access the default alternative, if it is chosen.

The list of effective fields of a union contains all alternative identifiers of the union. In addition to that, if the union contains a default alternative of a record or set type, the list of effective fields contains all field identifiers of that type and if the union contains a default alternative of a union type, the list of effective fields contains the effective list of that type.

Restrictions

In addition to the general static rules of TTCN-3 given in clause 5, the following restrictions apply:

a)	All identifiersin the list of effective fields shall be distinct.

b)	The @default alternative shall not be of the anytype.

EXAMPLE 2:

type union MyDefaultUnionType

{

@default integer 			number,

charstring 		string

};

// A valid instantiation of MyDefaultUnionType would be

var MyDefaultUnionType v_age, v_oneYearOlder;

v_age := 34; 	// implicit usage of the default alternative: the integer type is

// compatible with the default alternative; this is a shorthand notation

// for v_age.number := 34 or v_age := { number := 34 }

v_oneYearOlder := v_age+1;	// implicit selection of the default alternative: the union

// default alternative is compatible with integer, so that it

// can be used as an integer expression; this is equivalent to:

// v_oneYearOlder.number := v_age.number+1;

type union MyDefaultUnionType2 {

@default

MyDefaultUnionType ageInYears,

integer ageInDays

}

var MyDefaultUnionType2 v_age2 := 3;	// nested default usage: 3 is compatible with

// both alternatives, but only alternative ageInYears

// has @default, so this is equivalent to

// v_age2 := { ageInYears := 3 } which is equivalent

// to v_age2 := { ageInYears := { number := 3 } }

var integer v_result := v_age + v_age2;	// v_result is 37 as the expression is equivalent

// to v_age.number + v_age2.ageInYears

v_age := {string := "I feel young"};

v_result := v_age + v_age2;			// test case error: v_age would be treated as

// v_age.number, which is not the selected alternative

type union MyUnionTypeWithDefaultErr

{

@default

MyDefaultUnionType2 ageInYears,

charstring 		  string			// produces an error as the identifier "string"

};										// is in the list of effective fields of

// the default alternative

// from the type of the default alternative of

// MyDefaultUnionType2

#### 6.2.5.1	Referencing fields of a union type

Alternatives of a union type shall be referenced by the dot notation TypeIdOrExpression.AlternativeId, where TypeIdOrExpression resolves to the name of a union type or an expression of a union type such as variable, formal parameter, module parameter, constant, template, or function invocation. AlternativeId shall resolve to the name of an alternative in the union type or in case of an anytype value or template AlternativeId shall resolve to a known type name or a known type name qualified with a module name. Alternatives of union type definitions shall not reference themselves.

EXAMPLE 1:

v_myVar5 := v_myUnion1.myChoice1;

// If a union type is nested in another type then the reference may look like this

v_myVar6 := v_myRecord1.myElement1.myChoice2;

// Note, that the union type, of which the field with the identifier 'myChoice2' is referenced,

// is embedded in a record type

If an alternative in a union type or a subtype of a union type is referenced by the dot notation, the resulting type is the set of values allowed for that alternative imposed by the constraints of the alternative declaration itself (i.e. any constraints applied to the union type itself are ignored).

When an alternative of a union type is referenced on the right hand side of an assignment an error shall occur if the referenced alternative is not the currently chosen alternative or if the referenced union field or value is omitted or uninitialized.

EXAMPLE 2:

type union MyUnion2

{

integer choice1,

charstring choice2

}

type record MyRecordEmbedsUnion

{

MyUnion2 field1 optional

}

...

var MyUnion2 v_un2 := { choice1 := 1 }

var charstring v_char := v_un2.choice2; // causes an error as v_un.choice2 is not chosen

var MyRecordEmbedsUnion v_rec := { field1 := omit }

var integer v_int := v_rec.field1.choice1; // causes an error as v_rec.field1 is omitted

When referencing an alternative of a union type on the left hand side of an assignment, the referenced alternative shall become the chosen one. This rule shall apply recursively if the reference contains alternatives of nested unions, choosing all the referenced alternatives.

When referencing an alternative of an uninitialized union value or field or omitted field (including omitting a field at a higher level of the embedding hierarchy) on the left hand side of an assignment, the reference shall recursively be expanded up to and including the depth of the referenced alternative as follows:

- When expanding a value or value field of union type, the alternative referenced in the dot notation becomes the chosen one.

Expansion of record, record of, set, set of, and array values and intermediate fields shall follow the rules of item a) in clauses 6.2.1.1 and 6.2.7, and clause 6.2.2.1 correspondingly.

At the end of the expansion, the value at the right hand side of the assignment shall be assigned to the referenced alternative.

EXAMPLE 3:

type union MyUnion3

{

integer choice1,

union

{

bitstring subchoice1,

charstring subchoice2

} choice2

}

...

var MyUnion3 v_un3 := { choice1 := 1 };

var MyRecordEmbedsUnion v_rec2 := { field1 := omit };

v_un3.choice2.subchoice2 := "Hello!";

// after the assignment v_un3 equals to { choice2 := { subchoice2 := "Hello!" } }

v_rec2.field1.choice1 := 10; // after the assignment v_rec2 equals to
	                             // { field1 := { choice1 := 10 } }

#### 6.2.5.2	Option and union

Optional fields are not allowed for the union type, which means that the optional keyword shall not be used with union types.

#### 6.2.5.3	Nested type definition for field types

TTCN-3 supports the definition of types for union alternatives nested within the union definition, similar to the mechanism for record types described in clause 6.2.1.3.

### 6.2.6	The anytype

The special type anytype is defined as a shorthand for the union of all known data types and the address type in a TTCN-3 module. The definition of the term data type and known types is given in clause 3.1. The address type shall be included if it has been explicitly defined within that module.

The fieldnames of the anytype shall be uniquely identified by the corresponding type names.

NOTE 1:	As a result of this requirement imported types with clashing names (either with an identifier of a definition in the importing module or with an identifier imported from a third module) cannot be reached via the anytype of the importing module.

EXAMPLE:

// A valid usage of anytype would be

var anytype v_myVarOne, v_myVarTwo;

var integer v_myVarThree;

v_myVarOne.integer := 34;

v_myVarTwo := {integer := v_myVarOne.integer + 1};

v_myVarThree := v_myVarOne.integer * 12;

The anytype is defined locally for each module and (like the other predefined types) cannot be directly imported by another module. However, a user defined type of the type anytype can be imported by another module. The effect of this is that all types of that module are imported.

NOTE 2:	The user-defined type of anytype "contains" all types imported into the module where it is declared. Importing such a user-defined type into a module may cause side effects and hence due caution should be given to such cases.

### 6.2.7	Arrays

Arrays can be used in TTCN-3 as a shorthand notation to specify record of types. They may be specified also at the point of a variable, formal parameter or function return value declaration. Arrays may be declared as single or multi-dimensional. Array dimensions shall be specified using constant expressions, which shall evaluate to a positive integer values. Constants used in the constant expressions shall meet with the restrictions in clause 10.

EXAMPLE 1:

type integer MyArrayType1[3];                      // A type with 3 integer elements

type record length (3) of integer MyRecordOfType1; // The corresponding record of

var MyArrayType1 v_a1:= { 7, 8, 9 };

var MyRecordOfType1 v_r1:= v_a1;	// MyArrayType1 and MyRecordOfType1 are compatible

var integer v_myArray1[3]:= v_r1; 	// Instantiates an integer array of 3 elements

// with the index 0 to 2

// being compatible to MyArrayType1 and MyRecordOfType1

var integer v_myArray2[2][3];	    // Instantiates a two-dimensional integer array of 2 × 3									        // elements with indexes from (0,0) to (1,2)

Array elements are accessed by means of the index notation ([]), which shall specify a valid index within the array's range. Individual elements of multi-dimensional arrays can be accessed by repeated use of the index notation. An array or record of integer restricted to a single size can be used in the index notation as a short-hand for the repeated index notation. Accessing elements outside the array's range will cause a compile-time or test case error.

EXAMPLE 2:

v_myArray1[1] := 5;
	v_myArray2[1][2] := 12;

v_myArray1[4] := 12;    // ERROR: index shall be between 0 and 2
	v_myArray2[3][2] := 15; // ERROR: first index shall be 0 or 1

Array dimensions may also be specified using ranges (with inclusive boundaries only). In such cases, the lower and upper values of the range syntax define the lower and upper index values. The upper value shall not be lesser than the corresponding lower value. Such an array is corresponding to a record of with a fixed length restriction computed as the difference between upper and lower index bound plus 1 and indexing starting from the lower bound of the array definition.

EXAMPLE 3:

type integer MyArrayType2[2 .. 5]; // A type with 4 integer elements, indices starting with 2

type record length (4) of integer MyRecordOfType2; // The corresponding record of

var integer v_myArray3[1 .. 5];	// Instantiates an integer array of 5 elements

// with the index 1 to 5

v_myArray3[1] := 10; // Lowest index

v_myArray3[5] := 50; // Highest index

var integer v_myArray4[1 .. 5][2 .. 3 ];	// Instantiates a two-dimensional integer array of
											    // 5 × 2 elements with indexes from (1,2) to (5,3)

NOTE:	It is not possible to define an array type with a variable amount of elements. Neither is it possible to define an unlimited array with a lower bound on the array index.

The values of array elements shall be compatible with the corresponding variable or type declaration. Values may be assigned individually by a value list notation or index notation or more than one or all at once by a value list notation or index assignment notation. For using the value list or assignment notation for arrays, the rules described in clause 6.2.3 are valid for arrays as well.

Index notation can be used on both the right-hand side and left-hand side of assignments. The index of the first element shall be zero or the lower bound if an index range has been given. The index shall not exceed the limitations given by either the length or the upper bound of the index. If the value of the element indicated by the index at the right-hand of an assignment is undefined or if the index notation is applied to an uninitialized or omitted array value on the right hand side of an assignment, error shall be caused. Sending an array value with undefined elements shall cause an error. All elements in an array value that are not set explicitly are undefined. When referencing an element of an uninitialized array value or field or omitted field on the left hand side of an assignment, the rules for record of values specified in clause 6.2.3 apply.

For assigning values to multi-dimensional arrays, each dimension that is assigned shall resolve to a set of values enclosed in curly braces. When specifying values for multi-dimensional arrays, the leftmost dimension corresponds to the outermost structure of the value, and the rightmost dimension to the innermost structure. The use of array slices of multi-dimensional arrays, i.e. when the number of indexes of the array value is less than the number of dimensions in the corresponding array definition, is allowed. Indexes of array slices shall correspond to the dimensions of the array definition from left to right (i.e. the first index of the slice corresponds to the first dimension of the definition). Slice indexes shall conform to the related array definition dimensions.

EXAMPLE 4:

v_myArray1[0]:= 10;

v_myArray1[1]:= 20;

v_myArray1[3]:= 30;

// or using an value list

v_myArray1:= {10, 20, -, 30};

v_myArray4:= {{1, 2}, {3, 4}, {5, 6}, {7, 8}, {9, 10}};

// the array value is completely defined

var integer v_myArray5[2][3][4] :=

{
		{
			{1, 2, 3, 4}, // assigns a value to v_myArray5 slice [0][0]
			{5, 6, 7, 8}, // assigns a value to v_myArray5 slice [0][1]
			{9, 10, 11, 12} // assigns a value to v_myArray5 slice [0][2]
		}, // end assignments to v_myArray5 slice [0] 
		{
			{13, 14, 15, 16}, {17, 18, 19, 20}, {21, 22, 23, 24}

} // assigns a value to v_myArray5 slice [1]

};

v_myArray4[2] := {20, 20};

// yields {{1, 2}, {3, 4}, {20, 20}, {7, 8}, {9, 10}};
	v_myArray5[1] := { {0, 0, 0, 0}, {0, 0, 0, 0}, {0, 0, 0, 0}};
		// yields {{{1, 2, 3, 4}, {5, 6, 7, 8}, {9, 10, 11, 12}},
		//         {{0, 0, 0, 0}, {0, 0, 0, 0}, {0, 0, 0, 0}}};

v_myArray5[0][2] := {3, 3, 3, 3};

// yields {{{1, 2, 3, 4}, {5, 6, 7, 8}, {3, 3, 3, 3}},

//         {{0, 0, 0, 0}, {0, 0, 0, 0}, {0, 0, 0, 0}}};

var integer v_myArrayInvalid[2][2];

v_myArrayInvalid := { 1, 2, 3, 4 }

// causes an error as the dimension of the value notation

// does not correspond to the dimensions of the definition

v_myArrayInvalid[2] := { 1, 2 }

// causes an error as the index of the slice should be 0 or 1

### 6.2.8	The default type

TTCN-3 allows the activation of altsteps (see clause 16.1.5) as defaults to capture recurring behaviour. Default references are unique references to activated defaults. Such a unique default reference is generated by a test component when an altstep is activated as a default, i.e. a default reference is the result of an activate operation (see clause 20.5.2).

Default references have the special and predefined type default. Variables of type default can be used to handle activated defaults in test components. The special value null represents an unspecific default reference, e.g. can be used for the initialization of variables of default type.

Default references are used in deactivate operations (see clause 20.5.3) in order to identify the default to be deactivated.

Default references have meaning only within the test component instances they are activated, i.e. a default reference assigned to a default variable in test component instance "a1" of type "A" has no meaning in test component instance "a2" of type "A".

The actual data representation of the default type shall be resolved externally by the test system. This allows abstract test cases to be specified independently of any real TTCN-3 runtime environment, in other words TTCN-3 does not restrict the implementation of a test system with respect to the handling and identification of defaults.

Values of the default type are object references and follow specific rules for this kind of values.

### 6.2.9	Communication port types

Ports facilitate communication between test components and between test components and the test system interface.

TTCN-3 supports message-based and procedure-based ports. Each port shall be defined as being message-based or procedure-based. Message-based ports shall be identified by the keyword message and procedure-based ports shall be identified by the keyword procedure within the associated port type definition.

Ports are bidirectional. The directions are specified by the keywords in (for the in direction), out (for the out direction) and inout (for both directions). Operations allowed on a procedure present in the incoming port list are getcall, reply and raise. Operations allowed on a procedure present in the outcoming port list are call, getreply and catch. Directions shall be seen from the point of view of the test component owning the port with the exception of the test system interface, where directions shall be seen from the point of view of the test component port mapped to the test system interface port. The in list of the test system interface port contains message or procedure for which the mapped test component port allows the following operations: receive, trigger, getcall, reply or raise. The out list of the test system interface port contains message or procedure for which the mapped test component port allow the folowing operations: send, call, getreplay or catch.

Each port type definition shall have one or more lists indicating the allowed collection of (message) types or procedure signatures together with the allowed communication direction.

For configuration purposes the port type may have one map param and one unmap param declaration indicating the allowed additional parameters for the respective operation. These formal parameters shall be value parameters.

Whenever a signature (see also clause 14) is defined in the out direction of a procedure-based port, the types of all its inout and out parameters, its return type and its exception types are automatically part of the in direction of this port. Whenever a signature is defined in the in direction for a procedure-based port, the types of all its inout and out parameters, its return type and its exception types are automatically part of the out direction of this port.

Ports used for the communication with the SUT may need to address specific entities within the SUT. In addition, several address schemes may be supported by one SUT at different ports. To support such addressing schemes, TTCN-3 allows to bind an address type to a port. Values of this type may be used for addressing purposes in communication operations (see clause 22.1) and be stored in variables. The handling of address types bound to different ports by means of the dot notation is explained in clause 6.2.12.

Syntactical Structure

Message-based port:

type port PortTypeIdentifier message "{"

{ (address Type ";") |

(map param "(" { FormalValuePar [","] }+ ")") |

(unmap param "(" { FormalValuePar [","] }+ ")") |

((in | out | inout) { MessageType [ "," ] }+ ";") }

"}"

Procedure-based port:

type port PortTypeIdentifier procedure "{"

{ (address Type ";" ) |

(map param "(" { FormalValuePar [","] }+ ")") |

(unmap param "(" { FormalValuePar [","] }+ ")") |

((in | out | inout) { Signature [ "," ] }+ ";") }

"}"

TTCN-3 allows to define constants, variables and parameters of a port type. These constants, variables or parameters can contain a reference to an existing component port or a special value null. The special value null represents an unspecified port reference, i.e. it can be used to explicitly allow the referencing of no port.

Port type values are object references and follow specific rules for this kind of values.

Restrictions

In addition to the general static rules of TTCN-3 given in clause 5, the following restrictions apply:

a)	At most one address type shall be bound to a port type.

b)	At most one map parameter list shall be defined for a port type.

c)	At most one unmap parameter list shall be defined for a port type.

d)	Formal parameters of map param and unmap param declarations shall be value parameters of a data type.

e)	MessageType shall be a reference to a data type.

Examples

EXAMPLE 1:	Message-based port

// Message-based port which allows types MsgType1 and MsgType2 to be received at, MsgType3 to be

// sent via and any integer value to be send and received over the port

type port MyMessagePortTypeOne message

{

in		MsgType1, MsgType2;

out		MsgType3;

inout 	integer

}

EXAMPLE 2:	Procedure-based port

// Procedure-based port which allows the remote call of the procedures Proc1, Proc2 and Proc3.

// Note that Proc1, Proc2 and Proc3 are defined as signatures

type port MyProcedurePortType procedure

{

out		Proc1, Proc2, Proc3

}

EXAMPLE 3:	Message-based port with address type definition

type port MyMessagePortTypeTwo message

{

address	integer;		// if addressing is used on ports of type MyMessagePortTypeTwo

// the addresses have to be of type integer

inout	MsgType1, MsgType2;

}

NOTE:	The term message is used to mean both messages as defined by templates and actual values resulting from expressions. Thus, the list restricting what may be used on a message-based port is simply a list of type names.

EXAMPLE 4:	Usage of param in port declaration

// Message based port which allows MsgType4 to be send and received over the port

// and MsgType5 and MsgType6 as configuration parameter type

type port MyMessagePortType message

{

inout	MsgType4;

map param	(in MsgType5 p_p1, out MsgType6 p_p2);

}

// Procedure based port which allows the remote call of the procedure Proc1

// and MsgType5 as configuration parameter type

type port MyProcedurePortType procedure

{

out		Proc1;

unmap param	(MsgType5 p_p1);

}

### 6.2.10	Component types

#### 6.2.10.1	Component type definition

The component type defines which ports are associated with a component (see figure 3). The port names in a component type definition are local to that component type, i.e. another component type may have ports with the same names. Port names in the same component type definition shall all have unique names. If not stated otherwise, ports have the same semantics as constants of a port type.

Figure 3: Typical components

It is also possible to declare constants, variables, templates and timers local to a particular component type. These declarations are visible to all testcases, functions and altsteps that run on an instance of the given component type. This shall be explicitly stated using the runs on keyword (see clause 16) in the testcase, function or altstep header. Component type definitions are associated with the component instance and follow the scope rules defined in clause 5.2. Each new instance of a component type will thus have its own set of constants, variables, templates and timers as specified in the component type definition (including any initial values, if stated). Constants used in the constant expressions of type declarations for variables, constants or ports shall meet with the restrictions in clause 10, however constants used in the constant expressions of initial values for variables, constants, templates or timers do not have to obey these restrictions.

Syntactical Structure

type component ComponentTypeIdentifier "{"

{ ( PortInstance

| VarInstance

| TimerInstance

| ConstDef

| TemplateDef ) }

"}"

Semantic Description

Component type definitions specify the creation, declaration and initialization of ports and component constants, variables, templates and timers during the creation of an instance of a component type. These instances can be used as the main test component, as the test system interface or as a parallel test component. Every instance of a component type has its own new instances of the ports, constants, variables, templates and timers defined in the component type definition.

Component instances are object references and follow specific rules for this kind of values.

Restrictions

No specific restrictions in addition to the general static rules of TTCN-3 given in clause 5.

Examples

EXAMPLE 1:	Component type with port instances only

type component MyPTCType

{

port MyMessagePortType		pCO1, pCO4;

port MyProcedurePortType	pCO2

}

EXAMPLE 2:	Component type with variable, timer and port instance

type component MyMTCType

{

var integer vc_myLocalInteger;

timer tc_myLocalTimer;

port MyMessagePortType	pCO1

}

EXAMPLE 3:	Component type with port instance arrays

type component MyCompType

{

port MyMessagePortType pCOm[3];

port MyProcedurePortType pCOp[3][3]

// Defines a component type which has an array of 3 message ports and a two-dimensional

// array of 9 procedure ports.

}

#### 6.2.10.2	Reuse of component types

It is possible to define component types as the extension of other component types, using the extends keyword.

Syntactical Structure

type component ComponentTypeIdentifier extends ComponentTypeIdentifier 
		{ "," ComponentTypeIdentifier} "{"

{ ( PortInstance

| VarInstance

| TimerInstance

| ConstDef

| TemplateDef ) }

"}"

Semantic Description

In such a definition, the new type definition is referred to as the extended type, and the type definition following the extends keyword is referred to as the parent type. The effect of this definition is that the extended type will implicitly also contain all definitions from the parent type. It is called the effective type definition.

It is allowed to have one component type extending several parent types in one definition, which have to be specified as a comma-separated list of types in the definition. Any of the parent types may also be defined by means of extension. The effective component type definition of the extended type is obtained as the collection of all constant, variable, template, timer and port definitions contributed by the parent types (determined recursively if a parent type is also defined by means of an extension) and the definitions declared in the extended type directly. The effective component type definition shall be name clash free.

NOTE 1:	It is not considered to be a different declaration and hence causes no error if a specific definition is contributed to the extended type by different parent types (via different extension paths).

The semantics of component types with extensions are defined by simply replacing each component type definition by its effective component type definition as a pre-processing step prior to using it.

NOTE 2:	For component type compatibility, this means that a component reference c of type CT1, which extends CT2, is compatible with CT2, and test cases, functions and altsteps specifying CT2 in their runs on clauses can be executed on c (see clause 6.3.2.7).

Restrictions

In addition to the general static rules of TTCN-3 given in clause 5, the following restrictions apply:

a)	When defining component types by extension, there shall be no name clash between the definitions being taken from the parent type and the definitions being added in the extended type, i.e. there shall not be a port, variable, constant or timer identifier that is declared both in the parent type (directly or by means of extension) and the extended type. It is not considered to be a name clash if a specific definition is contributed to the extended type via different extension paths.

b)	When defining component types by extending more than one parent type, there shall be no name clash between the definitions of the different parent types, i.e. there shall not be a port, variable, constant or timer identifier that is declared in any two of the parent types (directly or by means of extension). It is not considered to be a name clash if a specific definition is contributed to the extended type via different extension paths.

c)	It is allowed to extend component types that are defined by means of extension, as long as no cyclic chain of definition is created.

Examples

EXAMPLE 1:	A component type extension and its effective type definition

type component MyMTCType

{

var integer vc_myLocalInteger;

timer tc_myLocalTimer;

port MyMessagePortType pCO1

}

type component MyExtendedMTCType extends MyMTCType

{

var float vc_myLocalFloat;

timer tc_myOtherLocalTimer;

port MyMessagePortType pCO2;

}

// effectively, the above definition is equivalent to this one:

type component MyExtendedMTCType

{

/* the definitions from MyMTCType */

var integer vc_myLocalInteger;

timer tc_myLocalTimer;

port MyMessagePortType pCO1

/* the additional definitions */

var float vc_myLocalFloat;

timer tc_myOtherLocalTimer;

port MyMessagePortType pCO2;

}

EXAMPLE 2:	A component type extension chain and forbidden cyclic extensions

type component MTCTypeA extends MTCTypeB { /* … */ };
	type component MTCTypeB extends MTCTypeC { /* … */ };
	type component MTCTypeC extends MTCTypeA { /* … */ };  // ERROR - cyclic extension

type component MTCTypeD extends MTCTypeD { /* … */ };  // ERROR - cyclic extension

EXAMPLE 3:	Component type extensions with name clashes

type component MyExtendedMTCType extends MyMTCType

{

var integer vc_myLocalInteger;    // ERROR - already defined in MyMTCType (see above)

var float tc_myLocalTimer;	      // ERROR - timer with that name exists in MyMTCType

port MyOtherMessagePortType pCO1; // ERROR - port with that name exists in MyMTCType

}

	type component MyBaseComponent { timer tc_myLocalTimer };
	type component MyInterimComponent extends MyBaseComponent { timer tc_myOtherTimer };

type component MyExtendedComponent extends MyInterimComponent

{

timer tc_myLocalTimer; // ERROR - already defined in MyInterimComponent via extension

}

EXAMPLE 4:	Component type extension from several parent types

type component MyCompB { timer tc_t };

type component MyCompC { var integer tc_t };

type component MyCompD extends MyCompB, MyCompC {}

// ERROR - name clash between MyCompB and MyCompC

// MyCompB is defined above

type component MyCompE extends MyCompB {

var integer vc_myVar1 := 10;

}

type component MyCompF extends MyCompB {

var float vc_myVar2 := 1.0;

}

type component MyCompG extends MyCompB, MyCompE, MyCompF {

// No name clash.

// All three parent types of MyCompG have a timer tc_t, either directly or via extension of

// MyCompB; as all these stem (directly or via extension) from timer tc_t declared in

// MyCompB, which make this form of collision legal.

/* additional definitions here */

}

### 6.2.11	Component references

Component references are unique references to the test components created during the execution of a test case.

Syntactical Structure

system | mtc | self | ObjectReference

Semantic Description

A unique component reference is generated by the test system at the time when a component is created. It is the result of a create operation (see clause 21.2.1). In addition, component references are returned by the predefined operations system (returns the component reference of the test system interface, which is automatically created when testcase execution is started), mtc (returns the component reference of the MTC, which is automatically created when testcase execution started) and self (returns the component reference of the component in which self is called).

Component references are used in the configuration operations such as connect, map and start (see clause 21) to set-up test configurations and in the from, to and sender parts of communication operations of ports connected to test components other than the test system interface for addressing purposes (see clause 22 and figure 6).

In addition, the special value null is available to indicate an undefined component reference, e.g. for the initialization of variables to handle component references.

The actual data representation of component references shall be resolved externally by the test system. This allows abstract test cases to be specified independently of any real TTCN-3 runtime environment, in other words TTCN-3 does not restrict the implementation of a test system with respect to the handling and identification of test components.

A component reference includes component type information. This means, for example, that a variable for handling component references shall use the corresponding component type name in its declaration.

The configuration operations (see clause 21) do not work directly on arrays of components. Instead a specific element of the array shall be provided as the parameter to these operations. For components, the effect of an array is achieved by using an array of component references and assigning the relevant array element to the result of the create operation.

Restrictions

In addition to the general static rules of TTCN-3 given in clause 5, the following restrictions apply:

a)	The only operations allowed on component references are assignment, equality and non-equality.

b)	The ObjectReference shall be of a component type and shall not resolve to a template.

Examples

EXAMPLE 1:	Component references with component type variables

// A component type definition

type component MyCompType {

port PortTypeOne pCO1;

port PortTypeTwo pCO2

}

// Declaring one variable for the handling of references to components of type MyCompType

// and creating a component of this type

var MyCompType v_myCompInst := MyCompType.create;

EXAMPLE 2:	Usage of component references in configuration operations

// referring to the component created above

connect(self:myPCO1, v_myCompInst:pCO1);

map(myCompInst:pCO2, system:extPCO1);

myCompInst.start(f_myBehavior(self));	// self is passed as a parameter to f_myBehavior

EXAMPLE 3:	Usage of component references in from- and to- clauses

MyPCO1.receive from v_myCompInst;

:

MyPCO2.receive(integer:?) -> sender v_myCompInst;

:

MyPCO1.receive(mw_myTemplate) from v_myCompInst;

:

MyPCO2.send(integer:5) to v_myCompInst;

EXAMPLE 4:	Usage of component references in one-to-many connections

// The following example explains the case of a one-to-many connection at a Port PCO1

// where values of type M1 can be received from several components of the different types

// MyCompType1, MyCompType2 and MyCompType3 and where the sender has to be retrieved.

// In this case the following scheme may be used:

:

var M1 v_myMessage, v_myResult;

var MyCompType1 v_myInst1 := null;

var MyCompType2 v_myInst2 := null;

var MyCompType3 v_myInst3 := null;

:

alt {

[] pCO1.receive(M1:?) from MyCompType1:? -> value v_myMessage sender v_myInst1 {}

[] pCO1.receive(M1:?) from MyCompType1:? -> value v_myMessage sender v_myInst2 {}

[] pCO1.receive(M1:?) from MyCompType1:? -> value v_myMessage sender v_myInst3 {}

}

:

v_myResult := f_myMessageHandling(v_myMessage);	// some result is retrieved from a function

:

if (v_myInst1 != null) {pCO1.send(v_myResult) to v_myInst1};

if (v_myInst2 != null) {pCO1.send(v_myResult) to v_myInst2};

if (v_myInst3 != null) {pCO1.send(v_myResult) to v_myInst3};

:

EXAMPLE 5:	Usage of self

var MyComponentType v_myAddress;

v_myAddress := self; // Store the current component reference

EXAMPLE 6:	Usage of component arrays

// This example shows how to model the effect of creating, connecting and running arrays of

// components using a loop and by storing the created component reference in an array of

// component references.

testcase TC_MyTestCase() runs on MyMtcType system MyTestSystemInterface

{

:

var integer v_i;

var MyPTCType1	v_myPtc[11];

:

for (v_i:= 0; v_i<=10; v_i:= v_i+1)

{

v_myPtc[v_i] := MyPTCType1.create;

connect(self:ptcCoordination, v_myPtc[v_i]:mtcCoordination);

v_myPtc[v_i].start(MyPtcBehaviour());

}

:

}

### 6.2.12	Addressing entities inside the SUT

An SUT may consist of several entities which can be addressed individually. The global address type may be used if only one type is needed. If several types at different ports are needed for addressing SUT entities, the type used for addressing via a port instance shall be declared in the corresponding port type definition.

Syntactical Structure

TemplateInstance

Semantic Description

The actual data representation of the global address type is resolved either by an explicit global address type definition within the test suite, address type definitions within port definitions, or externally by the test system (i.e. the address type is left as an open type within the TTCN-3 specification). This allows abstract test cases to be specified independently of any real address mechanism specific to the SUT.

If an address type is bound to a port type definition, addressing of SUT instances (i.e. to- and from-directives in communication operations) via instances of that port type shall be restricted to values of the bound address type.

If several address types exist within a test suite, ambiguities shall be resolved by means of the dot notation. For example, a type reference within a variable definition used to store an SUT address may be prefixed by a port type identifier or a module identifier. If both a global address type definition and port definitions with an address type definition exist in a module, the global address type shall only affect ports without an explicit address type definition. The consistent use of explicit address type definitions within port definitions is recommended over the use of global address type definitions.

Explicit SUT addresses for a globally defined address type shall only be generated inside a TTCN-3 module if the type is defined inside the module itself. If the type is not defined inside the module, explicit SUT addresses for a global address type shall only be passed in as parameters or be received in message fields or as parameters of remote procedure calls.

In addition, the special value null is available for the address type to indicate an undefined address, e.g. for the initialization of variables of the address type.

If a port type definition includes the declaration of a type that shall be used for addressing SUT entities, only values of that type shall be used in to, from and sender parts of receive and send operations of port instances of that type mapped to the test system interface.

Restrictions

In addition to the general static rules of TTCN-3 given in clause 5, the following restrictions apply:

a)	TemplateInstance shall be of type address or of the type of the address declaration in a port type definition. If TemplateInstance is of type address, it may be an address type value, an address type variable, etc.

b)	For addressing purposes, the address type shall only be used in the to, from and sender parts of receive and send operations of ports mapped to the test system interface.

c)	The address type shall not be used in the to, from and sender parts of receive and send operations of connected ports, i.e. ports used for the communication among test components.

Examples

EXAMPLE 1:	Global address type

// Associates the type integer to the open type address

type integer address;

:

// new address variable initialized with null

var address v_mySUTentity := null;

:

// receiving an address value and assigning it to variable MySUTentity

pCO.receive(address:?) -> value v_mySUTentity;

:

// usage of the received address for sending template m_myResult

pCO.send(m_myResult) to v_mySUTentity;

:

// usage of the received address for receiving a confirmation template

pCO.receive(mw_myConfirmation) from v_mySUTentity;

EXAMPLE 2:	Port type-specific address type

type record MyAddressType { 						// user-defined type

integer field1;

boolean field2;

}

type port MyPortType message {

address	MyAddressType;							// address declaration

inout	integer;

}

type component MyComponentType

{

port MyPortType	pCO;

}

function f_myFunction () runs on MyComponentType {

var MyPortType.address v_sUT_Address := {5, true};	// address value for addressing
															// via ports of MyPortType

:

pCO.send(integer: 5) to v_sUT_Address;				// use of address value in to

:

pCO.receive(integer: ?) from v_sUT_Address; 		// use of address value in from

:

}

EXAMPLE 3:	Elaborated address example

type AddressType1 address;				// address type definition on module level

type port MyPortType1 message {

inout	MsgType1;

}

// address types bound to port types

type port MyPortType2 message {

address	AddressType2;				// values of type AddressType2 can be

// used to address SUT entities.

inout	MsgType2;

}

type port MyMessagePort3 message {

address	AddressType3;				// values of type AddressType3 can be

// used to address SUT entities.

inout	MsgType3;

}

// component type definition

type component MyComponentType

{

port MyPortType1	pCO1;

port MyPortType2	pCO2;

port MyPortType3	pCO3

}

// The following behaviour is considered to be executed on an instance of MyComponentType.

// Furthermore, it is considered that the ports PCO1, PCO2 and PCO3 are mapped ports, i.e.

// used for the communication with the SUT.

:

// new address variable initialized with null

var MyPortType1.address v_mySUTentity1 := null;	// type of v_mySUTentity1 is AddressType1

var MyPortType2.address	v_mySUTentity2 := null;	// type of v_mySUTentity2 is AddressType2

var MyPortType3.address	v_mySUTentity3 := null;	 // type of v_mySUTentity3 is AddressType3

:

// receiving address values and assigning them to variables

pCO1.receive(MsgType1:?) from address:? -> sender v_mySUTentity1;

// Address type of module scope,

// no prefix needed

pCO2.receive(MsgType2:?) from MyPortType2.address:? -> sender v_mySUTentity2;

// Resolution of address type

// by means of a prefix

pCO3.receive(MsgType3:?) from MyPortType3.address:? -> sender v_mySUTentity3;

:

// usage of the received address values for addressing purposes

pCO1.send(v_myResult) to v_mySUTentity1;

:

pCO2.receive(mw_myConfirmation) from v_mySUTentity2;

:

pCO3.send(m_myRequest) to v_mySUTentity3;

:

### 6.2.13	Subtyping of structured types

#### 6.2.13.0	General

TTCN-3 allows subtyping of structured types as given in table 3.

#### 6.2.13.1	Length subtyping of record ofs and set ofs

TTCN-3 permits constraining the number of elements in instances of record of and set of types.

The length keyword followed by a value or a range (with inclusive boundaries only) within brackets and used between the record or set and the of keywords, restricts the allowed number of elements for the given record of or set of type. The value or the bounds within the brackets shall be non-negative integer values, except when the infinity keyword is used at the place of the upper bound, in which case the maximum number of the elements is not constrained. In case of the range syntax the upper bound shall not be lesser than the lower bound value.

Record of and set of type definitions may be used to define new record of or set of subtypes. In this case the rules of the previous paragraph apply, except that the length keyword and the value or range defining the allowed number of iterations (within brackets) shall be placed following the identifier of the new type.

Constants used in the constant expressions of length subtyping shall meet with the restrictions in clause 10.

EXAMPLE 1:	Length restrictions of record of and set of types

type record length(10) of integer MyRecordOfType10;

// is a record of exactly 10 integers

type record length(0..10) of integer MyRecordOfType0_10;

// is a record of a maximum of 10 integers

type record length(10..infinity) of integer MyRecordOfType10up;

// record of at least 10 integers

type record length(0..infinity) of integer MyRecordOfType0up;

// an unrestricted record of integer type

EXAMPLE 2:	Length subtyping of referenced record of types

type record of charstring StringArray;

// is an unlimited record of, each element shall be a charstring

type StringArray StringArray34 length(4 .. 5);

// is a record of 4 or 5 elements, each element is a charstring

// it is equivalent to

// type record length(4 .. 5) of charstring StringArray34a;

type StringArray34 StringArray34again length(4 .. 5);

// the same as StringArray34

type StringArray34 StringArray6 length(6);

// causes an error as record ofs with 6 elements are not legal values of StringArray34

EXAMPLE 3:	Length subtyping of referenced set of types

type record MyCapsule {

set of integer mySetOfInt

}

type MyCapsule.mySetOfInt MySetOfIntSub length(5..10);

// unordered list of 5 to 10 integers

#### 6.2.13.2	List subtyping of structured types and anytype

List subtyping is possible when defining a new type based on an existing parent type, but not directly at the declaration of the first parent type (see table 3).

Subtypes defined by a list subtyping restrict the allowed values of the subtype to the values matched by at least one of the constraints in the list. In case of list subtyping of record, set, record of, set of, union and anytype types, and arrays, the list may contain both subtypes and possibly partial templates of the parent types. Subtype references shall be resolved in a recursive way: the collection of templates denoted by the subtype(s) referenced in the list become members of the new subtype definition  with an expanded list containing only possibly partial templates. When constraining record of, set of, union and anytype types, and arrays, all templates of the expanded list (i.e. after resolving the subtype references) shall be valid (i.e. complete) templates of the first parent type. When constraining record and set types, templates of the expanded list defined using the value list notation shall be valid (i.e. complete) templates, while templates of the expanded list defined using the field assignment notation may be partial (i.e. incomplete). In the latter case, the fields that are not explicitly present shall be considered as containing AnyValue for mandatory fields and AnyValueOrNone for optional fields.

NOTE:	Users should assign new values to single fields of values/templates based on types using list subtyping cautiously: it may happen that the new field value would be valid with other combination(s) of the rest of the fields but causes an erroneous record/set value, when combining with the actual values of the other fields. See example 1 below.

In case of enumerated types, the template list subtyping shall contain only values of the parent type.

EXAMPLE 1:	List subtyping of record types

type record MyRecord {

integer		f1	optional,

charstring	f2,

charstring	f3

}

type MyRecord MyRecordSub1 (

{ f1 := omit, f2 := "user", f3 := "password" },

{ f1 := 1, f2 := "User", f3 := "Password" }

) // a valid subtype of MyRecord containing 2 values

type MyRecord MyRecordSub2 (

MyRecordSub1,

{ f1 := 2, f2 := "uname", f3 := "pswd" },

{ f1 := 3, f2 := "Uname", f3 := "Pswd" }

) // a valid subtype of MyRecord, containing 4 values; notice that values of

// MyRecordSub1 are identified by referencing MyRecordSub1

type MyRecordSub1 MyRecordSub3 (

{ f1 := 1, f2 := "user", f3 := "password" },

{ f1 := 1, f2 := "User", f3 := "Password" }

) // invalid type as { f1 := 1, f2 := "user", f3 := "password" } is not a legal value of

// MyRecordSub1 (notice field f1)

type MyRecord MyRecordSub4 (

{ f2 := "user", f3 := "password" },

{ f2 := "User", f3 := "Password" }

) // any valid value of MyRecord, where the combination of f2 and f3 is
	  // f2 := "user" AND f3 := "password" or f2 := "User" AND f3 := "Password"

// i.e. field f1 is considered as if it was present and contained AnyValueOrNone

type MyRecord MyRecordSub5 (

{ f2 := "user", f3 := pattern "password|Password" },

{ f1 := (1 .. 10), f2 := "User" }

) // a valid subtype of MyRecord containing all values which match one of the given

// templates

// { f1 := *, f2 := "user", f3 := pattern "password|Password" } or

// { f1 := (1 .. 10), f2 := "User", f3 := ? }

type record R { integer k, integer i, integer j }

type R R2 ({ k:= 1, i := 2}, { k:= 2, i := 3})

function f_inc(inout integer p_p) {

p_p := p_p + 1;

}

function f() {

var R2 v_x := { 1, 2, 5 }

v_x.k := 2; // error, as the value {2,2,5} is not allowed

inc(v_x.i); // error, as the value {1,3,5} is not allowed

// (previous erroneous assignment is ignored here)

inc(v_x.j); // allowed

}

EXAMPLE 2:	List subtyping of record of types

type record of charstring MyRecordOf;

type MyRecordOf MyRecordOfSub1(

{ "aa" },

{ "bbb", "cc" },

{ "ddd", "ee", "ff" }

); // valid subtype of MyRecordOf

type MyRecordOfSub1 MyRecordOfSub2(

{ "aa" },

{ "bbb", "cc" }

); // valid subtype of MyRecordOfSub1

type MyRecordOfSub1 MyRecordOfSub3(

MyRecordOfSub2,

{ "ddd", "ee", "ff" }

); // valid, but equivalent to MyRecordOfSub1

type MyRecordOfSub1 MyRecordOfSub4(

MyRecordOfSub2,

{ "ddd", "ee", "fff" }

); // empty type as { "ddd", "ee", "fff" } is not a value of MyRecordOfSub1

// (notice the extra character f in the third element)

EXAMPLE 3:	List subtyping of union types

type union MyUnion {

integer		c1,

charstring	c2,

charstring	c3

};

type MyUnion MyUnionSub1 (

{ c1 := 0 },

{ c1 := 1 }

); // a valid subtype of MyUnion containing two values

type MyUnion MyUnionSub2 (

MyUnionSub1,

{ c2 := "mine" },

{ c3 := "yours" }

); // a valid subtype of MyUnion containing four values; notice that values of

// MyUnionSub1 are identified by referencing MyUnionSub1

type MyUnionSub1 MyUnionSub3 (

{ c1 := 0 },

{ c1 := 2 }

); // causes an error as { c1 := 2 } is not a value of MyUnionSub1

EXAMPLE 4:	List subtyping of enumerated types

type enumerated MyEnum { e_first, e_second, e_third, e_fourth, e_fifth };

type MyEnum EnumSub1 ( e_first, e_second, e_third );

// a valid subtype of MyEnum

type EnumSub1 EnumSub2 ( e_first, e_second );

// a valid subtype of EnumSub1

type EnumSub1 EnumSub3 ( e_first, e_second, e_fourth );

// causes an error as e_fourth is not a value of EnumSub1

type MyEnum EnumSub4 ( EnumSub1, e_fourth);

// causes an error as type references are not allowed in the template list

// of enumerated types

EXAMPLE 5:	List subtyping of anytype

type anytype MyAnySub1 (

{ integer := 5 },

{ boolean := false },

{ bitstring := '0011'B },

{ charstring := "mine" },

{ MyEnum := first }

); // a valid subtype of anytype, consisting of 5 values

type MyAnySub1 MyAnySub2 (

{ integer := 5 },

{ boolean := false },

{ bitstring := '0011'B }

); // a valid subtype of MyAnySub1, consisting of 3 values

type anytype MyAnySub3 (

MyAnySub2,

{ octetstring := 'FF'O }

); // a valid subtype of anytype, consisting of 4 values, 3 of which are defined

// by referring to MyAnySub2

type MyAnySub1 MyAnySub4 (

{ integer := 5 },

{ boolean := false },

{ MyEnum := second }

); // causes an error as { MyEnum := second } is not a value of MyAnySub1

type MyAnySub1 MyAnySub5 (

MyAnySub3,

{ MyEnum := first }

); // causes an error as { octetstring := 'FF'O } (defined via referencing MyAnySub3) is

// not a value of MyAnySub1

EXAMPLE 6:	List subtyping of arrays

type charstring MyArray[1 .. 2];

type MyArray MyArraySub1 (

{ "aa", "cc" },

{ "bb", "cc" }

); // valid subtype of MyArray

type MyArraySub1 MyArraySub2 (

{ "aa", "cc" }

); // valid subtype of MyArraySub1

type MyArrayList1 MyArraySub3 (

MyArraySub2,

{ "bb", "cc" }

); // valid, also equivalent to MyArraySub1

type MyArraySub1 MyArraySub4 (

MyArraySub2,

{ "dd", "cc" }

); // empty type as { "dd", "cc" } is not a value of MyArraySub1

// (notice the dd in the first element)

#### 6.2.13.3	Subtyping of the iterated type of record ofs and set ofs

A type restriction following the identifier of a newly defined record of or set of type (i.e. when the keywords record and of or set and of are used in the definition) shall constrain the innermost type. The newly defined iterated type shall be a subset of the innermost type. If the innermost type is a basic type, the subtyping rules in clause 6.1.2 shall apply. If the innermost type is referencing a structured type or anytype, the rules in clauses 6.2.13.1 and 6.2.13.2 shall apply.

EXAMPLE 1:	Subtyping of basic innermost types of record ofs and set ofs

type record of charstring String23Array length(2 .. 3);

// is an unlimited record of, each element shall be a charstring of 2 or 3 characters

type record length(0..10) of charstring String12Array10 length(12);

// is a record of a maximum of 10 strings each with exactly 12 characters

type record of record of charstring String12Array2D length(12);

// is a two-dimensional unlimited array of strings each with exactly 12 characters

type set length(5) of set length(6) of charstring String23Array2D56 length(2..3);

// is an unordered two-dimensional array of the size 5*6 strings, each composed

// of 2 or 3  characters

const String23Array c_str23arr_a := { "aa", "bbb", "cc", "ddd", "ee", "ff" };

// valid, all charstrings are 2 or 3 characters long

const String23Array c_str23arr_b := { "a", "bbbb", "cc", "ddd", "ee", "ff" };

// causes an error as "a" and "bbbb" are not 2 or 3 characters long

const String23Array2D56 c_str12arr2D56_a := {

{ "aa", "aaa", "bb", "bbb", "cc", "ccc" },

{ "dd", "ddd", "ee", "eee", "ff", "fff" },

{ "gg", "ggg", "hh", "hhh", "ii", "iii" },

{ "jj", "jjj", "kk", "kkk", "ll", "lll" },

{ "mm", "mmm", "nn", "nnn", "oo", "ooo" }

}; // valid, a 5*6 matrix of charstrings being 2 or 3 characters long

const String23Array2D56 c_str12arr2D56_b := {

{ "a", "aaa", "bb", "bbbb", "cc", "ccc" },

{ "dd", "ddd", "ee", "eee", "ff", "fff" },

{ "gg", "ggg", "hh", "hhh", "ii", "iii" },

{ "jj", "jjj", "kk", "kkk", "ll", "lll" },

{ "mm", "mmm", "nn", "nnn", "oo", "ooo", "pp" }

}; // causes an error as "a" and "bbbb" are not 2 or 3 characters long and

// the 5th inner record of has 7 elements

EXAMPLE 2:	Length subtyping of structured innermost types of record ofs

type record of String23Array String23Array45 length(4 .. 5);

// is a two-dimensional array, the first dimension is unlimited,

// the second dimension is restricted to 4 or 5 elements and each element

// is a charstring of 2 or 3 characters. It is equivalent to:

// type record of record length(4 .. 5) of charstring String23Array45 length(2 .. 3);

const String23Array45 c_str23arr45_a := {

{ "aa", "bbb", "cc", "ddd" },

{ "ee", "fff", "gg", "hhh", "ii" }

}; // valid, 4 or 5 elements in the inner record of, all containing 2 or 3 characters

const String23Array45 c_str23arr45_b := {

{ "aa" , "bbb", "cc" }

}; //causes an error as there are only 3 elements in the inner record of

const String23Array45 c_str23arr45_c := {

{ "aa", "bbbb", "cc", "dd" }

}; //causes an error as "bbbb" contains 4 characters

type record length(0 .. 1) of String23Array String23Array0145 length(4 .. 5);

// is a two-dimensional array, the first dimension is limited to 0 or 1 elements,

// the second dimension is restricted to 4 or 5 elements, each element is a

// charstring of 2 or 3 characters.

const String23Array0145 c_str23arr0145_a := {

{ "aa", "bbb", "cc", "ddd" },

}; // a valid 1*4 array of charstrings, each of 2 or 3 characters

const String23Array0145 c_str23arr0145_a := {

{ "aa", "bbb", "cc", "ddd" },

{ "ee", "fff", "gg", "hhh", "ii" }

}; // causes an error as there are two elements in the outer record of

const String23Array0145 c_str23arr0145_b := {

{ "aa" , "bbb", "cc" }

}; // causes an error as there are only 3 elements in the inner record of

const String23Array0145 c_str23arr0145_c := {

{ "aa", "bbbb", "cc", "dd" }

}; // causes an error as "bbbb" contains 4 characters

type record of String23Array45 String23Array6 length(6);

// empty type as String23Array45 is restricted to 4 or 5 elements,

// thus length restriction 6 is outside the allowed range

#### 6.2.13.4	Mixing subtyping mechanisms

In the case of structured types and the special type anytype, it is forbidden to mix different subtyping mechanisms (e.g. list and length) in the same definition.

### 6.2.14	The timer type

TTCN-3 allows to define timer constants, variables and parameters. These constants, variables or parameters can contain a reference to an existing timer or a special value null. The special value null represents an unspecified timer reference, i.e. can be used for variables to explicitly reference no timer.

Timer references have meaning only within the test component instances where the timer is defined, i.e. a timer reference assigned to a timer variable in a test component instance "a1" of type "A" has no meaning in a test component instance "a2" of type "A".

The values of timer type are object references and follow specific rules for this kind of values.

### 6.2.15	Map types

#### 6.2.15.0	General

TTCN-3 supports the specification of map types that map from a set of keys to a set of values in such a way that each value in the set of key is associated with exactly one value in the set of values.

#### 6.2.15.1	Map Type Definition

Syntactical Structure

type map from Type to Type Identifier

Semantical Description

The Type following the from keyword is the type of the keys of the map type. The Type following the to keyword is the type of the values of the map type.

Restrictions

- Template definitions or expressions of type map or any structured type containing a field or element of type map directly or indirectly are not allowed.

Examples

EXAMPLE:

type record Connection { IPadress addr, integer portNr }

type record Client { charstring name, … }

type map from Connection to Client ConnectedClients; // associate each connection with a client

#### 6.2.15.2	Indexed Assignment Notation

Syntactical Structure

"{" { "[" Index "]" ":=" Value ","  } "}"

Semantic Description

Values of type map can be denoted with the indexed assignment notation where the indices are values of the key type of the map type and the right hand sides are values of the value type.

When using the indexed assignment notation on the right hand side of an assignment, elements for keys to be changed are identified explicitly and either a value or the not used symbol "-" can be assigned to them. Other elements, not referred to by keys in the notation, shall remain unchanged. Re-assigning a previously initialized value, using the not used symbol or just skipping a field or element in an assignment notation, will cause that field or element to remain unchanged.

Restrictions

- In the index assignment notation, every index expression shall evaluate to a different value than all the other ones.

Examples

EXAMPLE:

var ConnectedClients v_ myVariable:= {

[connection0] := client0,

[connection1] := client1,

[connection2] := client2

}

v_myVariable :=

{

[connection1] := client3,

}

// after this, v_myVariable contains:

//	{ [connection0] := client0 /* unchanged */,

[connection1] := client3,

[connection2] := client2 /* unchanged */ }

v_myVariable :=

{

[connection0] := -,

[connection1] := client2,

[connection2] := -

}

// after this, v_myVariable contains:

//	{ [connection0] := client0 /* unchanged */,

[connection1] := client2,

[connection2] := client2 /* unchanged */ }

#### 6.2.15.3	Unmapping Keys

Syntactical Structure

unmap "(" ValueRef "," SingleExpression ")"

Semantic Description

To remove a mapping from a key to its associated value in a variable of type map, the unmap statement may be used. If the map variable did not associate the given key with a value, the operation has no effect.

Restrictions

- ValueRef shall be an initialized variable of map type and SingleExpression shall be a value compatible with the key type of the map type of ValueRef.

#### 6.2.15.4	Index Notation

Syntactical Structure

ValueRef  "[" Index "]"

Semantic Description

Index notation can be used on both the right-hand side and left-hand side of assignments.

The index notation, when used on the right-hand side, refers to the value element that the map associates with the key given as the index.

When it is used at the left-hand side, only the value associated with the index key is changed, values with other keys remain unchanged. Using the index notation on the left hand side of an assignment with a key that is already mapped  causes the key to be mapped to the assigned value.

Presence of a mapping for a key can be checked by using the isbound, isvalue or ispresent with an index notation as its argument.

Restrictions

- The index expression used for index notation and index assignment notation shall be completely initialized and compatible with the from type of the map type.

- When an index operation of a map value is used as the left-hand side of an assignment, the assigned value shall be compatible with the to type of the map type.

- An index notation used on the right hand side with a key that has no associated value shall result in an error.

Examples

EXAMPLE:

if (not isbound(v_myVariable[connection0])) {

v_myVariable[connection0] := newClient; // adds mapping for connection0 to the map

}

if (isbound(v_myVariable[connection0])) {

unmap(v_myVariable, connection0); // deletes mapping for connection0 from the map

f(v_myVariable[connection0]); // results in a testcase error

}

#### 6.2.15.5	Accessing the Keys of a Map

The set of keys that the map associates with a value can be accessed using the from selector.

Syntactical Structure

MapValue "." from

MapValueType "." from

Semantic Description

If the MapValue is a value of type MapValueType defined as map from A to B then MapValue.from yields a result compatible with type set of A that contains all the key values from MapValue. This type can also be referenced as MapValueType.from.

NOTE:	It is unspecified in which order the keys of a map type value are enumerated.

EXAMPLE: 	Iterating over the keys of a map

var MapValueType v_map;

var MapValueType.from v_keys := v_map.from;

for (var integer i := 0; i < lengthof(v_keys); i := i + 1) {

// do something with v_map[v_keys[i]];

}

#### 6.2.15.6	Accessing the Values of a Map

The set of values that the map contains can be accessed using the to selector.

Syntactical Structure

MapValue "." to

MapValueType "." to

Semantic Description

If MapValue is a value of type MapValueType defined as map from A to B then MapValue.to yields a result compatible with type set of B that contains all the values from MapValue. This type can also be referenced as MapValueType.to;

EXAMPLE: 	Iterating over the values of a map

var MapValueType v_map;

var MapValueType.to v_values := v_map.to;

for (var integer i := 0; i < lengthof(v_values); i := i + 1) {

// do something with v_values[i];

}

#### 6.2.15.7	Referencing of Elements of a Map

When referencing an element of an uninitialized map value or field or omitted field (including omitting a field at a higher level of the embedding hierarchy) on the left-hand side of an assignment, the reference shall recursively be expanded up to and including the depth of the referenced element as follows:

- When expanding a value or value field of map type, the element referenced by the index notation shall be set to present.

- Expansion of record, record of and set of , union and set values and intermediate fields shall follow the rules of item a) in clauses 6.2.1.1, 6.2.3.0 and 6.2.5.1 and clause 6.2.2.1 correspondingly.

- At the end of the expansion, the value at the right-hand side of the assignment shall be assigned to the referenced element.

#### 6.2.15.8	Nested type definitions

TTCN-3 supports the definition of map types with nested structured types for the to part. The usage of structured types (record, set, enumerated, set of, record of and map) is allowed.

NOTE:	It is strongly recommended that only base types are used for the keys of map types.

EXAMPLE:

type map from charstring to map from charstring to charstring MapType;

### 6.2.16	The open type

The open type is represented by the keyword any. It shall onlz be used in formal parameters of external and predefined functions. Values of all existing TTCN-3 types can be directly passed as actual parameters to formal parameters of the open type without the need to explicitly specify a type context.

Restrictions

a)	The open type shall be used only in formal parameter of predefined or external functions.

NOTE:	The open type is forbidden in all other TTCN-3 statements, e.g. in:

Type declarations

Variable declarations

Template declarations

Constant declarations

Module parameter declarations

Formal parameters of templates, test cases, altsteps, non-external functions, map and unmap operations

Return clauses of functions

EXAMPLE:

external function fx_printf(charstring p_format, any p_data);

...

fx_printf("Numeric value: %d", 1);

## 6.3	Type compatibility

### 6.3.0	General

Generally, TTCN-3 requires type compatibility of values at assignments, instantiations and comparison.

For the purpose of this clause the actual value to be assigned, passed as parameter, etc., is called value "b". The type of value "b" is called type "B". The type of the formal parameter, which is to obtain the actual value of value "b" is called type "A".

NOTE:	As address is more a predefined type name than a distinct type with its own properties, the same type compatibility rules apply to an address type and to its derivatives as the rules were if the type was defined with a name different from address.

### 6.3.1	Compatibility of non-structured types

For variables, constants, templates, etc. of simple basic types and basic string types the value "b" is compatible to type "A" if type "B" resolves to the same root type as type "A" (e.g. integer) and it does not violate subtyping (e.g. ranges, length restrictions) of type "A". Compatibility between charstring and universal charstring is defined below.

EXAMPLE 1:	Compatibility of integers:

// Given

type integer MyInteger(1 .. 10);

:

var integer v_x;

var MyInteger v_y;

// Then

v_y := 5; // is a valid assignment

v_x := v_y;

// is a valid assignment, because v_y has the same root type as v_x and no subtyping is violated

v_x := 20; // is a valid assignment

v_y := v_x;

// is NOT a valid assignment, because the value of v_x is out of the range of MyInteger

v_x := 5; // is a valid assignment

v_y := v_x;

// is a valid assignment, because the value of v_x is now within the range of MyInteger

EXAMPLE 2:	Compatibility of floats:

// Given

type float PositiveFloats(0.0 .. infinity);

:

var PositiveFloats v_x;

var float v_y;

// Then

v_y := 5.0; // is a valid assignment

v_x := v_y;

// is a valid assignment, because v_y has the same root type as v_x and no subtyping is violated

v_y := -20.0; // is a valid assignment

v_x := v_y;

// causes an error, because the value of v_y is out of the range of PositiveFloats

v_y := not_a_number; // is a valid assignment

v_x := v_y;

// causes an error, because the value not_a_number is out of the range of PositiveFloats

EXAMPLE 3:	Compatibility of charstrings:

//Given

type charstring MyChar length (1);

type charstring MySingleChar length (1);

var MyChar v_myCharacter;

var charstring v_myCharString;

var MySingleChar v_mySingleCharString := "B";

//Then

v_myCharString := v_mySingleCharString;

//is a valid assignment as charstring restricted to length 1 is compatible with charstring.

v_myCharacter := v_mySingleCharString;

//is a valid assignment as two single-character-length charstrings are compatible.

//Given

v_myCharString := "abcd";

//Then

v_myCharacter := v_myCharString[1];

//is valid as the r.h.s. notation addresses a single element from the string

//Given

var charstring v_myCharacterArray [5] := {"A", "B", "C", "D", "E"}

//Then

v_myCharString := v_myCharacterArray[1];

//is valid and assigns the value "B" to v_myCharString;

For variables, constants, templates, etc. of charstring type, value 'b' is compatible with a universal charstring type 'A' unless it violates any type constraint specification (range, list or length) of type "A".

For variables, constants, templates, etc. of universal charstring type, value 'b' is compatible with a charstring type 'A' if all characters used in value 'b' have their corresponding characters (i.e. the same control or graphical character using the same character code) in the type charstring and it does not violate any type constraint specification (range, list or length) of type "A".

EXAMPLE 4:	Compatibility of character and universal character strings:

//Given

type charstring MyChar length (1);

...

var MyChar v_myCharacter;

var charstring v_myCharString;

var universal charstring v_myUnivCharString;

// Given

v_myCharString := "abcd";

// Then

v_myUnivCharString := v_myCharString

//is valid as charstring and universal charstring are compatible

v_myCharacter := v_myUnivCharString [1];

// is valid as the r.h.s. notation addresses a single element of the string,

// containing a character compatible with charstring

// Given

v_myUnivCharString := "bet" & char ( 0, 0, 1, 113);

// Then

v_myCharString := v_myUnivCharString;

// is invalid as v_myUnivCharString contains a character not in ISO 646.

// Given

var charstring v_myCharacterArray [5] := {"A", "B", "C", "D", "E"}

// Then

v_myCharString := v_myCharacterArray[1];

// is valid and assigns the value "B" to v_myCharString;

### 6.3.2	Compatibility of structured types

#### 6.3.2.0	General

This clause defines compatibility rules for structured types. In subsequent clauses, "value "b"" is called the value to be assigned, e.g. when passed as parameter, to an object of type "A".

#### 6.3.2.1	Compatibility of enumerated types

Enumerated types are only compatible with other enumerated types. An enumerated value "b" of an enumerated type "B" is compatible with enumerated type "A" if the identifier of the value "b" is also defined in "A" and the integer(s) associated with value "b" are also associated with the same identifier in "A".

EXAMPLE:	Assigning enumerated values:

// Given

type enumerated EWeekDays {

Mon, Tue, Wed, Thu, Fri, Sat, Sun

};

type enumerated EWorkDays {

Mon, Tue, Wed, Thu, Fri

};

type enumerated EDesWeekDays {

Tue, Wed, Thu, Fri, Sat, Sun, Mon

};

type enumerated EComplexValues {

e_num (1),

e_expr (2+2),

e_bin_conv (bit2int('0111'B)),

e_oct_conv (oct2int('34'O)),

e_hex_conv (hex2int('AC'H))

}

type enumerated ESimpleValues {

e_num (1),

e_expr (4),

e_bin_conv (7),

e_oct_conv (52),

e_hex_conv (172)

}

var EWeekDays v_myWeekDayMon := Mon

var EWeekDays v_myWeekDaySun := Sun

var EWorkDays v_myWorkDayMon := Mon

var EDesWeekDays v_myDesWeekDayMon := Mon

var EComplexValues v_myComplexValuedEnum := e_bin_conv;

var ESimpleValues v_mySimpleValuedEnum := e_bin_conv;

v_myWorkDayMon := v_myWeekDayMon

// works

v_mySimpleValuedEnum := v_myComplexValuedEnum;

// works

v_myWorkDayMon := v_myWeekDaySun

// causes an error as Sun is not a member of EWorkDays

v_myDesWeekDayMon := v_myWeekDayMon

// causes an error as Mon in EDesWeekDays and EWeekDays have different associated

// numbers; since this is true for all enumerated values in EWeekDays, these two

// types are fully incompatible

#### 6.3.2.2	Compatibility of record and record of types

record types are compatible if the number, and optional aspect of the fields in the textual order of definition are identical, the types of each field are compatible and the value of each existing field of the value "b" is compatible with the type of its corresponding field in type "A". The value of each field in the value "b" is assigned to the corresponding field in the value of type "A".

EXAMPLE 1:

// Given

type record AType {

integer		a(0..10)	optional,

integer		b(0..10)	optional,

boolean			c

}

type record BType {

integer			a		optional,

integer		b(0..10)	optional,

boolean			c

}

type record CType {		// type with different field names

integer			d	optional,

integer			e	optional,

boolean			f

}

type record DType {		// type with field c optional

integer			a	optional,

integer			b	optional,

boolean			c	optional

}

type record EType {		// type with an extra field d

integer			a	optional,

integer			b	optional,

boolean			c,

float			d	optional

}

var AType v_myVarA := { -, 1, true};

var BType v_myVarB := { omit, 2, true};

var CType v_myVarC := { 3, omit, true};

var DType v_myVarD := { 4, 4, true};

var EType v_myVarE := { 5, 5, true, omit};

// Then

v_myVarA := v_myVarB;	// is a valid assignment,

// new value of MyVarA is ( a :=omit, b:= 2, c:= true)

v_myVarC := v_myVarB;	// is a valid assignment

// new value of MyVarC is ( d :=omit, e:= 2, f:= true)

v_myVarA := v_myVarD;	// is NOT a valid assignment because the optionality of fields does not

// match

v_myVarA := v_myVarE;	// is NOT a valid assignment because the number of fields does not match

v_myVarC := { d:= 20 };	// actual value of MyVarC is { d:=20, e:=2,f:= true }

v_myVarA := v_myVarC	// is NOT a valid assignment because field 'd' of MyVarC violates

// subtyping of field 'a' of AType

record of types and arrays are compatible if their element types are compatible and value "b" does not violate any length subtyping of the record of type "A" or dimensions of the array type. Values of elements of the value "b" shall be assigned sequentially to the instance of type "A", including undefined elements.

Two array types are compatible if their corresponding record of types are compatible.

EXAMPLE 2:

// Given

type record of integer IType;

type record of float HType;

var HType v_myVarH := { 1, omit, 2};

var IType v_myVarI;

var integer v_myArrayVar[2];

// Then

v_myVarI := { 3, 4 };

v_myArrayVar := v_myVarI;
	// is a valid assignment as element types are compatible and the assigned value

// doesn't violate length restriction set by array dimension

v_myVarI2 := v_myArrayVar;

// is a valid assignment as element types are compatible and the target variable type has

// no length restriction

v_myVarI[2] := 5; // the value of v_myVarI is { 3, 4, 5 } now

v_myArrayVar := v_myVarI;

// is NOT a valid assignment as v_myVarI contains more elements than the array dimension

// allows

v_myVarH := v_myVarI;

// is NOT a valid assignment as element types are not compatible

#### 6.3.2.3	Compatibility of set and set of types

set types are only compatible with other set types and set of types are only compatible with other set of types. For set types the same compatibility rules shall apply as to record types and for set of types the same compatibility rules shall apply as to record of types.

NOTE 1:	This implies that though the order of elements at sending and receipt is unknown, when determining type compatibility for set types, the textual order of the fields in the type definition is decisive.

NOTE 2:	In set values the order of fields may be arbitrary, however this does not affect type compatibility as field names unambiguously identify, which fields of the related set type correspond to which set value fields.

EXAMPLE:

// Given

type set FType {

integer	a	optional,

integer	b	optional,

boolean	c

}

type set GType {

integer	d 	optional,

integer	e 	optional,

boolean	f

}

var FType v_myVarF := { a:=1, c:=true };

var GType v_myVarG := { f:=true, d:=7};

// Then

v_myVarF := v_myVarG;	// is a valid assignment as types FType and GType are compatible

v_myVarF := v_myVarA;	// is NOT a valid assignment as v_myVarA is a record type

#### 6.3.2.4	Compatibility of union types

The compatibility rules for union types are the following:

A union value "b" of union type "B" is compatible with union type "A" if the alternative selected in "b" has a corresponding alternative with identical name in "A" and the value of the selected alternative in "b" is compatible to the type of the corresponding alternative in "A".

Otherwise, the following rules apply. A union value "b" of union type "B" with a default alternative of type "C" is compatible with an arbitrary type "A" if the alternative selected in "b" is the default alternative and the value of the default alternative is compatible to "A". A value "a" of an arbitrary type "A" is compatible with a union type "B" with a default alternative of type "C" if value "a" is compatible to "C".

When considering the compatibility of two union types, initially the first rule (which is not dependent on the existence of a default alternative) shall be applied. The second rule shall only be used to check compatibility, when - using the first rule - no compatibility has been determined. This order shall avoid ambiguity in case that a default alternative would otherwise also be compatible with the union itself.

NOTE 1:	It is possible to have nested unions with default alternatives. The rules above make type compatibility along the default alternatives transitive, i.e. the outermost union type is compatible with the type of the innermost default union alternative if all containing alternatives are also default alternatives.

NOTE 2:	When a union with a default alternative is used in an expression it will be resolved to its long notation, before the expression is evaluated.

EXAMPLE 1:

type union U1 {integer i};

type union U2 {integer i, boolean b};

var U1 v_u1 := {i := 1};

var U2 v_u2 := v_u1; 		// correct

v_u1:= v_u2;				// correct as the alternative i is selected in v_u2 and is 
								// compatible to i in U1

v_u2:= {b := true};

v_u1:= v_u2;				// incorrect as v_u1 has no alternative b

var anytype v_x := v_u1; 	// incorrect as the anytype is not a union type.

EXAMPLE 2:	Using union values of unions with default alternatives

type union U3 { @default integer i, boolean b }

type union U4 { integer i, @default boolean b }

var U3 v_u3 := 3			// correct as i in U3 is declared with @default

v_u3 := v_u2;			// correct because all alternatives in U2 exist in U3

// and are compatible

v_u2 := 3;				// incorrect as 3 is not of a union type and there is

// no field in U2  declared with @default

v_u2 := v_u3;			// also correct

v_u2 := v_u1.i;			// incorrect

v_u3 := v_u1.i;			// correct

var integer v_int := v_u3 * 2	// v_int is 6 as v_u3 is treated as v_u3.i

var U3 v_u32 := {b := true};

var U4 v_u4 := true;

v_int := v_u4 *2;		// incorrect as v_u4 is treated as a boolean, and cannot be multiplied

v_int := v_u32 *2;		// test case error as "v_u32" would be treated as v_u32.i,

// which is not the selected alternative

log(v_u4);				// results in "{ b:=true}" logged; for backward compatibility when

// a union value is used in a log statement directly, no conversion

// is performed.

#### 6.3.2.5	Compatibility of anytype types

anytype types are only compatible with other anytype types. An anytype value "b" of anytype type "B" is compatible with anytype type "A" if the alternative selected in "b" is also contained in "A".

NOTE:	Only anytype types that are constrained to a fixed set of types via list subtyping can be a potential cause for anytype incompatibility, i.e. if the set of types contained in type "A" does not contain the type selected in "b".

EXAMPLE:

module A {

type integer I (0..2);

type float F;

type anytype Atype ({I:=?},{F:=?},{integer:=?});

//anytype composed of TTCN-3 built-in basic type integer, I, and F

}

module B {

type integer I (0..2);

type anytype Atype ({I:=?},{integer:=?},{float:=?});

}

module C {

import from A all;

import from B all;

type union U {

integer I (0..2)

}

control {

var A.Atype v_aa;

var A.Atype v_aaI := { A.I := 1 } // type I is imported from A and B

var A.Atype v_aaF := { F := 1.0 } // type F is only imported from A

var B.Atype v_ba := { integer := 1 }

var B.Atype v_baI := { B.I := 1 } // type I is imported from A and B

var U v_u := { I := 1 } // I is a field name in U

v_aa := v_ba;		// correct, the value of aa1 becomes { integer := 1 }

v_aa := v_baI;		// incorrect, type B.I is not present in the anytype A.Atype

v_aa := v_u; 		// incorrect, type of u is not anytype but a user defined union type

v_ba := { float := 1.0 };	// correct, assigning a literal value

v_ba := v_aaI;		// incorrect, type A.I is not present in the anytype B.Atype

v_ba := v_aaF;		// incorrect, type A.F is not present in the anytype B.Atype

}

}

#### 6.3.2.6	Compatibility between sub-structures

The rules defined in this clause for structured types compatibility are also valid for the sub-structure of such types.

EXAMPLE:

// Given

type record JType {

AType	a,

integer	b optional,

integer	c

}

var JType v_myVarJ;

// If considering the declarations above, then

v_myVarJ.a := v_myVarA;
	// is a valid assignment as the type of field a of JType and AType are compatible

v_myVarB := v_myVarJ.a;
	// is a valid assignment as BType and the type of field a of JType are compatible

#### 6.3.2.7	Compatibility of the open type

Values of all existing TTCN-3 types are compatible with the open type occurring in the left hand side of an assignment. Values of open type occurring on the right hand side of an assignment are compatible with a value "a" of a type "A", if the actual value contained in the open type value is compatible with "A".

### 6.3.3	Compatibility of component types

Type compatibility of component types has to be considered in different conditions:

Compatibility of a component reference value with a component type (e.g. when passing a component reference as an actual parameter to a function or an altstep or when assigning a component reference value to a variable of different component type): a component reference "b" of component type "B" is compatible with component type "A" if all definitions of "A" have identical definitions in "B".

Runs on compatibility: a function or altstep referring to component type "A" in its runs on clause may be called or started on a component instance of type 'B' if all the definitions of "A" have identical definitions in "B".

Mtc compatibility: a function or altstep referring to component type "A" in its mtc clause may be called or started in any context that has a mtc clause of type "B" or a testcase with a runs on clause of type "B" if all the port definitions of "A" have identical definitions in "B". If the type of the mtc is unknown in the calling function, this can lead to runtime errors if the component type "A" is not mtc-compatible with the type of the running mtc.

System compatibility: a function or altstep referring to component type "A" in its system clause may be called or started in any context that has a system clause of type "B" or a test case with a runs on clause of type "B" and no system clause if all the port definitions of "A" have identical definitions in "B". If the type of the system is unknown in the calling function, this can lead to runtime errors if the component type "A" is not system-compatible with the type of the system the current test case was started on.

Identity of definitions in "A" with definitions of "B" is determined based on the following rules:

- For port instances, both the type and the identifier shall be identical.

- For timer instances, identifiers shall be identical and either both shall have identical initial durations or both shall have no initial duration.

- For variable instances and constant definitions, the identifiers, the types and initialization values shall be identical (in case of variables this means that either the values are missing in both definitions or are the same).

- For local template definitions, the identifiers, the types, the formal parameter lists and the assigned template or template field values shall be identical.

### 6.3.4	Type compatibility of communication and connection operations

The communication operations (see clause 22) send, receive, trigger, call, getcall, reply, getreply and raise and connection operations connect, map, disconnect and unmap (see clause 21.1) are exceptions to the weaker rule of type compatibility and require strong typing. The types of values or templates directly used as parameters of the operations send, receive and trigger shall also be explicitly defined in the associated port type definition. The signature type of the parameter list given to the operations call, getcall, reply, getreply and the signature type given to the operations catch and raise shall also be explicitly defined in the associated port type definition. The types of values or templates directly used as exceptions to the operations catch and raise shall be explicitly defined in the exceptions part of the definition of the signature given to the operation.

EXAMPLE:

type record MyRec {...} 							// user defined type

type MyRec MyRecAlias; 							// a type alias

type port MyPort message { inout MyRec, MyRecAlias; } 	// port that can transport both types

type component MyComponent { port MyPort p; }

template MyRecAlias m_myRecAlias:= {...} 			// a template of the alias type

var MyComponent v_myComp1 := MyComponent.create, v_myComp2 := MyComponent.create;

connect (v_myComp1:p, v_myComp2:p)				// two connected PTCs via ports that can

// transport the user-defined and the alias type

// in v_myComp1:

p.send (m_myRecAlias); 							// sending of template of alias type

// in v_myComp2:

p.receive (MyRec:?);

// shall not match as the transmitted template is of the alias type and

// not of the user-defined type

// in v_myComp2:

var MyRec v_x;

p.receive (MyRecAlias:?) -> value v_x;

// shall not cause an error since storing the value requires no strong typing

### 6.3.5	Type conversion

If it is necessary to convert values of one type to values of another type, because their types have different root types, then either one of the predefined conversion functions defined in clause 16.1.2 or a user defined function shall be used.

EXAMPLE:

// To convert an integer value to a hexstring value use the predefined function int2hex

MyHstring := int2hex(123, 4);

### 6.3.6	Type compatibility of port types

For variables, constants and parameters of port types, the reference to a port "b" of type "B" is compatible to type "A" if type "B" and type "A" are equal or synonym types.

### 6.3.7	Type compatibility of timer types

For variables, constants and parameters of timer types, the reference to a timer is compatible with any other timer reference.

### 6.3.8	Type Compatibility of Map Types

map types are only compatible with other map types. A value "b" of a map type "B" is compatible with a map type "A" if the from type of  "A" is compatible with the from type of "B" and the to type of "B" is compatible with the to type of "A".

EXAMPLE:

type map from charstring to universal charstring Map1;

type map from universal charstring to charstring Map2;

// every value of Map2 can be used in a variable of Map1 but access to values

// associated with keys that are not charstring values is not possible

## 6.4	Type synonym

A type can be defined as a synonym to another type. Type synonyms can be defined for all kinds of types. Synonym types are compatible.

EXAMPLE:

type MyType1 MyType2; // MyType2 is synonym to MyType1

# 7	Expressions

## 7.0	General

TTCN-3 allows the specification of expressions. TTCN-3 expressions may be template references, value references or literals (i.e. no operation is involved), and may be composed of the operators defined in clause 7.1.

NOTE:	Templates can be used at the RHS of assignment, parameter passing and (predefined) functions where template passing is explicitly allowed.

Syntactical Structure

SingleExpression |

"{" { ( FieldReference ":=" ( Expression | "-" )) [","] } "}" | 	// compound expression

"{" [ { ( Expression | "-" ) [","] } ] "}"							// compound expression

Semantic Description

Expressions may be built from other (simple) expressions. Functions used in expressions shall have a return clause. The operands of the operators used in an expression shall be values and their root types shall be the types specified for the appropriate operator in the subsequent clauses.

Assignment or list notations are used for expressions of record, set, record of, set of, array, union and anytype types.

Restrictions

In addition to the general static rules of TTCN-3 given in clause 5, the following restrictions apply:

a)	At the point, when an expression is evaluated, the evaluated values of the operands used in expressions shall be completely initialized except where explicitly stated otherwise in the specific clause of the operator.

b)	The root types of the operands shall be the types specified for the appropriate operand.

c)	With the exception of the equality and non-equality operators, the special value null shall not be used as an operand of expressions (see clause 7.1.3).

This means also that all fields and elements of structured types referenced in an expression shall contain completely initialized values, while other fields and elements, not used in the expression, may be uninitialized or contain omit.

Examples

(c_x + c_y – f_increment(c_z))*3 	// single expression

{ a:= 1, b:= true }					// compound expression, assignment notation

{ 1, true }							// compound expression, list notation

## 7.1	Operators

### 7.1.0	General

TTCN-3 supports a number of predefined operators that may be used in the terms of TTCN-3 expressions. The predefined operators fall into seven categories:

a)	arithmetic operators;

b)	list operator;

c)	relational operators;

d)	logical operators;

e)	bitwise operators;

f)	shift operators;

g)	rotate operators.

h)	presence checking operators.

Restrictions

In addition to the general static rules of TTCN-3 given in clause 5, the following restrictions apply:

a)	When an expression is evaluated, the evaluated values used as the operands of operators shall be completely initialized, except for those operands for which it is explicitly allowed to be partially initialized (see clause 11.1).

These operators are listed in table 5.

Table 5: List of TTCN-3 operators

| Category | Operator | Symbol or Keyword |
| --- | --- | --- |
| Arithmetic operators | addition | + |
|  | subtraction | - |
|  | multiplication | * |
|  | division | / |
|  | modulo | mod |
|  | remainder | rem |
| String operators | concatenation | & |
| Relational operators | equal | == |
|  | less than | < |
|  | greater than | > |
|  | not equal | != |
|  | greater than or equal | >= |
|  | less than or equal | <= |
| Logical operators | logical not | not |
|  | logical and | and |
|  | logical or | or |
|  | logical xor | xor |
| Bitwise operators | bitwise not | not4b |
|  | bitwise and | and4b |
|  | bitwise or | or4b |
|  | bitwise xor | xor4b |
| Shift operators | shift left | << |
|  | shift right | >> |
| Rotate operators | rotate left | <@ |
|  | rotate right | @> |
| Presence checking operators | field presence check | ispresent |
|  | chosen alternative check | ischosen |
|  | value check | isvalue |
|  | bound check | isbound |

The precedence of these operators is shown in table 6. Within any row in this table, the listed operators have equal precedence. If more than one operator of equal precedence appears in an expression, the operations are evaluated from left to right. Parentheses may be used to group operands in expressions, in which case a parenthesized expression has the highest precedence for evaluation.

Table 6: Precedence of Operators

| Priority | Operator type | Operator |
| --- | --- | --- |
| highest Lowest | Unary Unary Binary Binary Unary Binary Binary Binary Binary Binary Binary Unary Binary Binary Binary | ( … ) ispresent, ischosen, isvalue, isbound +, - *, /, mod, rem +, -, & not4b and4b xor4b or4b <<, >>, <@, @> <, >, <=, >= ==, != not and xor or |

### 7.1.1	Arithmetic operators

The arithmetic operators represent the operations of addition, subtraction, multiplication, division, modulo and remainder. Operands of these operators shall be of integer values (including derivations of integer) or floating-point numbers (including derivations of float, containing numeric values only), except for mod and rem which shall be used with integer (including derivations of integer) types only.

The usage of the special float values infinity, -infinity and not_a_number in arithmetic operators shall follow the rules defined in IEEE 754 [6].

With integer types, the result type of arithmetic operations is integer. With float types, the result type of arithmetic operations is float.

In the case where plus (+) or minus (-) is used as the unary operator the rules for operands apply as well. The result of using the minus operator is the negative value of the operand if it was positive and vice versa. The result of using the plus operator is the value of the operand, i.e. a positive value if the operand value was positive and a negative value if the operand value was negative.

The result of performing the division operation (/) on two:

a)	integer values gives the whole integer part of the value resulting from dividing the first integer by the second (i.e. fractions are discarded);

b)	numeric float values gives the float value resulting from dividing the first float by the second (i.e. fractions are not discarded).

The operators rem and mod compute on operands of type integer and have a result of type integer. The operations x rem y and x mod y compute the rest that remains from an integer division of x by y. Therefore, they are only defined for non-zero operands y. For positive x and y, both x rem y and x mod y have the same result but for negative arguments they differ.

Formally, mod and rem are defined as follows:

x rem y = x - y * (x/y)

x mod y	= x rem |y|				when	x >= 0

= 0						when	x < 0	and	x rem |y| = 0

= |y| + x rem |y|		when	x < 0	and	x rem |y| < 0

Table 7 illustrates the difference between the mod and rem operator.

Table 7: Effect of mod and rem operator

| x | -3 | -2 | -1 | 0 | 1 | 2 | 3 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| x mod 3 | 0 | 1 | 2 | 0 | 1 | 2 | 0 |
| x rem 3 | 0 | -2 | -1 | 0 | 1 | 2 | 0 |

### 7.1.2	List operator

The predefined list operator (&) performs concatenation of values of string types, record of, set of, or array of the same root types. The operation is a simple concatenation from left to right. No form of arithmetic addition is implied. The result type is the root type of the operands.

NOTE 1:	In case of the list types, both the outer type (i.e. record of, set of or array) and the iterated inner type need to have the same root type in a recursive manner.

NOTE 2:	It is also possible to concatenate two or more value list notation expressions if the result is to be used as a record of or array of the same root type as the concatenated expressions.

Restrictions

In addition to the general static rules of TTCN-3 given in clause 5, the following restrictions apply:

a)	When the list concatenation operator is used for record of-s, set of-s and arrays, its operands shall be at least partially initialized.

EXAMPLE:

'1111'B & '0000'B & '1111'B gives '111100001111'B

{1,2} & {3,4} & {5,6} gives the following record of integer {1,2,3,4,5,6}

### 7.1.3	Relational operators

The predefined relational operators are equality (==), less than (<), greater than (>), non-equality to (!=), greater than or equal to (>=) and less than or equal to (<=). The result type of all these operations is boolean.

The relational operators less than (<), greater than (>), greater than or equal to (>=), and less than or equal to (<=) shall have only operands of type integer (including derivations of integer), float (including derivations of float), or instances of the same enumerated type. It is not allowed to compare instances of different root types.

The address type is allowed for the equality (==) and non-equality (!=) operators, independent of its actual type, but when its actual type differs from the types specified above, it can be compared to the literal special value null only.

Operands of equality (==) and non-equality (!=) shall be completely initialized values or field references, and with the exception of enumerated types, shall be of compatible root types. The values or field references being compared shall obey the following rules. This implies that instances of types not mentioned below shall not be operands of equality and non-equality:

Two field references are equal if the referenced fields are both optional fields and both fields are set to omit or if both referenced fields (regardless if they are optional or not) are initialized with values and these values are equal. A field reference is equal to a value if the referenced field is initialized with a value and both values are equal.

Two integer values are equal if and only if they contain the same value. Otherwise, normal mathematical ordering is applied.

Enumerated values of the same, or different types can be compared. In the case of different enumerated types, expression "b" of type "B" can be compared with expression "a" of type "A" if the two types "A" and "B" can be merged to a consistent larger enumerated type (i.e. where numbers are not associated with different identifiers, see also clause 6.3.2.1). Two enumerated values are equal if and only if both their identifiers and associated integer values (associated either explicitly or implicitly, see clause 6.2.4) are the same.

Two floating-point numbers are equal if and only if they contain the same value. The values minus zero and plus zero are two distinct values (e.g. they are encoded differently in some standardized languages) and minus zero is less than plus zero, which represents zero. Otherwise, normal mathematical ordering is applied. The special values -infinity, infinity and not_a_number are equal to themselves only. The special value -infinity is less than any other float value. The special value infinity is greater than any numerical float values and -infinity. The special value not_a_number is greater than any other float value (including infinity).

Two charstring or two universal charstring values are equal if and only if they have equal lengths and the characters at all positions are the same.

For values of bitstring, hexstring or octetstring types, the same equality rule applies as for charstring values with the exception, that fractions which shall equal at all positions are bits, hexadecimal digits or pairs of hexadecimal digits accordingly.

Two record values, or set values are equal respectively if and only if they are mutually compatible with the type of the other operand (see clause 6.2.15), the actual values of all present fields are equal to their corresponding fields and all fields corresponding to omitted fields are also omitted in the peer value.

Two record of values, set of values or array values, respectively, are equal if and only if they are mutually compatible with the type of the other operand (see clause 6.2.15), they both have the same length, and each element of one value is equal to the corresponding element of the other value. Record of values and array values may also be compared, in which case the corresponding record of type of the array is being considered.

Values of the same union type, and values of different union types in which at least one of the alternatives is compatible with the other type (see clause 6.3.2.4) can be compared (independent if a compatible alternative is the selected one or not). Two values of union types are equal if and only if in both values the name of the selected alternative is identical, they are compatible with the type of the other value, and the actual values of the chosen fields are equal.

Values of the same or any two anytype types can be compared. For anytype values the same rule apply as to union values, with the addition that names of user-defined types defined with the same name in different modules do not denote the same type name of the selected alternatives.

Two default, two port, two timer or two component values are equal if and only if they both are initialized with the special value null or they both contain a reference to the same entity (i.e. they designate the same default, port, timer or test component, independent of the actual state of the denoted object).

It is also possible to use compound expressions (field assignment or value list notation) directly as operands of comparison operations of structured types. If there is a compound expression on both sides of the comparison operator, they shall both be value list notation expressions where the elements shall be of the same root type and they shall be compared like record of values with elements of that root type. If only one operand of the comparison operation is a compound expression it shall be compatible with the root type of the other operand and they shall be compared like values of that root type.

EXAMPLE 1:	Comparing enumerated values

// Given

type enumerated EWeekDays {

Mon, Tue, Wed, Thu, Fri, Sat, Sun

};

type enumerated EWorkDays {

Mon, Tue, Wed, Thu, Fri

};

type enumerated EDesWeekDays {

Tue, Wed, Thu, Fri, Sat, Sun, Mon

};

var EWeekDays v_myWeekDayMon := Mon

var EWeekDays v_myWeekDaySun := Sun

var EWorkDays v_myWorkDayMon := Mon

var EDesWeekDays v_myDesWeekDayMon := Mon

// Then

v_myWeekDayMon ==  v_myWorkDayMon;

// returns true

v_myWeekDaySun == v_myWorkDayMon;

// returns false, because Sun is not a possible value in EworkDays

v_myDesWeekDayMon ==  v_myWeekDayMon;

// returns false: though the identifiers in both enumerated types are the same,

// but the integer values associated with the identifiers are different

EXAMPLE 2:	Comparing values of other structured types

// Given

type	set	S1	{

integer	a1	optional,

integer	a2	optional,

integer	a3	optional

};

type	set	S2	{

integer	b1	optional,

integer	b2	optional,

integer	b3	optional

};

type	set	S3	{

integer	c1	optional,

integer	c2	optional,

};

type	set of integer	SI;

type	union	U1	{

integer	d1,

integer	d2,

};

type	union	U2	{

integer	e1,

integer	e2,

};

type	union	U3	{

integer	d1,

integer	d2,

boolean	d3

};

// And

const	S1	c_s1	:=	{ a1 := 0, a2 := omit, a3 := 2 };

// Notice that the order of defining values of the fields does not matter

const	S2	c_s2a	:=	{ b1 := 0, b3 := 2, b2 := omit };

const	S2	c_s2b	:=	{ b2 := 0, b3 := 2, b1 := omit };

const	S3	c_s3	:=	{ c1 := 0, c2 :=2 };

var		SI	v_si:=	{ 0, -, 2 };

const	SI	c_si	:=	{ 0, 2 };

const	U1	c_u1	:=	{ d1:= 0 };

const	U2	c_u2	:=	{ e1:= 0 };

const	U3	c_u3;	:=	{ d1:= 0 };

// Then

c_s1 == c_s2a;

// returns true

c_s1 == c_s2b;

// returns false, because neither a1 nor a2 are equal to their counterparts

// (the corresponding element is not omitted)

c_s1 == c_s3;

// returns false, because the effective value structures of s1 and s3 are not compatible

c_s1 == v_si;

// causes test case error as v_si is not completely initialized
		// (2nd element is left uninitialized)

c_s1 == c_si;

// returns false, as the counterpart of the omitted a2 is 2,

// but the counterpart of a3 is undefined

c_s3 == c_si;

// returns true

c_u1 == c_u2;

// causes error as U1 and U2 have no common subset of alternatives

c_u1 == c_u3;

// returns true, as alternatives with the same names are chosen and

// the actual values in the selected alternatives are equal

{ 0, omit, 2 } == c_s1;

// returns true

c_s2a == { b1 := 0, b2:= omit, b3 := 2 };

// returns true

{ c_s1, c_s2b } == { c_s2a, c_s1 };

// returns false because c_s2b != c_s1

{ c_s1, c_s2b, c_s2a } == { c_s1 };

// returns false because of different length

c_s1.a1 == c_s2a.b1;

// returns true, both fields are initialized with values and the values are equal

c_s1.a2 == c_s2a.b2;

// returns true, both fields are omit

c_s1.a1 == c_s2a.b2;

// returns false, value vs. omit

c_s1.a1 == omit;

// error, omit is neither a value nor a field reference

c_s1.a2 == 3;

// false, omit vs. value

### 7.1.4	Logical operators

The predefined boolean operators perform the operations of negation, logical and, logical or and logical xor. Their operands shall be of root type boolean. The result type of logical operations is boolean.

The logical not is the unary operator that returns the value true if its operand was of value false and returns the value false if the operand was of value true.

The logical and returns the value true if both its operands are true; otherwise it returns the value false.

The logical or returns the value true if at least one of its operands is true; it returns the value false only if both operands are false.

The logical xor returns the value true if one of its operands is true; it returns the value false if both operands are false or if both operands are true.

Short circuit evaluation for boolean expressions is used, i.e. the evaluation of operands of logical operators is stopped once the overall result is known: in the case of the and operator, if the left argument evaluates to false, then the right argument is not evaluated and the whole expression evaluates to false. In the case of the or operator, if the left argument evaluates to true, then the right argument is not evaluated and the whole expression evaluates to true.

### 7.1.5	Bitwise operators

The predefined bitwise operators perform the operations of bitwise not, bitwise and, bitwise or and bitwise xor. These operators are known as not4b, and4b, or4b and xor4b respectively.

NOTE:	To be read as "not for bit", "and for bit", etc.

Their operands shall be of root type bitstring, hexstring or octetstring. In the case of and4b, or4b and xor4b the operands shall be of the same root types. The result type of the bitwise operators shall be the root type of the operands.

The bitwise not4b unary operator inverts the individual bit values of its operand. For each bit in the operand a 1 bit is set to 0 and a 0 bit is set to 1. That is:

not4b '1'B  gives  '0'B

not4b '0'B  gives  '1'B

EXAMPLE 1:

not4b '1010'B  gives  '0101'B

not4b '1A5'H  gives 'E5A'H

not4b '01A5'O  gives 'FE5A'O

The bitwise and4b operator accepts two operands of equal length. For each corresponding bit position, the resulting value is a 1 if both bits are set to 1, otherwise the value for the resulting bit is 0. That is:

'1'B and4b '1'B  gives  '1'B

'1'B and4b '0'B  gives  '0'B

'0'B and4b '1'B  gives  '0'B

'0'B and4b '0'B  gives  '0'B

EXAMPLE 2:

'1001'B and4b '0101'B  gives '0001'B

'B'H and4b '5'H  gives  '1'H

'FB'O and4b '15'O  gives  '11'O

The bitwise or4b operator accepts two operands of equal length. For each corresponding bit position, the resulting value is 0 if both bits are set to 0, otherwise the value for the resulting bit is 1. That is:

'1'B or4b '1'B gives  '1'B

'1'B or4b '0'B gives  '1'B

'0'B or4b '1'B gives  '1'B

'0'B or4b '0'B gives  '0'B

EXAMPLE 3:

'1001'B or4b '0101'B  gives '1101'B

'9'H or4b '5'H gives  'D'H

'A9'O or4b 'F5'O gives  'FD'O

The bitwise xor4b operator accepts two operands of equal length. For each corresponding bit position, the resulting value is 0 if both bits are set to 0 or if both bits are set to 1, otherwise the value for the resulting bit is 1. That is:

'1'B xor4b '1'B gives  '0'B

'0'B xor4b '0'B  gives  '0'B

'0'B xor4b '1'B  gives  '1'B

'1'B xor4b '0'B  gives  '1'B

EXAMPLE 4:

'1001'B xor4b '0101'B  gives '1100'B

'9'H xor4b '5'H  gives 'C'H

'39'O xor4b '15'O  gives '2C'O

### 7.1.6	Shift operators

The predefined shift operators perform the shift left (<<) and shift right (>>)operations. Their left-hand operand shall be of root type bitstring, hexstring or octetstring. Their right-hand operand shall be a non-negative integer. The result type of these operators shall be the same as the root type of the left operand.

The shift operators behave differently based upon the type of their left-hand operand. If the type of the left-hand operand is:

a)	bitstring then the shift unit applied is 1 bit;

b)	hexstring then the shift unit applied is 1 hexadecimal digit;

c)	octetstring then the shift unit applied is 1 octet.

The shift left (<<) operator accepts two operands. It shifts the left-hand operand by the number of shift units to the left as specified by the right-hand operand. Excess shift units (bits, hexadecimal digits or octets) are discarded. For each shift unit shifted to the left, a zero ('0'B, '0'H, or '00'O determined according to the type of the left-hand operand) is inserted from the right-hand side of the left operand.

EXAMPLE 1:

'111001'B << 2  gives  '100100'B

'12345'H << 2  gives  '34500'H

'1122334455'O << (1+1)  gives  '3344550000'O

The shift right (>>)operator accepts two operands. It shifts the left-hand operand by the number of shift units to the right as specified by the right-hand operand. Excess shift units (bits, hexadecimal digits or octets) are discarded. For each shift unit shifted to the right, a zero ('0'B, '0'H, or '00'O determined according to the type of the left-hand operand) is inserted from the left-hand side of the left operand.

EXAMPLE 2:

'111001'B >> 2  gives  '001110'B

'12345'H >> 2  gives  '00123'H

'1122334455'O >> (1+1)  gives  '0000112233'O

### 7.1.7	Rotate operators

The predefined rotate operators perform the rotate left (<@) and rotate right (@>) operators. Their left-hand operand shall be of root type bitstring, hexstring, octetstring, charstring, universal charstring, record of, or set of. Their right-hand operand shall be a non-negative integer. The result type of these operators shall be the same as the root type of the left-hand operand.

NOTE 1:	Please note that the root types of arrays is record of, therefore arrays are allowed as left-hand operands of rotate operators.

The rotate operators behave differently based upon the type of their left-hand operand. If the type of the left-hand operand is:

a)	bitstring then the rotate unit applied is 1 bit;

b)	hexstring then the rotate unit applied is 1 hexadecimal digit;

c)	octetstring then the rotate unit applied is 1 octet;

d)	charstring or universal charstring then the rotate unit applied is one character;

e)	record of, set of, or array then the rotate unit applied is one element.

The rotate left (<@) operator accepts two operands. It rotates the left-hand operand by the number of shift units to the left as specified by the right-hand operand. Excess shift units (bits, hexadecimal digits, octets, characters, or elements) are re-inserted into the left-hand operand from its right-hand side.

Restrictions

In addition to the general static rules of TTCN-3 given in clause 5, the following restrictions apply:

a)	When the rotate operator is used for record of-s, set of-s and arrays, its left hand operand shall be at least partially initialized.

NOTE 2:	Please note that for the right hand operand restriction a) in clause 7 further on applies.

EXAMPLE 1:

'101001'B <@ 2  gives  '100110'B

'12345'H <@ 2  gives  '34512'H

'1122334455'O <@ (1+2)  gives  '4455112233'O

"abcdefg" <@ 3  gives "defgabc"

The rotate right (@>) operator accepts two operands. It rotates the left-hand operand by the number of shift units to the right as specified by the right-hand operand. Excess shift units (bits, hexadecimal digits, octets, characters, or elements) are re-inserted into the left-hand operand from its left-hand side.

EXAMPLE 2:

'100001'B @> 2  gives  '011000'B

'12345'H @> 2  gives  '45123'H

'1122334455'O @> (1+2)  gives  '3344551122'O

"abcdefg" @> 3  gives "efgabcd"

### 7.1.8	Presence checking operators

#### 7.1.8.0	General

The presence checking operators include ispresent, ischosen, isvalue and isbound operators. The result type of the presence checking operators is boolean. The operand of these operators can be of any type.

Specific rules apply if the operand of a presence checking operator is a data object reference, function instance, template instance or a result of the valueof operation followed by an ExtendedFieldReference. In these cases, if resolving dot notation, index notation (see clause 7.2) or decoded field references (see clause 7.3) included in the ExtendedFieldReference would produce an error (e.g. if an index is out of range, when referencing a sub-field of an omitted field etc.), the following happens:

No error is produced.

Evaluation of all remaining unresolved fields in the ExtendedFieldReference is stopped. All remaining parts of the ExtendedFieldReference that are located right from the operation that would normally produce an error up to the end of the presence checking operator are not evaluated.

The presence checking operator yields the value false.

The rule on special handling of dot notation, index notation and decoded field references is not applied recursively. Errors occurring during resolving a value of an index inside index notation or a value of an encoding format parameter of a decoded field reference are not affected by this rule.

#### 7.1.8.1	The ispresent operator

The ispresent operator checks whether a referenced field is present in a data object.

Syntactical Structure

ispresent "(" TemplateInstance ")"

Semantic Description

The ispresent operator returns:

the value true if the operand fulfils the (present) template restriction as described in clause 15.8;

the value false otherwise.

NOTE 1:	When the argument of ispresent is a subfield of a template field to which the "?" (AnyValue) matching is assigned, the extension mechanism specified in clause 15.6.2 applies.

NOTE 2:	This means that whenever ispresent(m_myTemplate) returns true:

m_myTemplate can safely be assigned to a non-optional field of the type of the template in a template variable;

m_myTemplate can safely be used as an actual template(present) parameter or assigned to a variable of kind template(present).

Restrictions

General static rules of TTCN-3 given in clause 5 apply.

Examples

EXAMPLE:

// Given

type record MyRecord

{

record {

boolean innerField1 optional,

integer innerField2 optional,

MyRecord innerField3 optional

} field1 optional,

integer field2

}

var MyRecord v_myRecord := { field1 := {}, field2 := 5 }

// type of field1 is record with fields, therefore field1 remains uninitialized

// after this assignment (no value is assigned to any of the fields of vl_MyRecord.field1)

var boolean v_checkResult := ispresent(v_myRecord.field1) // yields false

v_myRecord.field1 := omit

v_checkResult := ispresent(v_myRecord.field1) // yields false

// and therefore, v_myRecord.field1.innerField1 is an inaccessible reference

v_checkResult := ispresent(v_myRecord.field1.innerField3.field2) // yields false because

// innerField3 is unintialized and therefore, v_myRecord.field1.innerField3.field2 is an

// inaccessible reference

v_checkResult := ispresent(v_myRecord.field1.innerField1) // yields false because field1
 // is omitted

var template MyRecord v_myRecordT := { field1 := ?, field2 := 5 }

v_checkResult := ispresent(v_myRecordT.field1) // yields true

v_checkResult := ispresent(v_myRecordT.field1.innerField1) // yields false because field1 is
 // AnyValue (pls. note, that at expansion of field1 the optional field innerField1 obtains "*"

// that can match both a present and an omitted field

type record R { integer f1 optional, integer f2 optional }

template R mw_t1 := {f1 := 1, f2 :=(2 .. 4) }

template R mw_t2 := { f1 := omit, f2 := (5, 7) ifpresent }

template R mw_t3 := {f1 := *, f2 :=? }

v_checkResult := ispresent(mw_t1.f1) // yields true

v_checkResult := ispresent(mw_t1.f2) // yields true

v_checkResult := ispresent(mw_t2.f1) // yields false

v_checkResult := ispresent(mw_t2.f2) // yields false

v_checkResult := ispresent(mw_t3.f1) // yields false

v_checkResult := ispresent(mw_t3.f2) // yields true

#### 7.1.8.2	The ischosen operator

The ischosen operator checks whether a referenced alternative is present in a data object.

Syntactical Structure

ischosen "(" TemplateInstance ")"

Semantic Description

This operator is allowed for data objects of all data types that are a union-field-reference or a type alternative of an anytype. The operator returns:

the value true if and only if the data object reference specifies the variant of the union type or the type alternative of the anytype that is actually selected for the given data object;

in all other cases false.

The ischosen operator is applicable to data objects of union types or of anytype containing a specific value or a value list. In case of a value list, the operator returns true if all data objects present in the value list have the given alternative selected. The result is false if there is another alternative of the union type or anytype on which ischosen would return true.

NOTE:	Please note that in case of anytype-s, no type compatibility is considered when determining the selected alternative; i.e. at the evaluation only the exact type chosen for the given value will satisfy the above criteria.

Restrictions

General static rules of TTCN-3 given in clause 5 apply.

Examples

EXAMPLE 1:	Using ischosen for union types

// inside module M:

type union U { integer f1, octetstring f2 }

template U m_u1 := {f1 := 1}

template U mw_u2 := {f2 := ?}

template U mw_u3 := ?

template U m_u4 := ({ f1 := 2 }, {f2 := 'AB'O })

template U mw_u5 := ({ f2 := '12?'O }, { f2 := '*34'O length(2) })

var boolean v_checkResult := ischosen(m_u1.f1) // returns true

v_checkResult := ischosen(m_u1.f2) // yields false

v_checkResult := ischosen(mw_u2.f1) // yields false

v_checkResult := ischosen(mw_u2.f2) // yields true

v_checkResult := ischosen(mw_u3.f1) // yields false

v_checkResult := ischosen(mw_u3.f2) // yields false

v_checkResult := ischosen(m_u4.f1) // yields false

v_checkResult := ischosen(m_u4.f2) // yields false

v_checkResult := ischosen(mw_u5.f1) // yields false

v_checkResult := ischosen(mw_u5.f2) // yields true

type record R { U u optional }

template R m_r1 := { omit }

var boolean v_checkResult := ischosen(m_r1.u.f1) // yields false

EXAMPLE 2:	Using ischosen for anytype

template anytype mw_a1 := { U := mw_u5 }

template anytype mw_a2 := { M.U := { f1 := m_u1.f1 } }

var boolean v_checkResult := ischosen(mw_a1.U) 		 // yields true

v_checkResult := ischosen(mw_a1.M.U) 	 // yields true

v_checkResult := ischosen(mw_a1.integer)  // yields false

v_checkResult := ischosen(mw_a2.U)        // yields true

EXAMPLE 3:

// Given

type union MyUnion

{	PDU_type1	p1,

PDU_type2	p2,

PDU_type	p3

}

// and given that mw_myPDU is a template of MyUnion type

// and v_receivedPDU is also of MyUnion type

// then

myPort.receive(mw_myPDU) -> value v_receivedPDU

var boolean v_checkResult := ischosen(v_receivedPDU.p2)

// yields true if the actual instance of mw_myPDU carries a PDU of the type PDU_type2

#### 7.1.8.3	The isvalue operator

The isvalue operator checks whether a referenced data object is a specific value.

Syntactical Structure

isvalue "(" TemplateInstance ")"

Semantic Description

The isvalue operator yields true, if the operand is completely initialized and resolves to a specific value. If the operand is of record or set type, omitted optional fields shall be considered as initialized, i.e. the operator shall also yield true if optional fields of the operand are set to omit. The operator shall yield false otherwise.

The specific value null is considered as concrete value.

Restrictions

General static rules of TTCN-3 given in clause 5 apply.

Examples

EXAMPLE 1:	Simple types:

template charstring m_char0 := "ABCD";  //template containing a specific value matching

template charstring m_char1 := "AB?D";  //template containing a specific value matching

//note, that "?" is not a matching symbol in this case

template charstring mw_char2 := pattern "ABCD"; // a pattern matching a single value only

template charstring mw_char3 := pattern "AB?D"; // pattern matching

template charstring m_char4 := ("ABCD"); // template containing a specific value (expression)

template charstring mw_char5 := ("ABCD","EFGH"); // a value list matching a single value only

var boolean v_checkResult := isvalue(m_char0);  // yields true

v_checkResult := isvalue(m_char1);  // yields true

v_checkResult := isvalue(mw_char2); // yields false

v_checkResult := isvalue(mw_char3); // yields false

v_checkResult := isvalue(m_char4);  // yields true

v_checkResult := isvalue(mw_char5); // yields false

EXAMPLE 2:	Special types:

var default v_default := null;

var boolean v_checkResult := isvalue(v_default); // yields true

EXAMPLE 3:	Record/set types:

type record MyRec {

integer f1 optional,

integer f2 optional

}

var MyRec v_myRec;

var template MyRec v_myRecT;

var boolean_checkResult := isvalue(v_myRec); 	   // yields false

v_checkResult := isvalue(v_myRecT);    // yields false

v_myRec   := { f1 := 5, f2 := omit }

v_myRecT  := { f1 := ?, f2 := 5 }

v_checkResult := isvalue(v_myRec); 	   // yields true

v_checkResult := isvalue(v_myRec.f2);  // yields false;

v_checkResult := isvalue(v_myRecT);    // yields false

v_checkResult := isvalue(v_myRecT.f1); // yields false

v_checkResult := isvalue(v_myRecT.f2); // yields true

v_myRecT.f2 := omit;

v_checkResult := isvalue(v_myRecT.f2); // yields false

EXAMPLE 4:	Union types:

type union MyUnion {

integer ch1,

integer ch2

}

template MyUnion m_myUnion := { ch1 := 5 }

template MyUnion mw_myUnion := { ch1 := ? }

var boolean v_checkResult := isvalue(m_myUnion);      // yields true

v_checkResult := isvalue(mw_myUnion);     // yields false

v_checkResult := isvalue(mw_myUnion.ch1); // yields false

// note, this is different from ischosen(mw_myUnion.ch1) as isvalue checks the content of the

// choice ch1, while ischosen is checking if ch1 has been selected or not

v_checkResult := isvalue(mw_myUnion.ch2); // yields false

EXAMPLE 5:	Nested types:

type record MyRecord {

MyUnion u optional

}

template MyRecord m_myRecord  := { u := m_myUnion }

template MyRecord mw_myRecord := { u := mw_myUnion }

template MyRecord m_myRecord2 := { u := omit }

var boolean v_checkResult := isvalue(m_myRecord.u.ch1);  // yields true

v_checkResult := isvalue(mw_myRecord.u.ch1); // yields false

v_checkResult := isvalue(mw_myRecord.u.ch2); // yields false

v_checkResult := isvalue(m_myRecord.u.ch2);  // yields false

#### 7.1.8.4	The isbound operator

The isbound operator checks whether a referenced data object is at least partially initialized.

Syntactical Structure

isbound "(" TemplateInstance ")"

Semantic Description

The isbound operator yields true, if the operand is at least partially initialized. If the operand is of a record or set type, omitted optional fields shall be considered as initialized, i.e. the operator shall also yield true if at least one optional field of the operand is set to omit. The operator shall yield false otherwise. Inaccessible fields (e.g. non-selected alternatives of union types, subfields of omitted record and set types or subfields of non-selected union fields) shall be considered as uninitialized, i.e. isbound shall yield for them false.

The specific value null is considered as concrete value.

Examples

EXAMPLE 1:	Simple types

var template charstring v_char;

var boolean_checkResult := isbound(v_char); // yields false as v_char is uninitialized

v_char := "AB?D";                           // template containing a specific value

checkResult := isbound(v_char);             // yields true

v_char := pattern "AB?D";                   // template containing a pattern matching

checkResult := isbound(v_char);             // yields true

EXAMPLE 2:	Special types

var default v_default := null;

var boolean_checkResult := isbound(v_default); // yields true

EXAMPLE 3:	Record/set types

type record MyRec {

integer f1,

MyRec f2 optional

}

var MyRec v_myRec;

var boolean_checkResult := isbound(v_myRec); // yields false

v_myRec.f2 := omit;

checkResult := isbound(v_myRec);           // yields true as v_myRec is partially initialized,

// field f2 is set to omit

v_myRec  := { f1 := 5, f2 := omit }

checkResult := isbound(v_myRec);           // yields true as v_myRec is completely initialized

checkResult := isbound(v_myRec.f2.f1);     // yields false as v_myRec.f2.f1 is inaccessible

checkResult := isbound(v_myRec.f1/0);      // shall cause an error already during evaluating the

// operand because the operand is not a reference and division by

// zero causes a runtime error

type union MyUnion {

integer ch1,

MyRec   ch2

}

var template MyUnion v_myUnion;

checkResult := isbound(v_myUnion);        // yields false, as v_myUnion is uninitialized

checkResult := isbound(v_myUnion.ch1);    // yields false, as alternative ch1 is uninitialized

v_myUnion := { ch1 := 5 };

checkResult := isbound(v_myUnion);        // yields true

checkResult := isbound(v_myUnion.ch1);    // yields true

checkResult := isbound(v_myUnion.ch2);    // yields false as the ch2 alternative is not selected

checkResult := isbound(v_myUnion.ch2.f1); // yields false as the field f1 is inaccessible

checkResult := isbound(v_myUnion.ch1/0);  // shall cause an error already during evaluating the

// operand because the operand is not a reference and division by

// zero causes a runtime error

## 7.2	Field references and list elements

Within expressions, fields of record and set types are referenced with the dot notation ".field". Elements of record of, set of, array and string types are referenced with the index notation "[index]". Dot and brackets have the same binding power. Field references and list elements are evaluated from left to right.

## 7.3	Decoded field reference

Decoded field reference is a specific notation that can be applied to expressions of bitstring, hexstring, octetstring, charstring or universal charstring types. It is used for accessing content of implicitly decoded payload fields.

Syntactical Structure

ReferencedValue "=>" ( PredefinedType | TypeIdentifier |

( "(" Type [ "," Expression ] ")" ) )

The string value preceding the => operator shall be decoded into a value of the type following the => operator. Failure of this decoding shall cause a test case error. In case the string operand is of the universal charstring type and the extended syntax with parentheses is used, the type operand can be followed by an optional parameter defining the encoding format. The parameter shall be of the charstring type and it shall contain one of the strings allowed for the decvalue_unichar function (specified in clause C.5.4). Any other value shall cause an error. In case the string operand is not a universal charstring, the optional parameter shall not be present.

Restrictions

In addition to the general static rules of TTCN-3 given in clause 5, the following restrictions apply:

a)	If the type operand is not enclosed into parentheses, only a built-in type or a type reference consisting of a single identifier can be used. Extended type references shall always use the extended syntax with parentheses.

EXAMPLE:

type record PDU {

PduHeader header,

bitstring outerPayload

}

type record OuterPayload {

OuterPayloadHeader header,

universal charstring innerPayload

}

type record InnerPayload {

integer data1,

charstring data2

}

...

var PDU v_pdu;

var InnerPayload v_inner;

... // v_pdu is filled with data;

v_inner := v_pdu.outerPayload=>OuterPayload.innerPayload=>(InnerPayload, "UTF-8");

// v_pdu.outerPayload field is first decoded into a value of the OuterPayload type

// the innerPayload field of the decoding result is subsequently decoded into a value

// of the InnerPayload type (using UTF-8 format for conversion into a bitstring)

# 8	Modules

## 8.0	General

The principal building blocks of TTCN-3 are modules. A module may define a fully executable test suite or just a library. A module may refer to the TTCN-3 language version and to package versions being used. A module contains definitions.

NOTE:	The term test suite is synonymous with a complete set of TTCN-3 modules containing test cases and a control function.

The transfer syntax of TTCN-3 modules shall be UTF-8, i.e. each character of the module shall be individually encoded and decoded according to the UCS Transformation Format 8 (UTF-8) as defined in annex R of ISO/IEC 10646 [2] and no characters not corresponding to any character of the module shall be present.

## 8.1	Definition of a module

A module is defined with the keyword module.

NOTE 1:	The treatment of TTCN-3 modules in files, repositories and alike is outside the scope of the present document.

Syntactical Structure

module ModuleIdentifier [ language FreeText { "," FreeText } ] "{"

[ ModuleDefinitionsPart ]

"}"

Semantic Description

A TTCN-3 module groups a set of (typically cohesive) TTCN-3 definitions. TTCN-3 modules have an explicit import interface to use definitions from other TTCN-3 or non-TTCN-3 modules. It is possible to hide definitions in a TTCN-3 module (see clause 8.2.5). TTCN-3 modules can be compiled/interpreted separately. They are reusable and parameterizable.

Module names are of the form of a TTCN-3 identifier.

NOTE 2:	The module identifier is the informal text name of the module.

In addition, a module specification can carry an optional attribute identified by the language keyword that identifies the edition of the TTCN-3 language, in which the module is specified. The following language strings are to be used:

"TTCN-3:2001" - to be used with modules complying with V1.1.2 [Fehler! Verweisquelle konnte nicht gefunden werden.] of the present document.
	"TTCN-3:2003" - to be used with modules complying with V2.2.1 [Fehler! Verweisquelle konnte nicht gefunden werden.] of the present document.
	"TTCN-3:2005" - to be used with modules complying with V3.1.1 [Fehler! Verweisquelle konnte nicht gefunden werden.] of the present document.
	"TTCN-3:2007" - to be used with modules complying with V3.2.1 [Fehler! Verweisquelle konnte nicht gefunden werden.] of the present document.
	"TTCN-3:2008" - to be used with modules complying with V3.3.2 [Fehler! Verweisquelle konnte nicht gefunden werden.] of the present document.
	"TTCN-3:2008 Amendment 1" - to be used with modules complying with V3.4.1 [Fehler! Verweisquelle konnte nicht gefunden werden.] of the present document.
	"TTCN-3:2009" - to be used with modules complying with V4.1.1 [Fehler! Verweisquelle konnte nicht gefunden werden.] of the present document.
	"TTCN-3:2010" - to be used with modules complying with V4.2.1 [Fehler! Verweisquelle konnte nicht gefunden werden.] of the present document.
	"TTCN-3:2011" - to be used with modules complying with V4.3.1 [Fehler! Verweisquelle konnte nicht gefunden werden.] of the present document.
	"TTCN-3:2012" - to be used with modules complying with V4.4.1 [Fehler! Verweisquelle konnte nicht gefunden werden.] of the present document.
	"TTCN-3:2013" - to be used with modules complying with V4.5.1 [Fehler! Verweisquelle konnte nicht gefunden werden.] of the present document.
	"TTCN-3:2014" - to be used with modules complying with V4.6.1 [Fehler! Verweisquelle konnte nicht gefunden werden.] of the present document.
	"TTCN-3:2015" - to be used with modules complying with V4.7.1 [Fehler! Verweisquelle konnte nicht gefunden werden.] of the present document.
	"TTCN-3:2016" - to be used with modules complying with V4.8.1 [Fehler! Verweisquelle konnte nicht gefunden werden.] of the present document.
	"TTCN-3:2017" - to be used with modules complying with V4.9.1 [Fehler! Verweisquelle konnte nicht gefunden werden.] of the present document.
	"TTCN-3:2018" - to be used with modules complying with the present document.

Furthermore, the optional attribute identified by the language keyword may identify package versions being used by this module. The package tags are defined in ETSI ES 202 781 [i.11], ETSI ES 202 782 [i.14], ETSI ES 202 784 [i.12], and ETSI ES 202 785 [i.13]. The language identifier and the package identifier are to be written as a comma-separated list.

Restrictions

In addition to the general static rules of TTCN-3 given in clause 5, the following restrictions apply:

- At most one language string per module shall be given to define the core language version in which the module is defined.

- Per extension package, at most one extension package string of that extension package shall be used by a module.

Examples

module MyTestSuite language "TTCN-3:2003"

{ … }

## 8.2	Module definitions part

### 8.2.0	General

The module definitions part specifies the top-level definitions of the module and may import visible identifiers from other modules. Visibility rules are given in clause 8.2.5. Scope rules for declarations made in the module definitions part and imported declarations are given in clause 5.3. Those language elements which may be defined in a TTCN-3 module are listed in table 1. Every definition can be associated with attributes using the with statement defined in clause 27. Visible module definitions may be imported by other modules.

Syntactical Structure

{

[ Visibility ] (

TypeDef |

ConstDef |

TemplateDef |

ModuleParDef |

FunctionDef |

SignatureDef |

TestcaseDef |

AltstepDef |

ImportDef |

GroupDef |

ExtFunctionDef |

FriendDef |

ModuleControlDef

) [ WithStatement ]

[ ";" ]

}+

Semantic Description

Definitions in the module definitions part may be made in any order.

Module definitions can be evaluated at runtime and can be evaluated in any order. A definition shall be evaluated latest before the first reference to it.

NOTE:	If a definition is not used, it may not be evaluated at all.

Such definitions, i.e. top-level definitions outside of other scope units, are globally visible within the module. They may be used elsewhere in the module. This includes identifiers imported from other modules.

Declarations of dynamic language elements such as variables or timers shall only be made in test cases, functions, altsteps or component types.

TTCN-3 does not support the declaration of variables in the module definitions part, i.e. global variables cannot be defined in TTCN-3. However, variables defined in a test component type may be used by all test cases, functions, etc. running on components of that component type.

Restrictions

No specific restrictions in addition to the general static rules of TTCN-3 given in clause 5.

Examples

module MyModule

{	// This module contains definitions only

:

const integer MyConstant := 1;

type record MyMessageType { … }

:

function TestStep(){ … }

:

}

### 8.2.1	Module parameters

Module parameters are values or templates that may be supplied by the test environment at runtime. Module parameters do not change during test execution. They can be used on the right hand side of assignments, in expressions, in actual parameters, and in template definitions, but not within type definitions.

Syntactical Structure

[ Visibility ] modulepar [ TemplateModifier ]

Type

{ Identifier [ ":=" TemplateBody ] "," }

Identifier [ ":=" TemplateBody ] ";"

Semantic Description

Module parameters behave as global constants or unparameterized templates at runtime. For module parameterization, TTCN-3 only supports value parameterization which has to be resolved static at start of runtime.

Module parameters allow to customize a TTCN-3 test suite for a specific IUT, test setup or test campaign.

It is allowed to specify a default value or template for each module parameter as part of the modulepar declaration.

If the test system does not provide an actual runtime value or template for a module parameter, the given default value or template shall be used during test execution, otherwise the actual value or template provided by the test system.

If functions are used for the initialization of module parameters, it is strongly advised to adhere to the rules defined in clause 16.1.4. Not following these rules may cause non-deterministic test executions.

Visible module parameters can be imported.

Optional fields of record and set module parameters or module parameter fields can be initialized explicitly or implicitly. For implicit initialization of the optional fields of a module parameter or a module parameter field, an optional attribute with the value "implicit omit" (see clause 27.7) shall be associated with it either directly or via the attribute distribution (scoping) mechanism (see clause 27.1.1).

Restrictions

In addition to the general static rules of TTCN-3 given in clause 5, the following restrictions apply:

a)	During test execution module parameters shall not be used as target of assignments or as actual out or inout parameters.

b)	Module parameters shall not be of port, default, timer or component type and shall not be of a structured type that contains a sub-element of port or timer type at any level of nesting.

c)	A module parameter shall only be of type address if the address type is explicitly defined within the associated module.

d)	Module parameters shall be declared within the module definition part only.

e)	More than one occurrence of module parameters declaration is allowed but each parameter shall be declared only once (i.e. redefinition of the module parameter is not allowed).

f)	The TemplateBody of a module parameter shall respect the limitations given in clause 16.1.4. and shall be compatible with the Type of the declaration and conform to the given TemplateModifier. If no TemplateModifier is present, the TemplateBody shall resolve to a value.

g)	Module parameters shall not be used in type or array definitions.

h)	All sub-elements of component or default type of a default value of a module parameter shall be initialized with the special value null.

Examples

module MyTestSuiteWithParameters

{

// single type, single module parameter, which is per default public

modulepar boolean PX_Par0 := true;

// single type, multiple module parameters with an explicit public visibility

public modulepar integer PX_Par1, PX_Par2 := 1 + char2int("a");

...

}

### 8.2.2	Groups of definitions

In the module definitions part, definitions can be collected in named groups. Grouping is done to aid readability and to add logical structure to the module if required. If necessary, the dot notation shall be used to identify sub-groups within the group hierarchy uniquely, e.g. for the import of a specific sub-group.

Syntactical Structure

[ public ] group GroupIdentifier "{"

{ ModuleDefinition [ ";" ] }

"}"

Semantic Description

A group of definitions can be specified wherever a single definition is allowed. Groups may be nested, i.e. groups may contain other groups. This allows the test suite specifier to structure, among other things, collections of test data or functions describing test behaviour.

Groups and nested groups have no scoping. Please note however, attributes given to a group by an associated with statement apply to all elements of a group (see clause 27). Import statements may import groups so that all visible elements of a group are imported (see clause 8.2.3.3).

Restrictions

In addition to the general static rules of TTCN-3 given in clause 5, the following restrictions apply:

- Group identifiers across the whole module need not necessarily be unique. However, top-level group identifiers and all group identifiers of subgroups of a single group shall be unique.

- Only public visibility can be defined for groups as they are always public.

Examples

module MyModule {

:

// A collection of definitions

group myGroup {

const integer c_myConst:= 1;

:

type record MyMessageType { … };

group myGroup1 {	// Sub-group with definitions

type record AnotherMessageType { … };

const boolean c_myBoolean := false

}

}

// A group of altsteps

group myStepLibrary {

group myGroup1 {	// Sub-group with the same name as the sub-group with definitions

altstep a_myStep11() { … }

altstep a_myStep12() { … }

:

altstep a_myStep1n() { … }

}

group myGroup2 {

altstep a_myStep21() { … }

altstep a_myStep22() { … }

:

altstep a_myStep2n() { … }

}

}

:

}

// An import statement that imports myGroup1 within myStepLibrary

import from MyModule {

group myStepLibrary.myGroup1

}

### 8.2.3	Importing from modules

#### 8.2.3.0	General

It is possible to re-use visible definitions specified in different modules using the import statement. Every definition in a TTCN-3 module has an associated visibility, which is by default public (see clause 8.2.5).

NOTE:	Groups are public only. Importing a group means that only the visible elements of the group are being imported.

#### 8.2.3.1	General format of import

An import statement can be used anywhere in the module definitions part.

Syntactical Structure

[ Visibility ] import from ModuleId  [-> LocalModuleName]

(

( all [ except "{" ExceptSpec "}" ] )

|

( "{" ImportSpec "}" )

)

[ ";" ]

Semantic Description

TTCN-3 supports the import of the following definitions: module parameters, user defined types, signatures, constants, data templates, signature templates, functions, external functions, altsteps and test cases. Each definition has a name (defines the identifier of the definition, e.g. a function name), a specification (e.g. a type specification or a signature of a function) and in the case of functions, altsteps and test cases an associated behaviour description. In addition, import statements of one module can be explicitly imported by another module (see clause 8.2.3.7). Only definitions or import statements visible from the importing module can be imported (see clause 8.2.5).

In contrast to module definitions, which are by default public, import statements are by default private.

EXAMPLE 1a:

|  | Name | Specification | Behaviour description |
| --- | --- | --- | --- |
| function | f_myFunction | (inout MyType1 p_myPar) return MyType2 runs on MyCompType | { const MyType3 c_myConst := …; : // further behaviour } |

|  | Specification | Name | Specification |
| --- | --- | --- | --- |
| type | record | MyRecordType | { MyType4 field1, integer field2 } |

|  | Specification | Name | Specification |
| --- | --- | --- | --- |
| template | MyType5 | m_myTemplate | := { field1 := 1, field2 := c_myConst, // c_myConst is a module constant field3 := PX_ModulePar // PX_ModulePar is module parameter } |

Behaviour descriptions have no effect on the import mechanism, because their internals are considered to be invisible to the importer when the corresponding functions, altsteps or test cases are imported. Thus, they are not considered in the following descriptions.

The specification part of an importable definition contains local definitions (e.g. field names of structured type definitions or values of enumerated types) and referenced definitions (e.g. references to type definitions, templates, constants or module parameters). For the examples above, this means:

|  | Name | Local definitions | Referenced definitions |
| --- | --- | --- | --- |
| function | f_myFunction | p_myPar | MyType1, MyType2, MyCompType |
| type | MyRecordType | field1, field2 | MyType4, integer |
| template | m_myTemplate |  | MyType5, field1, field2, field3, c_myConst, PX_ModulePar |

NOTE 1:	The local definitions column refers to identifiers only that are newly defined in the importable definition. Values assigned to individual fields of importable definitions, e.g. in template definitions, may also be considered as local definitions, but they are not important for the explanation of the import mechanism.

NOTE 2:	The referenced definitions field1, field2 and field3 of template MyTemplate are the field names of MyType5, i.e. they are referenced via MyType5.

Referenced definitions are also importable definitions, i.e. the source of a referenced definition can again be structured into a name and a specification part and the specification part also contains local and referenced definitions. In other words, an importable definition may be built up recursively from other importable definitions.

The TTCN-3 import mechanism is related to the local and referenced definitions used in the specification part of the importable definitions. Table 8 specifies the possible local and referenced definitions of importable definitions.

Table 8: Possible local and referenced definitions of importable definitions

| Importable Definition | Possible Local Definitions | Possible Referenced Definitions |
| --- | --- | --- |
| Module parameter |  | Module parameter type |
| User-defined type (for all) |  |  |
| - enumerated type | Concrete values |  |
| - structured type | Field names, nested type definitions | Field types |
| - port type |  | Message types, signatures |
| - component type | Constant names, variable names, timer names and port names | Constant types, variable types, port types |
| Signature | Parameter names | Parameter types, return type, types of exceptions |
| Constant |  | Constant type |
| Data Template | Parameter names | Template type, parameter types, constants, module parameters, functions |
| Signature template |  | Signature definition, constants, module parameters functions |
| Function | Parameter names | Parameter types, return type, component type (runs on clause) |
| External function | Parameter names | Parameter types, return type |
| Altstep | Parameter names | Parameter types, component type (runs on clause) |
| Test case | Parameter names | Parameter types, component types (runs on- and system clause) |
| NOTE 1: For the import of import statements see clause 8.2.3.7. NOTE 2: For the import of groups see clause 8.2.3.3. |  |  |

The TTCN-3 import mechanism distinguishes between the identifier of a referenced definition and the information necessary for the usage of a referenced definition within the imported definition. For the usage, the identifier of a referenced definition is not required and therefore not imported automatically.

EXAMPLE 1b:	Differentiation between information necessary for the usage and the identifier

module A {

type record MyRec1 {

integer		field1,

charstring	field2

}

}

module B {

import from A all;

type record MyRec2 {

MyRec1	myField1,

// "myField1" is the local definition, "MyRec1" is a referenced definition;

// the name "MyRec1" shall be imported in this case as is directly referenced

boolean	myField2

}

}

module C {

import from B all;

const MyRec2 c_myRec2 := {

myField1 := { field1 := 5, field2 := "A" },

// to define myField1 of MyRec2 the name "MyRec1" is not needed, the

// information necessary for the usage is its type information,

// i.e. names and types of its fields field1 and field2

// which is embeddded in the imported definition of MyRec2

myField2 := true

}

}

If an imported definition has attributes (defined by means of a with statement) then the attributes shall also be imported. The mechanism to change attributes of imported definitions is explained in clause 27.1.3.

NOTE 3:	If the module has global attributes they are associated to definitions without these attributes.

The use of import on single definitions, groups of definitions, definitions of the same kind, etc. may lead to situations where the same definition is referred to more than once. Such cases shall be resolved by the system and definitions shall be imported only once.

NOTE 4:	The mechanisms to resolve such ambiguities, e.g. overwriting and sending warnings to the user, are outside the scope of the present document and should be provided by TTCN-3 tools.

All import statements and definitions within import statements are considered to be treated independently one after the other in the order of their appearance.

All TTCN-3 modules shall have their own name space in which all definitions shall be uniquely identified. Name clashes may occur due to import, e.g. import from different modules. Name clashes shall be resolved using qualified name(s) for the imported definition(s), i.e. prefixing the imported definition (which causes the name clash) by the identifier of the module in which it has been defined; the prefix and the identifier shall be separated by a dot ("."). If the type of the component referenced in a connection operation is known (either when the component reference is a variable or value returned from a function or the type is defined the runs on, mtc or system clause of the calling function), the referenced port declaration shall be present in this component type.

There is one exception to this rule: when in the context of an enumerated type (see clause 6.2.4), an enumerated value is clashing with the name of a definition in the importing module, the enumerated value shall take precedence and the definition in the importing module shall be referenced by using its qualified name (see example 5 below in this clause).

In cases where there are no ambiguities the prefixing need not (but may) be present when the imported definitions are used. When the definition is referenced in the same module where it is defined, the module identifier of the module (the current module) also may be used for prefixing the identifier of the definition. For the latter case, prefixing shall only be used for definitions with global visibility for the module.

It is allowed to rename a module name during its import. The new name will be visible only in the importing module.

Restrictions

In addition to the general static rules of TTCN-3 given in clause 5, the following restrictions apply:

a)	An import statement shall only be used in the module definitions part and not be used within a function definition and alike.

b)	Only top-level visible definitions of a module may be imported. Definitions which are top-level but invisible to the importing module or which occur at a lower scope (e.g. local constants defined in a function) shall not be imported.

c)	A definition is imported together with its name and all local definitions.

NOTE 5:	A local definition, e.g. a field name of a user-defined record type or an enumerated value, has only meaning in the context of the definitions in which it is defined, e.g. a field name of a record type can only be used to access a field of the record type and not outside this context.

In particular, importing an enumerated type does not impose the restriction given in clause 6.2.4 on global names defined in the importing module.

d)	A definition is imported together with all information of referenced definitions that are necessary for the usage of the imported definition, independent of the visibility of the referenced definitions (see clause 8.2.5).

NOTE 6:	If module C imports a definition from module B that uses a type reference defined in module A, the corresponding information necessary for the usage of that type is automatically imported into module C (see example 6 below in this clause). Identifiers of referenced definitions are not automatically imported.

In particular, if module C imports global value or template definitions (e.g. constants, module parameters, templates) or local definitions (e.g. formal parameters of templates, functions, etc., or constants and variables of component types) of an enumerated type from module B, the enumerated values of this type (i.e. the identifiers) are implicitly and automatically imported to module C. That is, the enumerated values are known when an enumerated value or template is used in module C (e.g. when an actual parameter is passed or a value is assigned to a component variable). Note that this implicit importing does not impose the restriction given in clause 6.2.4 on global names defined in module C.

e)	If the referenced definitions are wished to be used in the importing module, they shall be explicitly imported either directly from its source module or indirectly by importing the import statements of a module importing it (see clause 8.2.3.7).

f)	When importing a function, altstep or test case the corresponding behaviour specifications and all definitions used inside the behaviour specifications remain invisible for the importing module.

g)	The language specification (see clause 8.1) of the import statement shall not override the language specification of the importing module.

h)	The language specification of the import statement shall be identical to the language specification of the source module from which definitions are imported (see clause 8.2.3.8) provided a language specification is defined in the source module. If not, the language specification in the import statement is taken as the language specification of the source module. If the source module uses however language concepts not being part of that language specification, this causes an error for the import statement.

i)	If an imported module is renamed in the import clause, the original module name is not imported and cannot be used for referencing the imported module. The imported module can be referenced using the new local name only.

j)	If an imported module is renamed in the import clause, the new local name of the module shall be unique in the scope of the importing module.

Examples

EXAMPLE 1:	Selected import examples:

module MyModuleA

{ 	:

// Scope of the imported definitions is global to MyModuleA

import from MyModuleB all; 	// import of all definitions from MyModuleB

import from MyModuleC {		// import of selected definitions from MyModuleC

type MyType1, MyType2;  // import of types MyType1 and MyType2

template all			// import of all templates

}

:

function f_myBehaviourC()

{

// import cannot be used here

:

}

:

control

{

// import cannot be used here

:

}

}

EXAMPLE 2:	Use of imported definitions and visibility of definitions referenced by them:

module ModuleONE {

modulepar integer ModPar1 := …;

type record RecordType_T1 {

integer	Field1_T1,

:

}

type record RecordType_T2 {

RecordType_T1	Field1_T2,

:

}

const integer c_myConst := …;

template RecordType_T2 m_t2 (RecordType_T1 p_tempParT2):= { // parameterized template

Field1_T2 := …,

:

}

} // end module ModuleONE

module ModuleTWO {

import from ModuleONE {

template m_t2

}

// Only the names m_T2 and p_tempParT2 will be visible in ModuleTWO. Please note, that

// the identifier p_tempParT2 can only be used when modifying m_t2. All information

// necessary for the usage of m_t2, e.g. for type checking purposes, are imported

// for the referenced definitions RecordType_T1, Field1_T2, etc., but their identifiers are

// not visible in ModuleTWO.

// This means, e.g. it is not possible to use the constant c_myConst or to declare a

// variable of type RecordType_T1 or RecordType_T2 in ModuleTWO without explicitly importing
		// these types.

import from ModuleONE {

modulepar ModPar2

}

// The module parameter ModPar2 of ModuleONE is imported from ModuleONE and

// can be used like an integer constant

} // end module ModuleTWO

module ModuleTHREE {

import from ModuleONE all; 	// imports all definitions from ModuleONE

type port MyPortType message {

inout RecordType_T2 	// Reference to a type defined in ModuleONE

}

type component MyCompType {

var integer v_myComponentVar := ModPar2;

// Reference to a module parameter of ModuleONE

:

}

function f_myFunction () return integer {

return c_myConst		// Reference to a module constant of ModuleONE

}

testcase TC_MyTestCase (out RecordType_T2 p_myPar) runs on MyCompType {

:

MyPort.send(m_t2); // Sending a template defined in ModuleONE

:

}

} // end ModuleTHREE

module ModuleFOUR {

import from ModuleTHREE {

testcase TC_MyTestCase

}

// Only the name TC_MyTestCase will be visible and usable in ModuleFOUR.

// Type information for RecordType_T2 is imported via ModuleTHREE from ModuleONE and

// Type information for MyCompType is imported from ModuleTHREE. All definitions

// used in the behaviour part of TC_MyTestCase remain hidden for the user of ModuleFOUR.

} // end ModuleFOUR

EXAMPLE 3:	Handling of name clashes:

module MyModuleA  {

:

type bitstring MyTypeA;

import from SomeModuleC {

type	MyTypeA,		// Where MyTypeA is of type character string

MyTypeB			// Where MyTypeB is of type character string

}

:

control {

:

var	SomeModuleC.MyTypeA v_myVar1 := "Test String"; 	// Prefix shall be used

var MyTypeA v_myVar2 := '10110011'B; 				// This is the original MyTypeA

:

var MyTypeB v_myVar3 := "Test String"; 			 	// Prefix need not be used …

var SomeModuleC.MyTypeB v_myVar3 := "Test String";	// … but it can be if wished

:

}

}

NOTE 7:	Definitions with the same name defined in different modules are always assumed to be different, even if the actual definitions in the different modules are identical. For example, importing a type that is already defined locally, even with the same name, would lead to two different types being available in the module.

EXAMPLE 4:	Renaming imported module:

module MyModuleA  {

import from VeryLongModuleNameB -> ShortNameB {

type	MyTypeA,		// Where MyTypeA is of type character string

}

:

control {

:

var	ShortNameB.MyTypeA v_myVar1 := "Test String";				// Is correct

var VeryLongModuleNameB MyTypeA v_myVar2 := "Test String";		// Causes an error

// as the original module name cannot be used for referencing if the

// imported module has been renamed.

}

}

EXAMPLE 5:	Name clash between enumerated values and global definitions:

module A {

type enumerated MyEnumType {enumX, enumY}

type enumerated MyEnumType2 {enumY, enumZ}

}

module B {

import from A all;

const MyEnumType enumY := enumX; // this is not allowed as enumerated values restrict

// global names (see clause 6.2.4)

const MyEnumType2 enumX := enumY;// this is allowed as MyEnumtype2 does not contain enumX

const MyEnumType enumZ := enumX; // allowed as MyEnumType does not contain enumZ

}

module C {

import from A all;

import from B all;

const integer enumZ := 0;

const integer enumY := 1;

const MyEnumType2 enumX := enumY;

modulepar MyEnumType PX_MyModulePar1 := enumY

// the default value of the module parameter will be the value enumY, as the type of

// PX_MyModulePar1 creates the context of MyEnumType and in this context enumerated values

// take precedence over global definition names; note that for the same context reason there

// is no name clash between the enumerated values defined in MyEnumType and in MyEnumType2

modulepar MyEnumType PX_MyModulePar2 := B.enumZ

// the default value of the module parameter will be the value enumX, as the prefix

// identifies the constant definition enumZ unambiguously, which has the value enumX

modulepar integer PX_IntegerPar := enumZ;

// the default value of the module parameter will be 0 as this assignment is not in the

// context of an enumerated type, hence no name clash occurs

modulepar MyEnumType PX_MyModulePar3 := C.enumX

// causes an error as PX_MyModulePar3 and the constant enumX in module C has different types

}

EXAMPLE 6:	Importing local definitions transitively:

module A {

type enumerated MyEnumType { enumX, enumY, enumZ}

type record MyRec { integer a, integer b }

type component MyComp { var MyRec v_rec := { a := 5 } }

}

module B {

import from A all;

modulepar MyEnumType PX_MyModulePar := enumY;

type component MyCompUser extends MyComp {}

}

module C {

import from B all;

testcase TC() runs on MyCompUser {

if (PX_MyModulePar == enumY) {

// the enumerated value enumY is known in C without explicitly importing it from A

setverdict(pass)

}

if (v_rec.a == 5) {

v_rec.b := v_rec.a;

// Both the variable name v_rec and the record field names are known in C without

// explicitly importing them from A

setverdict (pass)

}

}

}

#### 8.2.3.2	Importing single definitions

Single visible definitions can be imported by referring to the definition kind and the definition name(s). The import of single definitions can be used in combination with imports of groups (see clause 8.2.3.3), with imports of definitions of the same kind (see clause 8.2.3.4), and with imports of import statements (see clause 8.2.3.7).

Syntactical Structure

[ Visibility ] import from ModuleId "{"

{

(

( type 		{ TypeDefIdentifier 	[ "," ] } ) |

( template 	{ TemplateIdentifier 	[ "," ] } ) |

( const 	{ ConstIdentifier 		[ "," ] } ) |

( testcase 	{ TestcaseIdentifier 	[ "," ] } ) |

( altstep 	{ AltstepIdentifier 	[ "," ] } ) |

( function 	{ FunctionIdentifier 	[ "," ] } ) |

( signature { SignatureIdentifier 	[ "," ] } ) |

( modulepar { ModuleParIdentifier 	[ "," ] } )

)

[ ";" ]

}

"}" [ ";" ]

Semantic Description

See clause 8.2.3. Import of an invisible definition shall cause an error.

Restrictions

In addition to the general static rules of TTCN-3 given in clause 5, the following restrictions apply:

- The definition to be imported shall be defined in the module from which it is to be imported and shall be visible to the importing module.

See the restrictions given in clause 8.2.3.

Examples

import from MyModuleA {

type MyType1						// imports one type definition from MyModuleA only

}

import from MyModuleB {

type MyType2, Mytype3, MyType4;		// imports three types,

template m_myTemplate1;				// imports one template, and

const c_myConst1, c_myConst2		// imports two constants

}

#### 8.2.3.3	Importing groups

Groups of definitions may be imported. The import of groups can be used in combination with imports of single definitions (see clause 8.2.3.2), with imports of definitions of the same kind (see clause 8.2.3.4), and with imports of import statements (see clause 8.2.3.7).

It is allowed to import sub-groups (i.e. a group which is defined within another group) directly, i.e. without the groups in which the sub-group is embedded. If the name of a sub-group that should be imported is identical to the name of another sub-group in the same module (see clause 8.2.2), the dot notation shall be used to identify the sub-group to be imported uniquely.

If some visible definitions of a group are wished not to be imported, their kinds and identifiers shall be listed in the exception list within a pair of curly brackets following the except keyword. The all keyword is also allowed to be used in the exception list; this will exclude all definitions of the same kind from the import statement.

Syntactical Structure

[ Visibility ] import from ModuleId "{"

{

( group	{ QualifiedIdentifier [ except "{" ExceptSpec "}" ]	[ "," ] } )

[ ";" ]

}

"}" [ ";" ]

Semantic Description

The effect of importing a group is identical to an import statement that lists all visible definitions (including sub-groups) of this group except of those that are listed in the except specification. See also clause 8.2.3. Import statements contained in the group or in its subgroups are not part of this list, only definitions are.

It is important to point out, that the except statement does not exclude the definitions listed from being imported in general; all statements importing definitions of the same kind can be seen as a shorthand notation for an equivalent list of identifiers of single definitions. The except statement excludes definitions from this single list only.

Restrictions

In addition to the general static rules of TTCN-3 given in clause 5, the following restrictions apply:

- The group to be imported shall be defined in the module from which it is to be imported.

- See the restrictions given in clause 8.2.3.

Examples

import from MyModule { group myGroup } // includes all visible definitions from myGroup

import from MyModule {

group myGroup except {

type MyType3, MyType5;	// excludes the two types from the import statement,

template all			// excludes all templates defined in myGroup

// from the import statement

// but imports all other visible definitions of myGroup

}

}

import from MyModule {

group myGroup

except { type MyType3 };// imports all visible types of myGroup except MyType3

type MyType3				// imports MyType3 explicitly

}

#### 8.2.3.4	Importing definitions of the same kind

The all keyword may be used to import all visible definitions of the same kind of a module. The all keyword used with the constant keyword identifies all visible constants declared in the definitions part of the module the import statement refers to. Similarly the all keyword used with the function keyword identifies all visible functions, all visible external functions defined and the visible module control function in the module the import statement denotes.

If some visible declarations of a kind are wished to be excluded from the given import statement, their identifiers shall be listed following the except keyword.

The import of visible definitions of the same kind can be used in combination with imports of single visible definitions (see clause 8.2.3.2), with imports of groups (see clause 8.2.3.3), and with imports of import statements (see clause 8.2.3.7).

Syntactical Structure

[ Visibility ] import from ModuleId "{"

{

(

( type 		all [ except { TypeDefIdentifier 	[ "," ] } ] ) |

( template 	all [ except { TemplateIdentifier 	[ "," ] } ] ) |

( const 	all [ except { ConstIdentifier 		[ "," ] } ] ) |

( testcase 	all [ except { TestcaseIdentifier 	[ "," ] } ] ) |

( altstep 	all [ except { AltstepIdentifier 	[ "," ] } ] ) |

( function 	all [ except { FunctionIdentifier 	[ "," ] } ] ) |

( signature all [ except { SignatureIdentifier 	[ "," ] } ] ) |

( modulepar all [ except { ModuleParIdentifier 	[ "," ] } ] )

)

[ ";" ]

}

"}" [ ";" ]

Semantic Description

The effect of importing definitions of the same kind is identical to an import statement that lists all visible definitions of that kind except of those that are listed in the except specification. See also clause 8.2.3.

NOTE:	If the list of all visible definitions of that kind except of those that are listed in the except specification is empty, the import statement has no effect. This case does not lead to an error.

Restrictions

In addition to the general static rules of TTCN-3 given in clause 5, the following restrictions apply:

- See the restrictions given in clause 8.2.3.

Examples

import from MyModule {

type all;				// imports all types of MyModule

template all			// imports all templates of MyModule

}

import from MyModule {

type all except MyType3, MyType5;	// imports all types except MyType3 and MyType5

template all						// imports all templates defined in Mymodule

}

#### 8.2.3.5	Importing all definitions of a module

All visible definitions of a module definitions part may be imported using the all keyword next to the module name.

If some visible definitions are wished not to be imported, their kinds and identifiers shall be listed in the exception list within a pair of curly brackets following the except keyword. The all keyword is also allowed to be used in the exception list; this will exclude all visible declarations of the same kind from the import statement.

NOTE 1:	If the list of all visible definitions of a module except of those that are listed in the except specification is empty, the import statement has no effect. This case does not lead to an error.

NOTE 2:	Importing all definitions of a module imports only definitions declared directly in that module including the address type and module control function (if they are specified), but does not import the import statements of that module (see also clause 8.2.3.7).

Syntactical Structure

[ Visibility ] import from ModuleId

all

[

{

except "{"

( group 	{ QualifiedIdentifier 	[ "," ] } | all ) |

( type 		{ TypeDefIdentifier 	[ "," ] } | all ) |

( template 	{ TemplateIdentifier 	[ "," ] } | all ) |

( const 	{ ConstIdentifier 		[ "," ] } | all ) |

( testcase 	{ TestcaseIdentifier 	[ "," ] } | all ) |

( altstep 	{ AltstepIdentifier 	[ "," ] } | all ) |

( function 	{ FunctionIdentifier 	[ "," ] } | all ) |

( signature { SignatureIdentifier 	[ "," ] } | all ) |

( modulepar { ModuleParIdentifier 	[ "," ] } | all )

"}"

[ ";" ]

}

]

[ ";" ]

Semantic Description

The effect of importing all visible definitions of a module is identical to an import statement that lists all importable definitions of that module except of those that are listed in the except specification. See also clause 8.2.3.

Restrictions

In addition to the general static rules of TTCN-3 given in clause 5, the following restrictions apply:

- If all visible definitions of a module are imported by using the all keyword, no other form of import (import of single definitions, import of the same kind, etc.) shall be used for the same import statement.

- In the set of except statements for an all import, only one except statement per kind of definition (i.e. for a group, type, etc.) is allowed.

Examples

import from MyModule all; 	// includes all definitions from MyModule

import from MyModule all except {

type MyType3, MyType5;	// excludes these two types from the import statement and

template all			// excludes all templates declared in MyModule,

// from the import statement

// but imports all other definitions of MyModule

}

#### 8.2.3.6	Import definitions from other TTCN-3 editions and from non-TTCN-3 modules

In cases when visible definitions are imported from modules from other TTCN-3 editions or from other sources than TTCN-3 modules, the language specification (see clause 8.1) shall be used to denote the language (may be together with a version number) of the source (e.g. module, package, library or even file) from which definitions are imported. It consists of the language keyword and a subsequent textual declaration of the denoted language.

The use of the language specification is optional when importing from a TTCN-3 module of the same edition as the importing module. The TTCN-3 language identifiers defined in clause 8.1 are to be used. Package identifiers from ETSI ES 202 781 [i.11], ETSI ES 202 782 [i.14], ETSI ES 202 784 [i.12] and ETSI ES 202 785 [i.13] can be used in addition. Identifiers for other languages are defined in the language mapping parts of TTCN-3, i.e. in ETSI ES 201 873-7 [i.5], ETSI ES 201 873-8 [i.6] and ETSI ES 201 873-9 [i.7].

When an incompatibility is discovered between the language and/or package identification (including implicit identification by omitting the language specification) and the syntax of the module from which definitions are imported, tools shall provide reasonable effort to resolve the conflict.

Syntactical Structure

[ Visibility ] import from ModuleIdentifier [ LanguageSpec ] … [ ";" ]

Semantic Description

TTCN-3 supports the referencing of elements defined in other TTCN-3 editions (versioned elements) or other languages (foreign elements) from within TTCN-3 modules. Such elements can be used in a TTCN-3 module of a given edition only if they have a TTCN-3 view in that TTCN-3 edition. The term TTCN-3 view can be best explained by considering the case when the definition of a TTCN-3 element is based on another TTCN-3 element, the information content of the referenced element shall be available and is used for the new definition. For example, when a template is defined based on a structured type, the identifiers and types of fields of the base type shall be accessible and are used for the template definition. In a similar way, when a base type is a versioned or foreign element it shall provide the same information content as would be required from a TTCN-3 type declaration. The versioned or foreign element, naturally, may contain more information than required by TTCN-3. The TTCN-3 view of a versioned or foreign element means that part of the information carried by that element, which is necessary to use it in TTCN-3. Obviously, the TTCN-3 view of a versioned or foreign element may be the full set or a subset of the information content of that element but never a superset. There may be versioned or foreign element without a TTCN-3 view (zero TTCN-3 view), i.e. for some reason no TTCN-3 definition in the given edition could be based on them.

To make declarations of versioned or foreign element visible in TTCN-3 modules, their names shall be imported just like definitions in other TTCN-3 modules of the given edition. When imported, only the TTCN-3 view of the versioned or foreign element will be seen from the importing TTCN-3 module. There are two main differences between importing TTCN-3 elements of the same editions and versioned or foreign elements:

To import from a TTCN-3 module of another edition or from a non-TTCN-3 module, the import statement shall contain an appropriate language identifier string.

Only versioned or foreign elements with a TTCN-3 view of a given edition are importable into a TTCN-3 module of that edition.

Importing can be done automatically using the all directive, in which case all importable objects shall automatically be selected by the testing tool, or done manually by listing names of elements to be imported. Naturally, in the second case only importable elements are allowed in the list.

When importing definitions from a non-TTCN-3 language, two principle approaches exist:

With an implicit language mapping, non-TTCN-3 definitions are mapped internally in the TTCN-3 tool to the respective TTCN-3 definitions as defined by the language mapping; the importing module works with the internal representations of the imported definitions.

With an explicit language mapping, non-TTCN-3 definitions are mapped directly to separate TTCN-3 definitions; the importing module imports the generated TTCN-3 and works with the mapped TTCN-3 definitions.

These lead to three options when using non-TTCN-3 language modules in a TTCN-3 specification:

The import statement imports the non-TTCN-3 module; the tool uses the internal representation of the implicit mapping of the non-TTCN-3 module's definitions according to the language mapping specification of that language.

The import statement imports the non-TTCN-3 module; the tool imports from a TTCN-3 module which is an explicit mapping of the non-TTCN-3 module's definitions according to the language mapping specification of that language.

The import statement imports the explicit TTCN-3 representation of the non-TTCN-3 module; the tool imports the TTCN-3 module which is an explicit mapping of the non-TTCN-3 module according to the language mapping specification of that language.

Restrictions

In addition to the general static rules of TTCN-3 given in clause 5, the following restrictions apply:

- The language specification should only be omitted if the referenced module contains TTCN-3 notation and the TTCN-3 version is known.

- Definitions imported from non-TTCN-3 language sources have by default public visibility provided that no other rules are defined in the respective language mapping (see ETSI ES 201 873-7 [i.5], ETSI ES 201 873-8 [i.6] or ETSI ES 201 873-9 [i.7], respectively).

Examples

module MyNewModule {

import from MyOldModule language "TTCN-3:2003" {

type MyType

}

}

module MyNewestModule {

import from MyNewModule language "TTCN-3:2010" { import all };

// the language specifications shall be identical, see clause 8.2.3.8

}

NOTE:	The import mechanism is designed to allow the re-use of definitions from other TTCN-3 editions or from other non-TTCN-3 language sources. The rules for importing definitions from specifications written in other languages, e.g. SDL packages, may follow the TTCN-3 rules or may have to be defined separately.

#### 8.2.3.7	Importing of import statements from TTCN-3 modules

Visible import statements of TTCN-3 modules can be imported by other TTCN-3 modules.

Syntactical Structure

[ Visibility ] import from ModuleIdentifier [ LanguageSpec ]

"{" import all [ ";" ] "}" [ ";" ]

Semantic Description

TTCN-3 supports importing of visible import statements from other TTCN-3 modules. This means that import statements of the module, from which the import statements are imported, are re-imported to the importing module. For example, if module B imports the import statements of module A, everything that is imported by A using import statements visible for module B, is also imported by B. If another module C imports all import statements from B, then C imports all what A is importing - provided that the import statements are visible to modules B and C.

It is not possible to import individual import statements of another module.

The import of import statements can be used in combination with imports of single definitions (see clause 8.2.3.2), with imports of groups (see clause 8.2.3.3), and with imports of definitions of the same kind (see clause 8.2.3.4).

Restrictions

In addition to the general static rules of TTCN-3 given in clause 5, the following restrictions apply:

a)	The restrictions given in clause 8.2.3.1 apply.

b)	The restrictions given in clause 8.2.3.6 apply.

c)	Importing of import statements is only possible from other TTCN-3 modules, i.e. the language specification (see clause 8.1) shall denote a TTCN-3 edition only, not a non-TTCN-3 language.

Examples

EXAMPLE:	Importing of visible import statements

module A {

type integer T1;

type integer T2;

template T1 mw_t1 := ?;

template T2 mw_t2 := *;

:

}

module B {

public import from A { type T1 }

type charstring T2;

template T1 m_t1 := ( 1, 2, 3 );

:

}

module C {

public import from B { import all }  // imports the import statements only

public import from B { type T2 }     // imports the type B.T2

import from A { template all }

:

}

module D {

private import from C { import all } // imports the import statements only

:

}

module E {

import from D { import all }

:

}

// yields the following

// module A knows

// A.T1 	(defined)

// A.T2 	(defined)

// A.mw_t1 	(defined)

// A.mw_t2 	(defined)

//

// module B knows

// A.T1 	(imported)

// B.T2 	(defined)

// B.m_t1 	(defined)

//

// module C knows

// A.T1 	(imported from B importing it from A)

// B.T2 	(imported)

// A.mw_t1 	(imported)

// A.mw_t2 	(imported)

//

// module D knows

// A.T1 	(imported from C importing it from B importing it from A)

// B.T2 	(imported from C importing it from B)

// A.mw_t1 and A.mw_t2 are not imported as their imports are private to C

//

// module E "knows" nothing

// as the imports of D are private and not visible to E

#### 8.2.3.8	Compatibility of language specifications in imports

When importing into a TTCN-3 module, the language specification (see clause 8.1) of the importing module, the language specification of the import statement and the language specification of the source module, where the imported definitions are defined, have to be compatible according to the following rules.

Restrictions

In addition to the general static rules of TTCN-3 given in clause 5, the following restrictions apply:

a)	A TTCN-3 module of any TTCN-3 edition can import from a non-TTCN-3 language source provided that a TTCN-3 view for the non-TTCN-3 language exists (see clause 8.2.3.6).

b)	Definitions or import statements are imported according to the language specification in which the definition or the import statement is defined. If no language specification is given in this module, the language specification of the import statement with which those definitions or import statements are to be imported, is used instead. If the module, within which the definitions or the import statements are defined, and the import statement for these definitions or import statements provide both a language specification, then they shall be identical. If none of the two has a language specification, the language specification has to be known from other sources, which is tool specific.

c)	A TTCN-3 module shall only import from earlier or same editions of TTCN-3 but not from later editions, e.g. the TTCN-3 language specification in an import statement has to be lower or equal to the TTCN-3 language specification of the importing module.

### 8.2.4	Definition of friend modules

Modules can define other modules to be friends.

Syntactical Structure

[ private ] friend module ModuleIdentifier { "," ModuleIdentifier } ";"

Semantic Description

Friendship to modules is defined by the exporting module (the module that declares the definitions) not by the importing module (the module that uses the module definitions of another module). Friendship can be cyclic.

If a module is friend to a module from which it imports top-level definitions, all top-level definitions with public and friend visibility are visible to the friend module. For non-friend modules, public top-level definitions are visible only.

Missing friend modules shall not cause an error.

NOTE:	Friend modules can be checked by tools, however at most warning are to be issued if a friend module is missing.

Restrictions

In addition to the general static rules of TTCN-3 given in clause 5, the following restrictions apply:

a)	Only private visibility can be defined for friend definitions as they are always private.

Examples

module MyModuleA {

friend module MyModuleB,MyModuleC;

}

// MyModuleB and MyModuleC are friends of MyModuleA

module MyModuleB {

friend module MyModuleA;

}

// MyModuleA is friend of MyModuleB

module MyModuleC {

}

### 8.2.5	Visibility of definitions

Top-level module definitions and import statements have a visibility, which can be explicitly set. They are by default public except for imported and friend definitions. Import definitions are by default private. Friend definitions are private only. Group definitions are public only.

Syntactical Structure

[ public | friend | private ]

Semantic Description

The visibility controls whether a top-level definition or an import statement is importable by another module.

Three visibilities are distinguished:

A top-level definition or an import statement with public visibility is importable by any other module.

A top-level definition or an import statement with friend visibility is importable by friend modules only (see clause 8.2.4).

A top-level definition or an import statement with private visibility cannot be imported at all.

NOTE:	As specified in restriction e) of clause 8.2.3.1, this means that importable definitions are imported together with all information of referenced definitions that are necessary for the usage of the importable definition, even if the referenced definition is private. Only the identifier of the referenced definition is not visible in the importing TTCN-3 module.

The visibility of groups is always public. The visibility of imported definitions is by default private. All other module definitions are by default public.

The visibility of a top-level definition or an import statement defines their importability by another module. If the top-level definition or the import statement is part of a group, this has no effect on the importability of the module definition. The importability of a top-level definition by another module is summarized in table 9, the importability of import statements in table 10.

Table 9: Visibility and import of module definitions

| Visibility of module definition | Module definition importable directly by a non-friend module | Module definition importable directly by a friend module | Module definition importable via group import by a non-friend module | Module definition importable via group import by a friend module |
| --- | --- | --- | --- | --- |
| public | yes | yes | yes | yes |
| friend | no | yes | no | yes |
| private | no | no | no | no |

Table 10: Visibility and import of import statements

| Visibility of import | Import imported by a non-friend module | Import imported by a friend module |
| --- | --- | --- |
| public | yes | yes |
| friend | no | yes |
| private | no | no |

Restrictions

No specific restrictions in addition to the general static rules of TTCN-3 given in clause 5.

Examples

module MyModuleA {

friend module MyModuleC;

private type integer MyInteger;

// MyInteger is not visible to other modules

friend type charstring MyString;

// MyString is visible to friend modules

public type boolean MyBoolean;

// MyBoolean is visible to all modules

}

module MyModuleB {

import from MyModuleA all;

// MyString and MyInteger are not visible and are not imported

// MyBoolean is imported

}

module MyModuleC {

import from MyModuleA all;

// MyInteger is not visible and is not imported

// MyString and MyBoolean are imported

}

## 8.3	Module control part

The module control part is a shorthand notation to define a module control function.

Syntactical Structure

control StatementBlock

Semantic Description

The shorthand notation is equivalent to a module control function with the same statement block, no parameters, no runs on clause and no return clause (see clause 16.1). It is equal to the following syntax:

EXAMPLE 1:

control {

execute(TC_01());

}

// is equal to:

function control () {

execute(TC_01());

}

NOTE:	As the control part is a short hand notation of  a control function with the name "control", at most 1 control part can be defined in a TTCN-3 module.

When not used as an entry point for execution of the test suite, the module control function can be invoked explicitly in a similar way as a standard function, using the control keyword instead of the identifier. In this case, the control keyword may be optionally prefixed with a module identifier followed by a dot. If the referenced module control function does not have any parameters, the parameter list may also be omitted at the place of invocation.

The rules for declaring and use of the module control function are described in clause 16.1. The mechanism of module control is explained in more detail in clause 26.

EXAMPLE 2:

module MyTestSuite

{ 	// This module contains definitions …

:

const integer c_myConstant := 1;

type record MyMessageType { … }

template MyMessageType m_myMessage := { … }

:

function f_myFunction1() { … }

function f_myFunction2() { … }

:

testcase TC_MyTestcase1() runs on MyMTCType { … }

testcase TC_MyTestcase2() runs on MyMTCType { … }

:

// … the module control function provides an entry point for test suite execution

control

{

var boolean v_myVariable; // local control variable

:

execute( TC_MyTestCase1()); // sequential execution of test cases

execute( TC_MyTestCase2());

:

}

}

EXAMPLE 3:

function control(integer p_par) return integer {

execute(TC_01(p_par));			// typical use: test case execution

MyTestSuite.control();		// explicit execution of a control function from an imported

// module

MyTestSuite.control;		// shorthand call notation for control parts without parameters

return -1;

};

# 9	Port types, component types and test configurations

## 9.0	General

TTCN-3 allows the (dynamic) specification of concurrent test configurations (or configuration for short). A configuration consists of a set of inter-connected test components with well-defined communication ports and an explicit test system interface which defines the borders of the test system (see figure 4).

NOTE:	Additional configuration and deployment support for TTCN-3 is defined in the optional package [i.11].

Figure 4: Conceptual view of a typical TTCN-3 test configuration

Within every configuration there shall be one (and only one) Main Test Component (MTC). Test components that are not MTCs are called parallel test components or PTCs. The MTC shall be created by the system automatically at the start of each test case execution. The behaviour defined in the body of the test case shall execute on this component. During execution of a test case, other components can be created dynamically by the explicit use of the create operation.

Test case execution shall end when the MTC terminates. All other PTCs are treated equally i.e. there is no explicit hierarchical relationship among them and the termination of a single PTC terminates neither other components nor the MTC. When the MTC terminates, the test system has to stop all PTCs not terminated by the moment when the test case execution is ended.

Communication between test components and between the components and the test system interface is achieved via communication ports (see clause 9.1).

Test component types and port types, denoted by the keywords component and port, shall be defined in the module definitions part. The actual configuration of components and the connections between them is achieved by performing create and connect operations within the test case behaviour. The component ports are connected to the ports of the test system interface by means of the map operation (see clause 21.1.1).

## 9.1	Communication ports

Test components are connected via their ports, i.e. connections among components and between a component and the test system interface are port-oriented. Each port is modelled as an infinite FIFO queue which stores the incoming messages or procedure calls until they are processed by the component owning that port (see figure 5).

NOTE:	While TTCN-3 ports are infinite in principle in a real test system they may overflow. This is to be treated as a test case error (see clause 24.1).

Figure 5: The TTCN-3 communication port model

TTCN-3 connections are port-to-port and port-to-test system interface connections (see figure 6). There are no restrictions on the number of connections a component may maintain. One-to-many connections are also allowed (e.g. figure 6 (g) or (h)).

Restrictions

In addition to the general static rules of TTCN-3 given in clause 5, the following restrictions apply:

a)	The following connections are not allowed (see figure 7):

A port owned by a component A shall not be connected with two or more ports owned by the same component (figure 7 (a) and (e)).

A port of a test system interface cannot have connection with more than one port owned by a component A. This means, connections as shown in figure 7 (b) are not allowed.

A port owned by a component A shall not be connected with two or more ports owned by a component B (see figure 7 (c)).

A port owned by a component A can only have a one-to-one connection with the test system interface. This means, connections as shown in figure 7 (d) are not allowed.

Connections within the test system interface are not allowed (see figure 7 (f)).

A port that is connected shall not be mapped and a port that is mapped shall not be connected (see figure 7 (g)).

b)	Since TTCN-3 allows dynamic configurations and addresses, the restrictions on connections cannot always be checked at compile-time. The checks shall be made at runtime and shall lead to a test case error when failing.

Figure 6: Allowed connections

Figure 7: NOT allowed connections

## 9.2	Test system interface

TTCN-3 is used to test implementations. The object being tested is known as the Implementation Under Test or IUT. The IUT may offer direct interfaces for testing or it may be part of system in which case the tested object is known as a System Under Test or SUT. In the minimal case the IUT and the SUT are equivalent. In the present document the term SUT is used in a general way to mean either SUT or IUT.

In a real test environment test cases need to communicate with the SUT. However, the specification of the real physical connection is outside the scope of TTCN-3. Instead, a well defined (but abstract) test system interface shall be associated with each test case. A test system interface definition is identical to a component definition, i.e. it is a list of all possible communication ports through which the test case is connected to the SUT.

The test system interface statically defines the number and type of the port connections to the SUT during a test run. However, the connections between the test system interface and the TTCN-3 test components are dynamic in nature and may be modified during a test run by using map and unmap operations (see clause 21.1).

A component type definition is used to define the test system interface because, conceptually, component type definitions and test system interface definitions have the same form (both are collections of ports defining possible connection points). When used as test system interfaces, components cannot make use of any constants, variables and timers declared in the component type.

Syntactical Structure

The same as a component type definition (see clause 6.2.10.1).

Semantic Description

Generally, a component type reference defining the test system interface shall be associated with every test case using more than one test component. The ports of the test system interface shall automatically be instantiated by the system together with the MTC when the test case execution starts.

The operation returning the component reference of the test system interface is system. This shall be used to address the ports of the test system.

In the case where the MTC is the only component that is instantiated during test execution, a test system interface need not be associated to the test case. In this case, the component type definition associated with the MTC implicitly defines the corresponding test system interface.

Variables, timers and constants declared in component types, which are used as test system interfaces will have no effect.

Restrictions

In addition to the general static rules of TTCN-3 given in clause 5, the following restrictions apply:

- The same as for component type definitions (see clause 6.2.10.1).

Examples

EXAMPLE 1:	Explicit definition of a test system interface:

type component MyMTCType

{

var integer vc_myLocalInteger;

timer tc_myLocalTimer;

port MyMessagePortType pCO1

}

type component MyTestSystemInterface

{

port MyMessagePortType		pCO1, pCO2;

port MyProcedurePortType	pCO3

}

// MyTestSystemInterface is the test system interface

testcase TC_MyTestcase1 () runs on MyMTCType system MyTestSystemInterface {

// establishing the port connections

map(mtc:pCO1, system:pCO2);

// the testcase behaviour

// …

}

EXAMPLE 2:	Implicit definition of a test system interface:

// MyMTCType is the test system interface

testcase TC_MyTestcase2 () runs on MyMTCType {

// map statements are not needed

// the testcase behaviour

// …

}

# 10	Declaring constants

TTCN-3 constants are runtime constants. After value assignment, they do not change their value during test execution. They can be used on the right hand side of assignments, in expressions, in actual parameters, and in template definitions. Constants used within type definitions have to have values known at compile-time.

Syntactical Structure

const Type { ConstIdentifier [ ArrayDef ] ":=" ConstantExpression [ "," ] } [ ";" ]

Semantic Description

A constant assigns a name to a fixed value. A value is assigned only once to a constant, at the place of its declaration. The constant does not change its value during test execution. The constant is defined only once, but can be referenced multiple times in a TTCN-3 module.

If functions are used for the initialization of constants, it is strongly advised to adhere to the rules defined in clause 16.1.4. Not following these rules may cause non-deterministic test executions.

Optional fields of record and set constants or constant fields can be initialized explicitly or implicitly. For implicit initialization of the optional fields of a constant or a constant field, an optional attribute with the value "implicit omit" (see clause 27.7) shall be associated with it either directly or via the attribute distribution (scoping) mechanism (see clause 27.1.1).

Restrictions

In addition to the general static rules of TTCN-3 given in clause 5, the following restrictions apply:

a)	Void.

b)	Constant expressions initializing constants, which are used in type and array definitions, shall only contain literals, predefined functions except of rnd (see clause 16.1.2), operators specified in clause 7.1, and other constants obeying the limitations of this clause.

NOTE:	The only value that can be assigned to global constants of default, component, port or timer type or component constants of default or component types is the special value null.

c)	Using the dot notation (see clauses 6.2.1.1, 6.2.2.1 and 6.2.5.1) and index notation (see clauses 6.2.7 and 6.2.3.0) for referencing a field, alternative or element of an address value, which actual value is null shall cause an error.

d)	The right-hand side of the assignment that initializes a constant shall evaluate to an object that is at least partially initialized.

Examples

const integer c_myConst1 := 1;

const boolean c_myConst2 := true, c_myConst3 := false;

# 11	Declaring variables

## 11.0	General

TTCN-3 variables are statically typed variables. Variables are either value variables to store values or template variables to store templates.

Variables can be of simple basic types, basic string types, structured types, special data types (including subtypes derived from these types) as well as address, component or default, port or timer types.

Variables can be declared and used in test cases, functions and altsteps. Additionally, variables can be declared in component type definitions. These variables can be used in test cases, altsteps and functions which are running on a given component type.

Variables can be declared lazy using the @lazy modifier.

Alternatively, variables can be declared fuzzy using the @fuzzy modifier.

If a variable is declared fuzzy or lazy they can additionally declared with the @deterministic modifier to indicate that when used in a deterministic evaluation context, any evaluation of the variable would have no side effect and would yield the same result.

Lazy and fuzzy features are valid only in the scope, where the variables' names are visible. For example, if a fuzzy variable is passed to a formal parameter declared without a modifier, it loses its fuzzy feature inside the called function. Similarly, if it is passed to a lazy formal parameter, it becomes lazy within the called function.

Whenever a lazy or fuzzy variable is assigned, the TE is required to save the lexical environment (the set of directly or indirectly referenced values and templates) valid at the time of the assignment, so that it is possible to resolve the expression at the time of evaluation of the lazy or fuzzy value or template. If the assignment was made on a lower scope than the evaluation, saving the lexical environment extends lifetime of the referenced variables defined on that lower scope.

Example

var @fuzzy integer v_fuzzy := 1;

var integer v_var;

var boolean v_condition := true;

if (v_condition) {

var integer v_local := 0;

v_fuzzy := v_local;

v_local := 10;

}

// although v_local is no longer valid at this point, v_fuzzy still evaluates to 10 because

// the lexical environment is available to the fuzzy variable:

v_var := v_fuzzy;

## 11.1	Value variables

A TTCN-3 value variable stores values. It is declared by the var keyword followed by a type identifier and a variable identifier. An initial value can be assigned at variable declaration.

It may be used at the right hand side as well as at the left hand side of assignments, in expressions, following the return keyword in bodies of functions with a return clause in their headers and may be passed to both value and template-type formal parameters.

Syntactical Structure

var [ ( @lazy | @fuzzy ) [ @deterministic ] ] Type

{ VarIdentifier [ ArrayDef ] [ ":=" Expression ] [ "," ] }+ [ ";" ]

Semantic Description

A value variable associates a name with the location of a value. A value variable may change its value during test execution several times. A value can be assigned several times to a value variable. The value variable can be referenced multiple times in a TTCN-3 module.

Restrictions

In addition to the general static rules of TTCN-3 given in clause 5, the following restrictions apply:

a)	Expression shall be of type Type.

b)	Value variables shall store values only.

c)	Value variables shall not be declared or used in a module definitions part (i.e. global variables are not supported in TTCN-3).

d)	Use of uninitialized value variables at other places than the left hand side of assignments, in return statements, or as actual parameters passed to formal parameters shall cause an error.

e)	The initialization or assignment of a fuzzy or lazy variable shall not contain function calls of functions with inout or out parameters. The called functions may use other functions with inout or out parameters internally.

f)	If a lazy or fuzzy value variable is used in deterministic contexts (i.e. during the evaluation of a snapshot or initialization of global non-fuzzy templates), it shall be declared @deterministic and the expression assigned to the variable shall fulfill the restrictions imposed on content of functions used in special places given in clause 16.1.4.

g)	The expression assigned to a lazy or fuzzy variable might contain a direct or indirect reference to this variable. Evaluation of such an expression shall cause a dynamic error.

h)	Using the dot notation (see clauses 6.2.1.1, 6.2.2.1 and 6.2.5.1) and index notation (see clauses 6.2.7 and 6.2.3.0) for referencing a field, alternative or element of an address value, which actual value is null shall cause an error.

i)	The expression shall evaluate to a value, which is at least partially initialized.

Examples

var integer v_myVar0;

var integer v_myVar1 := 1;

var boolean v_myVar2 := true, v_myVar3 := false;

var @lazy integer v_myLazyVar1 := v_myVar1+1;

var timer v_timer1;

timer t_myTimer1;	v_myVar1 := 2;

v_myVar1 := v_myLazyVar1; // v_myLazyVar1 evaluates to 2 + 1

v_myLazyVar1 := v_myLazyVar1 + 1;

v_myVar1 := v_myLazyVar1; // causes an error as v_myLazyVar1 references itself

v_timer1 := t_myTimer1;

## 11.2	Template variables

A TTCN-3 template variable stores templates. They are declared by the var template keyword followed by a type identifier and a variable identifier. An initial content can be assigned at declaration. In addition to values, template variables may also store matching mechanisms (see clause 15.7).

Template variables may be used on the right hand side as well as on the left hand side of assignments, following the return keyword in bodies of functions defining a template-type return value in their headers and may be passed as actual parameters to template-type formal parameters. It is also allowed to assign a template instance to a template variable or a template variable field.

Syntactical Structure

var TemplateModifier [ (@lazy | @fuzzy) [ @deterministic ] ]

Type { VarIdentifier [ ArrayDef ] ":=" TemplateBody [ "," ] }+ [ ";" ]

Semantic Description

A template variable associates a name with the location of a template or a value (as every value is also a template). 
A template variable may change its template during test execution several times. A template or value can be assigned several times to a template variable. The template variable can be referenced multiple times in a TTCN-3 module.

The content of a template variable can be restricted to the matching mechanisms specific value and omit in the same way as formal template parameters, see clause 5.4.1.2. The restriction template (omit) can be replaced by the shorthand notation omit.

NOTE 1:	String and list type templates can be concatenated, see clause 15.11.

Restrictions

In addition to the general static rules of TTCN-3 given in clause 5, the following restrictions apply:

a)	Template variables shall not be declared or used in a module definitions part (i.e. global variables are not supported in TTCN-3).

b)	When used on the right hand side of assignments template variables shall not be operands of TTCN-3 operators (see clause 7.1) and the variable on the left hand side shall be a template variable too.

c)	When accessing element of template variables either on the left hand side or on the right hand side of assignments, the rules given in clause 15.6 shall apply.

NOTE 2:	While it is not allowed to directly apply TTCN-3 operations to template variables, it is allowed to use the dot notation and the index notation to inspect and modify template variable fields.

d)	Use of uninitialized template variables at other places than the left hand side of assignments, in return statements, or as actual parameters passed to formal parameters shall cause an error.

e)	Void.

f)	If the template variable is restricted, then the template used to initialize it shall contain only the matching mechanisms as described in clause 15.8.

g)	Template variables, similarly to global and local templates, shall be fully specified in order to be used in sending and receiving operations.

h)	Restrictions on templates in clause 15 shall apply.

i)	The initialization or assignment of a fuzzy or lazy variable shall not contain function calls of functions with inout or out parameters. The called functions may use other functions with inout or out parameters internally.

j)	If a lazy or fuzzy template variable is used in deterministic contexts (i.e. during the evaluation of a snapshot or initialization of global non-fuzzy templates), it shall be declared @deterministic and the template body assigned to the variable shall fulfill the restrictions imposed on content of functions used in special places given in clause 16.1.4.

k)	Using the dot notation (see clauses 6.2.1.1, 6.2.2.1 and 6.2.5.1) and index notation (see clause 6.2.7) for referencing a field, alternative or element of an address value, which actual value is null shall cause an error.

l)	The template body at the right-hand side of the  assignment symbol shall evaluate to a value or template, which is type compatible with the variable being declared.

m)	The template body at the right-hand side of the  assignment symbol shall evaluate to an object that is at least partially initialized.

Examples

var template integer  v_myVarTemp1 := ?;

var template MyRecord v_myVarTemp2 := { field1 := true, field2 := * },
				    	  v_myVarTemp3 := { field1 := ?, field2 := v_myVarTemp1 };

var template @fuzzy float v_fuzzTemp1 := rnd(); // evaluated on every usage

var template @fuzzy MyRecord v_fuzzTemp2 := { rnd() < 0.5, float2int(rnd()) };

var template @lazy float LazyTemp1 := v_fuzzTemp1; // evaluates v_fuzzTemp1

var template @lazy MyRecord v_lazyTemp2 :=

{ v_lazyTemp1 < 0.5, float2int(v_fuzzTemp1) }; // evaluates v_lazyTemp1 and v_fuzzTemp1

v_lazyTemp2.field1 := true; // evaluates v_lazyTemp2 and overwrites field1 with true

# 12	Declaring timers

TTCN-3 provides a timer mechanism. Timers can be declared and used in test cases, functions and altsteps. Additionally, timers can be declared in component type definitions. These timers can be used in test cases, functions and altsteps which are running on the given component type.

A timer declaration may have an optional default duration value assigned to it. The timer shall be started with this value if no other value is specified. The timer value shall be a non-negative float value (i.e. greater than or equal to 0.0) where the base unit is seconds.

In addition to single timer instances, timer arrays can also be declared. Default duration(s) of the elements of a timer array shall be assigned using a value array. Default duration(s) assignment shall use the array value notation as specified in clause 15.9. If the default duration assignment is wished to be skipped for some element(s) of the timer array, it shall explicitly be declared by using the not used symbol ("-").

Syntactical Structure

timer { TimerIdentifier [ ArrayDef ] ":=" TimerValue [ "," ] } [ ";" ]

Semantic Description

Timers are local to components. A component can start and stop a timer, check if a timer is running, read the elapsed time of a running timer and process timeout events after timer expiration. The timer value is interpreted with a base unit of seconds.

A timer declared and started in scope units such as functions ceases to exist when the scope unit is left unless there is a constant, variable or parameter defined in the current or higher scope unit or in an activated altstep that contains a reference to it. In this case, the timer is kept as long as at least one constant, variable or parameter of the current or higher scope unit or an activated altstep contain a reference to it. If a timer ceases to exist, it stops running, will never timeout and cannot be referenced via the any timer or all timer constructs.

If not stated otherwise, timers have the same semantics as constants of a timer type.

NOTE:	Timers that ceased to exist do not contribute to the test behaviour.

Restrictions

In addition to the general static rules of TTCN-3 given in clause 5, the following restrictions apply:

- In case of a single timer, the default duration value shall resolve to a non-negative numerical float value 
(i.e. the value shall be greater or equal 0.0, infinity and not_a_number are disallowed).

- In case of a timer array, it shall resolve to an array of float values obeying to restriction a) above of the same size as the size of the timer array.

Examples

EXAMPLE 1:	Single timer

timer t_myTimer1 := 5E-3;

// declaration of the timer t_myTimer1 with the default value of 5ms

timer t_myTimer2;	// declaration of t_myTimer2 without a default timer value i.e. a value has

// to be assigned when the timer is started

EXAMPLE 2:	Timer array

timer t_mytimer1[5] := { 1.0, 2.0, 3.0, 4.0, 5.0 }

// all elements of the timer array get a default duration.

timer t_mytimer2[5] := { 1.0, -, 3.0, 4.0, 5.0 }

// the second timer (t_mytimer2[1]) is left without a default duration.

# 13	Declaring messages

One of the key elements of TTCN-3 is the ability to send and receive simple or complex messages over message-based ports defined by the test configuration (see clauses 9 and 21). These messages may be those explicitly concerned with testing the SUT or with the internal co-ordination and control messages specific to the relevant test configuration.

Messages are instances of types declared in the in/out/inout clauses of message port type definition.

Any type can be declared as type of a message in a message port type definition, i.e. values of any basic or structured type (see clauses 6.1 and 6.2) can be sent or received. Received messages can also be declared as a combination of value and matching mechanisms (see clause 15.5). Instances of messages can be declared by global, local or in-line templates (see clause 15) or being constructed and passed via variables or template variables (see clause 11) and parameters or template parameters (see clause 5.4).

Syntactical Structure

See syntactical structure of types (see clause 6).

Semantic Description

See semantic description of types (see clause 6).

Restrictions

No specific restrictions in addition to the general static rules of TTCN-3 given in clause 5.

Examples

// a structured, ordered message with two fields

type record ARecord { integer i, float f }

# 14	Declaring procedure signatures

Procedure signatures (or signatures for short) are needed for procedure-based communication. Procedure-based communication may be used for the communication within the test system, i.e. among test components, or for the communication between the test system and the SUT. In the latter case, a procedure may either be invoked in the SUT (i.e. the test system performs the call) or in the test system (i.e. the SUT performs the call).

Syntactical Structure

signature SignatureIdentifier

"(" { [ in | inout | out ] Type ValueParIdentifier [ ","] } ")"

[ ( return Type ) | noblock ]

[ exception "(" ExceptionTypeList ")" ]

Semantic Description

For all used procedures, i.e. procedures used for the communication among test components, procedures called from the SUT and procedures called from the test system, a procedure signature shall be defined in the TTCN-3 module.

TTCN-3 supports blocking and non-blocking procedure-based communication. By default, signature definitions without the noblock keyword are assumed to be used for blocking procedure-based communication.

Signature definitions may have parameters. Parameters shall be of data type only, i.e. of a basic type, a structured type thereof or a subtype thereof. Within a signature definition the parameter list may include parameter identifiers, parameter types and their direction, i.e. in, out, or inout. The direction inout and out indicate that these parameters are used to retrieve information from the remote procedure.

NOTE 1:	The direction of the parameters is as seen by the called party rather than the calling party.

A remote procedure may return a value after its termination. The type of the return value shall be specified by means of a return clause in the corresponding signature definition.

Exceptions that may be raised by remote procedures are represented in TTCN-3 as values of a specific type. Therefore templates and matching mechanisms can be used to specify or check return values of remote procedures.

NOTE 2:	The conversion of exceptions generated by or sent to the SUT into the corresponding TTCN-3 type or SUT representation is tool and system specific and therefore beyond the scope of the present document.

The exceptions are defined in the form of an exception list included in the signature definition. This list defines all the possible different types associated with the set of possible exceptions (the meaning of exceptions themselves will usually only be distinguished by specific values of these types).

Restrictions

In addition to the general static rules of TTCN-3 given in clause 5, the following restrictions apply:

- Signature definitions for non-blocking communication shall use the noblock keyword, shall only have in parameters and shall have no return value but may raise exceptions.

- Signature parameters shall be of a data type.

Examples

signature MyRemoteProcOne ();			// MyRemoteProcOne will be used for blocking

// procedure-based communication. It has neither

// parameters nor a return value.

signature MyRemoteProcTwo () noblock;	// MyRemoteProcTwo will be used for non blocking

// procedure-based communication. It has neither

// parameters nor a return value.

signature MyRemoteProcThree (in integer Par1, out float Par2, inout integer Par3);

// MyRemoteProcThree will be used for blocking procedure-based communication. The procedure

// has three parameters: Par1 an in parameter of type integer, Par2 an out parameter of

// type float and Par3 an inout parameter of type integer.

signature MyRemoteProcFour (in integer Par1) return integer;

// MyRemoteProcFour will be used for blocking procedure-based communication. The procedure

// has the in parameter Par1 of type integer and returns a value of type integer after its

// termination

signature MyRemoteProcFive (inout float Par1) return integer

exception (ExceptionType1, ExceptionType2);

// MyRemoteProcFive will be used for blocking procedure-based communication. It returns a

// float value in the inout parameter Par1 and an integer value, or may raise exceptions of

// type ExceptionType1 or ExceptionType2

signature MyRemoteProcSix (in integer Par1) noblock

exception (integer, float);

// MyRemoteProcSix will be used for non-blocking procedure-based communication. In case of

// an unsuccessful termination, MyRemoteProcSix raises exceptions of type integer or float.

# 15	Declaring templates

## 15.0	General

Templates are used to either transmit a set of distinct values or to test whether a set of received values matches the template specification. Templates can be defined globally or locally.

Templates provide the following possibilities:

- they are a way to organize and to re-use test data, including a simple form of inheritance;

- they can be parameterized;

- they allow matching mechanisms;

- they can be used with either message-based or procedure-based communications.

Within a template values, ranges and matching attributes can be specified and then used in both message-based and procedure-based communications. Templates may be specified for any TTCN-3 type or procedure signature. The type-based templates are used for message-based communications and the signature templates are used in procedure-based communications.

A template can be declared fuzzy using the @fuzzy modifier.

NOTE:	Using a fuzzy template from a non-fuzzy template causes evaluation of the fuzzy template. Thus, for unparameterized non-fuzzy templates, the result of the used fuzzy templates will stay the same for every usage.

A fuzzy template can be declared deterministic using the @deterministic modifier. A deterministic template shall be evaluated to the same result in the same deterministic evaluation context whenever it is evaluated.

A modified template declaration (see clause 15.5) specifies only the fields to be changed from the base template, i.e. it is a partial specification.

Restrictions

In addition to the general static rules of TTCN-3 given in clause 5, the following restrictions apply:

- Templates shall be specified only for component data types and procedure signatures.

- Void.

- The expression or template body initializing a template shall evaluate to a value or template, which is type compatible with the template being declared.

- The expression or template body initializing a template shall evaluate to a value or a template that is at least partially initialized or to a matching mechanism.

- The body of a fuzzy template shall not contain function calls of functions with inout or out parameters. The called functions may use other functions with inout or out parameters internally.

- Fuzzy features are valid only in the scope, where the templates' names are visible. For example, if a fuzzy template is passed to a formal template parameter declared without a modifier, it loses its fuzzy feature inside the called function.

- For a fuzzy template that is declared deterministic the template body shall fulfill the restrictions imposed on content of functions used in special places given in clause 16.1.4.

Examples

type record MyRecord {

default def

}

type union MyUnion {

integer choice1,

MyRecord choice2

}

template MyUnion m_integerChosen := { choice1 := 5 }

// shall cause an error as the type MyUnion contains MyRecord, which includes

// a field of default type.

external function fx_garble(charstring p_str) return p_str;

template @fuzzy charstring m_fuzzy := fx_garble("foobar"); // every usage of m_fuzzy re-

// evaluates the function call

## 15.1	Declaring message templates

Instances of messages with actual values may be specified using templates. A template can be thought of as being a set of instructions to build a message for sending or to match a received message.

Syntactical Structure

See syntactical structure of global and local templates (see clause 15.3) and of in-line templates (see clause 15.4).

Semantic Description

A template used in a send operation defines a complete set of field values comprising the message to be transmitted over a port.

NOTE:	For sending templates, omitting an optional field is considered to be a value notation rather than a matching mechanism.

A template used in a receive, trigger or check operation defines a data template against which an incoming message is to be matched. Matching mechanisms, as defined in clauses 15.7 and 15.8 and in annex B, may be used in receive templates. No binding of the incoming values to the template shall occur.

Restrictions

In addition to restrictions in clause 15.0, the following restrictions apply:

a)	At the time of a send operation, the used template shall be completely initialized and all fields shall resolve to actual values or to omit and no other matching mechanisms shall be used in the template fields, neither directly nor indirectly.

At the time of a receiving operation, the matching template shall be completely initialized.

b)	Optional fields of record and set templates or template fields can be initialized explicitly or implicitly. For implicit initialization of the optional fields of a template or a template field, an optional attribute with the value "implicit omit" (see clause 27.7) shall be associated with it either directly or via the attribute distribution (scoping) mechanism (see clause 27.1.1).

Examples

EXAMPLE 1:	Template for sending messages

// Given the message definition

type record MyMessageType

{

integer 	field1	optional,

charstring 	field2,

boolean 	field3

}

// a message template could be

template MyMessageType m_myTemplate:=

{

field1 := omit,

field2 := "My string",

field3 := true

}

// and a corresponding send operation could be

myPCO.send(m_myTemplate);

EXAMPLE 2:	Template for receiving messages

// Given the message definition

type record MyMessageType

{

integer 	field1 	optional,

charstring	field2,

boolean		field3

}

// a message template might be

template MyMessageType mw_myTemplate:=

{

field1 := ?,

field2 := pattern "abc*xyz",

field3 := true

}

// and a corresponding receive operation could be

myPCO.receive(mw_myTemplate);

EXAMPLE 3:	Template for receiving messages

// When used in a receiving operation this template will match any integer value

template integer mw_myTemplate := ?;

// This template will match only the integer values 1, 2 or 3

template integer mw_myTemplate := (1, 2, 3);

## 15.2	Declaring signature templates

Instances of procedure parameter lists with actual values may be specified using templates. Templates may be defined for any procedure by referencing the associated signature definition.

Syntactical Structure

See syntactical structure of global and local templates (see clause 15.3) and of in-line templates (see clause 15.4).

Semantic Description

A signature template defines the values and matching mechanisms of the procedure parameters only, but not the return value. The values or matching mechanisms for a return have to be defined within the reply (see clause 22.3.3) or getreply operation (see clause 22.3.4).

A template used in a call or reply operation defines a complete set of field values for all in and inout parameters. At the time of the call operation, all in and inout parameters in the template shall resolve to actual values, no matching mechanisms shall be used in these fields, either directly or indirectly. Any template specification for out parameters is simply ignored, therefore it is allowed to specify matching mechanisms for these fields, or to omit them (see annex B).

A template used in a getcall operation defines a data template against which the incoming parameter fields are matched. Matching mechanisms, as defined in annex B, may be used in any templates used by this operation. No binding of incoming values to the template shall occur. Any out parameters shall be ignored in the matching process.

Restrictions

In addition to restrictions in clause 15.0, the following restrictions apply:

- At the time of a call, reply and raise operation, the used template shall be completely initialized and all in/inout parameters in a call, all out/inout parameters in a reply or raise operation shall resolve to specific values or to omit and no other matching mechanisms shall be used for these parameters, neither directly nor indirectly.

- The NotUsedSymbol shall only be used in signature templates for parameters which are not relevant and in modified template declarations and modified in-line templates to indicate no change for the specified field or element.

At the time of a getcall, getreply and catch operation, the matching template shall be completely initialized.

Optional fields of record and set parameters or parameter fields can be initialized explicitly or implicitly. For implicit initialization of a parameter or a parameter field, an optional attribute with the value "implicit omit" (see clause 27.7) shall be associated with it either directly or via the attribute distribution (scoping) mechanism (see clause 27.1.1).

Examples

EXAMPLE 1:	Templates for invoking and accepting procedures:

// signature definition for a remote procedure

signature RemoteProc(in integer Par1, out integer Par2, inout integer Par3) return integer;

// example templates associated to defined procedure signature

template RemoteProc s_template1:=

{

Par1 :=	1,

Par2 :=	2,

Par3 :=	3

}

template RemoteProc s_template2:=

{

Par1 := 1,

Par2 := ?,

Par3 := 3

}

template RemoteProc s_template3:=

{

Par1 := 1,

Par2 := ?,

Par3 := ?

}

template RemoteProc s_template4:=?;

EXAMPLE 2:	In-line templates for invoking procedures:

// Given example 1 in this clause

// Valid invocation since all in and inout parameters have a distinct value

myPCO.call(RemoteProc:s_template1);

// Valid invocation since all in and inout parameters have a distinct value

myPCO.call(RemoteProc:s_template2);

// Invalid invocation causing an error

// since the inout parameter Par3 has a matching attribute not a value

myPCO.call(RemoteProc:s_template3);

// Templates never return values. In the case of Par2 and Par3 the values returned by the

// call operation shall be retrieved using an assignment clause at the end of the call statement

EXAMPLE 3:	In-line templates for accepting procedure invocations:

// Given example 1 in this clause

// Valid getcall, it will match if Par1 == 1 and Par3 == 3

myPCO.getcall(RemoteProc:s_template1);

// Valid getcall, it will match if Par1 == 1 and Par3 == 3

myPCO.getcall(RemoteProc:s_template2);

// Valid getcall, it will match on Par1 == 1 and Any value of Par3

myPCO.getcall(RemoteProc:s_template3);

EXAMPLE 4:	In-line templates for accepting procedure replies:

// Given example 1 in this clause

// Valid getreply, in parameters will be ignored, matches if return value is 4

myPCO.getreply(RemoteProc:s_template2 value 4);

// Valid getreply, accepting any reply for RemoteProc

myPCO.getreply(RemoteProc:?);

// Valid getreply, also accepting any reply for RemoteProc

myPCO.getcall(RemoteProc:s_template4 value ?);

## 15.3	Global and local templates

TTCN-3 allows defining global templates and local templates.

Syntactical Structure

template [ TemplateRestriction ] [ @fuzzy ] [ @deterministic ] [ @abstract ] Type TemplateIdentifier

["(" TemplateFormalParList ")"] [ modifies TemplateRef ] ":=" BaseTemplateBody

NOTE 1:	The optional restriction part is covered by clause 15.8.

Semantic Description

Global templates shall be defined in the module definitions part. Local templates shall be defined in testcases, functions, altsteps or statement blocks. Both global and local templates shall adhere to the scoping rules specified in clause 5.

Both global and local templates can be parameterized. The actual parameters of a template can include values and templates. The rules for formal and actual parameter lists shall be followed as defined in clause 5.2.

Both global and local templates are initialized at the place of their declaration. This means, all template fields which are not affected by parameterization shall receive a value or matching mechanism. Template fields affected by parameterization are initialized at the time of template use.

If functions are used for the initialization of module parameters, it is strongly advised to adhere to the rules defined in clause 16.1.4. Not following these rules may cause non-deterministic test executions.

At the time of their use (e.g. in communication operations send, receive, call, getcall, etc.), it is allowed to change template fields by in-line modified templates, to pass in values via value parameters as well as to pass in templates via template parameters. The effects of these changes on the values of the template fields do not persist in the template subsequent to the corresponding communication event.

Restrictions

In addition to restrictions in clause 15.0, the following restrictions apply:

a)	The dot notation such as myTemplateId.fieldId shall not be used to set or retrieve values in templates in communication events. The "->" symbol shall be used for this purpose (see clause 23).

b)	Restrictions on referencing elements of templates or template fields are described in clause 15.6.

c)	There exist a number of restrictions on the functions used in expressions when specifying templates or template fields; these are specified in clause 16.1.4.

d)	After completing initialization, global and local templates that do not contain the @abstract modifier shall be fully initialized.

NOTE 2: 	Initialization of templates without parameters is completed at the place of their declaration. Initialization of parameterized templates is completed at the time of their use.

Examples

// The template

template MyMessageType mw_myTemplate (integer p_myFormalParam):=

{

field1 := p_myFormalParam,

field2 := pattern "abc*xyz",

field3 := true

}

// could be used as follows

pco1.receive(mw_myTemplate(123));

## 15.4	In-line Templates

Templates can be specified directly at the place they are used. Such templates are called in-line templates.

Syntactical Structure

[ Type ":" ] TemplateBody

NOTE 1:	An in-line template is an argument of a communication operation or an actual parameter of a testcase, function or altstep call, i.e. it is always placed within parenthesis and potentially separated with a comma.

Semantic Description

In-line templates can be defined directly at the place of its use.

In-line templates do not have names, therefore they cannot be referenced or reused. The lifetime of in-line templates is the TTCN-3 statement (an assignment, a testcase/function/alstep invocation, a return from a function, a communication operation), where they are defined.

Restrictions

In addition to restrictions in clause 15.0, the following restrictions apply:

a)	Void.

b)	The type field should only be omitted when the type is implicitly unambiguous.

NOTE 2:	For literal in-line templates, the following types may be omitted: integer, float, boolean, bitstring, hexstring, octetstring.

NOTE 3:	Types of constants, parameters and variables of the actual scope are always unambiguous and can hence always be omitted.

c)	In-line templates containing instead of values or inside values matching mechanisms (see clause 15.7) can only be defined in arguments of receiving communication operations (i.e. receive, trigger, check, getcall, getreply and catch), in arguments of the match and select case operations, in actual template parameters, at the right hand side of assignments (when there is a template variable at the left hand side of the assignment) and in return statements of template returning functions. In-line templates not containing matching mechanisms can be defined wherever values are allowed.

d)	When used in communication operations, the type of the in-line template shall be in the port list over which the template is sent or received. In the case where there is an ambiguity between the listed type and the type of the value provided (e.g. through subtyping) then the type name of the in-line template shall be included in the communication operation.

e)	There exist a number of restrictions on the functions used in expressions when specifying templates or template fields; these are specified in clause 16.1.4.

Examples

myPCO.receive(charstring:"abcxyz");

## 15.5	Modified templates

In cases where small changes are needed to specify a new template, it is possible to specify a modified template. A modified template specifies modifications to particular fields of the original template, either directly or indirectly. As well as creating explicitly named modified templates, TTCN-3 allows the definition of in-line modified templates.

Syntactical Structure

Global or local modified template:

template [ TemplateRestriction ] [ @fuzzy ] [ @deterministic ] [ @abstract ]

Type TemplateIdentifier

["(" TemplateFormalParList ")"] modifies TemplateRef ":=" BaseTemplateBody

NOTE 1:	The optional restriction part is covered by clause 15.8.

In-line modified template:

modifies BaseTemplateBody ":=" BaseTemplateBody

Semantic Description

The modifies keyword denotes the parent template from which the new modified template shall be derived. This parent template may be either an original template or a modified template or also a matching mechanism.

In case that the BaseTemplateBody on the right hand side of the modified template contains references to variables, these are evaluated before the modification and any uninitialized fields or elements in these variables are treated as unspecified, i.e. as if specified with the not used symbol "-" for the following modification algorithm.

The modifications occur in a linked fashion, eventually tracing back to the original template:

a)	In case of templates, template fields or list elements of simple types, union and enumerated types, the matching mechanism specified in the modified template is simply replacing its corresponding content in its parent.

b)	For templates, template fields and elements of record and set types, if a record or set field and its corresponding matching mechanism is specified in the modified template, then the specified matching mechanism replaces the one specified in the corresponding field of the parent template. If a record or set field or its corresponding matching mechanism is – implicitly or explicitly by using the not used symbol "-" - left unspecified in the modified template, then the matching mechanism in the corresponding field of the parent template shall be used. When the field to be modified is nested within a template field which is a structured field itself, no other field of the structured field is changed apart from the explicitly denoted one(s).

c)	For templates, template fields and elements of record of and set of types, the above rules specified for records and sets apply with the following deviations:

if the value list notation is used, only the number of elements listed in the modified template is inherited from the parent (i.e. the list is truncated at the last element of the list notation in the modified template);

when individual values of a modified template or a modified template field of record of or set of type wished to be changed, and only in these cases, the index assignment notation may also be used, where the left hand side of the assignment is the index of the element to be altered.

In case of record of and set of types first apply rule (c) to the complete structure (e.g. truncation) than apply further rules for the remaining individual type structure elements (see example 3).

Formal value or template parameters of modified templates inherit the default value or respectively template of the corresponding parameter of their parent templates only, if this is denoted by the dash (don't change) symbol at the place of the parameters' default value or respectively template.

Modified templates may also be restricted. Template restrictions are specified in clause 15.8.

A modified template may also be declared fuzzy using the @fuzzy modifier.

NOTE 2:	If a fuzzy modified template modifies a non-fuzzy unparameterized template, the inherited fields before modification will be the same for every evaluation of the fuzzy template.

Restrictions

In addition to the general static rules of TTCN-3 given in clause 5, the following restrictions apply:

a)	A modified template shall not refer to itself, either directly or indirectly, i.e. recursive derivation is not allowed.

b)	If a base template has a formal parameter list, the following rules apply to all modified templates derived from that base template, whether or not they are derived in one or several modification steps:

1)	the derived template shall not omit parameters and change types or names of parameters defined at any of the modification steps between the base template and the actual modified template;

2)	a template parameter restriction of a derived template specified at any of the modification steps between the base template and the actual modified template can be changed to a stricter one (see clause 15.8);

3)	a derived template can have additional (appended) parameters if wished;

4)	if the dash (don't change) symbol is used at the place of a default value or default template, the corresponding parameter of the parent template shall have a valid default value or default template, either assigned directly or inherited. If not, this shall cause an error.

c)	Restrictions on referencing elements of templates or template fields are described in clause 15.6: for modified templates the rules for the left hand side of assignments apply.

d)	Limitations on template restrictions described in clause 15.8 shall apply.

e)	After completing initialization, modified global and local templates that do not contain the @abstract modifier shall be fully initialized.

NOTE 3:	Initialization of modified templates without parameters is completed at the place of their declaration. Initialization of modified parameterized templates is completed at the time of their use.

Examples

EXAMPLE 1:	Modifying record templates (non-embedded case):

// Modifying records

type record MyRecordType

{

integer field1 optional,

charstring field2,

boolean field3

}

template MyRecordType m_myRecTemplate1 :=

{

field1 := 123,

field2 := "A string",

field3 := true

}

// then writing

template MyRecordType m_myRecTemplate2 modifies m_myRecTemplate1 :=

{

field1 := omit,					// field1 is optional but present in m_myTemplate1

field2 := "A modified string"

// field3 is unchanged

}

// is the same as writing

// template MyRecordType m_myRecTemplate2 :=

// {

//	  field1 := omit,

//	  field2 := "A modified string",

//	  field3 := true

// }

template MyRecordType m_myRecTemplate3 modifies m_myRecTemplate1 := {omit, "A modified string"}

//field3 is implicitly left unchanged;

//m_myRecTemplate3 has the same content as m_myRecTemplate2

template MyRecordType m_myRecTemplate4 modifies m_myRecTemplate1 := {omit,"A modified string",-}

//field3 is explicitly left unchanged;

//m_myRecTemplate4 has the same content as m_myRecTemplate2 and m_myRecTemplate3

EXAMPLE 2:	Modifying record of templates (non-embedded case):

type record of integer MyRecordOfType;

template MyRecordOfType m_myBaseTemplate := { 0, 1, 2, 3, 4, 5, 6, 7, 8, 9 };

template MyRecordOfType m_myRecOfTemplate1 modifies m_myBaseTemplate :=

{ -, -, 3, 2, -, -, -, -, -, - };
	// m_myRecOfTemplate1 contains { 0, 1, 3, 2, 4, 5, 6, 7, 8, 9 }

template MyRecordOfType m_myRecOfTemplate2 modifies m_myBaseTemplate := { -, -, 3, 2 };
	// m_myRecOfTemplate2 replaces m_myBaseTemplate with: { 0, 1, 3, 2 };

// elements 5 to 10 of m_myBaseTemplate are truncated

template MyRecordOfType m_myRecOfTemplate3 modifies m_myBaseTemplate := { [2] := 3, [3] := 2 };
	// m_myRecOfTemplate3 has the same content as m_myMod1Template: { 0, 1, 3, 2, 4, 5, 6, 7, 8, 9 }

EXAMPLE 3:	Modifying embedded record and record of templates:

//Modifying a record embedded in a record of

type record of record {
	  integer a,
	  integer b
	} MyListType

template MyListType mw_myBaseListTemplate := { ?, { a := 1, b := 2 }, ?, { a := 3, b := 4 } }

template MyListType mw_myListTemplate1 modifies mw_myBaseListTemplate := { [1] := { a := 42 } }
	//Content of field ″a″ of the second element is modified,

//the content of mw_myListTemplate1 is: { ?, { a := 42, b := 2 }, ?, { a := 3, b := 4 } }

template MyListType mw_myListTemplate2 modifies mw_myBaseListTemplate := { -, { a := 42 } ,- }
	//Content of field ″a″ of the second element is modified, and the

//record of is truncated after the third element: { ?, { a := 42, b := 2 }, ? }

EXAMPLE 4:	Modified in-line template:

// Given

template MyRecordType m_setup :=

{

field1 := 75,

field2 := "abc",

field3 := true

}

// Could be used to define an in-line modified template of Setup

// pco1.send (modifies m_setup := {field1:= 76});

EXAMPLE 5:	Modified parameterized template:

// Given

template MyRecordType m_myTemplate1(integer p_myPar):=

{

field1 := p_myPar,

field2 := "A string",

field3 := true

}

// then a modification could be

template MyRecordType m_myTemplate2(integer p_myPar) modifies m_myRecTemplate1 :=

// field1 is parameterized in m_myTemplate1 and remains also parameterized in m_myTemplate2

{

field2 := "A modified string"

}

EXAMPLE 6:	Default values of modified parameterized templates:

// Given

template MyRecordType m_myTemplate11 (integer p_int := 5 ):=

// p_int has the default value 5

{

field1 := p_int,

field2 := "A string",

field3 := true

}

// then possible template modifications are

template MyRecordType m_myTemplate12(integer p_int) modifies m_myTemplate11 :=

// p_int had a default value in m_myTemplate11 but has none in this template

{

field2 := "B string"

}

template MyRecordType m_myTemplate13(integer p_int := 0) modifies m_myTemplate12 := { }

// p_int has the default value 0

// no change is made to the template's content, but only to the default value of p_int

template MyRecordType m_myTemplate14(integer p_int := - ) modifies m_myTemplate13 :=

// p_int inherits the default value 0 from its parent m_myTemplate13

{

field2 := "C string"

}

template MyRecordType m_myTemplate15(integer p_int := - ) modifies m_myTemplate14 :=

// p_int inherits the default value 0 from m_myTemplate13 via m_myTemplate14

{

field2 := "D string"

}

template MyRecordType m_myTemplate16(integer p_int) modifies m_myTemplate15 := { }

// p_int has no default value; no change in the template's content

template MyRecordType m_myTemplate17(integer p_int := - ) modifies m_myTemplate16 :=

// causes an error as p_int has no default value in the parent template m_myTemplate16

{

field2 := "E string"

}

EXAMPLE 7:	Modifies with variables:

var template MyRecordType v_modification :=

{

field2 := "G string" // field1/field3 are uninitialized

}

var template MyRecordType v_myTemplate := modifies m_myTemplate1(5) := v_modification;

// no modification of field1/field3

// results in { field1 := 5, field2 := "G string", field3 := true }

## 15.6	Referencing elements of templates or template fields

### 15.6.0	General

This clause defines rules and restrictions for referencing elements of templates or template fields in case of unrestricted templates or templates with the present restriction. When referencing elements of templates or templates fields with the value or omit restriction, the rules for referencing elements of values are used.

### 15.6.1	Referencing individual string elements

It is not allowed to reference individual string elements inside templates or template fields. Instead, the substr function (see clause C.4.2) shall be used.

EXAMPLE:

var template charstring v_char1 := "MYCHAR";

var template charstring v_char2;

v_char2 := v_char1[1];

// shall cause an error as referencing individual string elements is not allowed

### 15.6.2	Referencing record and set fields

Both templates and template variables allow referencing sub-fields inside a template definition using the dot notation. However, the referenced field may be a subfield of a structured field to which a matching mechanism is assigned. This clause provides rules for such cases:

a)	Omit, AnyValueOrNone, template lists and complemented lists: referencing a subfield within a structured field to which Omit, AnyValueOrNone, a template list or a complemented list is assigned, at the right hand side of an assignment, shall cause an error.
When referencing a subfield within a structured field to which AnyValueOrNone or omit is assigned, at the left hand side of an assignment, the structured field is implicitly set to be present, it is expanded recursively up to and including the depth of the referenced subfield. During this expansion an AnyValue shall be assigned to mandatory subfields and AnyValueOrNone shall be assigned to optional subfields. After this expansion the value or matching mechanism at the right hand side of the assignment shall be assigned to the referenced subfield.
When referencing a subfield within a structured field to which template lists or complemented template lists are assigned, at the left hand side of an assignment, shall cause an error.

EXAMPLE 1:

type record R1 {

integer f1 optional,

R2      f2 optional

}

type record R2 {

integer g1,

R2      g2 optional

}

:

var template R1 v_r1 := {

f1 := 5,

f2 := omit

}

var template R2 v_r2 := v_r1.f2.g2;

// causes an error as omit is assigned to v_r1.f2

v_r1.f2 := *;

v_r2 := v_r1.f2.g2;

// causes an error as * is assigned to v_r1.f2

v_r1 := ({f1:=omit, f2:={g1:=0, g2:=omit}},{f1:=5, f2:={g1:=1, g2:={g1:=2, g2:=omit}}});

v_r2 := v_r1.f2;

v_r2 := v_r1.f2.g2;

v_r2 := v_r1.f2.g2.g2;

// all these assignments cause error as a template list is assigned to v_r1

v_r1 :=

complement({f1:=omit, f2:={g1:=0, g2:=omit}},{f1:=5, f2:={g1:=1, g2:={g1:=2, g2:=omit}}});

v_r2 := v_r1.f2;

v_r2 := v_r1.f2.g2;

v_r2 := v_r1.f2.g2.g2;

// all these assignments cause errors as a complemented list is assigned to v_r1

b)	AnyValue: when referencing a subfield within a structured field to which AnyValue is assigned, at the right hand side of an assignment, AnyValue shall be returned for mandatory subfields and AnyValueOrNone shall be returned for optional subfields.
When referencing a subfield within a structured field to which AnyValue is assigned, at the left hand side of an assignment, the structured field is implicitly expanded recursively up to and including, the depth of the referenced subfield. During this expansion an AnyValue shall be assigned to mandatory subfields and AnyValueOrNone shall be assigned to optional subfields. After this expansion the value or matching mechanism at the right hand side of the assignment shall be assigned to the referenced subfield.

EXAMPLE 2:

v_r1 := {f1:=0, f2:=?}

v_r2 := v_r1.f2.g2;

// after the assignment v_r2 will be {g1:=?, g2:=*}

v_r1.f2.g2.g2 := ({g1:=1, g2:=omit},{g1:=2, g2:=omit});

// first the field v_r1.f2 has hypothetically be expanded to {g1:=?,g2:={g1:=?,g2:=*}}
		// thus after the assignment v_r1 will be:
		//   {f1:=0, f2:={g1:=?,g2:={g1:=?,g2:=({g1:=1, g2:=omit},{g1:=2, g2:=omit})}}}

c)	Ifpresent attribute: referencing a subfield within a structured field to which the ifpresent attribute is attached, shall cause an error (irrespective of the value or the matching mechanism to which ifpresent is appended).

d)	Special value null: referencing a field of an address type, which actual value is null shall cause an error.

### 15.6.3	Referencing record of and set of elements

Both templates and template variables allow referencing elements of a record of, array or set of template or field using the index notation. However, a matching mechanism may be assigned to the template or field within which the element is referenced. This clause provides rules on handling such cases:

a)	Omit: referencing an element within a record of, set of or array field to which omit is assigned shall follow the rules specified in clause 6.2.3.

b)	Template lists, complemented lists, subset and superset: referencing an element within a record of or set of field to which a complemented list, a subset or a superset is assigned, shall cause an error.

EXAMPLE 1:

type record of integer RoI;

:

var template RoI v_roI;

var template integer v_int;

v_roI := ({},{0},{0,0},{0,0,0});

v_int := t_RoI[0];

// shall cause an error as template list is assigned to v_roI

c)	AnyValue: when referencing an element of a record of or set of template or field to which AnyValue is assigned (without a length attribute), at the right hand side of an assignment, AnyValue shall be returned. If a length attribute is attached to the AnyValue , the index of the reference shall not violate the length attribute.
When referencing an element within a record of or set of template or field to which AnyValue is assigned (without a length attribute), at the left hand side of an assignment, the value or matching mechanism at the right hand side of the assignment shall be assigned to the referenced element, AnyElement shall be assigned to all elements before the referenced one (if any) and a single AnyElementsOrNone shall be added at the end. When a length attribute is attached to AnyValue, the attribute shall be conveyed to the new template or field transparently. The index shall not violate type restrictions in any of the above cases.

EXAMPLE 2:

type record of integer RoI;

type record of RoI RoRoI;

:

var template RoI v_roI;

var template RoRoI v_roRoI;

var template integer v_int;

:

v_roI := ?;

v_int := v_roI[5];

// after the assignment v_int will be AnyValue(?);

v_roRoI := ?;

v_roI := v_roRoI[5];

// after the assignment v_roI will be AnyValue(?);

v_int := v_roRoI[5].[3];

// after the assignment v_int will be AnyValue(?);

v_roI := ? length (2..5);

v_int := v_roI[3];

// after the assignment v_int will be AnyValue(?);

v_int := v_roI[5];

// shall cause an error as the referenced index is outside the length attribute
		// (note that index 5 would refer to the 6th element);

v_roRoI[2] := {0,0};

// after the assignment v_roRoI will be {?,?,{0,0},*};

v_roRoI[4] := {1,1};

// after the assignment v_roRoI will be {?,?,{0,0},?,{1,1},*};

v_roI[0] := -5;

// after the assignment v_roI will be {-5,*} length(2..5);

v_roI := ? length (2..5);

v_roI[1] := 1;

// after the assignment v_roI will be {?,1,*} length(2..5);

v_roI[3] := ?;

// after the assignment v_roI will be {?,1,?,?,*} length(2..5);

v_roI[5] := 5;

// after the assignment v_roI will be {?,1,?,?,?,5,*} length(2..5); note that v_roI

// becomes an empty set but that shall cause no error;

d)	AnyValueOrNone: referencing an element within a record of, set of or array field to which AnyValueOrNone with or without a length attribute is assigned on the right hand side of an assignment shall cause an error. When referencing an element within a record of, set of or array field to which AnyValueOrNone is assigned on the left hand side of an assignment, the rules for AnyValue shall apply (see item c) for more details).

EXAMPLE 3:

type record of integer RoI;

type record R

{

RoI	field1 optional

}

:

var template R mw_t1 := { field1 := * };

var template integer mw_t2;

mw_t1.field1[2] := 2; // after the assignment, mw_t1 will be { field1 := { ?, ?, 2, * } }

mw_t1.field1 := *;

mw_t2 := mw_t1.field1[0];

// shall cause an error as mw_t1.field1 contains AnyValueOrNone

e)	Permutation: when referencing an element of a record of template or field, which is located inside a permutation (based on its index), this shall cause an error. Indexes of elements sheltered by a permutation shall be determined based on the number of permutation elements. AnyElementsOrNone as a permutation element causes that the permutation shelters all record of element indexes.

EXAMPLE 4:

v_roI:= {permutation(0,1,3,?),2,?};

v_int := v_roI[5];

// after the assignment v_int will be AnyValue(?)

v_roI:= {permutation(0,1,3,?),2,*};

v_int := v_roI[5];

// after the assignment v_int will be * (AnyValueOrNone)

v_int := v_roI[2];

// causes error as the third element (with index 2) is inside permutation

v_roI:= {permutation(0,1,3,*),2,?};

v_int := v_roI[5];

// causes error as the permutation contains AnyValueOrNone(*) that is able to

// cover any record of indexes

f)	Ifpresent attribute: referencing an element within a record of or set of field to which the ifpresent attribute is attached, shall cause an error (irrespective of the value or the matching mechanism to which ifpresent is appended).

g)	AnyElementsOrNone: when referencing an element of a record of or set of template or field that contains AnyElementsOrNone, the result of an operation is dependent on the position of AnyElementsOrNone, the referenced index and length attributes attached to AnyElementsOrNone.

When resolving the reference, a transformed form of the record of or set of template is used. The transformed form is equal to the original value where all occurrences of AnyElementsOrNone with a length restriction are replaced with a sequence of AnyElements of the same size as the lower bound. If the lower bound is greater than the upper bound, the sequence shall be followed by a single AnyElementsOrNone symbol with a length restriction. The lower bound of this restriction is zero and the upper bound is the difference between the lower and upper bound of the original restriction.

EXAMPLE 5:

type record of integer RoI;
template RoI mw_roI := {1, * length(2), 5}; 	// transformed form: {1, ?, ?, 5}
template RoI mw_roI := {1, * length(1..3), 5};  // transformed form: {1, ?, * length(0..2), 5}

h)	Special value null: referencing an element of an address type, which actual value is null shall cause an error.

When the reference is used at the right hand side of the assignment, the following applies:

If positions of all AnyElementsOrNone matching symbols in the transformed form are greater than the position of the referenced item, rules from the clause 6.2.3.2 are used for resolving the reference.

EXAMPLE 6:

type record of integer RoI;
:
var template RoI v_roI := {1, 2, * length(2), 5};	
	// transformed form: {1, 2, ?, ?, 5}
var template integer v_int;
v_int := v_roI[1]; // after the assignment, v_int will be 2
v_int := v_roI[2]; // after the assignment, v_int will be ?

If the position of the referenced item is greater or equal to the position of any AnyElementsOrNone symbol in the transformed template, an error is generated.

EXAMPLE 7:

type record of integer RoI;
:
var template RoI v_roI := {1, 2, *, 5};	
var template integer v_int := v_roI[3]; // produces an error

v_roI := {1, 2, *};	
v_int := v_roI[2]; 	// produces an error

When the reference is used at the left hand side of the assignment, the following applies:

If positions of all AnyElementsOrNone matching symbols in the transformed form are greater than the position of the referenced item the following rules are used. If the referenced item is not a result of transformation, the value or matching symbol at the right hand side of the assignment shall replace the referenced symbol in the original template. If the referenced element was a result of transformation, then the AnyValueOrNone symbol in the original template is replaced with its transformed form and the assignment is performed afterwards.

EXAMPLE 8:

type record of integer RoI;
:
var template RoI v_roI := {1, 2, * length(2), 5};	
	// transformed form: {1, 2, ?, ?, 5}
v_roI [1] := 10; 	// after the assignment, t_RoI will be {1, 10, * length(2), 5}
v_roI [2] := 3; 	// after the assignment, t_RoI will be {1, 10, 3, ?, 5}

If the position of the referenced item is greater or equal to the position of any AnyElementsOrNone symbol in the transformed form and this AnyElementsOrNone symbol is not the last element in the template, an error is generated.

EXAMPLE 9:

type record of integer RoI;
:
var template RoI v_roI:= {1, 2, *, 5};	
v_roI[3] := 4; // produces an error

If the position of the referenced item is greater or equal to the position of an AnyElementsOrNone symbol in the transformed form and this AnyElementsOrNone is the last symbol in the template, the value or matching symbol at the right hand side of the assignment shall be assigned to the referenced element. Then the AnyElementsOrNone symbol and all unbound values between it and the referenced symbol shall be replaced with AnyElement symbols. If the AnyElementsOrNone symbol had a length restriction, only as many AnyElement symbols can be added as is the value of the upper bound of the restriction. As the last step, an AnyElementsOrNone symbol can be appended to the end of the template. The symbol is always appended if the original AnyElementsOrNone symbol was unrestricted. If the original AnyElementsOrNone had a length restriction, the symbol is appended only if the restriction included items beyond the referenced item. In such a case, the appended symbol contains the original length restriction adjusted by the difference between the size of the template before and after assignment.

EXAMPLE 10:

type record of integer RoI;
:
var template RoI v_roI := {1, 2, * };
v_roI[4] := 5; // {1, 2, ?, ?, 5, *};	

v_roI := {1, * length(1..2)};
v_roI[4] := 5; // {1, ?, ?, -, 5};
	// short length restriction: only two ? symbols added and no * at the end

v_roI := {1, * length(1..5)};
v_roI[2] := 3; // {1, ?, 3, * length(0..3)};
	// adjusted length restriction at the end

The index of the referenced item shall not violate type restrictions in any of the above cases.

### 15.6.4	Referencing signature parameters

While signature templates do not allow referencing their parameters directly (e.g. using dot notation), such a reference is possible when modifying a signature template. However, there can be a matching mechanism assigned to the signature template. This clause provides rules for such cases.

a)	Value lists and complemented lists: referencing a parameter of a signature template to which a value list or a complemented list is assigned, at the left hand side of an assignment, shall cause an error.

EXAMPLE 1:

signature MySignature(in integer par1, in integer par2);
template MySignature s_mySign1 := ({ par1 := 1, par2 := 2 }, { par1 := 2, par2 := 1 });
template MySignature s_mySign2 modifies s_mySign1 := { par1 := ? };

// shall cause an error as s_mySign1 contains a value list template

b)	AnyValue: when referencing a parameter within a signature to which AnyValue is assigned, at the left hand side of an assignment, the signature template is implicitly expanded to the parameter level. During this expansion an AnyValue shall be assigned to all parameters of the template. After this expansion the value or matching mechanism at the right hand side of the assignment shall be assigned to the referenced parameter.

EXAMPLE 2:

template MySignature s_mySign3 := ?;
template MySignature s_mySign4 modifies s_mySign3 := { par1 := 3 }; 
// s_mySign3 is expanded to { par1 := ?, par2 := ? }, then 3 is assigned to par1,
// thus s_mySign4 will be { par1 := 3, par2 := ? }

### 15.6.5	Referencing union alternatives

Both templates and template variables allow referencing alternatives inside a union template definition using the dot notation. However, the referenced alternative may belong to template field containing a matching mechanism. This clause provides rules for such cases.

a)	Omit, AnyValueOrNone, template lists and complemented lists: referencing an alternative of a union template or template field to which Omit, AnyValueOrNone, a template list or a complemented list is assigned, at the right hand side of an assignment, shall cause an error.
When referencing an alternative of a union template or template field to which AnyValueOrNone or Omit is assigned, at the left hand side of an assignment, the template field is implicitly set to be present and the referenced alternative becomes the chosen one. If the referenced alternative is not the last element of the dot notation, rules in clause 15.6.2 valid for AnyValue shall apply recursively for further expansion. After this expansion the value or matching mechanism at the right hand side of the assignment shall be assigned to the referenced subfield.
Referencing an alternative of a union template field to which template lists or complemented template lists are assigned, at the left hand side of an assignment, shall cause an error.

EXAMPLE 1:

type record R1 {

integer f1,

integer f2

}

type union U {

integer c1,

R1      c2

}

type record R2 {

integer g1,

U       g2 optional

}

:

var template R2 v_t1 := {

g1 := 5,

g2 := *

}

var template integer v_t2;

v_t1.g2.f1 := 1;

// after the assignment v_t2.g2 is { g2 := { f1 := 1, f2 := ? } }

v_t1.g2 := omit;

v_t2 := v_t1.g2.c1;

// causes an error as omit is assigned to v_t1.g2

var template U v_u := {c1 := omit}; //causes an error as union alternatives can not be omitted

var template U v_u2 := omit; // after the assignment v_u2 will be omit

b)	AnyValue: when referencing an alternative of a union template or template field to which AnyValue is assigned, at the right hand side of an assignment, AnyValue shall be returned.
When referencing an alternative of a union template or template field to which AnyValue is assigned, at the left hand side of an assignment, the referenced alternative becomes the chosen one. If the referenced alternative is not the last element of the dot notation, rules in clause 15.6.2 valid for AnyValue shall apply recursively for further expansion. After this expansion the value or matching mechanism at the right hand side of the assignment shall be assigned to the referenced subfield.

EXAMPLE 2:

var template U v_t3 := ?;

v_t2 := v_t3.c1;

// after the assignment v_t2 will be ?

v_t3.c1.f1 := 1;

// after the assignment v_t3 will be { c1 := { f1 := 1, f2 := ? } }

c)	Ifpresent attribute: referencing an alternative of a union template field to which the ifpresent attribute is attached, shall cause an error (irrespective of the value or the matching mechanism to which ifpresent is appended).

d)	Special value null: referencing an alternative of an address type, which actual value is null shall cause an error.

## 15.7	Template matching mechanisms

### 15.7.0	General

Generally, matching mechanisms are used to replace values of single template fields or to replace even the entire contents of a template. Matching mechanisms may also be used in-line (see clause 15.4).

Matching mechanisms are arranged in four groups:

specific values;

special symbols that can be used instead of values;

special symbols that can be used inside values;

special symbols which describe attributes of values.

Some of the mechanisms may be used in combination.

The supported matching mechanisms and their associated symbols (if any) and the scope of their application are shown in table 11. The left-hand column of this table lists all the TTCN-3 types to which these matching mechanisms apply. A full description of each matching mechanism can be found in annex B.

Restrictions

In addition to the general static rules of TTCN-3 given in clause 5, the following restrictions apply:

- All other applications of matching mechanisms than the ones allowed in table 11 are forbidden.

Table 11: TTCN-3 Matching Mechanisms

| Used with values of | Value | Instead of values |  |  |  |  |  |  |  |  |  |  | Inside values |  |  | Attributes |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | S p e c i f i c V a l u e |  | O m i t | C o m p l e m e n t e d L i s t | T e m p l a t e L i s t | A n y V a l u e (?) | A n y V a l u e O r N o n e (*) | R a n g e | S u p e r s e t | S u b s e t | P a t t e r n | M a t c h d e c o d e d c o n t e n t | A n y E l e m e n t (?) | A n y E l e m e n t s O r N o n e (*) | P e r m u t a t i o n | L e n g t h R e s t r i c t i o n | I f P r e s e n t |
| boolean | Yes |  | Yes1 | Yes | Yes | Yes | Yes1 |  |  |  |  |  |  |  |  |  | Yes1 |
| integer | Yes |  | Yes1 | Yes | Yes | Yes | Yes1 | Yes |  |  |  |  |  |  |  |  | Yes1 |
| float | Yes |  | Yes1 | Yes | Yes | Yes | Yes1 | Yes |  |  |  |  |  |  |  |  | Yes1 |
| bitstring | Yes |  | Yes1 | Yes | Yes | Yes | Yes1 |  |  |  |  | Yes | Yes | Yes |  | Yes | Yes1 |
| octetstring | Yes |  | Yes1 | Yes | Yes | Yes | Yes1 |  |  |  |  | Yes | Yes | Yes |  | Yes | Yes1 |
| hexstring | Yes |  | Yes1 | Yes | Yes | Yes | Yes1 |  |  |  |  | Yes | Yes | Yes |  | Yes | Yes1 |
| character strings | Yes |  | Yes1 | Yes | Yes | Yes | Yes1 | Yes |  |  | Yes | Yes | Yes2 | Yes2 |  | Yes | Yes1 |
| record | Yes |  | Yes1 | Yes | Yes | Yes | Yes1 |  |  |  |  |  |  |  |  |  | Yes1 |
| record of | Yes |  | Yes1 | Yes | Yes | Yes | Yes1 |  |  |  |  |  | Yes | Yes | Yes | Yes | Yes1 |
| array | Yes |  | Yes1 | Yes | Yes | Yes | Yes1 |  |  |  |  |  | Yes | Yes | Yes | Yes | Yes1 |
| set | Yes |  | Yes1 | Yes | Yes | Yes | Yes1 |  |  |  |  |  |  |  |  |  | Yes1 |
| set of | Yes |  | Yes1 | Yes | Yes | Yes | Yes |  | Yes | Yes |  |  | Yes | Yes |  | Yes | Yes1 |
| enumerated | Yes |  | Yes1 | Yes | Yes | Yes | Yes1 |  |  |  |  |  |  |  |  |  | Yes1 |
| union | Yes |  | Yes1 | Yes | Yes | Yes | Yes1 |  |  |  |  |  |  |  |  |  | Yes1 |
| anytype | Yes |  | Yes1 | Yes | Yes | Yes | Yes1 |  |  |  |  |  |  |  |  |  | Yes1 |
| NOTE 1: Can be assigned to templates of any type as a whole or to optional fields of record and set templates. However when matching, it shall be applied to optional fields of record and set types only (without restriction on the type of that field). NOTE 2: Have matching mechanism meaning within character patterns only. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

### 15.7.1	Specific values

Specific values are the basic matching mechanism of TTCN-3 templates. Specific values in templates are expressions which do not contain any matching mechanisms.

Syntactical Structure

SingleExpression

Semantic Description

The matching mechanism for a specific value is an expression that evaluates to a specific value.

For further details please refer to clause 6 and to annex B.

Restrictions

In addition to the general static rules of TTCN-3 given in clause 5, the following restrictions apply:

- See the restrictions given in table 11 and in annex B.

Examples

myPCO.receive(charstring:"abcxyz");

myPCO.receive('AAAA'O);

### 15.7.2	Special symbols that can be used instead of values

These matching mechanisms can be used to characterize a set of values.

Syntactical Structure

omit |

"(" { (TemplateInstance | all from TemplateInstance) [","] } ")" |

complement "(" { (TemplateInstance | all from TemplateInstance) [","] } ")" |

"?" |

"	*" |

"(" ( ConstantExpression | -infinity ) ".." ( ConstantExpression | infinity ) ")" |

superset "(" { (TemplateInstance | all from TemplateInstance) [","] } ")" |

subset "(" { (TemplateInstance | all from TemplateInstance) [","] } ")" |

pattern [@nocase] Cstring

decmatch [ "(" Expression ]")" ] TemplateInstance

EnumValueIdentifier "(" TemplateBody {"," TemplateBody} ")"

Semantic Description

The matching mechanisms for special symbols that can be used instead of values are:

omit: the optional field, in which it is used, is not present;

NOTE 1:	omit can be assigned to templates of any type as a whole or to optional fields of record and set types. omit can only be used for matching optional fields.

(…): a list of values or templates;

complement (…): complement of a list of values or templates;

?: wildcard for any value;

*: wildcard for any value or no value at all, i.e. the field is not present;

NOTE 2:	* can be assigned to templates of any type as a whole or to optional fields of record and set types. * can only be used for matching optional fields.

(lowerBound .. upperBound): a range of integer or float values between and including the lower- and upper bounds;

superset: at least all of the elements listed, i.e. possibly more;

subset: at most the elements listed, i.e. possibly less;

pattern: a charstring or universal charstring that matches this format;

decmatch: used for matching of encoded payload fields;

EnumValueIdentifier with list of templates: used for matching of enumerated values with associated value list.

The matching mechanisms list, complemented list, subset, and superset can use the elements of a template using the all from clause.

For further details please refer to annex B.

Restrictions

In addition to the general static rules of TTCN-3 given in clause 5, the following restrictions apply:

- See the restrictions given in table 11 and in annex B.

- All templates and values used in the matching mechanisms above (including the referenced ones, e.g. within a pattern) shall be completely initialized.

Examples

myPCO.receive (integer:complement(1, 2, 3));

### 15.7.3	Special symbols that can be used inside values

These matching mechanisms allow to characterize value sets by varying values inside. The template containing a matching mechanism inside a value notation syntax is called a combined template.

Syntactical Structure

…	"?"… |

…	"*"… |

…	permutation "(" { ( TemplateBody | "?" | "*" | all from TemplateInstance )[","] } ")" …

Semantic Description

The matching mechanisms for special symbols that can be used inside values are:

?: wildcard for any single element in a string, array, record of or set of;

*: wildcard for any number of consecutive elements in a string, array, record of or set of, or no element at all (i.e. an omitted element);

permutation: all of the elements listed but in an arbitrary order (note, that ? and * are also allowed as elements of the permutation list and all elements of a template can be added to permutation using the all from clause).

For further details please refer to annex B.

Restrictions

In addition to the general static rules of TTCN-3 given in clause 5, the following restrictions apply:

- See the restrictions given in table 11 and in annex B.

- All templates or values used in the permutation matching mechanism shall be completely initialized.

Examples

template bitstring mw_b := '10???'B; 		// where each "?" may either be 0 or 1

type record of integer RI;

template RI mw_ri := {1, ?, 3}				// where ? may be any integer value

### 15.7.4	Special symbols which describe attributes of values

These matching mechanisms define properties of values.

Syntactical Structure

length "(" ConstantExpression [ ".." ( ConstantExpression | infinity ) ] ")" [ ifpresent ] |

ifpresent

Semantic Description

The matching mechanisms which describe attributes of values are:

length: restrictions for string length of string types and the number of elements for record of, set of and arrays;

ifpresent: for matching of optional field values (if not omitted).

NOTE 1:	ifpresent can be assigned to templates of any type as a whole or to optional fields of record and set types. ifpresent can only be used for matching optional fields.

NOTE 2:	Assigning ifpresent to a template that already matches the special value omit (i.e. it is either omit, an ifpresent template or AnyValueOrNone) has no effect; the resulting template will match the same set of values and the special value omit as the template the ifpresent is assigned to.

For further details please refer to annex B.

Restrictions

In addition to the general static rules of TTCN-3 given in clause 5, the following restrictions apply:

- See the restrictions given in table 11 and in annex B.

- All values used in the length matching attribute shall be completely initialized.

Examples

type record R {

record of integer ri optional

}

template R mw_r:=

{

ri := * length (1 .. 6) ifpresent	// any value containing 1, 2, 3, 4,

// 5 or 6 elements, provided it is present

}

## 15.8	Template Restrictions

Template restrictions allow to restrict the matching mechanisms that can be used with a template. Template restrictions are applicable to template definitions and template variables, formal template parameters, and return template types of functions. Template restrictions can be applied equally to message and signature templates.

Syntactical Structure

template "(" ( omit | present | value ) ")" Type

Semantic Description

The restrictions mean in case of:

(omit) the template shall resolve to a value matching mechanism (i.e. the fields of it shall resolve to a specific value or omit, and the whole template may also resolve to omit). Such a template can be used to define a field of a record and set template and the latter one could still be used in a send statement.

(value) the template shall resolve to a specific value (i.e. the fields of it shall resolve to a specific value or omit, but the whole template shall not resolve to omit). It can be used to define a mandatory field of a record or set template and the latter one could still be used in a send statement.

(present) the template as a whole shall not resolve to matching mechanisms that match omit (i.e. its fields may contain any of the matching mechanisms or matching attributes). Such a template can be used to define a mandatory field of a record or set template.

NOTE:	Template restrictions allow TTCN-3 tools to check more easily at compile time whether templates and matching expressions are used correctly. Whether the checks are performed at compile time and invalid code is rejected or whether the checks are performed at execution time and dynamic errors are raised, is outside the scope of the present document.

Restrictions

In addition to the general static rules of TTCN-3 given in clause 5, the following restrictions apply:

- Matching mechanisms can be used within restricted templates according to table 12.

Table 12: Using matching mechanisms with restricted templates

| Used with template restriction |  | Value |  | Instead of values |  |  |  |  |  |  |  |  |  | Inside values |  |  |  | Attributes |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | S p e c i f i c V a l u e | O m i t |  | C o m p l e m e n t e d L i s t | T e m p l a t e L i s t | A n y V a l u e (?) | A n y V a l u e O r N o n e (*) | R a n g e | S u p e r s e t | S u b s e t | P a t t e r n | M a t c h d e c o d e d c o n t e n t |  | A n y E l e m e n t (?) | A n y E l e m e n t s O r N o n e (*) | P e r m u t a t i o n |  | L e n g t h R e s t r i c t i o n | I f P r e s e n t |  |  |
| omit | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| value | Yes | Note 1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| present | Yes | Note 1 |  | Yes | Yes | Yes | Note 1 | Yes | Yes | Yes | Yes | Note 2 |  | Yes | Yes | Yes |  | Yes | Note 1 |  |  |
| NOTE 1: It is allowed to use the matching mechanism in fields of the template, but the template as a whole shall not resolve to this matching mechanism. NOTE 2: The matching mechanism is allowed only if the template following the decmatch keyword is fulfilling the given restriction. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

Restricted and unrestricted templates can be used as actual parameters of formal template parameters or assigned to templates or template variables according to table 13.

Table 13: Restrictions of formal and actual template parameters

|  | Actual parameter/right hand side of an assignment | value | template (omit) | template (value) | template (present) | template |
| --- | --- | --- | --- | --- | --- | --- |
| Formal parameter/left hand side of an assignment |  |  |  |  |  |  |
| template(omit) |  | Yes | Yes | Yes | No | No |
| template(value) |  | Yes | Rule 1 | Yes | No | No |
| template(present) |  | Yes | No | Yes | Yes | No |
| template |  | Yes | Yes | Yes | Yes | Yes |
| Rule 1: Not allowed with one exception: a template with an omit restriction is allowed to be used on the right hand side of an assignment if the left hand side resolves into an optional field of a template with a value restriction. |  |  |  |  |  |  |

Examples

// definitions of restricted templates

type record ExampleType {

integer a,

boolean b optional

}

template(omit) ExampleType m_exampleOmit := omit;

template(omit) ExampleType m_exampleOmitValue:= { 1, true };
	template(omit) ExampleType mw_exampleOmitAny := ?;           // incorrect

	template(value) ExampleType m_exampleValueomit := omit;     // incorrect
	template(value) ExampleType m_exampleValue := { 1, true };
	template(value) ExampleType m_exampleValueOptional := { 1, omit };

// omit assigned to a field is correct

	template(present) ExampleType mw_examplePresent := {1, ?};

template(present) ExampleType mw_examplePresentIfpresent := { 1, true } ifpresent;

// incorrect

template(present) ExampleType mw_examplePresentAny := ?;

// restricted template usage
	var template (omit) ExampleType v_omit;
	var template (present) ExampleType v_present;
	var template (value) ExampleType v_value;

v_omit := m_exampleOmit;
	v_omit := m_exampleValueOptional;
	v_omit := mw_examplePresentAny;     // incorrect, not a specific value

	v_present := m_exampleOmit;         // incorrect, shall not be omit
	v_present := mw_examplePresent;

	v_value := m_exampleOmit;           // incorrect, shall not be omit
	v_value := mw_examplePresentAny;    // incorrect, shall be a single value

## 15.9	Match Operation

The match operation allows to compare a value (specified in form of an expression) with a template.

Syntactical Structure

match "(" Expression "," TemplateInstance ")"

Semantic Description

The match operation returns a boolean value. It matches an expression, which shall denote a value or a field of a value against a template instance. Types of the expression and the template instance shall be compatible (see clause 6.2.15). The return value of the match operation indicates whether the expression matches the specified template instance. In the special case, matching a non-optional value expression (e.g. a value variable or non-optional field of a value) with a template instance that matches an omitted field (i.e. one of the matching mechanisms Omit, AnyValueOrNone, IfPresent) shall be allowed and shall be treated as if the value expression were an optional field. Thus, matching a value expression against a template instance which evaluates to the omit matching mechanism shall return false.

Restrictions

In addition to the general static rules of TTCN-3 given in clause 5, the following restrictions apply:

- The expression-parameter of the match operation shall evaluate to a value or shall denote an omitted optional field, i.e. the match operation cannot be used to compare two templates.

- The operands of the match operation shall be completely initialized.

- The type of the template instance-parameter shall be unambiguously identified. If the expression-parameter evaluates to a literal value without explicit or implicit identification of its type, the type of the template instance-parameter shall be used as the type governor for the expression-parameter.

NOTE:	In case of in-line templates, see restriction b) in clause 15.4.

Examples

EXAMPLE 1:	Using the match operation

template integer mw_lessThan10 := (-infinity..9);

:

myPort.receive(integer:?) -> value v_rxValue;

if(	match( v_rxValue, mw_lessThan10)) { … }

// true if the actual value of v_rxvalue is less than 10 and false otherwise

:

type record R { integer a, integer b optional, integer c optional }

const R c_r := { a := 1, b := omit, c := 1 }

const integer c_c := 1;

:

function f_f(template(omit) integer p_o) {

:

match(c_c, omit) 	// returns false

match(5, omit) 		// returns false

match(c_c, *) 		// returns true

match(c_r, c_c) 	// error (different types)

match(c_r.a, p_o) 	// returns true if p_o evaluates to 1, false, otherwise

match(c_r.b, p_o) 	// returns true, if p_o is not present, false, otherwise

match(c_r.c, p_o)  	// returns true, if p_o evaluates to 1, false, otherwise

match(c_c, p_o) 	// returns true, if p_o evaluates to 1, false, otherwise

match(c_c, 1)		// returns true (the syntax of the template parameter implicitly 
							// identifies its type, see clause 15.4)

}

EXAMPLE 2:	Using the match operation with enumerated types

type enumerated MyFirstEnumType { Monday, Tuesday, Wednesday, Thursday, Friday };

type enumerated MySecondEnumType { Saturday, Sunday, Monday };

control {

var MyFirstEnumType v_today := Tuesday;

match (v_today, Sunday) // causes an error, as the value Sunday alone does not specifies
		// the type context of the template instance-parameter

match (v_today, MySecondEnumType:Sunday) // returns false

match (Monday, v_today)

//returns false; in this case v_today is governing the type context for the match operation
		//(MyFirstEnumType), but its actual value is different from Monday

}

## 15.10	Valueof Operation

The valueof operation allows to return the value specified within a template. The returned value can be assigned to a variable, may be used in expressions, as an actual value parameter, etc.

Syntactical Structure

valueof "(" TemplateInstance ")"

Semantic Description

The valueof operation returns the value of a template instance.

Restrictions

In addition to the general static rules of TTCN-3 given in clause 5, the following restrictions apply:

a)	The template shall be completely initialized and resolve to a specific value.

Examples

EXAMPLE 1:

type record ExampleType

{

integer field1,

boolean field2

}

template ExampleType m_setupTemplate :=

{

field1 := 1,

field2 := true

}

:

var ExampleType v_rxValue := valueof(m_setupTemplate);

EXAMPLE 2:

function MyFunc() {
	  var template integer v_tInt := omit;

//is ok, but to be used for optional record or set fields only

var integer v_int := valueof(v_tInt)

//causes an error as omit is not a value and shall not be an argument of valueof
	  :
	}

## 15.11	Concatenating templates of string and list types

Templates of string and list types (bitstring, octetstring, hexstring, charstring, universal charstring, record of, set of, and array) can be concatenated from several single (in-line) templates using the concatenation operation. With the exception of charstring and universal charstring templates, each single template shall have the same root type.

The single templates of binary string types shall evaluate only to the matching mechanisms specific value, combined template, AnyValue without a length modifier, AnyValue or AnyValueOrNone, both constrained to a fixed length.

The concatenation of templates of binary string types results in the sequential concatenation of the single templates from left to right, with two exceptions: matching symbol AnyValue without a length modifier shall be replaced by a single AnyElementsOrNone matching symbol before concatenation and matching symbols AnyValue and AnyValueOrNone that are each constrained to a fixed length N shall be replaced by N AnyElement matching symbols before concatenation.

The single templates of list types shall evaluate only to the matching mechanisms specific value, combined template, AnyValue with or without a length modifier, AnyValueOrNone with a length modifier.

The concatenation of templates of list types results in the sequential concatenation of the single templates from left to right, with two exceptions: before concatenation, an AnyValue matching symbol without a length modifier shall be replaced by a single AnyElementsOrNone matching symbol and AnyValue and AnyValueOrNone matching symbols with a length modifier shall be replaced by an AnyElementsOrNone matching mechanism with the same length modifier.

Single templates of charstring and universal charstring types shall evaluate only to specific values, AnyValue with or without a length modifier, AnyValueOrNone with a length modifier or pattern. When concatenating templates of charstring and universal charstring types, each single template shall be either of the charstring or universal charstring type. When templates of charstring and universal charstring type are both present in the concatenation, the charstring values are implicitly converted to universal charstring values according to the rules specified in clause 6.3.1 before concatenation and the resulting template is of the universal charstring type.

The concatenation of templates of character string types results in the sequential concatenation of the single templates from left to right. In case all operands are specific values, the result of concatenation will be a specific value. In all other cases, if possible, a pattern will be produced. When producing the pattern, the templates are first transformed to a character string according to table 14. In the next step, the strings are concatenated and a pattern is created from the concatenation result. If one of the single templates is a pattern with the @nocase modifier, then all other single templates shall also be patterns with the @nocase modifier and the resulting pattern will also have the @nocase modifier.

Table 14: Transformation of character string templates before concatenation

| Concatenation operand | Transformed string |
| --- | --- |
| Specific values (character strings) | The character string is escaped by inserting the "\" character before each of the following characters: "#", "(", ")", "*", "+", "-", "?", "[", "\", "]", "^", "{", "¦", "}". |
| ?, ? length(0..infinity) or * length(0..infinity) | * |
| ? length(0) or * length(0) | empty string |
| ? length(1) or * length(1) | "?" |
| ? length(1 .. infinity) or * length(1 .. infinity) | "?+" |
| ? length(n) or * length(n) | "?#(n)" |
| ? length(n .. infinity) or * length(n .. infinity) | "?#(n,)" |
| ? length(n .. m) or * length(n .. m) | "?#(n,m)" |
| pattern "content" | "content" |

Concatenation of character strings in a pattern definition is described in clause B.1.5. Concatenation of character strings in a pattern definition always takes precedence over concatenation of templates. If it is necessary to concatenate a pattern definition with a following character string according to the template concatenation rules (thus automatically escaping the string), the pattern definition shall be enclosed into parentheses.

The concatenation shall be performed completely before using the resulting template (e.g. for assignment or matching) and the result shall be type-compatible with the place of its use.

The length matching attribute shall not follow a template or template field produced by concatenation directly, but in this case the concatenation shall be placed within a pair of parentheses.

Restrictions

In addition to the general static rules of TTCN-3 given in clause 5, the following restrictions apply:

a)	All operands of the concatenation operation shall be at least partially initialized.

EXAMPLE 1:	Composing templates of binary string types:

template bitstring mw_mybit := '010'B & ? & '1'B & ? length(1) & '1'B;

// results in the template '010*1?1'B

// note that & ? & turns to * within the resulting bitstring as the original ?

// stands for a bitstring of any length

template bitstring mw_mybit2 := '010'B & * length(2);

// results in the template '010??'B

// note that the ability of the AnyValueOrNone matching symbol to match omitted

// values is lost during concatenation

type bitstring ConstrainedBitString length(3);

// values of this type are always exactly 3 bits long

template ConstrainedBitString mw_mybit3 := ?;

template bitstring mw_mybit4 := '010'B & mw_mybit3;

// results in the template '010*'B

// note that the type constraint of the mw_mybit3 template has no impact on the

// concatenation operation; the AnyValue symbol without a length modifier is always

// replaced with a single AnyElementsOrNone

template octetstring mw_myoct1 := 'ABCD'O & 'EF'O & ? & ? length(1) & 'EF'O;

// results in the template 'ABCDEF*?EF'O

// note that & ? & turns to * within the resulting octetstring as the original ?

// stands for an octetstring of any length

template octetstring mw_myoct2 := 'ABCD'O & ? length (2) & 'EF'O;

// results in the template 'ABCD??EF'O

// (i.e. a 5 octets i.e. 10 hexadecimal digits long value)

template octetstring mw_myoctWrong := 'ABCD'O & ? length(2) length (4);

// causes an error, no length matching attribute shall directly follow a concatenation

template octetstring mw_myoct3 := ('ABCD'O & ? length(2)) length (4);

// results in the template 'ABCD??'O

template hexstring mw_myhexPar (integer N):=

'ABC'H & ? length(N) & 'E'H & ? length(1) & 'F'H;

function f_myFunc() runs on MyCompType {

var integer v_int := 3;

var template hexstring v_hstring;

:

v_hstring := 'ABC'H & ? length(v_int) & 'E'H & ? length(1) & 'F'H;

//results in the template 'ABC???E?F'H

p.receive (mw_myhexPar(4));

//actual content of mw_myhexPar is 'ABC????E?F'H

}

EXAMPLE 2:	Composing templates of list types:

type record of charstring RecofChar;

type set of integer SetofInt;

template RecofChar mw_myRecofChar := {"ABC"} & {"D?", "EF"};

// results in the template {"ABC", "D?", "EF" }

template SetofInt mw_mySetofInt := { 1, 2 } & ? length(2) & { 3, 4 };

// results in the template {1, 2, * length(2), 3, 4 }

template RecofInt mw_myRecofInt := { 1, 2 } & { * length(1..2), 3, 4 };

// results in the template {1, 2, * length(1..2), 3, 4 }

template RecofChar mw_myRecofCharPar (integer N):= { "ABC" } & ? & * length(N) & { "EF" };

function myFunc2() runs on MyCompType{

var integer v_int := 3;

var template RecofChar v_recofChar;

:

v_recofChar :=  { "ABC" } & ? length(v_int) & { "EF" };

//results in the template { "ABC", * length(3), "EF" }

p.receive ( mw_myRecofCharPar(3) );

//actual content of mw_myRecofCharPar is { "ABC", *, * length(3), "EF" }

}

EXAMPLE 3:	Composing templates of character string types:

template charstring mw_mychar1 := "ABC" & "DE*" & "F?";

// results in the template "ABCDE*F?"

// please note that "*" and "?" denote the characters "*" and "?"

template charstring mw_mychar2 := "ABC" & ? & "EF";

// results in the template pattern "ABC*EF"

template charstring mw_mychar3 := "ABC" & * length(1 .. infinity) & "EF";

// results in the template pattern "ABC?+EF"

template charstring mw_mychar4 := "ABC" & * length(2 .. 5) & "EF";

// results in the template pattern "ABC?#(2,5)EF"

template charstring mw_mychar5 := pattern "ABC" & "?EF";

// results in the template pattern "ABC?EF"

template charstring mw_mychar6 := (pattern "ABC") & "?EF";

// results in the template pattern "ABC\?EF"

template charstring mw_mycharPar (integer p_val):=

"ABC" & ? length(p_val) & "E" & ? length(1..10) & "F";

// the parameterized template is used inside the following function;

// concatenation to a pattern is performed during instantiation of the

// template when all actual parameter values as known

function f_myFunc3() runs on MyCompType {

var integer v_int := 3;

var template charstring v_cstring;

:

v_cstring := "ABC" & ? length(v_int .. v_int + 2) & "E" & ? length(1) & "F";

//results in the template pattern "ABC?#(3,5)E?F"

p.receive (mw_mycharPar(4));

//actual content of mw_mycharPar is pattern "ABC?#4E?#(1,10)F"

}

# 16	Functions, altsteps and testcases

## 16.0	General

In TTCN-3, functions, altsteps and testcases are used to specify and structure test behaviour, define default behaviour and to structure computation in a module, etc. as described in the following clauses.

## 16.1	Functions

### 16.1.0	General

Functions are used in TTCN-3 to express test behaviour, to organize test execution or to structure computation in a module, for example, to calculate a single value, to initialize a set of variables or to check some condition.

Syntactical Structure

function [ @deterministic | @control ]  ( FunctionIdentifier | control )

"(" [ { ( FormalValuePar | FormalTemplatePar ) [","] } ] ")"

[ runs on ComponentType ]

[ mtc ComponentType ]

[ system ComponentType ]

[ return [ TemplateModifier ] Type [ArrayDef] ]

StatementBlock

Semantic Description

Functions are portions of TTCN-3 behaviour, which perform a specific task and are relatively independent of the remaining behaviour.

Functions may return a value or a template. Value return is denoted by the return keyword followed by a type expression. Template return is denoted by the return keywords followed by a TemplateModifier and a type expression. Execution of a return statement in the body of the function causes evaluation of the return value or template,  the function to terminate and to return the result to the location of the call of the function.

The behaviour of a function can be defined by using statements and operations described in clauses 18 to 26.

Functions may be parameterized.

Functions may have an mtc clause. If a function has an mtc clause, the type referenced by this clause shall be mtc-compatible (see clause 6.3.2.7) with the type of the mtc component reference. If the mtc clause is not present, the type of the mtc component reference is unknown in the scope of this function.

Functions may have a system clause. If a function has a system clause, the type referenced by this clause shall be system-compatible (see clause 6.3.2.7) with the type of the system component reference. If the system clause is not present, the type of the system component reference is unknown in the scope of this function.

Using the @deterministic modifier, a function can be declared to be deterministic. Deterministic functions are safe to be used when called from specific places where non-determinism could lead to unexpected side effects (see clause 16.1.4).

NOTE 0:	The determination of determinism of a function is a semi-decidable problem and as such can and will not be exhaustively checked. As such, the annotation deterministic is mainly used for informational purposes and for allowing certain functions to be used during snapshot evaluation. Principally, a function can be seen as deterministic if it does not violate any of the restrictions from clause 16.1.4 which does not mean that violation of these restriction automatically leads to non-determinism.

Restrictions

In addition to the general static rules of TTCN-3 given in clause 5, the following restrictions apply:

- A function without runs on clause shall never invoke a function or altstep or activate an altstep as default with a runs on clause locally.

- Functions started by using the start test component operation shall always have a runs on clause (see clause 22.5) and are considered to be invoked in the component to be started, i.e. not locally. However, the start test component operation may be invoked within behaviour without a runs on clause.

NOTE 1:	The restrictions concerning the runs on clause are only related to functions and altsteps and not to test cases.

- Functions called directly or indirectly from a module control function shall have no mtc or system clause.

NOTE 2:	Nevertheless, functions called directly or indirectly from the module control function are allowed to execute test cases.

The rules for formal parameter lists shall be followed as defined in clause 5.4.

For return TemplateModifier statements the restrictions specified in clause 15 shall apply.

Template return can be restricted to the matching mechanisms specific value and omit, see clause 5.4.1.2.

A return statement in a value returning function shall always have a value expression compatible to the type specified in the function header return clause.

A return statement in a template returning function shall always have a template reference (including calling a value or template returning function)or template instance compatible to the type specified in the function header return clause. If the return clause has a template restriction, this restriction shall be adhered to by the returned template. The return statement shall return a template that is at least partially initialized.

If the function header includes a return clause, the function, when terminating, shall do so by executing a return statement. The function will cause a test case error if it terminates (i.e. reaches the end of the function body) without executing a return statement.

If a function references the names of definitions that are defined inside a component type definition, the component type shall be referenced using the runs on keywords in the function header. The one exception to this rule is if all the necessary component-wide information is passed in the function as parameters.

The additional restrictions in clause 16.1.5 shall apply to all explicit control functions.The list of statements and operations that are allowed to be used by control functions is provided by table 15.

Examples

EXAMPLE 1:	Function with return:

// Definition of f_myFunction which has no parameters

function f_myFunction() return integer

{

return 7; 	// returns the integer value 7 when the function terminates

}

EXAMPLE 2:	Function with template return:

// Definition of functions which may return matching symbols or templates

function f_myFunction2() return template integer

{

:

return ?; 	// returns the matching mechanism AnyValue

}

function f_myFunction3() return template octetstring

{

:

return 'FF??FF'O; 	// returns an octetstring with AnyValue inside it

}

EXAMPLE 3:	Function with runs on clause:

function f_myFunction3() runs on MyPTCType {

// f_myFunction3 does not return a value, but

var integer v_myVar := 5;	// does make use of the port operation

pCO1.send(v_myVar);			// send and therefore requires a runs on

// clause to resolve the port identifiers

}								// by referencing a component type

EXAMPLE 4:	Parameterized function:

function f_myFunction2(inout integer p_myPar1) {

// f_myFunction2 does not return a value

p_myPar1 := 10 * p_myPar1;	// but changes the value of p_myPar1 which

}								// is passed in by reference

EXAMPLE 5:	Function without return statement:

function f_myFunction5(inout integer p_myPar1) return integer {

if (p_myPar1 > 5) {

p_myPar1 := 5;

return p_myPar1;

}

// in case of p_myPar1 <= 5, f_myFunction5 does not terminate in a return statement

// and will cause a test case error

}

EXAMPLE 6:	Function with system and mtc:

type component MtcType { ... }

type component SystemType { ... }

function f_myFunction6() runs on MyPtcType mtc MtcType system SystemType {

var MtcType v_mtc := mtc;

var SystemType v_system := system;

f_myFunction3(); // allowed, f_myFunction3() has no mtc and system clause

f_myFunction6(); // allowed, f_myFunction6() has compatible mtc and system clause

}

function f_myFunction7() runs on MyPtcType system SystemType {

var MtcType v_mtc := mtc; // not allowed, mtc type unknown

f_myFunction6(); // possible runtime error, no mtc clause of f_myFunction7

}

function MyFunction8() runs on MyPtcType mtc MtcType {

var SystemType v_system := system; // not allowed, system type unknown

f_myFunction6(); // possible runtime error, no system clause of f_myFunction8

}

### 16.1.1	Invoking functions

A function is invoked by referring to its name and providing the actual list of parameters.

Syntactical Structure

FunctionRef "(" [ { ActualPar [","] } ] ")"

Semantic Description

A function invocation results in the execution of the statement block of the invoked function. The invoked function is performed by the test component invoking it. Actual parameters are passed into the statement block. If the function returns (upon termination and potentially with a return value), the test components continues its behaviour right after the function invocation.

Restrictions

In addition to the general static rules of TTCN-3 given in clause 5, the following restrictions apply:

a)	Functions that do not return values shall be invoked directly. Functions that return values may be invoked directly or inside expressions.

b)	The rules for actual parameter lists shall be followed as defined in clause 5.4.

c)	Special restrictions apply to functions bound to test components using the start test component operation. These restrictions are described in clause 21.3.2.

d)	When invoking a function, the compatibility to the test component type of the invoking test component as described in clause 6.3.2.7 need to be fulfilled.

e)	Restrictions on invoking functions from specific places are described in clause 16.1.4.

f)	When invoking a function, the mtc and system compatibility of the mtc and system components of the invoked function with the actual mtc and system types of the running test case as described in clause 6.3.2.7 need to be fulfilled.

Examples

v_myVar := f_myFunction4();	// The value returned by f_myFunction4 is assigned to v_myVar.

// The types of the returned value and v_myVar have to be compatible

f_myFunction2(v_myVar2);	// f_myFunction2 does not return a value and is called with the

// actual parameter v_myVar2, which may be passed in by reference

v_myVar3 := f_myFunction6(4) + f_myFunction7(v_myVar3);	// Functions used in expressions

### 16.1.2	Predefined functions

TTCN-3 contains a number of predefined (built-in) functions that need not be declared before use. These are summarized in table 15.

Table 15: List of TTCN-3 predefined functions

| Category | Function | Keyword |
| --- | --- | --- |
| Conversion functions | Convert integer value to charstring value | int2char |
|  | Convert integer value to universal charstring value | int2unichar |
|  | Convert integer value to bitstring value | int2bit |
|  | Convert integer value to enumerated value | int2enum |
|  | Convert integer value to hexstring value | int2hex |
|  | Convert integer value to octetstring value | int2oct |
|  | Convert integer value to charstring value | int2str |
|  | Convert integer value to float value | int2float |
|  | Convert float value to integer value | float2int |
|  | Convert charstring value to integer value | char2int |
|  | Convert charstring value to octetstring value | char2oct |
|  | Convert universal charstring value to octetstring value | unichar2oct |
|  | Convert universal charstring value to integer value | unichar2int |
|  | Convert bitstring value to integer value | bit2int |
|  | Convert bitstring value to hexstring value | bit2hex |
|  | Convert bitstring value to octetstring value | bit2oct |
|  | Convert bitstring value to charstring value | bit2str |
|  | Convert hexstring value to integer value | hex2int |
|  | Convert hexstring value to bitstring value | hex2bit |
|  | Convert hexstring value to octetstring value | hex2oct |
|  | Convert hexstring value to charstring value | hex2str |
|  | Convert octetstring value to integer value | oct2int |
|  | Convert octetstring value to bitstring value | oct2bit |
|  | Convert octetstring value to hexstring value | oct2hex |
|  | Convert octetstring value to charstring value | oct2str |
|  | Convert octetstring value to charstring value, version II | oct2char |
|  | Convert octetstring value to universal charstring value | oct2unichar |
|  | Convert charstring value to integer value | str2int |
|  | Convert charstring value to hexstring value | str2hex |
|  | Convert charstring value to octetstring value | str2oct |
|  | Convert charstring value to float value | str2float |
|  | Convert enumerated value to integer value | enum2int |
|  | Convert value or template to universal charstring value | any2unistr |
| Length/size functions | Return the length of a value or template of any string type, record of, set of or array | lengthof |
|  | Return the number of elements in a value or a template of a record or set | sizeof |
| Presence checking functions | Determine if a template contains certain matching mechanism | istemplatekind |
| String/list handling functions | Returns part of the input string matching the specified pattern group within a character pattern | regexp |
|  | Returns the specified portion of the input string/list value or template | substr |
|  | Replaces a substring of a string with or inserts the input string into a string, and similarly for lists | replace |
| Codec functions | Encode a value into a bitstring | encvalue |
|  | Decode a bitstring into a value | decvalue |
|  | Encode a value into a universal charstring | encvalue_unichar |
|  | Decode a universal charstring into a value | decvalue_unichar |
|  | Encode a value into a octetstring | encvalue_o |
|  | Decode a octetstring into a value | decvalue_o |
|  | Retrieve the type of string encoding | get_stringencoding |
|  | Remove BOMs of UCS encoding schemes | remove_bom |
| Other functions | Generate a random float number | rnd |
|  | Returns the name of the currently executing test case | testcasename |
|  | Returns the host id of the test component or module | hostid |

Syntactical Structure

int2char "(" SingleExpression ")" |

int2unichar "(" SingleExpression ")" |

int2bit "(" SingleExpression "," SingleExpression ")" |

int2enum "(" SingleExpression "," SingleExpression ")" |

int2hex "(" SingleExpression "," SingleExpression ")" |

int2oct "(" SingleExpression "," SingleExpression ")" |

int2str "(" SingleExpression ")" |

int2float "(" SingleExpression ")" |

float2int "(" SingleExpression ")" |

char2int "(" SingleExpression ")" |

char2oct "(" SingleExpression ")" |

unichar2int "(" SingleExpression ")" |

unichar2oct "(" SingleExpression ["," SingleExpression] ")" |

bit2int "(" SingleExpression ")" |

bit2hex "(" SingleExpression ")" |

bit2oct "(" SingleExpression ")" |

bit2str "(" SingleExpression ")" |

hex2int "(" SingleExpression ")" |

hex2bit "(" SingleExpression ")" |

hex2oct "(" SingleExpression ")" |

hex2str "(" SingleExpression ")" |

oct2int "(" SingleExpression ")" |

oct2bit "(" SingleExpression ")" |

oct2hex "(" SingleExpression ")" |

oct2str "(" SingleExpression ")" |

oct2char "(" SingleExpression ")" |

oct2unichar "(" SingleExpression ["," SingleExpression] ")" |

str2int "(" SingleExpression ")" |

str2hex "(" SingleExpression ")" |

str2oct "(" SingleExpression ")" |

str2float "(" SingleExpression ")" |

enum2int "(" SingleExpression ")" |

any2unistr "(" SingleExpression ["," SingleExpression] ")" |

lengthof "(" TemplateInstance ")" |

sizeof "(" TemplateInstance ")" |

istemplatekind "(" TemplateInstance "," TemplateInstance ")" |

regexp [@nocase] "(" TemplateInstance"," TemplateInstance"," SingleExpression ")" |

substr "(" TemplateInstance "," SingleExpression "," SingleExpression ")" |

replace "(" SingleExpression "," SingleExpression "," SingleExpression "," SingleExpression ")" |

encvalue "(" TemplateInstance ["," SingleExpression] ["," SingleExpression] ")" |

decvalue "(" SingleExpression "," SingleExpression

["," SingleExpression] ["," SingleExpression] ["," SingleExpression] ")" |

encvalue_unichar "(" TemplateInstance ["," SingleExpression]

["," SingleExpression] ["," SingleExpression] ")" |

decvalue_unichar "(" SingleExpression "," SingleExpression

["," SingleExpression] ["," SingleExpression] ["," SingleExpression] ")" |

encvalue_o "(" TemplateInstance ["," SingleExpression] ["," SingleExpression] ")" |

decvalue_o "(" SingleExpression "," SingleExpression ["," SingleExpression]

["," SingleExpression] ")" |

get_stringencoding "(" SingleExpression ")" |

remove_bom(" SingleExpression ") |

rnd "(" [ SingleExpression ] ")" |

testcasename "()" |

hostid "(" [ SingleExpression ] ")"

Semantic Description

The description of predefined functions is given in annex C.

Restrictions

In addition to the general static rules of TTCN-3 given in clause 5, the following restrictions apply:

a)	When a predefined function is invoked:

1)	the number of the actual parameters shall be the same as the number of the formal parameters; and

2)	each actual parameter shall evaluate to an element of its corresponding formal parameter's type; and

3)	all actual in and inout parameters shall be initialized with the following exceptions:

any parameters of the functions lengthof, substr and replace may be partially initialized;

the invalue parameter of the any2unistr function may be uninitialized or partially initialized;

the encoded_value parameter of the decvalue and decvalue_unichar function may be uninitialized.

b)	Restrictions on invoking functions from specific places are described in clause 16.1.4.

Examples

var hexstring v_h:= bit2hex ('111010111'B);

var octetstring v_o:= substr ('01AB23CD'O, 1, 2);

### 16.1.3	External functions

A function may be defined within a module or be declared as being defined externally (i.e. external).

Syntactical Structure

external function [ @deterministic | @control ] ExtFunctionIdentifier

"(" [ { ( FormalValuePar | FormalTemplatePar ) [","] } ] ")"

[ return  [ TemplateModifier ] Type ]

Semantic Description

For an external function only the function interface has to be provided in the TTCN-3 module. The realization of the external function is outside the scope of the present document.

Using the @deterministic modifier, an external function can be declared to be deterministic. Deterministic functions are safe to be used when called from specific places where non-determinism could lead to unexpected side effects (see clause 16.1.4).

The @control modifier is used in the same way as described in the clause 16.1.0.

Restrictions

In addition to the general static rules of TTCN-3 given in clause 5, the following restrictions apply:

- Restrictions on invoking functions from specific places are described in clause 16.1.4.

NOTE:	External functions should only exchange information with the test system via return values and parameter passing. Side-effects that change the status of the test system and may influence the test outcome should be avoided. Such side-effects can occur if an external function contains default handling, configuration, communication or timer operations.

Examples

external function fx_myFunction4() return integer;	// External function without parameters

// which returns an integer value

external function fx_initTestDevices();	// An external function which only has an

// effect outside the TTCN-3 module

### 16.1.4	Invoking functions from specific places

If value returning functions are called in receiving communication operations (in templates, template fields, in-line templates as actual parameters or when evaluating the port expression), in timeout operations (when evaluating the timer expression), in test component operations (in guards or events of alt statements or altsteps, see clause 20.2), or in initializations of altstep local definitions (see clause 16.1.5), the following operations shall not be present in functions called in the cases specified above, in order to avoid side effects that cause changing the state of the component or the actual snapshot and to prevent different results of subsequent evaluations on an unchanged snapshot:

- All component operations, i.e. create, start (component), stop (component), kill, running (component), alive, done and killed (see notes 1, 3, 4 and 6).

- All port operations, i.e. start (port), stop (port), halt, clear, checkstate, send, receive, trigger, call, getcall, reply, getreply, raise, catch, check, connect, disconnect, map and unmap (see notes 1, 2, 3, 4 and 6).

- The action operation (see notes 2 and 6).

- All timer operations, i.e. start (timer), stop (timer), running (timer), read, timeout (see notes 4 and 6).

- Calling non-deterministic external functions, i.e. external functions where the resulting values for actual inout or out parameters or the return value may differ for different invocations with the same actual in and inout parameters (see notes 4 and 6).

- Calling the rnd predefined function (see notes 4 and 6).

- Changing of component variables, i.e. using component variables on the left-hand side of assignments, and in the instantiation of out and inout parameters (see notes 4 and 6).

- Calling the setverdict operation (see notes 4 and 6).

- Activation and deactivation of defaults, i.e. the activate and deactivate statements (see notes 5 and 6).

- Calling functions and deterministic external functions with out or inout parameters (see notes 7 and 8).

- Calling functions and external functions with @fuzzy formal parameters and variables (see notes 4 and 9).

- The setencode operation (see note 8 and clause 27.9).

- Referencing lazy or fuzzy variables, parameters or templates that have not been declared deterministic.

NOTE 1:	The execution of the operations start, stop, done, killed, halt, clear, receive, trigger, getcall, getreply, catch and check can cause changes to the current snapshot.

NOTE 2:	The use of operations send, call, reply, raise, and action causes an error, i.e. all communication are to be made explicit and not as a side effect of another communication operation or the evaluation of a snapshot.

NOTE 3:	The use of operations map, unmap, connect, disconnect, create will lead to an error, i.e. all configuration operations are to be made explicit, and not as a side effect of a communication operation or the evaluation of a snapshot.

NOTE 4:	Calling of non-deterministic external functions, rnd, running, alive, read, checkstate, setverdict, referencing fuzzy objects and writing to component variables causes an error because this may lead to different results of subsequent evaluations of the same snapshot, thus, e.g. rendering deadlock detection impossible.

NOTE 5:	The use of operations activate and deactivate causes an error because they modify the set of defaults that is considered during the evaluation of the current snapshot.

NOTE 6:	Restrictions except the limitation on the use of out or inout parameterization in restriction j) apply recursively, i.e. it is disallowed to use them directly, or via an arbitrary long chain of function invocations.

NOTE 7:	The restriction of calling functions and deterministic external functions with out or inout parameters does not apply recursively, i.e. calling functions that themselves call functions with out or inout parameters is legal.

NOTE 8:	Using out or inout parameters and the setencode operation causes an error because this may lead to different results of subsequent evaluations of the same snapshot.

NOTE 9:	Calling functions and external functions with @fuzzy parameters causes an error, because fuzzy objects are re-evaluated each time referenced and this may lead to different results of subsequent evaluations of the same snapshot.

### 16.1.5	Explicit control functions

Explicit control functions are declared either by use of the name control or by use of the @control modifier.Restrictions

In addition to the general static rules of TTCN-3 given in clause 5 and the general restrictions on functions given in clause 16.1, the following restrictions apply to explicit control functions:

- No system and no mtc clause shall be declared.

The component type referenced in the runs on clause shall not contain any port definitions.

Parameters and the return type shall not be of a port type and shall not be of a structured type that contains fields of a port type on any level of nesting.

Parameters and the return type of module control functions shall not be of a default type and shall not be of a structured type that contais fields of a default type on any level of nesting.

The restrictions on the use of statements and operations usable in control functions given in table 15 shall be followed.

Module control functions used as an execution entry point shall have public visibility.

When invoking a module control function explicitly, the compatibility to the test component type of the invoking control component as described in clause 6.3.2.7 need to be fulfilled.

## 16.2	Altsteps

### 16.2.0	General

TTCN-3 uses altsteps to specify default behaviour or to structure the alternatives of an alt statement.

Syntactical Structure

altstep [ @control ] [ interleave ] AltstepIdentifier

"(" [ { ( FormalValuePar | FormalTemplatePar ) [","] } ] ")"

[ runs on ComponentType ]

[ mtc ComponentType ]

[ system ComponentType ]

"{"

{ ( VarInstance | TimerInstance | ConstDef | TemplateDef ) [";"] }

AltGuardList

"}"

Semantic Description

Altsteps are scope units similar to functions. The altstep body defines an optional set of local definitions and a set of alternatives, the so-called top alternatives, that form the altstep body. The syntax rules of the top alternatives are identical to the syntax rules of the alternatives of alt statements.

An altstep can also be used to define a named interleave statement by usage of the interleave keyword. In this case, the syntax rules of the top alternatives are identical to the syntax rules of the alternatives of the interleave statements.

NOTE:	As an interleave statement is semantically equivalent with an alt statement, there are no further restrictions on usages of interleave altsteps than on usages of normal altsteps. They can both be used as activated default alternatives and as top-level alternatives in other alt statement blocks.The behaviour of an altstep can be defined by using the program statements and operations summarized in clause 18. Altsteps may invoke functions and altsteps or activate altsteps as defaults.

Altsteps may be parameterized as defined in clause 5.4.

Altsteps may have an mtc clause. If an altstep has an mtc clause, the type referenced by this clause shall be mtc-compatible (see clause 6.3.2.7) with the type of the mtc component reference. If the mtc clause is not present, the type of the mtc component reference is unknown in the scope of this altstep.

Altsteps may have a system clause. If an altstep has a system clause, the type referenced by this clause shall by system-compatible (see clause 6.3.2.7) with the type of the system component reference. If the system clause is not present, the type of the system component reference is unknown in the scope of this altstep.

Altsteps with the @control modifier are allowed to be executed only on the control component (see clause 26). The restrictions specified for functions with the @control modifier (see clause 16.1.5) are valid for these altsteps as well.

Restrictions

In addition to the general static rules of TTCN-3 given in clause 5, the following restrictions apply:

a)	The local definitions of an altstep shall be defined before the set of alternatives.

b)	The evaluation of formal parameters' default values and initialization of local definitions by calling value returning functions may have side effects. To avoid side effects that cause an inconsistency between the actual snapshot and the state of the component, and to prevent different results of subsequent evaluations on an unchanged snapshot, restrictions given in clause 16.1.4 shall apply to the formal parameters' default values and the initialization of local definitions.

c)	If an altstep includes port operations or uses component variables, constants or timers the associated component type shall be referenced using the runs on keywords in the altstep header. The one exception to this rule is if all ports, variables, constants and timers used within the altstep are passed in as parameters.

d)	An altstep without a runs on clause shall never invoke a function or altstep or activate an altstep as default with a runs on clause locally.

e)	An altstep that is activated as a default shall only have in value or template parameters. An altstep that is only invoked as an alternative in an alt statement or as stand-alone statement in a TTCN-3 behaviour description may have in, out and inout parameters. The rules for formal parameter lists shall be followed as defined in clause 5.4.

f)	Altsteps started by using the start test component operation shall always have a runs on clause (see clause 22.5) and are considered to be invoked in the component to be started, i.e. not locally. However, the start test component operation may be invoked within behaviour without a runs on clause.

g)	If the altstep is an interleave alstep, all restrictions of the interleave statement (see clause 20.4) apply to the top alternatives of the altstep as well.

Examples

EXAMPLE 1:	Parameterized altstep with runs on clause:

// Given

type component MyComponentType {

var integer vc_myIntVar := 0;

timer tc_myTimer;

port myPortTypeOne pCO1, pCO2;

port myPortTypeTwo pCO3;

}

// Altstep definition using pCO1, pCO2, vc_myIntVar and tc_myTimer of MyComponentType

altstep a_altSet_A(in integer p_myPar1) runs on MyComponentType {

[] pCO1.receive(mw_myTemplate(p_myPar1, vc_myIntVar)) {

setverdict(inconc);

}

[] pCO2.receive {

if (p_myPar1 != 0) {

repeat

}

else {

break

}

}

[] tc_myTimer.timeout {

setverdict(fail);

stop

}

}

EXAMPLE 2:	Altstep with local definitions:

altstep a_anotherAltStep(in integer p_myPar1) runs on MyComponentType {

var integer v_myLocalVar := f_myFunction();			// local variable

const float c_myFloat := 3.41;						// local constant

[] pCO1.receive(MyTemplate(p_myPar1, v_myLocalVar)) {

setverdict(inconc);

}

[] pCO2.receive {

repeat

}

}

EXAMPLE 3:	Interleave Altstep:

altstep interleave a_interleaveAltStep(in integer p_myPar1, in integer p_myPar2)

runs on MyComponentType {

var integer v_myLocalVar := f_myFunction();			// local variable

[] pCO1.receive(MyTemplate(p_myPar1, v_myLocalVar)) {}

[] pCO1.receive(MyTemplate(p_myPar2, v_myLocalVar)) {}

}

### 16.2.1	Invoking altsteps

The invocation of an altstep is always related to an alt statement. The invocation may be done either implicitly by the default mechanism (see clause C.5) or explicitly by a direct call within an alt statement (see clause 20.2).

Syntactical Structure

[ @nodefault ] AltstepRef "(" [ { ActualPar [","] } ] ")"

Semantic Description

The invocation of an altstep causes no new snapshot and the evaluation of the top alternatives of an altstep is done by using the actual snapshot of the alt statement from which the altstep was called.

NOTE 1:	A new snapshot within an altstep will of course be taken, if within a selected top alternative a new alt statement is specified and entered.

For an implicit invocation of an altstep by means of the default mechanism, the altstep shall be activated as a default by means of an activate statement before the place of the invocation is reached.

An explicit call of an altstep within an alt statement looks syntactically like a function invocation as an alternative. When an altstep is called explicitly within an alt statement, the next alternative to be checked is the first alternative of the altstep. The alternatives of the altstep are checked and executed the same way as alternatives of an alt statement (see clause 20.1) with the exception that no new snapshot is taken when entering the altstep. An unsuccessful termination of the altstep (i.e. all top alternatives of the altstep have been checked and no matching branch is found) causes the evaluation of the next alternative or invocation of the default mechanism (if the explicit call is the last alternative of the alt statement). A successful termination may cause either the termination of the test component, i.e. the altstep ends with a stop statement, or a new snapshot and re-evaluation of the alt statement, i.e. the altstep ends with repeat (see clause 20.2) or a continuation immediately after the alt statement, i.e. the execution of the selected top alternative of the altstep ends with a break statement (see clause 19.12) or without explicit repeat or stop.

NOTE 2:	Due to the possibility of defining dynamic test configurations, an alternative in an explicitly invoked altstep may refer to a disconnected or unmapped port at the time of its evaluation. In TTCN-3, ports belong to the receiving component and matching is related to the top elements in the port queues. Dynamically unmapped and disconnected ports contribute to a snapshot in the same manner as mapped and connected ports. This means, an explicitly invoked altstep may execute receiving operations that empty the queues of unmapped and disconnected ports without causing a test case error.

An altstep can also be called as a stand-alone statement in a TTCN-3 behaviour description. In this case, the call of the altstep can be interpreted as shorthand for an alt statement with only one alternative describing the explicit call of the altstep. If the @nodefault modifier is placed before a stand-alone altstep call, the implicit alt statement also contains the @nodefault modifier.

Restrictions

In addition to the general static rules of TTCN-3 given in clause 5, the following restrictions apply:

a)	When invoking an altstep, the compatibility of the test component type of the invoking test component and of the altstep runs on clause (as described in clause 6.3.2.7) need to be fulfilled.

b)	Further restrictions on invoking altsteps in the activate statement are given in clause 20.5.2.

c)	When invoking an altstep, the mtc and system compatibility of the mtc and system components of the invoked altstep with the actual mtc and system types of the running test case as described in clause 6.3.2.7 need to be fulfilled.

Examples

EXAMPLE 1:	Implicit invocation of an altstep via a default activation

:

var default v_myDefVarTwo := activate(a_mySecondAltStep()); // Activation of an altstep as

// default

:

EXAMPLE 2:	Explicit invocation of an altstep within an alt statement

:

alt {

[] pCO3.receive {

…

}

[] a_anotherAltStep();	// explicit call of altstep a_anotherAltStep as an alternative

// of an alt statement

[] t_myTimer.timeout {}

}

EXAMPLE 3:	Explicit, stand-alone invocation of an altstep

// The statement

a_anotherAltStep(); // a_anotherAltStep is assumed to be a correctly defined altstep

//is a shorthand for

alt {

[] a_anotherAltStep();

}

## 16.3	Test cases

A test case is complete and independent specification of the actions required to achieve a specific test purpose. It typically starts in a stable testing state and ends in a stable testing state. It may involve one or more consecutive or concurrent connections to the SUT. The test case shall be complete in the sense that it is sufficient to enable a test verdict to be assigned unambiguously to each potentially observable test outcome (i.e. sequence of test events). The test case shall be independent in the sense that it shall be possible to execute the derived executable test case in isolation from other such test cases.

In TTCN-3, test cases are a special kind of function. Test cases define the behaviour, which have to be executed to check whether the SUT passes a test or not. This behaviour is performed by the MTC which is automatically created when a test case is being executed.

Syntactical Structure

testcase TestcaseIdentifier

"(" [ { ( FormalValuePar | FormalTemplatePar) [","] } ] ")"

runs on ComponentType

[ system ComponentType ]

StatementBlock

Semantic Description

A test case is considered to be a self-contained and complete specification that checks a test purpose. The result of a test case execution is a test verdict.

A test case header has two parts:

- interface part (mandatory): denoted by the keyword runs on which references the required component type for the MTC and makes the associated port names visible within the MTC behaviour; and

- test system part (optional): denoted by the keyword system which references the component type which defines the required ports for the test system interface. The test system part shall only be omitted if, during test execution, only the MTC is instantiated. In this case, the MTC type defines the test system interface ports implicitly;

- all formal parameter types of the test case shall be of a data type.

The behaviour of a test case can be defined by using the program statements and operations described in clause 18.

Test cases may be parameterized as described in clause 5.4. Test cases can be executed from control behaviour definitions (see clause 26).

Restrictions

In addition to the general static rules of TTCN-3 given in clause 5, the following restrictions apply:

a)	The rules for formal parameter lists shall be followed as defined in clause 5.4.

b)	Test cases may only be invoked with an execute statement in a function or altstep running on a control component as defined in clause 26.

Examples

testcase TC_MyTestCaseOne()

runs on MyMtcType1			// defines the type of the MTC

system MyTestSystemType		// makes the port names of the TSI visible to the MTC

{

:	// The behaviour defined here executes on the mtc when the test case invoked

}

// or, a test case where only the MTC is instantiated

testcase TC_MyTestCaseTwo() runs on MyMtcType2

{

:	// The behaviour defined here executes on the mtc when the test case invoked

}

# 17	Void

# 18	Overview of program statements and operations

The fundamental program elements of test cases, functions and altsteps are expressions, basic program statements such as assignments, loop constructs, etc., behavioural statements such as sequential behaviour, alternative behaviour, interleaving, defaults, etc., and operations such as send, receive, create, etc.

Statements can be either single statements (which do not include other program statements) or compound statements (which may include other statements and statement blocks).

Statements shall be executed in the order of their appearance, i.e. sequentially, as illustrated in figure 8.

Figure 8: Illustration of sequential behaviour

The individual statements in the sequence shall be separated by the delimiter ";".

EXAMPLE:

MyPort.send(Mymessage); MyTimer.start; log("Done!");

The specification of an empty statement block, i.e. {}, may be found in compound statements, e.g. a branch in an alt statement, and implies that no actions are taken.

Table 16 gives an overview of the TTCN-3 expressions, statements and operations and restrictions on their usage.

Table 16: Overview of TTCN-3 expressions, statements and operations

| Statement | Associated keyword or symbol | Can be invoked by control behaviour | Can be invoked by test behaviour | Can be directly or indirectly invoked from specific places (see note 1) |
| --- | --- | --- | --- | --- |
| Expressions | (…) | Yes | Yes | Yes |
| Basic program statements |  |  |  |  |
| Assignments | := | Yes | Yes | Yes (see note 3) |
| If-else | if (…) {…} else {…} | Yes | Yes | Yes |
| Select case | select case (…) { case (…) {…} case else {…}} | Yes | Yes | Yes |
| For loop | for (…) {…} | Yes | Yes | Yes |
| While loop | while (…) {…} | Yes | Yes | Yes |
| Do while loop | do {…} while (…) | Yes | Yes | Yes |
| Label and Goto | label / goto | Yes | Yes | Yes |
| Stop execution | stop | Yes | Yes |  |
| Returning control | return |  | Yes (see note 4) | Yes |
| Leaving a loop, alt, altstep or interleave | break | Yes | Yes | Yes |
| Next iteration of a loop | continue | Yes | Yes | Yes |
| Logging | log | Yes | Yes | Yes |
| Statements and operations for alternative behaviour |  |  |  |  |
| Alternative behaviour | alt {…} | Yes (see note 2) | Yes |  |
| Re-evaluation of alternative behaviour | repeat | Yes | Yes |  |
| Interleaved behaviour | interleave {…} | Yes (see note 2) | Yes |  |
| Activate a default | activate | Yes | Yes |  |
| Deactivate a default | deactivate | Yes | Yes |  |
| Configuration operations |  |  |  |  |
| Create parallel test component | create |  | Yes |  |
| Connect component port to component port | connect |  | Yes |  |
| Disconnect two component ports | disconnect |  | Yes |  |
| Map port to test interface | map |  | Yes |  |
| Unmap port from test system interface | unmap |  | Yes |  |
| Get MTC component reference value | mtc |  | Yes | Yes |
| Get test system interface component reference value | system |  | Yes | Yes |
| Get own component reference value | self |  | Yes | Yes |
| Start execution of test component behaviour | start |  | Yes |  |
| Stop execution of test component behaviour | stop |  | Yes |  |
| Terminating the testcase with an error verdict | testcase.stop |  | Yes | Yes |
| Remove a test component from the system | kill |  | Yes |  |
| Check termination of a PTC behaviour | running |  | Yes |  |
| Check if a PTC exists in the test system | alive |  | Yes |  |
| Wait for termination of a PTC behaviour | done |  | Yes |  |
| Wait a PTC cease to exist | killed |  | Yes |  |
| Communication operations |  |  |  |  |
| Send message | send |  | Yes |  |
| Invoke procedure call | call |  | Yes |  |
| Reply to procedure call from remote entity | reply |  | Yes |  |
| Raise exception (to an accepted call) | raise |  | Yes |  |
| Receive message | receive |  | Yes |  |
| Trigger on message | trigger |  | Yes |  |
| Accept procedure call from remote entity | getcall |  | Yes |  |
| Handle response from a previous call | getreply |  | Yes |  |
| Catch exception (from called entity) | catch |  | Yes |  |
| Check (current) message/call received | check |  | Yes |  |
| Clear port queue | clear |  | Yes |  |
| Clear queue and enable sending & receiving at a to port | start |  | Yes |  |
| Disable sending and disallow receiving operations to match at a port | stop |  | Yes |  |
| Disable sending and disallow receiving operations to match new messages/calls | halt |  | Yes |  |
| Check the state of a port | checkstate |  | Yes |  |
| Timer operations |  |  |  |  |
| Start timer | start | Yes | Yes |  |
| Stop timer | stop | Yes | Yes |  |
| Read elapsed time | read | Yes | Yes |  |
| Check if timer running | running | Yes | Yes |  |
| Timeout event | timeout | Yes | Yes |  |
| Verdict operations |  |  |  |  |
| Set local verdict | setverdict |  | Yes |  |
| Get local verdict | getverdict |  | Yes | Yes |
| External actions |  |  |  |  |
| Stimulate an (SUT) action externally | action | Yes | Yes |  |
| Execution of test cases |  |  |  |  |
| Execute test case | execute | Yes |  |  |
| NOTE 1: Specific places are defined in clause 16.1.4. Only operations that do not have any potential side effects on snapshot evaluation are allowed. NOTE 2: Can be used to control timer operations only. NOTE 3: Changing of component variables is disallowed. NOTE 4: Can be used in functions and altsteps but not in test cases. |  |  |  |  |

# 19	Basic program statements

## 19.0	General

Table 17 provides an overview of the TTCN-3 basic program statements.

Table 17: Overview of TTCN-3 basic program statements

| Basic program statements |  |
| --- | --- |
| Statement | Associated keyword or symbol |
| Assignments | := |
| If-else | if (…) {…} else {…} |
| Select case | select case (…) { case (…) {…} case else {…}} |
| For loop | for (…) {…} |
| While loop | while (…) {…} |
| Do while loop | do {…} while (…) |
| Label and Goto | label / goto |
| Stop execution | stop |
| Returning control | return |
| Leaving a loop, alt, altstep or interleave | break |
| Next iteration of a loop | continue |
| Logging | log |

Restrictions

In addition to the general static rules of TTCN-3 given in clause 5, the following restrictions apply:

a)	Unless specified differently in the relevant clause, all values and templates used in a basic program statement shall be completely initialized (for exemption see e.g. clause 19.1).

NOTE:	Note that the restriction applies to component of statements defined in the present document, like the boolean condition of if statements, but not to the content of statement blocks embedded into the statements.

## 19.1	Assignments

Values or templates may be assigned to variables or template variables (see clause 11). This is indicated by the symbol ":=".

Syntactical Structure

ValueRef ":=" ( Expression | TemplateBody )

Semantic Description

During execution of an assignment, the right-hand side of the assignment shall evaluate to a value or template that is at least partially initialized. The effect of an assignment is to bind the variable to the value of the expression or to a template. Assignments use the rules of passing by value. If the variable being assigned is of a type whose values are object references, only the reference is copied, but the referenced object (e.g. component, timer or port) is not. In all other cases, the content being assigned shall be a copy of the evaluated right-hand side.

Assignments are processed from left to right, i.e. expressions in the left hand side are evaluated before those in the right hand side. The evaluations obey the operator precedence defined in table 6. Unless the assignment is to a lazy or fuzzy variable or parameter, the right hand side is evaluated completely before the resulting value or template is bound to the evaluated left-hand side of the assignment. Whenever assignments are used within the right hand side of an assignment (due to assignment notation), these rules apply recursively.

A structured value on the right-hand side of the assignment shall be assigned completely to the variable on the left-hand side of the assignment, If a partially initialized value is assigned to a completely initialized variable, fields uninitialized at the right-hand side of the assignment shall also become uninitialized at the left-hand side.

When a direct or indirect element or field of a lazy or fuzzy variable is assigned, the variable is also evaluated as much as necessary before assignment, i.e. if an ancestor of that element or field is initialized with a function call, it shall be evaluated. Thus, if the variable is fully assigned, it does not need to be evaluated before assignment.

NOTE:	If a sub-field or sub-element of a fuzzy variable is assigned that has an ancestor which was formerly assigned a function call, this function call will be evaluated once before the assignment and replaced by its result inside the variable. Thus, the other sub-fields and sub-elements of that ancestor, apart from the field or element being assigned become non-fuzzy.

Restrictions

In addition to the general static rules of TTCN-3 given in clause 5 and shown in table 16, the following restrictions apply:

- The right-hand side of an assignment shall evaluate to a value, template, port or timer which is type compatible with the variable at the left-hand side of the assignment.

- When the right-hand side of the assignment evaluates to a template (global or local template, in-line template, template variable or a matching mechanism), the variable at the left hand side shall be a template variable.

- The right-hand side of an assignment shall evaluate to an object that is at least partially initialized.

- If the left-hand side of the assignment is a reference to a non-optional value object (i.e. a value definition, a mandatory field, a record/set of/array element, a union alternative, a value parameter), the right-hand side shall not be a reference to an omitted field or the omit symbol.

- Using a reference to an omitted field in the right-hand side of the assignment has the same effect as using the omit keyword.

Examples

EXAMPLE 1:

v_myVariable := (c_x + c_y – f_increment(c_z))*3;

EXAMPLE 2:

type record MyRecord {

record { float x, float y } c,

integer a

}

var @lazy MyRecord v_r := {

c := f_computeC(),

a := f_computeA()

} // not evaluated here

v_r.c.x := f_computeX();	// first replaces field c with result of f_computeC(),

// then replaces field c.x with unevaluated f_computeX()

// field while c.y remains fixed; field a remains unevaluated

EXAMPLE 3:

type record MyRecord {

charstring field1,

charstring field2,

charstring field3

}

var MyRecord v_myList1, v_myList2, v_myList3;

v_myList1 := {"value1", "value2", "value3" }; 	// v_myList1 is completely initialized

v_myList2 := v_myList1;							// v_myList2 is equal to  {"value1", "value2", 
													// "value3" }

v_myList2.field1 := "missing";					// only v_myList2 value changes to
													// {"missing", "value2", "value3" }; 
													// v_myList1 still contains {"value1", "value2", 
													// "value3" } after the assignment

v_myList3.field2 := "newvalue";					// v_myList3 is partially initialized

// field1 and field3 remain uninitialized

v_myList1 := v_myList3;							// v_myList1 become partially initialized,

// field2 has the value "newvalue"

// field1 and field3 are uninitialized

## 19.2	The If-else statement

The if-else statement, also known as the conditional statement, is used to denote branching in the control flow.

Syntactical Structure

if "(" BooleanExpression ")" StatementBlock

{ else if "(" BooleanExpression ")" StatementBlock }

[ else StatementBlock]

NOTE:	else if "(" BooleanExpression ")" StatementBlock [ else StatementBlock] is a shorthand notation for
else "{" if "(" BooleanExpression ")" StatementBlock [ else StatementBlock] "}".

Semantic Description

The branching of the control flow is decided upon the value of the Boolean expressions - the condition. A statement block - and only one - will be executed, if its condition evaluates to true. The optional else specifies a statement block that will be executed if all the "if" and "else if" conditions before are false.

Restrictions

No specific restrictions in addition to the general static rules of TTCN-3 given in clause 5 and shown in table 16.

Examples

if (v_date == "1.1.2005") { return ( fail ); }

if (v_myVar < 10) { v_myVar := v_myVar * 10; log ("v_myVar < 10"); }

else { v_myVar := v_myVar/5; }

## 19.3	The Select statements

### 19.3.1	The Select case statement

The select case statement is an alternative syntactic form of the if-else statement.

Syntactical Structure

select "(" SingleExpression ")" "{"

{ case "(" { TemplateInstance[","] } ")" StatementBlock }+

[ case else StatementBlock ]

"}"

Semantic Description

The select case statement is an alternative to using if .. else if .. else statements when comparing a value to one or several other values. The statement contains a header part and one or more branches. Never more than one of the branches is executed.

In the header part of the select case statement an expression shall be given. Each branch starts with the case keyword followed by a list of templateInstance (a list branch, which may also contain a single element) or in the last branch the case keyword may be followed by the else keyword (an else branch) and a statement block.

All templateInstance in all list branches shall be of a type compatible with the type of the expression in the header. 
A list branch is selected and the statement block of the selected branch is executed only, if any of the templateInstance matches the value of the expression in the header of the statement. On executing the statement block of the selected branch (i.e. not jumping out by a goto statement), execution continues with the statement following the select case statement.

The statement block of an else branch is always executed if no other branch textually preceding the else branch has been selected.

Branches are evaluated in their textual order. If none of the templateInstance-s matches the value of the expression in the header and the statement contains no else branch, execution continues without executing any of the select case branches.

NOTE 1:	In general, it cannot be decided if templateInstances overlap or not. However, it is advised to use in the branches templateInstances that don't overlap. In such situations tools might provide better runtime performance. The handling however is tool-specific.

NOTE 2:	When more than one branch could be selected (the templateInstances overlap) the textually first will be selected. For this reason overlapping is discouraged, handling however is tool-specific.

Restrictions

In addition to the general static rules of TTCN-3 given in clause 5 and shown in table 16, the following restrictions apply:

- The select SingleExpression and the case TemplateInstance-s shall be type compatible.

- When all templateInstances of all branches can be statically evaluated in compile time to specific values or value ranges no two branches shall match the same value.

Examples

select (PX_MyModulePar)  // where PX_MyModulePar is of charstring type

{
		case (charstring:"firstValue")
			{
			 log ("The first branch is selected");
			}
		case (v_myCharVar, c_myCharConst)
			{
			 log ("The second branch is selected");
			}
		case else
			{
			 log ("The value of the module parameter PX_MyModulePar is selected");
			}

}

// the above select statement is equivalent to the following nested if-else statement.

// Note: the following textual replacement of the select-case statement is described in

// the operational semantics of TTCN-3.

{

var charstring v_myLocalVar  := PX_MyModulePar;

if (match(v_myLocalVar , charstring:"firstValue")
			{
			 log ("The first branch is selected");
			}
		else if (match(v_myLocalVar , v_myCharVar) or match(v_myLocalVar , c_myCharConst))
			{
			 log ("The second branch is selected");
			}
		else 
			{
			 log ("The value of the module parameter PX_MyModulePar is selected");
			}

}

### 19.3.2	The Select union statement

To allow easier usage of the select statement for values of union types or anytype, a special form of the select statement exists.

Syntactical Structure

select union "(" SingleExpression ")" "{"

{ case "(" ({ Identifier [","] } | { TypeIdentifier [","] }) ")" StatementBlock }+

[ case else StatementBlock ]

"}"

Semantic Description

The statement contains a header part and one or more branches. Never more than one of the branches is executed.

In the header part of the select union statement a template instance of union type or anytype shall be given. If the template instance has a union type, each branch shall start with the case keyword followed by one or more identifiers of the alternatives (fields) of the union type (a list branch) or in the last branch the case keyword may be followed by the else keyword (an else branch) and a statement block. If the template instance has type anytype, each branch shall start with the case keyword followed by one or more type names (a list branch) or in the last branch the case keyword may be followed by the else keyword (an else branch) and a statement block. The StatementBlock of the list branch containing the identifier or type identifier of the chosen alternative is executed. If no case exists for the chosen alternative, the StatementBlock of the else branch, if it is present, is executed. Otherwise, the select union statement has no effect.

Restrictions

- The SingleExpression in the header of the select union statement shall be of a union type. It shall be at least partially initialized.

- Every Identifier in a case of the select union statement shall be an identifier of an alternative of the union type of the template instance given to the statement's header.

- No two cases in a select union statement shall have the same case Identifier or TypeIdentifier.

Examples

type union Messages {

MyMessageType1 msg1,

MyMessageType2 msg2,

MyMessageType3 msg3,

MyMessageType4 msg4,

MyMessageType5 msg5

}

function f_f(in Messages p_msg) {

select union (p_msg) {

case (msg1) { log(p_msg.msg1); }

case (msg2) { log(p_msg.msg2); }

case (msg3, msg4) { log("either msg3 or msg4"); }

case else { log("unhandled variant"); }

}

function f_g(in anytype p_msg) {

select union (p_msg) {

case (integer) { log(p_msg.integer); }

case (Messages) { f_f(p_msg.Messages); }

case else { log("unhandled anytype variant"); }

}

}

## 19.4	The For statement

The for statement defines a counter loop.

Syntactical Structure

for "(" ( VarInstance | Assignment ) ";" BooleanExpression ";" Assignment ")"

StatementBlock

Semantic Description

The for statement contains two assignments and a boolean expression. The first assignment is necessary to initialize the index (or counter) variable of the loop. The boolean expression terminates the loop and the second assignment is used to manipulate the index variable.

The value of the index variable is increased, decreased or manipulated in such a manner that after a certain number of execution loops a termination criteria is reached.

The termination criterion of the loop shall be expressed by a boolean expression. It is checked at the beginning of each new loop iteration. If it evaluates to true, the execution continues with the statement block in the for statement, if it evaluates to false, the execution continues with the statement which immediately follows the for loop. If a break statement is executed that is not within the body of an enclosed loop, alt, alststep or interleave, then the loop is terminated, too.

The index variable of a for loop can be declared before being used in the for statement or can be declared and initialized in the for statement header. If the index variable is declared and initialized in the for statement header, the scope of the index variable is limited to the loop body, i.e. it is only visible inside the loop body.

Restrictions

No specific restrictions in addition to the general static rules of TTCN-3 given in clause 5 and shown in table 16.

Examples

var integer v_j;							// Declaration of integer variable v_j

for (v_j:=1; v_j<=10; v_j:= v_j+1) { … }	// Usage of variable v_j as index variable of the

// for loop

for (var float v_i:=1.0; v_i<7.9; v_i:= v_i*1.35) { … }	// Index variable v_i is declared and

// initialized in the for loop header. Variable

// v_i only is visible in the loop body.

## 19.5	The While statement

A while statement defines a loop that is executed as long as the loop condition holds.

Syntactical Structure

while "(" BooleanExpression ")" StatementBlock

Semantic Description

The loop condition shall be checked at the beginning of each new loop iteration. If the loop condition does not hold, then the loop is exited and execution shall continue with the statement, which immediately follows the while loop. If a break statement is executed that is not within the body of an enclosed loop, alt, alststep or interleave, then the loop is terminated, too.

Restrictions

No specific restrictions in addition to the general static rules of TTCN-3 given in clause 5 and shown in table 16.

Examples

while (v_j<10){ … }

## 19.6	The Do-while statement

A do-while statement defines a loop that is executed up until the loop condition does not hold.

Syntactical Structure

do StatementBlock while "(" BooleanExpression ")"

Semantic Description

The do-while loop is identical to a while loop with the exception that the loop condition shall be checked at the end of each loop iteration. This means when using a do-while loop the behaviour is executed at least once before the loop condition is evaluated for the first time. If a break statement is executed that is not within the body of an enclosed loop, alt, alststep or interleave, then the loop is terminated, too.

Restrictions

No specific restrictions in addition to the general static rules of TTCN-3 given in clause 5 and shown in table 16.

Examples

do { … } while (v_j<10);

## 19.7	The Label statement

The label statement allows the specification of labels in test cases, functions and altsteps.

Syntactical Structure

label LabelIdentifier

Semantic Description

A label marks a statement. The label is used by the goto statement (see clause 19.8) to transfer control to a labelled statement.

Restrictions

In addition to the general static rules of TTCN-3 given in clause 5 and shown in table 16, the following restrictions apply:

- A label statement can be used freely like other TTCN-3 behavioural program statements according to the syntax rules defined in annex A. It can be used before or after a TTCN-3 statement but not as the first statement of an alternative or top alternative in an alt statement, interleave statement or altstep.

- Labels used following the label keyword shall be unique among all labels defined in the same test case, function or altstep.

Examples

label MyLabel;							// Defines the label MyLabel

// The labels L1, L2 and L3 are defined in the following TTCN-3 code fragment

:

label L1;								// Definition of label L1

alt{

[]	pCO1.receive(mw_mySig1)

{	label L2;						// Definition of label L2

pCO1.send(m_mySig2);

pCO1.receive(mw_ySig3)

}

[]	pCO2.receive(mw_mySig4)

{	pCO2.send(m_mySig5);

pCO2.send(m_mySig6);

label L3;						// Definition of label L3

pCO2.receive(mw_mySig7);

}

}

:

## 19.8	The Goto statement

A goto statement performs a jump to a label.

Syntactical Structure

goto LabelIdentifier

Semantic Description

The goto statement can be used in functions, test cases and altsteps to transfer control to a labelled statement.

The goto statement provides the possibility to jump freely, i.e. forwards and backwards, within a sequence of statements, to jump out of a single compound statement (e.g. a while loop) and to jump over several levels out of nested compound statements (e.g. nested alternatives).

Restrictions

In addition to the general static rules of TTCN-3 given in clause 5 and shown in table 16, the following restrictions apply:

a)	It is not allowed to jump out of or into functions, test cases and altsteps.

b)	It is not allowed to jump into a sequence of statements defined in a compound statement (i.e. alt statement, while loop, for loop, if-else statement, do-while loop and the interleave statement).

c)	It is not allowed to use the goto statement within an interleave statement.

Examples

// The following TTCN-3 code fragment includes

:

label L1;								// … the definition of label L1,

m_myVar := 2 * m_myVar;

if (m_myVar < 2000) { goto L1; }		// … a jump backward to L1,

m_myVar2 := f_myFunction(m_myVar);

if (m_myVar2 > m_myVar) { goto L2; }	// … a jump forward to L2,

pCO1.send(m_myVar);

pCO1.receive;

label L2; 								// … the definition of label L2,

pCO2.send(integer: 21);

alt {

[] pCO1.receive { }

[] pCO2.receive(integer: 67) {

label L3;						// … the definition of label L3,

pCO2.send(m_myVar);

alt {

[] pCO1.receive { }

[] pCO2.receive(integer: 90) {

pCO2.send(integer: 33);

pCO2.receive(integer: 13);

goto L4;			// … a jump forward out of two nested alt statements,

}

[] pCO2.receive(mw_myError) {

goto L3;			// … a jump backward out of the current alt statement,

}

[] any port.receive {

goto L2;			// … a jump backward out of two nested alt statements,

}

}

}

[] any port.receive {

goto L2;					// … and a long jump backward out of an alt statement.

}

}

label L4;

:

## 19.9	The Stop execution statement

The stop statement terminates execution of test components, a test case or a test control.

Syntactical Structure

stop

Semantic Description

The stop statement terminates execution in different ways depending on the context in which it is executed. When executed in a control behaviour, it terminates the execution of the control component. When invoked in a test case, altstep or function that are executed on a test component, it terminates the relevant test component.

NOTE:	The semantics of a stop statement that terminates a test component is identical to the stop component operation self.stop (see clause 21.3.3).

Restrictions

No specific restrictions in addition to the general static rules of TTCN-3 given in clause 5 and shown in table 16.

Examples

module MyModule {

: 	// Module definitions

testcase TC_MyTestCase() runs on MyMTCType system MySystemType{

var MyPTCType v_ptc:= MyPTCType.create;	// PTC creation

v_ptc.start(f_myFunction());				// start PTC execution

:			// test case behaviour continued

stop		// stops the MTC, all PTCs and the whole test case

}

function f_myFunction() runs on MyPTCType {

:

stop		// stops the PTC only, the test case continues

}

control {

:	// test execution

stop	// stops the test campaign

} // end control

} // end module

## 19.10	The Return statement

The return statement terminates execution of functions or altsteps.

Syntactical Structure

return [ Expression | TemplateInstance ]

Semantic Description

The return statement terminates execution of a function or altstep and returns control to the point from which the function or altstep was called. When used in functions, a return statement may be optionally associated with a return value or template.

TTCN-3 allows optional statement blocks that may follow altstep calls within alt statements. If there is a statement block, the return statement returns control to the beginning of this statement block and the statement block is executed before the alt statement is left. If there is no statement block, test execution continues with the first statement following the alt statement.

The return value or template is first evaluated before returning.

Restrictions

In addition to the general static rules of TTCN-3 given in clause 5 and shown in table 16, the following restrictions apply:

- The return statement shall not be present in testcase definitions.

Examples

function f_myFunction() return boolean {

:

if (v_date == "1.1.2005") {

return false; 	// execution stops on the 1.1.2005 and returns the boolean false

}

:

return true;		// true is returned

}

function f_myTemplateFunction() return template charstring {

:

if (v_date == "1.1.2005") {

return "2005"; 	// the string of the year is returned

}

:

return ?;			// the any template is returned

}

function f_myBehaviour() return verdicttype {

:

if (f_myFunction()) {

setverdict(pass); // use of f_myFunction in an if statement

}

else {

setverdict(inconc);

}

:

return getverdict; 	// explicit return of the verdict

}

## 19.11	The Log statement

The log statement provides the means to write logging information to some logging device. The information that can be logged is summarized in table 18.

Table 18: TTCN-3 language elements that can be logged

| Used in a log statement | What is logged | Comment |
| --- | --- | --- |
| module parameter identifier | actual value |  |
| literal value | value | This includes also free text. |
| data constant identifier | actual value |  |
| template instance | actual template or field values and matching symbols |  |
| variable identifier | actual value or "UNINITIALIZED" | See notes 3 and 4. |
| self, mtc, system or component type variable identifier | actual value and if assigned the component instance name otherwise "UNINITIALIZED" | On logging actual values see notes 2 to 4. Actual component states shall be logged according to note 5. |
| create operation | actual state and the component instance name | The actual state is always set to Inactive. |
| running operation (component or timer) | return value | true or false. In case of component or timer arrays, array element specification shall be included. |
| alive operation (component) | return value | true or false. In case of arrays, array element specifications shall be included. |
| checkstate operation | return value | true or false. |
| port instance | actual state | Port states shall be logged according to note 6. |
| default type variable identifier | actual state or "UNINITIALIZED" | Default states shall be logged according to note 7. See also notes 2 to 4. |
| activate operation | activate operation | The actual state is always set to activated. |
| timer name | actual state | Timer states shall be logged according to note 8. |
| read operation | return value | See clause 23.4. |
| match operation | return value | true or false. |
| valueof operation | return value |  |
| getverdict operation | return value | none, pass, inconc, or fail. |
| predefined functions | return value | See annex C. |
| function instance | return value | Only functions with return clause are allowed. |
| external function instance | return value | Only external functions with return clause are allowed. |
| formal parameter identifier | see comment column | Logging of actual parameters shall follow rules specified for the language elements they are substituting. In case of value parameters the actual parameter value, in case of template-type parameters the actual template or field values and matching symbols, in case of component type parameters the actual component reference, etc. shall be logged. |
| NOTE 1: Actual value/actual template is the value/template at the moment of the execution of the log statement. NOTE 2: The type of the logged value is tool dependent. NOTE 3: In case of array identifiers without array element specification, actual values and for component references names of all array elements shall be logged. NOTE 4: The string "UNINITIALIZED" is logged only if the log item is unbound (uninitialized). NOTE 5: Component states that can be logged are: Inactive, Running, Stopped and Killed (for further details see annex F). NOTE 6: Port states that can be logged are: Started and Stopped (for further details see annex F). NOTE 7: Default states that can be logged are: Activated and Deactivated. NOTE 8: Timer states that can be logged are: Inactive, Running and Expired (for further details see annex F). NOTE 9: As the logging of the fields of structured values and templates is not standardized, tools supplied by different vendors may differ in the logged information. |  |  |

Syntactical Structure

log "(" { ( FreeText | TemplateInstance ) [","] } ")"

Semantic Description

The log statement provides the means to write one or more log items to some logging device associated with the test control or the test component in which the statement is used. Items to be logged shall be identified by a comma-separated list in the argument of the log statement. Log items may be individual language elements specified in table 18 or expressions composed of such log items.

It is strongly recommended that the execution of the log statement has no effect on the test behaviour. In particular, functions used in a log statement should not (explicitly or implicitly) change component variable values, port or timer status, and should not change the value of any of its inout or out parameters.

NOTE:	It is outside the scope of the present document to define complex logging and trace capabilities which may be tool dependent.

Restrictions

No specific restrictions in addition to the general static rules of TTCN-3 given in clause 5 and shown in table 16.

Examples

var integer v_myVar:= 1;

log("Line 248 in PTC_A: ", v_myVar, " (actual value of v_myVar)");

// The string "Line 248 in PTC_A: 1 (actual value of v_myVar)" is written to some log device

// of the test system

## 19.12	The Break statement

A break statement causes the exit from a loop, from an altstep or from an alt or interleave statement.

Syntactical Structure

break

Semantic Description

On executing a break statement the innermost, currently executed loop, alt statement or interleave statement is left. Execution continues with the statement following the construct which is left. Using break outside the body of a loop (for, while, do-while) or an alternative of an alt or interleave statement shall cause an error.

Altsteps are always executed within a surrounding alt statement. If the execution of a top alternative of an altstep (see clause 16.1.5) ends with a break statement, the altstep and the surrounding alt statement are left. Execution continues with the statement following the surrounding alt statement.

NOTE:	TTCN-3 allows optional statement blocks that may follow altstep calls within alt statements. These statement blocks are not executed when the altstep is left by executing a break statement. A return statement has to be used, if such an optional statement block has to be executed (see clause 19.10).

Restrictions

No specific restrictions in addition to the general static rules of TTCN-3 given in clause 5 and shown in table 16.

Examples

do {

…

if (v_cond1) {

break;			// the do-while loop is left

}

…

for (var integer v_j:=1; v_j<=10; v_j:= v_j+1) {

…

if (v_cond2) {

break;			// the for-loop is left but the do-while loop is continued

}

…

}

…

}

while (v_j<10);

## 19.13	The Continue statement

A continue statement causes the start of the next iteration of a loop.

Syntactical Structure

continue

Semantic Description

On executing a continue statement, the subsequent statements of the body of the innermost, currently executed loop are skipped and the next iteration starts. Using continue outside the body of a loop (for, while, do-while) shall cause an error.

Restrictions

No specific restrictions in addition to the general static rules of TTCN-3 given in clause 5 and shown in table 16.

Examples

do {

…

if (v_cond) {

continue;		// execution continues with the next iteration of the do-while-loop

}

…

…

for (var integer v_j:=1; v_j<=10; v_j:= v_j+1) {

…

if (v_cond2) {

continue;	// continues with the next iteration of the for-loop

}

…

}

…

}

while (v_j<10);

## 19.14	Statement block

Statement blocks can be used like basic program statements to introduce a local scope in the flow of control of TTCN-3 behaviour. The declarations and statements in a statement block are executed in the order of their appearance, i.e. sequentially.

Syntactical Structure

"{" { LocalDefinition | Statement } "}"

Semantic Description

A statement block defines a local scope unit. Scoping rules for TTCN-3 are defined in clause 5.2.

Restrictions

No specific restrictions in addition to the general static rules of TTCN-3 given in clause 5 and shown in table 16.

Examples

var integer v_aVar:= 0;			// v_aVar is declared

{								// start of a statement block

var integer v_myVar:= 2;	// v_myVar is declared

v_aVar := 5 + v_myVar;		// v_myVar is used in an assignment

}								// end of statement block

// after leaving the statement block, v_aVar is still known, but v_myVar is not known anymore.

# 20	Statement and operations for alternative behaviours

## 20.0	General

Test behaviour cannot only be expressed sequentially, but also as a set of alternatives or combinations of both. An interleaving operator allows the specification of interleaved sequences or alternatives. Table 19 summarizes the statements and operations for alternative behaviours.

Table 19: Overview of TTCN-3 statements and operations for alternative behaviours

| Statements and operations for alternative behaviours |  |
| --- | --- |
| Statement/Operation | Associated keyword or symbol |
| Alternative behaviour | alt { … } |
| Re-evaluation of alt statements | repeat |
| Interleaved behaviour | interleave { … } |
| Activate a default | activate |
| Deactivate a default | deactivate |

## 20.1	The snapshot mechanism

A more complex form of behaviour is where sequences of statements are expressed as sets of possible alternatives to form a tree of execution paths, as illustrated in figure 9.

Figure 9: Illustration of alternative behaviour

This is done with the alt statement.

When entering an alt statement, a snapshot is taken. A snapshot is considered to be a partial state of a test component that includes all information necessary to evaluate the Boolean conditions that guard alternative branches, all relevant stopped test components, all relevant timeout events and the top messages, calls, replies and exceptions in the relevant incoming port queues. Any test component, timer and port which is referenced in at least one alternative in the alt statement, or in a top alternative of an altstep that is invoked as an alternative in the alt statement or activated as default is considered to be relevant. A detailed description of the snapshot semantics is given in the operational semantics of TTCN-3 (part 4 of the TTCN-3 standard - ETSI ES 201 873-4 [1]).

NOTE 1:	Snapshots are only a conceptual means for describing the behaviour of the alt statement. The concrete algorithms for the snapshot handling can be found in part 4 of the TTCN-3 standard (ETSI ES 201 873-4 [1]).

NOTE 2:	The TTCN-3 semantics assumes that taking a snapshot is instantaneous, i.e. has no duration. In a real implementation, taking a snapshot may take some time and race conditions may occur. The handling of such race conditions is outside the scope of the present document.

## 20.2	The Alt statement

An alt statement expresses sets of possible alternatives that form a tree of possible execution paths.

Syntactical Structure

alt [ @nodefault ] "{"

{

"[" [ BooleanExpression ] "]"

( ( TimeoutStatement |

ReceiveStatement |

TriggerStatement |

GetCallStatement |

CatchStatement |

CheckStatement |

GetReplyStatement |

DoneStatement |

KilledStatement ) StatementBlock )

|

( AltstepInstance [ StatementBlock ] )

}

[ "[" else "]" StatementBlock ]

"}"

Semantic Description

The alt statement denotes branching of test behaviour due to the reception and handling of communication and/or timer events and/or the termination of parallel test components, i.e. it is related to the use of the TTCN-3 operations receive, trigger, getcall, getreply, catch, check, timeout, done and killed. The alt statement denotes a set of possible events that are to be matched against a particular snapshot.

Execution of alternative behaviour:

When entering an alt statement, a snapshot is taken.

The alternative branches in the alt statement and the top alternatives of invoked altsteps and altsteps that are activated as defaults are processed in the order of their appearance. If several defaults are active, the reverse order of their activation determines the evaluation order of the top alternatives in the defaults. The alternative branches in active defaults are reached by the default mechanism described in clause 20.5. If the alt statement contains the @nodefault modifier, all active default alternatives are ignored for the execution of this alt statement.

The individual alternative branches are either branches that may be guarded by a Boolean expression or else-branches, i.e. alternative branches starting with [else].

Else-branches are always chosen and executed when they are reached (see below).

Branches that may be guarded by boolean expressions either invoke an altstep (altstep-branch), or start with a done operation (done-branch), a killed operation (killed-branch), timeout operation (timeout-branch) or a receiving operation (receiving-branch), i.e. receive, trigger, getcall, getreply, catch or a check operation. The evaluation of the Boolean guards shall be based on the snapshot. The Boolean guard is considered to be fulfilled if no Boolean guard is defined, or if the Boolean guard evaluates to true. The branches are processed and executed in the following manner.

An altstep-branch is selected if the Boolean guard is fulfilled. The selection of an altstep-branch causes the invocation of the referenced altstep, i.e. the altstep is invoked and the evaluation of the snapshot continues within the altstep. An altstep-branche may contain an optional statement block. The optional statement block shall be executed only, if an alternative of the altstep referenced in the altstep-branch has been selected and executed.

A done-branch is selected if the Boolean guard is fulfilled and if the specified test component is in the list of stopped components of the snapshot. The selection causes the execution of the statement block following the done operation. The done operation itself has no further effect.

A killed-branch is selected if the Boolean guard is fulfilled and if the specified test component is in the list of killed components of the snapshot. The selection causes the execution of the statement block following the killed operation. The killed operation itself has no further effect.

A timeout-branch is selected if the Boolean guard is fulfilled and if the specified timeout event is in the timeout-list of the snapshot. The selection causes execution of the specified timeout operation, i.e. removal of the timeout event from the timeout-list, and the execution of the statement block following the timeout operation.

A receiving-branch is selected if the Boolean guard is fulfilled and if the matching criteria of receiving operation is fulfilled by one of the messages, calls, replies or exceptions in the snapshot. The selection causes execution of the receiving operation, i.e. removal of the matching message, call, reply or exception from the port queue, maybe an assignment of the received information to a variable and the execution of the statement block following the receiving operation. In the case of the trigger operation the top message of the queue is also removed if the Boolean guard is fulfilled but the matching criteria is not. In this case the statement block of the given alternative is not executed.

NOTE 1:	The TTCN-3 semantics describe the evaluation of a snapshot as a series of indivisible actions of a test component. The semantics do not assume that the evaluation of a snapshot has no duration. During the evaluation of a snapshot, test components may stop, timers may timeout and new messages, calls, replies or exceptions may enter the port queues of the component However, these events do not change the actual snapshot and thus, are not considered for the snapshot evaluation.

NOTE 2:	Due to the possibility of defining dynamic test configurations, a receiving branch may refer to a disconnected or unmapped port at the time of its evaluation. In TTCN-3, ports belong to the receiving component and matching is related to the top elements in the port queues. Dynamically unmapped and disconnected ports contribute to a snapshot in the same manner as mapped and connected ports. This means, the execution of receiving operations may empty the queues of unmapped and disconnected ports without causing a test case error.

If none of the alternative branches in the alt statement and top alternatives in the invoked altsteps and active defaults can be selected and executed, the alt statement shall be executed again, i.e. a new snapshot is taken and the evaluation of the alternative branches is repeated with the new snapshot. This repetitive procedure shall continue until either an alternative branch is selected and executed, or the test case is stopped by another component or by the test system (e.g. because the MTC is stopped) or with a dynamic error.

The test case shall stop and indicate a dynamic error if a test component is completely blocked. This means none of the alternatives can be chosen, no relevant test component is running, no relevant timer is running and all relevant ports contain at least one message, call, reply or exception that do not match.

NOTE 3:	The repetitive procedure of taking a complete snapshot and re-evaluate all alternatives is only a conceptual means for describing the semantics of the alt statement. The concrete algorithm that implements this semantics is outside the scope of the present document.

Selecting/deselecting an alternative:

If necessary, it is possible to enable/disable an alternative by means of a Boolean expression placed between the ("[…]") brackets of the alternative.

Else branch in alternatives:

Any branch in an alt statement can be defined as an else branch by including the else keyword between the opening and closing brackets at the beginning of the alternative. The statement block of the else branch is always executed if no other alternative textually preceding the else branch has proceeded.

Default mechanism:

It should be noted that the default mechanism (see clause 20.5) is always invoked at the end of all alternatives unless the @nodefault modifier is present. If an else branch is defined, the default mechanism will never be called, i.e. active defaults will never be entered.

NOTE 4:	It is also possible to use else in altsteps.

NOTE 5:	It is allowed to use a repeat statement within an else branch.

NOTE 6:	It is allowed to define more than one else branch in an alt statement or in an altstep, however always only the first else branch is executed.

Re-evaluation of alt statements:

The re-evaluation of an alt statement can be specified by using a repeat statement (see clause 20.3).

Invocation of altsteps as alternatives:

TTCN-3 allows the invocation of altsteps as alternatives in alt statements (see clause 16.2.1). When an altstep is explicitly invoked as an alternative, the optional statement block following the altstep call shall also be executed.

Continue execution after the alt statement:

Behaviour execution continues with the statement following the alt statement when one of the branches of the alt or invoked defaults is selected and completely executed, or a branch of an altstep used in an altsteps-branch is selected and the branch and the optional statement block following the invoked altstep are completely executed.

Execution also continues with the statement following the alt statement if a break statement is reached in the statement block of the selected branch of an alt statement, of an altstep used in an altstep-branch, or of an altstep invoked as default.

The alt statement can also be left by using a goto statement in the selected branch of the alt (i.e. no branches of altsteps and defaults can be considered in this case), and execution continues with the statement following the label, goto is pointing to.

Restrictions

In addition to the general static rules of TTCN-3 given in clause 5 and shown in table 16, the following restrictions apply:

- The open and close square brackets ("[…]") shall be present at the start of each alternative, even if they are empty. This not only aids readability but also is necessary to syntactically distinguish one alternative from another.

The evaluation of a Boolean expression guarding an alternative shall not have side effects. To avoid side effects that cause an inconsistency between the actual snapshot and the state of the component, the same restrictions as the restrictions for the initialization of local definitions within altsteps (clause 16.1.5) and the restrictions imposed on the contents of functions called from special places (clause 16.1.4) shall apply.

The evaluation of the event of an alt branch shall not have side effects. To avoid side effects that cause an inconsistency between the actual snapshot and the state of the component or introduce indeterminism in the evaluation of the following alt branches or the re-evaluation of the same alt branch, the restrictions imposed on the contents of functions called from special places (clause 16.1.4) shall apply to expressions occurring in the matching part of an alternative.

The evaluation of an altstep invoked from an alt branch, if none of the alternatives in the altstep is chosen, shall not have side effects. To avoid side effects the restrictions imposed on the contents of functions called from special places (clause 16.1.4) shall apply to the actual parameters of the invoked altstep.

Void.

An alt statement used inside control behaviour shall only contain timeout statements.

Examples

EXAMPLE 1:	Nested alternatives:

alt {

[] myPort.receive (mw_myMessage) {

setverdict (pass);

t_myTimer.start;

alt {

[] myPort.receive (mw_mySecondMessage) {

t_myTimer.stop;

setverdict (pass);

}

[] t_myTimer.timeout {

myPort.send (m_myRepeat);

t_myTimer.start;

alt {

[] myPort.receive (mw_mySecondMessage) {

t_myTimer.stop;

setverdict (pass)

}

[] t_myTimer.timeout { setverdict (inconc) }

[] myPort.receive { setverdict (fail) }

}

}

[] myPort.receive { setverdict (fail) }

}

}

[] t_myTimer.timeout { setverdict (inconc) }

[] myPort.receive { setverdict (fail) }

}

EXAMPLE 2:	Alt statement with guards:

alt {

[v_x>1] l2.receive {					// Boolean guard/expression

setverdict(pass);

}

[v_x<=1] l2.receive {					// Boolean guard/expression

setverdict(inconc);

}

}

EXAMPLE 3:	Alt statement with else branch:

// Use of alternative with Boolean expressions (or guard) and else branch

alt {

:

[else] {						// else branch

f_myErrorHandling();

setverdict(fail);

stop;

}

}

EXAMPLE 4:	Re-evaluation with repeat:

alt {

[] pCO3.receive {

v_count := v_count + 1;

repeat					// usage of repeat

}

[] t_t1.timeout { }

[] any port.receive {

setverdict(fail);

stop;

}

}

EXAMPLE 5:	Alt statement with explicitly invoked altstep:

alt {

[] pCO3.receive { }

[] a_anotherAltStep() { 	// Explicit call of altstep a_anotherAltStep as alternative.

setverdict(inconc)	// Statement block executed if an alternative within

// altstep AnotherAltStep has been selected and executed.

}

[] t_myTimer.timeout { }

}

EXAMPLE 6:	Alt statement with forbidden function calls:

alt {

[] f_getPort().receive(t(p())) { }  // forbidden if f_getPort, t or p has side effects

[] a_anotherAltStep(f()); 	      // forbidden if f has side effects

[] t_myTimer[i(p())].timeout { }    // forbidden if i or p has side effects

[f_g()] f_getComponent(p()).done {} // forbidden if f_g, f_getComponent or p has side effects

}

## 20.3	The Repeat statement

The repeat statement is used for a re-evaluation of an alt statement.

Syntactical Structure

repeat

Semantic Description

The repeat statement, when used in the statement block of alternatives of alt statements, causes the re-evaluation of the alt statement, i.e. a new snapshot is taken and the alternatives of the alt statement are evaluated in the order of their specification.

When used in statement blocks of the response and exception handling parts of blocking procedure calls, the repeat statement causes the re-evaluation of the response and exception handling part of the call (see clause 22.3.1).

If a repeat statement is used in a top alternative in an altstep definition, it causes a new snapshot and the re-evaluation of the alt statement from which the altstep has been called. The call of the altstep may either be done implicitly by the default mechanism (see clause 20.5.1) or explicitly in the alt statement (see clause 20.2).

Restrictions

In addition to the general static rules of TTCN-3 given in clause 5 and shown in table 16, the following restrictions apply:

- The repeat statement shall only be used within alt statements, call statements or altsteps.

Examples

EXAMPLE 1:	Usage of repeat in an alt statement:

alt {

[] pCO3.receive {

v_count := v_count + 1;

repeat					// usage of repeat

}

[] t_t1.timeout { }

[] any port.receive {

setverdict(fail);

stop;

}

}

EXAMPLE 2:	Usage of repeat in an altstep:

altstep a_anotherAltStep() runs on MyComponentType {

[] pCO1.receive{

setverdict(inconc);

repeat					// usage of repeat

}

[] pCO2.receive {}

}

## 20.4	The Interleave statement

The interleave statement allows to specify the interleaved occurrence and handling of receiving events including done, killed, timeout, receive, trigger, getcall, getreply, catch and check.

Syntactical Structure

interleave "{"

{ "[]" ( TimeoutStatement |

ReceiveStatement |

TriggerStatement |

GetCallStatement |

CatchStatement |

CheckStatement |

GetReplyStatement |

DoneStatement |

KilledStatement ) StatementBlock

}

"}"

Semantic Description

The interleave statement allows to specify the interleaved occurrence and handling of the statements done, killed, timeout, receive, trigger, getcall, getreply, catch and check.

Interleaved behaviour can always be replaced by an equivalent set of nested alt statements. The procedures for this replacement and the operational semantics of interleaving are described in part 4 of the TTCN-3 standard (ETSI ES 201 873-4 [1]).

The rules for the evaluation of an interleaving statement are the following:

- Whenever a reception statement is executed, the following non-reception statements are subsequently executed until the next reception statement is reached, a break statement is reached, or the interleaved sequence ends.

NOTE 1:	Reception statements are TTCN-3 statements which may occur in sets of alternatives, i.e. receive, check, trigger, getcall, getreply, catch, done, killed and timeout. Non-reception statements denote all other non-control-transfer statements which can be used within the interleave statement.

If none of the alternatives of the interleave statement can be executed, the default mechanism will be invoked. This means, according to the semantics of the default mechanism, the actual snapshot will be used to evaluate those altsteps that have been activated before entering the interleave statement.

NOTE 2:	The complete semantics of the default mechanism within an interleave statement is given by replacing the interleave statement by an equivalent set of nested alt statements. The default mechanism applies for each of these alt statements.

The evaluation then continues by taking the next snapshot if no break statement was encountered.

The evaluation of the interleave statement is terminated if a break statement is executed.

The operational semantics of interleaving are fully defined in part 4 of the TTCN-3 standard (ETSI ES 201 873-4 [1]).

Restrictions

In addition to the general static rules of TTCN-3 given in clause 5 and shown in table 16, the following restrictions apply:

- Control transfer statements activate, deactivate,  repeat, all calls of altsteps and (direct and indirect) calls of user-defined functions, which include reception statements, shall not be present in interleave statements.

- In addition, it is not allowed to guard branches of an interleave statement with Boolean expressions (i.e. the '[ ]' shall always be empty). It is also not allowed to specify else branches in interleaved behaviour.

- An interleave used inside control behaviour shall only contain timeout statements in the event parts of the alternatives.

- The restricted use of the control transfer statements for, while, do-while, and goto within interleave statements is allowed under the following conditions:

The loop statements for, while, and do-while can be used within statements blocks that do not contain reception statements.

The goto statement can be used for defining jumps with statements blocks that do not contain reception statements and for specifying jumps out of interleave statements.

EXAMPLE:

// The following TTCN-3 code fragment

interleave {

[]	pCO1.receive(mw_mySig1) {

PCO1.send(m_mySig2);

PCO1.receive(mw_mySig3);

}

[]	pCO2.receive(mw_mySig4) {

pCO2.send(m_mySig5);

pCO2.send(m_mySig6);

pCO2.receive(mw_mySig7);

}

}

// is a shorthand for

alt {

[]	PCO1.receive(mw_mySig1) {

PCO1.send(m_mySig2);

alt {

[]	PCO1.receive(mw_mySig3) {

alt {

[]	PCO2.receive(mw_mySig4) {

PCO2.send(m_mySig5);

PCO2.send(m_mySig6);

PCO2.receive(mw_mySig7)

}

}

}

[]	PCO2.receive(mw_mySig4) {

PCO2.send(m_mySig5);

PCO2.send(m_mySig6);

alt {

[]	PCO1.receive(mw_mySig3) {

PCO2.receive(mw_mySig7);

}

[]	PCO2.receive(mw_mySig7) {

PCO1.receive(mw_mySig3);

}

}

}

}

}

[]	pCO2.receive(mw_mySig4) {

pCO2.send(m_mySig5);

pCO2.send(m_mySig6);

alt {

[]	pCO1.receive(mw_mySig1) {

pCO1.send(m_mySig2);

alt {

[]	pCO1.receive(mw_mySig3) {

pCO2.receive(mw_mySig7);

}

[]	pCO2.receive(mw_mySig7) {

pCO1.receive(mw_mySig3);

}

}

}

[]	pCO2.receive(mw_mySig7) {

alt {

[]	pCO1.receive(mw_mySig1) {

pCO1.send(m_mySig2);

pCO1.receive(mw_mySig3);

}

}

}

}

}

}

## 20.5	Default Handling

### 20.5.0	General

TTCN-3 allows the activation of altsteps (see clause 16.1.5) as defaults. For each test component the defaults, i.e. activated altsteps, are stored as an ordered list. The defaults are listed in the reversed order of their activation i.e. the last activated default is the first element in the list of active defaults. The TTCN-3 operations activate (see clause 20.5.2) and deactivate (see clause 20.5.3) operate on the list of defaults. An activate puts a new default as the first element into the list and a deactivate removes a default from the list. A default in the default list can be identified by means of default reference that is generated as a result of the corresponding activate operation.

### 20.5.1	The default mechanism

The default mechanism is evoked at the end of each alt statement not annotated with the @nodefault modifier, if due to the actual snapshot none of the specified alternatives could be executed. An evoked default mechanism invokes the first altstep in the list of defaults, i.e. the last activated default, and waits for the result of its termination. The termination can be successful or unsuccessful. Unsuccessful means that none of the top alternatives of the altstep (see clause 16.1.5) defining the default behaviour could be selected, successful means that one of the top alternatives of the default has been selected and executed.

NOTE 1:	An interleave statement is semantically equivalent to a nested set of alt statements and the default mechanism also applies to each of these alt statements. This means, the default mechanism also applies to interleave statements. Furthermore, the restrictions imposed on interleave statements in clause 20.4 do not apply to altsteps that are activated as default behaviour for interleave statements.

NOTE 2:	Due to the possibility of defining dynamic test configurations, an alternative in an altstep activated as default may refer to a disconnected or unmapped port at the time of its evaluation. In TTCN-3, ports belong to the receiving component and matching is related to the top elements in the port queues. Dynamically unmapped and disconnected ports contribute to a snapshot in the same manner as mapped and connected ports. This means, an altstep invoked as default may execute receiving operations that empty the queues of unmapped and disconnected ports without causing a test case error.

In the case of an unsuccessful termination, the default mechanism invokes the next default in the list. If the last default in the list has terminated unsuccessfully, the default mechanism will return to the place in the alt statement in which it has been invoked, i.e. at the end of the alt statement, and indicate an unsuccessful default execution. An unsuccessful default execution will also be indicated if the list of defaults is empty.

An unsuccessful default execution may cause a new snapshot or a dynamic error if the test component is blocked (see clause 20.1).

In the case of a successful termination, the default may either stop the test component by means of a stop statement, or the main control flow of the test component will continue immediately after the alt statement from which the default mechanism was called or the test component will take new snapshot and re-evaluate the alt statement. The latter has to be specified by means of a repeat statement (see clause 20.3). If the execution of the selected top alternative of the default ends with a break statement or without a repeat statement the control flow of the test component will continue immediately after the alt statement.

NOTE 3:	TTCN-3 does not restrict the implementation of the default mechanism. It may for example be implemented in form of a process that is implicitly called at the end of each alt statement or in form of a separate thread that is only responsible for the default handling. The only requirement is that defaults are called in the reverse order of their activation when the default mechanism has been invoked.

### 20.5.2	The Activate operation

The activate operation is used to activate altsteps as defaults.

Syntactical Structure

activate "(" 	AltstepRef "(" [ { ActualPar [","] } ] ")" ")"

Semantic Description

An activate operation will put the referenced altstep as the first element into the list of defaults and return a default reference. The default reference is a unique identifier for the default and may be used in a deactivate operation for the deactivation of the default.

The effect of an activate operation is local to the test component in which it is called. This means, a test component cannot activate a default in another test component.

The activate operation can be called without saving the returned default reference. This form is useful in test cases which do not require explicit deactivation of the activated default, i.e. deactivation of a default is done implicitly at MTC termination.

The actual parameters of a parameterized altstep (see clause 16.2.1) that should be activated as a default, shall be provided in the corresponding activate statement. This means the actual parameters are bound to the default at the time of its activation (and not e.g. at the time of its invocation by the default mechanism).

Restrictions

In addition to the general static rules of TTCN-3 given in clause 5 and shown in table 16, the following restrictions apply:

- Void.

Void.

An altstep that is activated as a default shall only have in parameters.

Examples

EXAMPLE 1:	Activation where the default reference is kept

// Declaration of a variable for the handling of defaults

var default v_myDefaultVar := null;

:

// Declaration of a default reference variable and activation of an altstep as default

var default v_myDefVarTwo := activate(a_mySecondAltStep());

:

// Activation of altstep MyAltStep as a default

v_myDefaultVar := activate(a_myAltStep()); // a_myAltStep is activated as default

:

// Usage of v_myDefaultVar for the deactivation of default a_myDefAltStep

deactivate(v_myDefaultVar);

EXAMPLE 2:	Simple activation

// Activation of an altstep as a default, without assignment of default reference

activate(a_myCommonDefault());

EXAMPLE 3:	Activation of a parameterized altstep

altstep a_myAltStep2 ( integer    p_value1, MyType p_value2, 
	                       MyPortType p_port,   timer  p_timer )
	{
	 :
	}

function f_myFunc () runs on MyCompType

{ :

var default v_myDefaultVar := null;

v_myDefaultVar := activate(a_myAltStep2(5, v_myVar, vc_myCompPort, tc_myCompTimer);

// MyAltStep2 is activated as default with the actual parameters 5 and

// the value of v_myVar. A change of v_myVar before a call of a_myAltStep2 by

// the default mechanism will not change the actual parameters of the call.

:

}

### 20.5.3	The Deactivate operation

The deactivate operation is used to deactivate defaults, i.e. previously activated altsteps.

Syntactical Structure

deactivate [ "(" ObjectReference ")" ]

Semantic Description

A deactivate operation will remove the referenced default from the list of defaults.

The effect of a deactivate operation is local to the test component in which it is called. This means, a test component cannot deactivate a default in another test component.

A deactivate operation without parameter deactivates all defaults of a test component.

Calling a deactivate operation with the special value null has no effect. Calling a deactivate operation with an undefined default reference, e.g. an old reference to a default that has already been deactivated or an uninitialized default reference variable, shall cause a runtime error.

Restrictions

In addition to the general static rules of TTCN-3 given in clause 5 and shown in table 16, the following restrictions apply:

- The ObjectReference shall be of default type and shall not resolve to a template.

Examples

var default v_myDefaultVar := null;

var default v_myDefVarTwo := activate(a_mySecondAltStep());

var default v_myDefVarThree := activate(a_myThirdAltStep());

:

v_myDefaultVar := activate(a_myAltStep());

:

deactivate(v_myDefaultVar); // deactivates a_myAltStep

:

deactivate;	// deactivates all other defaults, i.e. in this case a_mySecondAltStep

// and a_myThirdAltStep

# 21	Configuration Operations

## 21.0	General

Configuration operations are used to set up and control test components and their connections. They are summarized in table 20.

Restrictions

In addition to the general static rules of TTCN-3 given in clause 5, the following restrictions apply:

- These operations shall only be used in:

TTCN-3 test cases;

behaviour invoked directly or indirectly from a test case or from behaviour started on a ptc.

They shall not be present in:

control behaviour;

declarations inside component type definitions; or

functions invoked directly or indirectly from declarations inside component type definitions.

Table 20: Overview of TTCN-3 configuration operations

| Operation | Explanation | Syntax Examples |
| --- | --- | --- |
| Connection Operations |  |  |
| connect | Connects the port of one test component to the port of another test component | connect(ptc1:p1, ptc2:p2); |
| disconnect | Disconnects two or more connected ports | disconnect(ptc1:p1, ptc2:p2); |
| map | Maps the port of one test component to the port of the test system interface | map(ptc1:q, system:sutPort1); |
| unmap | Unmaps two or more mapped ports | unmap(ptc1:q, system:sutPort1); |
| Test Component Operations |  |  |
| create | Creation of a normal or alive test component, the distinction between normal and alive test components is made during creation (MTC behaves as a normal test component) | Non-alive test components: var PTCType c := PTCType.create; Alive test components: var PTCType c := PTCType.create alive; |
| start | Starting test behaviour on a test component, starting behaviour does not affect the status of component variables, timers or ports | c.start(PTCBehaviour()); |
| stop | Stopping test behaviour on a test component | c.stop; |
| kill | Causes a test component to cease to exist | c.kill; |
| alive | Returns true if the test component has been created and is ready to execute or is already executingbehaviour; otherwise returns false | if (c.alive) … |
| running | Returns true as long as the test component is executing behaviour; otherwise returns false | if (c.running) … |
| done | Checks whether the function running on a test component has terminated | c.done; |
| killed | Checks whether a test component has ceased to exist | c.killed { … } |
| Test Case Operations |  |  |
| stop | Terminates the test case with the test verdict error | testcase.stop ( … ); |
| Reference Operations |  |  |
| mtc | Gets the reference to the MTC | connect(mtc:p, ptc:p); |
| system | Gets the reference to the test system interface | map(c:p, system:sutPort); |
| self | Gets the reference to the test component that executes this operation | self.stop; |

## 21.1	Connection Operations

### 21.1.0	General

The ports of a test component can be connected to other components or to the ports of the test system interface (see figure 10). In the case of connections between two test components, the connect operation shall be used. When connecting a test component to a test system interface the map operation shall be used. The connect operation directly connects one port to another with the in side connected to the out side and vice versa. The map operation on the other hand can be seen purely as a name translation defining how communications streams can be referenced.

Figure 10: Illustration of the connect and map operations

### 21.1.1	The Connect and Map operations

The connect operation is used to setup connections between test components. The map operation are used to setup connections to the SUT.

Syntactical Structure

connect "(" ComponentRef ":" Port "," ComponentRef ":" Port ")"

map "(" ComponentRef ":" Port "," ComponentRef ":" Port ")"

[ param "(" [ { ActualPar [","] }+ ] ")" ]

Semantic Description

With both the connect operation and the map operation, the ports to be connected are identified by the component references of the components to be connected and the names of the ports to be connected.

The operation mtc identifies the MTC, the operation system identifies the test system interface and the operation self identifies the test component in which self has been called (see clause 6.2.11). All these operations can be used for identifying and connecting ports.

Both the connect and map operations shall be only invoked from places specified in 21.0.. Before either operation is called, the components to be connected shall have been created and their component references shall be known together with the names of the relevant ports.

Applying a map or connect operation to ports which are already mapped or connected has no effect on the test behaviour or test configuration, i.e. test execution continues as if the operation has not been invoked.

NOTE 1:	Please note that also triMap or tciConnect respectively will not be invoked in such a case.

The map operation provides an optional parameter list for configuration purposes. This allows to pass values needed for dynamic runtime configuration. If a parameter list is present, the actual parameters shall conform to the map param clause of the port type declaration of the system port used.

Restrictions

In addition to the general static rules of TTCN-3 given in clauses 5 and 21 and shown in table 16, the following restrictions apply:

a)	For both the connect and map operations, only consistent connections are allowed.

Assuming the following:

1)	ports PORT1 and PORT2 are the ports to be connected or mapped;

2)	inlist-PORT1 defines the messages or procedures of the in-direction of PORT1;

3)	outlist-PORT1defines the messages or procedures of the out-direction of PORT1;

4)	inlist-PORT2 defines the messages or procedures of the in-direction of PORT2; and

5)	outlist-PORT2 defines the messages or procedures of the out-direction of PORT2.

b)	The connect operation is allowed if and only if:

1)	outlist-PORT1  inlist-PORT2 and outlist-PORT2  inlist-PORT1; and

2)	neither PORT1 nor PORT2 are system port references; and

3)	atleast one of outlist-PORT1 or outlist-PORT2 is not empty.

c)	The map operation is allowed if and only if:

1)	PORT1 is a component port reference and PORT2 is a system port reference; and

2)	outlist-PORT1  outlist-PORT2 and inlist-PORT2  inlist-PORT1; and

3)	at least one of outlist-PORT1 or inlist-PORT2 is not empty.

NOTE 2:	Please note that PORT1 and PORT2 can occur in any order, thus the system adapter port can be either the first or the second operand of  the map operation.

d)	In all other cases, the operations shall not be allowed.

e)	Since TTCN-3 allows dynamic configurations and addresses, not all of these consistency checks can be made statically at compile-time. All checks, which could not be made at compile-time, shall be made at runtime and shall lead to a test case error when failing.

f)	In addition, the restrictions on allowed and disallowed connections described in clause 9.1 apply.

g)	In map operations, param clauses are optional. If in a map operation a param clause is present, exactly one of the components referenced by the operation shall be the system component reference, the type of the system component shall be known in the context of the  operation either via a system clause or via a runs on clause in a testcase without system clause, the type of the system port to which the operation is applied shall include a map param declaration, and the actual parameters shall conform to the map param clause of the port type declaration of the system port used.

h)	If the type of the component referenced in a connection operation is known (either when the component reference is a variable or value returned from a function or the type is defined in the runs on, mtc or system clause of the calling function), the referenced port declaration shall be present in this component type.

Examples

EXAMPLE 1:	Simple map and connect:

// It is assumed that the ports Port1, Port2, Port3 and PCO1 are properly defined and declared

// in the corresponding port type and component type definitions

:

var MyComponentType  v_myNewPTC;

v_myNewPTC := MyComponentType.create;

:

connect(v_myNewPTC:port1, mtc:port3);

map(v_myNewPTC:port2, system:pCO1);

:

// In this example a new component of type MyComponentType is created and its reference stored

// in variable v_myNewPTC. Afterwards in the connect operation, port1 of this new component

// is connected with port3 of the MTC. By means of the map operation, port2 of the new component

// is then connected to port pCO1 of the test system interface

EXAMPLE 2:	Parameterized map:

:

var MyConfigType v_myConfig := { option := 1, lock := false};

:

map(mtc:port4, system:pCO2) param (v_myConfig);

:

// In this example by means of the map operation, port4 of the MTC is connected to the port pCO2

// of the test system interface, and additionally a parameter containing configuration options

// for the connection is passed.

EXAMPLE 3:	Port visibility:

type port P message { inout integer; }

type component C1 { port P p1; }

type component C2 { port P p1, p2; }

testcase TC runs on C1 system C1

{

var C1 v_ptc := C2.create; // valid assignment, instance of C2 is compatible with C1 type

connect (self:p1, v_ptc:p1); // valid, p1 is present in C1 type definition

disconnect (self:p1, v_ptc:p1);

connect (self:p1, v_ptc:p2); // invalid, although the real instance in v_ptc is of the

// C2 type, the variable itself is of the C1 type making the p2 port invisible to the

// connection operation
	  connect (v_ptc:p1, system:p1); // invalid, connect parameters shall not contain

// a system port reference

}

### 21.1.2	The Disconnect and Unmap operations

The disconnect and unmap operations are the opposite operations of connect and map.

Syntactical Structure

disconnect [ ( "(" ComponentRef ":" Port "," ComponentRef ":" Port ")" ) |

( "(" PortRef ")" ) |

( "(" ComponentRef ":" all port ")" ) |

( "(" all component ":" all port ")" ) ]

unmap [ ( "(" ComponentRef ":" Port "," ComponentRef ":" Port ")"

[ param "(" [ { ActualPar [","] }+ ] ")" ] )  |

( "(" PortRef ")" [ param "(" [ { ActualPar [","] }+ ] ")" ] ) |

( "(" ComponentRef ":" all port ")" ) |

( "(" all component ":" all port ")" ) ]

Semantic Description

The disconnect and unmap operations perform the disconnection (of previously connected) ports of test components and the unmapping of (previously mapped) ports of test components and ports in the test system interface.

Both, the disconnect and unmap operations can be called from any component if the relevant component references together with the names of the relevant ports are known. A disconnect or unmap operation has only an effect if the connection or mapping to be removed has been created beforehand.

To ease disconnect and unmap operations related to all connections and mappings of a component or a port, it is allowed to use disconnect and unmap operations with one argument only. This one argument specifies one side of the connections to be disconnected or unmapped. The all port keyword can be used to denote all ports of a component.

The usage of a disconnect or unmap operation without any parameters is a shorthand form for using the operation with the parameter self:all port. It disconnects or unmaps all ports of the component that calls the operation.

The all component keyword shall only be used in combination with the all port keyword, i.e. all component:all port, and shall only be used by the MTC. Furthermore, the all component:all port argument shall be used as the one and only argument of a disconnect or unmap operation and it allows to release all connections and mappings of the test configuration.

Similar to the map operation, unmap provides an optional parameter list for configuration purposes. If a parameter list is present, the actual parameters shall conform to the unmap param clause of the port type declaration of the system port used. It allows to pass values needed for dynamic runtime configuration.

Restrictions

In addition to the general static rules of TTCN-3 given in clauses 5 and 21 and shown in table 16, the following restrictions apply:

a)	In an unmap operation, a param clause shall only be present if the system port to which the param clause belongs to is explicitly referenced.

b)	In unmap operations, param clauses are optional. If in an unmap operation a param clause is present, exactly one of the components referenced by the operation shall be the system component reference, the type of the system component shall be known in the context of the  operation either via a system clause or via a runs on clause in a testcase without system clause, the type of the system port to which the operation is applied shall include an unmap param declaration and the actual parameters shall conform to the unmap param clause of the port type declaration of the system port used.

c)	If the type of the component referenced in a connection operation is known (either when the component reference is a variable or value returned from a function or the type is defined the runs on, mtc or system clause of the calling function), the referenced port declaration shall be present in this component type.

d)	The disconnect operation parameters shall not contain a system port reference.

Examples

EXAMPLE 1:	Disconnect/unmap for specific connections:

connect(myNewComponent:port1, mtc:port3);

map(myNewComponent:port2, system:pCO1);

:

disconnect(myNewComponent:port1, mtc:port3);	// disconnect previously made connection

unmap(myNewComponent:port2, system:pCO1);		// unmap previously made mapping

EXAMPLE 2:	Disconnect/unmap for a component:

disconnect(myNewComponent:port1);				// disconnects all connections of Port1, which
													// is owned by component myNewComponent.

unmap(myNewComponent:all port);					// unmaps all ports of component myNewComponent

EXAMPLE 3:	Disconnect/unmap for "self":

disconnect;										// is a shorthand form for …

disconnect(self:all port);						// which disconnects all ports of the component

// that called the operation

:

unmap;											// is a shorthand form for …

unmap(self:all port);							// which unmaps all ports of the component

// that called the operation

EXAMPLE 4:	Disconnect/unmap for "all component":

disconnect(all component:all port);				// the MTC disconnects all ports of all

// components in the test configuration.

:

unmap(all component:all port);					// the MTC unmaps all ports of all

// components in the test configuration.

## 21.2	Test case operations

### 21.2.0	General

Test case operations address the entire test case by using the keyword testcase. Currently, the test case stop operation is the only test case operation. It specifies an immediate stop of the test case behaviour with an error verdict.

### 21.2.1	Test case stop operation

The testcase stop operation defines a user defined immediate termination of a test case with the test verdict error and an (optional) associated reason for the termination. Such an immediate stop of a test case is required for cases where a user defined behaviour that does not contribute to the test outcome behaves in an unexpected manner which leads to a situation where the continuation of the test case makes no more sense.

Syntactical Structure

testcase "." stop [ "(" { ( FreeText | TemplateInstance ) [","] } ")" ]

Semantic Description

The test case stop operation causes an immediate stop of the entire test case behaviour with the verdict error. In addition, the test case stop operation provides the means to specify the reason for the immediate termination of a test case by writing one or more items to some logging device associated with the test control or the test component in which the operation is used. Items to be logged shall be identified by a comma-separated list in the argument of the test case stop operation. The argument of the test case stop operation shall follow the same restrictions as the argument of the log statement (see clause 19.11).

Restrictions

No specific restrictions in addition to the general static rules of TTCN-3 given in clause 5 and shown in table 16.

Examples

testcase.stop("Unexpected Termination");

// The test case stops the an error verdict and the string "Unexpected Termination"

// is written to some log device of the test system

## 21.3	Test Component Operations

### 21.3.0	General

Test component operations are used to create, start, stop and kill test components. They can also be used to check if test components are alive, running, done or killed.

### 21.3.1	The Create operation

The create operation is used to create test components.

Syntactical Structure

ComponentType "." create [ "(" Expression ["," Expression] ")" ] [ alive ]

Semantic Description

The MTC is the only test component, which is automatically created when a test case starts. All other test components (the PTCs) shall be created explicitly during test execution by create operations. A component is created with its full set of ports of which the input queues are empty and with its full set of constants, variables and timers. Furthermore, if a port is defined to be of the type in or inout it shall be in a listening state ready to receive traffic over the connection.

All component variables and timers are reset to their initial value (if any) and all component constants are reset to their assigned values when the component is explicitly or implicitly created.

Two types of PTCs are distinguished: a PTC that can execute a behaviour only once and a PTC that is kept alive after termination of a behaviour and can be therefore reused to execute another behaviour. The latter is created using the additional alive keyword. An alive-type PTC shall be destroyed explicitly using the kill operation (see clause 21.3.4), whereas a non-alive PTC is destroyed implicitly after its behaviour terminates. Termination of a test case, i.e. the MTC, terminates all PTCs that still exist, if any.

Since all test components and ports are implicitly destroyed at the termination of each test case, each test case shall completely create its required configuration of components and connections when it is invoked.

The create operation shall return the unique component reference of the newly created instance. The unique reference to the component will typically be stored in a variable (see clause 6.2.10.1) and can be used for connecting instances and for communication purposes such as sending and receiving.

Optionally, a name can be associated with the newly created component instance. The test system shall associate the names 'MTC' to the MTC and 'SYSTEM' to the test system interface automatically at creation. Associated component names are not required to be unique.

The component instance name is used for logging purposes (see clause 19.11) only and shall not be used to refer to the component instance (the component reference shall be used for this purpose) and has no effect on matching.

Also optionally, a host id can be associated with the newly created component instance. If a host id is provided, the create operation shall cause a test case error, if the component cannot be deployed on the specified host.

Components can be created at any point in a behaviour definition providing full flexibility with regard to dynamic configurations (i.e. any component can create any other PTC). The visibility of component references shall follow the same scope rules as that of variables and in order to reference components outside their scope of creation the component reference shall be passed as a parameter or as a field in a message.

Restrictions

In addition to the general static rules of TTCN-3 given in clauses 5 and 21 and shown in table 16, the following restrictions apply:

a)	The name given by the first Expression shall be a charstring value and when assigned it shall appear as the first argument of the create function.

b)	The host id given by the second Expression shall be a charstring value and, when assigned, it shall appear as the second argument of the create function.

Examples

// This example declares variables of type MyComponentType, which is used to store the

// references of newly created component instances of type MyComponentType which is the

// result of the create operations. An associated name is allocated to some of the created

// component instances.

:

var MyComponentType v_myNewComponent;

var MyComponentType v_myNewestComponent;

var MyComponentType v_myAliveComponent;

var MyComponentType v_myAnotherAliveComponent;

var MyComponentType v_myDeployedComponent;

:

v_myNewComponent := MyComponentType.create;

v_myNewestComponent := MyComponentType.create("Newest");

v_myAliveComponent := MyComponentType.create alive;

v_myAnotherAliveComponent := MyComponentType.create("Another Alive") alive;

v_myDeployedComponent := MyComponentType.create(-, "Host4");

### 21.3.2	The Start test component operation

The start operation is used to associate a test behaviour to a test component, which is then being executed by that test component.

Syntactical Structure

ObjectReference  "." start "(" ( FunctionInstance | AltstepInstance ) ")"

Semantic Description

Once a PTC has been created and connected, behaviour has to be bound to this PTC and the execution of its behaviour has to be started. This is done by using the start operation (as PTC creation does not start execution of the component behaviour). The reason for the distinction between create and start is to allow connection operations to be done before actually running the test component.

The start operation shall bind the required behaviour to the test component. This behaviour is defined by reference to an already defined function or altstep.

An alive-type PTC may perform several behaviours in sequential order. Starting a second behaviour on a non-alive PTC or starting a behaviour on a PTC that is still running results in a test case error. If a behaviour is started on an alive-type PTC after termination of a previous behaviour, it uses variable values, timers, ports, and the local verdict as they were left after termination of the previous behaviour. In particular, if a timer was started in the previous behaviour, the subsequent behaviour should be enabled to handle a possible timeout event. In contrast to that, all active defaults are deactivated when the behaviour of an alive-type PTC is stopped. This means no default is activated when a new behaviour is started on an alive-type PTC.

NOTE 1:	The lifetime of variables and timers is bound to the scope in which they are declared. When an alive-type component is stopped, only the component scope is left. This means only variable values and timers declared in the component type definition of an alive-type PTC can be accessed by a behaviour with a corresponding runs on-clause that is started on an alive-type PTC.

Actual inout parameters will be passed to the function by value, i.e. like in-parameters.

If the function's formal parameter list includes any out parameter the actual parameter list may omit actual out parameters using the dash symbol ("-") or be omitted in the same manner as for actual in parameters with default values (see clause 5.4.2), i.e. they can be omitted in the list notation if all following actual parameters are also omitted and their assignment can be omitted altogether in assignment notation. If a variable is given as an actual out parameter, it will remain unchanged by the started behaviour, even if the behaviour changes the formal parameter during its execution.

Possible return values of a function invoked in a start test component operation, i.e. templates denoted by return keyword or inout and out parameters, have no effect when the started test component terminates.

Restrictions

In addition to the general static rules of TTCN-3 given in clauses 5 and 21 and shown in table 16, the following restrictions apply:

- The ObjectReference shall be of component type and shall not resolve to a template.

- The function or altstep invoked in a start test component operation shall have a runs on definition referencing a component type that is compatible with the newly created component (see clause 6.3.2.7).

All formal parameters of the function or altstep invoked in a start test component operation shall be of a component data type.

NOTE 2:	As in and inout ports starts listening when the component is created, at the moment, when it starts execution there may be messages in the incoming queues of such ports already waiting to be processed.

Examples

function f_myFirstBehaviour() runs on MyComponentType { … }

function f_mySecondBehaviour() runs on MyComponentType { … }

function f_myThirdBehaviour(out integer p_p1, inout integer p_p2) runs on MyComponentType { … }

altstep  a_myFourthBehaviour() runs on MyComponentType { ... }

:

var MyComponentType v_myNewPTC;

var MyComponentType v_myAlivePTC;

var integer v_int := 0;

:

v_myNewPTC := MyComponentType.create;			// Creation of a new non-alive test component.

v_myAlivePTC := MyComponentType.create alive;	// Creation of a new alive-type test component

:

v_myNewPTC.start(f_myFirstBehaviour());		// Start of the non-alive component.

v_myNewPTC.done;							// Wait for termination

v_myNewPTC.start(f_mySecondBehaviour());	// Test case error

:

v_myAlivePTC.start(f_myFirstBehaviour());	// Start of the alive-type component

v_myAlivePTC.done;							// Wait for termination

v_myAlivePTC.start(f_mySecondBehaviour());	// Start of the next function on the same component

:

v_myAlivePTC.start(f_myThirdBehaviour(-,v_int));	// v_int will not be changed by the function

v_myAlivePTC.done;

v_myAlivePTC.start(a_myFourthBehaviour());		// Direct start of an altstep behaviour<>

### 21.3.3	The Stop test behaviour operation

The stop test behaviour operation is used to stop the execution of a test component by itself or by another test component.

Syntactical Structure

stop |

( ( ObjectReference  | mtc | self ) "." stop ) |

( all component "." stop )

Semantic Description

By using the stop test component statement a test component can stop the execution of its own currently running test behaviour or the execution of the test behaviour running on another test component. If a component does not stop its own behaviour, but the behaviour running on another test component in the test system, the component to be stopped has to be identified by using its component reference. A component can stop its own behaviour by using a simple stop execution statement (see clause 19.9) or by addressing itself in the stop operation, e.g. by using the self operation.

NOTE 1:	While the create, start, running, done and killed operations can be used for PTC(s) only, the stop operation can also be applied to the MTC.

Stopping a test component is the explicit form of terminating the execution of the currently running behaviour. A test component behaviour terminates also by completing its execution upon reaching the end of the test behaviour that is started on this component or by an explicit return statement. This termination is also called implicit stop. The implicit stop has the same effects as an explicit stop, i.e. the global verdict is updated with the local verdict of the stopped test component (see clause 24).

If the stopped test component is the MTC, resources of all existing PTCs shall be released, the PTCs shall be removed from the test system and the test case shall terminate (see clause 26.1).

Stopping a non-alive-type test component (implicitly or explicitly) shall destroy it and all resources associated with the test component shall be released.

Stopping an alive-type component shall stop the currently running behaviour only but the component continues to exist and can execute new behaviour (started on it using the start operation). Stopping an alive-type component means that all variables, timers and ports declared in the component type definition of the alive-type component keep their value, contents or state. Furthermore, the local verdict of the component keeps its value. In contrast to that, all active defaults are automatically deactivated when the alive-type component is stopped. The component shall be left in a consistent state after stopping its behaviour.

For example, if the behaviour of an alive-type component is stopped during assigning a new value to an already bound variable, the variable shall remain bound after the component is stopped (with the old or the new value). Similarly, if the component is stopped during re-starting an already running timer, the timer shall be left in the running state after termination of the behaviour.

The all keyword can be used by the MTC only in order to stop all running PTCs but the MTC itself.

NOTE 2:	A PTC can stop the test case execution by stopping the MTC.

NOTE 3:	The concrete mechanism for stopping PTCs is outside the scope of the present document.

Restrictions

In addition to the general static rules of TTCN-3 given in clauses 5 and 21 and shown in table 16, the following restrictions apply:

- The ObjectReference shall be of component type and shall not resolve to a template.

Examples

EXAMPLE 1:	Stopping another test component and a test component by itself

var MyComponentType v_myComp := MyComponentType.create;	// A new test component is created

v_myComp.start(f_compBehaviour());							// The new component is started

:

if (v_date == "1.1.2005") {

v_myComp.stop;					// The component "v_myComp" is stopped

}

:

if (v_a < v_b ) {

:

self.stop;		// The test component that is currently executing stops its own behaviour

}

:

stop			// The test component stops its own behaviour

EXAMPLE 2:	Stopping all PTCs by the MTC

all component.stop		// The MTC stops all PTCs of the test case but not itself.

### 21.3.4	The Kill test component operation

The kill test component operation is used to destroy a test component by itself or by another test component. Kill and stop on a non-alive component have the same results, while they differ for alive components: stopping an alive components stops the test behaviour only, the test component continues to exist. Killing a test component destroys the test component.

Syntactical Structure

kill |

( ( ObjectReference | mtc | self ) "." kill ) |

( all component "." kill )

Semantic Description

The kill operation applied on a test component stops the execution of the currently running behaviour - if any - of that component and frees all resources associated to it (including all port connections of the killed component) and removes the component from the test system. The kill operation can be applied on the current test component itself by a simple kill statement or by addressing itself using the self operation in conjunction with the kill operation. The kill operation can also be applied to another test component. In this case the component to be killed shall be addressed using its component reference. If the kill operation is applied on the MTC, e.g. mtc.kill, it terminates the test case.

The all keyword can be used by the MTC only in order to stop and kill all running PTCs but the MTC itself.

Restrictions

In addition to the general static rules of TTCN-3 given in clause 5 and 21 and shown in table 16, the following restrictions apply:

- The ObjectReference shall be of component type and shall not resolve to a template.

Examples

EXAMPLE 1:	Killing another test component and a test component by itself

var PTCType v_myAliveComp := PTCType.create alive;	// Create an alive-type test component

v_myAliveComp.start(f_myFirstBehaviour());			// The new component is started

v_myAliveComp.done;									// Wait for termination

v_myAliveComp.start(f_mySecondBehavior());			// Start the component a 2nd time

v_myAliveComp.done;									// Wait for termination

v_myAliveComp.kill;									// Free its resources

EXAMPLE 2:	Killing all PTCs by the MTC

all component.kill;		// The MTC stops all (alive-type and normal) PTCs of the test case first
	// and frees their resources.

### 21.3.5	The Alive operation

The alive operation is a Boolean operation that checks whether a test component has been created and is ready to execute or is executing already a behaviour.

Syntactical Structure

( ObjectReference |

any component |

all component |

any from ComponentArrayRef ) "." alive

[ "->" @index value ValueRef ]

Semantic Description

Applied on a normal parallel test component, the alive operation returns true if the component is inactive or running a behaviour and false otherwise. Applied on an alive-type parallel test component, the operation returns true if the component is inactive, running or stopped. It returns false if the component has been killed. Applied on the mtc the operation returns true.

The alive operation can be used similar to the running operation (see clause 21.3.6). In particular, in combination with the all keyword it returns true if all (alive-type or normal) PTCs are alive.

The alive operation used in combination with the any keyword returns true if at least one PTC is alive.

When the any from component array notation is used, the components from the referenced array are iterated over and individually checked for being inactive or running a function from innermost to outermost dimension from lowest to highest index for each dimension. The first component to be found being inactive or running a behaviour causes the alive operation to return the true value. The index of the first component found alive can optionally be assigned to an integer variable for  single-dimensional component arrays or to an integer array or record of integer variable for multi-dimensional component arrays.

Restrictions

In addition to the general static rules of TTCN-3 given in clauses 5 and 21 and shown in table 16, the following restrictions apply:

- The ObjectReference shall be of component type and shall not resolve to a template.

- The ComponentArrayRef shall be a reference to a completely initialized component array.

- The index redirection shall only be used when the operation is used on an any from component array construct.

- If the index redirection is used for single-dimensional component arrays, the type of the integer variable shall allow storing the highest index of the respective array.

- If the index redirection is used for multi-dimensional component arrays, the size of the integer array or record of integer type shall exactly be the same as the dimension of the respective array, and its type shall allow storing the highest index (from all dimensions) of the array.

- If a variable referenced in the @index clause is a lazy or fuzzy variable, the expression assigned to this variable is equal to the result produced by the alive operation, i.e. later evaluation of the lazy or fuzzy variable does not lead to repeated invocation of the alive operation.

Examples

pTC1.done;								// Waits for termination of the component

if (pTC1.alive) {						// If the component is still alive …

pTC1.start(f_anotherFunction());	// … execute another function on it.

### 21.3.6	The Running operation

The running operation is a Boolean operation that checks whether a test component is executing already a behaviour.

Syntactical Structure

( ObjectReference |

any component |

all component |

any from ComponentArrayRef ) "." running

[ "->" @index value ValueRef]

Semantic Description

The running operation allows behaviour executing on a test component to ascertain whether behaviour running on a different test component has completed. The running operation returns true for the mtc and PTCs that have been started but not yet terminated or stopped. It returns false otherwise. The running operation is considered to be a boolean expression and, thus, returns a boolean value to indicate whether the specified test component (or all test components) has terminated. In contrast to the done operation, the running operation can be used freely in boolean expressions.

When the all keyword is used with the running operation, it will return true if all PTCs started but not stopped explicitly by another component are executing their behaviour. Otherwise it returns false.

NOTE:	The difference between the running operation applied to a single ptc and the usage of the all keyword leads to the situation that ptc.running is false if the ptc has never been started but all component.running is true at the same time as it considers only those components that ever have been started.

When the any keyword is used with the running operation, it will return true if at least one PTC is executing its behaviour. Otherwise it returns false.

When the any from component array notation is used, the components from the referenced array are iterated over and individually checked for executing currently from innermost to outermost dimension from lowest to highest index for each dimension. The first component to be found executing causes the running operation to succeed. The index of the matched component can optionally be assigned to an integer variable for single-dimensional arrays or to an integer array or record of integer variable for multi-dimensional component arrays.

Restrictions

In addition to the general static rules of TTCN-3 given in clauses 5 and 21 and shown in table 16, the following restrictions apply:

- The ObjectReference shall be of component type and shall not resolve to a template.

- The ComponentArrayRef shall be a reference to a completely initialized component array.

- The index redirection shall only be used when the operation is used on an any from component array construct.

- If the index redirection is used for single-dimensional component arrays, the type of the integer variable shall allow storing the highest index of the respective array.

- If the index redirection is used for multi-dimensional component arrays, the size of the integer array or record of integer type shall exactly be the same as the dimension of the respective array, and its type shall allow storing the highest index (from all dimensions) of the array.

- If a variable referenced in the @index clause is a lazy or fuzzy variable, the expression assigned to this variable is equal to the result produced by the running operation. Later evaluation of the lazy or fuzzy variable does not lead to repeated invocation of the running operation.

Examples

if (pTC1.running) 						// usage of running in an if statement

{

// do something!

}

while (all component.running != true) {	// usage of running in a loop condition

f_mySpecialFunction()

}

### 21.3.7	The Done operation

The done operation allows behaviour executing on a test component to ascertain whether the behaviour running on a different test component has completed. In addition, the done operation allows to retrieve the final local verdict of completed test components, i.e., the value of the local verdict at the time of test component completion.

Syntactical Structure

[ @nodefault ] ( ObjectReference |

any component |

all component |

any from ComponentArrayRef ) "." done
[ "->" [ value ValueRef] [ @index value ValueRef] ]

Semantic Description

The done operation shall be used in the same manner as a receiving operation or a timeout operation. This means it shall not be used in a boolean expression, but it can be used to determine an alternative in an alt statement or as stand-alone statement in a behaviour description. In the latter case a done operation is considered to be a shorthand for an alt statement with the done operation as the only alternative. If the @nodefault modifier is placed before a stand-alone done operation, the implicit alt statement also contains the @nodefault modifier.

When the done operation is applied to a PTC, it matches only if the behaviour of that PTC has been stopped (implicitly or explicitly) or the PTC has been killed. Otherwise, the match is unsuccessful.

NOTE 1:	The execution of a done operation does not change the state of the test component. Consecutive done operations applied to the same test component will give the same result as long as the test component does not change its state (see clause F.1.2).

When the done operation is applied to a PTC and matches, the final local verdict of the PTC can be retrieved and stored in variable of the type verdicttype. This is denoted by the symbol '->' the keyword value followed by the name of the variable into which the verdict is stored.

When the all keyword is used with the done operation, it matches if no one PTC is executing its behaviour. It also matches if no PTC has been created.

NOTE 2:	The difference between the done operation applied to a single ptc and the usage of the all keyword leads to the situation that ptc.done does not match if the ptc has never been started but all component.done matches at the same time as it considers only those components that ever have been started.

When the any keyword is used with the done operation, it matches if at least the behaviour of one PTC has been stopped or killed. Otherwise, the match is unsuccessful.

NOTE 3:	Stopping the behaviour of a non-alive component also results in removing that component from the test system, while stopping an alive-type component leaves the component alive in the test system. In both cases the done operation matches.

When the any from component array notation is used, the components from the referenced array are iterated over and individually checked for being stopped or killed from innermost to outermost dimension from lowest to highest index for each dimension. The first component to be found stopped or killed causes done operation to succeed. The index of the matched component can optionally be assigned to an integer variable for single-dimensional arrays or to an integer array or record of integer variable for multi-dimensional component arrays.

Restrictions

In addition to the general static rules of TTCN-3 given in clauses 5 and 21 and shown in table 16, the following restrictions apply:

- The done operation can be used for PTCs only.

- The ObjectReference followed by the done keyword, i.e. used for identifying a specific PTC, shall be of a component type and shall not resolve to a template.

- The ComponentArrayRef shall be a reference to a completely initialized component array.

- The variable used in the (optional) value clause for storing the final local verdict of a PTC shall be of the type verdicttype.

- The (optional) value clause for storing the final local verdict of a PTC shall not be used in combination with all component or any component.

- The index redirection shall only be used when the operation is used on an any from component array construct.

- If the index redirection is used for single-dimensional component arrays, the type of the integer variable shall allow storing the highest index of the respective array.

- If the index redirection is used for multi-dimensional component arrays, the size of the integer array or record of integer type shall exactly be the same as the dimension of the respective array, and its type shall allow storing the highest index (from all dimensions) of the array.

- If a variable referenced in the @index clause is a lazy or fuzzy variable, the expression assigned to this variable is equal to the result produced by the done operation. Later evaluation of the lazy or fuzzy variable does not lead to repeated invocation of the done operation.

- The @nodefault modifier is allowed only in stand-alone done statements.

Examples

// Use of done in alternatives

alt {

[]	myPTC.done {

setverdict(pass)

}

[]	any port.receive {

repeat

}

}

var MyComp v_c := MyComp.create alive;

v_c.start(f_myPTCBehaviour());

:

v_c.done;

// matches as soon as the function f_myPTCBehaviour (or function/altstep called by it) stops

v_c.done;

// matches again, even if the component has not been started again

if(v_c.running) {v_c.done}

// in case that some other component has started v_c in the meantime

// done here matches the end of the next behaviour only, not the previous one

// the following done as stand-alone statement:

@nodefault all component.done;

// has the following meaning:

alt @nodefault {

[]	all component.done {}

}

// and thus, blocks the execution until all parallel test components have terminated while

// ignoring all activated default alternatives

// Retrieving and using the final local verdict of a completed PTC

var MyComp v_myPTC := MyPTC.create alive;

var verdicttype v_myPTCverdict := none;

v_myPTC.start(f_myPTCBehaviour());

:

alt {

[]	v_myPTC.done -> value v_myPTCverdict {

if (v_myPTCverdict == fail) {

setverdict(fail);

stop;

}

else {

setverdict (pass);

}

}

[]	any port.receive {

repeat

}

}

### 21.3.8	The Killed operation

The killed operation allows to ascertain whether a different test component is alive or has been removed from the test system. In addition, the killed operation allows to retrieve the final local verdict of killed test components, i.e., the value of the local verdict at the time when the test component was killed.

Syntactical Structure

[ @nodefault ] ( ObjectReference |

any component |

all component |

any from ComponentArrayRef ) "." killed

[ "->" [ value ValueRef] [ @index value ValueRef] ]

Semantic Description

The killed operation shall be used in the same manner as receiving operations. This means it shall not be used in boolean expressions, but it can be used to determine an alternative in an alt statement or as a stand-alone statement in a behaviour description. In the latter case a killed operation is considered to be a shorthand for an alt statement with the killed operation as the only alternative. If the @nodefault modifier is placed before a stand-alone killed operation, the implicit alt statement also contains the @nodefault modifier.

NOTE 1:	When checking normal test components a killed operation matches if it stopped (implicitly or explicitly) the execution of its behaviour or has been killed explicitly, i.e. the operation is equivalent to the done operation (see clause 21.3.7). When checking alive-type test components, however, the killed operation matches only if the component has been killed using the kill operation. Otherwise the killed operation is unsuccessful.

NOTE 2:	The execution of a killed operation does not change the state of the test component. Consecutive killed operations applied to the same test component will give the same result as long as the test component does not change its state (see clause F.1.2).

When the all keyword is used with the killed operation, it matches if all PTCs of the test case have ceased to exist. It also matches if no PTC has been created.

When the killed operation is applied to a PTC and matches, the final local verdict of that PTC can be retrieved and stored in a variable of the type verdicttype. This is denoted by the symbol '->' the keyword value followed by the name of the variable into which the verdict is stored.

When the any keyword is used with the killed operation, it matches if at least one PTC ceased to exist. Otherwise, the match is unsuccessful.

When the any from component array notation is used, the components from the referenced array are iterated over and individually checked for being killed from innermost to outermost dimension from lowest to highest index for each dimension. The first component to be found killed causes the killed operation to succeed. The index of the matched component can optionally be assigned to an integer variable for single-dimensional component arrays or to an integer array or record of integer variable for multi-dimensional component arrays.

Restrictions

In addition to the general static rules of TTCN-3 given in clauses 5 and 21 and shown in table 16, the following restrictions apply:

- The killed operation can be used for PTCs only.

- The ObjectReference  followed by the killed keyword, i.e. used for identifying a specific PTC, shall be of a component type and shall not resolve to a template.

- The ComponentArrayRef shall be a reference to a completely initialized component array.

- The variable used in the (optional) value clause for storing the final local verdict of a PTC shall be of  the type verdicttype.

- The (optional) value clause for storing the final local verdict of a PTC shall not be used in combination with all component or any component.

- The index redirection shall only be used when the operation is used on an any from component array construct.

- If the index redirection is used for single-dimensional component arrays, the type of the integer variable shall allow storing the highest index of the respective array.

- If the index redirection is used for multi-dimensional component arrays, the size of the integer array or record of integer type shall exactly be the same as the dimension of the respective array, and its type shall allow storing the highest index (from all dimensions) of the array.

- If a variable referenced in the @index clause is a lazy or fuzzy variable, the expression assigned to this variable is equal to the result produced by the killed operation i.e. later evaluation of the lazy or fuzzy variable does not lead to repeated invocation of the killed operation.

- The @nodefault modifier is allowed only in stand-alone killed statements.

Examples

var MyPTCType v_ptc := MyPTCType.create alive;	// create an alive-type test component

timer t_T:= 10.0;								// create a timer

t_T.start;										// start the timer

v_ptc.start(f_myTestBehavior());				// start executing a function on the PTC

alt {

[] v_ptc.killed {								// if the PTC was killed during execution …

t_T.stop;									// … stop the timer and …

setverdict(inconc);							// … set the verdict to 'inconclusive'

}

[] v_ptc.done {									// if the PTC terminated regularly …

t_T.stop;									// … stop the timer and …

v_ptc.start(f_anotherFunction());			// … start another function on the PTC

}

[] t_T.timeout {								// if the timeout occurs before the PTC stopped

v_ptc.kill;									// … kill the PTC and …

setverdict(fail);							// … set the verdict to 'fail'

}

}

// Retrieving and using the final local verdict of a killed PTC

var MyComp v_myPTC := MyPTC.create alive;

var verdicttype v_myPTCverdict := none;

v_myPTC.start(f_myPTCBehaviour());

:

alt {

[]	v_myPTC.done {							// expected termination

setverdict (pass);

}

}

[]	v_myPTC.killed -> value v_myPTCverdict {

if (v_MyPTCverdict == none) {		// v_myPTC killed before verdict assignment

setverdict(fail);

stop;

}

else {

setverdict (inconc);			// further analysis is needed

stop;

}

}

[]	any port.receive {

repeat

}

}

### 21.3.9	Summary of the use of any and all with components

The keywords any and all may be used with configuration operations as indicated in table 21.

Table 21: Any and All with components

| Operation | Allowed |  | Example | Comment |
| --- | --- | --- | --- | --- |
|  | any (see note) | all (see note) |  |  |
| create |  |  |  |  |
| start |  |  |  |  |
| running | Yes but from MTC only | Yes but from MTC only | any component.running; all component.running; | Is there any PTC performing test behaviour? Are all PTCs performing test behaviour? |
| alive | Yes but from MTC only | Yes but from MTC only | any component.alive; all component.alive; | Is there any alive PTC? Are all PTCs alive? |
| done | Yes but from MTC only | Yes but from MTC only | any component.done; all component.done; | Is there any PTC that completed execution? Did all PTCs complete their execution? |
| killed | Yes but from MTC only | Yes but from MTC only | any component.killed; all component.killed; | Is there any PTC that ceased to exist? Did all PTCs cease to exist? |
| stop |  | Yes but from MTC only | all component.stop; | Stop the behaviour on all PTCs. |
| kill |  | Yes but from MTC only | all component.kill; | Kill all PTCs, i.e. they cease to exist. |
| NOTE: any and all refer to PTCs only, i.e. the MTC is not considered. |  |  |  |  |

### 21.3.10	The Call test component behaviour operation

The call operation is used start a test behaviour on a test component and wait until that behaviour has terminated.

Syntactical Structure

ObjectReference "." call "(" ( FunctionInstance | AltstepInstance )

["," SimpleExpression ] ")"

[ "->" [value Ref] [verdict Ref] ]

[ catch "(" timeout ")" StatementBlock ]

[ catch "(" stop ")" StatementBlock ]

Semantic Description

Similar to the start operation on test components which is not blocking, the blocking call operation implicitly uses a start operation, but waits until either the started behaviour has terminated or some timeout has occurred.

A timeout duration in seconds can be given explicitly in the form of a SimpleExpression as an additional parameter to the call operation. If no timeout duration is given, an infinite timeout duration is used.

The actions taken by the call operation are dependent on whether the execution of the started behaviour is complete or incomplete. Complete execution occurs when the started function is terminated by executing a return statement or if it reaches the end of the function body. If the started behaviour is terminated for any other reason, the execution is incomplete.

If the incomplete execution occurs because the called component was stopped or killed and a catch stop clause is added to the call operation, the StatementBlock of that clause is executed before the call operation terminates.

If the started behaviour does not terminate in the given timeout duration and a catch timout clause is added to the call operation, the called component is implicitly stopped and the StatementBlock of the catch timeout clause is executed before the call operation terminates.

In all other cases when the execution is incomplete, the call operation ends with a test case error.

After complete execution of the started behaviour, the out and inout actual parameters given to the actual parameter list of the called function or altstep instance will be updated in the same manner as if it was a normal function/altstep invocation.

Additionally, a redirect clause can be added to the operation which allows assignment of the return result (in case that the called function has a return clause) to a variable via the value clause and also the assignment of the termination verdict of the called component via the verdict clause.

In all cases of incomplete execution, the variables referenced in the value and verdict clause or in out and inout actual parameters will stay unchanged and no assignment will be made.

If the called component is not created alive and has already been started or called once or if it has been killed, additional call operations are not allowed.

Restrictions

In addition to the general static rules of TTCN-3 given in clauses 5 and 21 and shown in table 16, the following restrictions apply:

- The ObjectReference shall be of a component type.

- The function or altstep invoked in a call test component operation shall have a runs on definition referencing a component type to which the called component is compatible (see clause 6.3.2.7).

- All formal parameters of the function or altstep invoked in a call test component operation shall be of a component data type.

- The return value of the function  invoked from a call test component operation shall be of a component data type.

- The optional SimpleExpression representing the timer value shall be of a float type.

- The optional catch timeout clause may be present only if the timeout value has been defined.

- The variable in the value clause shall be compatible with the return value of the invoked function.

- The variable in the verdict clause shall be of type verdicttype.

Examples

function f_myFirstBehaviour() runs on MyComponentType { … }

function f_mySecondBehaviour() runs on MyComponentType { … }

function f_myThirdBehaviour(out integer p_p1, inout integer p_p2)

runs on MyComponentType

return integer { … }

altstep  a_myFourthBehaviour() runs on MyComponentType { ... }

:

var MyComponentType v_myNewPTC;

var MyComponentType v_myAlivePTC;

var integer v_out, v_inout := 0, v_result;

:

v_myNewPTC := MyComponentType.create;			// Creation of a new non-alive test component.

v_myAlivePTC := MyComponentType.create alive;	// Creation of a new alive-type test component

:

v_myNewPTC.call(f_myFirstBehaviour());			// Call to the non-alive component.

v_myNewPTC.call(f_mySecondBehaviour());			// Test case error

:

v_myAlivePTC.call(f_myFirstBehaviour());	// Call to the alive-type component

v_myAlivePTC.call(f_mySecondBehaviour());	// Another call to the same component

:

v_myAlivePTC.call(f_myThirdBehaviour(v_out,v_inout)) // v_out/v_inout can be changed

- -> value v_result verdict v_verdict; // v_result/v_verdict are assigned on successful
                                     // termination

v_myAlivePTC.call(a_myFourthBehaviour());		// Direct call of an altstep behaviour

# 22	Communication operations

## 22.0	General

TTCN-3 supports message-based and procedure-based unicast, multicast and broadcast communication. Furthermore, TTCN-3 allows to examine the top element of incoming port queues and to control the access to ports by means of controlling operations. The communication operations and restrictions on their usage are summarized in table 22.

Table 22: Overview of TTCN-3 communication operations

| Communication operations |  |  |  |
| --- | --- | --- | --- |
| Communication operation | Keyword | Can be used at message-based ports | Can be used at procedure-based ports |
| Message-based communication |  |  |  |
| Send message | send | Yes |  |
| Receive message | receive | Yes |  |
| Trigger on message | trigger | Yes |  |
| Procedure-based communication |  |  |  |
| Invoke procedure call | call |  | Yes |
| Accept procedure call from remote entity | getcall |  | Yes |
| Reply to procedure call from remote entity | reply |  | Yes |
| Raise exception (to an accepted call) | raise |  | Yes |
| Handle response from a previous call | getreply |  | Yes |
| Catch exception (from called entity) | catch |  | Yes |
| Examine top element of incoming port queues |  |  |  |
| Check msg/call/exception/reply received | check | Yes | Yes |
| Controlling operations |  |  |  |
| Clear port queue | clear | Yes | Yes |
| Clear queue and enable sending and receiving at a port | start | Yes | Yes |
| Disable sending and disallow receiving operations to match at a port | stop | Yes | Yes |
| Disable sending and disallow receiving operations to match new messages/calls | halt | Yes | Yes |
| Check the state of a port | checkstate | Yes | Yes |

## 22.1	The communication mechanisms

### 22.1.0	General

This clause explains the principles of TTCN-3 communication for message-based communication (see clause 22.1.1), for procedure-based communication (see clause 22.1.2), for unicast, multicast, and broadcast communication (see clause 22.1.3), as well as the general format of sending and receiving operations (see clause 22.1.4).

### 22.1.1	Principles of message-based communication

Message-based communication is communication based on an asynchronous message exchange. Message-based communication is non-blocking on the send operation, as illustrated in figure 11, where processing in the SENDER continues immediately after the send operation occurs. The RECEIVER is blocked on the receive operation until it processes the received message.

In addition to the receive operation, TTCN-3 provides a trigger operation that filters messages with certain matching criteria from a stream of received messages on a given incoming port. Messages at the top of the queue that do not fulfil the matching criteria are removed from the port without any further action.

Figure 11: Illustration of the asynchronous send, receive and trigger

### 22.1.2	Principles of procedure-based communication

The principle of procedure-based communication is to call procedures in remote entities. TTCN-3 supports blocking and non-blocking procedure-based communication. Blocking procedure-based communication is blocking on the calling and the called side, whereas non-blocking procedure-based communication is only blocking on the called side. Signatures of procedures that are used for non-blocking procedure-based communication shall be specified according to the rules in clause 13.

The communication scheme of blocking procedure-based communication is shown in figure 12. The CALLER calls a remote procedure in the CALLEE by using the call operation. The CALLEE accepts the call by means of a getcall operation and reacts by either using a reply operation to answer the call or by raising (raise operation) an exception. The CALLER handles the reply or exception by using getreply or catch operations. In figure 12, the blocking of CALLER and CALLEE is indicated by means of dashed lines.

Figure 12: Illustration of blocking procedure-based communication

The communication scheme of non-blocking procedure-based communication is shown in figure 13. The CALLER calls a remote procedure in the CALLEE by using the call operation and continues its execution, i.e. does not wait for a reply or exception. The CALLEE accepts the call by means of a getcall operation and executes the requested procedure. If the execution is not successful, the CALLEE may raise an exception to inform the CALLER. The CALLER may handle the exception by using a catch operation in an alt statement. In figure 13, the blocking of the CALLEE until the end of the call handling and possible raise of an exception is indicated by means of a dashed line.

Figure 13: Illustration of non-blocking procedure-based communication

### 22.1.3	Principles of unicast, multicast and broadcast communication

TTCN-3 supports unicast, multicast and broadcast communication:

Unicast communication means one sender to one receiver.

Multicast communication is from one sender to a list of receivers.

Broadcast communication is from one sender to all receivers (being connected or mapped to the sender).

The terms unicast, multicast and broadcast communication are related to port communication. This means, it is only possible to address one, several or all test components that are connected to the specified port. Unicast, multicast and broadcast can also be used for mapped ports. In this case, one, several or all entities within the SUT can be reached via the specified mapped port.

### 22.1.4	General format of communication operations

#### 22.1.4.0	General

Operations such as send and call are used for the exchange of information among test components and between an SUT and test components. For explaining the general format of these operations, they can be structured into two groups:

a)	a test component sends a message (send operation), calls a procedure (call operation), or replies to an accepted call (reply operation) or raises an exception (raise operation). These actions are collectively referred to as sending operations;

b)	a component receives a message (receive operation), awaits a message (trigger operation),accepts a procedure call (getcall operation), receives a reply for a previously called procedure (getreply operation) or catches an exception (catch operation). These actions are collectively referred to as receiving operations.

#### 22.1.4.1	General format of the sending operations

Sending operations consist of a send part and, in the case of a blocking procedure-based call operation, a response and exception handling part.

The send part:

specifies the port at which the specified operation shall take place;

defines the message or procedure call to be transmitted;

gives an (optional) address part that uniquely identifies one or more communication partners to which a message, call, reply or exception shall be sent.

The port name, operation name and value shall be present in all sending operations. The address part (denoted by the to keyword) is optional and need only be specified in cases of one-to-many connections where:

unicast communication is used and one receiving entity shall be explicitly identified;

multicast communication is used and a set of receiving entities has to be explicitly identified;

broadcast communication is used and all entities connected to the specified port have to be addressed.

EXAMPLE 1:

| Send part |  |  | (Optional) response and exception |
| --- | --- | --- | --- |
| Port and operation | Value part | (Optional) address part | handling part |
| myP1.send | (v_myVariable + v_yourVariable - 2) | to v_myPartner; |  |

Response and exception handling is only needed in cases of procedure-based communication. The response and exception handling part of the call operation is optional and is required for cases where the called procedure returns a value or has out or inout parameters whose values are needed within the calling component and for cases where the called procedure may raise exceptions which need to be handled by the calling component.

The response and exception handling part of the call operation makes use of getreply and catch operations to provide the required functionality.

EXAMPLE 2:

| Send part |  |  | (Optional) response and exception handling part |
| --- | --- | --- | --- |
| Port and operation | Value part | (Optional) address part |  |
| myP1.call | (MyProc:{s_myVar1}) |  | { [] myP1.getreply(MyProc:{s_myVar2}) {} [] myP1.catch(MyProc, ExceptionOne) {} } |

#### 22.1.4.2	General format of the receiving operations

A receiving operation consists of a receive part and an (optional) assignment part.

The receive part:

a)	specifies the port at which the operation shall take place;

b)	defines a matching part which specifies the acceptable input which will match the statement;

c)	gives an (optional) address expression that uniquely identifies the communication partner (in case of one-to-many connections).

The port name, operation name and value part of all receiving operations shall be present. The identification of the communication partner (denoted by the from keyword) is optional and need only be specified in cases of one-to-many connections where the receiving entity needs to be explicitly identified.

The assignment part in a receiving operation is optional. For message-based ports it is used when it is required to store received messages. In the case of procedure-based ports it is used for storing the in and inout parameters of an accepted call, for storing the return value or for storing exceptions. For the message or parameter value assignment part strong typing is not required, e.g. the variable used for storing a message shall be type-compatible to the type of the incoming message.

In addition, the assignment part may also be used to assign the sender address of a message, exception, reply or call to a variable. This is useful for one-to-many connections where, for example, the same message or call can be received from different components, but the message, reply or exception shall be sent back to the original sending component.

For receiving operations using the any port from a port array construction (see clause 22.2.2), the assignment part may also be used to store the indices that identify the specific port instance where the receiving operation matched.

If a receiving operation is used as a stand-alone statement, the @nodefault modifier can be placed before it to indicate that the implicit alt statement containing the operation as an alternative shall have the @nodefault modifier.

EXAMPLE:

| Receive part |  |  |  | (Optional) assignment part |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Port and operation | Matching part | (Optional) address expression |  | (Optional) value assignment | (Optional) parameter value assignment | (Optional) sender value assignment |
| myP1.getreply | (AProc:{?} value 5) |  | -> |  | param (v_v1) | sender v_aPeer |

| Receive part |  |  |  | (Optional) assignment part |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Port and operation | Matching part | (Optional) address expression |  | (Optional) value assignment | (Optional) parameter value assignment | (Optional) sender value assignment |
| myP2.receive | (mw_myTemplate(5,7)) | from v_aPeer | -> | value v_myVar |  |  |

| Receive part |  |  |  | (Optional) assignment part |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Port and operation | Matching part | (Optional) address expression |  | (Optional) value assignment | (Optional) parameter value assignment | (Optional) sender value assignment | (Optional) port index assignment |
| any from p.receive | (mw_myTemplate(5,7)) |  | -> |  |  |  | @index value v_i |

## 22.2	Message-based communication

### 22.2.0	General

The operations for message-based communication via asynchronous ports are summarized in table 23.

Table 23: Overview of TTCN-3 message-based communication

| Communication operation | Keyword |
| --- | --- |
| Send message | send |
| Receive message | receive |
| Trigger on message | trigger |
| Check message received | check |

### 22.2.1	The Send operation

The send operation is used to place a message on an outgoing message port.

Syntactical Structure

ObjectReference "." send "(" TemplateInstance ")"

[ to Address ]

NOTE:	Address may be an AddressRef, a list of AddressRef-s or "all component".

Semantic Description

The send operation places a message on an outgoing message port. The message may be specified by referencing a defined template or can be defined as an in-line template.

Sending unicast, multicast or broadcast

Unicast, multicast and broadcast communication can be determined by the optional to clause in the send operation. A to clause can be omitted in case of a one-to-one connection where unicast communication is used and the message receiver is uniquely determined by the test system structure.

Unicast communication is specified, if the to clause addresses one communication partner only. Multicast communication is used, if the to clause includes a list of communication partners. Broadcast is defined by using the to clause with all component keyword.

Restrictions

In addition to the general static rules of TTCN-3 given in clause 5 and shown in table 16, the following restrictions apply:

a)	The TemplateInstance (and all parts of it) shall have a specific value i.e. the use of matching mechanisms such as AnyValue is not allowed.

b)	When defining the message in-line, the optional type part shall be used if there is ambiguity of the type of the message being sent.

c)	The send operation shall only be used on message-based ports and the type of the template to be sent shall be in the list of outgoing types of the port type definition.

d)	A to clause shall be present in case of one-to-many connections.

e)	All AddressRef items in the to clause shall be of type address, component or of the address type bound to the port type (see clause 6.2.9) of the port instance referenced in the send operation. No AddressRef in the to clause shall contain the special value null at the time of the operation.

f)	Applying a send operation to an unmapped or disconnected port shall cause a test case error.

g)	The ObjectReference shall be of a port type.

h)	TemplateInstance shall be of a data type.

Examples

EXAMPLE 1:	Simple send (receiver is determined from the test configuration)

myPort.send(m_myTemplate(5,v_myVar));	// Sends the template m_myTemplate with the actual

// parameters 5 and v_myVar via myPort.

myPort.send(5);						// Sends the integer value 5 (which is an in-line template)

EXAMPLE 2:	Sending with explicit to clause

myPort.send(charstring:"My string") to v_myPartner;

// Sends the string "My string" to a component with a

// component reference stored in variable v_myPartner

myPCO.send(v_myVariable + v_yourVariable - 2) to v_myPartner;

// Sends the result of the arithmetic expression to v_myPartner.

myPCO2.send(m_myTemplate) to (v_myPeerOne, v_myPeerTwo);

// Specifies a multicast communication, where the value of

// m_myTemplate is sent to the two component references stored

// in the variables v_myPeerOne and v_myPeerTwo.

myPCO3.send(m_myTemplate) to all component;

// Broadcast communication: the value of m_mytemplate is sent to

// all components which can be addressed via this port. If

// myPCO3 is a mapped port, the components may reside inside

// the SUT.

### 22.2.2	The Receive operation

The receive operation is used to receive a message from an incoming message port queue.

Syntactical Structure

[ @nodefault ] ( ObjectReference | any port | any from PortArrayRef ) "." receive

[ "(" TemplateInstance ")" ]

[ from Address ]

[ "->" [ value ( ValueRef|

( "(" { ValueRef[ ":=" [ @decoded [ "(" Expression ")" ] ]

FieldOrTypeReference ][","] } ")" )

) ]

[ sender ValueRef]

[ @index value ValueRef] ]

NOTE 1:	Address may be an AddressRef, a list of AddressRef-s or "any component".

Semantic Description

The receive operation is used to receive a message from an incoming message port queue. The message may be specified by referencing a defined template or can be defined as an in-line template.

The receive operation removes the top message from the associated incoming port queue if, and only if, that top message satisfies all the matching criteria associated with the receive operation.

If the match is not successful, the top message shall not be removed from the port queue i.e. if the receive operation is used as an alternative of an alt statement and it is not successful, the execution of alt statement shall continue with its next alternative.

Matching criteria

The matching criteria are related to the type and value of the message to be received. The type and value of the message to be received are determined by the argument of the receive operation, i.e. may either be derived from the defined template or be specified in-line. An optional type field in the matching criteria to the receive operation shall be used to avoid any ambiguity of the type of the value being received.

NOTE 2:	Encoding attributes also participate in matching in an implicit way, by preventing the decoder to produce an abstract value from the received message encoded in a different way than specified by the attributes.

Receiving from a specific sender

In the case of one-to-many connections the receive operation may be restricted to a certain communication partner. This restriction shall be denoted using the from keyword followed by a specification of an address or component reference, a list of address or component references or any component.

NOTE 3:	The one-to-one connection is considered to be a simple case of the one-to-many connections and allows the usage of the from-clause.

Storing the received message and parts of the received message

If the match is successful, the value is removed from the port queue and/or parts of this value can be stored in variables or formal parameters. This is denoted by the symbol '->' and the keyword value.

When the keyword value is followed by a name of a variable or formal parameter, the whole received message shall be stored in the variable or formal parameter. The variable or formal parameter shall be type compatible with the received message.

When the keyword value is followed by a list enframed by a pair of parentheses, the whole received message and/or one or more parts of it can be stored. For each list element that consists only of a variable or formal parameter name the whole message shall be stored in that variable or formal parameter. The type of the variable or formal parameter shall be compatible with the type of the message. Each assignment notation member of the list allows storing the value of the field or element of the received message, which is referenced on the right hand side of the assignment notation (:=), in the variable or formal parameter on the left hand side. The variable or formal parameter shall be type compatible with the type of the referenced field or element.

When assigning individual fields of a message, encoded payload fields can be decoded prior to assignment using the @decoded modifier. In this case, the referenced field on the right hand side of the assignment shall be one of the bitstring, hexstring, octetstring, charstring or universal charstring types. It shall be decoded into a value of the same type as the variable on the left hand side of the assignment. Failure of this decoding shall cause a test case error. In case the referenced field is of the universal charstring type, the @decoded clause can contain an optional parameter defining the encoding format. The parameter shall be of the charstring type and it shall contain one of the strings allowed for the decvalue_unichar function (specified in clause C.5.4). Any other value shall cause an error. In case the referenced field is not a universal charstring, the optional parameter shall not be present.

NOTE 4:	The model of the behaviour of this implicit decoding is defined in clause B.1.2.9.

NOTE 5:	The @decoded clause is typically used together with the decmatch matching mechanism in the matching part of the receive statement. Since the decoding procedures for assignment and matching are virtually the same, TTCN-3 tools can be optimized in such a way that only one call to the decoder is made when the receiving statement contains both decmatch matching mechanism and @decoded assignment for the same payload field.

Storing the sender

It is also possible to retrieve and store the component reference or address of the sender of a message. This is denoted by the keyword sender.

When the message is received on a connected port, only the component reference is stored in the following the sender keyword, but the test system shall internally store the component name too, if any (to be used in logging).

Receive any message

A receive operation with no argument list for the type and value matching criteria of the message to be received shall remove the message on the top of the incoming port queue (if any) if all other matching criteria are fulfilled.

Receive on any port

To receive a message on any port, use the any port keywords.

Receive on any port from a port array

To receive a message on any port from a specific port array, use the any from PortArrayRef syntax where PortArrayRef shall be a reference to a  port array identifier. It is also possible to store the index of a port in a single-dimensional port array at which the operation was successful to a variable of type integer or, in case of multi-dimensional port arrays the index of the successful port to an integer array or record of integer variable. When checking the port array for matching messages, the port indices to be checked are iterated from lowest to highest. If the port array is multi-dimensional, then the ports are iterated over from innermost to outermost array dimension from lowest to highest index for each dimension, e.g. [0][0], [0][1], [1][0], [1][1]. The first port which matches all the criteria will cause the operation to be successful even if other ports in the array would also meet the criteria.

Stand-alone receive

The receive operation can be used as a stand-alone statement in a behaviour description. In this latter case the receive operation is considered to be shorthand for an alt statement with the receive operation as the only alternative.

Restrictions

In addition to the general static rules of TTCN-3 given in clause 5 and shown in table 16, the following restrictions apply:

a)	When defining the message in-line, the optional type part shall be present whenever the type of the message being received is ambiguous.

b)	The receive operation shall only be used on message-based ports and the type of the value to be received shall be included in the list of incoming types of the port type definition.

c)	No binding of the incoming values to the terms of the expression or to the template shall occur.

d)	A message received by receive any message shall not be stored, i.e. the value clause shall not be present.

e)	Type mismatch at storing the received value or parts of the received value and storing the sender shall cause an error.

NOTE 6:	An error due to a type mismatch may happen if the types in the receive part are not compatible to the types in the assignment part or, if the from clause is missing, but the type of the sender can be determined and it is not type compatible with the type in the sender clause.

f)	All AddressRef items in the from clause and all ValueRef items in the sender clause shall be of type address, component or of the address type bound to the port type (see clause C.5) of the port instance referenced in the receive operation. No AddressRef in the from clause shall contain the special value null at the time of the operation.

g)	The PortArrayRef shall be a reference to a completely initialized port array.

h)	The index redirection shall only be used when the operation is used on an any from port array construct.

i)	If the index redirection is used for single-dimensional port arrays, the type of the integer variable shall allow storing the highest index of the respective array.

j)	If the index redirection is used for multi-dimensional port arrays, the size of the integer array or record of integer type shall exactly be the same as the dimension of the respective array, and its type shall allow storing the highest index (from all dimensions) of the array.

k)	If a variable referenced in the value, sender or @index clause is a lazy or fuzzy variable, the expression assigned to this variable is equal to the result produced by the receive operation i.e. later evaluation of the lazy or fuzzy variable does not lead to repeated invocation of the receive operation.

l)	If the receive operation contains both from and sender clause, the variable or parameter referenced in the sender clause shall be type compatible with the template in the from clause.

m)	When assigning implicitly decoded message fields (by using the @decoded modifier) in cases where the value or template to be matched uses the MatchDecodedContent (decmatch) matching for the field to be stored, the type of the template in the MatchDecodedContent matching shall be type-compatible to the type of the variable the decoded field is stored into.

n)	The referenced value associated with ValueRef or the return type associated with FunctionInstance followed by the receive keyword, shall be of a port type.

o)	TemplateInstance shall be of a data type.

p)	The @nodefault modifier is allowed only in stand-alone receive statements.

Examples

EXAMPLE 1:	Basic receive

myPort.receive(mw_myTemplate(5, v_myVar));	// Matches a message that fulfils the conditions

// defined by template mw_myTemplate at port myPort.

myPort.receive(v_a<v_b);	// Matches a Boolean value that depends on the outcome of v_a<v_b

myPort.receive(integer:v_myVar);	// Matches an integer value with the value of v_myVar

// at port myPort

myPort.receive(v_myVar);				// Is an alternative to the previous example

EXAMPLE 2:	Receiving from a sender, storing the message, parts of the message or the sender

type MyPayloadType record {

integer		messageId,

ContentType	content

}

type MyType2 record {

Header		header,

octetstring	payload

}

template MyType mw_myTemplate := {

messageId := 42,

content := ?

}

...

var MyPayloadType v_myVar;

var integer v_myMessageIdVar, v_myIntegerVar;

var charstring v_myCharstringVar;

var address v_myPeer;

var octetstring v_myVarOne := '00ff'O;

MyPort.receive(charstring:"Hello")from v_myPeer;	// Matches charstring "Hello" from MyPeer

MyPort.receive(MyType:?) -> value v_myVar;	// The value of the received message is

// assigned to v_myVar.

MyPort.receive(MyType:?) -> value (v_myVar, v_myMessageIdVar:= messageId)

// The value of the received message is stored in the variable

// v_myVar and the value of the messageId field of the received

// message is stored in the variable v_myMessageIdVar.

MyPort.receive(anytype:?) -> value (v_myIntegerVar:= integer)

// If the received value is an integer, it is stored in the variable

// v_myIntegerVar, a test case error otherwise.

MyPort.receive(charstring:?) -> value (v_myCharstringVar)

// The received value is stored in the variable v_myCharstringVar;

// Note that it is the same as to write "value v_myCharstringVar"

MyPort.receive(A<B) -> sender v_myPeer;		// The address of the sender is assigned to v_myPeer

MyPort.receive(MyType:{5, v_myVarOne }) -> value v_myVar sender v_myPeer;

// The received message value is stored in v_myVar and the sender address is stored in

// v_myPeer.

MyPort.receive(MyType2:{header := ?, payload := decmatch mw_myTemplate}) 									-> value (v_myVar := @decoded payload);

// The encoded payload field of the received message is decoded and matched with

// mw_myTemplate; if the matching is successful the decoded payload is stored in v_myVar.

EXAMPLE 3:	Receive any message

myPort.receive;							// Removes the top value from myPort.

myPort.receive from myPeer;				// Removes the top message from myPort if its sender is 
											// myPeer

myPort.receive -> sender v_mySenderVar;	// Removes the top message from myPort and assigns

// the sender address to v_mySenderVar

EXAMPLE 4:	Receive on any port

any port.receive(mw_myMessage);

EXAMPLE 5:	Receive on any port from a port array

type port MyPort message { inout integer }

type component MyComponent {

port MyPort p[10][10];

}

var integer v_i[2];

any from p.receive(mw_myMessage) -> @index value v_i;

// checking receiving mw_myMessage on any port of the port array p and storing the index of the

// port on which the matching was successful first; if, for example MyMessage is matched first

// on p[4,2], the content of i will be {4,2}

### 22.2.3	The Trigger operation

The trigger operation is used to await a specific message on an incoming port queue.

Syntactical Structure

[ @nodefault ] ( ObjectReference | any port | any from PortArrayRef ) "." trigger

[ "(" TemplateInstance ")" ]

[ from Address ]

[ "->" [ value ( ValueRef|

( "(" { ValueRef[ ":=" [ @decoded [ "(" Expression ")" ] ]

FieldOrTypeReference ][","] } ")" )

) ]

[ sender ValueRef]

[ @index value ValueRef] ]

NOTE 1:	Address may be an AddressRef, a list of AddressRef-s or "any component".

Semantic Description

The trigger operation removes the top message from the associated incoming port queue. If that top message meets the matching criteria, the trigger operation behaves in the same manner as a receive operation. If that top message does not fulfil the matching criteria, it shall be removed from the queue without any further action.

The trigger operation requires the port name, matching criteria for type and value, an optional from restriction (i.e. selection of communication partner) and an optional assignment of the matching message and sender component to variables.

Matching criteria

The matching criteria as defined in clause 22.2.2 apply also to the trigger operation.

Trigger from a specific sender

In the case of one-to-many connections the trigger operation may be restricted to a certain communication partner. This restriction shall be denoted using the from keyword followed by a specification of an address or component reference, a list of address or component references or any component.

NOTE 2:	The one-to-one connection is considered to be a simple case of the one-to-many connections and allows the usage of the from-clause.

Trigger on any message

A trigger operation with no argument list shall trigger on the receipt of any message. Thus, its meaning is identical to the meaning of receive any message.

Trigger on any port

To trigger on a message at any port, use the any port keywords.

Trigger on any port from a port array

To trigger on a message at any port from a specific port array,  use the any from PortArrayRef syntax where PortArrayRef shall be a reference to a  port array identifier. It is also possible to store the index of a port in a single-dimensional port array at which the operation was successful to a variable of type integer or, in case of multi-dimensional port arrays the index of the successful port to an integer array or record of integer variable. When checking the port array for matching messages, the port indices to be checked are iterated from lowest to highest. If the port array is multi-dimensional, then the ports are iterated over from innermost to outermost array dimension from lowest to highest index for each dimension, e.g. [0][0], [0][1], [1][0], [1][1]. The first port which matches all the criteria will cause the operation to be successful even if other ports in the array would also meet the criteria.

If any port in the port array which is checked for matching contains a message that does not match, this message is removed and the containing alt statement is re-evaluated, regardless of whether or not other ports in the port array would meet the trigger criteria.

Stand-alone trigger

The trigger operation can be used as a stand-alone statement in a behaviour description. In this latter case the trigger operation is considered to be shorthand for an alt statement with two alternatives (one alternative expecting the message and another alternative consuming all other messages and repeating the alt statement, see ETSI ES 201 873-4 [1]).

Storing the received message, parts of the received message or the sender

Rules in clause 22.2.2 shall apply.

Restrictions

In addition to the general static rules of TTCN-3 given in clause 5 and shown in table 16, the following restrictions apply:

a)	The trigger operation shall only be used on message-based ports and the type of the value to be received shall be included in the list of incoming types of the port type definition.

b)	A message received by TriggerOnAnyMessage shall not be assigned to a variable.

c)	Type mismatch at storing the received value or parts of the received value and storing the sender shall cause an error.

NOTE 3:	An error due to a type mismatch may happen if the types in the receive part are not compatible to the types in the assignment part or, if the from clause is missing, but the type of the sender can be determined and it is not type compatible with the type in the sender clause.

d)	All AddressRef items in the from clause and all ValueRef items in the sender clause shall be of type address, component or of the address type bound to the port type (see clause C.5) of the port instance referenced in the trigger operation. No AddressRef in the from clause shall contain the special value null at the time of the operation.

e)	The PortArrayRef shall be a reference to a completely initialized port array.

f)	The index redirection shall only be used when the operation is used on an any from port array construct.

g)	If the index redirection is used for single-dimensional port arrays, the type of the integer variable shall allow storing the highest index of the respective array.

h)	If the index redirection is used for multi-dimensional port arrays, the size of the integer array or record of integer type shall exactly be the same as the dimension of the respective array, and its type shall allow storing the highest index (from all dimensions) of the array.

i)	If a variable referenced in the value, sender or @index clause is a lazy or fuzzy variable, the expression assigned to this variable is equal to the result produced by the trigger operation, i.e. later evaluation of the lazy or fuzzy variable does not lead to repeated invocation of the trigger operation.

j)	If the trigger operation contains both from and sender clause, the variable or parameter referenced in the sender clause shall be type compatible with the template in the from clause.

k)	The ObjectReference shall be of a port type.

l)	TemplateInstance shall be of a data type.

m)	The @nodefault modifier is allowed only in stand-alone trigger statements.

Examples

EXAMPLE 1:	Basic trigger

myPort.trigger(MyType:?);

// Specifies that the operation will trigger on the reception of the first message observed of

// the type MyType with an arbitrary value at port myPort.

EXAMPLE 2:	Trigger from a sender and with storing message or sender

myPort.trigger(MyType:?) from myPartner;

// Triggers on the reception of the first message of type MyType at port myPort

// received from myPartner.

myPort.trigger(MyType:?) from myPartner -> value v_myRecMessage;

// This example is almost identical to the previous example. In addition, the message which

// triggers i.e. all matching criteria are met, is stored in the variable v_myRecMessage.

myPort.trigger(MyType:?) -> sender myPartner;

// This example is almost identical to the first example. In addition, the reference of the

// sender component will be retrieved and stored in variable myPartner.

myPort.trigger(integer:?) -> value v_myVar sender v_myPartner;

// Trigger on the reception of an arbitrary integer value which afterwards is stored in

// variable v_myVar. The reference of the sender component will be stored in variable MyPartner.

EXAMPLE 3:	Trigger on any message

myPort.trigger;

myPort.trigger from myPartner;

myPort.trigger -> sender v_mySenderVar;

EXAMPLE 4:	Trigger on any port

any port.trigger

EXAMPLE 5:	Trigger on any port from port array

type port MyPort message { inout integer }

type component MyComponent {

port MyPort p[10][10];

}

var integer v_i[2];

any from p.trigger(mw_myMessage) -> @index value v_i;

// Checking if mw_myMessage has been received on any port of the port array p; if yes, the index

// of the port on which the matching was first successful is stored in the array v_i; if no port

// succeeds, the top messages are removed and the port array is re-checked.

## 22.3	Procedure-based communication

### 22.3.0	General

The operations for procedure-based communication via synchronous ports are summarized in table 24.

Table 24: Overview of procedure-based communication

| Communication operation | Keyword |
| --- | --- |
| Invoke procedure call | call |
| Accept procedure call from remote entity | getcall |
| Reply to procedure call from remote entity | reply |
| Raise exception (to an accepted call) | raise |
| Handle response from a previous call | getreply |
| Catch exception (from called entity) | catch |
| Check call/exception/reply received | check |

### 22.3.1	The Call operation

The call operation specifies the call of a remote operation on another test component or within the SUT.

Syntactical Structure

ObjectReference "." call "(" TemplateInstance [ "," ( CallTimerValue | nowait ) ] ")"

[ to Address ]

NOTE:	Address may be an AddressRef, a list of AddressRef-s or "all component".

Semantic Description

The call operation is used to specify that a test component calls a procedure in the SUT or in another test component.

The information to be transmitted in the send part of the call operation is a signature that may either be defined in the form of a signature template or be defined in-line.

Handling responses and exceptions to a call

In case of non-blocking procedure-based communication the handling of exceptions to call operations is done by using catch (see clause 22.3.6) operations as alternatives in alt statements.

If the nowait option is used, the handling of responses or exceptions to call operations is done by using getreply (see clause 22.3.4) and catch (see clause 22.3.6) operations as alternatives in alt statements.

In case of blocking procedure-based communication, the handling of responses or exceptions to a call is done in the response and exception handling part of the call operation by means of getreply (see clause 22.3.4) and catch (see clause 22.3.6) operations.

The response and exception handling part of a call operation looks similar to the body of an alt statement. It defines a set of alternatives, describing the possible responses and exceptions to the call.

If necessary, it is possible to enable/disable an alternative by means of a boolean expression placed between the "[ ]" brackets of the alternative.

The response and exception handling part of a call operation is executed like an alt statement without any active default. This means a corresponding snapshot includes all information necessary to evaluate the (optional) Boolean guards, may include the top element (if any) of the port over which the procedure has been called and may include a timeout exception generated by the (optional) timer that supervises the call.

Handling timeout exceptions to a call

The call operation may optionally include a timeout. This is defined as an explicit value or constant of float type and defines the length of time after the call operation has started that a timeout exception shall be generated by the test system. If no timeout value part is present in the call operation, no timeout exception shall be generated.

Nowait calls of blocking procedures

Using the keyword nowait instead of a timeout exception value in a call operation allows calling a procedure to continue without waiting either for a response or an exception raised by the called procedure or a timeout exception.

If the nowait keyword is used, a possible response or exception of the called procedure has to be removed from the port queue by using a getreply or a catch operation in a subsequent alt statement.

Calling blocking procedures without return value, out parameters, inout parameters and exceptions

A blocking procedure may have no return values, no out and inout parameters and may raise no exception. The call operation for such a procedure shall also have a response and exception handling part to handle the blocking in a uniform manner.

Calling non-blocking procedures

A non-blocking procedure has no out and inout parameters, no return value and the non-blocking property is indicated in the corresponding signature definition by means of a noblock keyword.

Possible exceptions raised by non-blocking procedures have to be removed from the port queue by using catch operations in subsequent alt or interleave statements.

Unicast, multicast and broadcast calls of procedures

Like for the send operation, TTCN-3 also supports unicast, multicast and broadcast calls of procedures. This can be done in the same manner as described in clause 22.2.1, i.e. the argument of the to clause of a call operation is for unicast calls the address of one receiving entity (or can be omitted in case of one-to-one connections), for multicast calls a list of addresses of a set of receivers and for broadcast calls the all component keyword. In case of one-to-one connections, the to clause may be omitted, because the receiving entity is uniquely identified by the system structure.

The handling of responses and exceptions for a blocking or non-blocking unicast call operation has been explained in this clause under "Handling timeout exceptions to a call". A multicast or broadcast call operation may cause several responses and exceptions from different communication partners.

In case of a multicast or broadcast call operation of a non-blocking procedure, all exceptions which may be raised from the different communication partners can be handled in subsequent catch, alt or interleave statements.

In case of a multicast or broadcast call operation of a blocking procedure, responses and exceptions can be handled in several ways: one possibility is to handle only one response or exception in the response and exception handling part of the call operation. All further responses and exceptions are handled in subsequent alt or interleave statements. Alternatively, several or all responses and exceptions are processed in the response and exception handling part of the call operation by means of nested alt statements or by using the repeat operation causing the re-evaluation of the call body. A further possibility is to use the nowait directive and to handle all responses and exceptions in subsequent alt or interleave statements.

Restrictions

In addition to the general static rules of TTCN-3 given in clause 5 and shown in table 16, the following restrictions apply:

a)	The call operation shall only be used on procedure-based ports. The type definition of the port shall include the name of the procedure to which the call operation belongs in its out or inout list i.e. it shall be allowed to call this procedure at this port.

b)	All in and inout parameters of the signature shall have a specific value i.e. the use of matching mechanisms such as AnyValue is not allowed.

c)	Only out parameters may be omitted or specified with a matching attribute.

d)	The signature arguments of the call operation are not used to retrieve variable names for out and inout parameters. The actual assignment of the procedure return value and out and inout parameter values to variables shall explicitly be made in the response and exception handling part of the call operation by means of getreply and catch operations. This allows the use of signature templates in call operations in the same manner as templates can be used for types.

e)	A to clause shall be present in case of one-to-many connections.

f)	All AddressRef items in the to clause shall be of type address, component or of the address type bound to the port type (see clause 6.2.9) of the port instance referenced in the call operation. No AddressRef in the to clause shall contain the special value null at the time of the operation.

g)	CallTimerValue shall be of type float.

h)	The selection of the alternatives to a call shall only be based on getreply and catch operations for the called procedure. Unqualified getreply and catch operations shall only treat replies from and exceptions raised by the called procedure. The use of else branches and the invocation of altsteps is not allowed.

i)	The evaluation of the Boolean expressions guarding the alternatives in the response and exception handling part may have side effects. In order to avoid unexpected side effects, the same rules as for the Boolean guards in alt statements shall be applied (see clause 20.2).

j)	The call operation for a blocking procedures without return value, out parameters, inout parameters and exceptions shall also have a response and exception handling part to handle the blocking in a uniform manner.

k)	In case of a multicast or broadcast call operation of a blocking procedure, where the nowait keyword is used, all responses and exceptions have to be handled in subsequent alt or interleave statements.

l)	The call operation for a non-blocking procedure shall have no response and exception handling part, shall raise no timeout exception and shall not use the nowait keyword.

m)	Applying a call operation to an unmapped or disconnected port shall cause a test case error.

n)	The ObjectReference shall be of a port type.

Examples

EXAMPLE 1:	Blocking call with getreply:

// Given …

signature MyProc (out integer MyPar1, inout boolean MyPar2);

:

// a call of MyProc

myPort.call(MyProc:{ -, v_myVar2}) {		// in-line signature template for the call of MyProc

[] myPort.getreply(MyProc:{?, ?}) { }

}

// … and another call of MyProc

myPort.call(s_myProcTemplate) {				// using signature template for the call of MyProc

[] myPort.getreply(MyProc:{?, ?}) { }

}

myPort.call(s_myProcTemplate) to myPeer {			// calling MyProc at myPeer

[] myPort.getreply(MyProc:{?, ?}) { }

}

EXAMPLE 2:	Blocking call with getreply and catch:

// Given

signature MyProc3 (out integer MyPar1, inout boolean MyPar2) return MyResultType

exception (ExceptionTypeOne, ExceptionTypeTwo);

:

// Call of MyProc3

myPort.call(MyProc3:{ -, true }) to myPartner {

[] myPort.getreply(MyProc3:{?, ?}) -> value v_myResult param (v_myPar1Var,v_myPar2Var) { }

[] myPort.catch(MyProc3, MyExceptionOne) {

setverdict(fail);

stop;

}

[] myPort.catch(MyProc3, ExceptionTypeTwo : ?) {

setverdict(inconc);

}

[MyCondition] myPort.catch(MyProc3, MyExceptionThree) { }

}

EXAMPLE 3:	Blocking call with timeout exception:

myPort.call(MyProc:{5,v_myVar}, 20E-3) {

[] myPort.getreply(MyProc:{?, ?}) { }

[] myPort.catch(timeout) {				// timeout exception after 20ms

setverdict(fail);

stop;

}

}

EXAMPLE 4:	Nowait call:

myPort.call(MyProc:{5, v_myVar}, nowait);	// The calling test component will continue

// its execution without waiting for the

// termination of MyProc

EXAMPLE 5:	Blocking call without return value, out parameters, inout parameters and exceptions:

// Given …

signature MyBlockingProc (in integer MyPar1, in boolean MyPar2);

:

// a call of MyBlockingProc

myPort.call(MyBlockingProc:{ 7, false }) {

[] myPort.getreply( MyBlockingProc:{ -, - } ) { }

}

EXAMPLE 6:	Broadcast call, accepting the first response only from certain communication partners:

var boolean v_allPeersHandled := false;

myPort.call(MyProc:{5,v_myVar}, 20E-3) to all component {	// Broadcast call of MyProc

// Handles the response from myPeerOne

[] myPort.getreply(MyProc:{?, ?}) from myPeerOne {

}

// Handles the response from myPeerTwo

[] myPort.getreply(MyProc:{?, ?}) from myPeerTwo {

}

[] myPort.catch(timeout) {				// timeout exception after 20ms

setverdict(inconc);

stop;

}

}

alt {

[] myPort.getreply(MyProc:{?, ?}) {		// Handles all other responses to the broadcast call

:

if (not(v_allPeersHandled)) {

repeat

}

}

}

EXAMPLE 7:	Multicast call:

myPort.call(MyProc:{5,v_myVar}, nowait) to (myPeer1, myPeer2);	// Multicast call of MyProc

interleave {

[] myPort.getreply(MyProc:{?, ?}) from myPeer1 { }	// Handles the response of myPeer1

[] myPort.getreply(MyProc:{?, ?}) from myPeer2 { }	// Handles the response of myPeer2

}

### 22.3.2	The Getcall operation

The getcall operation is used to accept calls.

Syntactical Structure

[ @nodefault ] ( ObjectReference | any port | any from PortArrayRef ) "." getcall

[ "(" TemplateInstance ")" ]

[ from Address ]

[ "->" [ param "(" { ( ValueRef":=" [ @decoded [ "(" Expression ")" ] ]												ParameterIdentifier ) "," } |

{ ( ValueRef| "-" ) "," }

")" ]

[ sender ValueRef]

[ @index value ValueRef] ]

NOTE 1:	Address may be an AddressRef, a list of AddressRef-s or "any component".

Semantic Description

The getcall operation is used to specify that a test component accepts a call from the SUT, or another test component.

The getcall operation shall remove the top call from the incoming port queue, if, and only if, the matching criteria associated to the getcall operation are fulfilled. These matching criteria are related to the signature of the call to be processed and the communication partner. The matching criteria for the signature may either be specified in-line or be derived from a signature template.

The assignment of in and inout parameter values to variables shall be made in the assignment part of the getcall operation. This allows the use of signature templates in getcall operations in the same manner as templates are used for types.

A getcall operation may be restricted to a certain communication partner in case of one-to-many connections. This restriction shall be denoted by using the from keyword followed by a specification of an address or component reference, a list of address or component references or any component.

NOTE 2:	The one-to-one connection is considered to be a simple case of the one-to-many connections and allows the usage of the from-clause.

The (optional) assignment part of the getcall operation comprises the assignment of in and inout parameter values to variables and the retrieval of the address of the calling component. The keyword param is used to retrieve the parameter values of a call.

When assigning individual parameters of a call, encoded parameters can be decoded prior to assignment using the @decoded modifier. In this case, the referenced parameter on the right hand sided of the assignment shall be one of the bitstring, hexstring, octetstring, charstring or universal charstring types. It shall be decoded into a value of the same type as the variable on the left hand side of the assignment. Failure of this decoding shall cause a test case error. In case the referenced field is of the universal charstring type, the @decoded clause can contain an optional parameter defining the encoding format. The parameter shall be of the charstring type and it shall contain one of the strings allowed for the decvalue_unichar function (specified in clause C.5.4). Any other value shall cause an error. In case the referenced field is not a universal charstring, the optional parameter shall not be present.

The keyword sender is used when it is required to retrieve the address of the sender (e.g. for addressing a reply or exception to the calling party in a one-to-many configuration).

Accepting any call

A getcall operation with no argument list for the signature matching criteria will remove the call on the top of the incoming port queue (if any) if all other matching criteria are fulfilled.

Getcall on any port

To getcall on any port is denoted by the any keyword.

Getcall on any port from a port array

To getcall on any port from a specific port array,  use the any from PortArrayRef syntax where PortArrayRef shall be a reference to a  port array identifier. It is also possible to store the index of a port in a single-dimensional port array at which the operation was successful to a variable of type integer or, in case of multi-dimensional port arrays the index of the successful port to an integer array or record of integer variable. When checking the port array for matching calls, the port indices to be checked are iterated from lowest to highest. If the port array is multi-dimensional, then the ports are iterated over from innermost to outermost array dimension from lowest to highest index for each dimension, e.g. [0][0], [0][1], [1][0], [1][1]. The first port which matches all the criteria will cause the operation to be successful even if other ports in the array would also meet the criteria.

Restrictions

In addition to the general static rules of TTCN-3 given in clause 5 and shown in table 16, the following restrictions apply:

a)	The getcall operation shall only be used on procedure-based ports. The type definition of the port shall include the name of the procedure to which the getcall operation belongs in its in or inout list.

b)	The signature argument of the getcall operation shall not be used to pass in variable names for in and inout parameters.

c)	The ParameterIdentifiers shall be from the corresponding signature definition.

d)	The value assignment part shall not be used with the getcall operation.

e)	Parameters of calls accepted by accepting any call shall not be assigned to a variable, i.e. the param clause shall not be present.

f)	All AddressRef items in the from clause and all ValueRef items in the sender clause shall be of type address, component or of the address type bound to the port type (see clause C.5) of the port instance referenced in the getcall operation. No AddressRef in the from clause shall contain the special value null at the time of the operation.

g)	The PortArrayRef shall be a reference to a completely initialized port array.

h)	The index redirection shall only be used when the operation is used on an any from port array construct.

i)	If the index redirection is used for single-dimensional port arrays, the type of the integer variable shall allow storing the highest index of the respective array.

j)	If the index redirection is used for multi-dimensional port arrays, the size of the integer array or record of integer type shall exactly be the same as the dimension of the respective array, and its type shall allow storing the highest index (from all dimensions) of the array.

k)	If a variable referenced in the param, sender or @index clause is a lazy or fuzzy variable, the expression assigned to this variable is equal to the result produced by the getcall operation, i.e. later evaluation of the lazy or fuzzy variable does not lead to repeated invocation of the getcall operation.

l)	If the getcall operation contains both from and sender clause, the variable or parameter referenced in the sender clause shall be type compatible with the template in the from clause. If the operation contains a sender clause but no from clause, the sender shall be type compatible with the type of the variable or parameter referenced in the sender clause.

NOTE 3:	An error due to a type mismatch may happen if the types in the receive part are not compatible to the types in the assignment part or, if the from clause is missing, but the type of the sender can be determined and it is not type compatible with the type in the sender clause.

m)	When assigning implicitly decoded parameters (by using the @decoded modifier) in cases where the value or template to be matched uses the MatchDecodedContent (decmatch) matching for the parameter to be stored, the type of the template in the MatchDecodedContent matching shall be type-compatible to the type of the variable the decoded field is stored into.

n)	The ObjectReference shall be of a port type.

o)	The @nodefault modifier is allowed only in stand-alone getcall statements.

Examples

EXAMPLE 1:	Basic getcall:

myPort.getcall(MyProc: s_myProcTemplate(5, v_myVar));	// accepts a call of MyProc at myPort

myPort.getcall(MyProc:{5, v_myVar}) from myPeer; // accepts a call of MyProc at myPort from

// myPeer

EXAMPLE 2:	Getcall with matching and assignments of parameter values to variables:

myPort.getcall(MyProc:{?, ?}) from myPartner -> param (v_myPar1Var, v_myPar2Var);

// The in or inout parameter values of MyProc are assigned to v_myPar1Var and v_myPar2Var.

myPort.getcall(MyProc:{5, v_myVar}) -> sender v_mySenderVar;

// Accepts a call of MyProc at myPort with the in or inout parameters 5 and v_myVar.

// The address of the calling party is retrieved and stored in v_mySenderVar.

// The following getcall examples show the possibilities to use matching attributes

// and omit optional parts, which may be of no importance for the test specification.

myPort.getcall(MyProc:{5, v_myVar}) -> param(v_myVar1, v_myVar2) sender v_mySenderVar;

myPort.getcall(MyProc:{5, ?}) -> param(v_myVar1, v_myVar2);

myPort.getcall(MyProc:{?, v_myVar}) -> param( - , v_myVar2);

// The value of the first inout parameter is not important or not used

// The following examples shall explain the possibilities to assign in and inout parameter

// values to variables. The following signature is assumed for the procedure to be called:

signature MyProc2(in integer A, integer B, integer C, out integer D, inout integer E);

myPort.getcall(MyProc2:{?, ?, 3, - , ?}) -> param (v_myVarA, v_myVarB, - , -, v_myVarE);

// The parameters A, B, and E are assigned to the variables v_myVarA, v_myVarB, and

// v_myVarE. The out parameter D needs not to be considered.

myPort.getcall(MyProc2:{?, ?, 3, -, ?}) -> param (v_myVarA:= A, v_myVarB:= B, v_myVarE:= E);

// Alternative notation for the value assignment of in and inout parameter to variables. Note,

// the names in the assignment list refer to the names used in the signature of MyProc2

myPort.getcall(MyProc2:{1, 2, 3, -, *}) -> param (v_myVarE:= E);

// Only the inout parameter value is needed for the further test case execution

// The following example demonstrates the use of encoded parameters:

signature MyProc3(in integer paramType, octetstring encodedParam);

template integer mw_int := ?;

…

var integer v_myVarX;

myPort.getcall(MyProc3:{1, decmatch mw_int}) -> param (v_myVarX := @decoded encodedParam);

// The parameters encodedParam is decoded into an integer and assigned to v_myVarX.

EXAMPLE 3:	Accepting any call:

myPort.getcall;					// Removes the top call from myPort.

myPort.getcall from myPartner;	// Removes a call from myPartner from port myPort

myPort.getcall -> sender v_mySenderVar;	// Removes a call from myPort and retrieves

// the address of the calling entity

EXAMPLE 4:	Getcall on any port:

any port.getcall(MyProc:?)

EXAMPLE 5:	Getcall on any port from port array:

type port MyPort procedure { inout MyProc }

type component MyComponent {

port MyPort p[10][10];

}

var integer v_i[2];

any from p.getcall(MyProc:?) -> @index value v_i;

// checking for an incoming call of the type MyProc on any port of the port array p and storing

// the index of the port on which the matching was successful first

### 22.3.3	The Reply operation

The reply operation is used to reply to a call.

Syntactical Structure

ObjectReference "." reply "(" TemplateInstance [ value TemplateBody ] ")"

[ to Address ]

NOTE 1:	Address may be an AddressRef, a list of AddressRef-s or "all component".

Semantic Description

The reply operation is used to reply to a previously accepted call according to the procedure signature.

NOTE 2:	The relation between an accepted call and a reply operation cannot always be checked statically. For testing it is allowed to specify a reply operation without an associated getcall operation.

The value part of the reply operation consists of a signature reference with an associated actual parameter list and (optional) return value. The signature may either be defined in the form of a signature template or it may be defined in-line.

Responses to one or more call operations may be sent to one, several or all peer entities connected to the addressed port. This can be specified in the same manner as described in clause 22.2.1. This means, the argument of the to clause of a reply operation is for unicast responses the address of one receiving entity, for multicast responses a list of addresses of a set of receivers and for broadcast responses the all component keywords.

In case of one-to-one connections, the to clause may be omitted, because the receiving entity is uniquely identified by the system structure.

A return value or template shall be explicitly stated with the value keyword and is first evaluated before returning.

Restrictions

In addition to the general static rules of TTCN-3 given in clause 5 and shown in table 16, the following restrictions apply:

- A reply operation shall only be used at a procedure-based port. The type definition of the port shall include the name of the procedure to which the reply operation belongs in its in or inout list.

- The TemplateInstance in the reply operation shall identify the signature definition and all signature parameters of the procedure to which the reply operation belongs.

- All out and inout parameters of the signature shall have a specific value i.e. the use of matching mechanisms such as AnyValue is not allowed.

- A to clause shall be present in case of one-to-many connections.

- All AddressRef items in the to clause shall be of type address, component or of the address type bound to the port type (see clause 6.2.9) of the port instance referenced in the reply operation. No AddressRef in the to clause shall contain the special value null at the time of the operation.

- The optional value clause for a return value shall only be present if the signature definition of the procedure to which the reply operation belongs defines a return type.

- The TemplateBody in the value clause shall conform to the template(value) restriction and it shall be type-compatible with the return type specified in the signature of the procedure to which the reply operation belongs.

- Applying a reply operation to an unmapped or disconnected port shall cause a test case error.

- The ObjectReference shall be of a port type.

Examples

myPort.reply(MyProc2:{ - ,5});			// Replies to an accepted call of MyProc2.

myPort.reply(MyProc2:{ - ,5}) to myPeer; // Replies to an accepted call of MyProc2 from myPeer

myPort.reply(MyProc2:{ - ,5}) to (myPeer1, myPeer2); // Multicast reply to myPeer1 and myPeer2

myPort.reply(MyProc2:{ - ,5}) to all component;	// Broadcast reply to all entities connected

// to myPort

myPort.reply(MyProc3:{5, v_myVar} value 20);	// Replies to an accepted call of MyProc3.

### 22.3.4	The Getreply operation

The getreply operation is used to handle replies from a previously called procedure.

Syntactical Structure

[ @nodefault ] ( ObjectReference | any port | any from PortArrayRef ) "." getreply

[ "(" TemplateInstance [ value TemplateInstance ]")" ]

[ from Address ]

[ "->" [ value (ValueRef|

( "(" { ValueRef[ ":=" [ @decoded [ "(" Expression ")" ] ]
 											  FieldOrTypeReference ][","] } ")" )

)]

[ param "(" { ( ValueRef":=" [ @decoded [ "(" Expression ")" ] ]
												ParameterIdentifier ) "," } |

{ ( ValueRef| "-" ) "," }

")" ]

[ sender ValueRef]

[ @index value ValueRef] ]

NOTE 1:	Address may be an AddressRef, a list of AddressRef-s or "any component".

Semantic Description

The getreply operation is used to handle replies from a previously called procedure.

The getreply operation shall remove the top reply from the incoming port queue, if, and only if, the matching criteria associated to the getreply operation are fulfilled. These matching criteria are related to the signature of the procedure to be processed and the communication partner. The matching criteria for the signature may either be specified in-line or be derived from a signature template.

Matching against a received return value can be specified by using the value keyword.

A getreply operation may be restricted to a certain communication partner in case of one-to-many connections. This restriction shall be denoted by using the from keyword followed by a specification of an address or component reference, a list of address or component references or any component.

NOTE 2:	The one-to-one connection is considered to be a simple case of the one-to-many connections and allows the usage of the from-clause.

The assignment of out and inout parameter values to variables shall be made in the assignment part of the getreply operation. This allows the use of signature templates in getreply operations in the same manner as templates are used for types.

The (optional) assignment part of the getreply operation comprises the assignment of out and inout parameter values to variables and the retrieval of the address of the sender of the reply. The keyword value is used to retrieve return values and the keyword param is used to retrieve the parameter values of a reply. The keyword sender is used when it is required to retrieve the address of the sender.

When assigning individual parameters or referenced fields of the return value of a reply, encoded parameters can be decoded prior to assignment using the @decoded modifier. In this case, the referenced parameter or field of the return value on the right hand sided of the assignment shall be one of the bitstring, hexstring, octetstring, charstring or universal charstring types. It shall be decoded into a value of the same type as the variable on the left hand side of the assignment. Failure of this decoding shall cause a test case error. In case the parameter or referenced field of the return value is of the  universal charstring type, the @decoded clause can contain an optional parameter defining the encoding format. The parameter shall be of the charstring type and it shall contain one of the strings allowed for the decvalue_unichar function (specified in clause C.5.4). Any other value shall cause an error. In case the parameter or referenced field of the return value is not a universal char string, the optional parameter shall not be present.

Get any reply

A getreply operation with no argument list for the signature matching criteria shall remove the reply message on the top of the incoming port queue (if any) if all other matching criteria are fulfilled.

If GetAnyReply is used in the response and exception handling part of a call operation, it shall only treat replies from the procedure invoked by the call operation.

Get a reply on any port

To get a reply on any port, use the any port keywords.

Get a reply on any port from a port array

To get a reply on any port from a specific port array,  use the any from PortArrayRef syntax where PortArrayRef shall be a reference to a  port array identifier. It is also possible to store the index of a port in a single-dimensional port array at which the operation was successful to a variable of type integer or, in case of multi-dimensional port arrays the index of the successful port to an integer array or record of integer variable. When checking the port array for matching replies, the port indices to be checked are iterated from lowest to highest. If the port array is multi-dimensional, then the ports are iterated over from innermost to outermost array dimension from lowest to highest index for each dimension, e.g. [0][0], [0][1], [1][0], [1][1]. The first port which matches all the criteria will cause the operation to be successful even if other ports in the array would also meet the criteria.

Restrictions

In addition to the general static rules of TTCN-3 given in clause 5 and shown in table 16, the following restrictions apply:

a)	A getreply operation shall only be used at a procedure-based port. The type definition of the port shall include the name of the procedure to which the getreply operation belongs.

b)	The signature argument of the getreply operation shall not be used to pass in variable names for out and inout parameters.

c)	Parameters or return values of responses accepted by get any reply shall not be assigned to a variable, i.e. the param and value clause shall not be present.

d)	All AddressRef items in the from clause and all ValueRef items in the sender clause shall be of type address, component or of the address type bound to the port type (see clause 6.2.9) of the port instance referenced in the getreply operation. No AddressRef in the from clause shall contain the special value null at the time of the operation.

e)	The PortArrayRef shall be a reference to a completely initialized port array.

f)	The index redirection shall only be used when the operation is used on an any from port array construct.

g)	If the index redirection is used for single-dimensional arrays, the type of the integer variable shall allow storing the highest index of the respective port array.

h)	If the index redirection is used for multi-dimensional arrays, the size of the integer array or record of integer type shall exactly be the same as the dimension of the respective port array, and the its type shall allow storing the highest index (from all dimensions) of the port array.

i)	If a variable referenced in the value, param, sender or @index clause is a lazy or fuzzy variable, the expression assigned to this variable is equal to the result produced by the getreply operation, i.e. later evaluation of the lazy or fuzzy variable does not lead to repeated invocation of the getreply operation.

j)	If the getreply operation contains both from and sender clause, the variable or parameter referenced in the sender clause shall be type compatible with the template in the from clause. If the operation contains a sender clause but no from clause, the sender shall be type compatible with the variable or parameter referenced in the sender clause.

NOTE 3:	An error due to a type mismatch may happen if the types in the receive part are not compatible to the types in the assignment part or, if the from clause is missing, but the type of the sender can be determined and it is not type compatible with the type in the sender clause.

k)	When assigning implicitly decoded parameters or referenced fields of the return value (by using the @decoded modifier) in cases where the value or template to be matched uses the MatchDecodedContent (decmatch) matching for the parameter to be stored, the type of the template in the MatchDecodedContent matching shall be type-compatible to the type of the variable the decoded field is stored into.

l)	The ObjectReference shall be of a port type.

m)	The @nodefault modifier is allowed only in stand-alone getreply statements.

Examples

EXAMPLE 1:	Basic getreply:

myPort.getreply(MyProc:{5, ?} value 20);	// Accepts a reply of MyProc with two out or

// inout parameters and a return value of 20

myPort.getreply(MyProc2:{ - , 5}) from myPeer;	// Accepts a reply of MyProc2 from myPeer

EXAMPLE 2:	Getreply with storing inout/out parameters and return values in variables:

myPort.getreply(MyProc1:{?, ?} value ?) -> value v_myRetValue param(v_myPar1, v_myPar2);

// The returned value is assigned to variable v_myRetValue and the value

// of the two out or inout parameters are assigned to the variables v_myPar1 and v_myPar2.

myPort.getreply(MyProc1:{?, ?} value ?)-> value v_myRetValue param(- ,v_myPar2) sender mySender;

// The value of the first parameter is not considered for the further test execution and

// the address of the sender component is retrieved and stored in the variable mySender.

// The following examples describe some possibilities to assign out and inout parameter values

// to variables. The following signature is assumed for the procedure which has been called

signature MyProc2(in integer A, integer B, integer C, out integer D, inout integer E);

myPort.getreply(s_aTemplate) -> param( - , - , - , v_myVarOut1, v_myVarInout1);

myPort.getreply(s_aTemplate) -> param(v_myVarOut1:=D, v_myVarOut2:=E);

myPort.getreply(MyProc2:{ - , - , - , 3, ?}) -> param(v_myVarInout1:=E);

// The following example demonstrates the use of encoded parameters:

signature MyProc3(out integer paramType, out octetstring encodedParam);

template integer mw_int := ?;

…

var integer v_myVarX;

myPort.getreply(MyProc3:{1, decmatch mw_int}) -> param (v_myVarX := @decoded encodedParam);

// The parameters encodedParam is decoded into an integer and assigned to v_myVarX.

EXAMPLE 3:	Get any reply:

myPort.getreply;				// Removes the top reply from myPort.

myPort.getreply from myPeer;	// Removes the top reply received from myPeer from myPort.

myPort.getreply -> sender v_mySenderVar;	// Removes the top reply from myPort and retrieves

// the address of the sender entity

EXAMPLE 4:	Get a reply on any port:

any port.getreply(Myproc:?)

EXAMPLE 5:	Get a reply on any port from port array:

type port MyPort procedure { inout MyProc }

type component MyComponent {

port MyPort p[10][10];

}

var integer v_i[2];

any from p.getreply(MyProc:?) -> @index value v_i;

// Getting a reply of the type MyProc on any port of the port array p and

// storing the index of the port on which the matching was successful first

### 22.3.5	The Raise operation

Exceptions are raised with the raise operation.

Syntactical Structure

ObjectReference "." raise "(" Signature "," TemplateInstance ")"

[ to Address ]

NOTE 1:	Address may be an AddressRef, a list of AddressRef-s or "all component".

Semantic Description

The raise operation is used to raise an exception.

NOTE 2:	The relation between an accepted call and a raise operation cannot always be checked statically. For testing it is allowed to specify a raise operation without an associated getcall operation.

The value part of the raise operation consists of the signature reference followed by the exception value.

Exceptions are specified as types. Therefore the exception value may either be derived from a template conforming to the template(value) restriction or be the value resulting from an expression (which of course can be an explicit value). The optional type field in the value specification to the raise operation shall be used in cases where it is necessary to avoid any ambiguity of the type of the value being sent.

Exceptions to one or more call operations may be sent to one, several or all peer entities connected to the addressed port. This can be specified in the same manner as described in clause 22.2.1. This means, the argument of the to clause of a raise operation is for unicast exceptions the address of one receiving entity, for multicast exceptions a list of addresses of a set of receivers and for broadcast exceptions the all component keywords.

In case of one-to-one connections, the to clause may be omitted, because the receiving entity is uniquely identified by the system structure.

Restrictions

In addition to the general static rules of TTCN-3 given in clause 5 and shown in table 16, the following restrictions apply:

a)	An exception shall only be raised at a procedure-based port. An exception is a reaction to an accepted procedure call the result of which leads to an exceptional event.

b)	The type of the exception shall be specified in the signature of the called procedure. The type definition of the port shall include in its in or inout list the name of the procedure to which the exception belongs.

c)	A to clause shall be present in case of one-to-many connections.

d)	All AddressRef items in the to clause shall be of type address, component or of the address type bound to the port type (see clause 6.2.9) of the port instance referenced in the raise operation. No AddressRef in the to clause shall contain the special value null at the time of the operation.

e)	Applying a raise operation to an unmapped or disconnected port shall cause a test case error.

f)	The TemplateInstance shall conform to the template(value) restriction (see clause 15.8).

g)	The ObjectReference shall be of a port type.

Examples

myPort.raise(MySignature, v_myVariable + v_yourVariable - 2);

// Raises an exception with a value which is the result of the arithmetic expression

// at myPort

myPort.raise(MyProc, integer:5});	// Raises an exception with the integer value 5 for MyProc

myPort.raise(MySignature, "My string") to myPartner;

// Raises an exception with the value "My string" at myPort for MySignature and

// send it to myPartner

myPort.raise(MySignature, "My string") to (myPartnerOne, myPartnerTwo);

// Raises an exception with the value "My string" at myPort and sends it to myPartnerOne and

// myPartnerTwo (i.e. multicast communication)

myPort.raise(MySignature, "My string") to all component;

// Raises an exception with the value "My string" at myPort for MySignature and sends it

// to all entities connected to myPort (i.e. broadcast communication)

### 22.3.6	The Catch operation

The catch operation is used to catch exceptions.

Syntactical Structure

[ @nodefault ] ( ObjectReference | any port | any from PortArrayRef ) "." catch

[ "(" ( Signature [ "," TemplateInstance ] ) | TimeoutKeyword ")" ]

[ from Address ]

[ "->" [ value ( ValueRef|

( "(" { ValueRef[ ":=" [ @decoded [ "(" Expression ")" ] ]												FieldOrTypeReference ][","] } ")" )

) ]

[ sender ValueRef]

[ @index value ValueRef] ]

NOTE 1:	Address may be an AddressRef, a list of AddressRef-s or "any component".

Semantic Description

The catch operation is used to catch exceptions raised by a test component or the SUT as a reaction to a procedure call. Exceptions are specified as types and thus, can be treated like messages, e.g. templates can be used to distinguish between different values of the same exception type. If a Signature is given in the parameter list, it is possible to omit the TemplateInstance part if the catch operation shall match any exception value of any of the exception types declared in the definition of the referenced Signature.

The catch operation removes the top exception from the associated incoming port queue if, and only if, that top exception satisfies all the matching criteria associated with the catch operation.

A catch operation may be restricted to a certain communication partner in case of one-to-many connections. This restriction shall be denoted by using the from keyword followed by a specification of an address or component reference, a list of address or component references or any component.

NOTE 2:	The one-to-one connection is considered to be a simple case of the one-to-many connections and allows the usage of the from-clause.

The (optional) redirection part of the catch operation comprises of storing the exception value and/or one or more parts of it and the retrieval of the address of the calling component. The keyword value is used to retrieve the value of an exception and/or the parts of it and the keyword sender is used when it is required to retrieve the address of the sender.

When assigning individual fields of an exception, encoded payload fields can be decoded prior to assignment using the @decoded modifier. In this case, the referenced field on the right hand sided of the assignment shall be one of the bitstring, hexstring, octetstring, charstring or universal charstring types. It shall be decoded into a value of the same type as the variable on the left hand side of the assignment. Failure of this decoding shall cause a test case error. In case the referenced field is of the universal charstring type, the @decoded clause can contain an optional parameter defining the encoding format. The parameter shall be of the charstring type and it shall contain one of the strings allowed for the decvalue_unichar function (specified in clause C.5.4). Any other value shall cause an error. In case the referenced field is not a universal charstring, the optional parameter shall not be present.

The catch operation may be part of the response and exception handling part of a call operation or be used to determine an alternative in an alt statement. If the catch operation is used in the accepting part of a call operation, the information about port name and signature reference to indicate the procedure that raised the exception is redundant, because this information follows from the call operation. However, for readability reasons (e.g. in case of complex call statements) this information shall be repeated.

The Timeout exception

There is one special timeout exception that can be caught by the catch operation. The timeout exception is an emergency exit for cases where a called procedure neither replies nor raises an exception within a predetermined time (see clause 22.3.1).

Catch any exception

A catch operation with no argument list allows any valid exception to be caught. The most general case is without using the from keyword. CatchAnyException will also catch the timeout exception.

Catch any exception for specific signature

A catch operation using only a Signature reference in the argument list allows any valid exception for that signature to be caught.

Catch on any port

To catch an exception on any port use the any keyword.

Catch on any port from a port array

To catch an exception on any port from a specific port array, indices use the any from PortArrayRef syntax where PortArrayRef shall be a reference to a  port array identifier. It is also possible to store the index of a port in a single-dimensional port array at which the operation was successful to a variable of type integer or, in case of multi-dimensional port arrays the index of the successful port to an integer array or record of integer variable. When checking the port array for matching exceptions, the port indices to be checked are iterated from lowest to highest. If the port array is multi-dimensional, then the ports are iterated over from innermost to outermost array dimension from lowest to highest index for each dimension, e.g. [0][0], [0][1], [1][0], [1][1]. The first port which matches all the criteria will cause the operation to be successful even if other ports in the array would also meet the criteria.

The catch on any port from a port array operation cannot be used to catch a call timeout.

Restrictions

In addition to the general static rules of TTCN-3 given in clause 5 and shown in table 16, the following restrictions apply:

- The catch operation shall only be used at procedure-based ports. The type of the caught exception shall be specified in the signature of the procedure that raised the exception.

- The type definition of the port shall include in its out or inout list the name of the procedure to which the exception belongs.

- No binding of the incoming values to the terms of the expression or to the template shall occur. The assignment of the exception values to variables shall be made in the assignment part of the catch operation.

- Catching timeout exceptions shall be restricted to the exception handling part of a call. No further matching criteria (including a from part) and no assignment part is allowed for a catch operation that handles a timeout exception.

- Exception values accepted by catch any exception shall not be assigned to a variable, i.e. the value clause shall not be present.

- If CatchAnyException is used in the response and exception handling part of a call operation, it shall only treat exceptions raised by the procedure invoked by the call operation.

- All AddressRef items in the from clause and all ValueRef items in the sender clause shall be of type address, component or of the address type bound to the port type (see clause 6.2.9) of the port instance referenced in the catch operation. No AddressRef in the from clause shall contain the special value null at the time of the operation.

- The PortArrayRef shall be a reference to a completely initialized port array.

- The index redirection shall only be used when the operation is used on an any from port array construct.

- If the index redirection is used for single-dimensional arrays, the type of the integer variable shall allow storing the highest index of the respective port array.

- If the index redirection is used for multi-dimensional arrays, the size of the integer array or record of integer type shall exactly be the same as the dimension of the respective port array, and the its type shall allow storing the highest index (from all dimensions) of the port array.

- If a variable referenced in the value, sender or @index clause is a lazy or fuzzy variable, the expression assigned to this variable is equal to the result produced by the catch operation, i.e. later evaluation of the lazy or fuzzy variable does not lead to repeated invocation of the catch operation.

- If the catch operation contains both from and sender clause, the variable or parameter referenced in the sender clause shall be type compatible with the template in the from clause. If the operation contains a sender clause but no from clause, the sender shall be type compatible with the variable or parameter referenced in the sender clause.

NOTE 3:	An error due to a type mismatch may happen if the types in the receive part are not compatible to the types in the assignment part or, if the from clause is missing, but the type of the sender can be determined and it is not type compatible with the type in the sender clause.

- When assigning implicitly decoded exception fields (by using the @decoded modifier) in cases where the value or template to be matched uses the MatchDecodedContent (decmatch) matching for the parameter to be stored, the type of the template in the MatchDecodedContent matching shall be type-compatible to the type of the variable the decoded field is stored into.

- The referenced value associated with Ref or the return type associated with FunctionInstance followed by the catch keyword, shall be of a port type.

- If no TemplateInstance is provided in the parameter list, then also no value clause shall be present in the redirection part.

- The @nodefault modifier is allowed only in stand-alone catch statements.

Examples

EXAMPLE 1:	Basic catch:

myPort.catch(MyProc, integer: v_myVar);	   // Catches an integer exception of value

// v_myVar raised by MyProc at port myPort.

myPort.catch(MyProc, v_myVar);			   // Is an alternative to the previous example.

myPort.catch(MyProc, v_a<v_b);			   // Catches a boolean exception

myPort.catch(MyProc, MyType:{5, v_myVar}); // In-line template definition of an exception value.

myPort.catch(MyProc, charstring:"Hello")from myPeer;	// Catches "Hello" exception from myPeer

EXAMPLE 2:	Catch with storing value and/or sender in variables:

myPort.catch(MyProc, MyType:?) from myPartner -> value v_myVar;

// Catches an exception from myPartner and assigns its value to v_myVar.

myPort.catch(MyProc, s_myTemplate(5)) -> value v_myVarTwo sender myPeer;

// Catches an exception, assigns its value to v_myVarTwo and retrieves the

// address of the sender.

myPort.catch(MyProc, s_myTemplate(5)) -> value (v_myVarThree:= f1)

sender myPeer;

// Catches an exception, assigns the value of its field f1 to v_myVarThree and retrieves the

// address of the sender.

// Handling encoded exception payload:

type MyException record {

...

}

type CommonException record {

integer		exceptionId,

octetstring	payload

}

signature S() exception (CommonException);

...

var MyException v_myVar;

myPort.catch (S, CommonException:{exceptionId := 25, payload := decmatch MyException:? }) 										-> value (v_myVar := @decoded payload);

// The encoded payload field of the caught exception is decoded and matched with m_excTemplate;

// if the matching is successful the decoded payload is stored in v_myVar.

EXAMPLE 3:	The Timeout exception:

myPort.call(MyProc:{5, v_myVar}, 20E-3) {

[] myPort.getreply(MyProc:{?, ?}) { }

[] myPort.catch(timeout) {				// timeout exception after 20ms

setverdict(fail);

stop;

}

}

EXAMPLE 4:	Catch any exception:

myPort.catch;

myPort.catch from myPartner;

myPort.catch -> sender v_mySenderVar;

myPort.catch(MyProc); // catch any exception raised by procedure MyProc

EXAMPLE 5:	Catch on any port:

any port.catch;

EXAMPLE 6:	Catch on any port from port array:

type port MyPort procedure { inout MyProc }

type component MyComponent {

port MyPort p[10][10];

}

var integer v_i[2];

any from p.catch(MyProc, MyType:?) -> @index value v_i;

// Catching an incoming exception of type MyType on any port in the port array p and

// storing the index of the port on which the matching was successful first

## 22.4	The Check operation

The check operation allows reading the top element of a message-based or procedure-based incoming port queue.

Syntactical Structure

[ @nodefault ] ( ObjectReference | any port | any from PortArrayRef ) "." check

[ "("

( PortReceiveOp | PortGetCallOp | PortGetReplyOp | PortCatchOp ) |

( [ from Address ]

[ "->" [ sender ValueRef]

[ @index value ValueRef] ] )

")" ]

NOTE 1:	Address may be an AddressRef, a list of AddressRef-s or "any component".

Semantic Description

The check operation is a generic operation that allows read access to the top element of message-based and procedure-based incoming port queues without removing the top element from the queue. The check operation has to handle values of a certain type at message-based ports and to distinguish between calls to be accepted, exceptions to be caught and replies from previous calls at procedure-based ports.

The receiving operations receive, getcall, getreply and catch together with their matching and value, sender or parameter storing parts, are used by the check operation to define the conditions that have to be checked and the information to be optionally extracted.

It is the top element of an incoming port queue that shall be checked (it is not possible to look into the queue). If the queue is empty the check operation fails. If the queue is not empty, a copy of the top element is taken and the receiving operation specified in the check operation is performed on the copy. The check operation fails if the receiving operation fails i.e. the matching criteria are not fulfilled. In this case the copy of the top element of the queue is discarded and test execution continues in the normal manner, i.e. the statement or alternative next to the check operation is evaluated. The check operation is successful if the receiving operation is successful. In this case, the value, sender or parameter storing parts of the receiving operation, if any, are executed, i.e. the message and/or a part of it, the sender's address or component reference, the parameter(s) of the call or reply or the value of the exception are stored in the associated variables.

If check is used as a stand-alone statement, it is considered to be a shorthand for an alt statement with the check operation as the only alternative.

Check from a specific sender

In the case of one-to-many connections the check operation may be restricted to a certain communication partner. This restriction shall be denoted using the from keyword followed by a specification of an address or component reference, a list of address or component references or any component.

NOTE 2:	The one-to-one connection is considered to be a simple case of the one-to-many connections and allows the usage of the from-clause.

Check any operation

A check operation with no argument list allows checking whether something waits for processing in an incoming port queue. The check any operation allows to distinguish between different senders (in case of one-to-many connections) by using a from clause and to retrieve the sender by using a shorthand assignment part with a sender clause.

Check on any port

To check on any port, use the any port keywords.

Check on any port from a port array

To check on any port from a specific port array, indices use the any from PortArrayRef syntax where PortArrayRef shall be a reference to a  port array identifier. It is also possible to store the index of a port in a single-dimensional port array at which the operation was successful to a variable of type integer or, in case of multi-dimensional port arrays the index of the successful port to an integer array or record of integer variable. When checking the port array for a matching message, call, reply or exception, the port indices to be checked are iterated from lowest to highest. If the port array is multi-dimensional, then the ports are iterated over from innermost to outermost array dimension from lowest to highest index for each dimension, e.g. [0][0], [0][1], [1][0], [1][1]. The first port which matches all the criteria will cause the operation to be successful even if other ports in the array would also meet the criteria.

Restrictions

In addition to the general static rules of TTCN-3 given in clause 5 and shown in table 16, the following restrictions apply:

- Using the check operation in a wrong manner, e.g. check for an exception at a message-based port shall cause a test case error.

- All AddressRef items in the from clause and all ValueRef items in the sender clause shall be of type address, component or of the address type bound to the port type (see clause C.5) of the port instance referenced in the check operation. No AddressRef in the from clause shall contain the special value null at the time of the operation.

- The PortArrayRef shall be a reference to a completely initialized port array.

- The index redirection shall only be used when the operation is used on an any from port array construct.

- If the index redirection is used for single-dimensional arrays, the type of the integer variable shall allow storing the highest index of the respective port array.

- If the index redirection is used for multi-dimensional arrays, the size of the integer array or record of integer type shall exactly be the same as the dimension of the respective port array, and the its type shall allow storing the highest index (from all dimensions) of the port array.

- If a variable referenced in the sender or @index clause is a lazy or fuzzy variable, the expression assigned to this variable is equal to the result produced by the check operation, i.e. later evaluation of the lazy or fuzzy variable does not lead to repeated invocation of the check operation.

- If the check operation contains both from and sender clause, the variable or parameter referenced in the sender clause shall be type compatible with the template in the from clause. If the operation contains a sender clause but no from clause, the sender shall be type compatible with the variable or parameter referenced in the sender clause.

- The ObjectReference shall be of a port type.

- The @nodefault modifier is allowed only in stand-alone check statements.

NOTE 3:	In most cases the correct usage of the check operation can be checked statically, i.e. before/during compilation.

NOTE 4:	An error due to a type mismatch may happen if the types in the receive part are not compatible to the types in the assignment part or, if the from clause is missing, but the type of the sender can be determined and it is not type compatible with the type in the sender clause.

Examples

EXAMPLE 1:	Basic check:

myPort1.check(receive(5));	// Checks for an integer message of value 5.

myPort1.check(receive(charstring:?) -> value v_myCharVar);

// Checks for a charstring message and stores the message if the message type is charstring

myPort2.check(getcall(MyProc:{5, v_myVar}) from myPartner);

// Checks for a call of MyProc at port myPort2 from myPartner

myPort2.check(getreply(MyProc:{5, v_myVar} value 20));

// Checks for a reply from procedure MyProc at myPort2 where the returned value is 20 and

// the values of the two out or inout parameters are 5 and the value of v_myVar.

myPort2.check(catch(MyProc, s_myTemplate(5, v_myVar)));

myPort2.check(getreply(MyProc1:{?, v_myVar} value *)-> value v_myReturnValue param(v_myPar1,-));

myPort.check(getcall(MyProc:{5, v_myVar}) from myPartner -> param (v_myPar1Var, v_myPar2Var));

myPort.check(getcall(MyProc:{5, v_myVar}) -> sender v_mySenderVar);

EXAMPLE 2:	Check any operation:

myPort.check;

myPort.check(from myPartner);

myPort.check(-> sender v_mySenderVar);

EXAMPLE 3:	Check on any port:

any port.check;

EXAMPLE 4:	Check on any port from port array:

type port MyPort procedure { inout MyProc }

type component MyComponent {

port MyPort p[10][10];

}

var integer v_i[2];

any from p.check(catch(MyProc, MyType:?)) -> @index value v_i;

// Checking for an incoming exception of the type MyType on any port of the port array p and

// storing the index of the port on which the matching was successful first

## 22.5	Controlling communication ports

### 22.5.0	General

TTCN-3 operations for controlling message-based and procedure-based ports are presented in table 25.

Table 25: Overview of TTCN-3 port operations

| Port operations |  |
| --- | --- |
| Statement | Associated keyword or symbol |
| Clear port | clear |
| Start port | start |
| Stop port | stop |
| Halt port | halt |
| Check the state of a port | checkstate |

### 22.5.1	The Clear port operation

The clear port operation empties incoming port queues.

Syntactical Structure

( ObjectReference | ( all port ) ) "." clear

Semantic Description

The clear operation removes the contents of the incoming queue of the specified port or of all ports of the test component performing the clear operation.

If a port queue is already empty then this operation shall have no action on that port.

Restrictions

In addition to the general static rules of TTCN-3 given in clause 5 and shown in table 16, the following restrictions apply:

- The ObjectReference shall be of a port type.

Examples

myPort.clear;	// clears port MyPort

### 22.5.2	The Start port operation

The start operation enables sending and receiving operations on the port(s).

Syntactical Structure

( ObjectReference | ( all port ) ) "." start

Semantic Description

If a port is defined as allowing receiving operations such as receive, getcall, etc., the start operation clears the incoming queue of the named port and starts listening for traffic over the port. If the port is defined to allow sending operations then the operations such as send, call, raise, etc., are also allowed to be performed at that port.

By default, all ports of a component shall be started implicitly when a component is created. The start port operation will cause unstopped ports to be restarted by removing all messages waiting in the incoming queue.

Restrictions

In addition to the general static rules of TTCN-3 given in clause 5 and shown in table 16, the following restrictions apply:

- The ObjectReference shall be of a port type.

Examples

myPort.start;	// starts myPort

### 22.5.3	The Stop port operation

The stop operation disables sending and disallow receiving operations to match at the port(s).

Syntactical Structure

( ObjectReference | ( all port ) ) "." stop

Semantic Description

If a port is defined as allowing receiving operations such as receive and getcall, the stop operation causes listening at the named port to cease. If the port is defined to allow sending operations then stop port disallows the operations such as send, call, raise, etc., to be performed.

To cease listening at the port means that all receiving operations defined before the stop operation shall be completely performed before the working of the port is suspended.

Restrictions

In addition to the general static rules of TTCN-3 given in clause 5 and shown in table 16, the following restrictions apply:

a)	The ObjectReference shall be of a port type.

Examples

myPort.receive (mw_myTemplate1) -> value v_recPDU;

// the received value is decoded, matched against
									    // MyTemplate1 and the matching value is stored
										// in the variable v_recPDU
	myPort.stop;						// No receiving operation defined following the stop
										// operation is executed (unless the port is restarted 
										// by a subsequent start operation)
	myPort.receive (mw_myTemplate2);	// This operation does not match and will block (assuming

// that no default is activated)

### 22.5.4	The Halt port operation

The halt operation is comparable to the stop operation, but allows entries being already in the queue to be processed with receiving operations.

Syntactical Structure

( ObjectReference | ( all port ) ) "." halt

Semantic Description

If a port allows receiving operations such as receive, trigger and getcall, the halt operation disallows receiving operations to succeed for messages and procedure call elements that enter the port queue after performing the halt operation at that port. Messages and procedure call elements that were already in the queue before the halt operation can still be processed with receiving operations. If the port allows sending operations then halt port immediately disallows sending operations such as send, call, raise, etc. to be performed. Subsequent halt operations have no effect on the state of the port or its queue.

NOTE 1:	The port halt operation virtually puts a marker after the last entry in the queue received when the operation is performed. Entries ahead of the marker can be processed normally. After all entries in the queue ahead of the marker have been processed, the state of the port is equivalent to the stopped state.

NOTE 2:	If a port stop operation is performed on a halted port before all entries in the queue ahead of the marker have been processed, further receive operations are disallowed immediately (i.e. the marker is virtually moved to the top of the queue).

NOTE 3:	A port start operation on a halted port clears all entries in the queue irrespectively if they arrived before or after performing the port halt operation. It also removes the marker.

NOTE 4:	A port clear operation on a halted port clears all entries in the queue irrespectively if they arrived before or after performing the port halt operation. It also virtually puts the marker at the top of the queue.

Restrictions

In addition to the general static rules of TTCN-3 given in clause 5 and shown in table 16, the following restrictions apply:

- The ObjectReference shall be of a port type.

Examples

myPort.halt;						// No sending allowed on myPort from this moment on;
										// processing of messages in the queue still possible.
	myPort.receive (mw_myTemplate1);	// If a message was already in the queue before the halt
										// operation and it matches mw_myTemplate1, it is processed;
										// otherwise the receive operation blocks.

### 22.5.5	The Checkstate port operation

The checkstate port operation allows to check the state of a port.

Syntactical Structure

( ObjectReference | ( all port ) | ( any port )) "." checkstate "(" SingleExpression ")"

Semantic Description

The checkstate port operation allows to examine the state of a port. If a port is in the state specified by the parameter, the checkstate operation returns the Boolean value true. If the port is not in the specified state, the checkstate operation returns the Boolean value false. Calling the checkstate operation with an invalid argument leads to an error.

The checkstate operation allows to check for different dimensions of a port state. It allows to check if a port is Started, Halted or Stopped, but also if a port is Connected, Mapped or Linked (i.e. Connected or Mapped).

NOTE 1:	The states Started, Halted and Stopped refer to the port states defined in the clauses F.3.1 and F.3.2. The states Connected, Mapped and Linked are related to the application of the connection operations connect, disconnect, map and unmap as defined in clause 21.1.

The checkstate port operation can be used with all port and any port. Using the checkstate operation with any port allows to test if at least one port of a test component is in the specified state. Using the checkstate operation with all port allows to check if all ports of a component are in the specified state.

Restrictions

In addition to the general static rules of TTCN-3 given in clause 5 and shown in table 16, the following restrictions apply:

a)	The parameter of the checkstate operation shall be of type charstring and shall have one of the following values:

1)	"Started"

2)	"Halted"

3)	"Stopped"

4)	"Connected"

5)	"Mapped"

6)	"Linked"

NOTE 2:	Clause E.2.2.4 includes the type definition objState and the constant definitions STARTED, HALTED, STOPPED, CONNECTED, MAPPED, and LINKED. It is recommended to use the checkstate operation in combination with this type and these constants to ease the checking of correct usage and to improve the readability of test specs.

b)	Calling the checkstate operation with a charstring parameter not listed in a) shall lead to an error.

c)	The ObjectReference shall be of a port type.

Examples

type component MyMTCType // Component type definition for an MTC

{

port MyPortType pCO1, pCO2

}

type component MyTestSystemInterface // Component type definition for a test system interface

{

port MyPortType	pCO3, pCO4, pCO5;

}

// Test case definition

testcase TC_MyTestcase1 () runs on MyMTCType system MyTestSystemInterface {

var boolean v_myPortState;

myPortState := all port.checkstate("Started");	// checkstate returns true, because all

// ports of a component are started after

// component creation and start

v_myPortState := any port.checkstate("Linked");	// checkstate returns false, no port is

// either connected nor mapped

map(mtc:pCO1, system:pCO3);

v_myPortState := pCO1.checkstate("Linked");		// checkstate returns true, pCO1 is mapped

v_myPortState := pCO1.checkstate("Mapped");		// checkstate returns true, pCO1 is mapped

v_myPortState := pCO1.checkstate("Connected");	// checkstate returns false, pCO1 is mapped

// and not connected

v_myPortState := any port.checkstate("Mapped");	// checkstate returns true, pCO1 is mapped

all port.stop;

v_myPortState := all port.checkstate("Started");// checkstate returns false, all ports

// are stopped

v_myPortState := pCO1.checkstate("Stopped");	// checkstate returns true, pCO1 is stopped

// further testcase behaviour

// …

}

## 22.6	Use of any and all with ports

The keywords any and all may be used with configuration and communication operations as indicated in table 26.

Table 26: Any and All with ports

| Operation | Allowed |  | Example |
| --- | --- | --- | --- |
|  | any | all |  |
| receive, trigger, getcall, getreply, catch, check) | yes |  | any port.receive |
| connect / map |  |  |  |
| disconnect / unmap |  | yes | unmap(self : all port) |
| start, stop, clear, halt |  | yes | all port.start |
| checkstate | yes | yes | any port.checkstate("Started") all port.checkstate("Connected") |

NOTE:	Ports are owned by test components and instantiated when a component is created. The keywords any port and all port address all ports owned by a test component and not only the ports known in the scope of the function or altstep that is executed on the component.

# 23	Timer operations

## 23.0	General

TTCN-3 supports a number of timer operations as given in table 27. These operations may be used in test cases, functions and altsteps.

Table 27: Overview of TTCN-3 timer operations

| Timer operations |  |
| --- | --- |
| Statement | Associated keyword or symbol |
| Start timer | start |
| Stop timer | stop |
| Read elapsed time | read |
| Check if timer running | running |
| Timeout event | timeout |

## 23.1	The timer mechanism

It is assumed that each test component and control component maintain their own running-timers list and timeout-list, i.e. a list of all timers that are actually running and a list of all timers that have timed out. The timeout-lists are part of the snapshots that are taken when a test case is executed. The running-timers list and timeout-list of a are updated if a timer of the component is started, is stopped, times out or the component executes a timeout operation.

NOTE 1:	The running-timers list and the timeout-list are only a conceptual lists and do not restrict the implementation of timers. Other data structures like a set, where the access to timeout events is not restricted by, e.g. the order in which the timeout events have happened, may also be used.

NOTE 2:	Conceptually, each test and control component maintain one running-timers list and one timeout-list only. However, within a given scope unit only timers known in the scope unit can be accessed individually, i.e. timers that are declared in the scope unit, passed in as parameters to the scope unit or known via a runs-on clause. In some special cases (e.g. for re-establishing a test component during a test run), it can be necessary to stop timers local to other scope units or to check if timers local to other scope units are running or have already timed out. This can be done by using the keywords all and any in combination with the timer operations stop, timeout and running. Allowed combinations are defined in clause 23.7.

When a timer expires, the timer becomes immediately inactive. A timeout event is placed in the timeout-list and the timer is removed from the running-timer list of the component for which the timer has been declared. Only one entry for any particular timer may appear in the timeout-list and running-timer list of the component for which the timer has been declared.

All running timers shall automatically be cancelled when a test component is explicitly or implicitly stopped.

## 23.2	The Start timer operation

The start timer operation is used to indicate that a timer shall start running.

Syntactical Structure

ObjectReference "." start [ "(" ObjectReference ")" ]

Semantic Description

When a timer is started, its name is added to the list of running timers (for the given scope unit).

The optional timer value parameter shall be used if no default duration is given, or if it is desired to override the default value specified in the timer declaration. When a timer duration is overridden, the new value applies only to the current instance of the timer, any later start operations for this timer, which do not specify a duration, shall use the default duration.

Starting a timer with the timer value 0.0 means that the timer times out immediately. Starting a timer with a negative timer value, e.g. the timer value is the result of an expression, or without a specified timer value shall cause a runtime error.

The timer clock runs from the float value zero (0.0) up to maximum stated by the duration parameter.

The start operation may be applied to a running timer, in which case the timer is stopped and re-started. Any entry in a timeout-list for this timer shall be removed from the timeout-list.

Restrictions

In addition to the general static rules of TTCN-3 given in clause 5 and shown in table 16, the following restrictions apply:

- The SingleExpression representing the timer value shall be a non-negative numerical float number (i.e. the value shall be greater than or equal to 0.0; infinity and not_a_number are disallowed).

- The ObjectReference shall be of the timer type.

Examples

t_myTimer1.start;	 		// t_myTimer1 is started with the default duration

t_myTimer2.start(20E-3);	// t_myTimer2 is started with a duration of 20 ms

// Elements of timer arrays may also be started in a loop, for example

timer t_myTimer [5];

var float v_timerValues [5];

for (var integer v_i := 0; v_i<=4; v_i:=v_i+1)

{ v_timerValues [v_i] := 1.0 }

for (var integer v_i := 0; v_i<=4; v_i:=v_i+1)

{t_myTimer [v_i].start ( v_timerValues [v_i])}

## 23.3	The Stop timer operation

The stop operation is used to stop a running timer.

Syntactical Structure

( ObjectReference |

all timer ) "." stop

Semantic Description

A stop operation removes a running timer from the list of running timers. A stopped timer becomes inactive and its elapsed time is set to the float value zero (0.0).

Stopping an inactive timer is a valid operation, although it does not have any effect. Stopping an expired timer causes the entry for this timer in the timeout-list to be removed.

The all keyword may be used to stop all timers that have been started on a test component or control component.

Restrictions

In addition to the general static rules of TTCN-3 given in clause 5 and shown in table 16, the following restrictions apply:

- The ObjectReference shall be of the timer type.

Examples

t_myTimer1.stop; 	// stops t_myTimer1

all timer.stop;		// stops all running timers

## 23.4	The Read timer operation

The read operation is used to retrieve the time that has elapsed since the specified timer was started.

Syntactical Structure

ObjectReference "." read

Semantic Description

The read operation returns the time that has elapsed since the specified timer was started. The returned value shall be of type float.

Applying the read operation on an inactive timer, i.e. on a timer not listed on the running-timer list, will return the float value zero (0.0).

Restrictions

In addition to the general static rules of TTCN-3 given in clause 5 and shown in table 16, the following restrictions apply:

- The ObjectReference shall be of the timer type.

Examples

var float v_myVar;

v_myVar := t_myTimer1.read; // assign to v_myVar the time that has elapsed since t_myTimer1

// was started

## 23.5	The Running timer operation

The running timer operation is used to check whether a timer is in the running-timer list.

Syntactical Structure

( ObjectReference ) | any timer | any from TimerArrayRef ) "." running

["->" @index value ValueRef]

Semantic Description

The running timer operation is used to check whether a specific timer visible in the given scope unit is listed on the running-timer list or not (i.e. that it has been started and has neither timed out nor been stopped). The operation returns the value true if the timer is listed on the list, false otherwise.

The any keyword may be used to check if any timer started on a test component or control component is running.

When the any from TimerArrayRef notation is used, where TimerArrayRef shall be a timer array identifier, the timers from the referenced array are iterated over and their states are checked individually, from innermost to outermost dimension from lowest to highest index for each dimension. The first timer to be found in the running state causes the operation returning with the true value. If no running timer is found in the array, the operation returns with the false value. The index of the first timer found running can optionally be stored in an integer variable for a single-dimensional array, or to an integer array or record of integer variable for multi-dimensional timer arrays.

Restrictions

In addition to the general static rules of TTCN-3 given in clause 5 and shown in table 16, the following restrictions apply:

- TimerArrayRef shall be a reference to a completely initialized timer array.

- The index redirection shall only be used for any from timer array running operations.

- If the index redirection is used for single-dimensional timer arrays, the type of the integer variable shall allow storing the highest index of the respective timer array.

- If the index redirection is used for multi-dimensional timer arrays, the size of the integer array or record of integer type shall exactly be the same as the dimension of the respective timer array, and its type shall allow storing the highest index (from all dimensions) of the timer array.

- The ObjectReference shall be of the timer type.

Examples

EXAMPLE 1:	Checking if a specific timer is running:

if (t_myTimer1.running) { … }

EXAMPLE 2:	Checking if an arbitrary timer is running:

if (any timer.running) { … }

EXAMPLE 3:	Checking if an arbitrary timer from a timer array is running:

timer t_myTimerArray[2][2];

var integer v_i[2];

if (any from t_myTimerArray.running -> @index value v_i;) { … }

// checks if any timer from array is running

// assigns index of matched timer to v_i

## 23.6	The Timeout operation

The timeout operation allows to check the expiration of timers.

Syntactical Structure

[ @nodefault ] ( ObjectReference | any timer | any from TimerArrayRef ) "." timeout

["->" @index value ValueRef]

Semantic Description

The timeout operation allows to check the expiration of a specific timer in the scope unit of a test component or control component in which the timeout operation has been called or of any timer that has been started on a test component or control component before entering the scope in which the timeout operation has been called.

When a timeout operation is processed, if a timer name is indicated, the timeout-list is searched according to the TTCN-3 scope rules. If there is a timeout event matching the timer name, that event is removed from the timeout-list, and the timeout operation succeeds.

The timeout can be used to determine an alternative in an alt statement or as stand-alone statement in a behaviour description. In the latter case a timeout operation is considered to be shorthand for an alt statement with the timeout operation as the only alternative. If the @nodefault modifier is placed before a stand-alone timeout operation, the implicit alt statement also contains the @nodefault modifier.

The any keyword used with the timeout operation succeeds if the timeout-list is not empty. In this case a randomly chosen timeout event is removed from the timeout-list.

When the any from TimerArrayRef notation is used, where TimerArrayRef shall be a timer array identifier, the timers from the referenced array are iterated over and individually checked for timeout from innermost to outermost dimension from lowest to highest index for each dimension. The first timer to be found in the timeout-list causes that timer to be removed from the list and the timeout operation succeeds. The index of the matched timer can be optionally stored in an integer variable for single-dimensional arrays or to an integer array or record of integer variable for multi-dimensional timer arrays.

Restrictions

In addition to the general static rules of TTCN-3 given in clause 5 and shown in table 16, the following restrictions apply:

- The timeout operation does not return any value and therefore shall not be used in an expression.

- TimerArrayRef shall be a reference to a completely initialized timer array.

- The index redirection shall only be used for any from timer array timeout operations.

- If the index redirection is used for single-dimensional timer arrays, the type of the integer variable shall allow storing the highest index of the respective timer array.

- If the index redirection is used for multi-dimensional timer arrays, the size of the integer array or record of integer type shall exactly be the same as the dimension of the respective timer array, and its type shall allow storing the highest index (from all dimensions) of the timer array.

- The ObjectReference shall be of the timer type.

- The @nodefault modifier is allowed only in stand-alone timeout statements.

Examples

EXAMPLE 1:	Timeout of a specific timer:

t_myTimer1.timeout;	// checks for the timeout of the previously started timer MyTimer1

EXAMPLE 2:	Timeout of an arbitrary timer:

any timer.timeout; // checks for the timeout of any previously started timer

EXAMPLE 3:	Timeout of a timer from a timer array:

timer t_myTimerArray[2][2];

var integer v_i[2];

any from t_myTimerArray.timeout -> @index value v_i;

// checks for the timeout of any timer from array

// assigns index of matched timer to v_i

## 23.7	Summary of use of any and all with timers

The keywords any and all may be used with timer operations as indicated in table 28.

Table 28: Any and All with Timers

| Operation | Allowed |  | Example |
| --- | --- | --- | --- |
|  | any | all |  |
| start |  |  |  |
| stop |  | yes | all timer.stop |
| read |  |  |  |
| running | yes |  | if (any timer.running) {…} |
| timeout | yes |  | any timer.timeout |

# 24	Test verdict operations

## 24.0	General

Verdict operations given in table 29 allow to set and retrieve verdicts. These operations shall only be used in test cases, altsteps and functions.

Table 29: Overview of TTCN-3 test verdict operations

| Test verdict operations |  |
| --- | --- |
| Statement | Associated keyword or symbol |
| Set local verdict | setverdict |
| Get local verdict | getverdict |

## 24.1	The Verdict mechanism

Each test component of the active configuration shall maintain its own local verdict. The local verdict is an object which is created for each test component at the time of its creation. It is used to track the individual verdict in each test component (i.e. in the MTC and in each and every PTC).

Additionally, there is a global test case verdict instantiated and handled by the test system that is updated when each test component (i.e. the MTC and each and every PTC) terminates execution (see figure 14). This verdict is not accessible to the getverdict and setverdict operations. The value of this verdict shall be returned by the test case when it terminates execution. If the returned verdict is not explicitly saved in the control component (e.g. assigned to a variable) then it is lost.

Figure 14: Illustration of the relationship between verdicts

NOTE 1:	TTCN-3 does not specify the actual mechanisms that perform the updating of the local and test case verdicts. These mechanisms are implementation dependent.

The verdict can have five different values: pass, fail, inconc, none and error, i.e. the distinguished values of the verdicttype (see clause 6.1).

NOTE 2:	inconc means an inconclusive verdict.

When a test component is instantiated, its local verdict object is created and set to the value none.

When changing the value of the local verdict (i.e. using the setverdict operation) the effect of this change shall follow the overwriting rules listed in table 30. The test case verdict is implicitly updated on the termination of a test component. The effect of this implicit operation shall also follow the overwriting rules listed in table 30.

Table 30: Overwriting rules for the verdict

| Current value of | New verdict assignment value |  |  |  |
| --- | --- | --- | --- | --- |
| Verdict | pass | inconc | fail | none |
| None | pass | inconc | fail | none |
| Pass | pass | inconc | fail | pass |
| Inconc | inconc | inconc | fail | inconc |
| Fail | fail | fail | fail | fail |

The error verdict is special in that it is set by the test system to indicate that a test case (i.e. runtime) error has occurred. It shall not be set by the setverdict operation and will not be returned by the getverdict operation. No other verdict value can override an error verdict. This means that an error verdict can only be a result of an execute test case operation.

Together with the local test verdict, each test component shall also maintain an implicit charstring variable to store information about the reasons for assigning the verdict. The implicit charstring variable shall have no effect on the overwriting rules and on the calculation of the final test case verdict. On the termination of the test component, the local verdict of the test component shall be logged together with the implicit charstring variable. The implicit charstring variable cannot be retrieved and read by any TTCN-3 function, it only provides additional information for logging.

## 24.2	The Setverdict operation

The local verdict is set with the setverdict operation.

Syntactical Structure

setverdict "(" SingleExpression { "," ( FreeText | TemplateInstance ) } ")"

Semantic Description

The value of the local verdict is changed with the setverdict operation. The effect of this change shall follow the overwriting rules listed in table 30.

The optional parameters allow to provide information that explain the reasons for assigning the verdict. This information is composed to a string and stored in an implicit charstring variable. On termination of the test component, the actual local verdict is logged together with the implicit charstring variable. Since the optional parameters can be seen as log information, the same rules and restrictions as for the parameters of the log statement (clause 19.11) apply.

As the result of the setverdict operation, the implicit charstring variable is overwritten whenever the local verdict of a test component is overwritten. A setverdict operation with a verdict only that overwrites the current local verdict, will also clear the implicit charstring variable. This means previously stored information gets lost.

Restrictions

In addition to the general static rules of TTCN-3 given in clause 5 and shown in table 16, the following restrictions apply:

a)	The setverdict operation shall only be used with the values pass, fail, inconc and none. It shall not be used to assign the value error, this is set by the test system only to indicate runtime errors.

b)	SingleExpression shall resolve to a value of type verdict.

c)	For FreeText and TemplateInstance, the same rules and restrictions apply as for the parameters of the log statement. Table 18 lists all language elements that can be used in a setverdict operation.

Examples

EXAMPLE 1:

setverdict(pass);	// the local verdict is set to pass

:

setverdict(fail);	// until this line is executed, which will result in the value

// of the local verdict being overwritten to fail

// When the ptc terminates the test case verdict is set to fail

EXAMPLE 2:

var integer v_myVar:= 1;

:

myPort.receive(integer:v_myVar);// Matches an integer value with the value of v_myVar

// at port myPort

setverdict(pass, "Value received: ", v_myVar ); // Provided the actual test component verdict is 									// none: local verdict is set to pass, the implicit

// charstring variable is set to "Value received: 5"

stop;							// The test component terminates. The local test verdict and

// implicit charstring variable are logged

## 24.3	The Getverdict operation

The value of the local verdict may be retrieved using the getverdict operation.

Syntactical Structure

getverdict

Semantic Description

The getverdict operation returns the actual value of the local verdict.

Restrictions

No specific restrictions in addition to the general static rules of TTCN-3 given in clause 5 and shown in table 16.

Examples

v_myResult := getverdict; // Where v_myResult is a variable of type verdicttype

# 25	External actions

In some testing situations some interface(s) to the SUT may be missing or unknown a priori (e.g. management interface) but it may be necessary that the SUT is stimulated to carry out certain actions (e.g. send a message to the test system). Also certain actions may be required from the test executing personnel (e.g. to change the environmental conditions of testing like the temperature, voltage of the power feeding, etc.).

The required action may be described as a string expression, i.e. the use of literal strings, string typed variables and parameters, etc. and any concatenation thereof are allowed.

Syntactical Structure

action "(" { ( FreeText | Expression ) ["&"] } ")"

Semantic Description

External actions can be used in test cases, functions and altsteps.

There is no specification of what is done to or by the SUT to trigger this action, only an informal description of the required action itself.

Restrictions

In addition to the general static rules of TTCN-3 given in clause 5 and shown in table 16, the following restrictions apply:

- Expression shall have the base type charstring or universal charstring.

Examples

var charstring v_myString:= " now."

action("Send MyTemplate on lower PCO" & v_myString);	// Informal description of the
															// external action

# 26	Module control

## 26.0	General

Test cases are defined in the module definitions part while the control functions manage their execution. The statements and operations that can be used in  control behaviour are summarized in table 31.

Table 31: Overview of TTCN-3 statements and operations in module control

| Statement | Associated keyword or symbol |
| --- | --- |
| Assignments | := |
| If-else | if (…) {…} else {…} |
| Select case | select case (…) { case (…) {…} case else {…}} |
| For loop | for (…) {…} |
| While loop | while (…) {…} |
| Do while loop | do {…} while (…) |
| Label and Goto | label / goto |
| Stop execution | stop |
| Leaving a loop, alt or interleave | break |
| Next iteration of a loop | continue |
| Logging | log |
| Alternative behaviour (see note) | alt {…} |
| Re-evaluation of alternative behaviour (see note) | repeat |
| Interleaved behaviour (see note) | interleave {…} |
| Activate a default (see note) | activate |
| Deactivate a default (see note) | deactivate |
| Start timer | start |
| Stop timer | stop |
| Read elapsed time | read |
| Check if timer running | running |
| Timeout event | timeout |
| Stimulate an (SUT) action externally | action |
| Execute test case | execute |
| NOTE: Can be used to control timer operations only. |  |  |

## 26.1	The Execute statement

Test cases are executed with an execute statement in the module control.

Syntactical Structure

execute "(" TestcaseRef "(" [ { ActualPar  [","] } ] ")" [ "," TimerValue [ "," HostId ] ] ")"

Semantic Description

The execute statement is used to start test cases (see clause 27.1) in control behaviour. The result of an executed test case is always a value of type verdicttype. Every test case shall contain one and only one MTC the type of which is referenced in the header of the test case definition. The behaviour defined in the test case body is the behaviour of the MTC.

When a test case is invoked the MTC is created, the ports of the MTC and the test system interface are instantiated and the behaviour specified in the test case definition is started on the MTC. All these actions shall be performed implicitly i.e. without explicit create and start operations.

Test case start

A test case is called using an execute statement. As the result of the execution of a test case, a test case verdict of either none, pass, inconc, fail or error shall be returned and may be assigned to a variable for further processing.

Optionally, the execute statement allows supervision of a test case by means of a timer duration.

Also optionally, the execute statement allows deployment of the MTC to a specific host before starting the execution. The host is identified by means of a host id.

Test case parameterization and configuration

All variables (if any) visible in the scope unit where the execute statement is used shall be passed into the test case by parameterization if they are to be used in the behaviour definition of that test case, i.e. TTCN-3 does not support global variables of any kind.

At the start of each test case, the test configuration shall be reset. This means that all components and ports conducted by create, connect, etc. operations in a previous test case were destroyed when that test case was stopped (hence are not "visible" to the new test case).

Test case termination

A test case terminates with the termination of the MTC. On termination of the MTC (explicitly or implicitly), all running parallel test components shall be removed by the test system.

NOTE 1:	The concrete mechanism for stopping all PTCs is tool specific and therefore outside the scope of the present document.

The final verdict of a test case is calculated based on the final local verdicts of the different test components according to the rules defined in clause 24.1. The actual local verdict of a test component becomes its final local verdict when the test component terminates itself or is stopped by itself, another test component or by the test system.

NOTE 2:	To avoid race conditions for the calculation of test verdicts due to the delayed stopping of PTCs, the MTC should ensure that all PTCs have stopped (by means of the done or killed statement) before it stops itself.

Test case timer

Timer may be used to supervise the execution of a test case. This may be done using an explicit timeout in the execute statement. If the test case does not end within this duration, the result of the test case execution shall be an error verdict and the test system shall terminate the test case. The timer used for test case supervision is a system timer and need not be declared or started.

Host id

A host id can be used to give a specific deployment location to the test system where the MTC shall be started and execute its behaviour. If a host id is provided, the execute statement shall end with a test case error if the MTC cannot be deployed on the specified host.

Restrictions

In addition to the general static rules of TTCN-3 given in clause 5 and shown in table 16, the following restrictions apply:

a)	The TimerValue shall resolve to a non-negative numerical float value (i.e. the value shall be greater or equal 0.0, infinity and not_a_number are disallowed).

b)	When the corresponding formal parameter is not of template type TemplateInstance shall resolve to an Expression.

c)	The execute statement shall not be called from within an existing executing test behaviour chain called from a test case, i.e. test cases can only be executed from a control behaviour.

d)	The HostId parameter shall resolve to a charstring value.

Examples

EXAMPLE 1:	Test case execution without keeping the test case verdict:

execute(TC_MyTestCase1());	// executes TC_MyTestCase1, without storing the

// returned test verdict and without time supervision

EXAMPLE 2:	Test case execution with keeping the test case verdict:

v_myVerdict := execute(TC_MyTestCase2());	// executes TC_MyTestCase2 and stores the resulting

// verdict in variable v_myVerdict

EXAMPLE 3:	Test case timer:

v_myVerdict := execute(TC_MyTestCase3(),5E-3);

// executes TC_MyTestCase3 and stores the resulting verdict in variable v_myVerdict.

// If the test case does not terminate within 5ms, v_myVerdict will get the value 'error'

EXAMPLE 4:	Host id:

v_myVerdict := execute(TC_MyTestCase3(), -, "Host1");

// executes TC_MyTestCase3 with unlimited time with MTC deployed to 'Host1'

## 26.2	Test suite execution

TTCN-3 test suite execution is controlled by the the module control function. The module control function defines, in which order, sequence, loop, under which preconditions, and with which parameters test cases are to be executed.

Execution and control component

The module control function is an entry point for execution of a TTCN-3 test suite. If the function contains formal parameters, their actual values shall be provided. When the control function is started, the TE creates a test component called control component. The component contains variables, constants, templates and times important for controlling of the execution of the test suite. The created control component is of the type specified in the runs on clause of the module control function. If the runs on clause is missing, an empty control component is created.

Sequence of test cases

Program statements specify such things like the order in which test cases are to be executed or the number of times a test case should run.

If no programming statements are used then, by default, the test cases are executed in the sequential order in which they appear in the module control function.

NOTE:	This does not preclude the possibility that certain tools may wish to override this default ordering to allow a user or tool to select a different execution order.

Timer operations may also be used explicitly to control test case execution.

Selection/deselection of test cases

The selection and deselection of test cases can also be used to control the execution of test cases.

There are different ways in TTCN-3 to select and deselect test cases. For example, boolean expressions may be used to select and deselect which test cases are to be executed. This includes, of course, the use of functions that return a boolean value.

Another way to execute test cases as a group is to collect them in a control function and invoke that function from the module control function.

As a test case returns a single value of type verdicttype, it is also possible to control the order of test case execution depending on the outcome of a test case. The use of the TTCN-3 verdicttype is another way to select test cases.

Examples

EXAMPLE 1:	Test case execution in a loop:

module MyTestSuite () {

:

control {

:

// Do this test 10 times

v_count:=0;

while (v_count < 10)

{	execute (TC_MySimpleTestCase1());

V_count := v_count+1;

}

}

}

EXAMPLE 2:	Test case execution controlled by a timer and a counter:

// Example of the use of the running timer operation

while (t_t1.running or v_x<10)	// Where t_t1 is a previously started timer

{	execute(TC_MyTestCase());

v_x := v_x+1;

}

// Example of the use of the start and timeout operations

timer t_t1:= 1.0;

:

execute(TC_MyTestCase1());

t_t1.start;

t_t1.timeout;	// Pause before executing the next test case

execute(TC_MyTestCase2());

EXAMPLE 3:	Selection/deselection of test cases with Boolean expressions:

module MyTestSuite () {

:

control {

:

if (f_mySelectionExpression1()) {

execute(TC_MySimpleTestCase1());

execute(TC_MySimpleTestCase2());

execute(TC_MySimpleTestCase3());

}

if (f_mySelectionExpression2()) {

execute(TC_MySimpleTestCase4());

execute(TC_MySimpleTestCase5());

execute(TC_MySimpleTestCase6());

}

:

}

}

EXAMPLE 4:	Selection/deselection of test cases with functions:

function f_myTestCaseGroup1()

{	execute(TC_MySimpleTestCase1());

execute(TC_MySimpleTestCase2());

execute(TC_MySimpleTestCase3());

}

function f_myTestCaseGroup2()

{	execute(TC_MySimpleTestCase4());

execute(TC_MySimpleTestCase5());

execute(TC_MySimpleTestCase6());

}

:

control

{	if (f_mySelectionExpression1()) { f_myTestCaseGroup1(); }

if (f_mySelectionExpression2()) { f_myTestCaseGroup2(); }

:

}

EXAMPLE 5:	Selection/deselection of test cases based on test case verdicts:

if ( execute (TC_MySimpleTestCase()) == pass )

{ execute (TC_MyGoOnTestCase()) }

else

{ execute (TC_MyErrorRecoveryTestCase()) };

# 27	Specifying attributes

## 27.0	General

TTCN-3 uses attributes to give special characterization/meaning to language elements such as specific presentation format, specific encoding and encoding variants, and user-defined properties.

## 27.1	The Attribute mechanism

### 27.1.0	General

Attributes can be associated with TTCN-3 language elements by means of the with statement. The with statement can be applied to modules, global module definitions and to local definitions in control, test cases, functions, altsteps, statement blocks and in component type definitions.

### 27.1.1	Scope of attributes

A with statement may associate attributes to a single language element or to elements or fields of structured types (in a recursive way) or to members of component or port types, the same way as specified in clauses 6.2.1.1 and 6.2.3.2. It is also possible to associate attributes to a number of language elements by, e.g. listing fields of a structured type in an attribute statement associated with a single type definition or associating a with statement to the surrounding scope unit or group of language elements.

A with statement can follow any module, any global definition inside module and group declarations as well as any local definition in component types and statement blocks inside behaviour definitions.

Attributes can be attached to synonym types (6.4). If the synonym type is a structured type, attributes in the with statement may reference fields or elements of this structured type.

EXAMPLE 1:	// attributes for single language elements and groups:

// MyPDU1 will be displayed as PDU

type record MyPDU1 { … } with { display "PDU"}

// MyPDU2 will be displayed as PDU with the application specific extension attribute MyRule

type record MyPDU2 { … }

with

{

display "PDU";

extension "MyRule"

}

// The following group definition …

group myPDUs {

type record MyPDU3 { … }

type record MyPDU4 { … }

}

with {display "PDU"}	// All types of group MyPDUs will be displayed as PDU

// is identical to

group myPDUs {

type record MyPDU3 { … } with { display "PDU"}

type record MyPDU4 { … } with { display "PDU"}

}

EXAMPLE 2:	// attributes for fields and elements:

type record MyRec {

integer field1,

record {

integer eField1,

boolean eField2

} field2

}

with { display (field2.eField1) "colour blue" }

// the embedded field eField1 is displayed blue

type record of integer MyRecOfInteger

with { display ([-]) "colour green"

// all integer elements are displayed green

type record of integer MyRecOfInteger2

with { display ([-]) "colour red" }

// integer elements are displayed red

const MyRecOfInteger c_MyRecordOfInt := {0, 1, 2, 3}

with { display ([0]) "colour blue" }

// the first element is displayed blue, the other elements are displayed green

### 27.1.2	Overwriting rules for attributes

#### 27.1.2.0	General

An attribute definition that is directly attached to a lower scope unit will override a general attribute definition in a higher scope and a type-specific attribute inherited from a type reference. Attributes inherited from a type reference will override general attributes from a higher scope unit containing the type reference. Additional overwriting rules for variant attributes are defined in clause 27.1.2.1.

EXAMPLE 1:

type record MyRecordA

{

:

} with { encode "RuleA" }

// In the following, MyRecordA is encoded according to "RuleA" and not according to

// "RuleB" because the attribute from the referenced type MyRecordA overrides

// the attribute from higher scope unit (surrounding MyRecordB type).

type record MyRecordB

{

:

MyRecordA 	field

} with { encode "RuleB" }

A with statement that is placed inside the scope of another with statement shall override the outermost with. This shall also apply to the use of the with statement with groups. If multiple attributes of the same type are allowed, all of them are overridden unless specified otherwise.

EXAMPLE 2:

// Example of the use of the overwriting scheme of the with statement

group myPDUs

{

type record MyPDU1 { … }

type record MyPDU2 { … }

group mySpecialPDUs

{

type record MyPDU3 { … }

type record MyPDU4 { … }

}

with {extension "MySpecialRule"}	// MyPDU3 and MyPDU4 will have the application

// specific extension attribute MySpecialRule

}

with

{

display "PDU";		// All types of group myPDUs will be displayed as PDU and

extension "MyRule";	// (if not overwritten) have the extension attribute MyRule

}

// is identical to …

group myPDUs

{

type record MyPDU1 { … } with {display "PDU"; extension "MyRule" }

type record MyPDU2 { … } with {display "PDU"; extension "MyRule" }

group mySpecialPDUs {

type record MyPDU3 { … } with {display "PDU"; extension "MySpecialRule" }

type record MyPDU4 { … } with {display "PDU"; extension "MySpecialRule" }

}

}

Attributes defined for a synonym type do not override existing attributes of fields or elements of this synonym type. The attributes are applied to the fields or elements of synonym types only if the fields or elements have no valid attributes.

EXAMPLE 3:

// Example of the use of attributes in synonym types

type record SourceType1 {

integer field1,

integer field2

} // neither the record nor its fields have a valid attribute

type record SourceType2 {

integer field1,

integer field2

} with { encode  "Rule1" }

// the record and its fields have a valid encode attribute "Rule1"

type SourceType1 SynonymType1 with { encode  "Rule2" }

// SynonymType1 and all its fields will be encoded with Rule2

type SourceType2 SynonymType2 with { encode  "Rule3" }

// SynonymType2 will be encoded with Rule3, but field1 and field2 will be encoded with

// Rule1 as SourceType2 definition already specifies the encode attribute of these fields

Attributes with the @local modifier override attributes from higher scope, but they are valid for the associated language element only. They do not affect definitions inside the associated language element as the @local attribute is completely transparent to lower scopes. Attributes from higher scope will still affect attributes in lower scopes even if the @local attribute is between them.

NOTE:	Attributes with the @local modifier associated to modules and groups are valid, but do not affect the definitions inside them.

EXAMPLE 4:

module M {

type record MyRec {

integer field1,

integer field1,

} with { encode @local "CodecB" }

// the record type MyRec will be encoded with CodecB, but its fields with CodecA,

// because the local attribute CodecB doesn't affect fields of the MyRec type.

} with { encode "CodecA" }

An attribute definition in a lower scope or those inherited from a referenced type can be overwritten in a higher scope by using the override directive.

EXAMPLE 5:

type record MyRecordA

{

:

} with { encode "RuleA" }

// In the following, fieldA of a MyRecordB instance is encoded according to RuleB

type record MyRecordB

{

:

MyRecordA 	fieldA

} with { encode override (fieldA) "RuleB" }

The override directive overrides the specified attribute for all declarations at all lower scopes that do not also declare the specified attribute. If the override directive is applied to a type reference, it does not affect the attributes of the original referenced type.

An attribute definition directly attached to a field or element of a structured type overrides the corresponding attribute of the structured type, as regards the identified field or element. Override attribute applied to a synonym type (clause 6.4) overrides attributes of all fields or elements of the synonym type unless the synonym type definition contains an explicit attribute definition for the field or element.

EXAMPLE 6:

// An instance of MyRecordA is encoded according to RuleA.

type record MyRecordA

{

:

} with { encode override "RuleA" }

// In the following, fieldA of a MyRecordB instance (and all its sub-fields) is encoded

// according to "RuleB".

type record MyRecordB

{

:

MyRecordA 	fieldA

} with { encode override "RuleB" }

// The following template will use "RuleA" as the override directive for MyRecordB affects only

// MyRecordB.fieldA, but not the original MyRecordA.

template MyRecordA mw_msg;

// In the following, rule "RuleB" is overridden by "RuleC" for fieldC, but it is

// not overridden by "RuleA" of the group because the direct attachment to fieldC and

// MyRecordC override the encode of the outer scope.

group myGroup {

type record MyRecordC

{

:

} with { encode override "RuleB" }

type record MyRecordD

{

:

MyRecordC 	fieldC

} with { encode override (fieldC) "RuleC" }

} with { encode override "RuleA" }

// In the following, the template mw_msg will be encoded with "RuleB", because the

// override directive doesn't override the encode attribute in references. However,

// all fields of the mw_msg template will be encoded with "RuleA", because the attributes

// from the references have higher precedence than attributes from a higher scope.

type record MyRecordE

{

:

} with { encode override "RuleA" }

template MyRecordE mw_msg :=

{

:

} with { encode "RuleB" }

// MyRecordG and its "field1" member will be encoded with "RuleB", but its field2 member

// will be encoded with "RuleA", because there's an encode attribute explicitly declared

// for this field.

type record MyRecordF {

integer field1,

integer field2

} with { encode "RuleA" }

type MyRecordF MyRecordG with {

encode override "RuleB";

encode(field2) "RuleA"

}

#### 27.1.2.1	Additional default overwriting rules for variant attributes

A variant attribute is always related to an encode attribute. Whereas a variant of an encoding may change, an encoding shall not change without overwriting all current variant attributes.

The present document defines the default rules for variant attributes. Extension packages of TTCN-3, for example specifying language mappings, may define their own overwriting rules for variant attributes. For variant attributes the following default overwriting rules apply:

a variant attribute overwrites a current variant attribute according to the rules defined in clause 27.1.2;

an encoding attribute, which overwrites a current encoding attribute according to the rules defined in clause 27.1.2, also overwrites a corresponding current variant attribute, i.e. no new variant attribute is provided, but the current variant attribute becomes inactive;

an encoding attribute, which changes a current encoding attribute of an imported language element according to the rules defined in clause 27.1.3, also changes a corresponding current variant attribute, i.e. no new variant attribute is provided, but the current variant attribute becomes inactive.

EXAMPLE:

module MyVariantEncodingModule {

:

type charstring MyType;	// Normally encoded according to "Encoding 1"

:

group myVariantsOne {

:

type record MyPDUone

{

integer		field1,	// field1 will be encoded according to "Encoding 2" only.

// "Encoding 2" overwrites "Encoding 1" and variant "Variant 1"

MyType		field3 	// field3 will be encoded according to "Encoding 1" with

// variant "Variant 1".

}

with { encode (field1) "Encoding 2" }

:

}

with { variant "Variant 1" }

group myVariantsTwo

{ 	:

type record MyPDUtwo

{

integer		field1,	// field1 will be encoded according to "Encoding 3"

// using encoding variant "Variant 3"

MyType		field3 	// field3 will be encoded according to "Encoding 1"

// using encoding variant "Variant 2"

}

with { variant (field1) "Variant 3" }

:

}

with { encode "Encoding 3"; variant "Variant 2"}

}

with { encode "Encoding 1" }

#### 27.1.2.2	Overwriting rules for multiple encoding

Explicitly listed encode attributes that occur on the higher scope and are not overwritten will retain all variants related to them.

An encoding related variant will overwrite only variants related to the same encoding.

EXAMPLE:

type integer Int with {

encode "CodecA"; variant "CodecA"."Rule1";

encode "CodecB"; variant "CodecB"."Rule2";

}

// Modifying list of allowed encodings

type Int Int2 with {

encode "CodecA"; // variant "CodecA"."Rule1" is kept

encode "CodecC"; variant "CodecC"."Rule6"; // new encoding and related variant

// "CodecB" encoding together with its variant are discarded as "CodecB" is not

// explicitly referenced

}

// Overwriting variant with an encoding reference

type Int Int3 with {

variant "CodecB"."Rule4"; // new variant for encoding "CodecB" overwrites

// the original variant "CodecB"."Rule2"

// Variant "CodecA"."Rule1" is unchanged as this definition contains no reference

// to "CodecB"

}

### 27.1.3	Changing attributes of imported language elements

In general, a language element is imported together with its attributes. In some cases these attributes may have to be changed when importing the language element, e.g. a type may be displayed in one module as ASP, then it is imported by another module where it should be displayed as PDU. For such cases it is allowed to change attributes on the import statement.

When resolving the attributes, the import statement works as an additional higher scope unit on the top of the imported module. Attributes set in the import statement are valid only within the importing module.

NOTE 1:	The import statement occurs inside an importing module and sometimes inside a group. Because of the scope rules, attributes of these scope units apply to the imported module too.

NOTE 2:	If a with statement is added to an import of a definition where a local definition also has a with statement, the local definition's attributes overwrite the attributes added to the import statement in the normal way. Thus, if the attributes of a local definition is to be changed via the import statement, the override directive needs to be used.

EXAMPLE:

import from MyModule {

type MyType

}

with { display "ASP" } 		// MyType will be displayed as ASP

import from MyModule {

group myGroup

}

with {

display "PDU";			// By default all types will be displayed as PDU

extension "MyRule"

}

## 27.2	The With statement

The with statement is used to associate attributes to TTCN-3 language elements (and sets thereof).

Syntactical Structure

with  "{"

{ ( encode | variant | display | extension | optional )

[ override | @local ]

["(" DefinitionRef | FieldReference | AllRef ")"]

[ ( FreeText | ("{" FreeText { "," FreeText } "}") ) "." ] FreeText [";"] }

"}"

Semantic Description

There are five kinds of attributes that can be associated to language elements:

a)	display: allows the specification of display attributes related to specific presentation formats;

b)	encode: allows references to specific encoding rules;

c)	variant: allows references to specific encoding variants;

d)	extension: allows the specification of user-defined attributes;

e)	optional: allows the implicit setting of optional fields in records and sets to omit.

The syntax for the argument of the with statement (i.e. the actual attributes) is defined as a free text string.

DefinitionRef and FieldReference identify a definition or field respectively which is within the module, group or definition to which the with statement is associated.

AllRef can be used to apply attributes to multiple language elements defined within the scope to which the with statement is associated. AllRef provides a flexible mechanism to select all language elements or all language elements of a certain kind defined in a given scope. Individual language elements that are not affected by an attribute can be excluded from a set of selected language elements in the except clause.

Restrictions

In addition to the general static rules of TTCN-3 given in clause 5, the following restrictions apply:

a)	DefinitionRef and FieldReference shall refer to a definition or field respectively which is within the module, group or definition to which the with statement is associated.

b)	In case multiple attributes of the same type are allowed, all of them shall be without an additional modifier (override, @local) or the modifier shall be the same for all attributes.

c)	Dot notation in the FreeText part is allowed for variant attributes only.

EXAMPLE:

type record MyService {

integer i,

float f

}

with { display "ServiceCall" } 		// MyRecord will be displayed as a ServiceCall

group G {

...

} with { encode(template all except {mw_msg1}) "Rule1" }

// with the exception of mw_msg1, all templates defined in this group will be encoded

// using the "Rule1" encoding

## 27.3	Display attributes

Display attributes allow the specification of display attributes related to specific presentation formats.

Syntactical Structure

display

Semantic Description

All TTCN-3 language elements can have display attributes to specify how particular language elements shall be displayed in, for example, a tabular format.

Special attribute strings related to the display attributes for the graphical presentation format can be found in ETSI ES 201 873-3 [i.2].

Other display attributes may be defined by the user.

NOTE:	Because user-defined attributes are not standardized, the interpretation of these attributes may differ between tools or even may not be supported.

Restrictions

In addition to the general static rules of TTCN-3 given in clause 5, the following restrictions apply:

- At most one display attribute shall be applied to each definition, each individual field reference or language element to which a with statement is associated.

EXAMPLE:

type record MyService {

integer i,

float f

}

with { display "ServiceCall" } 		// MyRecord will be displayed as a ServiceCall

## 27.4	Encoding attributes

In TTCN-3, general or particular encoding rules can be specified by using encode and variant attributes. Encoding attributes allow references to specific encoding rules.

Syntactical Structure

encode

Semantic Description

Encoding rules define how a particular value, template, etc. shall be encoded and transmitted over a communication port and how received signals shall be decoded. TTCN-3 does not have a default encoding mechanism. This means that encoding rules or encoding directives are defined in some external manner to TTCN-3.

The encode attribute allows the association of some referenced encoding rule or encoding directive to be made to a TTCN-3 definition.

The manner in which the actual encoding rules are defined (e.g. prose, functions, etc.) is outside the scope of the present document. If no specific rules are referenced then encoding shall be a matter for individual implementation.

In most cases encoding attributes will be used in a hierarchical manner. The top-level is the entire module, the next level is a group and the lowest is an individual type or definition:

a)	module: encoding applies to all types defined in the module, including TTCN-3 types (built-in types);

b)	group: encoding applies to a group of user-defined type definitions;

c)	type or definition: encoding applies to a single user-defined type or definition;

d)	field: encoding applies to a field in a record or set type or template.

The with statement may contain more than one encode attribute. In this case, multiple encodings are supported in the context where the attribute is used. The encoding used in the encoding and decoding operations can be selected dynamically by using the setencode operation (clause C.5), as a parameter of predefined codec functions (clause C.5) or inside the codec implementation.

Restrictions

No specific restrictions in addition to the general static rules of TTCN-3 given in clause 5.

EXAMPLE:

module MyFirstmodule

{	:

import from MySecondModule {

type MyRecord

}

with { encode "MyRule 1" } // Instances of MyRecord will be encoded according to MyRule 1

:

type charstring MyType;	// Normally encoded according to the "Global encoding rule"

:

group myRecords

{ 	:

type record MyPDU1

{

integer		field1,	 	// field1 will be encoded according to "Rule 3"

boolean		field2,		// field2 will be encoded according to "Rule 3"

Mytype		field3 		// field3 will be encoded according to "Rule 2"

}

with { encode (field1, field2) "Rule 3" }

:

}

with { encode "Rule 2" }

}

with { encode "Global encoding rule" }

## 27.5	Variant attributes

In TTCN-3, general or particular encoding rules can be specified by using encode and variant attributes. Variant attributes allow references to specific encoding variants.

Syntactical Structure

variant

Semantic Description

To specify a refinement of the currently specified encoding scheme instead of its replacement, the variant attribute shall be used. The variant attributes are different from other attributes, because they are closely related to encode attributes. Therefore, for variant attributes, additional overwriting rules apply (see clause 27.1.2.1).

Special variant strings:

The following strings are the predefined (standardized) variant attributes for simple basic types (see clause E.2.1):

a)	"8 bit" and "unsigned 8 bit" mean, when applied to integer and enumerated types, that the integer value or the integer numbers associated with enumerated values shall be handled as it was represented on 8-bits (single byte) within the system.

b)	"16 bit" and "unsigned 16 bit" mean, when applied to integer and enumerated types, that the integer value or the integer numbers associated with enumerated values shall be handled as it was represented on 16-bits (two bytes) within the system.

c)	"32 bit" and "unsigned 32 bit" mean, when applied to integer and enumerated types, that the integer value or the integer numbers associated with enumerated values shall be handled as it was represented on 32-bits (four bytes) within the system.

d)	"64 bit" and "unsigned 64 bit" mean, when applied to integer and enumerated types, that the integer value or the integer numbers associated with enumerated values shall be handled as it was represented on 64-bits (eight bytes) within the system.

e)	"IEEE754 float","IEEE754 double", "IEEE754 extended float" and "IEEE754 extended double" mean, when applied to a float type, that the value shall be encoded and decoded according to the standard IEEE 754 [6] (see annex E).

The following strings are the predefined (standardized) variant attributes for charstring and universal charstring (see clause E.2.2):

a)	"UTF-8" means, when applied to the universal charstring type, that the value shall be encoded and decoded according to the UCS encoding scheme UTF-8 as defined in clause 10.1 of ISO/IEC 10646 [2].

b)	"UTF-16" means, when applied to the universal charstring type, that the value shall be encoded and decoded according to the UCS encoding scheme UTF-16 as defined in clause 10.4 of ISO/IEC 10646 [2].

c)	"UTF-16LE" means, when applied to the universal charstring type, that each character of the value shall be individually encoded and decoded according to the UCS Encoding scheme UTF-16LE as defined in clause 10.3 of ISO/IEC 10646 [2].

d)	"UTF-16BE" means, when applied to the universal charstring type, that the value shall be encoded and decoded according to the UCS Encoding scheme UTF-16BE as defined in clause 10.2 of ISO/IEC 10646 [2].

e)	"UTF-32" means, when applied to the universal charstring type, that the value shall be encoded and decoded according to the UCS Encoding scheme UTF-32 as defined in clause 10.7 of ISO/IEC 10646 [2].

f)	"UTF-32LE" means, when applied to the universal charstring type, that the value shall be encoded and decoded according to the UCS Encoding scheme UTF-32LE as defined in clause 10.6 of ISO/IEC 10646 [2].

g)	"UTF-32BE" means, when applied to the universal charstring type, that the value shall be encoded and decoded according to the UCS Encoding scheme UTF-32BE as defined in clause 10.5 of ISO/IEC 10646 [2].

h)	"8 bit" means, when applied to charstring and universal charstring types, that each character of the value shall be individually encoded and decoded according to the coded representation as specified in ISO/IEC 10646 [2] (an 8-bit coding).

NOTE:	The UCS Encoding schemes allow an optional signature (also known as byte order mark, BOM) to be present in encoded character strings. The above UCS encoding scheme variant attributes does not specify, if signatures are present in the encoded values or not, this is an option for the encoder. It is expected that decoders are able to process signatures in the decoding process.

The following strings are the predefined (standardized) variant attributes for structured types (see clause E.2.2.4):

a)	"IDL:fixed FORMAL/01-12-01 v.2.6" means, when applied to a record type, that the value shall be handled as an IDL fixed point decimal value (see annex E).

These variant attributes can be used in combination with the more general encode attributes specified at a higher level. For example a universal charstring specified with the variant attribute "UTF-8" within a module which itself has a global encoding attribute "BER:1997" (see clause 12.2 of ETSI ES 201 873-7 [i.5]) will cause each character of the values within the string to first be encoded following the UTF-8 rules and then this UTF-8 value will be encoded following the more global BER rules.

Invalid encodings

If it is desired to specify invalid encoding rules then these shall be specified in a referenceable source external to the module in the same way that valid encoding rules are referenced.

Multiple encodings

If multiple encodings (see clause 27.4) are used, the variant attribute value shall be composed of two parts separated by a dot. Such variant attributes are called encoding related variant attributes. The first part of the attribute specifies the encodings the variant is related to. There are two possible notations: either a simple string when the variant is related to a single encode attribute or a comma separated list of strings enclosed in curly brackets if the variant is related to multiple encodings. The second part of the attribute (following the dot symbol) is a simple string that specifies the variant value.

The encoding related attributes are valid only when the related encoding is selected.

It is not allowed to define variant attributes with no encoding reference if multiple encodings are used.

Multiple variants

The with statement can contain any number of variant attributes.

Restrictions

In addition to the general static rules of TTCN-3 given in clause 5 , the following restrictions apply:

- When dot notation is used in the variant attribute value for an element, the strings preceding the dot symbol shall resolve into one of the encode attribute values associated with the same element.

EXAMPLE:

module MyTTCNmodule1

{	:

type charstring MyType;	// Normally encoded according to the "Global encoding rule"

:

group myRecords

{ 	:

type record MyPDU1

{

integer		field1,	 	// field1 will be encoded according to "Rule 2"

// using encoding variant "length form 3"

MyType		field3 		// field3 will be encoded according to

// "Global encoding rule" using any possible length

// encoding format

}

with { variant (field1) "length form 3" }

:

}

with { encode "Rule 2" }

type charstring Multi with {

encode "Codec1"; variant "Codec1"."Rule1";

encode "Codec2"; variant "Codec2"."Rule3";

}; 	// multiple encodings ("Codec1", "Codec2"), the variant "Rule1" is valid

// for the "Codec1" encoding only, while the variant "Rule3" applies only

// for the "Codec2" encoding

type charstring Multi2 with {

encode "Codec1";  encode "Codec2";

variant {"Codec1","Codec2"}."Rule1";

}; 	// multiple encodings ("Codec1", "Codec2"), variant "Rule1" applies to both of them

type charstring Multi3 with {

encode "Codec1";  encode "Codec2";

variant "Rule1";

} 	// the statement will produce an error as there are multiple encodings and the

// variant attribute doesn't specify encoding reference

}

with { encode "Global encoding rule" }

## 27.6	Extension attributes

Extension attributes can be used for proprietary extensions to TTCN-3. The with statement may contain any number of extension attributes.

Syntactical Structure

extension

Semantic Description

All TTCN-3 language elements can have extension attributes specified by the user.

NOTE:	Because user-defined attributes are not standardized the interpretation of these attributes between tools supplied by different vendors may differ or even not be supported.

Restrictions

No specific restrictions in addition to the general static rules of TTCN-3 given in clause 5.

Examples

testcase TC_MyTestcase() runs on MTCType {

:

}

with { extension "Test Purpose: This test case is used to check …" }

## 27.7	Optional attributes

The optional attribute can be used to indicate that optional fields of constants, module parameters, templates, variables and template variables of record and set types are implicitly set to omit.

Syntactical Structure

optional

Semantic Description

TTCN-3 constants, module parameters, templates, variables and template variables can have an optional attribute. Also, TTCN-3 language elements that contain such definitions, i.e. module, group, function, altstep, test case, control, and component type definitions can have an optional attribute. When an optional attribute is associated to a function, altstep, test case, control or component type definitions, it shall have effect on all the constants, module parameters, templates, variables and template variables declared within these definitions and not on the enframing definition itself.

Special optional strings:

The following strings are the predefined (standardized) optional attributes:

a)	"implicit omit" means that all optional fields, that have no assigned value definition in the statement on which the attribute operates, are set to omit. This applies recursively to the optional fields of the entity and to subfields of the mandatory fields.

b)	"explicit omit" means that all optional fields, that have no assigned value definition in the statement on which the attribute operates, are left undefined. This applies recursively to the optional fields of the entity and to subfields of the mandatory fields.

For variables and template variables associated with an "implicit omit" optional attribute, recursive procedure is applied to their optional fields after each assignment or usage as out or inout actual parameter in the scope of their declaration (e.g.after re-assigning parts or all of a variable's value) setting all optional fields that have no assigned value definition to omit.

NOTE:	Assigning the "implicit omit" attribute to a variable can have a negative runtime performance impact. Tool vendors are encouraged to identify and optimize particular cases where these operations are not needed (e.g. where it is possible to decide statically that no optional fields of the structure could have become undefined).

Restrictions

In addition to the general static rules of TTCN-3 given in clause 5, the following restrictions apply:

- Data type, port type, procedure signature and import statements shall not have an optional attribute associated to them directly. When an optional attribute is associated to module, group, function, altstep, test case, control or component type containing such definitions, it shall not have any effect on the included data type, port type, procedure signature or import statement.

At most one optional attribute shall be applied to each definition, each individual field reference or language element to which a with statement is associated.

EXAMPLE:

type record MyRecord1 {

integer a,

boolean b optional

}

type record MyRecord2 {

MyRecord1 m

}

// reference templates with explicitly set fields

template MyRecord1 mw_myTemplate1 := { a := ?, b := omit }

template MyRecord2 mw_myTemplate2 := { m := { a := ?, b := omit }}

// reference templates

template MyRecord1 mw_myTemplate1a := {a := ? } // b is undefined

template MyRecord1 mw_myTemplate1b := {a := ? } with {optional "explicit omit"} // b is 	undefined

template MyRecord2 mw_myTemplate2a := {} // m and its subfields are undefined

template MyRecord2 mw_myTemplate2b := { m := { a := ?}}; // m.b is undefined

// templates with attribute

template MyRecord1 mw_myTemplate11 := { a := ? } with {optional "implicit omit"}

// same as mw_myTemplate1, b is set to omit

template MyRecord2 mw_myTemplate21 := { m := { a := ?}} with {optional "implicit omit"}

// same as mw_myTemplate2, by recursive application of the attribute

template MyRecord2 mw_myTemplate22 := { m := mw_myTemplate1a } with {optional "implicit omit"}

// same as mw_myTemplate2, by recursive application of the attribute

template MyRecord2 mw_myTemplate23 := {} with {optional "implicit omit"}

// same as mw_myTemplate2a, m remains undefined

template MyRecord2 mw_myTemplate24 := { m := mw_myTemplate1b } with {optional "implicit omit"}

// same as mw_myTemplate2b, the attribute on the lower scope is not overwritten

template MyRecord2 mw_myTemplate25 := { m := MyTemplate1b }

with {optional override "implicit omit"}

// same as mw_myTemplate2, the attribute on the lower scope is overwritten

// implicitly omitted fields stay omitted after assignment

template MyRecord1 mw_myTemplate3a := mw_myTemplate1a with {optional "implicit omit"}

// same as mw_myTemplate1, b is set to omit

template MyRecord1 mw_myTemplate3b := mw_myTemplate3a;

// same as mw_myTemplate1, b is set to omit, by implicit omit attribute of mw_myTemplate3a

template MyRecord1 mw_myTemplate3c := mw_myTemplate3a with {optional "explicit omit"}

// same as mw_myTemplate1, b is set to omit, by implicit omit attribute of mw_myTemplate3a

// implicitly omitted fields stay omitted after assignment

template MyRecord1 mw_myTemplate3a := mw_myTemplate1a with {optional "implicit omit"}

// same as mw_myTemplate1, b is set to omit

template MyRecord1 mw_myTemplate3b := mw_myTemplate3a;

// same as mw_myTemplate1, b is set to omit, by implicit omit attribute of mw_myTemplate3a

template MyRecord1 mw_myTemplate3c := mw_myTemplate3a with {optional "explicit omit"}

// same as mw_myTemplate1, b is set to omit, by implicit omit attribute of mw_myTemplate3a

function f_helper1() return MyRecord1 {

var MyRecord1 v_temp := { 1, true };

return v_temp;

}

function f_helper2(out MyRecord1 p_par) {

p_par := { 1 };

// p_par is { 1, <undefined> }, no implicit omit attribute is in effect here

...

}

function f_function() {

var MyRecord2 v_var1;

v_var1.m.a := 5;

// at this time v_var1.m.b is set to omit for the "implicit omit" attribute

v_var1.m := f_helper1();

// v_var1.m.b is true, checking of v_var1 might be skipped given strong static checks

f_helper2( v_var1.m );

// at this time v_var1.m.b is set to omit for the "implicit omit" attribute

...

} with {optional "implicit omit"}

## 27.8	Retrieving attribute values

TTCN-3 provides a set of operations that can be used for retrieving attribute values associated with a type, template, variable, constant or module parameter.

Syntactical Structure

( Type | TemplateInstance ) "." ( display | encode | variant | extension | optional )

[ "(" Expression ")" ]

Semantic Description

The operation returns the actual value of an attribute associated with the type, template, variable, constant or module parameter that precedes the dot symbol. The value preceding the dot symbol may be unitialized. The attribute kind is denoted by the keyword following the dot symbol.

The return value of the operations for retrieving attribute values is of a universal charstring type in case of attributes that can be present only once (display, optional) . If such an attribute is not defined, the operation returns an empty string. If the attribute can occur multiple times (encode, variant, extension), the operation returns a record of universal charstring type. If such an attribute is not present, the operation returns an empty record of value.

The operation for getting a variant attribute value may be followed by an optional parameter. If no parameter is present, the operation returns only variants that are not bound to any particular encoding. If the parameter is present, the returned value will containt variants that are bound to the encoding referenced by the parameter.

Restrictions

In addition to the general static rules of TTCN-3 given in clause 5, the following restrictions apply:

- The optional parameter of the operation shall be used only for getting variant attributes.

- The Expression in optional parameter of the operation shall be of the universal charstring type.

- An error shall be produced if the Expression in the optional parameters is not one of the valid encode attributes.

EXAMPLE:

// MyPDU1 will be displayed as PDU

type record MyPDU1 { ... } with {

display "blue";

encode "Codec1";

variant "Codec1"."Rule1A";

variant "Codec1"."Rule1B";

encode "Codec2";

variant "Codec2"."Rule2A";

variant "Codec2"."Rule2B";

}

type record of universal charstring RUC;

control {

var MyPDU1 v_pdu;

var universal charstring v_display;

var RUC v_encoding, v_variants;

v_display := MyPDU1.display;		// v_display will contain "blue"

v_display := v_variants.display;	// v_display will contain "" as no display attribute is

// defined for v_variants

v_encoding := v_pdu.encode;			// v_encoding will contain { "Codec1", "Codec2" }

v_variants := v_pdu.variant;		// v_variants will contain {} as all variants are bound

// to encode attributes

// retrieve variants for all defined encodings

for (var integer i := 0; i < sizeof(v_encoding); i := i + 1) {

v_variants := v_pdu.variant(v_encoding[i]);

...

}

v_variants := v_variants.encode; // v_variants will contain {} as no encode attribute is

// defined for v_variants

v_variants := v_pdu.variant("UnknownCodec");	// produces an error as there is no such

// encode attribute as "UnknownCodec"

}

## 27.9	Dynamic configuration of encoding used by ports

The setencode operation can be used on a port or set of ports to dynamically select for the affected ports a single encode attribute value to be used for a type that has  multiple encode attributes attached to it.

Syntactical Structure

(SingleExpression | ( all port ) | self ) "." setencode"(" Type "," SingleExpression ")"

Semantic Description

The setencode operation dynamically restricts the number of encode attribute values of a referenced type or its fields or elements to a single value. Dependent on the language element preceding the dot, the encoding configuration is valid either for all sending and receiving operations of a single port (single port reference), sending and receiving operations of all ports of the current component (all port notation) or for all codec functions and communication operations of the current component (self keyword).

If the referenced type contains multiple encode attributes and the expression provided in the setencode operation is equal to one of these encode attribute values, the statement reduces the list of encode attributes to the selected one. The procedure is applied recursively to all elements and fields or the referenced type. After executing the operation, all other encode attributes and variants related to them are dynamically disabled and invisible to the codec.

Repeated call of the setencode operation always uses the static attributes that are valid for the referenced type. Previous calls of the setencode operation referencing the type are not considered in this case. This way it is possible to change the encoding during test execution using different encodings.

It is allowed to reference a field or element of a type using an extended type reference in the setencode operation. This operation is useful for payload fields of container protocols and allows dynamic configuration of the proper encoding for payload fields. If the extended type reference is used, following calls of the setencode operation for the whole type or any element that contains the the referenced payload field won't change the encoding that was dynamically configured for this field or element (and its sub-fields).

Restrictions

In addition to the general static rules of TTCN-3 given in clause 5, the following restrictions apply:

- If the setencode operation is applied to a single port, the referenced type shall be either listed in the in or out type list of the related port type or it shall be a reference to a field or element on any level of nesting of a type listed in the in or out type list of the related port type.

- The SingleExpression used in the second parameter of the setencode operation shall be compatible with the universal charstring type.

- If present, the SingleExpression preceding the setencode keyword shall be of a port type.

EXAMPLE:

type port P message {

inout PDU;

}

type component C {

port P p;

}

// Payload type with two encoding options

type record Payload {

...

} with { encode "PayloadCodec1"; encode "PayloadCodec2" }

// PDU type with two encoding options

type record PDU {

charstring source,

charstring destination,

Payload payload

} with { encode "PduCodec1"; encode "PduCodec2" }

template PDU m_msg := {

source := "source address",

destination := "destination address",

payload := { ... }

}

testcase TC01() runs on C {

p.setencode(PDU.payload, "PayloadCodec2");

p.setencode (PDU, "PduCodec1");

p.send(m_msg); // m_msg will be sent with its encode attribute set to "PduCodec1"

// and its payload field will have its encode attribute set to "PayloadCodec2"

}

Annex A (normative):
BNF and static semantics

# A.1	TTCN-3 BNF

## A.1.0	General

This annex defines the syntax of TTCN-3 using extended BNF (henceforth just called BNF).

## A.1.1	Conventions for the syntax description

Table A.1 defines the metanotation used to specify the extended BNF grammar for TTCN-3.

Table A.1: The syntactic metanotation

| ::= | is defined to be | definition of non-terminal |
| --- | --- | --- |
| abc xyz | abc followed by xyz | concatenation |
| ¦ | alternative | alternative |
| [abc] | 0 or 1 instances of abc | optional |
| {abc} | 0 or more instances of abc | repetition 1 |
| {abc}+ | 1 or more instances of abc | repetition 2 |
| {abc}#(n, m) | n to m instances of abc | repetition 3 |
| (...) | textual grouping | grouping |
| Abc | the non-terminal symbol abc | non-terminal |
| "abc" | a terminal symbol abc | terminal |

NOTE:	The metanotation defined in table A.1 is parsed from left to right. The metanotation operators have the following precedence, from highest (binding tightest) at the top, to lowest (loosest) at the bottom:

-	Repetition, Optional

-	Grouping

-	Concatenation

-	Alternative

-	Definition

## A.1.2	Statement terminator symbols

In general all TTCN-3 language constructs (i.e. definitions, declarations, statements and operations) are terminated with a semi-colon (;). The semi-colon is optional if the language construct ends with a right-hand curly brace (}) or the following symbol is a right-hand curly brace (}), i.e. the language construct is the last statement in a statement block.

## A.1.3	Identifiers

TTCN-3 identifiers are case sensitive and shall only contain lowercase letters (a-z) uppercase letters (A-Z), numeric digits (0-9) and the underscore ( _ ) symbol. An identifier shall begin with a letter (i.e. not with a number and not an underscore).

## A.1.4	Comments

Comments written in free text may appear anywhere in a TTCN-3 specification. Comments may contain any graphical character defined in ISO/IEC 10646 [2]. Block comments shall be opened by the symbol pair /* and closed by the symbol pair */.

EXAMPLE 1:

/* This is a block comment

spread over two lines */

Block comments shall not be nested.

/* This is not /* a legal */ comment */

Line comments shall be opened by the symbol pair // and closed by a <newline> or <end-of-file>.

EXAMPLE 2:

// This is a line comment

// spread over two lines

EXAMPLE 3:

// The following is not legal

const // This is MyConst integer c_myConst := 1;

// A block comment should have been used instead

const /* This is MyConst */ integer c_myConst := 1;

// A line comment like this works as well

const // This is MyConst

integer c_myConst := 1;

## A.1.5	TTCN-3 terminals

### A.1.5.0	General

TTCN-3 terminal symbols and reserved words are listed in tables A.2, A.3 and A.5.

Table A.2: List of TTCN-3 special terminal symbols

| Begin/end block symbols | { } |
| --- | --- |
| Begin/end list symbols | ( ) |
| Element specifier symbols | [ ] |
| Range symbol | .. |
| Line and block comments | /* */ // |
| Statement separator symbol | ; |
| Arithmetic operator symbols | + / - * |
| Concatenation operator symbol | & |
| Relational operator symbols | != == >= <= < > |
| Shift operator symbols | << >> |
| Rotate operator symbols | <@ @> |
| String enclosure symbols | " ' |
| Wildcard/matching symbols | ? * |
| Assignment symbol | := |
| Communication operation assignment | -> |
| Bitstring, hexstring and Octetstring values | B H O |
| Float exponent | E |
| List element separator symbol | , |
| Field reference | . |
| Decoded field reference | => |

The predefined function identifiers defined in table 15 and described in annex C shall also be treated as reserved words.

Table A.3: List of TTCN-3 terminals which are reserved words

| action activate address alive all alt altstep and and4b any anytype bitstring boolean break case call catch char charstring check clear complement component connect const continue control create deactivate decmatch default disconnect display do done else encode enumerated error except exception execute extends extension external | fail false float for friend from function getverdict getcall getreply goto group halt hexstring if ifpresent import in inconc infinity inout integer interleave isbound ischosen ispresent isvalue kill killed label language length log map match message mixed mod modifies module modulepar mtc | noblock none not not_a_number not4b nowait null octetstring of omit on optional or or4b out override param pass pattern permutation port present private procedure public raise read receive record recursive rem repeat reply return running runs | select self send sender set setencode setverdict signature start stop subset superset system template testcase timeout timer to trigger true type union universal unmap value valueof var variant verdicttype while with xor xor4b |
| --- | --- | --- | --- |

The TTCN-3 terminals listed in table A.3 shall not be used as identifiers in a TTCN-3 module. These terminals shall be written in all lowercase letters.

Additionally, there are special TTCN-3 terminals consisting of an @-symbol, directly followed by an identifier. These terminals shall also be written in all lowercase letters.

NOTE:	These terminals can be used in combination with the @-symbol, which results in a specific semantics for the annotated language element. They can also be used like any other identifier without any special meaning.

Table A.4: List of TTCN-3 terminals which are modifiers

| @abstract @control | @decoded @default @deterministic @fuzzy | @index @lazy @local | @nocase @nodefault |
| --- | --- | --- | --- |

Table A.5: List of TTCN-3 terminals which are reserved words in extension packages

| apply assert at configuration conjunct cont delta disjunct duration finished | history implies inv mode notinv now onentry onexit | par prev realtime seq setstate static stepsize stream | timestamp until values wait |
| --- | --- | --- | --- |

The TTCN-3 terminals listed in table A.5 are used as keywords inside the TTCN-3 extension packages. These terminals shall not be used as identifiers in a TTCN-3 module..

These terminals shall be written in all lowercase letters.

### A.1.5.1	Use of whitespaces and newlines

The elements of the TTCN-3 syntax (reserved words, identifiers, terminal symbols and literal values) shall be separated by whitespace or by special terminal symbols listed in table A.2 according to the TTCN-3 syntax.

In representing whitespace, any one or more of the following characters of the C0 set of Recommendation ITU-T T.50 [4] and of annex A of Recommendation ITU-T T.50 [4] may be used in any combination:

HT - HORIZONTAL TABULATION (9)

LF - LINE FEED (10)

VT -VERTICAL TABULATION (11)

FF - FORM FEED (12)

CR - CARRIAGE RETURN (13)

SP - SPACE (32)

The characters of the C0 set of Recommendation ITU-T T.50 [4] and of annex A of Recommendation ITU-T T.50 [4] below are denoting newline (end of line). A single CR(13) character directly followed by an LF(10) character denote a single end of line (i.e. the sequence CRLFCRLFVT denotes 3 lines):

LF - LINE FEED (10)

VT - VERTICAL TABULATION (11)

FF - FORM FEED (12)

CR - CARRIAGE RETURN (13)

Any character or character sequence that is a valid newline is also a valid whitespace.

NOTE:	It is recommended that for newline only the CR and LF and for whitespace only the HT, LF, CR and SP control characters are used as the VT and FF characters may cause problems with some conventional text editors.

## A.1.6	TTCN-3 syntax BNF productions

### A.1.6.0	TTCN-3 module

TTCN3Module ::= TTCN3ModuleKeyword ModuleId "{" [ModuleDefinitionsList]

"}" [WithStatement] [SemiColon]

TTCN3ModuleKeyword ::= "module"

ModuleId ::= Identifier [LanguageSpec]

LanguageSpec ::= LanguageKeyword FreeText {"," FreeText}

LanguageKeyword ::= "language"

### A.1.6.1	Module definitions part

#### A.1.6.1.0	General

ModuleDefinitionsList ::= {ModuleDefinition [SemiColon]}+

ModuleDefinition ::= (([Visibility] (TypeDef |

ConstDef |

TemplateDef |

ModuleParDef |

FunctionDef |

SignatureDef |

TestcaseDef |

AltstepDef |

ImportDef |

ExtFunctionDef |

ModuleControlDef

)) |

(["public"] GroupDef) |

(["private"] FriendModuleDef)

) [WithStatement]

Visibility ::= "public" |

"friend" |

"private"

#### A.1.6.1.1	Typedef definitions

TypeDef ::= TypeDefKeyword TypeDefBody

TypeDefBody ::= StructuredTypeDef | SubTypeDef

TypeDefKeyword ::= "type"

StructuredTypeDef ::= RecordDef |

UnionDef |

SetDef |

RecordOfDef |

SetOfDef |

EnumDef |

PortDef |

ComponentDef |

MapDef

MapDef ::= MapKeyword FromKeyword Type ToKeyword (Type | NestedTypeDef | MapDef)

RecordDef ::= RecordKeyword StructDefBody

RecordKeyword ::= "record"

StructDefBody ::= IdentifierOrAddr "{" [StructFieldDef

{"," StructFieldDef}]

"}"

StructFieldDef ::= (Type | NestedTypeDef) Identifier [ArrayDef] [SubTypeSpec]

[OptionalKeyword]

NestedTypeDef ::= NestedRecordDef |

NestedUnionDef |

NestedSetDef |

NestedRecordOfDef |

NestedSetOfDef |

NestedEnumDef

NestedRecordDef ::= RecordKeyword "{" [StructFieldDef {"," StructFieldDef}]

"}"

NestedUnionDef ::= UnionKeyword "{" UnionFieldDef {"," UnionFieldDef}

"}"

NestedSetDef ::= SetKeyword "{" [StructFieldDef {"," StructFieldDef}]

"}"

NestedRecordOfDef ::= RecordKeyword [StringLength] OfKeyword (Type |

NestedTypeDef)

NestedSetOfDef ::= SetKeyword [StringLength] OfKeyword (Type | NestedTypeDef)

NestedEnumDef ::= EnumKeyword "{" EnumerationList "}"

OptionalKeyword ::= "optional"

UnionDef ::= UnionKeyword UnionDefBody

UnionKeyword ::= "union"

UnionDefBody ::= IdentifierOrAddr "{" UnionFieldDef {","

UnionFieldDef}

"}"

UnionFieldDef ::= [DefaultModifier] (Type | NestedTypeDef) Identifier [ArrayDef] [SubTypeSpec]

/** STATIC SEMANTICS: at most one UnionFieldDef of UnionDefBody or NestedUnionDef shall contain a DefaultModifier */

SetDef ::= SetKeyword StructDefBody

SetKeyword ::= "set"

RecordOfDef ::= RecordKeyword [StringLength] OfKeyword StructOfDefBody

OfKeyword ::= "of"

StructOfDefBody ::= (Type | NestedTypeDef) IdentifierOrAddr

[SubTypeSpec]

SetOfDef ::= SetKeyword [StringLength] OfKeyword StructOfDefBody

EnumDef ::= EnumKeyword IdentifierOrAddr "{" EnumerationList

"}"

EnumKeyword ::= "enumerated"

EnumerationList ::= Enumeration {"," Enumeration}

Enumeration ::= Identifier ["(" IntegerValueOrRange {"," IntegerValueOrRange } ")"]

IntegerValueOrRange ::= IntegerValue [".." IntegerValue]

IntegerValue ::= [Minus] Number

SubTypeDef ::= Type IdentifierOrAddr [ArrayDef] [SubTypeSpec]

SubTypeSpec ::= AllowedValuesSpec [StringLength] | StringLength

/* STATIC SEMANTICS - AllowedValues shall be of the same type as the field being subtyped */

AllowedValuesSpec ::= "(" ((TemplateOrRange {"," TemplateOrRange}) |

CharStringMatch) ")"

TemplateOrRange ::= RangeDef |

TemplateBody |

Type

/* STATIC SEMANTICS - RangeDef production shall only be used with integer, charstring, universal charstring or float based types */

/* STATIC SEMANTICS - When subtyping charstring or universal charstring range and values shall not be mixed in the same SubTypeSpec */

RangeDef ::= Bound ".." Bound

StringLength ::= LengthKeyword "(" SingleExpression [".."(SingleExpression | InfinityKeyword) ] ")"

/* STATIC SEMANTICS - StringLength shall only be used with String types or to limit set of and record of. SingleExpression and Bound shall evaluate to non-negative integer values (in case of Bound including infinity) */

LengthKeyword ::= "length"

PortDef ::= PortKeyword PortDefBody

PortDefBody ::= Identifier PortDefAttribs

PortKeyword ::= "port"

PortDefAttribs ::= MessageAttribs |

ProcedureAttribs |

MixedAttribs

MessageAttribs ::= MessageKeyword "{" {(AddressDecl |

MessageList |

ConfigParamDef

) [SemiColon]}+ "}"

ConfigParamDef ::= MapParamDef | UnmapParamDef

MapParamDef ::= MapKeyword ParamKeyword "(" FormalValuePar {"," FormalValuePar}

")"

UnmapParamDef ::= UnmapKeyword ParamKeyword "(" FormalValuePar {","

FormalValuePar}

")"

AddressDecl ::= AddressKeyword Type

MessageList ::= Direction AllOrTypeList

Direction ::= InParKeyword |

OutParKeyword |

InOutParKeyword

MessageKeyword ::= "message"

AllOrTypeList ::= AllKeyword | TypeList

/* NOTE: The use of AllKeyword in port definitions is deprecated */

AllKeyword ::= "all"

TypeList ::= Type {"," Type}

ProcedureAttribs ::= ProcedureKeyword "{" {(AddressDecl |

ProcedureList |

ConfigParamDef

) [SemiColon]}+ "}"

ProcedureKeyword ::= "procedure"

ProcedureList ::= Direction AllOrSignatureList

AllOrSignatureList ::= AllKeyword | SignatureList

SignatureList ::= Signature {"," Signature}

MixedAttribs ::= MixedKeyword "{" {(AddressDecl |

MixedList |

ConfigParamDef

) [SemiColon]}+ "}"

MixedKeyword ::= "mixed"

MixedList ::= Direction ProcOrTypeList

ProcOrTypeList ::= AllKeyword | (ProcOrType {"," ProcOrType})

ProcOrType ::= Signature | Type

ComponentDef ::= ComponentKeyword Identifier [ExtendsKeyword ComponentType

{"," ComponentType}] "{"

[ComponentDefList] "}"

ComponentKeyword ::= "component"

ExtendsKeyword ::= "extends"

ComponentType ::= ExtendedIdentifier

ComponentDefList ::= {ComponentElementDef [WithStatement] [SemiColon]}

ComponentElementDef ::= PortInstance |

VarInstance |

TimerInstance |

ConstDef |

TemplateDef

PortInstance ::= PortKeyword ExtendedIdentifier PortElement {"," PortElement}

PortElement ::= Identifier [ArrayDef]

#### A.1.6.1.2	Constant definitions

ConstDef ::= ConstKeyword Type ConstList

ConstList ::= SingleConstDef {"," SingleConstDef}

SingleConstDef ::= Identifier [ArrayDef] AssignmentChar ConstantExpression

ConstKeyword ::= "const"

#### A.1.6.1.3	Template definitions

TemplateDef ::= TemplateKeyword [TemplateRestriction] [FuzzyModifier [DeterministicModifier]]

[AbstractModifier] BaseTemplate [DerivedDef] AssignmentChar BaseTemplateBody

BaseTemplate ::= (Type | Signature) Identifier ["(" TemplateOrValueFormalParList

")"]

TemplateKeyword ::= "template"

DerivedDef ::= ModifiesKeyword ExtendedIdentifier

ModifiesKeyword ::= "modifies"

TemplateOrValueFormalParList ::= TemplateOrValueFormalPar {"," TemplateOrValueFormalPar}

TemplateOrValueFormalPar ::= FormalValuePar | FormalTemplatePar

/* STATIC SEMANTICS - FormalValuePar shall resolve to an in parameter */

TemplateBody ::= DerivedTemplateBody | BaseTemplateBody

BaseTemplateBody ::= (SimpleSpec |

FieldSpecList |

ArrayValueOrAttrib

) [ExtraMatchingAttributes]

/* STATIC SEMANTICS - Within BaseTeplateBody the ArrayValueOrAttrib can be used for array, record, record of and set of types. */

SimpleSpec ::= (SingleExpression ["&" SimpleTemplateSpec]) | SimpleTemplateSpec

SimpleTemplateSpec ::= SingleTemplateExpression ["&" SimpleSpec]

SingleTemplateExpression ::= MatchingSymbol |

({TemplateRefWithParList [ExtendedFieldReference]) |

ExtendedIdentifier EnumTemplateExtension

/** STATIC Semantics: ExtendedIdentifier shall refer to an enumerated value with associated value */

EnumTemplateExtension ::= "(" (BaseTemplateBody | Range) {"," (BaseTemplateBody | Range) } ")"

/** STATIC Semantics: each TemplateBody shall be an integer template template and the limits of each Range  an integer value*/

FieldSpecList ::= "{" FieldSpec {"," FieldSpec} "}"

FieldSpec ::= FieldReference AssignmentChar (TemplateBody | Minus)

FieldReference ::= StructFieldRef |

IndexRef |

ParRef

StructFieldRef ::= Identifier |

PredefinedType |

TypeReference

/* STATIC SEMANTICS - PredefinedType and TypeReference shall be used for anytype value notation only. PredefinedType shall not be AnyTypeKeyword.*/

ParRef ::= Identifier

/* STATIC SEMANTICS - Identifier in ParRef shall be a formal parameter identifier from the associated signature definition */

IndexRef ::= "[" SingleExpression "]"

/* STATIC SEMANTICS - IndexRef shall be optionally used for array types and TTCN-3 record of, set of and map types. The same notation can be used for a string element reference inside an TTCN-3 charstring, universal charstring, bitstring, octetstring and hexstring type */

/* STATIC SEMANTICS - SingleExpression will resolve to a value of integer type */

ArrayValueOrAttrib ::= "{" [ArrayElementSpecList] "}"

ArrayElementSpecList ::= ArrayElementSpec {"," ArrayElementSpec}

ArrayElementSpec ::= Minus |

PermutationMatch |

TemplateBody

MatchingSymbol ::= Complement |

(AnyValue [WildcardLengthMatch]) |

(AnyOrOmit [WildcardLengthMatch]) |

ListOfTemplates |

Range |

BitStringMatch |

HexStringMatch |

OctetStringMatch |

CharStringMatch |

SubsetMatch |

SupersetMatch |

DecodedContentMatch

DecodedContentMatch ::= DecodedMatchKeyword ["(" [Expression] ")"] TemplateInstance

DecodedMatchKeyword ::= "decmatch"

/* STATIC SEMANTIC – WildcardLengthMatch shall be used when MatchingSymbol is used in fractions of a concatenated string or list (see clause 15.11) and shall not be used in other cases. In this case, the Complement, ListOfTemplates, Range, BitStringMatch, HexStringMatch, OctetStringMatch, CharStringMatch, SubsetMatch and SupersetMatch productions shall not be used. */

ExtraMatchingAttributes ::= StringLength |

IfPresentKeyword |

(StringLength IfPresentKeyword)

BitStringMatch ::= "'" {BinOrMatch} "'" "B"

BinOrMatch ::= Bin |

AnyValue |

AnyOrOmit

HexStringMatch ::= "'" {HexOrMatch} "'" "H"

HexOrMatch ::= Hex |

AnyValue |

AnyOrOmit

OctetStringMatch ::= "'" {OctOrMatch} "'" "O"

OctOrMatch ::= Oct |

AnyValue |

AnyOrOmit

CharStringMatch ::= PatternKeyword [CaseInsenModifier] PatternParticle {"&" PatternParticle}

PatternParticle ::= Pattern | ReferencedValue

PatternKeyword ::= "pattern"

Pattern ::= """ {PatternElement} """

PatternElement ::= (("\" ("?" | "*" | "\" | "[" | "]" | "{" | "}" |

""" | "|" | "(" | ")" | "#" | "+" | "d" |

"w" | "t" | "n" | "r" | "s" | "b"

)) | ("?" | "*" | "\" | "|" | "+"

) | ("[" ["^"] [{PatternClassChar ["-"

PatternClassChar]}]

"]") |

("{" ["\"] ReferencedValue "}") | ("\" "N" "{"

(ReferencedValue |

Type) "}") |

(""" """) |

("(" PatternElement ")") |

("#" (Num |

("(" Number "," [Number] ")") |

("(" "," Number ")") |

("(" [","] ")") Num ")"

))

) | PatternChar

PatternChar ::= NonSpecialPatternChar | PatternQuadruple

/* STATIC SEMANTICS: Characters "?", "*", "\", "[", "]", "{", "}", """, "|", "(", ")", "#", "+", "d", "^", "N" have special semantics – they are metacharacters for the definition of pattern elements – only if they follow the BNF as defined above, if not they are interpreted like normal characters */

NonSpecialPatternChar ::= Char

PatternClassChar ::= NonSpecialPatternClassChar |

PatternQuadruple |

"\" EscapedPatternClassChar

NonSpecialPatternClassChar ::= Char

/* STATIC SEMANTICS: Characters "[", "-", "^", "]", "\", "q", ","have special semantics – they are metacharacters for the definition of pattern class characters – only if they follow the BNF as defined above, if not they are interpreted like normal characters */

EscapedPatternClassChar ::= "[" | "-" | "^" | "]"

PatternQuadruple ::= "\" "q" "(" Number "," Number "," Number ","

Number ")"

Complement ::= ComplementKeyword ListOfTemplates

ComplementKeyword ::= "complement"

ListOfTemplates ::= "(" TemplateListItem {"," TemplateListItem} ")"

TemplateListItem ::= TemplateBody | AllElementsFrom

AllElementsFrom ::= AllKeyword FromKeyword TemplateBody

SubsetMatch ::= SubsetKeyword ListOfTemplates

SubsetKeyword ::= "subset"

SupersetMatch ::= SupersetKeyword ListOfTemplates

SupersetKeyword ::= "superset"

PermutationMatch ::= PermutationKeyword ListOfTemplates

/* STATIC SEMANTICS: Restrictions on the content of TemplateBody within the ListOfTemplates are given in clause B.1.3.3. */

PermutationKeyword ::= "permutation"

AnyValue ::= "?"

AnyOrOmit ::= "*"

WildcardLengthMatch ::= LengthKeyword "(" SingleExpression ")"

/* STATIC SEMANTICS: SingleExpression shall evaluate to type integer */

IfPresentKeyword ::= "ifpresent"

PresentKeyword ::= "present"

Range ::= "(" Bound ".." Bound ")"

Bound ::= (["!"] SingleExpression) | ([Minus] InfinityKeyword)

/* STATIC SEMANTICS - Bounds shall evaluate to types integer, charstring, universal charstring or float. In case they evaluate to types charstring or universal charstring, the string length shall be 1. infinity as lower bound and –infinity as upper bound are allowed for float types only. */

InfinityKeyword ::= "infinity"

ActualParAssignment ::= Identifier ":=" TemplateInstance

/* STATIC SEMANTICS – if a value parameter is used, an in-line template shall evaluate to a value */ TemplateRefWithParList ::= ExtendedIdentifier [ActualParList]

TemplateInstance ::= [(Type | Signature) Colon]

TemplateBody

DerivedTemplateBody ::= ModifiesKeyword BaseTemplateBody AssignmentChar BaseTemplateBody

ActualParList ::= "(" [(ActualPar {"," ActualPar })

{"," ActualParAssignment} |

(ActualParAssignment {"," ActualParAssignment})]

")"

ActualPar ::= TemplateInstance | Minus

/* STATIC SEMANTICS - When the corresponding formal parameter is not of template type the TemplateInstance production shall resolve to one or more SingleExpressions */

TemplateOps ::= MatchOp | ValueofOp

MatchOp ::= MatchKeyword "(" Expression "," TemplateInstance ")"

MatchKeyword ::= "match"

ValueofOp ::= ValueofKeyword "(" TemplateInstance")"

ValueofKeyword ::= "valueof"

#### A.1.6.1.4	Function definitions

FunctionDef ::= FunctionKeyword [ DeterministicModifier | ControlModifier ]

IdentifierOrControl

"(" [FunctionFormalParList] ")" [RunsOnSpec] [MtcSpec]

[SystemSpec] [ReturnType] StatementBlock

FunctionKeyword ::= "function"

FunctionFormalParList ::= FunctionFormalPar {"," FunctionFormalPar}

FunctionFormalPar ::= FormalValuePar |

FormalTemplatePar

ReturnType ::= ReturnKeyword [TemplateModifier]

Type [ArrayDef]

ReturnKeyword ::= "return"

RunsOnSpec ::= RunsKeyword OnKeyword ComponentType

RunsKeyword ::= "runs"

OnKeyword ::= "on"

MtcSpec ::= MTCKeyword ComponentType

MTCKeyword ::= "mtc"

StatementBlock ::= "{" [FunctionDefOrStatementList] "}"

FunctionDefOrStatementList ::= {( FunctionDef | FunctionStatement) [SemiColon]}+

FunctionDef ::= (FunctionLocalDef | FunctionLocalInst) [WithStatement]

FunctionLocalInst ::= VarInstance | TimerInstance

FunctionLocalDef ::= ConstDef | TemplateDef

FunctionStatement ::= ConfigurationStatements |

TimerStatements |

CommunicationStatements |

BasicStatements |

BehaviourStatements |

SetLocalVerdict |

SUTStatements |

TestcaseOperation

FunctionInstance ::= FunctionRef [ "(" [ActualParList] ")" ]

/* STATIC SEMANTICS – the part is only optional if the FunctionRef uses the ControlKeyword and the referenced control function has no formal parameters */

FunctionRef ::= [Identifier Dot] (Identifier | PreDefFunctionIdentifier | ControlKeyword )

PreDefFunctionIdentifier ::= Identifier [CaseInsenModifier]

/* STATIC SEMANTICS - The Identifier shall be one of the pre-definedpredefined TTCN-3 function identifiers from Annex C of ES 201 873-1. CaseInsenModifier shall be present only if Identifier is "regexp". */

/* STATIC SEMANTICS – if a value parameter is used, an in-line template shall evaluate to a value */

#### A.1.6.1.5	Signature definitions

SignatureDef ::= SignatureKeyword Identifier "(" [SignatureFormalParList]

")" [ReturnType | NoBlockKeyword] [ExceptionSpec]

SignatureKeyword ::= "signature"

SignatureFormalParList ::= FormalValuePar {"," FormalValuePar}

ExceptionSpec ::= ExceptionKeyword "(" TypeList ")"

ExceptionKeyword ::= "exception"

Signature ::= ExtendedIdentifier

NoBlockKeyword ::= "noblock"

#### A.1.6.1.6	Testcase definitions

TestcaseDef ::= TestcaseKeyword Identifier "(" [TemplateOrValueFormalParList]

")" ConfigSpec StatementBlock

TestcaseKeyword ::= "testcase"

ConfigSpec ::= RunsOnSpec [SystemSpec]

SystemSpec ::= SystemKeyword ComponentType

SystemKeyword ::= "system"

TestcaseInstance ::= ExecuteKeyword "(" ExtendedIdentifier "(" [ActualParList]

")" ["," (Expression | Minus) ["," SingleExpression]]

")"

ExecuteKeyword ::= "execute"

#### A.1.6.1.7	Altstep definitions

AltstepDef ::= AltstepKeyword [ ControlModifier ] [ InterleavedKeyword ] Identifier

"(" [FunctionFormalParList]  ")" [RunsOnSpec] [MtcSpec] [SystemSpec]

"{" AltstepLocalDefList AltGuardList "}"

AltstepKeyword ::= "altstep"

AltstepLocalDefList ::= {AltstepLocalDef [WithStatement] [SemiColon]}

AltstepLocalDef ::= VarInstance |

TimerInstance |

ConstDef |

TemplateDef

AltstepInstance ::= ExtendedIdentifier "(" [ActualParList]

")"

#### A.1.6.1.8	Import definitions

ImportDef ::= ImportKeyword ImportFromSpec [PortRedirectSymbol Identifier]

(AllWithExcepts | ("{" ImportSpec "}"))

ImportKeyword ::= "import"

AllWithExcepts ::= AllKeyword [ExceptsDef]

ExceptsDef ::= ExceptKeyword "{" ExceptSpec "}"

ExceptKeyword ::= "except"

ExceptSpec ::= {ExceptElement [SemiColon]}

ExceptElement ::= ExceptGroupSpec |

ExceptTypeDefSpec |

ExceptTemplateSpec |

ExceptConstSpec |

ExceptTestcaseSpec |

ExceptAltstepSpec |

ExceptFunctionSpec |

ExceptSignatureSpec |

ExceptModuleParSpec

ExceptGroupSpec ::= GroupKeyword (QualifiedIdentifierList | AllKeyword)

IdentifierListOrAll ::= IdentifierList | AllKeyword

TypeIdListOrAll ::= TypeIdentifierList | AllKeyword

FuncIdListOrAll ::= FuncIdentifierList | AllKeyword

ExceptTypeDefSpec ::= TypeDefKeyword TypeIdListOrAll

ExceptTemplateSpec ::= TemplateKeyword IdentifierListOrAll

ExceptConstSpec ::= ConstKeyword IdentifierListOrAll

ExceptTestcaseSpec ::= TestcaseKeyword IdentifierListOrAll

ExceptAltstepSpec ::= AltstepKeyword IdentifierListOrAll

ExceptFunctionSpec ::= FunctionKeyword FuncIdListOrAll

ExceptSignatureSpec ::= SignatureKeyword IdentifierListOrAll

ExceptModuleParSpec ::= ModuleParKeyword IdentifierListOrAll

ImportSpec ::= {ImportElement [SemiColon]}

ImportElement ::= ImportGroupSpec |

ImportTypeDefSpec |

ImportTemplateSpec |

ImportConstSpec |

ImportTestcaseSpec |

ImportAltstepSpec |

ImportFunctionSpec |

ImportSignatureSpec |

ImportModuleParSpec |

ImportImportSpec

ImportFromSpec ::= FromKeyword ModuleId

ImportGroupSpec ::= GroupKeyword (GroupRefListWithExcept | AllGroupsWithExcept)

GroupRefListWithExcept ::= QualifiedIdentifierWithExcept {"," QualifiedIdentifierWithExcept}

AllGroupsWithExcept ::= AllKeyword [ExceptKeyword QualifiedIdentifierList]

QualifiedIdentifierWithExcept ::= QualifiedIdentifier [ExceptsDef]

IdentifierListOrAllWithExcept ::= IdentifierList | AllWithExcept

TypeIdListOrAllWithExcept ::= TypeIdentifierList | AllTypesExcept

FuncIdListOrAllWithExcept ::= FuncIdentifierList | AllFunctionsExcept

ImportTypeDefSpec ::= TypeDefKeyword TypeIdListOrAllWithExcept

AllWithExcept ::= AllKeyword [ExceptKeyword IdentifierList]

AllTypesExcept ::= AllKeyword [ExceptKeyword TypeIdentifierList]

AllFunctionsExcept ::= AllKeyword [ExceptKeyword FuncIdentifierList]

ImportTemplateSpec ::= TemplateKeyword IdentifierListOrAllWithExcept

ImportConstSpec ::= ConstKeyword IdentifierListOrAllWithExcept

ImportAltstepSpec ::= AltstepKeyword IdentifierListOrAllWithExcept

ImportTestcaseSpec ::= TestcaseKeyword IdentifierListOrAllWithExcept

ImportFunctionSpec ::= FunctionKeyword FuncIdListOrAllWithExcept

ImportSignatureSpec ::= SignatureKeyword IdentifierListOrAllWithExcept

ImportModuleParSpec ::= ModuleParKeyword IdentifierListOrAllWithExcept

ImportImportSpec ::= ImportKeyword AllKeyword

TypeIdentifierList ::= IdentifierOrAddr {"," IdentifierOrAddr }

IdentifierOrAddr ::= Identifier | AddressKeyword

FuncIdentifierList ::= IdentifierOrControl {"," IdentifierOrControl }

IdentifierOrControl ::= Identifier | ControlKeyword

#### A.1.6.1.9	Group definitions

GroupDef ::= GroupKeyword Identifier "{" [ModuleDefinitionsList] "}"

GroupKeyword ::= "group"

#### A.1.6.1.10	External function definitions

ExtFunctionDef ::= ExtKeyword FunctionKeyword [DeterministicModifier | ControlModifier]

Identifier "(" [FunctionFormalParList] ")" [ReturnType]

ExtKeyword ::= "external"

#### A.1.6.1.11	Void

#### A.1.6.1.12	Module parameter definitions

ModuleParDef ::= ModuleParKeyword (ModulePar | ("{" MultitypedModuleParList

"}"))

ModuleParKeyword ::= "modulepar"

MultitypedModuleParList ::= {ModulePar [SemiColon]}

ModulePar ::= [ TemplateModifier ] Type ModuleParList

ModuleParList ::= Identifier [AssignmentChar TemplateBody] {","

Identifier [AssignmentChar TemplateBody]}

#### A.1.6.1.13	Friend module definitions

FriendModuleDef ::= "friend" "module" IdentifierList [SemiColon]

### A.1.6.2	Module control function

ModuleControlDef ::= ControlKeyword StatementBlock

ControlKeyword ::= "control"

### A.1.6.3	Local definitions

#### A.1.6.3.1	Variable instantiation

VarInstance ::= VarKeyword (([(LazyModifier | FuzzyModifier) [DeterministicModifier] ]

Type VarList) | ( TemplateModifier

[(LazyModifier | FuzzyModifier) ) [DeterministicModifier] ]

Type TempVarList) )

VarList ::= SingleVarInstance {"," SingleVarInstance}

SingleVarInstance ::= Identifier [ArrayDef] [AssignmentChar Expression]

VarKeyword ::= "var"

TempVarList ::= SingleTempVarInstance {"," SingleTempVarInstance}

SingleTempVarInstance ::= Identifier [ArrayDef] [AssignmentChar TemplateBody]

ValueRef ::= Identifier [ExtendedFieldReference]

#### A.1.6.3.2	Timer instantiation

TimerInstance ::= TimerKeyword VarList

TimerKeyword ::= "timer"

ArrayIdentifierRef ::= Identifier {IndexRef}

### A.1.6.4	Operations

#### A.1.6.4.1	Component operations

ConfigurationStatements ::= ConnectStatement |

MapStatement |

DisconnectStatement |

UnmapStatement |

[ NoDefaultModifier ] DoneStatement |

[ NoDefaultModifier ] KilledStatement |

StartTCStatement |

StopTCStatement |

KillTCStatement |

SetEncodeStatement

ConfigurationOps ::= CreateOp |

SelfOp |

SystemKeyword |

MTCKeyword |

RunningOp |

AliveOp

CreateOp ::= ComponentType Dot CreateKeyword ["(" (SingleExpression |

Minus) ["," SingleExpression] ")"] [AliveKeyword]

SelfOp ::= "self"

DoneStatement ::= ComponentOrAny Dot DoneKeyword [ PortRedirectSymbol

[ ValueStoreSpec ] [ IndexSpec ] ]

/*STATIC SEMANTICS – If PortRedirectSymbol is present, at least one of ValueStoreSpec and IndexSpec shall be present*/

ComponentOrAny ::= ObjectReference |

(AnyKeyword (ComponentKeyword | FromKeyword ValueRef)) |

(AllKeyword ComponentKeyword)

ValueStoreSpec ::= ValueKeyword ValueRef

IndexAssignment ::= PortRedirectSymbol IndexSpec

IndexSpec ::= IndexModifier ValueStoreSpec

KilledStatement ::= ComponentOrAny Dot KilledKeyword [ PortRedirectSymbol

[ ValueStoreSpec ] [ IndexSpec] ]

/*STATIC SEMANTICS – If PortRedirectSymbol is present, at least one of ValueStoreSpec and IndexSpec shall be present*/

DoneKeyword ::= "done"

KilledKeyword ::= "killed"

RunningOp ::= ComponentOrAny Dot RunningKeyword [IndexAssignment]

RunningKeyword ::= "running"

AliveOp ::= ComponentOrAny Dot AliveKeyword [IndexAssignment]

CreateKeyword ::= "create"

AliveKeyword ::= "alive"

ConnectStatement ::= ConnectKeyword SingleConnectionSpec

ConnectKeyword ::= "connect"

SingleConnectionSpec ::= "(" PortRef "," PortRef ")"

PortRef ::= ComponentRef Colon ArrayIdentifierRef

ComponentRef ::= ObjectReference |

SystemKeyword |

SelfOp |

MTCKeyword

DisconnectStatement ::= DisconnectKeyword [SingleConnectionSpec |

AllConnectionsSpec |

AllPortsSpec |

AllCompsAllPortsSpec

]

AllConnectionsSpec ::= "(" PortRef ")"

AllPortsSpec ::= "(" ComponentRef ":" AllKeyword PortKeyword ")"

AllCompsAllPortsSpec ::= "(" AllKeyword ComponentKeyword ":" AllKeyword

PortKeyword ")"

DisconnectKeyword ::= "disconnect"

MapStatement ::= MapKeyword SingleConnectionSpec [ParamClause]

ParamClause ::= ParamKeyword ActualParList

MapKeyword ::= "map"

UnmapStatement ::= UnmapKeyword [SingleConnectionSpec [ParamClause] |

AllConnectionsSpec [ParamClause] |

AllPortsSpec |

AllCompsAllPortsSpec |

"(" ValueRef "," SingleExpression ")"

]

UnmapKeyword ::= "unmap"

StartTCStatement ::= ObjectReference Dot StartKeyword

"(" (FunctionInstance | AltstepInstance)  ")"

StartKeyword ::= "start"

StopTCStatement ::= StopKeyword | (ComponentReferenceOrLiteral | AllKeyword

ComponentKeyword) Dot StopKeyword

ComponentReferenceOrLiteral ::= ObjectReference |

MTCKeyword |

SelfOp

KillTCStatement ::= KillKeyword | ((ComponentReferenceOrLiteral |

AllKeyword ComponentKeyword) Dot KillKeyword)

ObjectReference ::= ValueRef | FunctionInstance

KillKeyword ::= "kill"

SetEncodeStatement ::= ( SingleExpression | ( AllKeyword PortKeyword ) | SelfOp ) | "."
							SetEncodeKeyword "(" Type "," SingleExpression ")"

SetEncodeKeyword ::= "setencode"

#### A.1.6.4.2	Port operations

CommunicationStatements ::= SendStatement |

CallStatement |

ReplyStatement |

RaiseStatement |

[ NoDefaultModifier ] ReceivingStatement |

ClearStatement |

StartStatement |

StopStatement |

HaltStatement |

CheckStateStatement

ReceivingStatement ::= ReceiveStatement |

TriggerStatement |

GetCallStatement |

GetReplyStatement |

CatchStatement |

CheckStatement

SendStatement ::= ObjectReference Dot PortSendOp

PortSendOp ::= SendOpKeyword "(" TemplateInstance")" [ToClause]

SendOpKeyword ::= "send"

ToClause ::= ToKeyword (TemplateInstance|

AddressRefList |

AllKeyword ComponentKeyword

)

AddressRefList ::= "(" TemplateInstance{"," TemplateInstance} ")"

ToKeyword ::= "to"

CallStatement ::= ObjectReference Dot PortCallOp [PortCallBody]

PortCallOp ::= CallOpKeyword "(" CallParameters ")" [ToClause]

CallOpKeyword ::= "call"

CallParameters ::= TemplateInstance ["," CallTimerValue]

CallTimerValue ::= Expression | NowaitKeyword

NowaitKeyword ::= "nowait"

PortCallBody ::= "{" CallBodyStatementList "}"

CallBodyStatementList ::= {CallBodyStatement [SemiColon]}+

CallBodyStatement ::= CallBodyGuard StatementBlock

CallBodyGuard ::= AltGuardChar CallBodyOps

CallBodyOps ::= GetReplyStatement | CatchStatement

ReplyStatement ::= ObjectReference Dot PortReplyOp

PortReplyOp ::= ReplyKeyword "(" TemplateInstance [ReplyValue] ")" [ToClause]

ReplyKeyword ::= "reply"

ReplyValue ::= ValueKeyword TemplateBody

/* STATIC SEMANTICS - TemplateBody shall be type compatible with the return type. It shall evaluate to a value or template (literal or template instance)  conforming to the template(value) restriction. */

RaiseStatement ::= ObjectReference Dot PortRaiseOp

PortRaiseOp ::= RaiseKeyword "(" Signature "," TemplateInstance")"

[ToClause]

NoDefaultModifier ::= "@nodefault"

RaiseKeyword ::= "raise"

ReceiveStatement ::= PortOrAny Dot PortReceiveOp

PortOrAny ::= ObjectReference | (AnyKeyword (PortKeyword | FromKeyword ValueRef))

PortReceiveOp ::= ReceiveOpKeyword ["("TemplateInstance")"] [FromClause] [PortRedirect]

ReceiveOpKeyword ::= "receive"

FromClause ::= FromKeyword (TemplateInstance |

AddressRefList |

AnyKeyword ComponentKeyword

)

FromKeyword ::= "from"

PortRedirect ::= PortRedirectSymbol ((ValueSpec [SenderSpec] [IndexSpec]) |

(SenderSpec [IndexSpec]) |

IndexSpec

)

PortRedirectSymbol ::= "->"

ValueSpec ::= ValueKeyword (ValueRef | ("(" SingleValueSpec {"," SingleValueSpec} ")"))

SingleValueSpec ::= ValueRef [AssignmentChar [ DecodedModifier ["(" [Expression] ")"] ]

FieldReference ExtendedFieldReference]

/*STATIC SEMANTICS – FieldReference shall not be ParRef and ExtendedFieldReference shall not be TypeDefIdentifier*/

ValueKeyword ::= "value"

SenderSpec ::= SenderKeyword ValueRef

SenderKeyword ::= "sender"

TriggerStatement ::= PortOrAny Dot PortTriggerOp

PortTriggerOp ::= TriggerOpKeyword ["(" TemplateInstance  ")"] [FromClause]

[PortRedirect]

TriggerOpKeyword ::= "trigger"

GetCallStatement ::= PortOrAny Dot PortGetCallOp

PortGetCallOp ::= GetCallOpKeyword ["(" TemplateInstance ")"] [FromClause]

[PortRedirectWithParam]

GetCallOpKeyword ::= "getcall"

PortRedirectWithParam ::= PortRedirectSymbol RedirectWithParamSpec

RedirectWithParamSpec ::= (ParamSpec [SenderSpec] [IndexSpec]) |

(SenderSpec [IndexSpec]) |

IndexSpec

ParamSpec ::= ParamKeyword ParamAssignmentList

ParamKeyword ::= "param"

ParamAssignmentList ::= "(" (AssignmentList | VariableList) ")"

AssignmentList ::= VariableAssignment {"," VariableAssignment}

VariableAssignment ::= ValueRef AssignmentChar [ DecodedModifier ["(" Expression] ")"]

Identifier

VariableList ::= VariableEntry {"," VariableEntry}

VariableEntry ::= ValueRef | Minus

GetReplyStatement ::= PortOrAny Dot PortGetReplyOp

PortGetReplyOp ::= GetReplyOpKeyword ["(" TemplateInstance [ValueMatchSpec]

")"] [FromClause] [PortRedirectWithValueAndParam]

PortRedirectWithValueAndParam ::= PortRedirectSymbol RedirectWithValueAndParamSpec

RedirectWithValueAndParamSpec ::= (ValueSpec [ParamSpec] [SenderSpec]

[IndexSpec]) | RedirectWithParamSpec

GetReplyOpKeyword ::= "getreply"

ValueMatchSpec ::= ValueKeyword TemplateInstance

CheckStatement ::= PortOrAny Dot PortCheckOp

PortCheckOp ::= CheckOpKeyword ["(" CheckParameter ")"]

CheckOpKeyword ::= "check"

CheckParameter ::= CheckPortOpsPresent |

FromClausePresent |

RedirectPresent

FromClausePresent ::= FromClause [PortRedirectSymbol ((SenderSpec

[IndexSpec]) |

IndexSpec)]

RedirectPresent ::= PortRedirectSymbol ((SenderSpec [IndexSpec]) |

IndexSpec)

CheckPortOpsPresent ::= PortReceiveOp |

PortGetCallOp |

PortGetReplyOp |

PortCatchOp

CatchStatement ::= PortOrAny Dot PortCatchOp

PortCatchOp ::= CatchOpKeyword ["(" CatchOpParameter ")"] [FromClause] [PortRedirect]

CatchOpKeyword ::= "catch"

CatchOpParameter ::= Signature [ "," TemplateInstance ] | TimeoutKeyword

ClearStatement ::= PortOrAll Dot ClearOpKeyword

PortOrAll ::= ObjectReference | AllKeyword PortKeyword

ClearOpKeyword ::= "clear"

StartStatement ::= PortOrAll Dot StartKeyword

StopStatement ::= PortOrAll Dot StopKeyword

StopKeyword ::= "stop"

HaltStatement ::= PortOrAll Dot HaltKeyword

HaltKeyword ::= "halt"

AnyKeyword ::= "any"

CheckStateStatement ::= PortOrAllAny Dot CheckStateKeyword "(" SingleExpression

")"

PortOrAllAny ::= PortOrAll | AnyKeyword PortKeyword

CheckStateKeyword ::= "checkstate"

#### A.1.6.4.3	Timer operations

TimerStatements ::= StartTimerStatement |

StopTimerStatement |

[ NoDefaultModifier ] TimeoutStatement

TimerOps ::= ReadTimerOp | RunningTimerOp

StartTimerStatement ::= ObjectReference Dot StartKeyword ["(" Expression ")"]

StopTimerStatement ::= TimerRefOrAll Dot StopKeyword

TimerRefOrAll ::= ObjectReference | AllKeyword TimerKeyword

ReadTimerOp ::= ObjectReference Dot ReadKeyword

ReadKeyword ::= "read"

RunningTimerOp ::= TimerRefOrAny Dot RunningKeyword [IndexAssignment]

TimeoutStatement ::= TimerRefOrAny Dot TimeoutKeyword [IndexAssignment]

TimerRefOrAny ::= ObjectReference |

(AnyKeyword TimerKeyword) |

(AnyKeyword FromKeyword Identifier)

TimeoutKeyword ::= "timeout"

#### A.1.6.4.4	Testcase operation

TestcaseOperation ::= TestcaseKeyword "." StopKeyword ["(" { LogItem [","] } ")"]

### A.1.6.5	Type

Type ::= PredefinedType | ReferencedType

PredefinedType ::= BitStringKeyword |

BooleanKeyword |

CharStringKeyword |

UniversalCharString |

IntegerKeyword |

OctetStringKeyword |

HexStringKeyword |

VerdictTypeKeyword |

FloatKeyword |

AddressKeyword |

DefaultKeyword |

AnyTypeKeyword |

TimerKeyword

BitStringKeyword ::= "bitstring"

BooleanKeyword ::= "boolean"

IntegerKeyword ::= "integer"

OctetStringKeyword ::= "octetstring"

HexStringKeyword ::= "hexstring"

VerdictTypeKeyword ::= "verdicttype"

FloatKeyword ::= "float"

AddressKeyword ::= "address"

DefaultKeyword ::= "default"

AnyTypeKeyword ::= "anytype"

CharStringKeyword ::= "charstring"

UniversalCharString ::= UniversalKeyword CharStringKeyword

UniversalKeyword ::= "universal"

ReferencedType ::= ExtendedIdentifier [ExtendedTypeFieldReference]

TypeReference ::= ExtendedIdentifier

ArrayDef ::= {"[" SingleExpression [".." SingleExpression] "]"}+

ExtendedTypeFieldReference ::= {(Dot (Identifier | PredefinedType | FromKeyword | ToKeyword)) |

("[" Minus "]") }+

/* STATIC SEMANTICS - ArrayBounds will resolve to a non negative value of integer type */

### A.1.6.6	Value

Value ::= PredefinedValue | ReferencedValue

PredefinedValue ::= Bstring |

BooleanValue |

CharStringValue |

Number | /* IntegerValue */

Ostring |

Hstring |

VerdictTypeValue |

FloatValue |

AddressValue |

OmitKeyword

BooleanValue ::= "true" | "false"

VerdictTypeValue ::= "pass" |

"fail" |

"inconc" |

"none" |

"error"

CharStringValue ::= Cstring | Quadruple | USIlikeNotation

Quadruple ::= CharKeyword "(" Number "," Number "," Number "," Number ")"

USIlikeNotation ::= CharKeyword "(" UIDlike { "," UIDlike } ")"

UIDlike ::= ("U"|"u") {"+"} {Hex}#(1,8)

CharKeyword ::= "char"

FloatValue ::= FloatDotNotation |

FloatENotation |

NaNKeyword

NaNKeyword ::= "not_a_number"

FloatDotNotation ::= Number Dot DecimalNumber

FloatENotation ::= Number [Dot DecimalNumber] Exponential [Minus] Number

Exponential ::= "E"

ReferencedValue ::= (ExtendedIdentifier [ExtendedFieldReference] ) | ReferencedEnumValue

/* STATIC SEMANTICS – The second option is used only for referencing enumerated values, in all other cases, the first option is used.

ReferencedEnumValue ::= [ReferencedType Dot] Identifier [ExtendedEnumReference]

/** STATIC Semantics: ExtendedEnumReference shall be present if and only if Identifier refers to an enumerated value with an attached value list */

ExtendedEnumReference ::= "(" IntegerValue ")"

Number ::= (NonZeroNum {Num}) | "0"

NonZeroNum ::= "1" | "2" | "3" | "4" | "5" | "6" | "7" | "8" | "9"

DecimalNumber ::= { Num }+

Num ::= "0" | NonZeroNum

Bstring ::= "'" { Bin | BinSpace } "'" "B"

Bin ::= "0" | "1"

Hstring ::= "'" { Hex | BinSpace } "'" "H"

Hex ::= Num | "A" | "B" | "C" | "D" | "E" | "F" | "a" | "b" | "c" |

"d" | "e" | "f"

Ostring ::= "'" { Oct | BinSpace } "'" "O"

Oct ::= Hex Hex

Cstring ::= """ {Char} """

Char ::= /* REFERENCE - A character defined by the relevant CharacterString type. For charstring a character from the character set defined in ITU-T T.50. For universal charstring a character from any character set defined in ISO/IEC 10646 */

Identifier ::= Alpha {AlphaNum | Underscore}

Alpha ::= UpperAlpha | LowerAlpha

AlphaNum ::= Alpha | Num

UpperAlpha ::= "A" | "B" | "C" | "D" | "E" | "F" | "G" | "H" | "I" |

"J" | "K" | "L" | "M" | "N" | "O" | "P" | "Q" | "R" |

"S" | "T" | "U" | "V" | "W" | "X" | "Y" | "Z"

LowerAlpha ::= "a" | "b" | "c" | "d" | "e" | "f" | "g" | "h" | "i" |

"j" | "k" | "l" | "m" | "n" | "o" | "p" | "q" | "r" |

"s" | "t" | "u" | "v" | "w" | "x" | "y" | "z"

ExtendedAlphaNum ::= /* REFERENCE - A graphical character from the BASIC LATIN or from the LATIN-1 SUPPLEMENT character sets defined in ISO/IEC 10646 (characters from char (0,0,0,32) to char (0,0,0,126), from char (0,0,0,161) to char (0,0,0,172) and from char (0,0,0,174) to char (0,0,0,255) */

FreeText ::= """ {ExtendedAlphaNum} """

AddressValue ::= "null"

OmitKeyword ::= "omit"

BinSpace ::=  " " | "\" NLChar

NLChar ::= /* REFERENCE - Any sequence of newline characters that constitute a newline by using the following C0 control characters: LF(10), VT(11), FF(12), CR(13) (see Recommendation ITU-T T.50 [4]) (jointly called newline characters, see clause A.1.5.1) from the character set defined in Recommendation ITU-T T.50 [4].*/

### A.1.6.7	Parameterization

InParKeyword ::= "in"

OutParKeyword ::= "out"

InOutParKeyword ::= "inout"

FormalValuePar ::= [InParKeyword | InOutParKeyword | OutParKeyword )]

[(LazyModifier | FuzzyModifier) [DeterministicModifier] ]

Type Identifier [ArrayDef] [":=" (Expression | Minus)]

FormalTemplatePar ::= [(InParKeyword | OutParKeyword | InOutParKeyword)] TemplateModifier

[(LazyModifier | FuzzyModifier) [DeterministicModifier] ]

Type Identifier [ArrayDef] [":=" (TemplateInstance | Minus) ]

TemplateModifier ::= TemplateKeyword | RestrictedTemplate

RestrictedTemplate ::= OmitKeyword | (TemplateKeyword TemplateRestriction)

TemplateRestriction ::= "(" (OmitKeyword |

ValueKeyword |

PresentKeyword

) ")"

### A.1.6.8	Statements

#### A.1.6.8.1	With statement

WithStatement ::= WithKeyword WithAttribList

WithKeyword ::= "with"

WithAttribList ::= "{" MultiWithAttrib "}"

MultiWithAttrib ::= {SingleWithAttrib [SemiColon]}

SingleWithAttrib ::= 	StandardAttribute |

VariantAttribute

StandardAttribute ::= AttribKeyword [OverrideKeyword | LocalModifier] [AttribQualifier]

FreeText

VariantAttribute ::= VariantKeyword [( OverrideKeyword | LocalModifier )]

[AttribQualifier] [ RelatedEncoding "." ] FreeText

RelatedEncoding ::= FreeText | ( "{" FreeText { "," FreeText } "}" )

AttribKeyword ::= EncodeKeyword |

DisplayKeyword |

ExtensionKeyword |

OptionalKeyword

EncodeKeyword ::= "encode"

VariantKeyword ::= "variant"

DisplayKeyword ::= "display"

ExtensionKeyword ::= "extension"

OverrideKeyword ::= "override"

LocalModifier ::= "@local"

AttribQualifier ::= "(" DefOrFieldRefList ")"

DefOrFieldRefList ::= DefOrFieldRef {"," DefOrFieldRef}

DefOrFieldRef ::= QualifiedIdentifier |

((FieldReference | "[" Minus "]") [ExtendedFieldOrTypeReference]) |

AllRef

QualifiedIdentifier ::= {Identifier Dot} Identifier

ExtendedFieldOrTypeReference ::= {(Dot (Identifier | PredefinedType)) |

IndexRef | ("[" Minus "]") }+

/* STATIC SEMANTIC - The Identifier refers to a type definition if the type of the VarInstance or ReferencedValue in which the ExtendedFieldOrReference is used is anytype. IndexRef shall be used when referencing elements of values or arrays. The square brackets with dash shall be used when referencing inner types of a record of, set of or array type. */

AllRef ::= (GroupKeyword AllKeyword [ExceptKeyword "{" QualifiedIdentifierList

"}"]) | ((TypeDefKeyword |

TemplateKeyword |

ConstKeyword |

AltstepKeyword |

TestcaseKeyword |

FunctionKeyword |

SignatureKeyword |

ModuleParKeyword

) AllKeyword [ExceptKeyword

"{" IdentifierList

"}"])

#### A.1.6.8.2	Behaviour statements

BehaviourStatements ::= TestcaseInstance |

FunctionInstance |

ReturnStatement |

AltConstruct |

InterleavedConstruct |

LabelStatement |

GotoStatement |

RepeatStatement |

DeactivateStatement |

AltstepInstance |

ActivateOp |

BreakStatement |

ContinueStatement

SetLocalVerdict ::= SetVerdictKeyword "(" SingleExpression {"," LogItem}

")"

SetVerdictKeyword ::= "setverdict"

GetLocalVerdict ::= "getverdict"

SUTStatements ::= ActionKeyword "(" ActionText {StringOp ActionText}

")"

ActionKeyword ::= "action"

ActionText ::= FreeText | Expression

ReturnStatement ::= ReturnKeyword [TemplateInstance]

/* STATIC SEMANTICS - TemplateInstance shall evaluate to a value of a type compatible with the return type for functions returning a value. It shall evaluate to a value, template (literal or template instance), or a matching mechanism compatible with the return type for functions returning a template. */

AltConstruct ::= AltKeyword "{" AltGuardList "}"

AltKeyword ::= "alt"

AltGuardList ::= {GuardStatement | ElseStatement [SemiColon]}

GuardStatement ::= AltGuardChar (AltstepInstance [StatementBlock] |

GuardOp StatementBlock)

ElseStatement ::= "[" ElseKeyword "]" StatementBlock

AltGuardChar ::= "[" [BooleanExpression] "]"

GuardOp ::= TimeoutStatement |

ReceiveStatement |

TriggerStatement |

GetCallStatement |

CatchStatement |

CheckStatement |

GetReplyStatement |

DoneStatement |

KilledStatement

InterleavedConstruct ::= InterleavedKeyword "{" InterleavedGuardList

"}"

InterleavedKeyword ::= "interleave"

InterleavedGuardList ::= {InterleavedGuardElement [SemiColon]}+

InterleavedGuardElement ::= InterleavedGuard StatementBlock

InterleavedGuard ::= "[" "]" GuardOp

LabelStatement ::= LabelKeyword Identifier

LabelKeyword ::= "label"

GotoStatement ::= GotoKeyword Identifier

GotoKeyword ::= "goto"

RepeatStatement ::= "repeat"

ActivateOp ::= ActivateKeyword "(" AltstepInstance ")"

ActivateKeyword ::= "activate"

DeactivateStatement ::= DeactivateKeyword ["(" ObjectReference ")"]

DeactivateKeyword ::= "deactivate"

BreakStatement ::= "break"

ContinueStatement ::= "continue"

#### A.1.6.8.3	Basic statements

BasicStatements ::= Assignment |

LogStatement |

LoopConstruct |

ConditionalConstruct |

SelectCaseConstruct |

StatementBlock

Expression ::= SingleExpression | CompoundExpression

CompoundExpression ::= FieldExpressionList | ArrayOrMixedExpression

/* STATIC SEMANTICS - Within CompoundExpression the ArrayOrMixedExpression can be used for Arrays, record, record of, set and set of types. */

FieldExpressionList ::= "{" FieldExpressionSpec {"," FieldExpressionSpec}

"}"

FieldExpressionSpec ::= FieldReference AssignmentChar NotUsedOrExpression

ArrayOrMixedExpression ::= "{" [ArrayElementExpressionList {"," FieldExpressionSpec}] "}"

ArrayElementExpressionList ::= NotUsedOrExpression {"," NotUsedOrExpression}

NotUsedOrExpression ::= Expression | Minus

ConstantExpression ::= SingleExpression | CompoundConstExpression

BooleanExpression ::= SingleExpression

/* STATIC SEMANTICS - BooleanExpression shall resolve to a Value of type Boolean */

CompoundConstExpression ::= FieldConstExpressionList | ArrayConstExpression

/* STATIC SEMANTICS - Within CompoundConstExpression the ArrayConstExpression can be used for arrays, record, record of and set of types. */

FieldConstExpressionList ::= "{" FieldConstExpressionSpec {"," FieldConstExpressionSpec} "}"

FieldConstExpressionSpec ::= FieldReference AssignmentChar ConstantExpression

ArrayConstExpression ::= "{" [ArrayElementConstExpressionList] "}"

ArrayElementConstExpressionList ::= ConstantExpression {"," ConstantExpression}

Assignment ::= ValueRef AssignmentChar  TemplateBody

/* STATIC SEMANTICS - The Templatebody on the right hand side of Assignment shall evaluate to an explicit value of a type compatible with the type of the left hand side for value variables and shall evaluate to an explicit value, template (literal or a template instance) or a matching mechanism compatible with the type of the left hand side for template variables. */

SingleExpression ::= XorExpression {"or" XorExpression}

/* STATIC SEMANTICS - If more than one XorExpression exists, then the XorExpressions shall evaluate to specific values of compatible types */

XorExpression ::= AndExpression {"xor" AndExpression}

/* STATIC SEMANTICS - If more than one AndExpression exists, then the AndExpressions shall evaluate to specific values of compatible types */

AndExpression ::= NotExpression {"and" NotExpression}

/* STATIC SEMANTICS - If more than one NotExpression exists, then the NotExpressions shall evaluate to specific values of compatible types */

NotExpression ::= ["not"] EqualExpression

/* STATIC SEMANTICS - Operands of the not operator shall be of type boolean or derivatives of type Boolean. */

EqualExpression ::= RelExpression {EqualOp RelExpression}

/* STATIC SEMANTICS - If more than one RelExpression exists, then the RelExpressions shall evaluate to specific values of compatible types. If only one RelExpression exists, it shall not derive to a CompoundExpression. */

RelExpression ::= ShiftExpression [RelOp ShiftExpression] | CompoundExpression

/* STATIC SEMANTICS - If both ShiftExpressions exist, then each ShiftExpression shall evaluate to a specific integer, Enumerated or float Value or derivatives of these types */

ShiftExpression ::= BitOrExpression {ShiftOp BitOrExpression}

/* STATIC SEMANTICS - Each Result shall resolve to a specific Value. If more than one Result exists the right-hand operand shall be of type integer or derivatives and if the shift op is "<<" or ">>" then the left-hand operand shall resolve to either bitstring, hexstring or octetstring type or derivatives of these types. If the shift op is " */

BitOrExpression ::= BitXorExpression {"or4b" BitXorExpression}

/* STATIC SEMANTICS - If more than one BitXorExpression exists, then the BitXorExpressions shall evaluate to specific values of compatible types */

BitXorExpression ::= BitAndExpression {"xor4b" BitAndExpression}

/* STATIC SEMANTICS - If more than one BitAndExpression exists, then the BitAndExpressions shall evaluate to specific values of compatible types */

BitAndExpression ::= BitNotExpression {"and4b" BitNotExpression}

/* STATIC SEMANTICS - If more than one BitNotExpression exists, then the BitNotExpressions shall evaluate to specific values of compatible types */

BitNotExpression ::= ["not4b"] AddExpression

/* STATIC SEMANTICS - If the not4b operator exists, the operand shall be of type bitstring, octetstring or hexstring or derivatives of these types. */

AddExpression ::= MulExpression {AddOp MulExpression}

/* STATIC SEMANTICS - Each MulExpression shall resolve to a specific Value. If more than one MulExpression exists and the AddOp resolves to StringOp then the MulExpressions shall be valid operands for StringOp. If more than one MulExpression exists and the AddOp does not resolve to StringOp then the MulExpression shall both resolve to type integer or float or derivatives of these types. If only one MulExpression exists, it shall not derive to a CompoundExpression. */

MulExpression ::= UnaryExpression {MultiplyOp UnaryExpression} | CompoundExpression

/* STATIC SEMANTICS - Each UnaryExpression shall resolve to a specific Value. If more than one UnaryExpression exists then the UnaryExpressions shall resolve to type integer or float or derivatives of these types. */

UnaryExpression ::= [UnaryOp] Primary

/* STATIC SEMANTICS - Primary shall resolve to a specific Value of type integer or float or derivatives of these types.*/

Primary ::= OpCall |

PresenceCheckingOp |

Value |

"(" SingleExpression ")"

ExtendedFieldReference ::= {(Dot (Identifier | PredefinedType | FromKeyword | ToKeyword)) |

IndexRef |

DecodedFieldReference

}+

/* STATIC SEMANTIC - The Identifier refers to a type definition if the type of the VarInstance or ReferencedValue in which the ExtendedFieldReference is used is anytype. IndexRef shall be used when referencing elements of values or arrays. DecodedFieldReference shall not appear on the LHS of assignments and in type references*/

DecodedFieldReference ::= "=>" DecodedFieldType

DecodedFieldType ::= PredefinedType |

Identifier |

"(" Type [ "," Expression ] ")"

/* The Identifier shall resolve into a type */

OpCall ::= ConfigurationOps |

GetLocalVerdict |

TimerOps |

TestcaseInstance |

(FunctionInstance [ExtendedFieldReference]) |

(TemplateOps [ExtendedFieldReference]) |

ActivateOp |

GetAttributeOp

PresenceCheckingOp := ("ispresent" | "ischosen" | "isvalue" | "isbound")

"(" TemplateInstance ")"

AddOp ::= "+" |

"-" |

StringOp

/* STATIC SEMANTICS - Operands of the "+" or "-" operators shall be of type integer or float or derivations of integer or float (i.e. subrange) */

MultiplyOp ::= "*" | "/" | "mod" | "rem"

/* STATIC SEMANTICS - Operands of the "*", "/", rem or mod operators shall be of type integer or float or derivations of integer or float (i.e. subrange) */

UnaryOp ::= "+" | "-"

/* STATIC SEMANTICS - Operands of the "+" or "-" operators shall be of type integer or float or derivations of integer or float (i.e. subrange) */

RelOp ::= "<" | ">" | ">=" | "<="

/* STATIC SEMANTICS - the precedence of the operators is defined in Table 6 */

EqualOp ::= "==" | "!="

StringOp ::= "&"

/* STATIC SEMANTICS - Operands of the list operator shall be bitstring, hexstring, octetstring, (universal) character string, record of, set of, or array types, or derivates of these types */

ShiftOp ::= "<<" | ">>" | "<@" | "@>"

LogStatement ::= LogKeyword "(" LogItem {"," LogItem} ")"

LogKeyword ::= "log"

LogItem ::= FreeText | TemplateInstance

LoopConstruct ::= ForStatement |

WhileStatement |

DoWhileStatement

ForStatement ::= ForKeyword "(" Initial SemiColon BooleanExpression

SemiColon Assignment ")" StatementBlock

ForKeyword ::= "for"

Initial ::= VarInstance | Assignment

WhileStatement ::= WhileKeyword "(" BooleanExpression ")" StatementBlock

WhileKeyword ::= "while"

DoWhileStatement ::= DoKeyword StatementBlock WhileKeyword "(" BooleanExpression

")"

DoKeyword ::= "do"

ConditionalConstruct ::= IfKeyword "(" BooleanExpression ")" StatementBlock

{ElseIfClause} [ElseClause]

IfKeyword ::= "if"

ElseIfClause ::= ElseKeyword IfKeyword "(" BooleanExpression ")" StatementBlock

ElseKeyword ::= "else"

ElseClause ::= ElseKeyword StatementBlock

SelectCaseConstruct ::= SelectKeyword [UnionKeyword] "(" SingleExpression ")" SelectCaseBody

SelectKeyword ::= "select"

SelectCaseBody ::= "{" {SelectCase}+ [CaseElse] "}"

SelectCase ::= CaseKeyword ("("TemplateInstance {"," TemplateInstance}

")" | ElseKeyword) StatementBlock

/** STATIC SEMANTICS TemplateInstance-s shall be Identifier-s if the UnionKeyword is present in the surrounding SelectCaseConstruct (see clause 19.3.2)*/

CaseElse ::= CaseKeyword ElseKeyword StatementBlock

CaseKeyword ::= "case"

ExtendedIdentifier ::= [Identifier Dot] Identifier

IdentifierList ::= Identifier {"," Identifier}

QualifiedIdentifierList ::= QualifiedIdentifier {"," QualifiedIdentifier}

GetAttributeOp ::= (Type | TemplateInstance) "." GetAttributeSpec

GetAttributeSpec ::= EncodeKeyword |

VariantKeyword ["(" FreeText ")"] |

DisplayKeyword |

ExtensionKeyword |

OptionalKeyword

### A.1.6.9	Miscellaneous productions

Dot ::= "."

Minus ::= "-"

SemiColon ::= ";"

Colon ::= ":"

Underscore ::= "_"

AssignmentChar ::= ":="

IndexModifier ::= "@index"

DeterministicModifier ::= "@deterministic"

LazyModifier ::= "@lazy"

FuzzyModifier ::= "@fuzzy"

CaseInsenModifier ::= "@nocase"

DecodedModifier ::= "@decoded"

DefaultModifier ::= "@default"

ControlModifier ::= "@control"

AbstractModifier ::= "@abstract"

Annex B (normative):
Matching values

# B.1	Template matching mechanisms

## B.1.0	General

This annex specifies the matching mechanisms that may be used in TTCN-3 templates (and only in templates).

## B.1.1	Matching specific values

Specific values are the basic matching mechanism of TTCN-3 templates. Specific values in templates are expressions which do not contain any matching mechanisms or wildcards.

Unless otherwise specified, a template field matches the corresponding field value if, and only if, the field value has exactly the same value as the value to which the expression in the template evaluates.

Restrictions

No specific restrictions in addition to the general static rules of TTCN-3 given in clause 5.

Examples

// Given the message type definition

type record MyMessageType

{

integer 	field1,

charstring 	field2,

boolean 	field3 optional,

integer 	field4[4]

}

// A message template using specific values could be

template MyMessageType m_myTemplate:=

{

field1 := 3+2,			// specific value of integer type

field2 := "My string", 	// specific value of charstring type

field3 := true,			// specific value of boolean type

field4 := {1,2,3,4}		// specific value of integer array

}

## B.1.2	Matching mechanisms instead of values

### B.1.2.0	General

The following matching mechanisms may be used in place of explicit values.

### B.1.2.1	Template list

A template list denotes  a list of acceptable values. It can be used for values of all types. A template list may contain values, templates obeying the template (present) restriction (see clause 15.8), and members added by all from clauses. An all from clause comprises all elements of an existing record of or set of template into the template list.

A template field that uses a template list matches the corresponding field if, and only if, the field value matches any one of the values or templates in the template list, after resolving all all from clauses. Each value or template in the template list shall be of the type declared for the template field in which this mechanism is used.

Restrictions

a)	The type of the template list and the member type of the template in the all from clause shall be compatible.

b)	The template in the all from clause shall be either a specific value or combined template. Elements of the combined template may contain any of the matching mechanisms or matching attributes with the exception of those described in the following restriction.

c)	Individual elements of the combined template in the all from clause shall not resolve to any of the following matching mechanisms: AnyElementsOrNone, permutation.

d)	Each value or template in the template list shall be of the type declared for the template field in which this mechanism is used.

e)	Templates in the template list shall obey the template (present) restriction (see clause 15.8).

Examples

EXAMPLE 1:

template MyMessageType mw_myTemplate:=

{

field1 := (2,4,6),				    // list of integer values

field2 := ("String1", "String2"),	// list of charstring values

:

:

}

EXAMPLE 2:

type record of integer RoI;

template RoI mw_roI1 := {1, 2, (6..9)};

template RoI mw_roI2 := {1, *, 3};

template integer mw_i1 := (all from mw_roI1, 100);

// results in (1, 2, (6..9), 100)

template integer mw_i2 := (0, all from mw_roI2);

// causes an error because mw_roI2 contains AnyElementsOrNone

template RoI mw_roI3 := (all from mw_roI1);

// causes an error because member type of mw_roI1 (integer)

// is not compatible with the list template type (RoI)

template RoI mw_roI4 := ?;

template RoI mw_roI5 := (all from mw_roI4);

// causes an error, because mw_roI4 as a whole resolves into a matching mechanism

### B.1.2.2	Complemented template list

The keyword complement denotes a list of values that will not be accepted as values (i.e. it is the complement of a template list). It can be used on all values of all types. A complemented value list may also contain templates obeying the present template restriction (see clause 15.8).

A template field that uses complement matches the corresponding field if and only if the corresponding field's value does not match any of the values or templates listed in the template list. The template list may be a single value, of course.

Besides specifying individual values, it is possible to add all elements of an existing record of or set of template into a complement template list using an all from clause.

Restrictions

a)	The type of the complemented template list and the member type of the template in the all from clause shall be compatible.

b)	The template in the all from clause shall resolve either into a specific value or combined template. Elements of the combined template may contain any of the matching mechanisms or matching attributes with the exception of those described in the following restriction.

c)	Individual elements of the combined template in the all from clause shall not resolve to any of the following matching mechanisms: AnyElementsOrNone, permutation.

d)	Each value or template in the list shall be of the type declared for the template field in which the complement is used.

e)	The complement of a template list shall not match omit.

f)	Templates in the complement of a template list shall obey the present template restriction (see clause 15.8).

Examples

EXAMPLE 1:

type record MyMessageType

{

integer	field1,

boolean	field2

}

template MyMessageType mw_myTemplate:=

{

field1 := complement (1,3,5),	// list of unacceptable integer values

field2 := complement(true)		        // will match false

}

EXAMPLE 2:

type record of integer RoI;

template RoI mw_roI1 := {1, 2, (6..9)};

template RoI mw_roI2 := {1, *, 3};

template integer mw_i1 := complement(all from mw_roI1, 100);

// matches integer values different from 1, 2, 6, 7, 8, 9 and 100

template integer mw_i2 := complement(0, all from mw_roI2);

// causes an error because mw_roI2 contains AnyElementsOrNone

template RoI mw_roI3 := complement(all from mw_roI1);

// causes an error because member type of mw_roI1 (integer) is not compatible

// with the complemented list template type (RoI)

template RoI mw_roI4 := ?;

template RoI mw_roI5 := complement (all from mw_roI4);

// causes an error because mw_roI4 resolves into a matching mechanism

### B.1.2.3	Any value

The matching symbol "?" (AnyValue) matches any value of the specified type. It can be used on values of all types.

A template field that uses the any value mechanism matches the corresponding field if, and only if, the field evaluates to a single element of the specified type.

Restrictions

No specific restrictions in addition to the general static rules of TTCN-3 given in clause 5.

Examples

template MyMessageType mw_myTemplate:=

{

field1 := ?,		// will match any integer

field2 := ?,		// will match any non-empty charstring value

field3 := ?,		// will match true or false

field4 := ?		    // will match any sequence of integers

}

### B.1.2.4	Any value or none

The matching symbol "*" (AnyValueOrNone) is used to indicate that any valid value and the omission of the given optional field are acceptable. It can be assigned to templates of any type as a whole or to optional fields of set or record templates.

A template field that uses this symbol matches the corresponding field if, and only if, either the field evaluates to any element of the specified type, or if the field is absent.

Restrictions

a)	It can be assigned to templates of any type as a whole or to optional fields of set or record templates.

b)	At the time of matching during a receiving operation, it shall be applied to optional fields of record and set templates only.

Examples

type record MyMessageType2

{

integer			field1,

MyRecordofType		field2	optional,

boolean			field3	optional

}

type record of integer MyRecordofType;

const MyMessageType2 c_myMessage := { {42}, omit, false }

template MyMessageType2 mw_myMessageTemplate:=

{

:

field3 := *		// matches true or false or omitted field3

}

template MyMessageType2 mw_myMessageTemplate2:=

{

field1 := *,		// causes an error as field1 is mandatory

:

}

template MyRecordofType mw_myRecofTemplate := *; 	// this assignment is allowed

template boolean mw_myBoolTemplate := *; 			// this assignment is allowed as well

template MyMessageType2 mw_myMessageTemplate3:=

{

field1 := 42,

field2 := mw_myRecofTemplate,

// matches any valid value allowed by mw_myRecordof or absent field2

field3 := mw_myBoolTemplate

// matches true or false or omitted field3

}

v_mybooleanVar := match (c_myMessage.field2, mw_myRecofTemplate)

// matches and returns true

v_mybooleanVar := match ({},mw_myRecofTemplate);

// matches and returns true

v_mybooleanVar := match (false, mw_myBoolTemplate);

// matches and returns true

v_mybooleanVar := match ({42,omit,true},mw_myMessageTemplate3);

// matches and returns true

### B.1.2.5	Value range

Ranges indicate a bounded range of acceptable values, including or excluding the boundaries.

A template field that uses a range matches the corresponding field if, and only if, the field value is equal to one of the values in the range.

Restrictions

a)	When used for values of integer or float types (and integer or float subtypes), a boundary value shall be either:

1)	infinity or -infinity;

2)	an expression that evaluates to a specific integer or float value.

b)	The lower boundary shall be put on the left side of the range, the upper boundary at the right side. The lower boundary shall be less than the upper boundary.

c)	When used in templates or template fields of charstring or universal charstring types, the boundaries shall evaluate to valid character positions according to the coded character set table(s) of the type (e.g. the given position shall not be empty).

d)	Empty positions between the lower and the upper boundaries are not considered to be valid values of the specified range.

Examples

template MyMessageType mw_myTemplate:=

{

field1 := (1 .. !6),	// range of integer type from 1 to 5

:

:

:

}

// other entries for field1 might be (-infinity to 8) or (!12 to infinity)

### B.1.2.6	SuperSet

SuperSet is denoted by the keyword superset. SuperSet matches a set of values if, and only if, the set of values contains at least all of the elements defined within the SuperSet, and may contain more. The successful match shall be produced only if there exists such a one-to-one mapping from the SuperSet elements to the elements of the set of values where each SuperSet element matches the element of set of values it is mapped to. The SuperSet matching mechanism may contain templates (including template variables) obeying the present template restriction (see clause 15.8) and matching mechanisms with the restrictions given below. However, the length matching attribute may be attached to the SuperSet itself.

NOTE:	The SuperSet matching mechanism imposes an implicit length restriction on the matched set of values: the set of values shall contain at least as many elements as the SuperSet template in order to produce a successful match.

Besides specifying individual values, it is possible to add all elements of a record of or set of template into SuperSets using an all from clause.

Restrictions

a)	SuperSet is an operation for matching that shall be used only on values of set of types.

b)	Individual members of the SuperSet's argument shall be of the type replicated by the set of.

c)	The member type of the set of associated with the SuperSet template and the member type of the template in the all from clause shall be compatible.

d)	The template in the all from clause shall resolve either into a specific value or combined template. Elements of the combined template may contain any of the matching mechanisms or matching attributes with the exception of those described in the following restriction.

e)	The individual members of the SuperSet's argument and the elements of the combined template in the all from clause shall not be the matching mechanisms omit, SuperSet, SubSet and the matching attributes (length restriction and ifpresent). In addition, the individual members shall not resolve to AnyValueOrNone and individual elements of the combined template in the all from clause shall not resolve to AnyElementsOrNone or permutation.

f)	If the length matching attribute is attached to the SuperSet, the minimal length allowed by the length attribute shall not be less than the number of the elements in the SuperSet.

g)	Templates in SuperSet's argument shall obey the present template restriction (see clause 15.8).

Examples

EXAMPLE 1:

type set of integer MySetOfType (0 .. 10);

template MySetOfType mw_myTemplate1 := superset (1, 2, 3);

// matches any sequence of integers which contains at least one occurrences of the numbers

// 1, 2 and 3 in any order and position

template MySetOfType mw_myTemplate2_AnyValue := superset (1, 2, ?);

// matches any sequence of integers which contains at least one occurrences of the numbers

// 1, 2 and at least one more valid integer value (i.e. between 0 and 10, inclusively), in any

// order and position

template MySetOfType mw_myTemplate3 := superset (1, 2, (3, 4));

// matches any sequence of integers which contains at least one occurrences of the numbers

// 1, 2 and a number with the value 3 or 4, in any order and position

template MySetOfType mw_myTemplate4 := superset (1, 2, complement(3, 4));

// any sequence of integers matches which contains at least one occurrences of the numbers

// 1, 2 and a valid integer value which is not 3 or 4, in any order and position

template MySetOfType mw_myTemplate6 := superset (1, 2, 3) length (7);

// matches any sequence of 7 integers which contains at least one occurrences of the numbers

// 1, 2 and 3 in any order and position

template MySetOfType mw_myTemplate7 := superset (1, 2, ?) length (7 .. infinity);

// matches any sequence of at least 7 integers which contains at least one occurrences of the

// numbers 1, 2 and at least 5 more valid integer values (i.e. between 0 and 10, inclusively) in any order and position

template MySetOfType mw_myTemplate8 := superset (1, 2, 3) length (2 .. 7);

// causes an error, the lower bound of the length attribute contradicts to the minimum number

// of elements imposed by the superset argument

EXAMPLE 2:

type record of integer RoI;

type set of integer SoI;

template RoI mw_roI1 := {1, 2, ?};

template SoI mw_soI1 := superset(all from mw_roI1);

// results in superset(1, 2, ?)

### B.1.2.7	SubSet

SubSet is denoted by the keyword subset. SubSet matches a set of values if, and only if, the set of values contains only elements defined within the SubSet, and may contain less. The successful match shall be produced only if there exists such a one-to-one mapping from the elements of the set of values to the SubSet elements where each element of the set of values is matched by the SubSet element it is mapped to. The SubSet matching mechanism may contain templates (including template variables) obeying the present template restriction (see clause 15.8) and matching mechanisms with the restrictions given below. However, the length matching attribute may be attached to the SubSet itself.

The SubSet matching mechanism imposes an implicit length restriction on the matched set of values: the set of values shall contain at most as many elements as the SubSet template in order to produce a successful match.

Besides specifying individual values, it is possible to add all elements of a record of or set of template into SubSets using an all from clause.

Restrictions

a)	SubSet is an operation for matching that can be used only on values of set of types.

b)	Individual members of the SubSet's argument shall be of the type replicated by the set of.

c)	The member type of the set of type associated with the SubSet and the member type of the template in the all from clause shall be compatible.

d)	The template in the all from clause shall resolve either into a matching mechanism or combined template. Elements of the combined template may contain any of the matching mechanisms or matching attributes with the exception of those described in the following restriction.

e)	The individual members of the SubSet's argument and the elements of the combined template in the all from clause shall not be the matching mechanisms omit, SuperSet, SubSet and the matching attributes (length restriction and ifpresent). In addition, individual members shall not resolve to AnyValueOrNone and individual elements of the combined template in the all from clause shall not resolve to AnyElementsOrNone or permutation.

f)	If the length matching attribute is attached to the SubSet, the maximum length allowed by the length attribute shall not exceed the number of the elements in the SubSet.

g)	Templates in SubSet's argument shall obey the present template restriction (see clause 15.8).

Examples

EXAMPLE 1:

template MySetOfType mw_myTemplate1:= subset (1, 2, 3);

// matches any sequence of integers which contains zero or one occurrences of the numbers

// 1, 2 and 3 in any order and position

template MySetOfType mw_myTemplate1:= subset (1, 2, ?);

// matches any sequence of integers which contains zero or one occurrences of the numbers

// 1, 2 and a valid integer value (i.e. between 0 and 10, inclusive) in any order and position

template MySetOfType mw_myTemplate1:= subset (1, 2, (3, 4));

// matches any sequence of integers which contains zero or one occurrences of the numbers

// 1, 2 and one of the numbers 3 or 4, in any order and position

template MySetOfType mw_myTemplate1:= subset (1, 2, complement (3, 4));

// matches any sequence of integers which contains zero or one occurrences of the numbers

// 1, 2 and a valid integer number which is not 3 or 4, in any order and position

template MySetOfType mw_myTemplate1:= subset (1, 2, 3) length (2);

// matches any sequence of two integers which contains zero or one occurrences of

// the numbers 1, 2 and 3, in any order and position

template MySetOfType mw_myTemplate1:= subset (1, 2, ?) length (0 .. 2);

// matches any sequence of zero, one or two integers which contains zero or one occurrences of

// the numbers 1, 2 and of a valid integer value, in any order and position

template MySetOfType mw_myTemplate1:= subset (1, 2, 3) length (0 .. 4);

// causes an error, the upper bound of length attribute contradicts to the maximum number of

// elements imposed by the subset argument

EXAMPLE 2:

type record of integer RoI;

type set of integer SoI;

template RoI mw_roI1 := {1, 2, ?};

template SoI mw_soI1 := subset(all from mw_roI1);

// results in subset(1, 2, ?)

### B.1.2.8	Omitting optional fields

The keyword omit denotes that an optional field shall be absent. If used as a matching mechanism, it matches an optional field if and only if it is absent.

Restrictions

a)	It can be assigned to templates of any type as a whole or to optional fields of set or record templates.

b)	At the time of matching during a receiving operation, it shall be applied to optional fields of record and set templates only.

Examples

type record MyMessageType2

{

integer			field1,

MyRecordofType		field2	optional,

boolean			field3	optional

}

const MyMessageType2 c_myMessage := { 42, omit, false }

template MyMessageType2 m_myMessageTemplate:=

{

:

field3 := omit		// omits the optional field field3

}

template MyMessageType2 m_myMessageTemplate2:=

{

field1 := omit,		// causes an error as field1 is mandatory

:

}

template MyRecordof m_myRecofTemplate := omit; 	// this assignment is allowed

template boolean m_myBoolTemplate := omit; 		// this assignment is allowed as well

template MyMessageType2 m_myMessageTemplate3:=

{

field1 := 42,

field2 := m_myRecofTemplate,

// matches if field2 is absent

field3 := m_myBoolTemplate

// matches if field3 is absent

}

v_myBooleanVar := match (c_myMessage.field2, m_myRecofTemplate)

// matches and returns true

v_myBooleanVar := match ({},m_myRecofTemplate)

// does not match and returns false

v_myBooleanVar := match (false, m_myBoolTemplate);

// does not match and returns false

v_myBooleanVar := match ({42,omit,omit},m_myMessageTemplate3)

// matches and returns true

### B.1.2.9	Matching decoded content

The matching symbol MatchDecodedContent decmatch is used for checking encoded payload fields. The matching symbol is composed of the decmatch keyword, an optional encoding format parameter and a mandatory template instance called decoding target.

A template field that uses this symbol matches the corresponding field if, and only if, the field can be successfully decoded as an instance of the same type as the decoding target and if the decoded instance can be successfully matched by the decoding target.

The optional encoding format parameter may specify one of the UCS encoding formats (see clause C.5.4) that shall be used for the decoding trial, i.e. it overrides any variant attribute attached to the decoding target or the type of the decoding target (for example, for predefined variant attributes see clause 27.5).

Restrictions

a)	It can be assigned to templates and template fields of bitstring, hexstring, octetstring, charstring and universal charstring types.

b)	The decoding target can be a template of any data type.

c)	The optional encoding format parameter can be used only for fields of r universal charstring types. The parameter value shall be of the charstring type and it shall contain one of the strings allowed for the decvalue_unichar predefined function (specified in clause C.5.4). Any other value shall cause an error.

d)	If the template field is of charstring type or is of universal charstring type and the encoding format is missing, the default value "UTF-8" shall be used.

NOTE:	The model of the behaviour of this implicit decoding is the following. At first, hexstring and octetstring values are implicitly converted to a bitstring value using the predefined hex2bit and oct2bit functions (specified in clauses C.1.18 and C.1.22) and charstring values are implicitly converted to universal charstring values. Prior to decoding, the bitstring and universal charstring values are stored into a temporary anonymous variable. Decoding is then performed by implicitly calling the predefined decvalue function (specified in clause C.5.2) for bitstring values and decvalue_unichar function for  universal charstring values. The anonymous variable containing the encoded value is passed as the first parameter to the function, the second parameter contains another temporary variable called decoded instance. The decoded instance is of the same type as the decoding target. If the optional encoding format parameter is present, it is passed as the third parameter to the decvalue_unichar function. Decoding is successful only if the decoding function returns 0 and the first parameter contains an empty string (i.e. the whole encoded value has been successfully decoded). The matching mechanism will generate an unsuccessful match if decoding hasn't succeeded.

Examples

type record MyBinaryMessageType

{

...,

octetstring	payload

}

type record MyTextMessageType

{

...,

universal charstring	payload

}

type record MyPayloadType

{

integer		field1,

integer		field2

}

template MyBinaryMessageType mw_t1 :=

{

:

// The payload field can be matched only if it contains an encoded value of the MyPayload

// type and if the field1 of the decoded value is equal to 1.

payload := decmatch MyPayload:{field1 := 1, field2 := ? }

}

template MyTextMessageType mw_t2 :=

{

:

// The payload field can be matched only if it contains an encoded value of the MyPayloadType

// type in the UTF-8 format and if the field1 of the decoded value is equal to 2 or 3.

payload := decmatch("UTF-8") MyPayloadType:{field1 := (2, 3), field2 := ? }

}

### B.1.2.10	Matching enumerated value with value list

To match an enumerated value with an associated value list in its definition, the enumerated value name shall be referenced followed by a non-empty list of integer templates in parenthesis.

The template matches only those enumerated values of the same name where the associated integer values is matched by at least one of the integer templates.

Examples

type enumerated Days

{

Christmas(0), Easter(1), Other(2..365)

}

template integer mw_greater20 := complement(0 .. 20);

template mw_days1 := Other(5..6, greater20); // matches Other(5), Other(6) and

// Other(21) .. Other(365)

template mw_days2 := Other(?); // matches Other(2) .. Other(365)

## B.1.3	Matching mechanisms inside values

### B.1.3.0	General

The following matching mechanisms may be used inside explicit values of strings, records, records of, sets, sets of and arrays. The template that contains a matching mechanism inside a value is called a combined template.

### B.1.3.1	Any element

#### B.1.3.1.0	General

The matching symbol "?" (AnyElement) is used to indicate that it replaces single elements of a string (except character strings, see table 4 for the lengths of the units being matched by "?" in a string), a record of, a set of or an array.

Restrictions

a)	It shall be used only within values of string types, record of types, set of types and arrays.

Examples

template MyMessageType mw_myTemplate:=

{	:

field2 := "abcxyz",

field3 := '10???'B, 	// where each "?" may either be 0 or 1

field4 := {1, ?, 3}		// where ? may be any integer value

}

NOTE:	The "?" in field4 can be interpreted as AnyValue as an integer value, or AnyElement inside a record of, set of or array. Since both interpretations lead to the same match no problem arises.

#### B.1.3.1.1	Using single character wildcards

If it is required to express the "?" wildcard in character strings it shall be done using character patterns (see clause B.1.5). For example: "abcdxyz", "abccxyz", "abcxxyz" etc. will all match pattern "abc?xyz". However, "abcxyz", "abcdefxyz", etc. will not.

### B.1.3.2	Any number of elements or no element

#### B.1.3.2.0	General

The matching symbol "*" (AnyElementsOrNone) is used to indicate that it replaces none or any number of consecutive elements of a string (except character strings), a record of, a set of or an array. The "*" symbol matches the longest sequence of elements possible, according to the pattern as specified by the symbols surrounding the "*".

If a "*" appears at the highest level inside a binary string, a record of, set of or array, it shall be interpreted as AnyElementsOrNone.

NOTE:	This rule prevents the otherwise possible interpretation of "*" as AnyValueOrNone that replaces an element inside a binary string, record of, set of or array.

Restrictions

a)	It shall be used only within values of binary string types, record of types, set of types and arrays and inside the permutation matching mechanism.

Examples

template MyMessageType mw_myTemplate:=

{	:

field2 := "abcxyz",

field3 := '10*11'B, 	// 	where "*" may be any sequence of bits (possibly empty)

field4 := {*, 2, 3}		// where "*"may be any number of integer values or omitted

}

type charstring MyStrings[4];

myPCO.receive(MyStrings:{"abyz", *, "abc" });

#### B.1.3.2.1	Using multiple character wildcards

If it is required to expressed the "*" wildcard in character strings it shall be done using character patterns (see clause B.1.5). For example: "abcxyz", "abcdefxyz" "abcabcxyz" etc. will all match pattern "abc*xyz".

### B.1.3.3	Permutation

Permutation is an operation for matching that shall be used only on values of record of and array types. Permutation is denoted by the keyword permutation. Permutation elements shall obey the restrictions given below.

A permutation without AnyElementsOrNone in place of a single record of element means that any series of elements is acceptable provided that there is a one to one mapping between elements in the record of and in the permutation list such that each element matches its corresponding element in the permutation list.

AnyElementsOrNone used inside permutation (directly or via reference) replaces none or any number of elements within the segment of the record of value matched by permutation. The permutation matching is successful, if a subset of the elements in the record of matches the permutation list without the AnyElementsOrNone. If both permutation and AnyElementsOrNone are used in a record of template, they shall be evaluated jointly.

NOTE 1:	AnyElementsOrNone used inside permutation has a different effect as AnyElementsOrNone used in conjunction with permutation as in the latter AnyElementsOrNone replaces consecutive elements only. For example, {permutation(1,2,*)} is equivalent to ({*,1,*,2,*},{*,2,*,1,*}), while {permutation(1,2),*} is equivalent to ({1,2,*},{2,1,*}).

NOTE 2:	When AnyElementsOrNone is inside a permutation, a length attribute may be applied to AnyElementsOrNone to restrict the number of elements matched by AnyElementsOrNone (see also clause B.1.4.1).

Besides specifying all individual values, it is possible to add all elements of a record of or set of template into permutations using an all from clause.

Restrictions

a)	Each individual member listed in the permutation shall be of the type replicated by the record of or array type.

b)	The member type of the permutation and the member type of the template in the all from clause shall be compatible.

c)	The template referenced in the all from clause shall resolve either into specific value (see clause B.1.1) or combined template. The combined template shall not contain permutations.

d)	Void.

e)	Templates except AnyElementsOrNone listed in the permutation shall obey the present template restriction (see clause 15.8).

Examples

EXAMPLE 1:

type record of integer MySequenceOfType;

template MySequenceOfType mw_myTemplate1 := { permutation ( 1, 2, 3 ), 5 };

// matches any of the following sequences of 4 integers: 1,2,3,5; 1,3,2,5; 2,1,3,5;

// 2,3,1,5; 3,1,2,5; or 3,2,1,5

template MySequenceOfType mw_myTemplate2 := { permutation ( 1, 2, ? ), 5 };

// matches any sequence of 4 integers that ends with 5 and contains 1 and 2 at least once in

// other positions

template MySequenceOfType mw_myTemplate3 := { permutation ( 1, 2, 3 ), * };

// matches any sequence of integers starting with 1,2,3; 1,3,2; 2,1,3; 2,3,1; 3,1,2 or 3,2,1

template MySequenceOfType mw_myTemplate4 := { *, permutation ( 1, 2, 3 )};

// matches any sequence of integers ending with 1,2,3; 1,3,2; 2,1,3; 2,3,1; 3,1,2 or 3,2,1

template MySequenceOfType mw_myTemplate5 := { *, permutation ( 1, 2, 3 ),* };

// matches any sequence of integers containing any of the following substrings at any position:

// 1,2,3; 1,3,2; 2,1,3; 2,3,1; 3,1,2 or 3,2,1

template MySequenceOfType mw_myTemplate6 := { permutation ( 1, 2, * ), 5 };

// matches any sequence of integers that ends with 5 and containing 1 and 2 at least once in

// other positions

template MySequenceOfType mw_myTemplate7 := { permutation ( 1, 2, 3 ), * length (0..5)};

// matches any sequence of three to eight integers starting with 1,2,3; 1,3,2; 2,1,3; 2,3,1;

// 3,1,2 or 3,2,1

template integer mw_myInt1 := (1,2,3);

template integer mw_myInt2 := (1,2,?);

template integer mw_myInt3 := ?;

template integer mw_myInt4 := *;

template MySequenceOfType mw_myTemplate10 := { permutation (mw_myInt1, 2, 3 ), 5 };

// matches any of the sequences of 4 integers:

//		1,3,2,5; 2,1,3,5; 2,3,1,5; 3,1,2,5; or 3,2,1,5;

//		2,3,2,5; 2,2,3,5; 2,3,2,5; 3,2,2,5; or 3,2,2,5;

//		3,3,2,5; 2,3,3,5; 2,3,3,5; 3,3,2,5; or 3,2,3,5;

template MySequenceOfType mw_myTemplate11 := { permutation (mw_myInt2, 2, 3 ), 5 };

// matches any sequence of 4 integers that ends with 5 and contains 2 and 3 at least once in

// other positions

template MySequenceOfType mw_myTemplate12 := { permutation (mw_myInt3, 2, 3 ), 5 };

// matches any sequence of 4 integers that ends with 5 and contains 2 and 3 at least once in

// other positions

template MySequenceOfType mw_myTemplate13 := { permutation (mw_myInt4, 2, 3 ), 5 };

// matches any sequence of integers that ends with 5 and containing 2 and 3 at least once in

// other positions

template MySequenceOfType mw_myTemplate14 := { permutation (mw_myInt3, 2, ? ), 5 };

// matches any sequence of 4 integers that ends with 5 and contains 2 at least once in

// other positions

template MySequenceOfType mw_myTemplate15 := { permutation (mw_myInt4, 2, * ), 5 };

// matches any sequence of integers that ends with 5 and contains 2 at least once in

// other positions

template MySequenceOfType mw_myTemplate16 := { permutation (2, 2, 3 ), 5 };

// matches any sequence of integers of length 4 that ends with 5 and contains 2 in

// two other positions and 3 in the remaining position

EXAMPLE 2:

type record of integer RoI;

template RoI mw_roI1 := {1, 2, *};

template RoI mw_roI2 := {permutation(0, all from mw_roI1), 4, 5};

// results in {permutation(0, 1, 2, *), 4, 5}

## B.1.4	Matching attributes of values

### B.1.4.0	General

The following attributes may be associated with matching mechanisms.

### B.1.4.1	Length restrictions

The length restriction attribute is used to restrict the length of string values matching the template or the number of elements in a set of, record of or array structure.

It can also be used in conjunction with the ifpresent matching attribute. The syntax for length can be found in clause 6.2.3.

NOTE:	When the length attribute is used with a template list, elements of the list may be disabled by the attribute.

The units of length are to be interpreted according to table 4 in the main body of the present document in the case of string values. For set of, record of types and arrays the unit of length is the replicated type.

A template field that uses length as an attribute of a symbol matches the corresponding field if, and only if, the field matches both the symbol and its associated attribute. The length attribute matches if the length of the field is greater than or equal to the specified lower bound and less than or equal to the upper bound. In the case of a single length value the length attribute matches only if the length of the received field is exactly the specified value.

It is allowed to use a length restriction in conjunction with the special value omit, however in this case the length attribute has no effect (i.e. with omit it is redundant). With AnyValueOrNone and ifpresent it places a restriction on the value, if any.

If a length restriction is added to an AnyElementsOrNone matching mechanism, it constrains the number of elements matched by this matching mechanism. The minimum and maximum number of elements matched by the AnyElementsOrNone matching mechanism is equal to the lower bound and upper bound of the length restriction.

Restrictions

a)	The length restriction shall be used only as an attribute of the following matching mechanisms: specific value, combined template, template list, complemented template list, AnyValue, AnyValueOrNone, AnyElement, AnyElementsOrNone, superset, subset, and pattern.

b)	It shall not be used directly with templates and template fields produced by concatenation (see clause 15.11). If the length of a template or template field produced by concatenation is wished to be restricted, the concatenation shall be enclosed into a pair of parentheses.

c)	The boundaries of the length restriction shall be denoted by expressions which resolve to specific non-negative integer values. Alternatively, the keyword infinity can be used as a value for the upper boundary in order to indicate that there is no upper limit of length.

d)	The length specifications for the template shall not conflict with the length for restrictions (if any) of the corresponding type.

e)	When both the complement and the length restriction matching mechanisms are used for a template or template field, restrictions implied by them shall apply to the template or template field independently.

Examples

// Given the message type definition

type record MyMessageType3

{

record of integer 	field1,

record of integer 	field2,

charstring 			field3,

charstring 			field4,

charstring 			field5

}

template MyMessageType3 mw_myTemplate:=

{

field1 := complement ({4,5},{1,4,8,9}) length (1 .. 6),	// any value containing 1, 2, 3,

//4, 5 or 6 elements is accepted provided it is not {4,5} or {1,4,8,9}

field2 := { 1, * length(2), 3 }  // matches the same values as {1, ?, ?, 3 }

field3 := "ab*ab" length(5),   // matches the character string "ab*ab" only

field4 := "ab*ab" length(13),  // never matches as the specific value is of length 5

// and not of length 13

Field5 := pattern "ab*ab" length(13),

// max length of the AnyElementsOrNone string is 9 characters

}

### B.1.4.2	The IfPresent indicator

The ifpresent indicates that a match may be made if an optional field is present (i.e. not omitted).

A template field that uses ifpresent matches the corresponding field if, and only if, the field matches according to the associated matching mechanism, or if the field is absent.

Restrictions

a)	This IfPresent indicator shall be used only for matching mechanisms in templates of any type as a whole or for optional fields of set or record templates.

b)	At the time of matching during a receiving operation, it shall be applied to optional fields of record and set templates only.

Examples

type record MyMessageType2

{

integer			field1,

MyRecordofType		field2	optional,

boolean			field3	optional

}

template MyMessageType2 mw_myMessageTemplate:=

{

:

field2 := { 1, 2, 3 } ifpresent, 	// matches { 1, 2, 3 } if not omitted

:

}

template MyMessageType mw_myMessageTemplate2:=

{

field1 := 1 ifpresent,				// causes an error as field1 is mandatory

:

}

template MyRecordofType mw_myRecofTemplate := { 1, 2, 3 } ifpresent; // this assignment is 	allowed

template boolean mw_myBoolTemplate := true ifpresent; // this assignment is also allowed

template MyMessageType2 mw_myMessageTemplate3:=

{

field1 := 42,

field2 := mw_myRecofTemplate,

// if field2 is not absent, it matches the value { 1, 2, 3 }

field3 := mw_myBoolTemplate

// if field3 is not absent, it matches the value true

}

v_mybooleanVar := match ({},mw_myRecofTemplate);

// mayches and returns true

v_mybooleanVar := match ({42,omit,true},mw_myMessageTemplate3);

// matches and returns true

NOTE:	AnyValueOrNone has exactly the same meaning as ? ifpresent.

## B.1.5	Matching character pattern

### B.1.5.0	General

Character patterns can be used in templates to define the format of a required character string to be received. Character patterns can be used to match charstring and universal charstring values. In addition to literal characters, character patterns allow the use of meta-characters (e.g. ? and * within a character pattern means matching any character and any number of any character respectively).

EXAMPLE 1:

template charstring mw_myTemplate:= pattern "ab??xyz*0";

This template will match any character string that consists of the characters "ab", followed by any two characters, followed by the characters "xyz", followed by any number of any characters (including any number of "0"-s) before the closing character "0".

If it is required to interpret any metacharacter literally it shall be preceded with the metacharacter "\".

EXAMPLE 2:

template charstring mw_myTemplate:= pattern "ab?\?xyz*";

This template would match any character string which consists of the characters "ab", followed by any character, followed by the characters "?xyz", followed by any number of any characters.

The list of meta characters for TTCN-3 patterns is shown in table B.1. Metacharacters shall not contain whitespaces except a whitespace preceded by a newline character before or inside a set expression.

Table B.1: List of TTCN-3 pattern metacharacters

| Metacharacter | Description |
| --- | --- |
| ? | Match any character (see notes 1 and 2) |
| * | Match any character zero or more times; shall match the longest possible number of characters (see example 1 above) (see notes 1 and 2) |
| \ | Cause the following metacharacter to be interpreted as a literal (see note 3). When preceding a character without defined metacharacter meaning "\" and the character together match the character following the "\" (see note 4) |
| [ ] | Match any character within the specified set, see clause B.1.5.1 for more details |
| - | Has a metacharacter meaning in a set expression. It allows to specify a range of characters; see clause B.1.5.1 for more details |
| ^ | Has a metacharacter meaning in a set expression. It causes to match any character complementing the set of characters following this metacharacter; see clause B.1.5.1 for more details |
| \q{group,plane,row,cell} or \q{Uxxxx, Uxxx ….} | Match one or more universal character. Both the quadruple and the USI-like syntaxes specified in clause 6.1.1 can be used |
| {reference} | Insert the referenced user defined string and interpret it as a regular expression. See clause 27.9 for more details |
| {\reference} | Insert the referenced user defined string and interpret it as a set of literals. See clause 27.9 for more details |
| \N{reference} | Matches a single character from the (sub)set of characters denoted; see clause B.1.5.4 for more details |
| \d | Match any numerical digit (equivalent to [0-9]) |
| \w | Match any alphanumeric character (equivalent to [0-9a-zA-Z]) |
| \t | Match the C0 control character HT(9) (see Recommendation ITU-T T.50 [4]) |
| \n | Match any of the following C0 control characters: LF(10), VT(11), FF(12), CR(13) (see Recommendation ITU-T T.50 [4]) (jointly called newline characters, see clause A.1.5.1) |
| \r | Match the C0 control character CR (see Recommendation ITU-T T.50 [4]) |
| \s | Match any one of the following C0 control characters: HT(9), LF(10), VT(11), FF(12), CR(13), SP(32) (see Recommendation ITU-T T.50 [4]) (jointly called white-space characters, see clause A.1.5.1) |
| \b | Match a word boundary (any graphical character except SP or DEL is preceded or followed by any of the whitespace or newline characters) |
| \" | Match the double quote character |
| "" | Match the double quote character |
| ¦ | Used to denote two alternative expressions |
| ( ) | Used to group an expression |
| #(n, m) | Match the preceding expression at least n times but no more than m times (postfix). See clause B.1.5.3 for more details |
| #n | Match the previous expression exactly n times (where n is a single digit) (postfix); the same as #(n). See clause B.1.5.3 for more details |
| + | Match the preceding expression one or several times (postfix); the same as #(1,). See clause B.1.5.3 for more details |
| NOTE 1: Metacharacters ? and * are able to match any characters of the character set of the root type of the template or template field in which they are used (i.e. not considering type constraints applied). However, it shall not be forgotten, that receiving operations require type checking of the received message before attempting to match it. Therefore received values not complying with the subtype specification of the template or template field are never provided for matching. NOTE 2: In some other languages/notations ? and * has different meaning as metacharacters. However inTTCN-3 these characters are traditionally used for matching in the sense as specified in this table. NOTE 3: Consequently the backslash character can be matched by a pair of backslash characters without space between them (\\), e.g. the pattern "\\d" will match the string "\d"; opening or closing square brackets can be matched by "\[" and "\]" respectively, etc. NOTE 4: Such use of the metacharacter "\" is deprecated as further metacharacters can be defined later. |  |

The symbols that can appear as lexical marks in metacharacter definitions are called metacharacter symbols. They include the following characters: "#", "(", ")", "*", "+", "-", "?", "[", "\", "]", "^", "{","|","}". When any of the metacharacter symbols are present in a pattern, but do not form a valid metacharacter, they retain their literal value.

NOTE:	This rule assures that no format error can occur during pattern template instantiation. However, errors caused by invalid references can still appear (see clauses 27.9 and B.1.5.4 for more details).

Character patterns may be composed from several fragments using the concatenation operation. The fragments of the pattern shall be concatenated before any evaluation of the pattern expression. See also the shorthand notation for referenced definitions at concatenation in clause 27.9.

EXAMPLE 3:

template charstring mw_myTemplate:= pattern "ab?\?" & "xyz*"; // results in the same pattern as

// in example 2

Pattern definitions may contain references to values or templates. The referred value or template shall be of the charstring or universal charstring type and it shall contain either a specific value or pattern. When the referenced template contains a pattern, the character pattern definition of this pattern is used as a fragment for creating the new pattern.

EXAMPLE 4:

template charstring mw_template1 := "ab?";

template charstring mw_template2 := pattern "?xyz*0";

template charstring mw_template3 := ?;

template charstring mw_template4 := pattern mw_template1 & mw_template2;

// the same template as in example 1, i.e. pattern "ab??xyz*0"

template charstring mw_template5 := pattern mw_template3

// produces as error as mw_template3 doesn't contain a value or pattern

### B.1.5.1	Set expression

A list of characters enclosed by a pair of "[" and "]" matches any single character in that list. The set expression is delimited by the "[" "]" symbols. In addition to character literals, it is possible to specify character ranges using the hyphen "-" as separator. The range consist of the character immediately before the separator, the character immediately after it and all characters with a character code between the codes of the two bordering characters. A hyphen character "-" inside the list but without preceding or following character loses its special meaning.

The set expression can also be negated by placing the caret "^" character as the first character after the opening square bracket. Negation takes precedence over character ranges. Therefore a hyphen "-" immediately following a negating caret "^" shall be processed as a literal character.

An empty list and an empty negated list are not allowed. Therefore a closing square bracket "]" immediately following an opening square bracket "[" or a caret following the opening square bracket "[" and immediately followed by a closing square bracket "]" shall be processed as literal characters.

All metacharacters, except those listed below, lose their special meaning inside the list:

"]" not at the first position and not immediately following a "^" at the first position;

"-" not at the first or last positions in the list;

"^" at the first position in the list except when immediately followed by a closing square bracket;

"\", "\d", "\t", "\w", "\r", "\n", "\s" and "\b";

"\q{group,plane,row,cell}";

"\N{reference}".

NOTE 1:	Embedded lists are not allowed. For example in pattern "[ab[r-z]]" the second "[" denotes a literal "[", the first "]" closes the list and the second "]" retains its literal value as no related opening bracket precedes it in the pattern. The pattern will match character strings containing two elements, with the first element equal to "a", "b", "[" or anything in the range "r"-"z" and the second character equal to "]".

NOTE 2:	To include a literal caret character "^", place it anywhere except in the first position or precede it with a backslash. To include a literal hyphen "-", place it first or last in the list, or precede it with a backslash. To include a literal closing square bracket "]", place it first or precede it with a backslash. If the first character in the list is the caret "^", then the characters "-" and "]" also match themselves when they immediately follow that caret.

EXAMPLE:

template charstring mw_regExp1:= pattern "[a-z]";  //this will match any character from a to z

template charstring mw_regExp2:= pattern "[^a-z]"; //this will match any character except a to z

template charstring mw_regExp3:= pattern "[AC-E][0-9][0-9][0-9]YKE";

// mw_regExp3 will match a string which starts with the letter A or a letter between

// C and E (but not e.g. B) then has three  digits and the letters YKE

### B.1.5.2	Reference expression

In addition to direct string values, it is also possible within the pattern to use references to templates, constants, variables, formal parameters, module parameters, or to their fields, containing either a character string value or pattern matching. The reference shall be enclosed within the "{"  "}" characters and reference shall resolve a compatible character string type. The opening bracket can be optionally followed by a backslash.

If the backslash character is missing, the referenced character string or pattern shall be inserted into the pattern being constructed and shall be handled as a regular expression. Each expression shall be dereferenced only once, before the insertion (i.e. the expression dereferenced and inserted into the referencing pattern shall not be dereferenced again).

If the backslash character is present, the referenced item shall contain a character string value in this case. The character string is inserted into the pattern being constructed so that it all characters contained in it can keep their literal value (i.e. all metacharacter symbols are automatically escaped).

If the reference cannot be resolved or if the referenced symbol does not fulfil the requirements set by this clause, an error shall be generated.

EXAMPLE 1:

const charstring c_myString:= "ab?";

template charstring mw_myTemplate:= pattern "{c_myString}";

//matches any character string that consists of the characters "ab" followed by any character

template charstring mw_myTemplate2:= pattern "{\c_myString}";

//resolves into pattern "ab\?" and matches the string"ab?" only

template universal charstring mw_myTemplate3:= pattern "{c_myString}de\q{1, 1, 13, 7}";

//matches any universal character string which consists of the characters "ab", followed by

//any character, followed by the characters "de", followed by the character in ISO10646-1 with

//group=1, plane=1, row=13 and cell=7.

If a referenced definition or field of a definition contains one or more reference expressions, then these references shall recursively be dereferenced before inserting their contents into the referencing pattern.

If a fragment of a pattern contains a single reference only, it is allowed, as a shorthand notation, to reference the definition or the field of the definition directly, i.e. leave out double quotes (" ") and the pair of curly brackets ({ }).

EXAMPLE 2:

const charstring c_myConst2 := "ab";

template charstring mw_regExp1 := pattern "{c_myConst2}";

// matches the string "ab"

template charstring mw_regExp1a := pattern c_myConst2;

// the same as above, matches the string "ab"

template charstring mw_regExp2 := pattern "{mw_regExp1}{mw_regExp1}";

// matches the string "abab"

template charstring mw_regExp2a := pattern "{mw_regExp1}" & "{mw_regExp1}";

// the same as above, matches the string "abab"

template charstring mw_regExp2b := pattern mw_regExp1 & mw_regExp1;

// the same as above, matches the string "abab"

template charstring mw_regExp3 := pattern "c{mw_regExp2}d";

// matches the string "cababd"

template charstring mw_regExp4 := pattern "{mw_reg";

template charstring mw_regExp5 := pattern "Exp1}";

template charstring mw_regExp6 := pattern "{mw_regExp4}{mw_regExp5}";

// matches the string "{mw_regExp1}" only (i.e. shall not be handled as a reference

// expression after insertion)

template charstring mw_regExp7 := pattern "{mw_reg" & "Exp1}";

// note the difference to the previous example; in this case the fragments of the

// pattern are joined before any evaluation, i.e. this template will match the string "ab"

EXAMPLE 3:

template charstring m_ref0:= "My String";

template charstring m_ref1:= "{m_re";

template charstring m_ref2:= "f0}";

template charstring m_ref3:= "{m_ref1}{m_ref2}";

//this matches "{m_ref0}"

//i.e. there is no further dereferencing

//as m_ref1 and m_ref2 do not contain a reference

template charstring m_ref4:= "{m_ref0}";

template charstring m_ref5:= "";

template charstring m_ref6:= "{m_ref4}{m_ref5}";

//this matches "My String" – here m_ref0 is dereferenced, because m_ref4 contains

//the reference expression {m_ref0} with the reference m_ref0

EXAMPLE 4:

type record MyRecordType {

integer i,

charstring c

}

const MyRecordType c_referencedRecord:= {1,"this"}

const charstring c_referencedConstant := c_referencedRecord.c;

template charstring m_referencingPattern := pattern "{c_referencedConstant}"

//this matches "this" as the c_referencedConstant is dereferenced

### B.1.5.3	Match expression n times

To specify that the preceding expression should be matched a number of times one of the following syntaxes shall be used: "#(n, m)", "#(n, )", "#( , m)", "#(n)", "#n", "#(,)", "#()" or "+".

The form "#(n, m)" specifies that the preceding expression shall be matched at least n times but not more than m times.

The metacharacter postfix "#(n, )" specifies that the preceding expression shall be matched at least n times while
"#( , m)" indicates that the preceding expression shall be matched not more than m times.

Metacharacters (postfixes) "#(n)" and "#n" specify that the preceding expression shall be matched exactly n times (they are equivalent to "#(n, n)"). In the form "#n" n shall be a single digit.

The forms "#(,)" and "#()" are shorthand notations for "#(0,)", i.e. matches the preceding expression any number of times.

The metacharacter postfix "+" denotes that the preceding expression shall be matched at least 1 time (equivalent to "#(1,)").

EXAMPLE:

template charstring mw_regExp4:= pattern "[a-z]#(9, 11)"; //match at least 9 but no more than 11

// characters from a to z

template charstring mw_regExp5a:= pattern "[a-z]#(9)";    // match exactly 9

// characters from a to z

template charstring mw_regExp5b:= pattern "[a-z]#9";  	  // match exactly 9

// characters from a to z

template charstring mw_regExp6:= pattern "[a-z]#(9, )";   // match at least 9

// characters from a to z

template charstring mw_regExp7:= pattern "[a-z]#(, 11)";  // match no more than 11

// characters from a to z

template charstring mw_regExp8:= pattern "[a-z]+";		  // match at least 1

// characters from a to z,

### B.1.5.4	Match a referenced character set

A notation of the form "\N{reference}", where reference is denoting a one-character-length template, constant, variable, formal parameter or module parameter, matches the character in the referenced value or template.

If the reference cannot be resolved or if the referenced symbol is anything else than a template, constant, variable, formal parameter or module parameter containing a character string of length 1, an error shall be generated.

A notation of the form "\N{typereference}", where "typereference" is a reference to a charstring or universal charstring type, matches any character of the character set denoted by the referenced type.

NOTE 1:	Cases when the referenced set of characters is not a subset of values allowed by the type definition of the template or template field for which the character pattern is used, are not be treated as an error (but e.g. matching never can occur if the two sets do not overlap).

NOTE 2:	\N{charstring} is equivalent to ? when the latter is applied to a template or template field of charstring type and \N{universal charstring} is equivalent to ? when the latter is applied to a template or template field of universal charstring type (but causes an error if applied to a template or template field of charstring type).

EXAMPLE:

type charstring MyCharRangeType ("a".."z");

type charstring MyCharListType ("a", "z");

const MyCharRangeType c_myCharR := "r";

template charstring mw_myTempPatt1 := pattern "\N{c_myCharR}";

// mw_myTempPatt1 shall match the string "r" only

template charstring mw_myTempPatt2 := pattern "\N{MyCharRange}";

// mw_myTempPatt2 shall match any string containing a single character from a to z

template MyCharRangeType mw_myTempPatt3 := pattern "\N{MyCharList}";

// mw_myTempPatt3 shall match strings "a" or "z" only

### B.1.5.5	Type compatibility rules for patterns

For the purpose of referenced patterns (see clause 27.9) and references character sets (see clause B.1.5.3) specific type compatibility rules apply: a referenced type, template, constant, variable or module parameter of the type charstring always can be used in the pattern specification of a template or template field of universal charstring type; a referenced type, template or value of the type universal charstring can be used in the pattern specification of a template or template field of charstring type if all characters used in the referenced template or value and the character set allowed by the referenced type has their corresponding characters in the charstring type (see definition of corresponding characters in clause 6.3.1).

### B.1.5.6	Case insensitive pattern matching

When the "@nocase" modifier is used after the pattern keyword, the matching is evaluated in a case insensitive way, i.e. at positions, where without the "@nocase" modifier a small letter alphabetical character would be matched, with the "@nocase" modifier also capital letter counterpart - but only that - shall be accepted. For example, at positions where the pattern matches the character đ (latin small letter d with stroke), also its counterpart Đ (latin capital letter d with stroke) shall be accepted, but the similarly looking graphical characters Ð (latin capital letter eth) and Ɖ (latin capital letter african d) shall not.

EXAMPLE 1:

template charstring mw_myTemplateNoCase:= pattern @nocase "ab??xyz*0";

//This template would match any character string that start with the characters "ab" or "Ab"

//or "aB" or "AB", followed by any two characters, followed by the characters "xyz" or "Xyz"

//or "xYz" or "xyZ" or "XYz" or "xYZ" or "XyZ" or "XYZ", followed by any number of any

//characters (including any number of "0"-s) before the closing character "0".

When referencing a pattern from inside another pattern (see clause 27.9), the case sensitivity property of the referenced pattern is not inherited. I.e. - after dereferencing, possibly recursively - only the resulting string part of the referenced pattern is inserted into the referencing pattern. The whole resulting pattern is always evaluated according to the case-sensitivity of the referencing pattern.

EXAMPLE 2:

const charstring c_myString:= "ab?";

template charstring mw_myTemplate:= pattern @nocase "{c_myString}";

//matches any character string that consists of the characters "ab" or "Ab" or "aB" or "AB",

// followed by any character

template universal charstring mw_myTemplate3:= pattern "{mw_myTemplate}de\q{1, 1, 13, 7}";

//matches any character string which consists of the characters "ab", followed by any

//character, followed by the characters "de", followed by the character in ISO10646-1 with

//group=1, plane=1, row=13 and cell=7.

Annex C (normative):
Predefined TTCN-3 functions

# C.0	General exception handling procedures

This annex defines the TTCN-3 predefined functions.

When the general restrictions specified in clause 16.1.2 are not met, this shall cause a compile time or runtime error. Error situations for which no explicit exception-handling rule is defined in the relevant clauses of this annex shall cause a TTCN-3 compile-time or runtime error. Which error situation causes compile-time and which one runtime error is a tool implementation option.

# C.1	Conversion functions

## C.1.1	Integer to character

int2char(in integer invalue) return charstring

This function converts an integer value in the range of 0 to 127 (8-bit encoding) into a single-character-length charstring value. The integer value describes the 8-bit encoding of the character.

In addition to the general error causes in clause 16.1.2, error causes are:

invalue is less than 0 or greater than 127.

## C.1.2	Integer to universal character

int2unichar(in integer invalue) return universal charstring

This function converts an integer value in the range of 0 to 2 147 483 647 (32-bit encoding) into a single-character-length universal charstring value. The integer value describes the 32-bit encoding of the character.

In addition to the general error causes in clause 16.1.2, error causes are:

invalue is less than 0 or greater than 2147483647.

## C.1.3	Integer to bitstring

int2bit(in integer invalue, in integer length) return bitstring

This function converts a single integer value to a single bitstring value. The resulting string is length bits long.

For the purposes of this conversion, a bitstring shall be interpreted as a positive base 2 integer value. The rightmost bit is least significant, the leftmost bit is the most significant. The bits 0 and 1 represent the decimal values 0 and 1 respectively. If the conversion yields a value with fewer bits than specified in the length parameter, then the bitstring shall be padded on the left with zeros.

In addition to the general error causes in clause 16.1.2, error causes are:

invalue is less than zero;

the conversion yields a return value with more bits than specified by length.

## C.1.4	Integer to enumerated

int2enum ( in integer inpar, out Enumerated_type outpar)

This function converts an integer value into an enumerated value of a given enumerated type. The integer value shall be provided as in parameter and the result of the conversion shall be stored in an out parameter. The type of the out parameter determines the type into which the in parameter is converted.

The general error causes in clause 16.1.2 apply.

EXAMPLE:

type enumerated MyFirstEnumType {

Monday, Tuesday, Wednesday, Thursday, Friday, Weekend(6..7)

};

type enumerated MySecondEnumType {

Saturday(-3), Sunday (0), Monday

};

//within a dynamic language element:

var MyFirstEnumType v_firstEnum := Tuesday;

var MySecondEnumType v_secondEnum := Sunday;

int2enum(0, v_firstEnum) 	// v_firstEnum == Monday

int2enum(1, v_secondEnum)   // v_secondEnum == Monday

int2enum(6, v_firstEnum)    // v_firstEnum == Weekend(6)

## C.1.5	Integer to hexstring

int2hex(in integer invalue, in integer length) return hexstring

This function converts a single integer value to a single hexstring value. The resulting string is length hexadecimal digits long.

For the purposes of this conversion, a hexstring shall be interpreted as a positive base 16 integer value. The rightmost hexadecimal digit is least significant, the leftmost hexadecimal digit is the most significant. The hexadecimal digits 0 to F represent the decimal values 0 to 15 respectively. If the conversion yields a value with fewer hexadecimal digits than specified in the length parameter, then the hexstring shall be padded on the left with zeros.

In addition to the general error causes in clause 16.1.2, error causes are:

invalue is less than zero;

the conversion yields a return value with more hexadecimal characters than specified by length.

## C.1.6	Integer to octetstring

int2oct(in integer invalue, in integer length) return octetstring

This function converts a single integer value to a single octetstring value. The resulting string is length octets long.

For the purposes of this conversion, an octetstring shall be interpreted as a positive base 16 integer value. The rightmost hexadecimal digit is least significant, the leftmost hexadecimal digit is the most significant. The number of hexadecimal digits provided shall be multiples of 2 since one octet is composed of two hexadecimal digits. The hexadecimal digits 0 to F represent the decimal values 0 to 15 respectively. If the conversion yields a value with fewer hexadecimal digits than specified in the length parameter, then the hexstring shall be padded on the left with zeros.

In addition to the general error causes in clause 16.1.2, error causes are:

invalue is less than zero;

the conversion yields a return value with more octets than specified by length.

## C.1.7	Integer to charstring

int2str(in integer invalue) return charstring

This function converts the integer value into its string equivalent (the base of the return string is always decimal).

The general error causes in clause 16.1.2 apply.

EXAMPLE:

int2str(66) 	// will return the charstring value "66"

int2str(-66) 	// will return the charstring value "-66"

int2str(0) 		// will return the charstring value "0"

## C.1.8	Integer to float

int2float(in integer invalue) return float

This function converts an integer value into a float value.

The general error causes in clause 16.1.2 apply.

EXAMPLE:

int2float(4) = 4.0

## C.1.9	Float to integer

float2int(in float invalue) return integer

This function converts a float value into an integer value by removing the fractional part of the argument and returning the resulting integer.

In addition to the general error causes in clause 16.1.2, error causes are:

invalue is infinity, -infinity or not_a_number.

- EXAMPLE:

float2int(3.12345E2) = float2int(312.345) = 312

## C.1.10	Character to integer

char2int(in charstring invalue) return integer

This function converts a single-character-length charstring value into an integer value in the range of 0 to 127. The integer value describes the 8-bit encoding of the character.

In addition to the general error causes in clause 16.1.2, error causes are:

length of invalue does not equal 1.

## C.1.11	Character to octetstring

char2oct(in charstring invalue) return octetstring

This function converts a charstring invalue to an octetstring. Each octet of the octetstring will contain the Recommendation ITU-T T.50 [4] codes (according to the IRV) of the appropriate characters of invalue.

The general error causes in clause 16.1.2 apply.

EXAMPLE:

char2oct ("Tinky-Winky") = '54696E6B792D57696E6B79'O

## C.1.12	Universal character to integer

unichar2int(in universal charstring invalue) return integer

This function converts a single-character-length universal charstring value into an integer value in the range of 0 to 2 147 483 647. The integer value describes the 32-bit encoding of the character.

In addition to the general error causes in clause 16.1.2, error causes are:

length of invalue does not equal 1.

## C.1.13	Bitstring to integer

bit2int(in bitstring invalue) return integer

This function converts a single bitstring value to a single integer value.

For the purposes of this conversion, a bitstring shall be interpreted as a positive base 2 integer value. The rightmost bit is least significant, the leftmost bit is the most significant. The bits 0 and 1 represent the decimal values 0 and 1 respectively.

NOTE:	On real test systems the integer interpretation of invalue may lead to an overflow problem that causes compile time or runtime error. However, this is out of the scope of the present document.

The general error causes in clause 16.1.2 apply.

## C.1.14	Bitstring to hexstring

bit2hex(in bitstring invalue) return hexstring

This function converts a single bitstring value to a single hexstring. The resulting hexstring represents the same value as the bitstring.

For the purpose of this conversion, a bitstring shall be converted into a hexstring, where the bitstring is divided into groups of four bits beginning with the rightmost bit. Each group of four bits is converted into a hex digit as follows:

'0000'B  '0'H,	'0001'B  '1'H,	'0010'B  '2'H,	'0011'B  '3'H,	'0100'B  '4'H,	'0101'B  '5'H,

'0110'B  '6'H,	'0111'B  '7'H,	'1000'B  '8'H,	'1001'B  '9'H,	'1010'B  'A'H,	'1011'B  'B'H,

'1100'B  'C'H,	'1101'B  'D'H,	'1110'B  'E'H, and '1111'B  'F'H.

When the leftmost group of bits does contain less than 4 bits, this group is filled with '0'B from the left until it contains exactly 4 bits and is converted afterwards. The consecutive order of hex digits in the resulting hexstring is the same as the order of groups of 4 bits in the bitstring.

The general error causes in clause 16.1.2 apply.

EXAMPLE:

bit2hex ('111010111'B)= '1D7'H

## C.1.15	Bitstring to octetstring

bit2oct(in bitstring invalue) return octetstring

This function converts a single bitstring value to a single octetstring. The resulting octetstring represents the same value as the bitstring.

For the conversion the following holds: bit2oct(value)=hex2oct(bit2hex(value)).

The general error causes in clause 16.1.2 apply.

EXAMPLE:

bit2oct('111010111'B)= '01D7'O

## C.1.16	Bitstring to charstring

bit2str(in bitstring invalue) return charstring

This function converts a single bitstring value to a single charstring. The resulting charstring has the same length as the bitstring and contains only the characters '0' and '1'.

For the purpose of this conversion, a bitstring shall be converted into a charstring. Each bit of the bitstring is converted into a character '0' or '1' depending on the value 0 or 1 of the bit. The consecutive order of characters in the resulting charstring is the same as the order of bits in the bitstring.

The general error causes in clause 16.1.2 apply.

EXAMPLE:

bit2str('1110101'B) will return "1110101"

## C.1.17	Hexstring to integer

hex2int(in hexstring invalue) return integer

This function converts a single hexstring value to a single integer value.

For the purposes of this conversion, a hexstring shall be interpreted as a positive base 16 integer value. The rightmost hexadecimal digit is least significant, the leftmost hexadecimal digit is the most significant. The hexadecimal digits 0 to F represent the decimal values 0 to 15 respectively.

NOTE:	On real test systems the integer interpretation of invalue may lead to an overflow problem that causes compile time or runtime error. However, this is out of the scope of the present document.

The general error causes in clause 16.1.2 apply.

## C.1.18	Hexstring to bitstring

hex2bit(in hexstring invalue) return bitstring

This function converts a single hexstring value to a single bitstring. The resulting bitstring represents the same value as the hexstring.

For the purpose of this conversion, a hexstring shall be converted into a bitstring, where the hex digits of the hexstring are converted in groups of bits as follows:

'0'H  '0000'B,	'1'H  '0001'B,	'2'H  '0010'B,	'3'H  '0011'B,	'4'H  '0100'B,	'5'H  '0101'B,

'6'H  '0110'B,	'7'H  '0111'B,	'8'H  '1000'B,	'9'H  '1001'B,	'A'H  '1010'B,	'B'H  '1011'B,

'C'H  '1100'B,	'D'H  '1101'B,	'E'H  '1110'B, and 'F'H  '1111'B.

The consecutive order of the groups of 4 bits in the resulting bitstring is the same as the order of hex digits in the hexstring.

The general error causes in clause 16.1.2 apply.

EXAMPLE:

hex2bit('1D7'H)= '000111010111'B

## C.1.19	Hexstring to octetstring

hex2oct(in hexstring invalue) return octetstring

This function converts a single hexstring value to a single octetstring. The resulting octetstring represents the same value as the hexstring.

For the purpose of this conversion, a hexstring shall be converted into a octetstring, where the octetstring contains the same sequence of hex digits as the hexstring when the length of the hexstring modulo 2 is 0. Otherwise, the resulting octetstring contains 0 as leftmost hex digit followed by the same sequence of hex digits as in the hexstring.

The general error causes in clause 16.1.2 apply.

EXAMPLE:

hex2oct('1D7'H)= '01D7'O

## C.1.20	Hexstring to charstring

hex2str(in hexstring invalue) return charstring

This function converts a single hexstring value to a single charstring. The resulting charstring has the same length as the hexstring and contains only the characters '0' to '9'and 'A' to 'F'.

For the purpose of this conversion, a hexstring shall be converted into a charstring. Each hex digit of the hexstring is converted into a character '0' to '9' and 'A' to 'F' depending on the value 0 to 9 or A to F of the hex digit. The consecutive order of characters in the resulting charstring is the same as the order of digits in the hexstring.

The general error causes in clause 16.1.2 apply.

EXAMPLE:

hex2str('AB801'H) // will return "AB801"

## C.1.21	Octetstring to integer

oct2int(in octetstring invalue) return integer

This function converts a single octetstring value to a single integer value.

For the purposes of this conversion, an octetstring shall be interpreted as a positive base 16 integer value. The rightmost hexadecimal digit is least significant, the leftmost hexadecimal digit is the most significant. The number of hexadecimal digits provided shall be multiples of 2 since one octet is composed of two hexadecimal digits. The hexadecimal digits 0 to F represent the decimal values 0 to 15 respectively.

NOTE:	On real test systems the integer interpretation of invalue may lead to an overflow problem that causes compile time or runtime error. However, this is out of the scope of the present document.

The general error causes in clause 16.1.2 apply.

## C.1.22	Octetstring to bitstring

oct2bit(in octetstring invalue) return bitstring

This function converts a single octetstring value to a single bitstring. The resulting bitstring represents the same value as the octetstring.

For the conversion the following holds: oct2bit(value)=hex2bit(oct2hex(value)).

The general error causes in clause 16.1.2 apply.

EXAMPLE:

oct2bit ('01D7'O)='0000000111010111'B

## C.1.23	Octetstring to hexstring

oct2hex(in octetstring invalue) return hexstring

This function converts a single octetstring value to a single hexstring. The resulting hexstring represents the same value as the octetstring.

For the purpose of this conversion, a octetstring shall be converted into a hexstring containing the same sequence of hex digits as the octetstring.

The general error causes in clause 16.1.2 apply.

EXAMPLE:

oct2hex('1D74'O)= '1D74'H

## C.1.24	Octetstring to character string

oct2str(in octetstring invalue) return charstring

This function converts an octetstring invalue to an charstring representing the string equivalent of the input value. The resulting charstring shall have the same length as the incoming octetstring.

For the purpose of this conversion each hex digit of invalue is converted into a character '0', '1', '2', '3', '4', '5', '6', '7', '8', '9', 'A', 'B', 'C', 'D', 'E' or 'F' echoing the value of the hex digit. The consecutive order of characters in the resulting charstring is the same as the order of hex digits in the octetstring.

The general error causes in clause 16.1.2 apply.

EXAMPLE:

oct2str('4469707379'O) = "4469707379"

## C.1.25	Octetstring to character string, version II

oct2char(in octetstring invalue) return charstring

This function converts an octetstring invalue to a charstring. The input parameter invalue shall not contain octet values higher than 7F. The resulting charstring shall have the same length as the input octetstring. The octets are interpreted as Recommendation ITU-T T.50 [4] codes (according to the IRV) and the resulting characters are appended to the returned value.

The general error causes in clause 16.1.2 apply.

EXAMPLE:

oct2char('4469707379'O) = "Dipsy"

NOTE:	The character string returned may contain non-graphical characters, which cannot be presented between the double quotes.

## C.1.26	Charstring to integer

str2int(in charstring invalue) return integer

This function converts a charstring representing an integer value to the equivalent integer.

In addition to the general error causes in clause 16.1.2, error causes are:

invalue contains characters other than "0", "1", "2", "3", "4", "5", "6", "7", "8", "9" and "-".

invalue contains the character "-" at another position than the leftmost one.

NOTE:	On real test systems the integer interpretation of invalue may lead to an overflow problem that causes compile time or runtime error. However, this is out of the scope of the present document.

EXAMPLE:

str2int("66") 	// will return the integer value 66

str2int("-66") 	// will return the integer value -66

str2int("6-6") 	// will cause an error

str2int("abc") 	// will cause an error

str2int("0") 	// will return the integer value 0

## C.1.27	Character string to hexstring

str2hex(in charstring invalue) return hexstring

This function converts a string of the type charstring to a hexstring. The string invalue shall contain the "0", "1", "2", "3", "4", "5", "6", "7", "8", "9", "a", "b", "c", "d", "e" "f", "A", "B", "C", "D", "E" or "F" graphical characters only. Each character of invalue shall be converted to the corresponding hexadecimal digit. The resulting hexstring will have the same length as the incoming charstring.

In addition to the general error causes in clause 16.1.2, error cause is:

invalue contains characters other than specified above.

EXAMPLE:

str2hex("54696E6B792D57696E6B7") = '54696E6B792D57696E6B7'H

## C.1.28	Character string to octetstring

str2oct(in charstring invalue) return octetstring

This function converts a string of the type charstring to an octetstring. The string invalue shall contain the "0", "1", "2", "3", "4", "5", "6", "7", "8", "9", "a", "b", "c", "d", "e" "f", "A", "B", "C", "D", "E" or "F" graphical characters only. When the string invalue contains even number characters the resulting octetstring contains 0 as leftmost character followed by the same sequence of characters as in the charstring.

lengthof (see clause C.2.1 for the resulting octetstring) will return half of lengthof of the incoming charstring. In addition to the general error causes in clause 16.1.2, error causes is:

invalue contains characters other than specified above.

EXAMPLE:

str2oct("54696E6B792D57696E6B79") = '54696E6B792D57696E6B79'O

str2oct("1D7")= '01D7'O

NOTE:	The semantic of the str2oct function cause asymmetric behaviour:

oct2str(str2oct("1D7"))// results in the charstring value "01D7"

## C.1.29	Character string to float

str2float(in charstring invalue) return float

This function converts a charstring comprising a number into a float value. The format of the number in the charstring shall follow rules in clause 6.1.0, items a) or b) with the following exceptions:

leading zeros are allowed;

leading "+" sign before positive values is allowed;

"-0.0" is allowed;

no numbers after the dot in the decimal notation are allowed.

In addition to the general error causes in clause 16.1.2, error causes are:

the format of invalue is different than defined above.

NOTE:	On real test systems the float interpretation of invalue may lead to an overflow problem that causes compile time or runtime error. However, this is out of the scope of the present document.

EXAMPLE:

str2float("12345.6") 	// is the same as str2float("123.456E+02")

str2float("1.6") 		// returns a float value equal to 1.6

str2float("+001.") 		// returns a float value equal to 1.0

str2float("+001") 		// returns a float value equal to 1.0

str2float("-0.0") 		// returns a float value equal to -0.0

## C.1.30	Enumerated to integer

enum2int(in Enumerated_type inpar) return integer

This function accepts an enumerated value and returns the integer value associated to the enumerated value (see also clause 6.2.4). The actual parameter passed to inpar always shall be a typed object (see clause 6.2.4 and the definition "type context" in clause 3.1).

The general error causes in clause 16.1.2 apply.

EXAMPLE:

type enumerated MyFirstEnumType {

Monday, Tuesday, Wednesday, Thursday, Friday, Weekend(5..6)

};

type enumerated MySecondEnumType {

Saturday(-3), Sunday (0), Monday

};

//within a dynamic language element:

var MyFirstEnumType v_firstEnum := Monday;

var MySecondEnumType v_secondEnum := Monday;

enum2int(v_firstEnum)  // returns 0

enum2int(v_secondEnum) // returns 1

v_firstEnum := Wednesday;

v_secondEnum := Saturday;

enum2int(v_firstEnum)  // returns 2

enum2int(v_secondEnum) // returns -3

v_firstEnum := Friday;

v_secondEnum := Sunday;

enum2int(v_firstEnum)  // returns 4

enum2int(v_secondEnum) // returns 0

v_firstEnum := Weekend(6);

enum2int(v_firstEnum)  // returns 6

## C.1.31	Octetstring to universal character string

oct2unichar(in octetstring invalue, in charstring string_encoding := "UTF-8")

return universal charstring

This function converts an octetstring invalue to a universal charstring by use of the given string_encoding.  The octets are interpreted as mandated by the standardized mapping associated with the given string_encoding and the resulting characters are appended to the returned value. If the optional string_encoding parameter is omitted, the default value "UTF-8".

The following values (see ISO/IEC 10646 [2]) are allowed as string_encoding actual parameters (for the description of the codepoints see clause 27.5):

- "UTF-8"

"UTF-16"

"UTF-16LE"

"UTF-16BE"

"UTF-32"

"UTF-32LE"

"UTF-32BE"

The invalue parameter shall not include the optional signature (see clause 10 of ISO/IEC 10646 [2], also known as byte order mark).

In case of "UTF-16" and "UTF-32" big-endian ordering shall be used (as described in clauses 10.4 and 10.7 of ISO/IEC 10646 [2]).

The general error causes in clause 16.1.2 apply.

EXAMPLE:

oct2unichar('C384C396C39CC3A4C3B6C3BC'O) = "ÄÖÜäöü";

oct2unichar('00C400D600DC00E400F600FC'O,"UTF-16BE") = "ÄÖÜäöü";

oct2unichar('C400D600DC00E400F600FC00'O,"UTF-16LE") = "ÄÖÜäöü";

NOTE:	The character string returned may contain non-graphical characters, which cannot be presented between the double quotes.

## C.1.32	Universal character string to octetstring

unichar2oct(in universal charstring invalue, in charstring string_encoding := "UTF-8")

return octetstring

This function converts a universal charstring invalue to an octetstring. Each octet of the octetstring will contain the octets mandated by mapping the characters of invalue using the standardized mapping associated with the given string_encoding in the same order as the characters appear in inpar. If the optional string_encoding parameter is omitted, the default value "UTF-8".

The following values (see ISO/IEC 10646 [2]) are allowed as string_encoding actual parameters (for the description of the UCS encoding scheme see clause 27.5):

- "UTF-8"

- "UTF-16"

- "UTF-16LE"

- "UTF-16BE"

- "UTF-32"

- "UTF-32LE"

- "UTF-32BE"

The general error causes in clause 16.1.2 apply.

EXAMPLE:

unichar2oct("ÄÖÜäöü") = 'C384C396C39CC3A4C3B6C3BC'O;

unichar2oct("ÄÖÜäöü","UTF-16BE") = '00C400D600DC00E400F600FC'O;

unichar2oct("ÄÖÜäöü","UTF-16LE") = 'C400D600DC00E400F600FC00'O;

## C.1.33	Value or template to universal charstring

any2unistr(in template any invalue,

in universal charstring format := "") return universal charstring

This function converts the content of a value or template to a single universal charstring. The resulting universal charstring is the same as the string produced by the log operation containing the same operand as the one passed to the any2unistr function. The value or template passed as a parameter to the any2unichar function may be uninitialized, partially or completely initialized.

The optional format parameter is used for dynamic selection of how the resulting universal charstring should be produced from the provided invalue. When the optional format parameter is missing or is set to an empty string the resulting universal charstring is the same as the string produced by the log operation for the same operand. When the optional format parameter takes the value "canonical": unbound fields are represented in the output as "-", the fields and members of structured types are represented recursively in assignment notation.

If the actual value of the format parameter is different from the empty string and "canonical", the format of the result produced by the function is either tool-specific if the tool supports the specified custom format or it is the same as the string produced by the log operation (i.e. the same as if the parameter value was the empty string).

The general error causes in clause 16.1.2 apply.

EXAMPLE:

var integer v_int1 := 5, v_int2;

var template integer mw_int1 := ?;

var template integer mw_int2 := -1 ifpresent;

var universal charstring v_chr1, v_chr2, v_chr3, v_chr4;

v_chr1 := any2unistr(v_int1); // after the assignment v_chr1 will be "5"

v_chr2 := any2unistr(v_int2); // after the assignment v_chr2 will be "UNINITIALIZED"

v_chr3 := any2unistr(mw_int1); // after the assignment v_chr3 will be "?"

v_chr4 := any2unistr(mw_int2); // after the assignment v_chr3 will be "-1 ifpresent"

type record MyRecord {

integer field1,

integer field2

}

var template MyRecord t1;

var template MyRecord t2 := {1};

v_chr1 := any2unistr(t1); // after the assignment v_chr1 will be "UNINITIALIZED"

v_chr1 := any2unistr(t2);

// after the assignment v_chr1 could be "{ field1 := 1, field2 := UNINITIALIZED }"

v_chr1 := any2unistr(t1, "canonical"); // after the assignment v_chr1 will be "-"

v_chr1 := any2unistr(t2, "canonical");

// after the assignment v_chr1 will be "{ fielfd1 := 1, field2 := - }"

# C.2	Length/size functions

## C.2.1	Length of strings and lists

lengthof(in template (present) any inpar) return integer

This function returns the length of a value or template that is of type bitstring, hexstring, octetstring, charstring, universal charstring, record of, set of, or array. The units of length for each string type are defined in table 4 in the present document.

For values or templates of record of or set of type, the value to be returned is the maximum of the minimal length restriction value of the type, or 0 for types with no minimal length restriction, and the index of the last initialized element plus 1.

The length value returned in case of length restricted string or list type shall be at least the minimum length according to the type definition. In particular, the length of a fixed length record of or set of value will always be the fixed length according to the type definition. For array values or templates, the value to be returned is the fixed length of the corresponding record of type.

NOTE 1:	As in formal parameters does not allow passing in uninitialized values or templates, even in these cases inpar will be at least partially initialized.

The length of an universal charstring shall be calculated by counting each combining character and hangul syllable character (including fillers) on its own (see ISO/IEC 10646 [2], clauses 23 and 24).

When the function lengthof is applied to string-type templates, inpar shall only contain the following matching mechanisms: specific value, value list, complemented value list, pattern, "?" (AnyValue instead of value), "*" (AnyValueOrNone instead of value), combined template and the length restriction matching attribute. In case of string-type templates inpar shall match values of the same length only.

When the function lengthof is applied to templates of record of or set of types, inpar shall only contain the following matching mechanisms: specific value, value list, complemented value list, "?" (AnyValue instead of value), "*" (AnyValueOrNone instead of value), SuperSet, SubSet, combined template, permutation and the length restriction matching attribute. The parameter inpar shall only match values, for which the lengthof function would give the same result. If inpar contains uninitialized elements, each of them shall be counted as 1 element, i.e. they shall be matched as if they contained the "?" (AnyElement inside value) matching character.

NOTE 2:	In case of record ofs and set ofs and arrays only elements of the TTCN-3 object, which is the parameter of the function are calculated; i.e. no elements of nested types are taken into account when determining the return value.

In addition to the general error causes in clause 16.1.2, error causes are:

inpar is a string-type template and it can match string values with different length or the length restriction matching attribute contradicts the number of string elements in the template body;

inpar is a record of or set of type template and it can match values of different lengths or the length restriction matching attribute contradicts the number of elements in the template body;

the template passed to the invalue parameter is not of type bitstring, hexstring, octetstring, charstring, universal charstring, record of, set of, or array.

NOTE 3:	On real test systems the length calculation of inpar may lead to an overflow problem that causes compile time or runtime error. However, this is out of the scope of the present document.

EXAMPLE 1:	Using lengthof for values

lengthof('010'B) 	// returns 3

lengthof('F3'H) 	// returns 2

lengthof('F2'O) 	// returns 1

lengthof (universal charstring : "Length_of_Example") // returns 17

// Given

type record length(0..10) of integer MyList;

var MyList v_myListVar := { 0, 1, -, 2, - };

lengthof(v_myListVar);

// returns 4 without respect to the fact, that the element v_myListVar[2] is not initialized

EXAMPLE 2:	Using lengthof for string-type templates

lengthof(charstring : "HELLO") 			// returns 5

lengthof(octetstring : ('12'O, '34'O)) 	// returns 1

lengthof('1??1'B) 						// returns 4

lengthof(universal charstring : ? length(8)) // returns 8

lengthof('1*F'H)  						// shall cause an error

lengthof('1*F'H length (8))  			// returns 8

lengthof(bitstring : ? length(2..infinity)) // shall cause an error

lengthof('00*FF'O length(1..2)) 		    // returns 2

lengthof('1*49'H length(1..2)) 		    // shall cause an error

lengthof('1'B length(3)) 				// shall cause an error

lengthof('1*1'B length(10..20)) 		    // shall cause an error

EXAMPLE 3:

type record of integer RoI;

template RoI mw_roI1 := { 1, permutation(2, 3), ? }

template RoI mw_roI2 := {1, *, (2, 3) }

template RoI mw_roI3 := { 1, *, 10 } length(5)

template RoI mw_roI4 := { 1, 2, 3, * } length(1..2)

template RoI mw_roI5 := { 1, 2, 3, * } length(1..3)

lengthof (mw_roI1) // returns 4

lengthof (mw_roI2) // shall cause an error

lengthof (mw_roI3) // returns 5

lengthof (mw_roI4) // shall cause an error

lengthof (mw_roI5) // returns 3

## C.2.2	Number of elements in a structured value

sizeof(in template (present) any inpar) return integer

This function returns the actual number of elements of a value or template of a record or set type (see note).

The function sizeof is applicable to templates of record and set types. The function is applicable only if the sizeof function gives the same result on all values that match the template.

NOTE:	Only elements of the TTCN-3 object, which is the parameter of the function are calculated; i.e. no elements of nested types/values are taken into account at determining the return value.

In addition to the general error causes in clause 16.1.2, error causes are:

when inpar is a template and it can match values of different sizes;

the template passed to the inpar parameter is not of a record or set type.

EXAMPLE:

// Given

type record MyPDU

{ 	boolean field1  optional,

integer field2

};

template MyPDU m_myTemplate :=

{ field1 := omit,

field2 :=	5

};

sizeof(m_myTemplate);	// returns 1

type set S {

integer f1,

bitstring f2 optional,

charstring f3 optional

}

template S mw_s1 := { f1 := (0..99), f2 := omit, f3 := ? }

template S mw_s2 := { f3 := *, f1 := 1, f2 := '00'B ifpresent }

template S mw_s3 := ({ f1 := 1, f2 := omit, f3 := "ABC" },

{ f1 := 2, f3 := omit, f2 := '1'B },

{ f3 := omit, f1 := 3, f2 := '1?1'B }

)

template S mw_s4 := ?

sizeof(mw_s1) // returns 2

sizeof(mw_s2) // shall cause an error

sizeof(mw_s3) // returns 2

sizeof(mw_s4) // shall cause an error

# C.3	Presence checking functions

## C.3.1	Void

## C.3.2	Void

## C.3.3	Void

## C.3.4	Void

## C.3.5	Matching mechanism detection

istemplatekind (in template any_type invalue, in charstring kind) return boolean

This function allows to examine if a template contains a certain kind of the matching mechanisms.

If the matching mechanism kind enquired is matching a specific value (clause B.1.1), a matching mechanism instead of values (clause B.1.2) or matching character pattern (clause B.1.5), the function shall return true if the content of the invalue parameter is of the same kind.

If the matching mechanism kind enquired is a matching mechanism inside values (clause B.1.3), the function shall return true if the template in the invalue parameter contains this kind of matching mechanism on the first level of nesting.

If the matching mechanism kind enquired is a matching attribute (clause B.1.4), the function shall return true if the template in the invalue parameter has this kind of matching attribute attached to it directly (i.e. it doesn't count if the attribute is attached to a field of invalue at any level of nesting).

In all other cases the function returns false.

The kind parameter shall be one of the strings listed in table C.1.

Table C.1: Allowed values of kind parameter

| Value of kind parameter | Searched matching mechanism |  |
| --- | --- | --- |
|  | Name | Clause reference |
| "value" | Specific value | B.1.1 |
| "list" | Template list | B.1.2.1 |
| "complement" | Complemented template list | B.1.2.2 |
| "AnyValue", "?" | Any value | B.1.2.3 |
| "AnyValueOrNone", "*" | Any value or none | B.1.2.4 |
| "range" | Value range | B.1.2.5 |
| "superset" | SuperSet | B.1.2.6 |
| "subset" | SubSet | B.1.2.7 |
| "omit" | Omit | B.1.2.8 |
| "decmatch" | Matching decoded content | B.1.2.9 |
| "AnyElement" | Any element | B.1.3.1 |
| "AnyElementsOrNone" | Any number of elements or none | B.1.3.2 |
| "permutation" | Permutation | B.1.3.3 |
| "length" | Length restriction | B.1.4.1 |
| "ifpresent" | The IfPresent indicator | B.1.4.2 |
| "pattern" | Matching character pattern | B.1.5 |

NOTE:	Clause E.2.2.5 includes the type definition TemplateKind and a constant for each of the allowed values of the kind parameter. It is recommended to use the istemplatekind function in combination with this type and these constants to ease the checking of correct usage and to improve the readability of test specs.

Restrictions

In addition to the general error causes given in clause 16.1.2, the following restrictions apply:

- Calling the istemplatekind function with a different second parameter than stated in table C.1 shall lead to an error.

EXAMPLE:

type record of integer RoI;

...

var template integer v_t1 := ?, v_t2 := (0,1,2) ifpresent;

var template RoI v_t3:= { permutation(1, 2, 3), ? };

var boolean v_res;

...

v_res := istemplatekind(v_t1, "AnyValue");       // true

v_res := istemplatekind(v_t1, "AnyValueOrNone"); // false

v_res := istemplatekind(v_t2, "complement");     // false

v_res := istemplatekind(v_t2, "list");           // true

v_res := istemplatekind(v_t2, "ifpresent");      // true

v_res := istemplatekind(v_t3, "permutation");    // true

v_res := istemplatekind(v_t3, "AnyElement");     // true

# C.4	String/list handling functions

## C.4.1	The Regexp function

regexp [@nocase] (

in template (value) any inpar,

in template (present) any expression,

in integer groupno

) return any_character_string_type

This function first matches the parameter inpar (or in case inpar is a template, its value equivalent) against the expression in the second parameter according to the pattern matching specified in clause B.1.5. If expression is not a template containing a pattern matching mechanism, it shall be processed by this predefined function as if it was a character pattern as described in clause B.1.5. If the @nocase modifier is present, this and all subsequent matchings shall be done in a case-insensitive way, as specified in clause B.1.5.6. If inpar is a literal (i.e. type is not explicitly given) the corresponding type shall be retrieved from the value contents.

If this matching is unsuccessful, an empty string shall be returned.

If this matching is successful, the substring of inpar shall be returned, which matched the groupno-s group of expression during the matching. Group numbers are assigned by the order of occurrences of the opening bracket of a group and counted starting from 0 by step 1.

The parameters inpar and expression shall be a value or a template of charstring or universal charstring types. In case inpar is a template, it shall contain the specific value matching mechanism only. When expression is a template it shall contain the specific value or pattern matching mechanisms only. The parameter groupno shall be a non-negative integer. The type of the character string returned is the root type of inpar.

NOTE:	This function differs from other well-known regular expression matching implementations in that:

a)	It shall match the whole inpar string instead of only a substring.

b)	It starts counting groups from 0, while in some other implementations the first group is referenced by 1 and the whole substring matched by the expression is referenced by 0.

In addition to the general error causes in clause 16.1.2, error causes are:

when inpar is a template, it contains other matching mechanism than specific value or character pattern;

when expression is a template, it contains other matching mechanism than specific value or character pattern;

inpar is of charstring type and expression is of universal charstring type;

groupno is a negative integer;

there is no groupno -s group in expression.

EXAMPLE:

// Given

var charstring v_myInput := "   simple text for a regexp example   ";

var charstring v_myString;

v_myString := regexp(v_myInput,charstring:"?+(text)?+",0);

// will return "text"

v_myString := regexp(v_myInput,charstring:"?+(text)?+",1);

// causes an error as there is no group with index 1

v_myString := regexp(v_myInput,charstring:"(?+)(text)(?+)",0);

// will return "   simple "

v_myString := regexp(v_myInput,charstring:"(?+)(text)(?+)",2);

// will return " for a regexp example   "

v_myString := regexp(v_myInput,charstring:"((?+)(text)(?+))",0);

// will return the whole inpar, i.e. "   simple text for a regexp example   "

v_myString := regexp(v_myInput,charstring:"(([ ]+)(text)(?+))",0);

// will return an empty string as expression does not matches inpar

v_myString := regexp(v_myInput,universal charstring:"?+(text)?+",0);

// will cause an error as inpar is of type charstring, while

// expression is of type universal charstring

v_myInput := "      date: 2001-10-20 ;  msgno: 17; exp  ";

var template charstring v_myPattern :=

pattern "([ \t]#(0,)date:[ \d\-]#(0,);[ \t]#(0,)msgno: (\d#(1,3)); (exp)#(0,1)) [ \t]#(0,)";

// please note, that only the very first opening bracket and the bracket before "\d#(1,3)"

// denotes groups; "#(0,)", "#(1,3)" and "#(0,1)" denotes matching the preceding expression

// several time

v_myString := regexp(v_myInput, v_myPattern,0);

// will return the input string but the whitespace at the end,

// i.e. "      date: 2001-10-20 ;  msgno: 17; exp"

v_myString := regexp(v_myInput, v_myPattern,1);

// will return the value "17"

//An example of a wrapper function to count groups from 1 and return the complete p_inpar

//if p_groupno equals 0

function f_regexp0(

in template charstring p_inpar,

in template charstring p_expression,

in integer p_groupno)

return charstring {

var template charstring v_extendedExpr := pattern "({p_expression})";

return regexp(p_inpar, v_extendedExpr, p_groupno )

}

## C.4.2	The Substring function

substr(

in template (present) any inpar,

in integer index,

in integer count

) return input_string_or_sequence_type

This function returns a substring or subsequence from a value that is of a binary string type (bitstring, hexstring, octetstring), a character string type (charstring, universal charstring), or a sequence type (record of, set of or array). If inpar is a literal (i.e. type is not explicitly given) the corresponding type shall be retrieved from the value contents. The type of the substring or subsequence returned is the root type of the input parameter. The starting point of substring or subsequence to return is defined by the second parameter (index). Indexing starts from zero. The third input parameter (count) defines the length of the substring or subsequence to be returned. The units of length for string types are as defined in table 4 of the present document. For sequence types, the unit of length is element.

NOTE:	Please note that the root types of arrays is record of, therefore if inpar is an array the returned type is record of. This, in some cases, may lead to different indexing in inpar and in the returned value.

When used on templates of character string types, specific values and patterns that contain literal characters and the following metacharacters: "?", "*" are allowed in inpar and the function shall return the character representation of the matching mechanisms. When inpar is a template of binary string or sequence type or is an array, only the specific value combined templates whose elements are specific values, and AnyElement matching mechanisms or combined templates are allowed and the substring or subsequence to be returned shall not contain AnyElement or combined template.

In addition to the general error causes in clause 16.1.2, error causes are:

index is less than zero;

count is less than zero;

index + count is greater than lengthof(inpar);

inpar is a template of a character string type and contains a matching mechanism other than a specific value or pattern; or if the pattern contains other metacharacters than "?", "*";

inpar is a template of a binary string or sequence type or array and it contains other matching mechanism as specific value or combined template; or if the elements of combined template are any other matching mechanisms than specific values, and AnyElement or combined templates;

inpar is a template of a binary string or sequence type or array and the substring or subsequence to be returned contains the AnyElement matching mechanism or combined templates;

the template passed to the inpar parameter is not of type bitstring, hexstring, octetstring, charstring, universal charstring, record of, set of, or array.

EXAMPLE:

substr('00100110'B, 3, 4)		// returns '0011'B

substr('ABCDEF'H, 2, 3)		    // returns 'CDE'H

substr('01AB23CD'O, 1, 2)		// returns 'AB23'O

substr("My name is JJ", 11, 2)	// returns "JJ"

substr({ 4, 5, 6 }, 1, 2)		// returns {5, 6}

## C.4.3	The Replace function

replace(

in any inpar,

in integer index,

in integer len,

in any repl

) return any_string_or_sequence type

This function replaces the substring or subsequence of value inpar at index index of length len with the string or sequence value repl and returns the resulting string or sequence. inpar shall not be modified. If len is 0 the string or sequence repl is inserted. If index is 0, repl is inserted at the beginning of inpar. If index is lengthof(inpar), repl is inserted at the end of inpar. If inpar is a literal (i.e. type is not explicitly given) the corresponding type shall be retrieved from the value contents. inparand repl, and the returned string or sequence shall be of the same root type. The function replace can be applied to bitstring, hexstring, octetstring, or any character string, record of, set of, or arrays. Note that indexing in strings starts from zero.

NOTE:	Please note that the root types of arrays is record of, therefore if inpar or repl or both are an array, the returned type is record of. This, in some cases, may lead to different indexing in inpar and/or repl and in the returned value.

In addition to the general error causes in clause 16.1.2, error causes are:

inpar or repl are not of string, record of, set of, or array type;

inpar and repl are of different root type;

index is less than 0 or greater than lengthof(inpar);

len is less than 0 or greater than lengthof(inpar);

index+len is greater than lengthof(inpar).

EXAMPLE:

replace ('00000110'B, 1, 3, '111'B)	    // returns '01110110'B

replace ('ABCDEF'H, 0, 2, '123'H)		// returns '123CDEF'H

replace ('01AB23CD'O, 2, 1, 'FF96'O)	// returns '01ABFF96CD'O

replace ("My name is JJ", 11, 1, "xx")	// returns "My name is xxJ"

replace ("My name is JJ", 11, 0, "xx")	// returns "My name is xxJJ"

replace ("My name is JJ", 2, 2, "x")	// returns "Myxame is JJ",

replace ("My name is JJ", 12, 2, "xx")	// produces test case error

replace ("My name is JJ", 13, 2, "xx")	// produces test case error

replace ("My name is JJ", 13, 0, "xx")	// returns "My name is JJxx"

# C.5	Codec functions

## C.5.1	The encoding function

encvalue(in template (value) any inpar,

in universal charstring encoding_info := "",

in universal charstring dynamic_encoding := "") return bitstring

The encvalue function encodes a value or template into a bitstring. When the actual parameter that is passed to inpar is a template, it shall resolve to a specific value (the same restrictions apply as for the argument of the send statement). The returned bitstring represents the encoded value of inpar, however, the TTCN-3 test system need not make any check on its correctness. The optional encoding_info parameter is used for passing additional encoding information to the codec and, if it is omitted, no additional information is sent to the codec.

The optional dynamic_encoding parameter is used for dynamic selection of encode attribute of the inpar value for this single encvalue call. The rules for dynamic selection of the encode attribute are described in clause 27.9.

In addition to the general error causes in clause 16.1.2, error causes are:

Encoding fails due to a runtime system problem (i.e. no encoding function exists for the actual type of inpar).

## C.5.2	The decoding function

decvalue(inout bitstring encoded_value,

out any decoded_value,

in universal charstring decoding_info := "",

in universal charstring dynamic_encoding := "") return integer

The decvalue function decodes a bitstring into a value. The test system shall suppose that the bitstring encoded_value represents an encoded instance of the actual type of decoded_value. The optional decoding_info parameter is used for passing additional decoding information to the codec and, if it is omitted, no additional information is sent to the codec.

The optional dynamic_encoding parameter is used for dynamic selection of encode attribute of the decoded_value parameter for this single decvalue call. The rules for dynamic selection of the encode attribute are described in clause 27.9.

If the decoding was successful, then the used bits are removed from the parameter encoded_value, the rest is returned (in the parameter encoded_value), and the decoded value is returned in the parameter decoded_value. If the decoding was unsuccessful, the actual parameters for encoded_value and decoded_value are not changed. The function shall return an integer value to indicate success or failure of the decoding below:

The return value 0 indicates that decoding was successful.

The return value 1 indicates an unspecified cause of decoding failure. This value is also returned if the encoded_value parameter contains an unitialized value.

The return value 2 indicates that decoding could not be completed as encoded_value did not contain enough bits.

The restrictions in clause 16.1.2 apply.

## C.5.3	The encoding to universal charstring function

encvalue_unichar(in template (value) any inpar,

in charstring string_serialization := "UTF-8",

in universal charstring encoding_info := "",

in universal charstring dynamic_encoding := "")

return universal charstring

The encvalue_unichar function encodes a value or template into a universal charstring. When the actual parameter that is passed to inpar is a template, it shall resolve to a specific value (the same restrictions apply as for the argument of the send statement). The returned universal charstring  represents the encoded value of inpar, however, the TTCN-3 test system need not make any check on its correctness. If the optional string_serialization parameter is omitted, the default value "UTF-8" is used. The optional encoding_info parameter is used for passing additional encoding information to the codec and, if it is omitted, no additional information is sent to the codec.

The optional dynamic_encoding parameter is used for dynamic selection of encode attribute of the inpar value for this single encvalue_unichar call. The rules for dynamic selection of the encode attribute are described in clause 27.9.

The following values (see ISO/IEC 10646 [2]) are allowed as string_serialization actual parameters (for the description of the UCS encoding scheme see clause 27.5):

- "UTF-8"

- "UTF-16"

- "UTF-16LE"

- "UTF-16BE"

- "UTF-32"

- "UTF-32LE"

- "UTF-32BE"

The serialized bitstring shall not include the optional signature (see clause 10 of ISO/IEC 10646 [2], also known as byte order mark).

In case of "UTF-16" and "UTF-32" big-endian ordering shall be used (as described in clauses 10.4 and 10.7 of ISO/IEC 10646 [2]).

The specific semantics of this function are explained by the following TTCN-3 definition:

function encvalue_unichar(in template(value) any inpar,

in charstring enc

in universal charstring encoding_info := "",

in universal charstring dynamic_encoding := "")

return universal charstring {

return oct2unichar(bit2oct(encvalue(inpar, encoding_info, dynamic_encoding)), enc);

}

The encvalue_unichar function first invokes the encvalue function in order to encode the value passed in the inpar parameter to a bitstring. The bitstring is then converted to an octetstring by the bit2oct function and subsequently to a universal charstring using the oct2unichar function. The string_serialization parameter defines how the encoded octets (in fact the encoded bitstring received from the codec) contain the characters. The universal charstring value is then returned as the result of the encvalue_unichar function.

In addition to the general error causes in clause 16.1.2, error causes are:

Encoding fails due to a runtime system problem (i.e. no encoding function exists for the actual type of inpar).

The given string encoding is not recognized.

## C.5.4	The decoding from universal charstring function

decvalue_unichar(inout universal charstring encoded_value,

out any decoded_value,

in charstring string_serialization:= "UTF-8",

in universal charstring decoding_info := "",

in universal charstring dynamic_encoding := "")

return integer

The decvalue_unichar function decodes (part of) a universal charstring  into a value. The test system shall suppose that a prefix of the universal charstring encoded_value represents an encoded instance of the actual type of decoded_value. The optional decoding_info parameter is used for passing additional decoding information to the codec and, if it is omitted, no additional information is sent to the codec.

The optional dynamic_encoding parameter is used for dynamic selection of encode attribute of the decoded_value parameter for this single decvalue_unichar call. The rules for dynamic selection of the encode attribute are described in clause 27.9.

If the decoding was successful, then the characters used for decoding are removed from the parameter encoded_value, the rest is returned (in the parameter encoded_value), and the decoded value is returned in the parameter decoded_value. If the decoding was unsuccessful, the actual parameters for encoded_value and decoded_value are not changed. The function shall return an integer value to indicate success or failure of the decoding below:

The return value 0 indicates that decoding was successful.

The return value 1 indicates an unspecified cause of decoding failure. This value is also returned if the encoded_value parameter contains an unitialized value.

The return value 2 indicates that decoding could not be completed as encoded_value did not contain enough bits.

If the optional string_serialization parameter is omitted, the default value "UTF-8" is used.

The following values (see ISO/IEC 10646 [2]) are allowed as string_serialization actual parameters (for the description of the UCS encoding scheme see clause 27.5):

- "UTF-8"

- "UTF-16"

- "UTF-16LE"

- "UTF-16BE"

- "UTF-32"

- "UTF-32LE"

- "UTF-32BE"

The serialized bitstring shall not include the optional signature (see clause 10 of ISO/IEC 10646 [2], also known as byte order mark).

- In case of "UTF-16" and "UTF-32" big-endian ordering shall be used (as described in clauses 10.4 and 10.7 of ISO/IEC 10646 [2]).

- The semantics of the function can be explained by the following TTCN-3 function:

function decvalue_unichar (inout universal charstring encoded_value,

out any decoded_value,

in charstring string_encoding := "UTF-8"",

in universal charstring decoding_info := "",

in universal charstring dynamic_encoding := "") return integer {

var bitstring v_str = oct2bit(unichar2oct(encoded_value, string_encoding));

var integer v_result := decvalue(v_str, decoded_value, decoding_info, dynamic_encoding);

if (v_result == 0) { // success

encoded_value := oct2unichar(bit2oct(v_str), string_encoding);

}

return v_result;

}

The decvalue_unichar function first converts the universal charstring value passed in the encoded_value parameter into an octetstring using the unichar2oct function. The string_encoding parameter controls how the characters are converted into octets (in fact how the bitstring sent to the codec contains the characters). The octetstring is subsequently converted into a bitstring by the oct2bit function. This bitstring is then passed as a parameter to the standard decvalue function that performs the actual decoding. In case of successful decoding, the undecoded part of the message is automatically converted from bitstring to octetstring by the bit2oct function and then to universal charstring using the oct2unichar function. This universal charstring is then assigned to the encoded_value parameter. The result of decoding is then returned to the TE, finishing the decvalue_unichar call.

The restrictions in clause 16.1.2 apply.

## C.5.5	The encoding to octetstring function

encvalue_o(in template (value) any inpar,

in universal charstring encoding_info := "",

in universal charstring dynamic_encoding := "",

out integer bit_length) return octetstring

The encvalue_o function encodes a value or template into an octetstring. When the actual parameter that is passed to inpar is a template, it shall resolve to a specific value (the same restrictions apply as for the argument of the send statement). The returned octetstring represents the encoded value of inpar, however, the TTCN-3 test system need not make any check on its correctness. In case the encoded message is not octet-based and has a bit length not divisable by 8, the encoded message will be left-aligned in the returned octetstring and the least significant (8 - (bit length mod 8)) bits in the least significant octet will be 0. The bit length can be assigned to a variable by usage of the formal out parameter bit_length. The optional encoding_info parameter is used for passing additional encoding information to the codec and, if it is omitted, no additional information is sent to the codec.

The optional dynamic_encoding parameter is used for dynamic selection of encode attribute of the inpar value for this single encvalue_o call. The rules for dynamic selection of the encode attribute are described in clause 27.9.

In addition to the general error causes in clause 16.1.2, error causes are:

Encoding fails due to a runtime system problem (i.e. no encoding function exists for the actual type of inpar).

## C.5.6	The decoding from octetstring function

decvalue_o(inout octetstring encoded_value,

out any decoded_value,

in universal charstring decoding_info := "",

in universal charstring dynamic_encoding := "") return integer

The decvalue_o function decodes an octetstring into a value. The test system shall suppose that the octetstring encoded_value represents an encoded instance of the actual type of decoded_value. The optional decoding_info parameter is used for passing additional decoding information to the codec and, if it is omitted, no additional information is sent to the codec.

The optional dynamic_encoding parameter is used for dynamic selection of encode attribute of the decoded_value parameter for this single decvalue_o call. The rules for dynamic selection of the encode attribute are described in clause 27.9.

If the decoding was successful, then the used octets are removed from the parameter encoded_value, the rest is returned (in the parameter encoded_value), and the decoded value is returned in the parameter decoded_value. If the decoding was unsuccessful, the actual parameters for encoded_value and decoded_value are not changed. The function shall return an integer value to indicate success or failure of the decoding below:

The return value 0 indicates that decoding was successful.

The return value 1 indicates an unspecified cause of decoding failure. This value is also returned if the encoded_value parameter contains an unitialized value.

The return value 2 indicates that decoding could not be completed as encoded_value did not contain enough octets.

The restrictions in clause 16.1.2 apply.

## C.5.7	Retrieving the type of string encoding

get_stringencoding(in octetstring encoded_value) return charstring

The get_stringencoding function analyses the encoded_value and returns the UCS encoding scheme according to clause 10 of ISO/IEC 10646 [2] (see also clause 27.5 of the present document). The identified encoding scheme, or the value "<unknown>", if the type of encoding cannot be determined unanimously, shall be returned as a character string.

NOTE:	The initial octet sequence (also known as byte order mark, BOM), when present, allows identifying the encoding scheme unanimously. When it is not present, other symptoms may be used to identify the encoding scheme unanimously; for example, only UTF-8 may have odd number of octets and bit distribution according to table 2 of clause 9.1 of ISO/IEC 10646 [2].

EXAMPLE:

match ( get_stringencoding('6869C3BA7A'O),charstring:"UTF-8") // true

//(the octetstring contains the UTF-8 encoding of the character sequence "hiúz")

## C.5.8	Removing BOMs of UCS encoding schemes

remove_bom(in octetstring encoded_value) return octetstring

The remove_bom function removes the optional FEFF ZERO WIDTH NO-BREAK SPACE sequence that may be present at the beginning of a stream of serialized (encoded) universal character strings to indicate the order of the octets within the encoding form, as defined in clause 10 of ISO/IEC 10646 [2]. If no FEFF ZERO WIDTH NO-BREAK SPACE sequence present in the encoded_value parameter, the function shall return the value of the parameter without change.

Table C.2: Overview of initial octet sequences used for BOM

| Coding scheme | initial octet sequence | comments |
| --- | --- | --- |
| UTF-8 | EF BB BF | signature not required / no effect |
| UTF-16BE | FE FF | no signature meaning |
| UTF-16LE | FF FE | no signature meaning |
| UTF-16 | FE FF FF FE | signature (default FE FF) |
| UTF-32BE | 00 00 FE FF | no signature meaning |
| UTF-32LE | FF FE 00 00 | no signature meaning |
| UTF-32 | 00 00 FE FF FF FE 00 00 | signature (default 00 00 FE FF) |

EXAMPLE:

remove_bom('FEFF0068006900FA007A'O) // returns '0068006900FA007A'O

remove_bom('BC'O) ) // returns 'BC'O

// note that this octetstring doesn't contain valid UCS character

//example use: automatic decoding of encoded character strings:

oct2unichar(remove_bom(v_myEncodedCharacterSequence),

get_stringencoding(v_myEncodedCharacterSequence))

# C.6	Other functions

## C.6.1	The random number generator function

rnd([in float seed]) return float

The rnd function returns a (pseudo) random number less than 1 but greater or equal to 0. The random number generator is initialized per test component and for the control part by means of an optional seed value (a numerical float value). If no new seed is provided, the last generated number will be used as seed for the next random number. Without a previous initialization a value calculated from the system time will be used as seed value when rnd is used the first time in a test component or the control part.

Each time the rnd function is initialized with the same seed value, it shall repeat the same sequence of random numbers.

NOTE:	For the purpose of keeping parallel testing deterministic, each test component, as well as the control part has its own random seed. This allows for better reproducibility of test executions. Thus, the rnd function will always use the seed of the component or control part which calls it.

To produce a random integers in a given range, the following formula can be used:

float2int(int2float(upperbound - lowerbound +1)*rnd()) + lowerbound

// Here, upperbound and lowerbound denote highest and lowest number in range.

In addition to the general error causes in clause 16.1.2, error causes are:

seed is infinity, -infinity or not_a_number.

## C.6.2	The testcasename function

testcasename() return charstring

The testcasename function shall return the unqualified name of the actually executing test case.

EXAMPLE 1:

module MyTCModule {

:

testcase TC_MyTestCase1 () runs on MTC system TSI

{

var charstring v_tcName := testcasename ();

// will return the charstring "TC_MyTestCase1"

:

}

:

testcase TC_MyTestCase2 () runs on MTC system TSI

{

:

}

:

}

module MyTSModule {

:

function f_myStartAPTC() runs on PTC {

var charstring v_tcName := testcasename ();

// will return charstring "TC_MyTestCase1", if the function is

// called by a test component during the execution of TC_MyTestCase1

// will return charstring "TC_MyTestCase2", if the function is

// called by a test component when TC_MyTestCase2 is being executed

}

:

}

When the function testcasename is called if the control part is being executed but no testcase, it shall return the empty string.

EXAMPLE 2:

module MyModule {

:

control

{

var charstring v_tcName := testcasename () // will return charstring ""

:

}

:

}

The general error causes in clause 16.1.2 apply.

## C.6.3	The hostId function

hostid(in charstring idkind := "Ipv4orIPv6") return charstring

The hostid function shall return the host id of the test component or module control executing the hostid function in form of a character string. The in parameter idkind allows to specify the expected id format to be returned.

Predefined idkind values are:

"Ipv4orIPv6": The contents of the returned character string is an Ipv4 address. If no Ipv4 address, but an Ipv6 address is available, a character string representation of the Ipv6 address is returned.

"Ipv4": The contents of the returned character string shall be an Ipv4 address.

"Ipv6": The contents of the returned characterstring shall be an Ipv6 address.

The hostid function shall return the empty string, if it cannot retrieve any host id or a host id of a kind different from the kind defined by the actual idkind parameter.

The general error causes in clause 16.1.2 apply.

EXAMPLE:

// assume

testcase TC_MyTestCase () runs on MTC system TSI

{

:

var charstring v_myHostId := hostid ("Ipv4");

:

}

// assume further the following statement in module control

execute(TC_MyTestCase(), -, "127.0.0.1");

// In this setting, v_myHostId will have the value "127.0.0.1" after the execution of hostid

Annex D (normative):
Preprocessing macros

# D.0	General

This annex defines a set of preprocessing macros. A preprocessing macro is a macro that is replaced by a preprocessor or a compiler with a charstring or integer value respectively before compilation. Preprocessing macros shall not be replaced inside literal charstring values and templates and not in TTCN-3 comments. In the TTCN-3 code, it can be used like a charstring or an integer value respectively.

# D.1	Preprocessing macro __MODULE__

The __MODULE__ preprocessing macro denotes the module name in which the macro is used. A preprocessor or compiler shall replace all occurrences of __MODULE__ with the actual module name in form of a charstring value.

# D.2	Preprocessing macro __FILE__

The __FILE__ preprocessing macro denotes the canonical (absolute) file name, i.e. the full path and the basic file name, in which the macro is used. A preprocessor or compiler shall replace all occurrences of __FILE__ with the actual canonical (absolute) file name in form of a charstring value.

NOTE:	The format of the canonical file name depends on the operating system and is not specified by the present document.

EXAMPLE:

const charstring c_myConst:= __FILE__;

//c_myConst is for example "/home/myhome/MyTest.ttcn"

# D.3	Preprocessing macro __BFILE__

The __BFILE__ preprocessing macro denotes the basic (relative) file name, i.e. without path, in which the macro is used. A preprocessor or compiler shall replace all occurrences of __BFILE__ with the actual basic (relative) file name in form of a charstring value.

NOTE:	The format of the basic file name depends on the operating system and is not specified by the present document.

EXAMPLE:

const charstring c_myConst:= __BFILE__;

// c_myConst is for example "MyTest.ttcn"

# D.4	Preprocessing macro __LINE__

The __LINE__ preprocessing macro denotes the line number of the file in which the macro is used. A preprocessor or compiler shall replace each occurrence of __LINE__ with the actual line number in form of an integer value.

A file starts with line number 1. Each newline shall increase the line number by 1 (see clause A.1.5.1). Also newlines of commented lines shall increase the line number by 1.

# D.5	Preprocessing macro __SCOPE__

The __SCOPE__ preprocessing macro denotes the unqualified name of the lowest named basic scope unit in which the macro is used. According to clause 5.2, basic scope units of TTCN-3 are module definitions part, module control function, component types, functions, altsteps, test cases, statement blocks, templates and user defined named types. Statement blocks have no name and therefore, a __SCOPE__ preprocessing macro used in a statement block refers to the next higher named basic scope unit.

A preprocessor or compiler shall replace all occurrences of __SCOPE__ with a charstring value which includes:

- the module name, if the lowest named scope unit is the module definitions part;

- "control", if the lowest named scope unit is the module control function;

- a component type name, if the lowest named scope unit is a component type definition;

- a test case name, if the lowest named scope unit is a test case definition;

- an altstep name, if the lowest named scope is an altstep definition;

- a function name, if the lowest named scope is a function definition;

- a template name, if the lowest named scope is a template definition (local or global); or

- the type name, if the lowest named scope is a user defined named type definition.

NOTE:	The __SCOPE__ preprocessing macro cannot be used to retrieve the names of other kinds of definitions, like for example names of groups of definitions or names of global constants.

EXAMPLE 1:	Using __SCOPE__ in constant and template definitions:

module MyModule

{

const charstring c_myConst := __SCOPE__;  		// c_myConst contains "MyModule"

template charstring m_myTemplate := __SCOPE__;	// m_myTemplate contains "m_myTemplate"

type record MyRecord1

{

charstring field11,

charstring field12

}

template MyRecord1 m_myTemplate1 (charstring p_p := __SCOPE__) :=

{

field11 := p_p,

field12 := __SCOPE__							// field12 contains "m_myTemplate1"

}

function f_myFunction() {

var template MyRecord1 v_myvar1 := m_myTemplate1;

// field11 of m_myTemplate1 will contain the default value of parameter p_p,

// i.e. "m_myTemplate1"

};

control {

const charstring c_myLocalConst := __SCOPE__;  		// c_myLocalConst contains "control"

}

}

EXAMPLE 2:	Using __SCOPE__ in a structured type scope:

type record MyRecord2 {

charstring field21,

charstring field22 ("a", "b", __SCOPE__)

// list constrained field: a legal values are "a", "b" or "MyRecord2"

}

template MyRecord2 m_myTemplate2 := {

field21 := "a",

field22 := "MyRecord2" 							// a valid specific value matching

}

template MyRecord2 m_myTemplate3 := {

field21 := "a",

field22 := __SCOPE__

// Causes an error as __SCOPE__ is replaced with "m_myTemplate3",

// which is violating the list constraint of field22

}

EXAMPLE 3:	Using __SCOPE__ in an embedded structured type scope:

type record MyRecord3 {

charstring field31,

record {

charstring field321 ("a", "b", __SCOPE__)

// list constrained field: a legal value shall be "a", "b" or "MyRecord3"

} field32

}

template MyRecord3 m_myTemplate4 :=

{

field31 := "a",

field32 :=

{

field321 := "MyRecord3"						// a valid specific value matching

}

}

template MyRecord3 m_myTemplate5 :=

{

field31 := "a",

field32 :=

{

field321 := __SCOPE__

// Causes and error as __SCOPE__ is replaced with "m_myTemplate5",

// which is violating the list constraint of field321

}

}

Annex E (informative):
Library of Useful Types

# E.1	Limitations

Names of types added to this library are to be unique within the whole language and within the library (i.e. are not to be one of the names defined in annex C). Names defined in this library are not to be used by TTCN-3 users as identifiers of other definitions than given in this annex.

NOTE:	Therefore type definitions given in this annex may be repeated in TTCN-3 modules but no type distinct from the one specified in this annex can be defined with one of the identifiers used in this annex.

# E.2	Useful TTCN-3 types

## E.2.1	Useful simple basic types

### E.2.1.0	Signed and unsigned single byte integers

These types support integer values of the range from -128 to 127 for the signed and from 0 to 255 for the unsigned type. The value notation for these types is the same as the value notation for the integer type. Values of these types are to be encoded and decoded as they were represented on a single byte within the system independently from the actual representation form used.

NOTE:	Encoding of values of these types may be the same or may differ from each other and from the encoding of the integer type (the root type of these useful types) depending on the actual encoding rules used. Details of encoding rules are out of the scope of the present document.

Type definitions for these types are:

type integer	byte			(-128 .. 127)	with { variant "8 bit" };

type integer	unsignedbyte	(0 .. 255)		with { variant "unsigned 8 bit" };

### E.2.1.1	Signed and unsigned short integers

These types support integer values of the range from -32 768 to 32 767 for the signed and from 0 to 65 535 for the unsigned type. The value notation for these types is the same as the value notation for the integer type. Values of these types are to be encoded and decoded as they were represented on two bytes within the system independently from the actual representation form used.

NOTE:	Encoding of values of these types may be the same or may differ from each other and from the encoding of the integer type (the root type of these useful types) depending on the actual encoding rules used. Details of encoding rules are out of the scope of the present document.

Type definitions for these types are:

type integer	short			(-32768 .. 32767)	with { variant "16 bit" };

type integer	unsignedshort	(0 .. 65535)		with { variant "unsigned 16 bit" };

### E.2.1.2	Signed and unsigned long integers

These types support integer values of the range from -2 147 483 648 to 2 147 483 647 for the signed and from 0 to 4 294 967 295 for the unsigned type. The value notation for these types is the same as the value notation for the integer type. Values of these types are to be encoded and decoded as they were represented on four bytes within the system independently from the actual representation form used.

NOTE:	Encoding of values of these types may be the same or may differ from each other and from the encoding of the integer type (the root type of these useful types) depending on the actual encoding rules used. Details of encoding rules are out of the scope of the present document.

Type definitions for these types are:

type integer	long	(-2147483648 .. 2147483647)

with { variant "32 bit" };

type integer	unsignedlong	(0 .. 4294967295)

with { variant "unsigned 32 bit" };

### E.2.1.3	Signed and unsigned longlong integers

These types support integer values of the range from -9 223 372 036 854 775 808 to 9 223 372 036 854 775 807 for the signed and from 0 to 18 446 744 073 709 551 615 for the unsigned type. The value notation for these types is the same as the value notation for the integer type. Values of these types are to be encoded and decoded as they were represented on eight bytes within the system independently from the actual representation form used.

NOTE:	Encoding of values of these types may be the same or may differ from each other and from the encoding of the integer type (the root type of these useful types) depending on the actual encoding rules used. Details of encoding rules are out of the scope of the present document.

Type definitions for these types are:

type integer	longlong	(-9223372036854775808 .. 9223372036854775807)

with { variant "64 bit" };

type integer	unsignedlonglong	(0 .. 18446744073709551615)

with { variant "unsigned 64 bit" };

### E.2.1.4	IEEE 754 floats

These types support the ANSI/IEEE 754 [6] for binary floating-point arithmetic. The type IEEE 754 [6] float supports floating-point numbers with base 10, exponent of size 8, mantissa of size 23 and a sign bit. The type IEEE 754 [6] double supports floating-point numbers with base 10, exponent of size 11, mantissa of size 52 and a sign bit. The type IEEE 754 [6] extfloat supports floating-point numbers with base 10, minimal exponent of size 11, minimal mantissa of size 32 and a sign bit. The type IEEE 754 [6] extdouble supports floating-point numbers with base 10, minimal exponent of size 15, minimal mantissa of size 64 and a sign bit.

Values of these types are to be encoded and decoded according to the IEEE 754 [6] definitions. The value notation for these types is the same as the value notation for the float type (base 10).

NOTE:	Precise encoding of values of this type depends on the actual encoding rules used. Details of encoding rules are out of the scope of the present document.

Type definitions for these types are:

type	float	IEEE754float		with { variant "IEEE754 float" };

type	float	IEEE754double		with { variant "IEEE754 double" };

type	float	IEEE754extfloat		with { variant "IEEE754 extended float" };

type	float	IEEE754extdouble	with { variant "IEEE754 extended double" };

## E.2.2	Useful character string types

### E.2.2.0	UTF-8 character string "utf8string"

This type supports the whole character set of the TTCN-3 type universal charstring (see paragraph d) of clause 6.1.1). Its distinguished values are zero, one, or more characters from this set. Values of this type are entirely (e.g. each character of the value individually) to be encoded and decoded according to the UCS Transformation Format 8 (UTF-8) as defined in annex R of ISO/IEC 10646 [2]. The value notation for this type is the same as the value notation for the universal charstring type.

The type definition for this type is:

type universal charstring utf8string with { variant "UTF-8" };

### E.2.2.1	BMP character string "bmpstring"

This type supports the Basic Multilingual Plane (BMP) character set of ISO/IEC 10646 [2]. The BMP represents all characters of plane 00 of group 00 of the Universal Multiple-octet coded Character Set. Its distinguished values are zero, one, or more characters from the BMP. Values of this type are entirely (e.g. each character of the value individually) to be encoded and decoded according to the UTF-16 coded representation form (see clause 9.2 of ISO/IEC 10646 [2]). The value notation for this type is the same as the value notation for the universal charstring type.

NOTE:	The type "bmpstring" supports a subset of the TTCN-3 type universal charstring.

The type definition for this type is:

type universal charstring bmpstring	( char ( 0,0,0,0 ) .. char ( 0,0,255,255) )

with { variant "UTF-16" };

### E.2.2.2	UTF-16 character string "utf16string"

This type supports all characters of planes 00 to 16 of group 00 of the Universal Multiple-octet coded Character Set (see ISO/IEC 10646 [2]). Its distinguished values are zero, one, or more characters from this set. Values of this type are entirely (e.g. each character of the value individually) to be encoded and decoded according to the UCS Transformation Format 16 (UTF-16) as defined in annex Q of ISO/IEC 10646 [2]. The value notation for this type is the same as the value notation for the universal charstring type.

NOTE:	The type "utf16string" supports a subset of the TTCN-3 type universal charstring.

The type definition for this type is:

type universal charstring utf16string	( char ( 0,0,0,0 ) .. char ( 0,16,255,255) )

with { variant "UTF-16" };

### E.2.2.3	ISO/IEC 10646 character string "iso8859string"

This type supports all characters in all alphabets defined in the multiparty standard ISO/IEC 10646 [2]. Its distinguished values are zero, one, or more characters from the ISO/IEC 10646 [2] character set. Values of this type are entirely (e.g. each character of the value individually) to be encoded and decoded according to the coded representation as specified in ISO/IEC 10646 [2] (an 8-bit coding). The value notation for this type is the same as the value notation for the universal charstring type.

NOTE 1:	The type "iso8859string" supports a subset of the TTCN-3 type universal charstring.

NOTE 2:	In each ISO/IEC 10646 [2] alphabet the lower part of the character set table (positions 02/00 to 07/14) is compatible with the Recommendation ITU-T T.50 [4] character set. Hence all extra language specific characters are defined for the upper part of the character table only (positions 10/00 to 15/15).

The type definition for this type is:

type universal charstring iso8859string	( char (0,0,0,0) .. char (0,0,0,255) )

with { variant "8 bit" };

### E.2.2.4	Status values for TTCN-3 objects

Type and constants defined in this clause support the secure usage of the checkstate port operation defined in clause 22.5.5.

The type definition for this type is:

type charstring objState	("Started", "Halted", "Stopped", "Connected", "Mapped", "Linked");

Useful constant definitions for working with object states are:

const objState STARTED := "Started";

const objState HALTED := "Halted";

const objState STOPPED := "Stopped";

const objState CONNECTED := "Connected";

const objState MAPPED := "Mapped";

const objState LINKED := "Linked";

### E.2.2.5	Template kinds of TTCN-3 objects

Type and constants defined in this clause support the secure usage of the predefined istemplatekind function, described in clause C.3.5.

The type definition for this type is:

type charstring TemplateKind ("value", "list", "complement", "AnyValue", "?", "AnyValueOrNone", 		"*", "range", "subset", "superset", "omit", "decmatch", "AnyElement", "AnyElementsOrNone", 		"permutation", "length", "ifpresent", "pattern");

Useful constant definitions for working with template kinds are:

const TemplateKind VALUE := "value";

const TemplateKind LIST := "list";

const TemplateKind COMPLEMENT := "complement";

const TemplateKind ANY_VALUE := "AnyValue";

const TemplateKind ANY_VALUE_OR_NONE := "AnyValueOrNone";

const TemplateKind RANGE := "range";

const TemplateKind SUBSET := "subset";

const TemplateKind SUPERSET := "superset";

const TemplateKind OMIT := "omit";

const TemplateKind DECMATCH := "decmatch";

const TemplateKind ANY_ELEMENT := "AnyElement";

const TemplateKind ANY_ELEMENTS_OR_NONE := "AnyElementsOrNone";

const TemplateKind PERMUTATION := "permutation";

const TemplateKind LENGTH := "length";

const TemplateKind IFPRESENT := "ifpresent";

const TemplateKind PATTERN := "pattern";

## E.2.3	Useful structured types

### E.2.3.0	Fixed-point decimal literal

This type supports the use of fixed-point decimal literal as defined in the IDL Syntax and Semantics version 2.6 [i.10]. It is specified by an integer part, a decimal point and a fraction part. The integer and fraction parts both consist of a sequence of decimal (base 10) digits. The number of digits is stored in "digits" and the size of the fraction part is given in "scale". The digits itself are stored in "value_". Value notation for this type is the same as the value notation for the record type. Values of this type are to be encoded and decoded as IDL fixed point decimal values.

NOTE:	Precise encoding of values of this type depends on the actual encoding rules used. Details of encoding rules are out of the scope of the present document.

The type definition for this type is:

type record	IDLfixed {

unsignedshort	digits,

short	scale,

charstring	value_

}

with { variant "IDL:fixed FORMAL/01-12-01 v.2.6" };

## E.2.4	Useful atomic string types

### E.2.4.1	Single Recommendation ITU-T T.50 character type

A type whose distinguished values are single characters of the version of Recommendation ITU-T T.50 [4] complying to the International Reference Version (IRV) as specified in clause 8.2 of Recommendation ITU-T T.50 [4] (see also note 1 to clause 6.1.1).

The type definition for this type is:

type charstring char646 length (1);

NOTE:	The special string "8 bit" defined in clause 27.5 may be used with this type to specify a given encoding for its values. Also, other properties of the base type can be changed by using attribute mechanisms.

### E.2.4.2	Single universal character type

A type whose distinguished values are single characters from ISO/IEC 10646 [2].

The type definition for this type is:

type universal charstring uchar length (1);

NOTE:	Special strings defined in clause 27.5 except "8 bit" may be used with this type to specify a given encoding for its values. Also, other properties of the base type can be changed by using attribute mechanisms.

### E.2.4.3	Single bit type

A type whose distinguished values are single binary digits.

The type definition for this type is:

type bitstring bit length (1);

### E.2.4.4	Single hex type

A type whose distinguished values are single hexadecimal digits.

The type definition for this type is:

type hexstring hex length (1);

### E.2.4.5	Single octet type

A type whose distinguished values are pairs of hexadecimal digits.

The type definition for this type is:

type octetstring octet length (1);

Annex F (informative):
Operations on TTCN-3 active objects

# F.0	General

This annex describes in a short form the semantics of operations on active objects in TTCN-3 being test components, timers and ports. This dynamic behaviour is written in the form of state machines with:

the states being named and identified as nodes;

the initial state being identified by an incoming arrow;

transitions between states connecting two states (not necessarily different states) and identified as arrows;

transitions being marked with the enabling condition for that transition (i.e. operation or statement calls) and the resulting condition (for example a test case error), both are separated by '/':

operation and statement calls are the TTCN-3 operations and statements applicable to the object (written in bold);

error as a resulting condition means testcase error (written in bold);

null as a resulting condition means that except of a possible state change no other results apply (written in bold);

match/no match refers to the matching result of a transition (written in bold);

concrete values are boolean or float results (written in bold italics);

all other resulting conditions are textually described (written in standard font);

notes are used to explain further details of the state machine.

For further details, please refer to the operational semantics of TTCN-3 [1]. In case of any contradiction between this annex and the operational semantics of TTCN-3 [1] the latter takes precedence.

# F.1	Test components

## F.1.1	Test component references

Variables of test component types, the self and mtc operations are used to reference test components. The start, stop, done and running operations are not directly applied on test components but on component references. The test system has to decide if the operation requested should affect the component object itself or other action is appropriate (e.g. an error occurs when the reference of a stopped PTC is used in a component start operation). The create operation used to create PTCs returns a unique reference to the created PTC, which is typically bound to a variable of component type. The behaviour related to variables of component type themselves is shown in figure F.1.

NOTE:	Whenever a test component enters its error state, the error verdict is assigned to its local verdict, the test case terminates and the overall test case result will be error.

Figure F.1: Handling of test component references

## F.1.2	Dynamic behaviour of PTCs

PTCs can be of non-alive type or alive-type. Non-alive type PTCs can be in Inactive, Running and Killed states. Their dynamic behaviour is shown in figure F.2.

NOTE 1:	(a)	Stop can be either a stop, self.stop or a stop from another test component.

(b)	Kill can be either a kill, self.kill, a kill from another test component or a kill from the test system 	(in error cases).

NOTE 2:	(a)	Stop can be from another test component only.

(b)	Kill can be from another test component or from the test system (in error cases) only.

NOTE 3:	Whenever a test component enters its error state, the error verdict is assigned to its local verdict, the test case terminates and the overall test case result will be error.

Figure F.2: Dynamic behaviour of non-alive type PTCs

Alive-type PTCs can be in Inactive, Running, Stopped and Killed states. Their dynamic behaviour is shown in figure F.3.

NOTE 1:	(a)	Stop can be either a stop, self.stop or a stop from another test component.

(b)	Kill can be either a kill, self.kill, a kill from another test component or a kill from the test system 	(in error cases).

NOTE 2:	(a)	Stop can be from another test component only.

(b)	Kill can be from another test component or from the test system (in error cases) only.

NOTE 3:	Whenever a test component enters its error state, the error verdict is assigned to its local verdict, the test case terminates and the overall test case result will be error.

Figure F.3: Dynamic behaviour of alive-type PTCs

## F.1.3	Dynamic behaviour of the MTC

The MTC can be in Running or Killed state. The dynamic behaviour of the MTC is shown in figure F.4.

NOTE 1:	(a)	Stop can be either a stop, self.stop, a stop from another test component.

(b)	Kill can be either a kill, self.kill, a kill from another test component or a kill from the test system

(in error cases).

NOTE 2:	All remaining PTCs are to be killed as well and the testcase terminates.

NOTE 3:	Whenever the MTC enters its error state, the error verdict is assigned to its local verdict,the test case terminates and the overall test case result will be error.

Figure F.4: Dynamic behaviour of the MTC

# F.2	Timers

Timers can be in Inactive, Running or Expired state. The dynamic behaviour of a timer is shown in figure F.5.

NOTE 1:	For any scope unit, all timers in that scope being in Running state constitute the running-timer list.

NOTE 2:	For any scope unit, all timers in that scope being in Expired state constitute the timeout-list.

NOTE 3:	Whenever a timer enters its error state, the test component it belongs to enters also its error state,assigns a local error verdict, the test case terminates and the overall test case result will be error.

Figure F.5: Dynamic behaviour of timers

# F.3	Ports

## F.3.0	General

Ports can be in Started or Stopped state. As their behaviour is rather complex, the state machine has been split into a state machine giving the dynamic behaviour of configuration operations (i.e. connect, disconnect, map and unmap), of port controlling operations (i.e. start, stop and clear) and of communication operations (i.e. send, receive, call, getcall, raise, catch, reply, getreply and check). As trigger is a shorthand for an alt together with receive it is not considered here.

## F.3.1	Configuration Operations

The port configuration operations (i.e. connect, disconnect, map and unmap) are indifferent to the state of the port. They show the behaviour shown in figure F.6.

NOTE 1:	When creating a PTC the ports of that PTC are created and started; when creating the MTC the ports of the MTC and the ports of the TSI are created and started.

NOTE 2:	Whenever a port enters its error state, the test component it belongs to enters also its error state, assigns a local error verdict, the test case terminates and the overall test case result will be error.

Figure F.6: Dynamic behaviour of ports: port configuration operations

The transitions do not change the main state of the port, i.e. the port remains in the Started or Stopped state.

## F.3.2	Port Controlling Operations

The results of port controlling operations are shown in figure F.7.

NOTE:	When creating a PTC the ports of that PTC are created and started; when creating the MTC the ports of the MTC and the ports of the TSI are created and started.

Figure F.7: Dynamic behaviour of ports: port controlling operations

## F.3.3	Communication Operations

The results of the communication operations send, receive, call, getcall, raise, catch, reply, getreply, check are shown in figure F.8.

NOTE 1:	When creating a PTC the ports of that PTC are created and started; when creating a MTC the ports of the MTC and the ports of the TSI are created and started.

NOTE 2:	A unique receiver exists if there is only one link for this port or if the to address expression references a test component whose port is linked to this port (a terminated test component is not a legal receiver).

NOTE 3:	Whenever a port enters its error state, the test component it belongs to enters also its error state, assigns a local error verdict, the test case terminates and the overall test case result will be error.

NOTE 4:	As trigger is a shorthand for an alt together with receive it is not considered here.

Figure F.8: Dynamic behaviour of ports: communication operations

Annex G (informative):
Deprecated language features

# G.1	Group style definition of module parameters

Previous versions of the present document (up to and including V2.2.1) required to use a group-like syntax shown in the example below to declare module parameters. The module parameter syntax has been unified with constant and variable declaration syntax in this version but group-like syntax is not fully removed to leave a time period for tool providers and users to change from the old syntax to the new one. The group-like syntax of module parameter declarations may be fully removed in a future edition of the present document.

EXAMPLE (superfluous syntax):

module MyModuleWithParameters

{

modulepar { integer PX_Par0, PX_Par1 := 0;

boolean PX_Par2 := true

};

modulepar { hexstring PX_Par3 };

}

# G.2	Void

# G.3	Using all in port type definitions

Previous versions of the present document (up to and including V2.2.1) allowed to use the all keyword in port type definitions instead of an explicit list of types and signatures allowed via the given port. This feature is deprecated and may be fully removed in a future edition of  the present document.

# G.4	sizeof for length of lists

Previous versions of the present document (up to and including V3.2.1) allowed to use the built-in function sizeof to compute the length of record of, set of, and array. This has been replaced by lengthof. The use of sizeof for list like types is deprecated and is planned to be fully removed in the next published version.

# G.5	Void

# G.6	Mixed ports

Previous versions of the present document (up to and including V3.2.1) allowed to use mixed ports. This feature is deprecated and may be fully removed in a future edition of the present document.

# G.7	Void

# G.8	Void

# G.9	Void

# G.10	Void

# G.11	Void

# G.12	Void

# G.13	Assignment of less restrictive templates to more restrictive templates

Previous versions of the present document (up to and including V4.9.1) allowed assignment of less restrictive templates to more restrictive templates and the use of less restrictive templates as actual parameters of formal parameters with stronger template restriction if the actual content of the less restrictive template fulfilled requirements of the stronger template restriction. This feature is deprecated and may be fully removed in the future versions of the document.

# G.14	Mixing case and case else branches in select statements

Previous versions of the present document (up to and including V4.9.1) allowed to use several case else branches in the same select statement. It was also allowed to have case else branches in between other case statements. 
Using several case else branches in the same select statement, and using a case else branch not as the textually last branch is deprecated and may be fully removed in a future edition of the present document.

EXAMPLE (superfluous syntax):

select (PX_MyModulePar)  // where PX_MyModulePar is of charstring type

{
		case (charstring:"firstValue")
			{
			 log ("The first branch is selected");
			}

case else
			{
			 log ("This else branch would make all later branches unreachable");
			}
		case (v_myCharVar, c_myCharConst)
			{
			 log ("The second branch is selected");
			}
		case else
			{
			 log ("The value of the module parameter PX_MyModulePar is selected");
			}

}

# G.15	Partially initialized global and local templates

Previous versions of the present document (up to and including V4.11.1) allowed global and local templates without the @abstract modifier to be partially initialized after their initialization has been completed. This feature is deprecated and may be fully removed in a future edition of the present document.

Annex H (informative):
Bibliography

ISO/IEC 10646 (2003): "Information technology -- Universal Coded Character Set (UCS)".

ETSI ES 201 873-2: "Methods for Testing and Specification (MTS); The Testing and Test Control Notation version 3; Part 2: TTCN-3 Tabular presentation Format (TFT)".

# History

| Document history |  |  |
| --- | --- | --- |
| V1.1.1 | March 2001 | Publication |
| V1.1.2 | June 2001 | Publication |
| V2.2.1 | February 2003 | Publication |
| V3.1.1 | June 2005 | Publication |
| V3.2.1 | February 2007 | Publication |
| V3.3.2 | April 2008 | Publication |
| V3.4.1 | September 2008 | Publication |
| V4.1.1 | June 2009 | Publication |
| V4.2.1 | July 2010 | Publication |
| V4.3.1 | June 2011 | Publication |
| V4.4.1 | April 2012 | Publication |
| V4.5.1 | April 2013 | Publication |
| V4.6.1 | June 2014 | Publication |
| V4.7.1 | June 2015 | Publication |
| V4.8.1 | July 2016 | Publication |
| V4.9.1 | May 2017 | Publication |
| V4.10.1 | May 2018 | Publication |
| V4.11.1 | April 2019 | Publication |
| V4.12.1 | February 2020 | Membership Approval Procedure MV 20200428: 2020-02-28 to 2020-04-28 |
| V4.12.1 | May 2020 | Publication |