TD <>

Draft ETSI TS 102 868-1V<0.0.5> (2010-10)

Technical Specification

Intelligent Transport System (ITS);

Testing;

Conformance test specification for Co-operative Awareness Messages (CAM);

Part 1: Test requirements and Protocol Implementation Conformance Statement (PICS) proforma;

<

Reference

DTS/ITS-0010007-1

Keywords

ITS, PICS, TESTING

ETSI

650 Route des Lucioles

F-06921 Sophia Antipolis Cedex - FRANCE

Tel.: +33 4 92 94 42 00   Fax: +33 4 93 65 47 16

Siret N° 348 623 562 00017 - NAF 742 C

Association à but non lucratif enregistrée à la

Sous-Préfecture de Grasse (06) N° 7803/88

Important notice

Individual copies of the present document can be downloaded from:
http://www.etsi.org

The present document may be made available in more than one electronic version or in print. In any case of existing or perceived difference in contents between such versions, the reference version is the Portable Document Format (PDF). In case of dispute, the reference shall be the printing on ETSI printers of the PDF version kept on a specific network drive within ETSI Secretariat.

Users of the present document should be aware that the document may be subject to revision or change of status. Information on the current status of this and other ETSI documents is available at http://portal.etsi.org/tb/status/status.asp

If you find errors in the present document, please send your comment to one of the following services:
http://portal.etsi.org/chaircor/ETSI_support.asp

Copyright Notification

Reproduction is only permitted for the purpose of standardization work undertaken within ETSI.
The copyright and the foregoing restriction extend to reproduction in all media.

© European Telecommunications Standards Institute yyyy.

All rights reserved.

DECTTM, PLUGTESTSTM, UMTSTM, TIPHONTM, the TIPHON logo and the ETSI logo are Trade Marks of ETSI registered for the benefit of its Members.
3GPPTM is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners.
LTE™ is a Trade Mark of ETSI currently being registered
for the benefit of its Members and of the 3GPP Organizational Partners.
GSM® and the GSM logo are Trade Marks registered and owned by the GSM Association.

# Contents

Contents	3

Intellectual Property Rights	4

Foreword	4

1	Scope	5

2	References	5

2.1	Normative references	5

2.2	Informative references	5

3	Definitions and abbreviations	6

3.1	Definitions	6

3.2	Abbreviations	6

4	Conformance requirement concerning PICS	6

Annex A (normative): CAM PICS Proforma	7

A.1	Guidance for completing the ICS proforma	7

A.1.1	Purposes and structure	7

A.1.2	Abbreviations and conventions	7

A.1.3	Instructions for completing the ICS proforma	9

A.2	Identification of the implementation	9

A.2.1	Date of the statement	9

A.2.2	Implementation Under Test (IUT) identification	9

A.2.3	System Under Test (SUT) identification	10

A.2.4	Product supplier	10

A.2.5	Client (if different from product supplier)	10

A.2.6	ICS contact person	11

A.3	Identification of the protocol	12

A.4	Global statement of conformance	12

A.5	Tables	12

A.5.1	ITS Station type	12

A.5.2	CAM vehicle profiles	12

A.5.3	CAM road side station profile	13

A.5.4	CAM Basic Functionality	13

A.5.5	CAM Interfaces to data provisioning services	13

A.5.6	CAM Message	13

A.5.6.1	stationCharacteristics element	14

A.5.6.2	referencePosition element	14

A.5.6.2.1	referencePosition sub-elements	14

A.5.6.3	taggedList element	16

A.5.6.3.1	taggedList  sub-elements	17

A.5.6.3.2	PTLineDescription sub-elements	21

A.5.6.4	common sub-elements	22

A.5.7	Protocol parameters	23

A.5.7.1	Timing requirements	23

History	24

# Intellectual Property Rights

IPRs essential or potentially essential to the present document may have been declared to ETSI. The information pertaining to these essential IPRs, if any, is publicly available for ETSI members and non-members, and can be found in ETSI SR 000 314: "Intellectual Property Rights (IPRs); Essential, or potentially Essential, IPRs notified to ETSI in respect of ETSI standards", which is available from the ETSI Secretariat. Latest updates are available on the ETSI Web server (http://webapp.etsi.org/IPR/home.asp).

Pursuant to the ETSI IPR Policy, no investigation, including IPR searches, has been carried out by ETSI. No guarantee can be given as to the existence of other IPRs not referenced in ETSI SR 000 314 (or the updates on the ETSI Web server) which are, or may be, or may become, essential to the present document.

# Foreword

This Technical Specification (TS) has been produced by ETSI Technical Committee Intelligent Transport System (ITS).

The present document is part 1 of a multi-part deliverable covering Conformance test specification for Co-operative Awareness Messages (CAM) as identified below:

Part 1:	"Test requirements and Protocol Implementation Conformance Statement (PICS) proforma";

Part 2:	" Test Suite Structure and Test Purposes (TSS&TP)";

Part 3:	"Abstract Test Suite (ATS) and Protocol Implementation eXtra Information for Testing (PIXIT)".

# 1	Scope

The present document provides the Protocol Implementation Conformance Statement (PICS) proforma for Conformance test specification for Co-operative Awareness Messages (CAM) as defined in TS 102 637-2 [2] in compliance with the relevant requirements and in accordance with the relevant guidance given in ISO/IEC 9646-7 [6].

The supplier of an implementation which is claimed to conform to TS 102 637-2 [2] is required to complete a copy of the PICS proforma provided in the annex A of the present document.

# 2	References

References are either specific (identified by date of publication and/or edition number or version number) or non-specific. For specific references,only the cited version applies. For non-specific references, the latest version of the referenced document (including any amendments) applies.

Referenced documents which are not found to be publicly available in the expected location might be found at http://docbox.etsi.org/Reference.

## 2.1	Normative references

The following referenced documents are necessary for the application of the present document.

[1]	ETSI TS 102 637-1: "Intelligent Transport Systems (ITS); Vehicular Communications; Basic Set of Applications; Part 1: Functional Requirements".

[2]	ETSI TS 102 637-2: "Intelligent Transport Systems (ITS); Vehicular Communications; Basic Set of Applications; Part 2: Specification of Cooperative Awareness Basic Service".

[3]	ETSI TS 102 637-3: "Intelligent Transport Systems (ITS); Vehicular Communications; Basic Set of Applications; Part 3: Specifications of Decentralized Environmental Notification Basic Service".

[4]	ETSI TS 102 637-4: "Intelligent Transport Systems (ITS); Vehicular Communications; Basic Set of Applications; Part 4: Operational Requirements".

[5]	ISO/IEC 9646-1 (1995): "Information technology - Open Systems Interconnection - Conformance testing methodology and framework - Part 1: General concepts".

[6]	ISO/IEC 9646-7 (1995): "Information technology - Open Systems Interconnection - Conformance testing methodology and framework - Part 7: Implementation Conformance Statements".

[7]	ETS 300 406 (1995): "Methods for testing and Specification (MTS); Protocol and profile conformance testing specifications; Standardization methodology".

## 2.2	Informative references

The following referenced documents are not necessary for the application of the present document but they assist the user with regard to a particular subject area.

# 3	Definitions and abbreviations

## 3.1	Definitions

For the purposes of the present document, the following terms and definitions apply:

-	terms given in TS 102 637-2 [2].

-	terms given in ISO/IEC 9646-1 [5] and in ISO/IEC 9646-7 [6].

In particular, the following terms given in ISO/IEC 9646-1 [5] apply:

Implementation Conformance Statement (ICS): statement made by the supplier of an implementation or system claimed to conform to a given specification, stating which capabilities have been implemented. The ICS can take several forms: protocol ICS, profile ICS, profile specific ICS, information object ICS, etc.

ICS proforma: document, in the form of a questionnaire, which when completed for an implementation or system becomes an ICS

Protocol ICS (PICS): PICS for an implementation or system claimed to conform to a given protocol specification

## 3.2	Abbreviations

For the purposes of the present document, the following abbreviations apply:

BSA		Basic Set of Applications

C2CCC		Car to Car Communication Consortium

CAM	Cooperative Awareness Message

CAN	Controller Area Network

DENM	Decentralized Environmental Notification Message

I2V	Infrastructure-to-Vehicle

ICS	Implementation Conformance Statement

ITS	Intelligent Transportation Systems

ITS-G5	5 GHz wireless communication

IUT	Implementation Under Test

OBU	On Board Unit

PICS	Protocol Implementation Conformance Statement

RSU	Road Side Unit

SCS	System Conformance Statement

SUT	System Under Test

VANET	Vehicular Ad Hoc Networks

V2I	Vehicle-to-Infrastructure

V2V	Vehicle-to-Vehicle

V2X	V2V and/or V2I

# 4	Conformance requirement concerning PICS

If it claims to conform to the present document, the actual PICS proforma to be filled in by a supplier shall be technically equivalent to the text of the PICS proforma given in annex A, and shall preserve the numbering/naming and ordering of the proforma items.

An ICS which conforms to the present document shall be a conforming PICS proforma completed in accordance with the instructions for completion given in clause A.1.

Annex A (normative):
CAM PICS Proforma

Notwithstanding the provisions of the copyright clause related to the text of the present document, ETSI grants that users of the present document may freely reproduce the CAM PICS proforma in this annex so that it can be used for its intended purposes and may further publish the completed CAM PICS.

# A.1	Guidance for completing the ICS proforma

## A.1.1	Purposes and structure

The purpose of this PICS proforma is to provide a mechanism whereby a supplier of an implementation of the requirements defined in TS 102 637-2 [2] may provide information about the implementation in a standardized manner.

The PICS proforma is subdivided into clauses for the following categories of information:

-	guidance for completing the ICS proforma;

-	identification of the implementation;

-	identification of the TS 102 637-2 [2];

-	global statement of conformance;

-	PICS proforma tables.

## A.1.2	Abbreviations and conventions

The ICS proforma contained in this annex is comprised of information in tabular form in accordance with the guidelines presented in ISO/IEC 9646-7 [6].

Item column

The item column contains a number which identifies the item in the table.

Item description column

The item description column describes in free text each respective item (e.g. parameters, timers, etc.). It implicitly means "is <item description> supported by the implementation?".

Status column

The following notations, defined in ISO/IEC 9646-7 [6], are used for the status column:

m	mandatory - the capability is required to be supported.

o	optional - the capability may be supported or not.

n/a	not applicable - in the given context, it is impossible to use the capability.

x	prohibited (excluded) - there is a requirement not to use this capability in the given context.

o.i	qualified optional - for mutually exclusive or selectable options from a set. "i" is an integer which identifies an unique group of related optional items and the logic of their selection which is defined immediately following the table.

ci	conditional - the requirement on the capability ("m", "o", "x" or "n/a") depends on the support of other optional or conditional items. "i" is an integer identifying an unique conditional status expression which is defined immediately following the table.

i	irrelevant (out-of-scope) - capability outside the scope of the reference specification. No answer is requested from the supplier.

NOTE 1:	This use of "i" status is not to be confused with the suffix "i" to the "o" and "c" statuses above.

Reference column

The reference column makes reference to TS 102 637-2 [2]., except where explicitly stated otherwise.

Support column

The support column shall be filled in by the supplier of the implementation. The following common notations, defined in ISO/IEC 9646-7 [6], are used for the support column:

Y or y	supported by the implementation.

N or n	not supported by the implementation.

N/A, n/a or - 	no answer required (allowed only if the status is n/a, directly or after evaluation of a conditional status).

NOTE 2:	As stated in ISO/IEC 9646-7 [6], support for a received PDU requires the ability to parse all valid parameters of that PDU. Supporting a PDU while having no ability to parse a valid parameter is non-conformant. Support for a parameter on a PDU means that the semantics of that parameter are supported.

Values allowed column

The values allowed column contains the type, the list, the range, or the length of values allowed. The following notations are used:

-	range of values:	<min value> .. <max value>

example: 	5 .. 20

-	list of values:	<value1>, <value2>, ..., <valueN>

example: 	2 ,4 ,6 ,8, 9

example:	'1101'B, '1011'B, '1111'B

example:	'0A'H, '34'H, '2F'H

-	list of named values:	<name1>(<val1>), <name2>(<val2>), ..., <nameN>(<valN>)

example:	reject(1), accept(2)

-	length:	size (<min size> .. <max size>)

example:	size (1 .. 8)

Values supported column

The values supported column shall be filled in by the supplier of the implementation. In this column, the values or the ranges of values supported by the implementation shall be indicated.

References to items

For each possible item answer (answer in the support column) within the ICS proforma a unique reference exists, used, for example, in the conditional expressions. It is defined as the table identifier, followed by a solidus character "/", followed by the item number in the table. If there is more than one support column in a table, the columns are discriminated by letters (a, b, etc.), respectively.

EXAMPLE 2:	A.5/4 is the reference to the answer of item 4 in table 5 of annex A.

EXAMPLE 3:	A.6/3b is the reference to the second answer (i.e. in the second support column) of item 3 in table 6 of annex A.

Prerequisite line

A prerequisite line takes the form: Prerequisite: <predicate>.

A prerequisite line after a clause or table title indicates that the whole clause or the whole table is not required to be completed if the predicate is FALSE.

## A.1.3	Instructions for completing the ICS proforma

The supplier of the implementation shall complete the ICS proforma in each of the spaces provided. In particular, an explicit answer shall be entered, in each of the support or supported column boxes provided, using the notation described in clause A.1.2.

If necessary, the supplier may provide additional comments in space at the bottom of the tables or separately.

More detailed instructions are given at the beginning of the different clauses of the ICS proforma.

# A.2	Identification of the implementation

Identification of the Implementation Under Test (IUT) and the system in which it resides (the System Under Test (SUT)) should be filled in so as to provide as much detail as possible regarding version numbers and configuration options.

The product supplier information and client information should both be filled in if they are different.

A person who can answer queries regarding information supplied in the ICS should be named as the contact person.

## A.2.1	Date of the statement

## A.2.2	Implementation Under Test (IUT) identification

IUT name:

IUT version:

## A.2.3	System Under Test (SUT) identification

SUT name:

Hardware configuration:

Operating system:

## A.2.4	Product supplier

Name:

Address:

Telephone number:

Facsimile number:

E-mail address:

Additional information:

## A.2.5	Client (if different from product supplier)

Name:

Address:

Telephone number:

Facsimile number:

E-mail address:

Additional information:

## A.2.6	ICS contact person

(A person to contact if there are any queries concerning the content of the ICS)

Name:

Telephone number:

Facsimile number:

E-mail address:

Additional information:

# A.3	Identification of the protocol

This ICS proforma applies to the following standard:

ETSI TS 102 637-2; "Intelligent Transport Systems (ITS); Vehicular Communications; Basic Set of Applications; Part 2: Specification of Cooperative Awareness Basic Service".

# A.4	Global statement of conformance

Are all mandatory capabilities implemented? (Yes/No)

NOTE:	Answering "No" to this question indicates non-conformance to the CAM standard specification. Non-supported mandatory capabilities are to be identified in the ICS, with an explanation of why the implementation is non-conforming, on pages attached to the ICS proforma.

# A.5	Tables

## A.5.1	ITS Station type

The supplier of the implementation shall state the support of type of the implementation according to the following station type, in the table below.

Table A.1: ITS Station type

| Item | Type | Reference | Status | Support |
| --- | --- | --- | --- | --- |
| 1 | central ITS station | 6.1[1] | n/a |  |
| 2 | road side ITS station | 6.1[1] | o.101 |  |
| 3 | Vehicle ITS station | 6.1[1] | o.101 |  |
| o.101: It is mandatory to support only one of these type. |  |  |  |  |

## A.5.2	CAM vehicle profiles

The supplier of the implementation shall state the support of the implementation for each of the following vehicle type, in the table below.

Table A.2: CAM vehicle profiles supported

| Prerequisite: A.3/3 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Item | CAM vehicle profile | Reference | Status | Support |
| 1 | basicVehicle | 7.2 | o.301 |  |
| 2 | emergencyVehicle | 7.2 | o.301 |  |
| 3 | publicTransportVehicle | 7.2 | o.301 |  |
| o.301: It is mandatory to support only one of these type. |  |  |  |  |

## A.5.3	CAM road side station profile

The supplier of the implementation shall state the support of the implementation for each of the following road side station profile, in the table below.

Table A.4: CAM road side station profile supported

| Prerequisite: A.5/2 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Item | CAM road side station profile | Reference | Status | Support |
| 1 | basicIRS | 7.2 | m |  |

## A.5.4	CAM Basic Functionality

The supplier of the implementation shall state the support of the implementation for each of the following functionalities, in the table below.

Table A. 6: CAM Basic Functionality supported

| Item | Vehicle Type | Reference | Status | Support |
| --- | --- | --- | --- | --- |
| 1 | Generation of CAM Messages | 4.1 | m |  |
| 2 | Transmission of CAM Messages | 4.1 | m |  |
| 3 | Reception of CAM Messages | 4.1 | m |  |

## A.5.5	CAM Interfaces to data provisioning services

The supplier of the implementation shall state the support of the implementation for each of the following interfaces to data provisioning services, in the table below.

Table A. 7: CAM Interfaces supported

| Item | Vehicle Type | Reference | Status | Support |
| --- | --- | --- | --- | --- |
| 1 | Station State Monitoring | 6.2 | o |  |
| 2 | Mobile Station Dynamic Monitoring | 6.2 | o |  |
| 3 | Time Management | 6.2 | o |  |
| 4 | LDM Management | 6.2 | o |  |

## A.5.6	CAM Message

Table A.8: Fields of CAM message supported

| It. | Name of field | Ref. | Status | Sp. | Value allowed | Value sp. |
| --- | --- | --- | --- | --- | --- | --- |
| 1 | protocolVersion | 7 | m |  | 0..255 |  |
| 2 | messageID | 7 | m |  | 0..255 |  |
| 3 | generationTime | 7 | m |  | 0.. 281474976710655 |  |
| 4 | stationed | 7 | m |  | 0..4294967295 |  |
| 5 | referencePosition | 7 | m |  | referencePosition |  |
| 6 | stationCharacteristics | 7 | m |  | stationCharacteristics |  |
| 7 | taggedList | 7 | m |  | taggedList |  |

### A.5.6.1	stationCharacteristics element

Table A.9: stationCharacteristics

| Prerequisite: A.10/6 |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| It. | Name of field | Ref. | Status | Sp. | Value allowed | Value sp. |
| 1 | mobile | 7 | m |  | Boolean |  |
| 2 | private | 7 | m |  | Boolean |  |
| 3 | physicalRelevant | 7 | m |  | Boolean |  |

### A.5.6.2	referencePosition element

Table A.11: referencePosition

| Prerequisite: A.12/5 |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Column Prerequisite |  |  | A.13/1 |  | A.14/2 |  | A.15/3 |  | A.16/1 |  |
| It. | Name of field | Ref. | Status | Sp. | Status | Sp. | Status | Sp. | Status | Sp. |
| 1 | longitude | 7.2 | m |  | m |  | m |  | m |  |
| 2 | latitude | 7.2 | m |  | m |  | m |  | m |  |
| 3 | elevation | 7.2 | m |  | m |  | m |  | m |  |
| 4 | heading | 7.2 | o |  | o |  | o |  | x |  |
| 5 | streetName | 7.2 | o |  | o |  | o |  | o |  |
| 6 | positionConfidence | 7.2 | c.1601 |  | c.1601 |  | c.1601 |  | x |  |
| 7 | elevationConfidence | 7.2 | c.1601 |  | c.1601 |  | c.1601 |  | x |  |
| 8 | roadSegmentID | 7.2 | o |  | o |  | o |  | o |  |
| c.1601: IF value of A.17/1 is TRUE THEN m ELSE o |  |  |  |  |  |  |  |  |  |  |

#### A.5.6.2.1	referencePosition sub-elements

Table A.18: Longitude

| Prerequisite: A.19/1 |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| It. | Name of field | Ref. | Status | Sp. | Value allowed | Value sp. |
| 1 | isEast | Annex A | m |  | Boolean |  |
| 2 | Degree | Annex A | m |  | 0..1800000000 |  |

Table A.20: Latitude

| Prerequisite: A.21/2 |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| It. | Name of field | Ref. | Status | Sp. | Value allowed | Value sp. |
| 1 | isSouth | Annex A | m |  | Boolean |  |
| 2 | Degree | Annex A | m |  | 0..900000000 |  |

Table A.22: Elevation

| Prerequisite: A.23/3 |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| It. | Name of field | Ref. | Status | Sp. | Value allowed | Value sp. |
| 1 | Elevation | Annex A | m |  | -10000..16767215 |  |

Table A.24: Heading

| Prerequisite: A.25/4 |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| It. | Name of field | Ref. | Status | Sp. | Value allowed | Value sp. |
| 1 | Heading | Annex A | m |  | Direction |  |

Table A.26: StreetName

| Prerequisite: A.27/5 |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| It. | Name of field | Ref. | Status | Sp. | Value allowed | Value sp. |
| 1 | StreetName | Annex A | m |  | 32 characters max 1 character min |  |

Table A.28: ElevationConfidence

| Prerequisite: A.29/7 |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| It. | Name of field | Ref. | Status | Sp. | Value allowed | Value sp. |
| 1 | elevationConfidence | 7.2 | m |  | Confidence |  |

Table A.30: RoadSegmentID

| Prerequisite: A.31/8 |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| It. | Name of field | Ref. | Status | Sp. | Value allowed | Value sp. |
| 1 | RoadSegmentID | Annex A | m |  | 0..99999999 |  |

### A.5.6.3	taggedList element

Table A.32: taggedList

| Prerequisite: A.33/7 |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Column Prerequisite |  |  | A.34/1 |  | A.35/2 |  | A.36/3 |  | A.37/1 |  |
| It. | Name of field | Ref. | Status | Sp. | Status | Sp. | Status | Sp. | Status | Sp. |
| 1 | VehicleType | Annex A | m |  | m |  | m |  | x |  |
| 2 | PublicVehicleType | Annex A | x |  | x |  | m |  | x |  |
| 3 | LightBarInUse | Annex A | x |  | o |  | x |  | x |  |
| 4 | SireneInUse | Annex A | x |  | o |  | x |  | x |  |
| 5 | EmergencyResponseType | Annex A | x |  | m |  | x |  | x |  |
| 6 | StationLength | Annex A | m |  | m |  | m |  | x |  |
| 7 | StationLengthConfidence | 7.2 | m, see Note |  | m, see Note |  | m, see Note |  | x |  |
| 8 | StationWidth | Annex A | m |  | m |  | m |  | x |  |
| 9 | StationWidthConfidence | 7.2 | m, see Note |  | m, see Note |  | m, see Note |  | x |  |
| 10 | VehicleSpeed | Annex A | m |  | m |  | m |  | x |  |
| 11 | VehicleSpeedConfidence | 7.2 | m |  | m |  | m |  | x |  |
| 12 | LongAcceleration | Annex A | m |  | m |  | m |  | x |  |
| 13 | LongAccelerationConfidence | 7.2 | m |  | m |  | m |  | x |  |
| 14 | YawRate | Annex A | m |  | m |  | m |  | x |  |
| 15 | YawRateConfidence | 7.2 | m |  | m |  | m |  | x |  |
| 16 | AccelerationControl | Annex A | m |  | m |  | m |  | x |  |
| 17 | ExteriorLights | Annex A | m |  | m |  | m |  | x |  |
| 18 | CauseCode | Annex A | x |  | x |  | x |  | x |  |
| 19 | AmbientAirTemperature | Annex A | x |  | x |  | x |  | x |  |
| 20 | Speed | Annex A | x |  | x |  | x |  | x |  |
| 21 |  |  |  |  |  |  |  |  |  |  |
| 22 | PTLineDescription | Annex A | x |  | x |  | o |  | x |  |
| 23 | TurnAdvice | Annex A | o |  | o |  | o |  | x |  |
| 24 | DistanceToStopLine | Annex A | o |  | o |  | o |  | x |  |
| 25 | DistanceToStopLineConfidence | 7.2 | o |  | o |  | o |  | x |  |
| 26 | Occupancy | Annex A | o |  | o |  | o |  | x |  |
| 27 | ScheduleDeviation | Annex A | x |  | x |  | o |  | x |  |
| 28 | TrafficLightPriority | Annex A | x |  | x |  | o |  | x |  |
| 29 | DoorOpen | Annex A | o |  | o |  | m |  | x |  |
| 30 | DataReference | Annex A | x |  | x |  | x |  | x |  |
| 31 | PosConfidenceEllipse | Annex A | m |  | m |  | m |  | x |  |
| 32 | Curvature | Annex A | m |  | m |  | m |  | x |  |
| 33 | CurvatureChange | Annex A | o |  | o |  | o |  | x |  |
| 34 | CurvatureConfidence | 7.2 | m |  | m |  | m |  | x |  |
| 35 | WiperSystemFront | Annex A | x |  | x |  | x |  | x |  |
| 36 | CrashStatus | Annex A | m, see Note |  | m, see Note |  | m, see Note |  | x |  |
| 37 | HeadingConfidence | 7.2 | m |  | m |  | m |  | x |  |
| 38 | DangerousGoods | Annex A | m, see Note |  | m, see Note |  | m, see Note |  | x |  |
| Note: Support of the following features is mandatory: StationLengthConfidence, StationWidthConfidence, DoorOpen, CrashStatus, DangerousGoods. Presence of the fields in the message depends on the situation. |  |  |  |  |  |  |  |  |  |  |

#### A.5.6.3.1	taggedList  sub-elements

Table A.38: AccelerationControl

| Prerequisite: A.39/16 |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| It. | Name of field | Ref. | Status | Sp. | Value allowed | Value sp. |
| 1 | brakePedal | Annex A | m |  | Boolean |  |
| 2 | throttlePedal | Annex A | m |  | Boolean |  |
| 3 | cruiseControl | Annex A | m |  | Boolean |  |
| 4 | acc | Annex A | m |  | Boolean |  |
| 5 | limiter | Annex A | m |  | Boolean |  |
| 6 | brakeAssist | Annex A | m |  | Boolean |  |

Table A.40: AmbientAirTemperature

| Prerequisite: A.41/19 |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| It. | Name of field | Ref. | Status | Sp. | Value allowed | Value sp. |
| 1 | AmbientAirTemperature | Annex A | m |  | Temperature |  |

Table A.42: CauseCode

| Prerequisite: A.43/18 |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| It. | Name of field | Ref. | Status | Sp. | Value allowed | Value sp. |
| 1 | CauseCode | Annex A | m |  | 0..100 |  |

Table A.44: CrashStatus

| Prerequisite: A.45/36 |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| It. | Name of field | Ref. | Status | Sp. | Value allowed | Value sp. |
| 1 | CrashStatus | Annex A | m |  | Boolean |  |

Table A.46: Curvature

| Prerequisite: A.47/32 |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| It. | Name of field | Ref. | Status | Sp. | Value allowed | Value sp. |
| 1 | Curvature | Annex A | m |  | -32765..32765 |  |

Table A.48: CurvatureChange

| Prerequisite: A.49/33 |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| It. | Name of field | Ref. | Status | Sp. | Value allowed | Value sp. |
| 1 | CurvatureChange | Annex A | m |  | -1023..1023 |  |

Table A.50: CurvatureConfidence

| Prerequisite: A.51/34 |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| It. | Name of field | Ref. | Status | Sp. | Value allowed | Value sp. |
| 1 | CurvatureConfidence | Annex A | m |  | Confidence |  |

Table A.52: DataReference

| Prerequisite: A.53/30 |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| It. | Name of field | Ref. | Status | Sp. | Value allowed | Value sp. |
| 1 | DataReference | Annex A | m |  | 128 characters max 1 character min |  |

Table A.54: DangerousGoods

| Prerequisite: A.55/38 |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| It. | Name of field | Ref. | Status | Sp. | Value allowed | Value sp. |
| 1 | DangerousGoods | Annex A | m |  | 0..8191 |  |

Table A.56: DistanceToStopLine

| Prerequisite: A.57/24 |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| It. | Name of field | Ref. | Status | Sp. | Value allowed | Value sp. |
| 1 | DistanceToStopLine | Annex A | m |  | Distance |  |

Table A.58: DistanceToStopLineConfidence

| Prerequisite: A.59/25 |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| It. | Name of field | Ref. | Status | Sp. | Value allowed | Value sp. |
| 1 | DistanceToStopLineConfidence | Annex A | m |  | Confidence |  |

Table A.60: DoorOpen

| Prerequisite: A.61/29 |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| It. | Name of field | Ref. | Status | Sp. | Value allowed | Value sp. |
| 1 | driver | Annex A | m |  | Boolean |  |
| 2 | passenger | Annex A | m |  | Boolean |  |
| 3 | maintenance | Annex A | m |  | Boolean |  |
| 4 | luggage | Annex A | m |  | Boolean |  |

Table A.62: EmergencyResponseType

| Prerequisite: A.63/4 |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| It. | Name of field | Ref. | Status | Sp. | Value allowed | Value sp. |
| 1 | EmergencyResponseType | Annex A | m |  | 0..3 |  |

Table A.64: ExteriorLights

| Prerequisite: A.65/17 |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| It. | Name of field | Ref. | Status | Sp. | Value allowed | Value sp. |
| 1 | lowBeamHeadlightsOn | Annex A | m |  | Boolean |  |
| 2 | highBeamHeadlightsOn | Annex A | m |  | Boolean |  |
| 3 | leftTurnSignalOn | Annex A | m |  | Boolean |  |
| 4 | rightTurnSignalOn | Annex A | m |  | Boolean |  |
| 5 | automaticLightControlOn | Annex A | m |  | Boolean |  |
| 6 | daytimeRunningLightsOn | Annex A | m |  | Boolean |  |
| 7 | fogLightOn | Annex A | m |  | Boolean |  |
| 8 | parkingLightsOn | Annex A | m |  | Boolean |  |

Table A.66: HeadingConfidence

| Prerequisite: A.67/37 |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| It. | Name of field | Ref. | Status | Sp. | Value allowed | Value sp. |
| 1 | HeadingConfidence | Annex A | m |  | Confidence |  |

Table A.68: LightBarInUse

| Prerequisite: A.69/3 |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| It. | Name of field | Ref. | Status | Sp. | Value allowed | Value sp. |
| 1 | LightBarInUse | Annex A | m |  | SimpleSystemState |  |

Table A.70: LongAcceleration

| Prerequisite: A.71/12 |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| It. | Name of field | Ref. | Status | Sp. | Value allowed | Value sp. |
| 1 | LongAcceleration | Annex A | m |  | -2000..2000 |  |

Table A.72: LongAccelerationConfidence

| Prerequisite: A.73/13 |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| It. | Name of field | Ref. | Status | Sp. | Value allowed | Value sp. |
| 1 | LongAccelerationConfidence | Annex A | m |  | Confidence |  |

Table A.74: Occupancy

| Prerequisite: A.75/26 |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| It. | Name of field | Ref. | Status | Sp. | Value allowed | Value sp. |
| 1 | Occupancy | Annex A | m |  | 0..255 |  |

Table A.76: PosConfidenceEllipse

| Prerequisite: A.77/31 |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| It. | Name of field | Ref. | Status | Sp. | Value allowed | Value sp. |
| 1 | semiMajorConfidence | Annex A | m |  | PositionConfidence |  |
| 2 | semiMinorConfidence | Annex A | m |  | PositionConfidence |  |
| 3 | semiMajorOrientation | Annex A | m |  | Direction |  |

Table A.78: PTLineDescription

| Prerequisite: A.79/22 |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| It. | Name of field | Ref. | Status | Sp. | Value allowed | Value sp. |
| 1 | courseOfJourney | Annex A | m |  | CourseOfJourney |  |
| 2 | lineRef | Annex A | m |  | LineRef |  |
| 3 | routeRef | Annex A | m |  | RouteRef |  |

Table A.80: PublicVehicleType

| Prerequisite: A.81/2 |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| It. | Name of field | Ref. | Status | Sp. | Value allowed | Value sp. |
| 1 | PublicVehicleType | Annex A | m |  | 0..255 |  |

Table A.82: ScheduleDeviation

| Prerequisite: A.83/27 |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| It. | Name of field | Ref. | Status | Sp. | Value allowed | Value sp. |
| 1 | ScheduleDeviation | Annex A | m |  | -900..3600 |  |

Table A.84: SireneInUse

| Prerequisite: A.85/4 |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| It. | Name of field | Ref. | Status | Sp. | Value allowed | Value sp. |
| 1 | SireneInUse | Annex A | m |  | SimpleSystemState |  |

Table A.86: Speed

| Prerequisite: A.87/20 or A.88/1 |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| It. | Name of field | Ref. | Status | Sp. | Value allowed | Value sp. |
| 1 | Speed | Annex A | m |  | -32765..32765 |  |

Table A.89: StationLength

| Prerequisite: A.90/6 |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| It. | Name of field | Ref. | Status | Sp. | Value allowed | Value sp. |
| 1 | StationLength | Annex A | m |  | Dimension |  |

Table A.91: StationLengthConfidence

| Prerequisite: A.92/7 |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| It. | Name of field | Ref. | Status | Sp. | Value allowed | Value sp. |
| 1 | StationLengthConfidence | Annex A | m |  | Confidence |  |

Table A.93: StationWidth

| Prerequisite: A.94/8 |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| It. | Name of field | Ref. | Status | Sp. | Value allowed | Value sp. |
| 1 | StationWidth | Annex A | m |  | Dimension |  |

Table A.95: StationWidthConfidence

| Prerequisite: A.96/9 |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| It. | Name of field | Ref. | Status | Sp. | Value allowed | Value sp. |
| 1 | StationWidthConfidence | Annex A | m |  | Confidence |  |

Table A.97: TrafficLightPriority

| Prerequisite: A.98/28 |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| It. | Name of field | Ref. | Status | Sp. | Value allowed | Value sp. |
| 1 | TrafficLightPriority | Annex A | m |  | Priority |  |

Table A.99: TurnAdvice

| Prerequisite: A.100/23 |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| It. | Name of field | Ref. | Status | Sp. | Value allowed | Value sp. |
| 1 | direction | Annex A | m |  | TurnDirection |  |
| 2 | distance | Annex A | m |  | Distance |  |

Table A.101: VehicleSpeed

| Prerequisite: A.102/10 |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| It. | Name of field | Ref. | Status | Sp. | Value allowed | Value sp. |
| 1 | VehicleSpeed | Annex A | m |  | Speed |  |

Table A.103: VehicleSpeedConfidence

| Prerequisite: A.104/11 |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| It. | Name of field | Ref. | Status | Sp. | Value allowed | Value sp. |
| 1 | VehicleSpeedConfidence | Annex A | m |  | Confidence |  |

Table A.105: VehicleType

| Prerequisite: A.106/1 |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| It. | Name of field | Ref. | Status | Sp. | Value allowed | Value sp. |
| 1 | VehicleType | Annex A | m |  | 0..255 |  |

Table A.107: WiperSystemFront

| Prerequisite: A.108/35 |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| It. | Name of field | Ref. | Status | Sp. | Value allowed | Value sp. |
| 1 | WiperSystemFront | Annex A | m |  | 0..4 |  |

Table A.109: YawRate

| Prerequisite: A.110/14 |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| It. | Name of field | Ref. | Status | Sp. | Value allowed | Value sp. |
| 1 | YawRate | Annex A | m |  | -32765..32765 |  |

A Table A.111: YawRateConfidence

| Prerequisite: A.112/15 |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| It. | Name of field | Ref. | Status | Sp. | Value allowed | Value sp. |
| 1 | YawRateConfidence | Annex A | m |  | Confidence |  |

#### A.5.6.3.2	PTLineDescription sub-elements

Table A.113: CourseOfJourney

| Prerequisite: A.114/1 |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| It. | Name of field | Ref. | Status | Sp. | Value allowed | Value sp. |
| 1 | CourseOfJourney | Annex A | m |  | 32 characters max |  |

Table A.115: LineRef

| Prerequisite: A.116/2 |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| It. | Name of field | Ref. | Status | Sp. | Value allowed | Value sp. |
| 1 | LineRef | Annex A | m |  | 32 characters max |  |

Table A.117: RouteRef

| Prerequisite: A.118/3 |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| It. | Name of field | Ref. | Status | Sp. | Value allowed | Value sp. |
| 1 | RouteRef | Annex A | m |  | 32 characters max |  |

### A.5.6.4	common sub-elements

Table A.119: Confidence

| Prerequisite: A.120/1 or A.121/1 or A.122/1 or A.123/1 or A.124/1 or A.125/1 or A.126/1 or A.127/1 or A.128/1 or A.129/1 |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| It. | Name of field | Ref. | Status | Sp. | Value allowed | Value sp. |
| 1 | Confidence | Annex A | m |  | 0..15 |  |

Table A.130: Dimension

| Prerequisite: A.131/1 or A.132/1 |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| It. | Name of field | Ref. | Status | Sp. | Value allowed | Value sp. |
| 1 | Dimension | Annex A | m |  | 0..16383 |  |

Table A.133: Direction

| Prerequisite: A.134/1 or A.135/1 |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| It. | Name of field | Ref. | Status | Sp. | Value allowed | Value sp. |
| 1 | Direction | Annex A | m |  | 0..28800 |  |

Table A.136: Distance

| Prerequisite: A.137/1 or A.138/2 |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| It. | Name of field | Ref. | Status | Sp. | Value allowed | Value sp. |
| 1 | Distance | Annex A | m |  | 0..65535 |  |

Table A.139: PositionConfidence

| Prerequisite: A.140/6 or A.141/1 or A.142/2 |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| It. | Name of field | Ref. | Status | Sp. | Value allowed | Value sp. |
| 1 | PositionConfidence | Annex A | m |  | Confidence |  |

Table A.143: Priority

| Prerequisite: A.144/1 |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| It. | Name of field | Ref. | Status | Sp. | Value allowed | Value sp. |
| 1 | Priority | Annex A | m |  | 0..7 |  |

Table A.145: SimpleSystemState

| Prerequisite: A.146/1 or A.147/1 |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| It. | Name of field | Ref. | Status | Sp. | Value allowed | Value sp. |
| 1 | SimpleSystemState | Annex A | m |  | 0..3 |  |

Table A.148: Temperature

| Prerequisite: A.149/1 |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| It. | Name of field | Ref. | Status | Sp. | Value allowed | Value sp. |
| 1 | Temperature | Annex A | m |  | -40..215 |  |

Table A.150: TurnDirection

| Prerequisite: A.151/1 |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| It. | Name of field | Ref. | Status | Sp. | Value allowed | Value sp. |
| 1 | TurnDirection | Annex A | m |  | 0..7 |  |

## A.5.7	Protocol parameters

### A.5.7.1	Timing requirements

The supplier of the implementation shall provide information about the timing requirements.

Table A.152: Timing requirements

| It. | Name of field | Ref. | Status | Sp. | Value allowed | Value sp. |
| --- | --- | --- | --- | --- | --- | --- |
| 1 | Maximum interval of CAM generations | 5.1 | m |  | 1 s if no Use Case has been selected |  |
| 2 | Minimum interval of CAM generations | 5.1 | m |  | 0.1 s |  |
| 3 | Max processing time of CAM construction | 5.1 | m |  | 50 ms |  |
| 4 | Time to check generation rules | Annex A | o |  | 100 ms |  |
| 5 | Duration of doors closure for situational mandatory TaggedValue | Annex A | c.15201 |  | 30 s |  |
| c.15201: IF A.153/3 and A.154/29 THEN m ELSE n/a |  |  |  |  |  |  |

# History

| Document history |  |  |
| --- | --- | --- |
| V0.0.1 | May 2010 | First draft |
| V0.0.2 | May 2010 | Second draft |
| V0.0.3 | June 2010 | Third draft |
| V0.0.4 | July 2010 | Fourth draft |
| V0.0.5 | October 2010 | Review |